3GPP TSG RAN WG1 #112bis-e		R1-2302774

e-Meeting, April 17th - April 26th, 2023

Source:	OPPO

Title:	Discussion on self-evaluation methodology for IMT-2020 satellite radio interface

Agenda Item:	9.15.1

Document for:	Discussion and Decision

Introduction

In RANP meeting #99, a new study item [1] was agreed to provide the self-evaluation results towards IMT-2020 submission to ITU-R WP 4B against the technical performance requirements defined by Report ITU-R M.2514 [2], which should be based on Rel-17 NTN (including both NR NTN and IoT NTN). Detailed objectives of this study item include:

- Complete all required submission templates as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc] - Provide self-evaluation results against technical performance requirements for eMBB-s as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2], including - Peak data rate - Peak spectral efficiency - User experienced data rate - 5th percentile user spectral efficiency - Average spectral efficiency - Area traffic capacity - Latency, including user plane latency and control plane latency - Energy efficiency, including both network and device - Mobility - Mobility interruption time - Provide self-evaluation results against technical performance requirements for mMTC-s as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2], including - Connection density - Provide self-evaluation results against technical performance requirements for HRC-s as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2], including - Reliability - Provide self-evaluation results for other requirements (including bandwidth) as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2, RAN4] IoT NTN will at least target self-evaluation against bullets c) and e) technical requirements, and NR NTN will target self-evaluation against all technical requirements (in bullets b) to e)). This study shall start with evaluating features that are supported by Rel-17 NTN (NR NTN + IoT NTN), as relevant for the above aspects.

In this contribution, we provide our considerations on self-evaluation methodology for IMT-2020 satellite radio interface, including general technical features, performance requirements and evaluation assumptions/ methodology for both NR NTN and IoT NTN.

Discussion

According to the guidance in Report ITU-R M.2514, the new satellite-based service categories are reflected in the usage scenarios for the satellite component of IMT-2020 including enhanced Mobile Broadband via satellite (eMBB-s), massive Machine Type Communications via satellite (mMTC-s) and High Reliability Communications via satellite (HRC-s). The IMT-2020 satellite radio interface is mainly expected to support eMBB-s and mMTC-s services, and also expected to support HRC-s service.

Self-evaluation of eMBB-s

This section discusses general technical features, performance requirements and evaluation assumptions/ methodology for eMBB-s self-evaluation.

General technical features for eMBB-s

The following features as shown in Table 1 can be considered as a starting point for eMBB-s evaluation.

Table 1. Technical features for eMBB-s

| General features | FR1 (2 GHz) |
| --- | --- |
| Rel-17 NR NTN on single-band | Paired spectrum: FDD |

The above features are based on the 3GPP Rel-17 NR NTN and are considered for Rural-eMBB-s usage scenario, i.e., an environment with large and continuous area coverage, supporting pedestrian and vehicular users with high data rates, to employ eMBB-s evaluation for IMT-2020 submission.

Proposal 1: Rel-17 NR NTN on single-band is considered as a starting point for eMBB-s evaluation towards IMT-2020 submission.

Performance requirements for eMBB-s

According to the SID and Report ITU-R M.2514, the following performance requirements and the corresponding high-level assessment method summarized in Table 2 are considered for eMBB-s evaluation.

Table 2. Performance requirements for eMBB-s

| Performance requirements | High-level assessment method |
| --- | --- |
| Peak data rate | Analytical |
| Peak spectral efficiency | Analytical |
| User experienced data rate | Simulation and Analytical |
| 5th percentile user spectral efficiency | Simulation |
| Average spectral efficiency | Simulation |
| Area traffic capacity | Simulation and Analytical |
| Latency, including user plane latency and control plane latency | Analytical and Inspection |
| Energy efficiency, including both network and device | Inspection |
| Mobility | Simulation |
| Mobility interruption time | Analytical |

Proposal 2: Performance requirements in Table 2 should be considered for eMBB-s evaluation towards IMT-2020 submission.

Evaluation assumptions/ methodology for eMBB-s

System-level simulation is considered for eMBB-s evaluation. The Set-1 satellite with satellite orbit of LEO-600 as well as handheld UE defined in TR38.821 (Table 6.1.1.1-1 and Table 6.1.1.1-3) can be considered as a starting point. The beam layout definition for single satellite simulation in TR38.821 Table 6.1.1.1-4 and Table 6.1.1.1-6 can also be considered. In addition, the following evaluation parameters in Table 3 and Table 4 for DL and UL transmissions can be considered.

Table 3. Evaluation parameters for DL eMBB-s

| DL FDD – FR1 |  |  | Rel-17 NR NTN |
| --- | --- | --- | --- |
| Carrier frequency |  |  | 2 GHz |
| Waveform |  |  | OFDM based |
| Multiple access |  |  | OFDMA |
| Modulation |  |  | Up to 64QAM |
| Coding on PDSCH |  |  | LDPC |
| Numerology | Slot/non-slot |  | 14 OFDM symbol slot |
|  | SCS |  | 15kHz |
| Simulation bandwidth |  |  | 30 MHz |
| Frame structure |  |  | Slot Format 0 (all downlink) for all slots |
| MIMO |  | Transmission mode | Closed loop SU-MIMO adaptation |
|  |  | Codebook | Detailed configuration to be reported |
|  |  | PRB bundling | 4PRB, or allocated PRB; (It impacts DMRS channel estimation, but does not limit the scheduling unit.) |
|  |  | SU dimension | Rank adaptation up to 8 layers (depending on UE antenna elements) |
|  |  | Codeword (CW)-to-layer mapping | For 1~4 layers, CW1; For 5 layers or more, two CWs |
|  |  | DMRS transmission for overhead | Up to 2 OFDM symbols per slot according to the number of layers |
|  |  | CSI-RS transmission | Non-precoded/precoded according to CSI measurement (with codebook selection) and interference measurement |
|  |  | CSI feedback | For slot/non-slot: PMI, CQI: every [5] slot; RI: every [5] slot, CRI: every [5] slot Subband based |
|  |  | Interference measurement | SU-CQI |
| HARQ |  | HARQ-ACK feedback | Option 1: feedback enabled Option 2: feedback disabled |
|  |  | Re-transmission delay | Companies to report the minimum delay assumed in the simulation |
| PDCCH resource sharing |  |  | Resource sharing with PDSCH might be considered in overhead calculation |
| Overhead |  |  | SS/PBCH block ([1 SS/PBCH block in every 20ms]) CSI-RS, DMRS and TRS, CSI-IM (if used) PDCCH Companies to report the overhead in the simulation |

Table 4. Evaluation parameters for UL eMBB-s

| UL FDD – FR1 |  |  | NR |
| --- | --- | --- | --- |
| Carrier frequency |  |  | 2 GHz |
| Waveform |  |  | OFDM based |
| Multiple access |  |  | OFDMA Companies report power back-off model if used. |
| Modulation |  |  | Up to 64QAM |
| Coding on PDSCH |  |  | LDPC |
| Numerology |  | Slot/non-slot | 14 OFDM symbol slot |
|  |  | SCS | 15kHz |
| Simulation bandwidth |  |  | 30 MHz |
| Frame structure |  |  | Slot Format 1 (all uplink) for all slots / non-slots |
| MIMO | Transmission scheme |  | UL codebook based SU-MIMO |
|  | UL Codebook |  | - NR 2Tx/4Tx codebook |
|  | SU dimension |  | Rank adaptation up to 4 layers depending on UE antenna |
|  | Codeword (CW)-to-layer mapping |  | For 1~4 layers, CW1 |
|  | DMRS transmission for overhead |  | Up to 2 OFDM symbols per slot according to the number of layers |
|  | SRS transmission |  | Non-precoded 1 slot is configured to up to 6 OFDM symbols for SRS transmissions. |
| HARQ | Re-transmission delay |  | Companies to report the minimum delay assumed in the simulation |
| Power control schemes |  |  | Companies to report |
| Overhead |  |  | DMRS and SRS PUCCH DL feedback Companies to report the overhead in the simulation |

Proposal 3: Evaluation parameters in TR38.821, Table 3 and Table 4 can be considered as a starting point for eMBB-s evaluation towards IMT-2020 submission.

Self-evaluation of mMTC-s

This section discusses general technical features, performance requirements and evaluation assumptions/ methodology for mMTC-s self-evaluation.

General technical features for mMTC-s

The following features as shown in Table 5 can be considered as a starting point for mMTC-s evaluation.

Table 5. Technical features for mMTC-s

| General features | FR1 (2 GHz) |
| --- | --- |
| Rel-17 NB-IoT NTN | Paired spectrum: FDD |
| Rel-17 eMTC NTN | Paired spectrum: FDD |

The above features are based on the 3GPP Rel-17 IoT NTN and are considered for Rural-mMTC-s usage scenario, i.e., an environment targeting large and continuous coverage focusing on a high number of connected machine type devices, to employ mMTC-s evaluation for IMT-2020 submission.

Proposal 4: Rel-17 IoT NTN is considered as a starting point for mMTC-s evaluation towards IMT-2020 submission.

Performance requirements for mMTC-s

According to the SID and Report ITU-R M.2514, the following performance requirement and the corresponding high-level assessment method summarized in Table 6 are considered for mMTC-s evaluation.

Table 6. Performance requirement for mMTC-s

| Performance requirement | High-level assessment method |
| --- | --- |
| Connection density | Simulation |

Proposal 5: Performance requirement in Table 6 should be considered for mMTC-s evaluation towards IMT-2020 submission.

Evaluation assumptions/ methodology for mMTC-s

According to Report ITU-R M.2514, connection density is the total number of devices fulfilling a specific quality of service (QoS) per unit area (per km2). A connection density of at least 500 devices per km2 should be supported. The requirement was derived assuming up to and including a 30 MHz bandwidth.

For mMTC-s evaluation, the IoT NTN reference scenario parameters and the Set-1 satellite parameters for scenario A (GEO) defined in TR36.763 (Table 6.1-1 and Table 6.2-4) can be considered as a starting point. To evaluate mMTC-s in satellite environment, the procedure and delay modeling for NB-IoT and eMTC [5] with needed adaptions can be considered as a starting point.

Proposal 6: The procedure and delay modeling for NB-IoT and eMTC with needed adaptions can be considered as a starting point for mMTC-s evaluation towards IMT-2020 submission.

Self-evaluation of HRC-s

This section discusses general technical features, performance requirements and evaluation assumptions/ methodology for HRC-s self-evaluation.

General technical features for HRC-s

The following features as shown in Table 7 can be considered as a starting point for HRC-s evaluation.

Table 7. Technical features for HRC-s

| General features | FR1 (2 GHz) |
| --- | --- |
| Rel-17 NR NTN on single-band | Paired spectrum: FDD |

The above features are based on the 3GPP Rel-17 NR NTN and are considered for Rural-HRC-s usage scenario, i.e., an environment targeting large and continuous coverage for highly reliable communications, to employ HRC-s evaluation for IMT-2020 submission.

Proposal 7: Rel-17 NR NTN on single-band is considered as a starting point for HRC-s evaluation towards IMT-2020 submission.

Performance requirements for HRC-s

According to the SID and Report ITU-R M.2514, the following performance requirement and the corresponding high-level assessment method summarized in Table 8 are considered for HRC-s evaluation.

Table 8. Performance requirement for HRC-s

| Performance requirement | High-level assessment method |
| --- | --- |
| Reliability | Simulation |

Proposal 8: Performance requirement in Table 8 should be considered for HRC-s evaluation towards IMT-2020 submission.

Evaluation assumptions/ methodology for HRC-s

As defined in Report ITU-R M.2514, reliability is the success probability of transmitting a layer 2/3 packet within a required maximum time, which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality.

Considering that the satellite component of IMT-2020 will not address the URLLC but will cover the HRC-s usage scenario due to the inherent distance of satellites to associated terminals or earth stations, to evaluate the reliability requirement for HRC-s, the evaluation method for URLLC reliability [6] with enhancements can be considered as a starting point. In addition, the simulation parameters for eMBB-s evaluation can be reused as much as possible for HRC-s evaluation.

Proposal 9: The evaluation method for URLLC reliability with enhancements can be considered as a starting point for HRC-s evaluation towards IMT-2020 submission.

Conclusion

In this contribution, we provide our considerations on self-evaluation methodology for IMT-2020 satellite radio interface, including general technical features, performance requirements and evaluation assumptions/ methodology for both NR NTN and IoT NTN. The following proposals are made.

Proposal 1: Rel-17 NR NTN on single-band is considered as a starting point for eMBB-s evaluation towards IMT-2020 submission.

Proposal 2: Performance requirements in Table 2 should be considered for eMBB-s evaluation towards IMT-2020 submission.

Proposal 3: Evaluation parameters in TR38.821, Table 3 and Table 4 can be considered as a starting point for eMBB-s evaluation towards IMT-2020 submission.

Proposal 4: Rel-17 IoT NTN is considered as a starting point for mMTC-s evaluation towards IMT-2020 submission.

Proposal 5: Performance requirement in Table 6 should be considered for mMTC-s evaluation towards IMT-2020 submission.

Proposal 6: The procedure and delay modeling for NB-IoT and eMTC with needed adaptions can be considered as a starting point for mMTC-s evaluation towards IMT-2020 submission.

Proposal 7: Rel-17 NR NTN on single-band is considered as a starting point for HRC-s evaluation towards IMT-2020 submission.

Proposal 8: Performance requirement in Table 8 should be considered for HRC-s evaluation towards IMT-2020 submission.

Proposal 9: The evaluation method for URLLC reliability with enhancements can be considered as a starting point for HRC-s evaluation towards IMT-2020 submission.

Reference

- RP-230736, “New SID: Study on Self-Evaluation towards the 3GPP submission of an IMT-2020 Satellite Radio Interface Technology”, Ericsson, Qualcomm, Thales, MediaTek Inc.

- Report ITU-R M.2514, “Vision, requirements and evaluation guidelines for satellite radio interface(s) of IMT-2020”.

- 3GPP TR 38.821, “Solutions for NR to support non-terrestrial networks (NTN) (Release 16)”.

- 3GPP TR 36.763, “Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN) (Release 17)”.

- R1-1805934, “Considerations and evaluation results for IMT-2020 for mMTC connection density”, Huawei, HiSilicon.

- R1-1807760, “Summary on offline discussion on URLLC evaluation method and parameters for IMT-2020 self-evaluation”, Huawei.