3GPP TSG RAN WG1 #112bis-e	                    	R1-2302950

e-Meeting, April 17th – April 26th, 2023

Source: 	ZTE, Sanechips

Title:	Discussion on RedCap remaining issues

Agenda item: 	7.2

Document for:	Discussion and decision

Introduction

According to the last RAN1#112 meeting discussion [1], the following RedCap remaining issues still need to be further discussed.

- Occasion validation

- PUSCH TDRA table

For the occasion validation, some remaining cases need to be addressed in this meeting. For the PUSCH TDRA table issue, which has been discussed for NR UE. Due to the NBC issue and gNB implementation method, no further optimization in RAN1 is made. However, for RedCap UE, this issue would be different. Therefore, in this contribution, we discuss the above two issues and provide the analysis for reference.

Discussion on remaining issues

Occasion validation

Agreement Discuss the need to clarify PRACH/PUSCH/PUCCH occasion validation for the following cases: - Issue 5.1: A RedCap UE performing random access in idle/inactive state in RedCap-specific initial DL BWP without CD-SSB or NCD-SSB - Issue 5.2: A RedCap UE in connected state operating in a DL BWP without CD-SSB but with NCD-SSB. - Issue 5.3: A RedCap UE in connected state operating in a DL BWP without CD-SSB or NCD-SSB. Conclusion For TDD, RedCap UE in a BWP without any SSB should apply CD-SSB for determining the following in all RRC states: - PRACH occasion validation (in Clause 8.1, TS38.213), - MsgA PUSCH occasion validation (in Clause 8.1A, TS38.213) Note: No specification impact is expected

PUCCH

For unpaired spectrum, the UE determines the NPUCCHrepeat slots for a PUCCH transmission starting from a slot indicated to the UE as described in clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in clause 9.2.4 for SR reporting or in clause 5.2.1.4 of [6, TS 38.214] for CSI reporting and having - an UL symbol, as described in clause 11.1, or flexible symbol that is not SS/PBCH block symbol provided by startingSymbolIndex as a first symbol, and - consecutive UL symbols, as described in clause 11.1, or flexible symbols that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols

For PUCCH repetition, it is encouraged to discuss whether the SS/PBCH block symbol in yellow highlight is NCD-SSB or CD-SSB when only NCD-SSB is configured for this RedCap UE.

From our perspective, it refers to NCD-SSB for RedCap UE when NCD-SSB is configured. If the BWP contains CD-SSB, it also can refer to CD-SSB for RedCap UE. Therefore, we think the SS/PBCH block symbol here just refers to the configured one in this BWP for RedCap.

There is no need to consider the case that NR UE’s PUCCH would collide with NCD-SSB if the NR UE is not aware of the NCD-SSB, since the gNB scheduling could void the collision by the following two methods.

Method 1: gNB configures the offset between SSB and NCD-SSB as N*T ms, where N=0,1,2, ... and T=5, 10,15,20ms and so on.

Method 2: gNB would avoid the collision when it schedules NR UE’s PUCCH

Additionally, from the gNB’s perspective, it would not schedule a DL channel and a UL channel on the same flexible symbol.

Observation 1: In the flexible symbol, gNB doesn’t receive RedCap UE’s PUCCH and transmit SSB for non-RedCap UE at the same time.

Proposal 1: For RedCap UE’s PUCCH on flexible symbol, the flexible symbol is not the symbol of available SSB in this active BWP.

PUSCH

A PUSCH occasion is valid if it does not overlap in time and frequency with any valid PRACH occasion associated with either a Type-1 random access procedure or a Type-2 random access procedure. Additionally, for unpaired spectrum and for SS/PBCH blocks with indexes provided by ssb-PositionsInBurst in SIB1 or by ServingCellConfigCommon - if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion - does not precede a SS/PBCH block in the PUSCH slot, and - starts at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2 and, if channelAccessMode = "semiStatic" is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where the UE does not transmit [15, TS 37.213]. - if a UE is provided tdd-UL-DL-ConfigurationCommon, a PUSCH occasion is valid if the PUSCH occasion - is within UL symbols, or - does not precede a SS/PBCH block in the PUSCH slot, and - starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2 and, if channelAccessMode = "semiStatic" is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where the UE does not transmit [15, TS 37.213].

For PUSCH, it is similar with PUCCH. We think the SS/PBCH block symbol here just refers to the configured one in this BWP for RedCap. If the BWP contains CD-SSB, it also can refer to CD-SSB for RedCap UE.

Proposal 2: SS/PBCH block symbol for RedCap PUSCH validation refers to the applicable SSB within this active BWP.

PRACH

[Clause 8.1 of TS38.213] For unpaired spectrum, - if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last SS/PBCH block reception symbol, where Ngap is provided in Table 8.1-2 and, if channelAccessMode = "semiStatic" is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where the UE does not transmit [15, TS 37.213]. - the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon , as described in clause 4.1 - If a UE is provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if - it is within UL symbols, or - it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2, and if channelAccessMode = "semiStatic" is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where there shall not be any transmissions, as described in [15, TS 37.213] - the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon, as described in clause 4.1. For preamble format B4 [4, TS 38.211], Ngap=0.

If the PRACH validation is based on CD-SSB, for a BWP with NCD-SSB in connected mode, the PRACH would collide with the NCD-SSB in this active BWP in TDD.

If the PRACH validation is based on NCD-SSB, for a separate initial BWP in RACH procedure before connected mode, the PRACH may collide with NCD-SSB because the NCD-SSB is not known for this UE.

The above two cases can be avoided by configuring an appropriate NCD-SSB offset or by RO and NCD-SSB configuration. Of course, if the collision happens, the RACH also could be retransmitted. If the PRACH validation is based on CD-SSB and NCD-SSB, more symbols would be wasted for PRACH transmission.

Observation 2: gNB configuration could avoid collision between SSB and PRACH in TDD.

Proposal 3: SS/PBCH block for PRACH validation refers to the applicable SSB in the active BWP.

TDRA for PUSCH

According to the current spec, the applicable PUSCH time domain resource allocation for common search space and DCI format 0_0 in UE specific search space is determined as following table in TS38.214.

Table 6.1.2.1.1-1: Applicable PUSCH time domain resource allocation for common search space and DCI format 0_0 in UE specific search space

| RNTI | PDCCH search space | pusch-ConfigCommon includes pusch-TimeDomainAllocationList | pusch-Config includes pusch-TimeDomainAllocationList | PUSCH time domain resource allocation to apply |
| --- | --- | --- | --- | --- |
| PUSCH scheduled by MAC RAR as described in clause 8.2 of [6, TS 38.213] or MAC fallbackRAR as described in clause 8.2A of [6, 38.213] or for MsgA PUSCH transmission |  | No | - | Default A |
|  |  | Yes |  | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI | Any common search space associated with CORESET 0 | No | - | Default A |
|  |  | Yes |  | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
| C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI | Any common search space not associated with CORESET 0, DCI format 0_0 in UE specific search space | No | No | Default A |
|  |  | Yes | No | pusch-TimeDomainAllocationList provided in pusch-ConfigCommon |
|  |  | No/Yes | Yes | pusch-TimeDomainAllocationList provided in pusch-Config |

For a RedCap UE, for a separate initial DL BWP without CORESET#0 and SSB, it is not used for paging and configured with NCD-SSB. After the UE completes initial access, the applicable PUSCH TDRA list should be based on the switch ‘Any common search space not associated with CORESET 0, DCI format 0_0 in UE specific search space’. However, in this case, it is hard for the gNB to distinguish whether the UE’s CBRA procedure is initiated in connected mode or idle mode when the UE performs the CBRA procedure in the separate initial DL BWP, since the UE in connected mode or idle mode may share the RA-CSS scrambled by TC-RNTI.

Observation 3: When RedCp UE initiate a CBRA procedure and type-1 CSS is monitored in a separate initial DL BWP without CORESET#0 and CD-SSB, it is hard for gNB to distinguish whether the UE’s CBRA procedure is in connected mode or idle mode before PUSCH scheduling, which causes misalignment of TDRA list assumption for PUSCH scheduling between gNB side and UE side.

Moreover, based on the discussion in R1- 2210546 in RAN1 110bis-e meeting, when CSS is configured in the dedicated DL BWP and not associated with CORESET#0, different RedCap UE may share the same type-1 CSS and has different TDRA list configuration. In this case, the gNB cannot differentiate the different RedCap UE, and then the gNB does not know how to use the TDRA table for each UE. However, the same situation is observed for NR UE. It is quite hard to differentiate the different UEs with one type.

Additionally, when RedCp UE initiate a CBRA procedure and type-1 CSS is monitored in a dedicated DL BWP, if the RedCap UE and non-RedCap UE has the same PRACH resources, it is hard for gNB to distinguish which TDRA table is assumed for the RedCap UE or non-RedCap UEs who share the CSS in the dedicated BWP.

Observation 4: When CBRA procedure is initiated and type-1 CSS is monitored in a dedicated DL BWP, if RedCap specific PRACH resources can be used to differentiate the RedCap UE and non-RedCap UE, then gNB can separately deal with the PUSCH scheduling by TC-RNTI scrambled DCI for RedCap UE.

Based on observation 3, the RedCap UE and non-RedCap UE can be differentiated, since non-RedCap UE would not be configured with separate initial BWP. Based on observation 4, in fact, without msg1 identification, the UE type also can be identified when msg3 is retransmitted. In this case that the RedCap UE is identified via msg1, any optimization for RedCap UE would not have an impact on non-RedCap UE.

Unlike NR UE, there still is a chance to correct this without NBC issue for RedCap before deployment. Therefore, some unified solutions are put on the table for further discussion.

| Options | Options description | Brief comments |
| --- | --- | --- |
| Option 1: gNB implementation | 1-1: In separate initial DL BWP without CORESET#0 and SSB or in dedicated DL BWP, pusch-TimeDomainAllocationList would not be provided in pusch-Config 1-2: pusch-TimeDomainAllocationList provided in pusch-Config is the same with pusch-TimeDomainAllocationList provided in pusch-ConfigCommon 1-3: at least one common SLIV in dedicated TDRA table, and the gNB only indicate the common SLIV. | Option 1-1 and option 1-2 are similar since the common TDRA table is assumed for PUSCH scheduling in separate initial DL BWP or dedicated DL BWP. Based on Option 1-1 and option 1-2, UE specific scheduling for other PUSCH scheduling in connected mode is not available Option 1-3 can provide some kind of flexibility for other PUSCH scheduling in connected mode, but full flexibility for other PUSCH cannot be achieved and scheduling for PUSCH in CBRA procedure is also restricted. |
| Option 2: spec corrections | 2-1: In separate initial DL BWP without CORESET#0 and SSB, the type-1 CSS is configured. The applicable PUSCH TDRA list scheduled by DCI in common search space not associated with CORESET 0 is determined by Default A or pusch-TimeDomainAllocationList provided in pusch-ConfigCommon 2-2: In any active DL BWP for RedCap UE, if the type-1 CSS is configured and not associated with CORESET#0, the applicable PUSCH TDRA list scheduled by DCI scrambled by TC-RNTI in common search space is determined by Default A or pusch-TimeDomainAllocationList provided in pusch-ConfigCommon | Option 2-1 can only solve the problem for separate initial DL BWP. Option 2-2 is more comprehensive and RedCap UE should be identified via msg1. |

For RedCap UE, it is still suggested to solve this problem before widely deployment, which can help avoid the PUSCH scheduling restriction in connected mode due to the type-1 CSS configuration especially when the separate initial DL/UL BWP is configured without CORESET#0.

Observation 5: If this issue is left to gNB implementation, the PUSCH scheduling would be limited in connected mode when separate initial BWP without CORESET#0 even is configured with NCD-SSB.

Proposal 4: Discuss the following case

- When RedCp UE initiate a CBRA procedure and type-1 CSS is monitored in a separate initial DL BWP or any other active BWP without CORESET#0 and CD-SSB, it is hard for gNB to distinguish whether the UE’s CBRA procedure is in connected mode or idle mode before PUSCH scheduling, how the gNB or UE assume the TDRA list, e.g., based on pusch-Config or pusch-ConfigCommon or default table, for PUSCH scheduling.

Conclusion

In this contribution, we discuss the remaining issues of Redcap. We have the following observations and proposals.

Occasion validation

Observation 1: In the flexible symbol, gNB doesn’t receive RedCap UE’s PUCCH and transmit SSB for non-RedCap UE at the same time.

Observation 2: gNB configuration could avoid collision between SSB and PRACH in TDD.

Proposal 1: For RedCap UE’s PUCCH on flexible symbol, the flexible symbol is not the symbol of available SSB in this active BWP.

Proposal 2: SS/PBCH block symbol for RedCap PUSCH validation refers to the applicable SSB within this active BWP.

Proposal 3: SS/PBCH block for PRACH validation refers to the applicable SSB in the active BWP.

TDRA for PUSCH

Observation 3: When RedCp UE initiate a CBRA procedure and type-1 CSS is monitored in a separate initial DL BWP without CORESET#0 and CD-SSB, it is hard for gNB to distinguish whether the UE’s CBRA procedure is in connected mode or idle mode before PUSCH scheduling, which causes misalignment of TDRA list assumption for PUSCH scheduling between gNB side and UE side.

Observation 4: When CBRA procedure is initiated and type-1 CSS is monitored in a dedicated DL BWP, if RedCap specific PRACH resources can be used to differentiate the RedCap UE and non-RedCap UE, then gNB can separately deal with the PUSCH scheduling by TC-RNTI scrambled DCI for RedCap UE.

Observation 5: If this issue is left to gNB implementation, the PUSCH scheduling would be limited in connected mode when separate initial BWP without CORESET#0 even is configured with NCD-SSB.

Proposal 4: Discuss the following case

- When RedCp UE initiate a CBRA procedure and type-1 CSS is monitored in a separate initial DL BWP or any other active BWP without CORESET#0 and CD-SSB, it is hard for gNB to distinguish whether the UE’s CBRA procedure is in connected mode or idle mode before PUSCH scheduling, how the gNB or UE assume the TDRA list, e.g., based on pusch-Config or pusch-ConfigCommon or default table, for PUSCH scheduling.

Reference

- R1-2301881, RAN1 agreements for Rel-17 NR RedCap, RAN1 112, Ericsson