3GPP TSG-WG1 Meeting #112-bis-e			R1-2303394

e-Meeting, April 17th-26th, 2023

Agenda item:	7.2

Title:	RedCap support of SDT

Source:	Nokia, Nokia Shanghai Bell

Document for:	Discussion and Decision

1. Introduction

At the RAN1#112 meeting, several issues regarding the support of SDT with REDCAP were deferred until RAN2 made progress on the same topic.

In this contribution, we analyse RAN2 progress on these issues and conclude that is no further work is required from RAN1.

2. Discussion

2.1   Analysis of RAN2 agreements

The previous RAN2 meeting started with a discussion of the following 4 options.

RedCap & SDT

Option 1: CG/RA-SDT can only be performed if the initial DL BWP includes the CD-SSB

Option 2: CG/RA-SDT can also be performed if the initial DL BWP does not include the CD-SSB but a NCD-SSB (to be signalled to the UE). A corresponding UE capability is introduced

Option 3: CG/RA-SDT can be performed even if the initial DL BWP does not include any SSB. It’s up to UE implementation whether to perform a new RSRP measurement on CB-SSB before CG transmission. A corresponding UE capability could be introduced

Option 4: If the network configures a REDCAP-specific initial DL BWP that does not include the CD-SSB, the UE monitors PDCCH on initialDownlinkBWP during the CG/RA-SDT procedure.

As noted in RAN1, during the meeting, RAN2 had at least eliminated Options 3 and 4.

Agreements:

- Option 4 is no longer considered

- Option 3 is no longer considered for this issue

Subsequent RAN2 discussions, led to the following agreements and CRs, R2-2302117 (38.331) & R2-2302118 (38.306).

Agreements:

- RAN2 confirms it will focus only on option 2.

- 1-week post meeting email discussion to check the technical details of R2-2301954 and R2-2301956. The intention is to allow companies to check internally and attempt to have a RAN2 agreed CR at the end of the email disc. (Of course, this does not preclude companies to raise concerns in the RAN plenary)

In summary, our understanding of the RAN2 agreements and CRs, translates to the following observations.

Observation 1:    	A RedCap UE does not expect to perform subsequent CG/RA-SDT transmission in a RedCap-specific separate initial DL BWP that does not include any SSB.

Observation 2:    	A RedCap UE supporting the ncd-SSB-forRedCapInitialBWP-SDT-r17 capability, can perform subsequent CG/RA-SDT transmission in a RedCap-specific separate initial DL BWP without CD-SSB but with NCD-SSB

Whilst these observations are in line with the RAN2 agreements and RAN1’s previous attempts to draft proposals for subsequent CG/RA-SDT transmission scenarios, the RAN2 agreement seems to go further and extend to initial (non-subsequent) RA-SDT transmissions.     Given RAN1’s previous conclusion (below) for initial RA-SDT transmissions, the RAN2 latest agreements/CRs can be interpreted as being unnecessarily restrictive.

Conclusion:   	No issue is identified for RedCap UEs supporting RA-SDT to support initial (non-subsequent) RA-SDT transmission in a RedCap-specific separate initial BWP without CD-SSB.

This leads us to the following proposal.

Proposal 1:	RAN1 discuss if the restriction to not support initial (non-subsequent) RA-SDT transmission in a RedCap-specific separate initial BWP without SSB is correct and acceptable.

If RAN1 deems this restriction correct but unacceptable, one possible step forward would be an LS to RAN2.

3. Conclusion

Given the latest RAN2 agreements on the operation of SDT with REDCAP, the following observations and proposal can be made:

Observation 1:    	A RedCap UE does not expect to perform subsequent CG/RA-SDT transmission in a RedCap-specific separate initial DL BWP that does not include any SSB.

Observation 2:   	A RedCap UE supporting the ncd-SSB-forRedCapInitialBWP-SDT-r17 capability, can perform subsequent CG/RA-SDT transmission in a RedCap-specific separate initial DL BWP without CD-SSB but with NCD-SSB

In our view, these observations have no RAN1 specification impacts.

Proposal 1:	RAN1 discuss if the restriction to not support initial (non-subsequent) RA-SDT transmission in a RedCap-specific separate initial BWP without SSB is correct and acceptable.

If RAN1 deems this restriction correct but unacceptable, one possible step forward would be an LS to RAN2.

4. References

[1]       Draft Report of 3GPP TSG RAN WG2 meeting #121,  Athens, Greece,  27 February - 3 March, 2023