3GPP TSG RAN WG1 #112bis-e								R1-2303454

e-Meeting, April 17th – April 26th, 2023

Agenda Item:	9.12.2

Source:	InterDigital, Inc.

Title:	Discussion on power domain enhancements

Document for:	Discussion and Decision

# Introduction

In RAN1#112 meeting [1], the following were agreed to facilitate higher power transmissions:

Agreement Further discussions in RAN1 concerning means to facilitate higher power transmissions in CA and DC, if applicable, can target increasing gNB awareness of UE’s Tx power, e.g., PHR reporting enhancement such as current power class, power class change, or application of P-MPR by UE (subject to RAN4’s input). - FFS: details.

In this contribution we discuss signaling to enable higher power transmission in CA and DC and we present link level simulation results for both Tone reservation and FDSS techniques.

# Discussion

## Higher power transmissions

In NR carrier aggregation, the UE performs power control per each serving cell and ensures that the total transmitted power is below the limit set for CA power class i.e., Pcmax_CA. If the total power exceeds the limit, the UE scales the allocated power according to defined priorities order. In Rel-15/16, Pcmax_CA has the same value regardless of the number of combinations of UL carriers, UE capability or the activated UL carriers. In Rel-17, the UE can indicate to the network a higher power limit capability for CA for a band combination semi-statically and the Pcmax_CA range limits can be increased accordingly. To increase the awareness at gNB of power availability at the UE and to facilitate higher power transmission in CA and DC, enhanced signalling between UE and gNB was agreed to be considered for study in RAN1#111 meeting. One potential enhancement to consider in Rel-18 is to dynamically adapt the Pcmax based on the power availability at the UE. To enable a dynamic way to change Pcmax, the UE can send an indication to the gNB regarding the power class changes and the gNB can adapt/optimize its scheduling according to the signalled CA power class.

Proposal 1: Support UE indicating the power class change to the gNB.

To assist the gNB in scheduling according to the power class and based on the power availability at the UE, to achieve a dynamic power class adaptation, enhanced power headroom reports can be used. The UE can determine that power class has changed due to some triggering events and indicates the power class change to the gNB. For example, an enhanced PHR with more information can be transmitted by the UE. The enhanced PHR can indicate to the gNB a suggested power class depending on the triggering events.

Proposal 2: Support UE indication of power class change in power headroom report.

One event that could trigger power class change is the SAR requirements changes for the used UL duty cycle. The UL duty cycle has a correlation with the SAR compliance and thus in some cases the UE will be falling back to a different power class in case the duty cycle changed (increased). Another event that can trigger UE power class change is the change of the RSRP measurements. For example, a UE that has strong SSB-RSRP may not need the higher power class whereas a UE at cell edge may need a higher power class. The UE can be using a default power class and change to higher power class. Another event could be a change of the set of active UL carriers. For example, when the UE receives MAC CE activating/deactivating CCs, the CA UE power class can be changed.

Proposal 3: Study events that can trigger UE to report power class change.

## Tone reservation and FDSS link level evaluation

Tone reservation:

Tone reservation is a technique that can reduce PAPR of a signal carrying data transmission. It consists of using additional frequency resources to generate tones that reduce the peaks of data signal. In RAN1#110b-e meeting it was agreed to study candidate solutions to reduce MPR PAR and tone reservation technique was among the candidate solutions [2]. In RAN1#111 meeting, LLS evaluation assumption for MPR-PAR reduction solutions was agreed and are captured in [1]. In this section we present our LLS results comparing the tone reservation technique with Rel-17 baseline. To implement tone reservation, we used an iterative algorithm to generate the peak reduced signal with target peak threshold.

We compared the performance of tone reservation technique using spectrum extension against Rel-17 baseline with no extension. The LLS results for 700 MHz, 4GHz and 28 GHz are shown in Table 1, Table 2 and Table 3 respectively. We show the SNR required to achieve 10% BLER target as well as the 1% CCDF PAPR for different TBS, MCS and number of PRBs values agreed in RAN1#111 meeting. The comparison takes into account the additional PRBs used for tone reservation by using those PRBs for data transmission in case of no spectrum extension to reduce the coding rate. As we can see in the tables, the PAPR is significantly decreased with tone reservation technique. Depending on the number of PRBs, we can achieve up to 3dB reduction. This increase comes at the expense of higher BLER. We can observe that SINR loss can be up to 2dB to achieve the same 10% BLER target. Because the PAPR reduction can allow the UE to transmit with higher transmit power, this SINR loss can be compensated by using higher transmission power. The coverage gains by using tone reservation depends on how much the transmission power is increased. It is up to RAN4 to determine the possible transmission power increase for each PAPR reduction value.

Observation 1: The BLER increase due to using some of the PUSCH resources for spectrum extension can be overcome by increasing the transmission power and potentially results in higher coverage when using the tone reservation technique.

Table 1: 10 % BLER and 1% CCDF PAPR of PUSCH for 700 MHz Rural

|  | No spectrum extension |  |  |  | Tone reservation with spectrum extension |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBS value | PRBs | MCS | 10% BLER SINR | 1% CCDF PAPR | PRBs before extension | PRBs after extension | MCS | 10% BLER SINR | 1% CCDF PAPR | SINR Loss | PAPR reduction |
| 2408 | 16 | 7 | -0.5 | 6.9 | 14 | 16 | 8 | -1.74 | 4.62 | 0.88 | 2.3 |
| 5376 | 32 | 8 | -0.11 | 6.99 | 28 | 32 | 9 | -1.12 | 4.61 | 0.95 | 2.3 |
| 272 | 8 | 0 | -6.22 | 6.8 | 6 | 8 | 1 | -8.02 | 3.79 | 1.28 | 3.12 |
| 1032 | 8 | 6 | -1.13 | 6.82 | 6 | 8 | 8 | -1.51 | 3.79 | 1.94 | 3.12 |
| 2152 | 40 | 2 | -5.35 | 6.91 | 30 | 40 | 3 | -6.44 | 4.23 | 1.26 | 2.69 |
| 4992 | 40 | 6 | -1.83 | 6.92 | 30 | 40 | 8 | -2.12 | 4.22 | 1.74 | 2.69 |
| 1736 | 32 | 2 | -5.24 | 6.89 | 20 | 32 | 4 | -5.54 | 4.08 | 2.12 | 2.83 |
| 432 | 8 | 2 | -4.67 | 6.7 | 6 | 8 | 3 | -6.2 | 3.79 | 1.29 | 3.13 |
| 808 | 24 | 0 | -6.73 | 6.87 | 18 | 24 | 1 | -8.46 | 4.18 | 0.99 | 2.73 |
| 272 (BPSK) | 8 | 0 | -8.29 | 5.5 | 6 | 8 | 1 | -6.75 | 3.45 | 1.54 | 2.05 |
| 808 (BPSK) | 24 | 0 | -8.9 | 5.51 | 18 | 24 | 1 | -7.2 | 3.72 | 1.7 | 1.79 |

Table 2: 10 % BLER and 1% CCDF PAPR of PUSCH for 4GHz Urban

|  | No spectrum extension |  |  |  | Tone reservation with spectrum extension |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBS value | PRBs | MCS | 10% BLER SINR | 1% CCDF PAPR | PRBs before extension | PRBs after extension | MCS | 10% BLER SINR | 1% CCDF PAPR | SINR Loss | PAPR reduction |
| 2408 | 16 | 7 | -0.21 | 6.87 | 14 | 16 | 8 | 0.22 | 4.62 | 0.93 | 2.3 |
| 5376 | 32 | 8 | 0.35 | 6.91 | 28 | 32 | 9 | 0.58 | 4.61 | 0.9 | 2.31 |
| 272 | 8 | 0 | -5.63 | 6.89 | 6 | 8 | 1 | -4.78 | 3.8 | 1.22 | 3.11 |
| 1032 | 8 | 6 | -0.72 | 6.81 | 6 | 8 | 8 | 1.02 | 3.79 | 1.95 | 3.12 |
| 2152 | 40 | 2 | -4.67 | 6.93 | 30 | 40 | 3 | -4.36 | 4.21 | 1.14 | 2.71 |
| 4992 | 40 | 6 | -1.3 | 6.9 | 30 | 40 | 8 | -0.37 | 4.21 | 1.63 | 2.71 |
| 1736 | 32 | 2 | -4.55 | 6.9 | 20 | 32 | 4 | -3.54 | 4.08 | 1.9 | 2.83 |
| 432 | 8 | 2 | -4.05 | 6.97 | 6 | 8 | 3 | -3.22 | 3.8 | 1.3 | 3.11 |
| 808 | 24 | 0 | -5.99 | 6.86 | 18 | 24 | 1 | -6.03 | 4.18 | 0.94 | 2.74 |
| 272 (BPSK) | 8 | 0 | -5.39 | 5.5 | 6 | 8 | 1 | -3.88 | 3.46 | 1.51 | 2.04 |
| 808 (BPSK) | 24 | 0 | -6.65 | 5.51 | 18 | 24 | 1 | -5.1 | 3.72 | 1.55 | 1.79 |

Table 3: 10 % BLER and 1% CCDF PAPR of PUSCH for 28 GHz Urban

|  | No spectrum extension |  |  |  | Tone reservation with spectrum extension |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBS value | PRBs | MCS | 10% BLER SINR | 1% CCDF PAPR | PRBs before extension | PRBs after extension | MCS | 10% BLER SINR | 1% CCDF PAPR | SINR Loss | PAPR reduction |
| 2408 | 16 | 7 | 4.51 | 6.88 | 14 | 16 | 8 | 5.17 | 4.62 | 0.97 | 2.3 |
| 5376 | 32 | 8 | 4.99 | 6.91 | 28 | 32 | 9 | 5.66 | 4.64 | 1.11 | 2.28 |
| 272 | 8 | 0 | -1.97 | 6.79 | 6 | 8 | 1 | -1.12 | 3.79 | 1.42 | 3.13 |
| 1032 | 8 | 6 | 3.96 | 6.85 | 6 | 8 | 8 | 5.79 | 3.79 | 2.22 | 3.12 |
| 2152 | 40 | 2 | -1.04 | 6.95 | 30 | 40 | 3 | -0.11 | 4.57 | 1.53 | 2.35 |
| 4992 | 40 | 6 | 2.97 | 6.98 | 30 | 40 | 8 | 4.53 | 4.56 | 2.05 | 2.36 |
| 1736 | 32 | 2 | -0.81 | 6.85 | 20 | 32 | 4 | 0.99 | 4.09 | 2.45 | 2.83 |
| 432 | 8 | 2 | -0.1 | 6.87 | 6 | 8 | 3 | 0.78 | 3.79 | 1.48 | 3.12 |
| 808 | 24 | 0 | -2.51 | 6.83 | 18 | 24 | 1 | -2.06 | 4.19 | 1.12 | 2.73 |
| 272 (BPSK) | 8 | 0 | -1.79 | 5.5 | 6 | 8 | 1 | 0.04 | 3.44 | 1.83 | 2.06 |
| 808 (BPSK) | 24 | 0 | -2.85 | 5.51 | 18 | 24 | 1 | -1.02 | 3.71 | 1.83 | 1.8 |

FDSS with spectrum extension:

FDSS with spectrum extension uses the excess bandwidth as part of the total allocated bandwidth and uses it for the pulse shaping of the transform pre-coded data. First, an M-point DFT is applied to the data, and then the output of the M-point DFT is cyclically extended to K additional tones on each side as illustrated in Figure 1. Lastly, this cyclically extended output is multiplied with the frequency-domain coefficients corresponding to a pulse shaping filter (3-taps [-0.28 1 -0.28]). The output of this operation is then mapped to M+2*K contiguous tones located in the middle of the transmission bandwidth before conversion to the time domain (N-point IFFT).

Figure 1: FDSS with spectrum extension

We compared the performance of FDSS using spectrum extension against Rel-17 baseline with no extension. The LLS results for 700 MHz, 4GHz and 28 GHz are shown in Table 4, Table 5 and Table 6 respectively. Both SINR for 10% BLER and 1% CCDF PAPR are shown in the tables for the agreed TBS and MCS values.

Observation 2: Similar to tone reservation, FDSS-SE can potentially result in increased uplink coverage if higher transmission power is supported.

Table 4: 10 % BLER and 1% CCDF PAPR of PUSCH for 700 MHz Rural

|  | No spectrum extension |  |  |  | FDSS with spectrum extension |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBS value | PRBs | MCS | 10% BLER SINR | 1% CCDF PAPR | PRBs before extension | PRBs after extension | MCS | 10% BLER SINR | 1% CCDF PAPR | SINR Loss | PAPR reduction |
| 2408 | 16 | 7 | -0.5 | 6.9 | 14 | 16 | 8 | -0.75 | 3.98 | 1.87 | 2.94 |
| 5376 | 32 | 8 | -0.11 | 6.99 | 28 | 32 | 9 | -0.02 | 5.39 | 2.05 | 1.52 |
| 272 | 8 | 0 | -6.22 | 6.8 | 6 | 8 | 1 | -8.7 | 3.57 | 0.6 | 3.34 |
| 1032 | 8 | 6 | -1.13 | 6.82 | 6 | 8 | 8 | -1.66 | 3.57 | 1.79 | 3.34 |
| 2152 | 40 | 2 | -5.35 | 6.91 | 30 | 40 | 3 | -6.89 | 5.65 | 0.81 | 1.27 |
| 4992 | 40 | 6 | -1.83 | 6.92 | 30 | 40 | 8 | -2.25 | 5.64 | 1.61 | 1.27 |
| 1736 | 32 | 2 | -5.24 | 6.89 | 20 | 32 | 4 | -6.79 | 6.19 | 0.87 | 0.72 |
| 432 | 8 | 2 | -4.67 | 6.7 | 6 | 8 | 3 | -6.74 | 3.57 | 0.75 | 3.35 |
| 808 | 24 | 0 | -6.73 | 6.87 | 18 | 24 | 1 | -9.17 | 5.9 | 0.28 | 1.01 |
| 272 (BPSK) | 8 | 0 | -8.29 | 5.5 | 6 | 8 | 1 | -7.53 | 2.41 | 0.76 | 3.09 |
| 808 (BPSK) | 24 | 0 | -8.9 | 5.51 | 18 | 24 | 1 | -8.05 | 4.26 | 0.85 | 1.25 |

Table 5: 10 % BLER and 1% CCDF PAPR of PUSCH for 4GHz Urban

|  | No spectrum extension |  |  |  | FDSS with spectrum extension |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBS value | PRBs | MCS | 10% BLER SINR | 1% CCDF PAPR | PRBs before extension | PRBs after extension | MCS | 10% BLER SINR | 1% CCDF PAPR | SINR Loss | PAPR reduction |
| 2408 | 16 | 7 | -0.5 | 6.9 | 14 | 16 | 8 | 1.21 | 3.99 | 1.92 | 2.93 |
| 5376 | 32 | 8 | -0.11 | 6.99 | 28 | 32 | 9 | 1.82 | 5.39 | 2.14 | 1.53 |
| 272 | 8 | 0 | -6.22 | 6.8 | 6 | 8 | 1 | -5.32 | 3.57 | 0.68 | 3.34 |
| 1032 | 8 | 6 | -1.13 | 6.82 | 6 | 8 | 8 | 0.98 | 3.58 | 1.91 | 3.33 |
| 2152 | 40 | 2 | -5.35 | 6.91 | 30 | 40 | 3 | -4.81 | 5.65 | 0.69 | 1.27 |
| 4992 | 40 | 6 | -1.83 | 6.92 | 30 | 40 | 8 | -0.39 | 5.64 | 1.61 | 1.28 |
| 1736 | 32 | 2 | -5.24 | 6.89 | 20 | 32 | 4 | -4.73 | 6.19 | 0.71 | 0.72 |
| 432 | 8 | 2 | -4.67 | 6.7 | 6 | 8 | 3 | -3.67 | 3.57 | 0.85 | 3.34 |
| 808 | 24 | 0 | -6.73 | 6.87 | 18 | 24 | 1 | -6.66 | 5.89 | 0.31 | 1.03 |
| 272 (BPSK) | 8 | 0 | -8.29 | 5.5 | 6 | 8 | 1 | -4.55 | 2.41 | 0.84 | 3.09 |
| 808 (BPSK) | 24 | 0 | -8.9 | 5.51 | 18 | 24 | 1 | -5.75 | 4.26 | 0.9 | 1.25 |

Table 6: 10 % BLER and 1% CCDF PAPR of PUSCH for 28 GHz Urban

|  | No spectrum extension |  |  |  | FDSS with spectrum extension |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBS value | PRBs | MCS | 10% BLER SINR | 1% CCDF PAPR | PRBs before extension | PRBs after extension | MCS | 10% BLER SINR | 1% CCDF PAPR | SINR Loss | PAPR reduction |
| 2408 | 16 | 7 | -0.5 | 6.9 | 14 | 16 | 8 | 6.1 | 3.98 | 1.9 | 2.94 |
| 5376 | 32 | 8 | -0.11 | 6.99 | 28 | 32 | 9 | 6.61 | 5.4 | 2.06 | 1.52 |
| 272 | 8 | 0 | -6.22 | 6.8 | 6 | 8 | 1 | -1.74 | 3.57 | 0.8 | 3.35 |
| 1032 | 8 | 6 | -1.13 | 6.82 | 6 | 8 | 8 | 5.58 | 3.57 | 2.01 | 3.34 |
| 2152 | 40 | 2 | -5.35 | 6.91 | 30 | 40 | 3 | -0.58 | 5.64 | 1.06 | 1.28 |
| 4992 | 40 | 6 | -1.83 | 6.92 | 30 | 40 | 8 | 4.35 | 5.64 | 1.87 | 1.28 |
| 1736 | 32 | 2 | -5.24 | 6.89 | 20 | 32 | 4 | -0.26 | 6.18 | 1.2 | 0.74 |
| 432 | 8 | 2 | -4.67 | 6.7 | 6 | 8 | 3 | 0.26 | 3.57 | 0.96 | 3.34 |
| 808 | 24 | 0 | -6.73 | 6.87 | 18 | 24 | 1 | -2.71 | 5.89 | 0.47 | 1.03 |
| 272 (BPSK) | 8 | 0 | -8.29 | 5.5 | 6 | 8 | 1 | -0.7 | 2.41 | 1.09 | 3.09 |
| 808 (BPSK) | 24 | 0 | -8.9 | 5.51 | 18 | 24 | 1 | -1.74 | 4.26 | 1.11 | 1.25 |

FDSS vs Tone reservation:

We can see from the results summarized in the above tables that both tone reservation and FDSS techniques have the potential to reduce PAPR and thus increase the coverage. When the number of allocated PRBs is more than 16, it is more beneficial to use tone reservation as it provides higher PAPR reduction. On the other hand, with 16 or less PRBs, FDSS provides better PAPR reduction performance.

As for BLER, we can see that tone reservation provides better BLER performance when the baseline MCS is higher than 6, and FDSS performance is better at lower coding rates, i.e., ≤ MCS 6.

Based on the above observations, our view is that both techniques can be supported depending on the BW/MCS of the scheduled uplink grant and the gNB can enable FDSS or tone reservation.

Proposal 4: Support FDSS and tone reservation with spectrum extension.

# Conclusion

In this contribution, we have discussed and proposed the following:

Proposal 1: Support UE indicating the power class change to the gNB.

Proposal 2: Support UE indication of power class change in power headroom report.

Proposal 3: Study events that can trigger UE to report power class change.

Observation 1: The BLER increase due to using some of the PUSCH resources for spectrum extension can be overcome by increasing the transmission power and potentially result in higher coverage.

Observation 2: Similar to tone reservation, FDSS-SE can potentially result in increased uplink coverage if higher transmission power is supported.

Proposal 4: Support FDSS and tone reservation with spectrum extension.

# References

Draft Report of 3GPP TSG RAN WG1 #111 v0.2.0

R1-2210801, “Final Report of 3GPP TSG RAN WG1 #110bis-e v1.0.0.”

# Appendix: simulation parameters

| Channel Bandwidth | 100MHz for Urban 20MHz for Rural, |
| --- | --- |
| SCS | 30 kHz (4GHz), 120 kHz (28GHz) 15 kHz (700 MHz), |
| Channel model | TDL-C 300ns for FR1 Urban (4GHz), TDL-A 30ns for FR2 Urban (28GHz), TDL-D 30ns for Rural |
| UE speed | 3km/h |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 4 for FR1 Urban, 2 for FR2, 4 for FR1 Rural, |
| PUSCH length | 14 symbols (12 for data and 2 for DMRS) |
| Frequency hopping | disabled |