3GPP TSG-RAN WG1 Meeting #112-bis-e	R1-2304256

# Online, April 17th - April 26th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0477 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on reference SCS for availability indication in eIAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Qualcomm), Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2023-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Agreement from RAN1#106 is missing and clarification of the reference SCS used for availability indication in time- and frequency-domain is needed. Agreement A Reference SCS is configured for frequency domain H/S/NA configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the specification to include the RAN1 agreement from the RAN1#106-e meeting. Clarification on the the reference SCS from gNB-DU Cell Resource Configuration is used for the slot of a cell in availabilityCombinations-r16 an availabilityCombinationsRBGroups-r17, while the reference SCS from RB set Configuration is used for the RB sets of a cell in availabilityCombinationsRBGroups-r17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 14 Integrated access-backhaul operation

<Unchanged parts are omitted>

For each cell of an IAB-DU in a set of cells of the IAB-DU, the IAB-DU can be provided:

-	an identity of the IAB-DU cell by iab-DU-CellIdentity

-	a location of an availability indicator (AI) index field in DCI format 2_5 by positionInDCI-AI or by positionInDCI-AI-r17

-	a set of availability combinations by availabilityCombinations-r16 or by availabilityCombinationsRBGroups-r17, where each availability combination in the set of availability combinations includes

-	resourceAvailability-r16 indicating availability of soft symbols in one or more slots for the IAB-DU cell, or one resourceAvailability-r17 indicating availability of soft resources in all RB sets in one or more slots for the IAB-DU cell, or one or multiple RB set groups by rb-SetGroups-r17 with each RB set groups by RB-SetGroup-r17 indicating resourceAvailability-r17 for soft resources in one or more slots for the associated rb-Sets-r17, and

-	a mapping for the soft symbol, and/or for soft resources, availability combinations provided by resourceAvailability-r16 or resourceAvailability-r17 to a corresponding AI index field value in DCI format 2_5 provided by availabilityCombinationId-r16 or availabilityCombinationId-r17, respectively

With reference to a slot of an IAB-DU cell, if the IAB-DU is not provided an indication of hard, soft or unavailable type per RB set by Frequency-Domain HSNA Configuration List [16, TS 38.473], the indication of availability for the slot is based solely on availabilityCombinations-r16.

The IAB-DU can assume a same SCS configuration for availabilityCombinations-r16 or availabilityCombinationsRBGroups-r17 for slots of a cell as an SCS configuration provided by gNB-DU Cell Resource Configuration for the cell.

The IAB-DU can assume a same SCS configuration for availabilityCombinationsRB-Groups-r17 for RB sets of a cell as a SCS configuration provided by RB Set Configuration for the cell.

<Unchanged parts are omitted>