3GPP TSG RAN WG1 #113			       R1-2304431

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:		9.1.4.2

Source:				Moderator (InterDigital, Inc.)

Title:						FL Summary on SRI/TPMI Enhancements; Third Round

Document for:		Discussion and Decision

# Background

In RAN plenary #94, the WID for Rel-18 MIMO enhancements was finalized [1]. According to the WID, some enhancements for SRI/TPMI are necessary to enable 8 TX UE transmission.

Objective 5: Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

To accomplish the objective, the scope of this agenda item centers on codebook design for 8TX, enhancements for dual CW operation, enhancements on SRS configuration, impacts resulted from coherency characteristics of such UEs as well as UE operation with full power.

Based on the progress and agreements made in the last meeting [2-3], the following topics are the focal point of the discussion in this meeting.

High Priority Topics

- Fully-coherent CB-based precoding

  - TPMI indication

- Partially-coherent CB-based precoding

  - Ng=2

    - Precoder selection and TPMI indication

  - Ng=4

    - Decision for supported cases of layer splitting for rank 3, 4, 5, 6 and 7

    - Precoder selection and TPMI indication

- Non-coherent CB-based precoding

  - Down-selection between support of the full set of precoders or a subset of it

- Non-CB-based precoding

  - Details for legacy-based SRI indication for when with NSRS>4, and lmax=8

Other Topics

- Codebook subset discussion

  - Down-selection from the identified alternatives

- Remaining details for support of dual CW transmission

  - Confirming the range of N values for CBG-based transmission

  - Identifying remaining issues, if any

- Full power

  - Discuss specification impact for Modes 1 and 2

# Codebook Design for Coherent 8TX UE

Void

# Codebook Design for Partially/Non-Coherent UE

Working Assumption (#112b-e) For partially coherent uplink precoding by an 8TX UE, Ng=2, - At least the following combinations of layer splitting are supported - FFS: For rank>4, all the layers for each CW is mapped to only one antenna group Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (1,2), (2,1). 4; (4,0), (0,4). 4; (2,2). 5; (2,3), (3,2). 6; (3,3). 7; (3,4), (4,3). Agreement (110b-e) For codebook design of an 8TX partial-coherent UE, configured with an 8-port SRS resource - For when Ng=2, down-select of the following convention for assumption of port coherency scheme is used - Alt 1: two coherent groups of {0,2,4,6} and {1,3,5,7} - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7} - Alt 3: two coherent groups of {0,1,2,3} and {4,5,6,7} - For when Ng=4, down-select of the following convention for assumption of port coherency scheme is used - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7} - Alt 2: four coherent groups of {0,1}, {2,3}, {4,5}, and {6,7} - Alt 3: four coherent groups of {0, 2}, {4, 6}, {1, 3} and {5,7}

Based on the discussion and contributions from the last meeting, the following proposals are considered for the discussion in this meeting.

Proposal 3.1: For partially coherent uplink precoding by an 8TX UE, Ng=2, following cases are supported

For codebook design of an 8TX partial-coherent UE, configured with an 8-port SRS resource

- For when Ng=2, following convention for assumption of port coherency scheme is used

  - Alt 1: two coherent groups of {0,2,4,6} and {1,3,5,7}

  - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7}

  - Alt 3: two coherent groups of {0,1,2,3} and {4,5,6,7}

- For when Ng=4, following convention for assumption of port coherency scheme is used

  - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7}

  - Alt 2: four coherent groups of {0,1}, {2,3}, {4,5}, and {6,7}

  - Alt 3: four coherent groups of {0, 2}, {4, 6}, {1, 3} and {5,7}

Supporting: vivo, NTT, Samsung, NEC, MediaTek, Spreadtrum, OPPO, Lenovo, Xiaomi, LG, Nokia, CMCC, Ericsson, InterDigital, Apple,

Concern: ZTE, CATT, Huawei

Proposal 3.2: For partially coherent uplink precoding by an 8TX UE, Ng=2, confirm the Working Assumptions with updates:

| Rank | All layers in one Antenna Group | Layers split across 2 Antenna Groups |
| --- | --- | --- |
| 2 | (2,0), (0,2) |  |
| 2 |  | (1,1) |
| 3 | (3,0), (0,3) |  |
| 3 |  | (1,2), (2,1) |
| 4 | (4,0), (0,4) |  |
| 4 |  | (2,2) |
| 5 |  | (2,3), (3,2) |
| 6 |  | (3,3) |
| 7 |  | (3,4), (4,3) |

Confirm WA with removing the indicated cases: ZTE, Qualcomm, MediaTek, OPPO, CATT, Lenovo, Xiaomi, CMCC, Ericsson(keep (2,1)), InterDigital

Confirm WA without removing the indicated cases: vivo, NTT, Samsung, NEC, Spreadtrum, Google, LG, Nokia, HW, Apple (we are fine with reverting with WA and include all the combinations due to the same DCI overhead)

Proposal 3.3: For partially coherent uplink precoding by an 8TX UE, Ng=2, when rank>4, down-select from,

- Option 1: Layers for each CW can be shuffled and mapped to either of antenna groups

  - FFS CW-layer to antenna group mapping, e.g., fixed, layer scrambling, etc.

- Option 2: Layers of each CW is mapped to only a single antenna group

Option 1: Spreadtrum, Xiaomi, Google, Lenovo, Huawei, ZTE, NTT, Samsung, MediaTek, OPPO, CATT, Apple

Option 2: vivo, Nokia, Ericsson, InterDigital

Proposal 3.4: For partially coherent uplink precoding by an 8TX UE, Ng=4,

- The following rank and layer splitting cases are supported,

| Rank | All layers in one Antenna Group | Layers split across 4 Antenna Groups (All possible permutations) |
| --- | --- | --- |
| 3 |  | Transmission by 2 of the 4 antenna groups: (2,1,0,0), (2,0,1,0), (2,0,0,1), (0,2,1,0), (0,2,0,1), (0,0,2,1), (1,2,0,0), (1,0,2,0), (1,0,0,2), (0,1,2,0), (0,1,0,2), (0,0,1,2) Transmission by 3 of the 4 antenna groups: (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1) |
| 4 |  | Transmission by 3 of the 4 antenna groups: (2,1,1,0), (0,2,1,1), (1,0,2,1), (1,1,0,2) (1,2,1,0), (1,1,2,0), (0,1,2,1), (0,1,1,2), (1,0,1,2), (2,0,1,1), (2,1,0,1), (1,2,0,1) |
| 5 |  | Transmission by 3 of the antenna groups: (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1), (2,1,2,0), (1,2,2,0), (2,1,0,2), (1,2,0,2), (2,0,1,2), (1,0,2,2), (0,2,1,2), (0,1,2,2) Transmission by 4 of the 4 antenna groups: (1,1,2,1), (1,1,1,2), (2,1,1,1), (1,2,1,1) |
| 6 |  | Transmission by 3 of the 4 antenna groups: (2,2,2,0), (2,2,0,2), (2,0,2,2), (0,2,2,2) Transmission by 4 of the 4 antenna groups: (2,1,2,1), (1,2,1,2), (1,2,2,1), (2,1,1,2), (2,2,1,1), (1,1,2,2) |
| 7 |  | Transmission by 4 of the 4 antenna groups: (2,1,2,2), (2,2,2,1), (1,2,2,2), (2,2,1,2) |

Table 1 - Companies’ views

| Company | Perspective |
| --- | --- |
| FL | Proposals 3.1, 3.2, 3.3: Please check whether your position is captured correctly. No further comment is required, however please feel free to further discuss if you would like to. Proposals 3.4: Based on companies’ proposals, layer splitting for Ng=4 is updated. |
| CATT | Proposal 3.1: We prefer Alt 1 for Ng = 2 and Alt 1 for Ng = 2. In RAN1 #112bis-e meeting, it was agreed that for an 8-port SRS resource with all of 8 ports mapped to all of OFDM symbols of the SRS resource: - If kTC=2, ports {1000, 1002, 1004, 1006} are mapped on the first comb offset, and {1001, 1003, 1005, 1007} on the second comb offset - If kTC=4, ports {1000, 1004} are mapped on the first comb offset, {1001, 1005} on the second comb offset, {1002, 1006} on the third comb offset, and {1003, 1007} on the fourth comb offset. On the number of comb offsets that the SRS ports for the same coherent group are mapped into, we think it shall be as small as possible. Therefore Alt 1 for Ng = 2 and Alt 1 for Ng = 2 is preferred. Proposal 3.2: As we figured it out that precoders with all layers come from one coherent antenna group are rarely used, we do not support the all layers in one Antenna Group cases. Thus, we suggest the following modifications based on the current proposal: Proposal 3.2: For partially coherent uplink precoding by an 8TX UE, Ng=2, confirm the Working Assumptions with updates: Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2) -. 2; (1,1). 3; (3,0), (0,3) -. 3; (1,2), (2,1). 4; (4,0), (0,4) -. 4; (2,2). 5; (2,3), (3,2). 6; (3,3). 7; (3,4), (4,3). Proposal 3.3: Not Support. In RAN1 #110bis-e meeting, agreement on CW to layer mapping has been achieved for dual CW transmission. The CW to antenna groups mapping can be determined by the mapping of layers and antenna ports. Therefore, no further agreement is needed. Agreement If dual CW is supported for uplink transmission with Rank>4 by an 8TX UE, reuse DL Rel-15 codeword to layer mapping for both codebook-based and non-codebook-based transmission.. |
| NTT DOCOMO | For partially-coherent precoders, we have one question. Is the current assumption to use all the 4TX/2TX fully-coherent precoders to generate new 8TX precoders? Or is it still open for discussion whether to further reduce the candidates from current 4TX/2TX fully-coherent precoders? |
| LG Electronics | For proposal 3.3, we are more likely to option 1, however we are not sure on the current wording in option 1. As commented by CATT, we don’t need to agree further regarding CW to layer mapping. Even we go with option 1, there are FFS point which may cause more confusion. To NTT Docomo’s question, our understanding is it is still open whether we use all of fully-coherent precoders or not. The proposal only deals with layer split combination. |
| Huawei, HiSilicon | For Proposal 3.2: We don’t support it. We have evaluated the performance of full layer splitting and partial layer splitting in various layouts, where two antenna groups are arranged as two 4TX ULA arrays with different directions. The angle θ between two panel directions can be 0 (pointing the same direction), or 60 degrees. Three precoder sets are evaluated: - Full: precoders with all layer splitting - Partial 1: precoders with layer splitting in the working assumption - Partial 2: precoders with layer splitting in proposal 3.2 From the results, it can be observed, the performance is degraded when more layer splitting is removed, especially when there’s angle between panels. Hence, to accommodate various UE layouts and dynamically varied channel environments, we prefer to support all possible layer splitting schemes for each rank. In addition, we agree with companies’ view that the pruning of layer splitting would only be meaningful if it reduces overhead. According to the following table, it can be seen that the pruning of layer splitting in WA and Proposal 3.2 do not reduce overhead, where 10 bits are still required. r; Full; Partial 1; Partial 2. 1; 32; 32; 32. 2; 272; 272; 272. 3; 264; 264; 136. 4; 196; 68; 68. 5; 128; 64; 32. 6; 48; 16; 16. 7; 16; 16; 8. 8; 4; 4; 4. ≤8; 960; 736; 568. For Proposal 3.3: We agree with CATT that no further agreement is needed. For Proposal 3.4: not support. It can be seen that rank 4-6 have larger number of precoders. Hence, to reduce overhead, the pruning the layer splitting of rank 4, rank 5 and rank 6 should be prioritized. For rank 4, we prefer to remove all unbalanced layer splitting cases of transmission by 3 of the 4 antenna groups, which can reduce to 280 precoders. For rank 5, considering the probability that one antenna group out of the four has bad channel quality, it should be enough to support the first 4 layer splitting cases (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1) of transmission by 3 of the 4 antenna groups in the agreement. For transmission by 4 of the 4 antenna groups, we prefer to support only two layer splitting cases, such as the first 2 layer splitting cases (1,1,2,1), (1,1,1,2). With such pruning, the number of precoders of rank 5 can reduce to 320 precoders. For rank 6, all layer splitting cases of transmission by 3 of the 4 antenna groups need to be supported. For transmission by 4 of the 4 antenna groups, we prefer to support only four layer splitting cases, such as the first 4 layer splitting cases (2,1,2,1), (1,2,1,2), (1,2,2,1), (2,1,1,2). With such pruning, the number of precoders of rank 6 can reduce to 288 precoders. |
| QC | Proposal 3.1: We are fine with the proposal. Proposal 3.2: We support the FL proposal. Proposal 3.3: Thank FL clarify option 1 means layers are “shuffled” before map to antenna groups for precoding. If so, we don’t support option 1, because there is no motivation to shuffle the layers. Plus, how to perform the shuffling needs more work. Comparing the left and right figure, the scheme in left figure is completed. The scheme in right figure seems incomplete, because it is still open how to do the shuffling. However, the formulation of option 2 seems a little inaccurate as well. If 5 layer split into 3+2 is also supported in proposal 3.2. Then even the left figure will lead to layers of one CW map to either antenna groups. In our view, the key issue not about map layers of one CW to one or multiple antenna groups. The key issue is whether “shuffle” the layers before map to antenna groups. We suggest feature lead to reformulate the two options in that way, which might clarify some confusion and help companies to do down-selection. Proposal 3.4: we are in general supportive to the proposal. We just have a minor question/comment for clarification: why we keep both “(2,1,2,1), (1,2,1,2)” for rank 6, while only 1 split is allowed for transmission by 4 of 4 antenna group, with rank 5, and 7? In our opinion, only one of “(2,1,2,1), (1,2,1,2)” is needed, to be consistent with other ranks. But we are open to keep both, if motivation can be justified. |
| ZTE | Proposal 3.1: We share the same concerns as mentioned by CATT about Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7} In our original thought, if Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7} is adopted, port indexes grouping for two 4Tx groups can be supported by either of the followings. - Alt 1: two coherent groups of {0,2,4,6} and {1,3,5,7} - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7} That means either one is fine to us, and we slightly preferred Alt2 before. However, in last meeting, i.e., RAN1#112b-e, a similar issue was discussed in AI 9.1.3.2, considering how to group port indexes for 8-port SRS for a same comb offset. The ports sharing same comb offset are more likely to be coherent. Note that in legacy 4Tx SRS scheme, ports {0, 2} are allocated to one comb offset, ports {1, 3} are allocated to the same comb offset or different comb offsets from that for ports {0,2}. During that discussion, we mentioned this pendent discussion in AI 9.1.4.2, and preferred to group ports as Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7} according to the majority of views in AI 9.1.4.2, but no other companies liked this grouping scheme. Finally, in A.I. 9.1.3.2 an agreement was reached as shown below. Agreement For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is assigned with kTC>1 comb offsets, determine the mapping from the ports to comb offsets as follows: - If kTC=2, ports {1000, 1002, 1004, 1006} are mapped on the first comb offset, and {1001, 1003, 1005, 1007} on the second comb offset - If kTC=4, ports {1000, 1004} are mapped on the first comb offset, {1001, 1005} on the second comb offset, {1002, 1006} on the third comb offset, and {1003, 1007} on the fourth comb offset.. Under this situation, we wonder why we need to define a port grouping for 8Tx PUSCH precoder that is: - Same as port indexing groups for 4 groups as for 8-port SRS - But different from port indexing groups for 2 groups as for 8-port SRS Note that there should be same assumption for coherent relation among ports for 8Tx PUSCH and 8Tx SRS. Therefore, we tend to prefer a same port index grouping method as for SRS. That is: Alt 1 for Ng=2, and Alt 1 for Ng=4. Highly appreciated, if the proponents of Alt 2 for Ng=2 can reconsider above. Proposal 3.2: layer splitting pruning is one way to reduce the number of candidates of precoders. But what is more important should be the number of precoders at last, e.g., with considering #. of precoders of FC 4Tx TPMI for rank 1, 2, 3, and 4. For Ng=2, the number of candidate precoders are listed as: All FC 4Tx TPMI, i.e., 16, 8, 4, 2 for rank 1, 2, 3, 4; Half of FC 4Tx TPMI i.e., 8, 4, 2, 1 for rank 1, 2, 3, 4. Without pruning of layers splitting; 960; 255. Current WA, i.e., keeping (2,1) and (3,2) (4,3); 736; 199. Proposal 3.2, i.e., removing (2,1) and (3,2) (4,3); 568; 157. Current WA+keeping (2,1) and removing (3,2) (4,3); 696; 189. It is obvious that reducing number of candidate FC rank 1-4 for 4Tx TPMI is more effective than pruning layer splitting candidates on reducing the candidates of precoders. Now there are divergent opinions on whether to further remove {2, 1}, (3, 2), and (4, 3), we can live with more layers splitting candidates for more flexibility. Proposal 3.4: Generally we are fine. Similar observation for Ng=4 to Ng=2. For Ng=2, the number of candidate precoders are listed as: All FC 2Tx TPMI, i.e., 4, 2 for rank 1, 2; Half of FC 2Tx TPMI i.e., 2, 1 for rank 1, 2. Without pruning of layers splitting; 2400; 255. Proposal 3.4; 1232; 145. ZTE’s proposal [4]; 1472; 135. We noticed that companies have very divergent opinions on pruning, and they are actually not very different from perspective of reducing candidate of precoders, not as effective as reducing number of 2Tx TPMIs. Therefore, we suggest not to spend too much time on layer splitting pruning: prune some unlikely combinations, and allow less candidates for higher ranks than lower ranks. |
| Lenovo | Proposal 3.1: Support Proposal 3.2: Support Proposal 3.3: Support Proposal 3.4: For rank 4, suggest to add (1,1,1,1). Reduction of the codeword combination for (1,1,1,1) can be FFS. |
| OPPO | Proposal 3.1: Support. Proposal 3.2: Support. We are also fine with CATT’s proposal for further overhead reduction. Proposal 3.3: In our understanding, if RAN1 cannot agree on option 2, the consequence is no restriction for CW to antenna port group mapping. Then the mapping is depended on the supported layer splitting. Hence, we think the necessity of this proposal is unclear. Proposal 3.4: We have similar comment as QC on (2,1,2,1), (1,2,1,2) for rank 6. We think only one layer split is sufficient. |
| IDC | P3.1: Support P3.2: Support P3.3: Not support. Share similar views as Qualcomm. Option 1 allowing the shuffling requires more second level discussions on how to shuffle (as captured in the FFS bullet). P3.4: OK in principle. |
| Ericson | P3.3: Do not agree. The details of the FFS need to be spelled out for Option 1. These details are significant details e.g. how the layer mapping is done, and also how scrambling works. These need to be identified. We are not aware of simulation results showing the gains of shuffling layers among codewords. P3.4: Our comments are: - Rank 3: We can be OK with the change, as it is similar to the rank 3 case for Ng=2, where we did not find much gains for (2,1) on top of (1,2) layer splits. To be conservative, our first preference would still be to keep {(1,2,0,0), (1,0,2,0), (1,0,0,2), (0,1,2,0), (0,1,0,2), (0,0,1,2)}, though. - Rank 4: Suggest to also remove :{(2,1,1,0), (0,2,1,1), (1,0,2,1), (1,1,0,2)}, i.e no transmission by 3 of the 4 antenna groups is needed for rank 4. We have agreed to have the various permutations of group selection precoders (2,2,0,0) and one balanced (1,1,1,1). We saw no gain in including something in between like the splits:{ (2,1,1,0), (0,2,1,1), (1,0,2,1), (1,1,0,2)}. - Rank 5: Agree it’s good to remove most of the splits, as not so many precoders should be needed for higher ranks. Suggest to remove (2,2,1,0), (2,2,0,1) as well: there should not be so much gain and this also complicates the CW2L mapping, since then the first group pair has rank 4 while the second has rank 1, which is inconsistent with the (CW1=rank2,CW2=rank3) DL CW2L mapping. We somewhat prefer to keep (1,1,1,2), but expect it could be removed also without much loss in performance. - Rank 6: Suggest to also remove {(2,2,2,0), (2,2,0,2), (2,0,2,2), (0,2,2,2)}, i.e no transmission by 3 of the 4 antenna groups is needed for rank 6. Similar to rank 5, there should not be so much gain and this also complicates the CW2L mapping, since then the first group pair has rank 4 while the second has rank 2, which is inconsistent with the (CW1=rank3,CW2=rank3) DL CW2L mapping. Agree with keeping only (2,1,2,1), (1,2,1,2) for the 4 of 4 group case; good to reduce precoders at these higher ranks. - Rank 7: Somewhat prefer to keep (1,2,2,2), as this has some potential performance gains by switching ranks among groups. However, at rank 7 we expect these gains may hard to see at the system level. So we can go with the majority view on if (1,2,2,2) is kept or not. |
| Apple | I updated Apple’s views in your summary above. P3.1: we are fine with the current proposal. We would also be fine with CATT’s proposal. P3.3: not support. Do not think this proposal is needed at all. Earlier agreement is sufficient already. P3.4: we are generally fine with the direction. But we think the down-selection should keep the number of precoders <= 1028 to make this exercise worth the effort. So we think some further down-selection would be necessary. |
| Samsung | P3.1: support P3.2: do not support - why are we supporting (2,3) but not (3,2)? What is the technical benefits? - If overhead saving is the main goal, then we should first discuss precoders for a layer split (l1,l2), then discuss whether we support both (l1,l2) and (l2,l1) when l1≠l2. - One simple way to support both (l1,l2) and (l2,l1) is to separate layer ordering/mapping to groups from TPMI, i.e., the mapping can be provided with the codebook subset, and TPMI indicates multiple TPMIs, and UE applies those TPMIs, according to configured ordering. An example is shown in table below, where mapping/ordering x1,x2∈{1,2,2,1}. - Besides, based on our sls results (copied below), we observe that the codebook size can be reduced significantly (e.g. subsampling by 2 or even 4), with a very small loss (1-2%) in avg. UPT. Hence, we don’t need to remove a layer combination. We can support layer splits and reduce the 4Tx FC TPMIs instead. - In short, we are open to discuss pruning (reducing) codebook size, but not by removing layer splits arbitrarily. Rank; Rank of One 4Tx TPMI; Rank of Two 4Tx TPMIs; All 4Tx FC precoders; Subsampled (1/2) of 4Tx FC precoders. 1; lx1=1; 16. 2; lx1=2; 8; lx1,lx2=(1,1); 256; 64. 3; lx1=3; 4; lx1,lx2=(2,1); 96; 32. 4; lx1=4; 2; lx1,lx2=(2,2); 64; 16. 5; lx1,lx2=(3,2); 32; 8. 6; lx1,lx2=(3,3); 16; 4. 7; lx1,lx2=(4,3); 8; 2. 8; lx1,lx2=(4,4); 4; 1. Total; 28; 456; 127. Proposal 3.2: For partially coherent uplink precoding by an 8TX UE, Ng=2, confirm the Working Assumptions with updates: FFS: Ordering/mapping of layers (l1,l2) to groups (G1,G2) or (G2,G1) is configured. P3.3 - Same view as other companies, this proposal is not needed. The layers are determined by the measured channel, hence both (l1, l2) = (2, 3) and (3,2) possible (based on the channel measurements). And in legacy (4Tx PC precoders), the layer indices are determined based on antenna port indices, this is legacy behavior. For example, in this precoder , 1st column is layer 1 and it maps to 1st antenna port group (1,3), and 2nd and 3rd columns are layer 2-3, and they map to 2nd antenna group (2,4). - So, based on the above (which is legacy), it is clear we don’t need any further agreement. P3.4: - We are OK to prune and remove layer combinations. But, we have same comment as Ng=2, if we support a layer (l1,l2,l3,l4), we should support all permutations, not just 1-2 of them. - Re pruning, we have the same view as E/// that layers corresponding to 3 of the 4 antenna groups can be removed for all ranks, at least for rank 3-5. As we can see below, they correspond to far too many (256, 128, 64) rank 3-5 precoders, which is not needed. As shown by several companies, we don’t need very large size codebook for high ranks. - So, we prefer to remove them. - Finally, same comments as Ng=2, the ordering/mapping of antenna groups of layers l1-l4 can be part of codebook subset, this can reduce the overhead a lot. Also, we can support more layer combinations. 3; Transmission by 3 of the 4 antenna groups: (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1) 256. 4; Transmission by 3 of the 4 antenna groups: (2,1,1,0), (0,2,1,1), (1,0,2,1), (1,1,0,2) 128 (1,2,1,0), (1,1,2,0), (0,1,2,1), (0,1,1,2), (1,0,1,2), (2,0,1,1), (2,1,0,1), (1,2,0,1). 5; Transmission by 3 of the antenna groups: (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1), 64 (2,1,2,0), (1,2,2,0), (2,1,0,2), (1,2,0,2), (2,0,1,2), (1,0,2,2), (0,2,1,2), (0,1,2,2) Transmission by 4 of the 4 antenna groups: (1,1,2,1), (1,1,1,2), (2,1,1,1), (1,2,1,1). Proposal 3.4: For partially coherent uplink precoding by an 8TX UE, Ng=4, - The following rank and layer splitting cases are supported, FFS: Ordering/mapping of layers (l1,..,l4) to groups (G1,…,G4) or (G4,…G1) is configured. |
| Nokia, NSB | Proposal 3.3: not agree. Option 1 is a wide open design with too much FFS, while Option 2 is very clear. Shuffling layers would have significant performance impact. Proposal 3.4: not agreeable and might need further work during the meeting. Some comments: Rank 3: “Transmission by 2 of the 4 antenna groups:” has 96 precoders; while “Transmission by 3 of the 4 antenna groups” has 256 precoders. We need to trim “Transmission by 3 of the 4 antenna groups” for rank 3, while the proposal does not. Other reducing is in the right direction. However, we need to associate the number of precoders to have a general picture on the overhead. |
| FL | Updated proposals based on offline session: Proposal 3.4: For partially coherent uplink precoding by an 8TX UE, Ng=4, - The following rank and layer splitting cases are supported, Rank; All layers in one Antenna Group; Layers split across 4 Antenna Groups (All possible permutations). 3; Transmission by 2 of the 4 antenna groups: (2,1,0,0), (2,0,1,0), (2,0,0,1), (0,2,1,0), (0,2,0,1), (0,0,2,1), Transmission by 3 of the 4 antenna groups: (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1). 5; Transmission by 3 of the antenna groups: (2,0,2,1), (0,2,2,1), Transmission by 4 of the 4 antenna groups: (1,1,2,1). 6; Transmission by 3 of the 4 antenna groups: (2,2,2,0), (2,0,2,2) Transmission by 4 of the 4 antenna groups: (2,1,2,1). 7; Transmission by 4 of the 4 antenna groups: (2,1,2,2). Proposal 3.2a: For partially coherent uplink precoding by an 8TX UE, Ng=2, confirm the Working Assumptions with updates: - Following combinations of layer splitting are supported Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (1,2), (2,1). 4; (4,0), (0,4). 4; (2,2). 5; (2,3), (3,2). 6; (3,3). 7; (3,4), (4,3). Support: CATT, ZTE, Xiaomi, Ericsson, Qualcomm, MediaTek, OPPO, Lenovo, InterDigital, CMCC, vivo, Nokia, NTT DOCOMO Proposal 3.2b: For partially coherent uplink precoding by an 8TX UE, Ng=2, - At least the following combinations of layer splitting are supported - For rank>4, all the layers for each CW is mapped to only one antenna group Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (1,2), (2,1). 4; (4,0), (0,4). 4; (2,2). 5; (2,3), (3,2). 6; (3,3). 7; (3,4), (4,3). |

# Codebook Configuration Aspects

Agreement (#110) For 8TX PUSCH, at least support - Ng=1, 2, 4 Note: The above does not restrict the Ng for the non-coherent case Agreement (112b-e) For codebook -based 8TX PUSCH transmission, down-select from, - Alt1 - A fully-coherent UE (Ng =1) can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =1, 2, 4, 8 - FFS which combinations of Ng value(s), to be considered - A partially-coherent UE , with Ng =2 can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =2, 4, 8 - FFS which combinations of Ng value(s), to be considered - A partially-coherent UE , with Ng =4, can be configured with precoders considered for at least one or more Ng cases, i.e., Ng= 4, 8 - FFS which combinations of Ng value(s), if any, to be considered - A non-coherent UE , Ng =8, can only be configured with precoders considered for Ng = 8 - Alt2 - A fully-coherent UE (Ng =1) can only be configured with precoders considered for one of Ng cases, i.e., Ng =1, 2, 4, 8 - FFS which Ng value(s), to be considered - A partially-coherent UE , with Ng =2, can only be configured with precoders considered for one of Ng cases, i.e., Ng =2, 4, 8 - FFS which Ng value(s), to be considered - A partially-coherent UE , with Ng =4, can only be configured with precoders considered for one of Ng cases, i.e., Ng =4, 8 - FFS which Ng value(s), to be considered - A non-coherent UE , with Ng =8, can only be configured with precoders considered for Ng = 8 - FFS whether/how the configuration can be done via RRC or MAC-CE. - Alt3 - A fully-coherent UE (Ng =1) can only be configured with precoders considered for Ng =1 - A partially-coherent UE , with Ng =2, can only use precoders considered for Ng =2 - A partially-coherent UE , with Ng =4, can only use precoders considered for Ng =4 - A non-coherent UE , with Ng =8, can only use precoders considered for Ng = 8 - Other alternatives are not precluded Note: For an 8TX UE, Ng =8 can represent a non-coherent UE.

Based on the agreement from the last meeting, there are three alternatives for down-selection in this meeting. Based on the discussion, it seems that Alt2 may be a reasonable compromise between Alt1 and Alt3. Once, an alternative is agreed, discussion on details will be followed.

Proposal 4.1: For codebook-based 8TX PUSCH transmission, Alt2 is supported, where

- Alt2

  - A fully-coherent UE (Ng =1) can only be configured with precoders considered for one of Ng cases, i.e., Ng =1, 2, 4, 8

    - FFS which Ng value(s), to be considered

  - A partially-coherent UE, with Ng =2, can only be configured with precoders considered for one of Ng cases, i.e., Ng =2, 4, 8

    - FFS which Ng value(s), to be considered

  - A partially-coherent UE, with Ng =4, can only be configured with precoders considered for one of Ng cases, i.e., Ng =4, 8

    - FFS which Ng value(s), to be considered

  - A non-coherent UE, with Ng =8, can only be configured with precoders considered for Ng = 8

  - FFS whether/how the configuration can be done via RRC or MAC-CE.

Proposal 4.1: For codebook-based 8TX PUSCH transmission, Alt1 is supported, where

- Alt1

  - A fully-coherent UE (Ng =1) can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =1, 2, 4, 8

    - FFS which combinations of Ng value(s), to be considered

  - A partially-coherent UE , with Ng =2 can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =2, 4, 8

    - FFS which combinations of Ng value(s), to be considered

  - A partially-coherent UE , with Ng =4, can be configured with precoders considered for at least one or more  Ng cases, i.e., Ng= 4, 8

    - FFS which combinations of Ng value(s), if any, to be considered

  - A non-coherent UE , Ng =8, can only be configured with precoders considered for Ng = 8

Alt1: ZTE, Qualcomm, NEC, Spreadtrum, CATT, Lenovo, Xiaomi, CMCC, Huawei, Ericsson, Apple

Alt2: vivo, NTT, Samsung, MediaTek, LG, InterDigital

Over the last several meetings, companies have discussed configuration aspect related to the 8TX UE. One relevant aspect is whether a UE with a higher coherence capability can also support transmission associated with lower coherency. Therefore, Proposal 4.2 is intended to resolve this issue. It should be noted that Proposal 4.2 does not have any overlap or conflict with proposal 4.1.

Proposal 4.2: For an 8TX UE, down-select from Option 1 is supported,

- Option 1 – Subject to its capability, an 8TX UE may report more than one Ng value as part of its UE capability. Based on which, gNB may select and configure UE with one of supported Ng values.

- Option 2 – An 8TX UE may report only a single Ng value as part of its UE capability. No RRC configuration is needed.

Table 2 - Companies’ views

| Company | Perspective |
| --- | --- |
| FL | Proposals 4.1 and 4.2 are updated based on companies’ inputs. |
| CATT | Proposal 4.1: Support. We prefer to extend the legacy Rel-15 scheme to the Rel-18 8TX design thus Alt1 is supported which aligns with all the scenarios. Proposal 4.2: Not support. For Option 1, it is our view that gNB can select and configure UE with one or multiple Ng values. Besides, Regarding the UE capability, for fully-coherent UEs, partially-coherent UEs and non-coherent UEs, at least the following configurations is supported by the UEs: For fully-coherent UEs (Ng =1): - One Ng cases from Ng = 1,2, 4,8, i.e. (1), (2), (4), (8); For partially-coherent UEs with Ng =2: - One Ng cases from Ng = 2, 4,8, i.e. (2), (4), (8); For partially-coherent UEs with Ng =4: - One Ng cases from Ng = 4,8, i.e. (4), (8); For non-coherent UEs with Ng =8:Ng=8. Other combinations of Ng values can be UE capability: |
| NTT DOCOMO | For Proposal 4.1, if we go with Alt1, we should add some limitations on the nested codebook and the final precoder number of the nested codebook. For example, for fully-coherent UEs, only consider nested codebook of fully-coherent precoders and non-coherent precoders. And the nested codebook could not cause TPMI field increase or the TPMI field increase is up to 1 bit. For Proposal 4.2, we do not think additional UE capability is needed. For example, a fully-coherent UE can support non-coherent precoders without additional UE capability. |
| LG Electronics | For proposal 4.1, we have preference on Alt 2 which is middle ground of Alt 1 and 3. If we go with Alt1, more discussion on # of codebook size is needed. For proposal 4.2, we also don’t support. Additional UE capability on Ng is not needed. |
| FGI | For proposal 4.1, we support Alt 1. Furthermore, the size of the codebook should be limited while considering flexible codebook subset configuration. For example, a partial-coherent UE, with Ng=2, can only be configured with precoders corresponding to Ng=2 and/or Ng=8 cases. For proposal 4.2, agree with CATT. |
| Huawei, HiSilicon | For proposal 4.1, support. For proposal 4.2, to be aligned with proposal 4.1, we think the following modification is needed: Proposal 4.2: For an 8TX UE, down-select from Option 1 is supported, - Option 1 – Subject to its capability, an 8TX UE may report more than one Ng value as part of its UE capability. Based on which, gNB may select and configure UE with one or more of supported Ng values. - Option 2 – An 8TX UE may report only a single Ng value as part of its UE capability. No RRC configuration is needed. |
| QC | We support Alt 1 in general. But to reduce the complexity of gNB scheduler to do precoder selection, and also reduce UE’s complexity to do capability reporting, we suggest to simplify Alt 1 by allowing only one additional Ng, which is Ng=8. In our understand, the main motivation for this codebook subset restriction is avoid power waste in case of hand-blockage. Allowing an additional noncoherent codebook for each Ng (Ng=1, or 2, or 4) can serve the purpose of avoid power waste very well. And it does not need complicated UE capability reporting neither. UE just need to report a single Ng, rather than a set of Ngs. Then we don’t need to discuss which combinations of Ngs are allowed for reporting. |
| ZTE | Proposal 4.1: Agree. Proposal 4.2: Not support. - Regarding UE capability report, we prefer UE to report one or more supported Ng values, then gNB can configure one or more Ng values with subjective to UE capabilities. - Therefore, the update from Huawei looks good to match proposal 4.1. |
| vivo | Proposal 4.1, do not support. The reason is that partial-coherent and non-coherent precoders designs are not complete yet. We don’t know what proponents are considering on TMPI size, if all precoders for FC and PC and NC are supported in the set then TPMI size could be very large. Proposal 4.2, do not support. |
| Lenovo | Proposal 4.1: Do not support. This should be discussed after the codebooks are stabilized, so the benefit for configuring different codebooks can be analyzed. Proposal 4.2: Do not support. Only a single Ng value should be reported. For the reported Ng value, the codebooks supported can be reported as a separate UE capability. |
| CMCC | For proposal 4.1: Support. For proposal 4.2: We also think that gNB may select and configure UE with one or more of supported Ng values. |
| OPPO | Proposal 4.1: Do not support. Alt.1 can be discussed when the codebook design is finalized. If we need to make a decision now, Alt.2 is the safer way. Proposal 4.2: We also think UE only needs to report single value of Ng, and the codebook subset capability can be extended from Rel-15 UE capability reporting. |
| IDC | P4.1: Our concern on Alt1 is that it may be possible to enable two different codebook construction methods can co-exist in the DCI field(s), e.g., both Ng=1 and Ng=2 based precoder sets are needed in the same DCI, similarly mentioned by vivo. So, we prefer Alt2 that is a middle ground and flexible enough. P4.2: Support, and Huawei’s update is OK. |
| Ericsson | P4.1: OK. We think Ng=(1,8) is the most logical/important. Ng=(2,8) or (4,8) also make sense to us, but may not be so crucial. Other combinations are certainly possible, but the need is not as clear to us. P4.2: Option 1 is OK, but P4.2 still seems to us to overlap with P4.1. Can we clarify, e.g: - Option 1 – Subject to its capability, an 8TX UE may report more than one Ng value as part of its UE capability. Based on which, gNB may select and configure UE with one of supported Ng values. - Whether a UE reporting one value of Ng capability supports a codebook containing precoders for more than one Ng case is to be discussed separately. |
| Apple | P4.1: OK in principle, but we would suggest “FFS: further down-selection of precoders for codebook subset restriction”. We have some sympathy on the comment that given the codebook design is not completed yet, it is not possible to assess the full impact of agreeing to an alternative. P4.2: OK in general. Whether gNB may configured one or more supported Ng values depends on the outcome of P4.1. |
| Samsung | P4.1: we prefer Alt2, and can live with Alt1 for the following (since codebook size can be OK for these): - FC UE: Ng=1,8 - When codebook subset Ng=1 and 8, codebook size <=256 - PC UE: Ng=2,8 - When codebook subset Ng=2 and 8, codebook size <=256 - PC UE: Ng=4,8 - When codebook subset Ng=4 and 8, codebook size <=256 - NC UE: - codebook size <=32 For the remaining, the codebook size can be very large, so we prefer Alt2. - FC UE: Ng=1,2,8 or 1,4,8 or 1,2,4,8 - PC UE: Ng=2,4,8 |
| Nokia, NSB | P4.1: We are concerned about more than one Ng cases. This is certainly associated with UE capability. We are supportive this this modification: Proposal 4.1: For codebook-based 8TX PUSCH transmission, Alt1 is supported, where - Alt1 - A fully-coherent UE (Ng =1) can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =1, 2, 4, 8 - FFS which combinations of Ng value(s), to be considered - A partially-coherent UE , with Ng =2 can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =2, 4, 8 - FFS which combinations of Ng value(s), to be considered - A partially-coherent UE , with Ng =4, can be configured with precoders considered for at least one or more Ng cases, i.e., Ng= 4, 8 - FFS which combinations of Ng value(s), if any, to be considered - A non-coherent UE , Ng =8, can only be configured with precoders considered for Ng = 8 P4.2: support, with the above P4.1 |

# Support of Two Codewords

Agreement To support dual CW PUSCH operation by an 8TX UE, if CBG-based transmission is configured, the DL principle for CBGTI DCI field is reused where, - The first half of CBGTI field bits is used to indicate the transmission state of CBGs of the first transport block, while the second half of CBGTI field bits is used to indicate the transmission state of CBGs of the second transport block. - The bit field may be configured to have a length of N bits that can support operation of N/2 CBGs , where N=[2, 4, 6 or 8].

In the last meeting, support of CBG-based transmssion for dual CW PUSCH transmission was agreed, however the range of values for the CBGTI bit field was left for confirmation in this meeting.

Proposal 5.1: To support CBG-based transmission for dual CW PUSCH operation, the range N=[2, 4, 6 or 8] is confirmed for the CBGTI bit field.

Support of CG PUSCH transmission has been proposed by several companies. Given the limited time left for completion of Rel-18, a decision on this topic needs to be made in this meeting.

Proposal 5.2:

Version A: Support CG for dual CW PUSCH transmission by an 8TX UE,

- For Type-1 CG: A second mcsAndTBS parameters is configured in rrc-ConfiguredUplinkGrant

- For Type-2 CG: A second MCS field is added in DCI formats (0_1, 0_2)  activating a Type-2 CG-PUSCH

Version B: Conclusion: For dual CW PUSCH transmission by an 8TX UE, there is no consensus to support CG transmission.

Concern: vivo, Ericsson

Proposal 5.3: For dual CW PUSCH transmission by an 8TX UE, PHY layer priority indicator (if configured) is applied on both codewords.

Proposal 5.4: For dual CW PUSCH transmission by an 8TX UE, UL-SCH indicator is applied on both codewords.

Table 3 - Companies’ views

| Company | Perspective |
| --- | --- |
| FL | Proposal 5.1 is stable, and no further comment is required. Proposal 5.2 is updated based on companies’ inputs. |
| CATT | Proposal 5.1: Support. Proposal 5.2: Not support. We prefer to version A. |
| NTT DOCOMO | Proposal 5.2: a conclusion on no consensus is too early. Let’s finish the design on CB first. |
| LG Electronics | For proposal 5.2, agree with Docomo. |
| FGI | For proposal 5.2, agree with DCM. |
| QC | Proposal 5.1: we support the proposal. Proposal 5.2: We are surprised version B is recommended as a WF. Given vast majority supporting version A, I think RAN1 should go with version A, because there is no technical reason to exclude CG out of the scope of this WI. Also, we don’t think there are large amount of additional work needed to specific CG-PUSCH with 2 CW. Can VIVO or Ericsson clarify what is the main concerns to take version A? By the way, the wording in WID and existing agreement on 2CW already implies CG-PUSCH with 2 CWs are supported in Rel-18, because the wording we used in WID and previous agreements are generic. We never limit the scope to only DG-PUSCH, which means both CG-PUSCH and DG-PUSCH are included. |
| ZTE | Proposal 5.2: Not support. It is obvious that version A reflects the majority of views according to last round opinion. It does not need much additional spec efforts to support 2CW for CG PUSCH on basis of dual CW of DG PUSCH. At least version A should be at least supported. Then the following part we discussed in [4] can be further determined. - FFS: Whether RV sequence parameter (repK-RV) is commonly or individually configured to two codewords. |
| vivo | Fine with proposal 5.2 |
| Lenovo | Proposal 5.1: Support Proposal 5.2: Fine with us. |
| CMCC | Proposal 5.1: Support. Proposal 5.2: Not support. Prefer version A. |
| KDDI | Proposal 5.1: We would like to confirm majority’s understanding of range N (=2, 4, 6 or 8). Does the “N” correspond to the RRC parameter maxCodeBlockGroupsPerTransportBlock which is defined in TS 38.331? If that understanding is correct, in PDSCH, where dual CW transmission has already been specified, the value range is limited to 2 and 4 for dual CW, as described in TS 38.331 highlighted in red below. ---------- PDSCH-CodeBlockGroupTransmission ::= SEQUENCE { maxCodeBlockGroupsPerTransportBlock ENUMERATED {n2, n4, n6, n8}, codeBlockGroupFlushIndicator BOOLEAN, ... } maxCodeBlockGroupsPerTransportBlock Maximum number of code-block-groups (CBGs) per TB. In case of multiple CW, the maximum CBG is 4 (see TS 38.213 [13], clause 9.1.1). ---------- On the contrary, if “N/2” in the agreement of previous RAN1 meeting corresponds to maxCodeBlockGroupsPerTransportBlock and the value range is the same as that in the PDSCH specification described above, then “N/2” is limited to be 2 and 4, which means that “N” is restricted to be 4 and 8 in case of dual CW. Proposal 5.2: agree with Docomo. |
| IDC | P5.1: Support. P5.2: Prefer Version A. |
| Apple | P5.1: agree with KDDI’s comment. Should clarify. My understanding is that we simply follow PDSCH design. P5.2: also surprised to see the current proposal. Alt 1 seemed to be a clear majority. |
| Samsung | P5.1: OK |
| Nokia, NSB | P5.2: also prefer Version A |
| FL | Proposal 5.3: For dual CW PUSCH transmission by an 8TX UE, PHY layer priority indicator (if configured) is applied on both codewords. Proposal 5.4: For dual CW PUSCH transmission by an 8TX UE, UL-SCH indicator is applied on both codewords. |

# Full Power Operation

Agreement Framework for full power PUSCH transmission by an 8TX UE - To support full power transmission with Mode0, Rel-16 Mode0 (fullPower ) is re-used. - FFS if any change is required in the specifications. - Working Assumption To support full power transmission with Mode1, Rel-16 Mode1 (fullPowerMode1) is re-used. - FFS if more than one of the 8TX full coherent precoders is used per rank. - Working Assumption To support full power transmission with Mode2, Rel-16 Mode2 (fullPowerMode2) is re-used. - FFS definition of precoder groups (G0, G1, …) - FFS enhancements for SRS configuration

Since the design of the full-coherent precoder is near complete, support of full power transmission using Rel-16 Mode1 can be agreed. As for support of Mode2, first at least one PA architecture needs to be agreed. Based on the discussions in the last several meetings, following proposal can be considered.

Proposal 6.1: For full power PUSCH transmission by an 8TX UE, confirm the Working Assumptions with updates:

- To support full power transmission with Mode1, Rel-16 Mode1 (fullPowerMode1) is re-used.

  - FFS if more than one of the 8TX full coherent precoders is used per rank.

- To support full power transmission with Mode2, Rel-16 Mode2 (fullPowerMode2) is re-used.

  - At least the PA powers per TX chain of [-3 -3 -3 -3 -3 -3 -3 -3] dB relative to their power class is considered for

    - definition of precoder groups (G0, G1, …)

    - identifying enhancements for SRS configuration

Proposal 6.1a: For full power PUSCH transmission by an 8TX UE, confirm the Working Assumptions for Mode1 with updates:

- To support full power transmission with Mode1, Rel-16 Mode1 (fullPowerMode1) is re-used.

  - FFS if more than one of the 8TX full coherent precoders is used per rank.

Proposal 6.1b: For full power PUSCH transmission by an 8TX UE, confirm the Working Assumptions for Mode2 with updates:

- To support full power transmission with Mode2, Rel-16 Mode2 (fullPowerMode2) is re-used.

  - At least following PA architectures are considered,

    - [-9 -9 -9 -9 -9 -9 -3 -3] dB relative to their power class, (Low priority Example 2a from RAN1#110)

    - [-9 -9 -9 -9 -6 -6 -6 -6] dB relative to their power class, (Low priority Example 3a from RAN1#110)

  - definition of precoder groups (G0, G1, …)

  - identifying enhancements for SRS configuration

Table 4 - Companies’ views

| Company | Perspective |
| --- | --- |
| FL | Proposal 6.1 is split to two separate proposals, each addressing a specific mode. |
| CATT | We prefer to postpone the discussion on whether and how to support full power transmission Mode 1 and Mode 2 after the codebook design is finalized. |
| NTT DOCOMO | For Proposal 6.1a, before discussing the FFS on more than one, maybe we need a bullet on whether support to add one fully-coherent precoder per rank first? |
| LG Electronics | For proposal 6.1a, Ok with proposal. For proposal 6.1b, it can be discussed after finalizing the codebook. |
| Huawei, HiSilicon | We prefer to discuss full mode 1 and 2 after the CB is finalized. |
| QC | Proposal 6.1a: we support the proposal. Proposal 6.1b: We don’t support the proposal. We are not sure why [-9 -9 -9 -9 -9 -9 -3 -3] dB or [-9 -9 -9 -9 -6 -6 -6 -6] dB are prioritized. This prioritization looks quite ad hoc and we don’t support it. We are either OK to confirm the working assumption on mode 2 as it is, or deprioritize mode 2 due to the fact there is no clear picture on PA architecture available. |
| ZTE | Proposal 6.1a: Agree. Proposal 6.1b: Generally fine. But for mode 2, if adopting full power precoder groups such as G0, G1, ... like in Rel-16, it would cause huge spec efforts to list possible precoders for a group due to large number of precoders for 8-Tx. We suggest to keep FFS as in WA of last meeting as follows: Proposal 6.1b: For full power PUSCH transmission by an 8TX UE, confirm the Working Assumptions for Mode2 with updates: - To support full power transmission with Mode2, Rel-16 Mode2 (fullPowerMode2) is re-used. - At least following PA architectures are considered, - [-9 -9 -9 -9 -9 -9 -3 -3] dB relative to their power class, (Low priority Example 2a from RAN1#110) - [-9 -9 -9 -9 -6 -6 -6 -6] dB relative to their power class, (Low priority Example 3a from RAN1#110) - FFS definition of precoder groups (G0, G1, …) - identifying enhancements for SRS configuration |
| Lenovo | Proposal 6.1a: Support Proposal 6.1b: Support the main bullet. The sub-bullets need more discussion, including where to place the ports with -3 or -6dB. |
| CMCC | Proposal 6.1a: Support. Proposal 6.1b: Prefer to firstly discuss enhancements for SRS configuration which has no relationship with PA architectures. |
| OPPO | Proposal 6.1a: Support. Proposal 6.1b: Not support. We think mode 2 can be discussed after the codebook is finalized. |
| IDC | P6.1a: Support. P6.1b: OK in principle. |
| Ericsson | P6.1a and P6.1b: Do not support splitting Mode 1 from Mode 2. Mode 1, Mode 2 w/ full power TPMI, and Mode 2 with SRI indication can all be supported with prioritized PA configurations, with a few simple configurations. P6.1a: OK with the FFS, but don’t see a need for more than one full coherent precoder per rank. P6.1b: The last 3(?) meetings there have been proposals to add PA architectures, and companies consistently have had concerns. Adding the two new ones here only adds work. To be clear: supporting more PA architectures is fine for us in principle, but given the concerns from many companies on progress, any new PA architectures should be FFS, and then we can address them once the basic precoder groups and SRS configurations are agreed. The most straightforward solution here to us again is to agree to a simple set of configurations, and then build on that: Proposal: For full power Modes 1 and 2, support the following: - Full power mode 1 can be configured with at most one TPMI per each of ranks 1-8. - Full power mode 2 with full power TPMIs can be configured with at most one TPMI per each of ranks 1-8. - Full power mode 2 with multiple SRS resources can be configured with one or more of 1, 2, 4, or 8 SRS port resources in an SRS resource set |
| Apple | In generally we prefer to discuss this after CB design is done. We think it is good to discuss Mode 1 and Mode 2 separately. |
| Samsung | P6.1a: OK P6.1b: OK, but not sure why the two PA architectures need to be included in the proposal. Since the definition of precoder groups is still FFS. Proposal 6.1b: For full power PUSCH transmission by an 8TX UE, confirm the Working Assumptions for Mode2 with updates: - To support full power transmission with Mode2, Rel-16 Mode2 (fullPowerMode2) is re-used. - At least following PA architectures are considered, - [-9 -9 -9 -9 -9 -9 -3 -3] dB relative to their power class, (Low priority Example 2a from RAN1#110) - [-9 -9 -9 -9 -6 -6 -6 -6] dB relative to their power class, (Low priority Example 3a from RAN1#110) - definition of precoder groups (G0, G1, …) - identifying enhancements for SRS configuration |
| Nokia, NSB | P6.1a: support P6.1b: we can confirm the WA. Regarding to the PA architectures part, we can remove them to make the proposal agreeable. |

# TRI/SRI/TPMI Indication for Codebook UL Transmission

Conclusion For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook (CodebookMode=1), there is no consensus to support any optional over-sampling ratio.

| Configuration parameters | Number of precoders using NR Rel-15 single panel DL Type I |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (N1, N2, O1, O2) | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | Total |
| (2, 2, 1, 1) | 16 | 32 | 24 | 24 | 8 | 8 | 8 | 8 | 128 |
| (4, 1, 1, 1) | 16 | 32 | 24 | 24 | 8 | 8 | 4 | 4 | 120 |

The design of fully-coherent precoder based on Rel-15 DL codebook was concluded. Based on the agreed codebook and (N1, N2) and (O1, O2) values, precoder indication can now be discussed.

Proposal 7.1: For PUSCH transmission by a full-coherent 8TX UE, support the following tables for indication of full-coherent precoders,

- For (N1, N2) = (4, 1)

| Index | Rank | i1,1 | i1,2 | i1,3 | i2 |
| --- | --- | --- | --- | --- | --- |
| 0-15 | 1 | {0, 1, 2, 3} | {0} | - | {0, 1, 2, 3} |
| 16-47 | 2 |  |  | {0, 1, 2, 3} | {0, 1} |
| 48-71 | 3 |  |  | {0, 1, 2} |  |
| 72-95 | 4 |  |  |  |  |
| 96-103 | 5 |  |  | - |  |
| 104-111 | 6 |  |  |  |  |
| 112-115 | 7 | {0, 1} |  |  |  |
| 116-119 | 8 | {0, 1} |  |  |  |

- For (N1, N2) = (2, 2)

| Index | Rank | i1,1 | i1,2 | i1,3 | i2 |
| --- | --- | --- | --- | --- | --- |
| 0-15 | 1 | {0, 1} | {0, 1} | - | {0, 1, 2, 3} |
| 16-47 | 2 |  |  | {0, 1, 2, 3} | {0, 1} |
| 48-71 | 3 |  |  | {0, 1, 2} |  |
| 72-95 | 4 |  |  |  |  |
| 96-103 | 5 |  |  | - |  |
| 104-111 | 6 |  |  |  |  |
| 112-119 | 7 |  |  |  |  |
| 120-127 | 8 |  |  |  |  |

Note: Details related to capturing of the agreement, including formulation, ordering, indexing, etc., is up to Editor’s discretion. The indexing may be further refined by the editor.

Proposal 7.2: For non-coherent uplink precoding by an 8TX UE, support Alt2.,

- Alt1. – All 255 combinations from 8 non-coherent rank1 precoders are supported

- Alt2. – Only a subset of Alt1. is supported, striving for a substantial reduction in the number of precoders

  - Up to 5 bits is used for TPMI indication, where

| Rank | Number of precoders |
| --- | --- |
| 1 | 8 |
| 2 | 8 |
| 3 | 6 |
| 4 | 4 |
| 5 | 2 |
| 6 | 2 |
| 7 | 1 |
| 8 | 1 |
| Total | 32 |

  - FFS identification of precoders for each rank ≥ 2

Proposal 7.3: For partially coherent 8TX precoding with Ng =2, the precoder is based on up to two full-coherent 4TX precoders. Option 3 is supported, where

- Option 3 – Up to two 4TX TPMIs are indicated,

  - When two TMPIs are indicated, the first is applied on one of antenna group, and the second is applied on the other antenna group,

  - FFS : details of TPMI indication when one antenna group is used

- Option 4 – A single 8TX TPMI is indicated

- Other options are not precluded

Proposal 7.4: For PUSCH transmission by an 8TX UE, the sum of precoders for partial and non-coherent precoders is at most the same as the number of supported full-coherent precoders, e.g., 128.

Table 5 - Companies’ views

| Company | Perspective |
| --- | --- |
| FL | Proposal 7.1: The Proposal is further clarified. Proposal 7.2: New proposal based on companies’ contributions. Proposal 7.3: New proposal based on companies’ contributions. Proposal 7.4: New proposal based on companies’ contributions. |
| CATT | Proposal 7.1: Not support. For full-coherent precoders, we support to reuse the legacy TPMI/TRI indication scheme as in Rel-15, i.e., a single 8TX TPMI is jointly indicated with TRI. Proposal 7.2: We prefer to alt1. Still, we are open for the discussion and are fine with only a subset of Alt1 if reasonable subsets are provided. Proposal 7.3: Not support. We think at least when maxRank = 1 or 2 is configured for the 8TX PUSCH, option 4 is adopted since significant overhead can be reduced by option 4 compared to option 3. Option 3 is only acceptable for the case with maxRank > 2 configured for the 8TX PUSCH Proposal 7.4: Not support. More discussion is needed. |
| NTT DOCOMO | Proposal 7.1: Generally okay. But it is not a complete solution if nested codebook is supported. Proposal 7.2: We have concern on whether 32 non-coherent precoders could have good performance for different types of UE implementations on antenna layout. Proposal 7.3: Not support. First, if nested codebook is supported, unified DCI indication method should be considered for partially-coherent precoders and non-coherent precoders. Second, for Ng=2, since some layer split cases have been excluded, there could be unused codepoints by two TPMI fields, which increases DCI indication overhead. Proposal 7.4: Not support. Do not understand the reason to compare precoder number of fully-coherent precoders with sum number of partial and non-coherent precoders. |
| LG Electronics | Proposal 7.1: Not support. It can be totally upto editor how to capture in the specification. Proposal 7.2: We have preference on Alt1. This is also related to the codebook subset design. If the codebook subset design in Alt 2 is supported, there are less limitation on supporting codebook. Proposal 7.3: Support. Proposal 7.4: Not support. It needs more discussion. |
| Huawei, HiSilicon | For Proposal 7.1: For fully coherent precoders, we prefer to separately indicate rank and TPMI to simply reuse DL indication scheme. For Proposal 7.2: The performance of 5-bit precoders is not clear, whether there’s large performance loss should be evaluated. In addition, considering Proposal 4.1, we think it is necessary to align the indication overhead for different Ng. Because fully coherent precoders require 7 bits and partially coherent precoders require 8~10 bits, it is more favourable to use similar number (7~10) of bits for non-coherent precoders. For Proposal 7.3: support. For Proposal 7.4: not support. We think the number of partially coherent precoders with Ng=2 and Ng=4 should be comparable with that of fully coherent precoders. Further discussion is needed. |
| QC | Proposal 7.1: Maybe we missed something. But it seems to us that there is a bug in the agreed 8 Tx CB for (N1=4, N2=1). Let’s take a simple example, for rank 8, with i2=0, the two precoders generated with i11=0, and i11=1 are essentially identical, with only shifted columns (because the l,l’,l’’,l’’’ = 0,1,2,3 with i11=0; and l,l’,l’’,l’’’ = 1,2,3,0 with i11=1, which is just a shift). And we all know that shifting rows does not change a precoding matrix. The bug seems exist for other rank as well. We did the exercise for all ranks and listed the # unique precoders for each rank (excluding those duplicated ones with shift columns) in the following tables. We really hope this is just a false alarm due to we missed something. But before the group verifying this, we don’t think we should agree with proposal 7.1. ULA 4*1. Rank; # precoders w/ duplication; # Unique precoders w/o duplication; Notes of codebook construction method. Rank 1; 16; 16; CodebookMode=1, N1=4, N2=1, O1=1, O2=1, i2=0,1,2,3. Rank 2; 32; 32; CodebookMode=1, N1=4, N2=1, O1=1, O2=1, i2=0,1, i1,3=0,1,2,3. Rank 3; 24; 24; CodebookMode=1, N1=4, N2=1, O1=1, O2=1, i2=0,1, i1,3=0,1,2. Rank 4; 24; 12; CodebookMode=1, N1=4, N2=1, O1=1, O2=1, i2=0,1, i1,3=0,1,2. Rank 5; 8; 8; CodebookMode=1, N1=4, N2=1, O1=1, O2=1, i2=0,1. Rank 6; 8; 5; CodebookMode=1, N1=4, N2=1, O1=1, O2=1, i2=0,1. Rank 7; 4; 3; CodebookMode=1, N1=4, N2=1, O1=1, O2=1, i2=0,1. Rank 8; 4; 3; CodebookMode=1, N1=4, N2=1, O1=1, O2=1, i2=0,1. Sum; 120. UPA 2*2. Rank; # precoders w/ duplication; # Unique precoders w/o duplication; Notes of codebook construction method. Rank 1; 16; 16; CodebookMode=1, N1=2, N2=2, O1=1, O2=1, i2=0,1,2,3. Rank 2; 32; 32; CodebookMode=1, N1=2, N2=2, O1=1, O2=1, i2=0,1, i1,3=0,1,2,3. Rank 3; 24; 24; CodebookMode=1, N1=2, N2=2, O1=1, O2=1, i2=0,1, i1,3=0,1,2. Rank 4; 24; 12; CodebookMode=1, N1=2, N2=2, O1=1, O2=1, i2=0,1, i1,3=0,1,2. Rank 5; 8; 8; CodebookMode=1, N1=2, N2=2, O1=1, O2=1, i2=0,1. Rank 6; 8; 5; CodebookMode=1, N1=2, N2=2, O1=1, O2=1, i2=0,1. Rank 7; 8; 5; CodebookMode=1, N1=2, N2=2, O1=1, O2=1, i2=0,1. Rank 8; 8; 5; CodebookMode=1, N1=2, N2=2, O1=1, O2=1, i2=0,1. Sum; 128. Proposal 7.2: Don’t support this proposal. We’d like to discuss simulation results before go with Alt 2. Even we go with Alt 2, we are not sure why the # precoders per rank should go as indicated in the proposal. We think the robustness of noncoherent codebook in case of handblocking need to be considered in codebook size reduction. Assuming once a port is blocked, the corresponding port on the other polarization is blocked as well. A more reasonable # precoder distribution per rank can be the following. rank; TPMI size; Ports selection. 1; 8; (0),(1),(2),(3),(4),(5),(6),(7). 2; 4; (0,4),(1,5),(2,6),(3,7). 3; 6; (0,4,1) ,(0,4,2) ,(0,3,7) ,(1,5,2),(1,5,3), (2,6,3). 4; 6; (0,4,1,5),(0,4,2,6),(0,4,3,7),(1,5,2,6),(1,5,3,7),(2,6,3,7). 5; 4; (0,1,5,2,6), (0,4,1,3,7), (0,4,2,6,3), (1,5,2,3,7). 6; 2; (0,4,1,5,3,7),(0,4,2,6,3,7). 7; 1; (0,4,1,5,2,6,3). 8; 1; (0,4,1,5,2,6,3,7). total; 32. Proposal 7.4: We don’t support this proposal. Like other companies commented, this needs more discussion. We don’t follow the logic of the proposal. |
| ZTE | Proposal 7.2: Support. Proposal 7.3: Support. Proposal 7.4: Not support. As observed in section 3, even half of FC 2Tx/4Tx TPMI and the most restrict layers splitting cannot meet this requirement of less than 128. So this proposal of less than 128 is meaningless. Maybe a larger number, such as 256 is more possible. |
| vivo | Proposal 7.2: do not support. We are open to discuss subset of alt1 if there is consensus. Current proposal of 32 precoders is too little Proposal 7.3: support Proposal 7.4: do not support. At least consider 128 precoders for PC and 128 or less precoders for NC |
| Lenovo | Proposal 7.1: Support. Proposal 7.2: Do not support. The design of the table (the number of precoders for each rank) needs to be justified. Because the rank 1 precoders for NC UE are just port selection codewords, and the channels from these ports to the TRP are completely determined by the channel (no precoding is possible), any codeword combination of a certain rank is just as good as another. We do not see a clear justification for choosing a subset of the codewords. Proposal 7.3: This should be discussed after the codebook is stable. Proposal 7.4: This should be discussed after the codebooks are stable. |
| CMCC | Proposal 7.2: Prefer Alt1. The principle of subset selection of Alt2 and the performance loss of Alt 2 are not clear to us. Proposal 7.3: Not support. Since only part of layer splitting are supported, the overhead of second TPMI can be further reduced. Proposal 7.4: More discussion is needed. |
| OPPO | Proposal 7.1: Support in principle. However, separate indication of RI and TPMI is also fine to us, which would not increase the overhead. Proposal 7.2: We think more evaluation is needed. Alt.1 is preferred. Proposal 7.4: Not support. We don’t think this can be achieved. Based on current design, partial coherent codebook is much larger than full coherent codebook. |
| IDC | P7.1: Support to make an agreement, so that the editor can capture this to be fit into the spec. P7.2: Support. P7.3: Support. P7.4: More discussion is needed. |
| Ericsson | P7.1: This overlaps with P4.1 and P4.2, and seems to preclude nested codebooks as written. Can we clarify room can be made for nesting, if supported,, e.g. with: Proposal 7.1: For PUSCH transmission by a full-coherent 8TX UE, support the following tables for indication of full-coherent precoders, where - If a codebook containing only full-coherent precoders is supported, the full set of precoders is used. If a codebook does not contain only full-coherent precoders, a subset of the full-coherent precoders is in a codebook. P7.2: Also overlaps with P4.1 and P4.2, and seems to preclude nested codebooks as written. Can we clarify that this is a non-coherent-only codebook? Also, we may want to use fewer than 32 precoders in an FC+NC codebook, so which non-coherent precoders are in a codebook with full-coherent precoders should be FFS. Then we think that there is some benefit for rank 5 to allow some panel/antenna selection, and prefer to have 4, rather than 2 precoders for rank 5, while rank 2 can be OK with 6 instead of 8 precoders. We expect this can be resolved with some short discussion in this meeting. We propose the following revision to P7.2: Proposal 7.2: For non-coherent-only uplink precoding by an 8TX UE, support Alt2., - Alt1. – All 255 combinations from 8 non-coherent rank1 precoders are supported - Alt2. – Only a subset of Alt1. is supported, striving for a substantial reduction in the number of precoders - Up to 5 bits is used for TPMI indication, where which alternative of each of [8,6] and {2,4] is supported is to be determined in RAN1#113 Rank; Number of precoders. 1; 8. 2; [8,6]. 3; 6. 4; 4. 5; [2,4]. 6; 2. 7; 1. 8; 1. Total; 32. - FFS identification of precoders for each rank ≥ 2 - FFS: which, if any, non-coherent precoders are in a codebook containing full-coherent precoders P7.3: Not support. The details of TPMI indication should be clearly spelled out, and so it is unclear what Option 3 actually is. Moreover, two TPMIs will double the overhead compared to 4 Tx, while a single TPMI can have less. Also, it’s unclear how e.g. rank 1 works : if a single 4 Tx precoder is applied, are only half the Tx chains active? And to repeat our and others prior comments, the codebook design should be done prior to deciding how to carry the TPMIs in DCI. P7.4 OK in principle, but prefer it be a working assumption (would like to double check this small a design), and would like a clarification that this is where a nested codebook is configured: Proposal 7.4: Working Assumption: For PUSCH transmission by an 8TX UE configured to transmit using both partial and non-coherent precoders, the sum of the number of precoders for partial and non-coherent precoders is at most the same as the number of supported full-coherent precoders, e.g., 128. |
| Apple | P7.1: our first preference is to have separate RI and TPMI indication. But we could be fine with the current proposal. P7.2: our preference is Alt 1. It is not clear why 32 precoders for Alt 2. The performance impact can be significant. This should be discussed more. P7.3: support P7.4: not support. The motivation and the performance impact is not clear. |
| Samsung | P7.1: it should be up to the editor P7.2, 7.4: OK P7.3: OK in principle. One clarification: how many TPMI field do we have, one field indicating up to 2 4Tx TPMIs or two separate fields? |
| Nokia, NSB | P7.1: okay P7.2: prefer Alt. 1; okay with Alt 2. P7.3: support P7.4: The 128 number comes from 8Tx fully coherent design? This needs more discussion but however this number is important for the CB design. |

# SRS Configuration

Void

# Feature-lead Proposals for Approval

# Round 1

Agreement

For codebook-based 8TX PUSCH transmission, Alt3 is supported, where

- A fully-coherent UE (Ng =1) can only be configured with precoders considered for Ng =1

- A partially-coherent UE, with Ng =2, can only use precoders considered for Ng =2

- A partially-coherent UE, with Ng =4, can only use precoders considered for Ng =4

- A non-coherent UE, with Ng =8, can only use precoders considered for Ng = 8

Agreement

To support CBG-based transmission for dual CW PUSCH operation, the range N=2, 4, 6, or 8 is confirmed for the CBGTI bit field.

Agreement

For non-coherent uplink precoding by an 8TX UE, support Alt1.,

- Alt1. – All 255 combinations from 8 non-coherent rank1 precoders are supported

# Round 2

Agreement

For partially coherent uplink precoding by an 8TX UE, Ng=4,

- The following rank and layer splitting cases are supported,

| Rank | All layers in one Antenna Group | Layers split across 4 Antenna Groups (All possible permutations) |
| --- | --- | --- |
| 3 |  | Transmission by 2 of the 4 antenna groups: (2,1,0,0), (2,0,1,0), (2,0,0,1), (0,2,1,0), (0,2,0,1), (0,0,2,1), Transmission by 3 of the 4 antenna groups: (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1) |
| 5 |  | Transmission by 3 of the antenna groups: (2,0,2,1), (0,2,2,1), Transmission by 4 of the 4 antenna groups: (1,1,2,1) |
| 6 |  | Transmission by 3 of the 4 antenna groups: (2,2,2,0), (2,0,2,2) Transmission by 4 of the 4 antenna groups: (2,1,2,1) |
| 7 |  | Transmission by 4 of the 4 antenna groups: (2,1,2,2) |

# Round 3

Proposal 3.2c: Confirming the Working Assumption,

For partially coherent uplink precoding by an 8TX UE, Ng=2,

- At least the following combinations of layer splitting are supported

  - FFS: For rank>4, all the layers for each CW is mapped to only one antenna group

| Rank | All layers in one Antenna Group | Layers split across 2 Antenna Groups |
| --- | --- | --- |
| 2 | (2,0), (0,2) |  |
| 2 |  | (1,1) |
| 3 | (3,0), (0,3) |  |
| 3 |  | (1,2), (2,1) |
| 4 | (4,0), (0,4) |  |
| 4 |  | (2,2) |
| 5 |  | [(2,3), (3,2)] |
| 6 |  | (3,3) |
| 7 |  | [(3,4), (4,3)] |

Note: At least one permutation will be selected in RAN1#114.

Proposal 4.2: For an 8TX UE, Option 1 is supported,

- Option 1 – Subject to its capability, an 8TX UE may report more than one Ng value, based on which, gNB may select and configure UE with one of the supported Ng values.

- Option 2 – An 8TX UE may report only a single Ng value as part of its UE capability. No RRC configuration is needed.

Proposal 5.2:

Version A:

Support CG for dual CW PUSCH transmission by an 8TX UE,

- For Type-1 CG: A second mcsAndTBS parameters is configured in rrc-ConfiguredUplinkGrant

- For Type-2 CG: A second MCS field is added in DCI formats (e.g., 0_1, 0_2)  activating a Type-2 CG-PUSCH

Version B:

Conclusion: In Rel-18, there is no consensus to support CG transmission with dual CW PUSCH by an 8TX UE.

Proposal 5.3: For dual CW PUSCH transmission by an 8TX UE, PHY layer priority indicator (if configured) is applied on both codewords.

Proposal 5.4: For dual CW PUSCH transmission by an 8TX UE, UL-SCH indicator is applied on both codewords.

Proposal 6.1a: For full power PUSCH transmission by an 8TX UE, confirm the Working Assumption for Mode1 with updates:

- To support full power transmission with Mode1, Rel-16 Mode1 (fullPowerMode1) is re-used.

  - FFS if more than one of the 8TX At most one full coherent precoders is used per rank.

Proposal 6.1b: For full power PUSCH transmission by an 8TX UE, confirm the Working Assumption for Mode2 with updates:

- To support full power transmission with Mode2, Rel-16 Mode2 (fullPowerMode2) is re-used.

  - FFS definition of precoder groups (G0, G1, …)

  - FFS enhancements for SRS configuration

  - At least the following PA architecture (Agreed in RAN1#110) is considered,

    - [-9 -9 -9 -9 -9 -9 -9 0] dB relative to their power class

Proposal 3.1: For partially coherent uplink precoding by an 8TX UE, Ng=2, following cases are supported

For codebook design of an 8TX partial-coherent UE, configured with an 8-port SRS resource

- For when Ng=2, following convention for assumption of port coherency scheme is used

  - Alt 1: two coherent groups of {0,2,4,6} and {1,3,5,7}

  - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7}

  - Alt 3: two coherent groups of {0,1,2,3} and {4,5,6,7}

- For when Ng=4, following convention for assumption of port coherency scheme is used

  - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7}

  - Alt 2: four coherent groups of {0,1}, {2,3}, {4,5}, and {6,7}

  - Alt 3: four coherent groups of {0, 2}, {4, 6}, {1, 3} and {5,7}

Supporting: vivo, NTT, Samsung, NEC, MediaTek, Spreadtrum, OPPO, Lenovo, Xiaomi, LG, Nokia, CMCC, Ericsson, InterDigital, Apple,

Concern: ZTE, CATT, Huawei

Proposal 7.1: For PUSCH transmission by a full-coherent 8TX UE, support the following full-coherent precoders, where

- If a codebook containing only full-coherent precoders is supported, the full set of precoders is used. If a codebook does not contain only full-coherent precoders, a subset of the full-coherent precoders is in a codebook.

- For (N1, N2) = (4, 1)

| Index | Rank | i1,1 | i1,2 | i1,3 | i2 |
| --- | --- | --- | --- | --- | --- |
| 0-15 | 1 | {0, 1, 2, 3} | {0} | - | {0, 1, 2, 3} |
| 16-47 | 2 |  |  | {0, 1, 2, 3} | {0, 1} |
| 48-71 | 3 |  |  | {0, 1, 2} |  |
| 72-95 | 4 |  |  |  |  |
| 96-103 | 5 |  |  | - |  |
| 104-111 | 6 |  |  |  |  |
| 112-115 | 7 | {0, 1} |  |  |  |
| 116-119 | 8 | {0, 1} |  |  |  |

- For (N1, N2) = (2, 2)

| Index | Rank | i1,1 | i1,2 | i1,3 | i2 |
| --- | --- | --- | --- | --- | --- |
| 0-15 | 1 | {0, 1} | {0, 1} | - | {0, 1, 2, 3} |
| 16-47 | 2 |  |  | {0, 1, 2, 3} | {0, 1} |
| 48-71 | 3 |  |  | {0, 1, 2} |  |
| 72-95 | 4 |  |  |  |  |
| 96-103 | 5 |  |  | - |  |
| 104-111 | 6 |  |  |  |  |
| 112-119 | 7 |  |  |  |  |
| 120-127 | 8 |  |  |  |  |

Note: Details related to capturing of the agreement, including formulation, ordering, indexing, etc., is up to Editor’s discretion.

Note: Above is not relevant to how precoders are indicated.

# Additional Info for (3,2)

Proposal 3.2a: For partially coherent uplink precoding by an 8TX UE, Ng=2, confirm the Working Assumptions with updates:

- Following combinations of layer splitting are supported

| Rank | All layers in one Antenna Group | Layers split across 2 Antenna Groups |
| --- | --- | --- |
| 2 | (2,0), (0,2) |  |
| 2 |  | (1,1) |
| 3 | (3,0), (0,3) |  |
| 3 |  | (1,2), (2,1) |
| 4 | (4,0), (0,4) |  |
| 4 |  | (2,2) |
| 5 |  | (2,3), (3,2) |
| 6 |  | (3,3) |
| 7 |  | (3,4), (4,3) |

Support: CATT, ZTE, Xiaomi, Ericsson, Qualcomm, MediaTek, OPPO, Lenovo, InterDigital, CMCC, vivo, Nokia, NTT DOCOMO

Proposal 3.2b: For partially coherent uplink precoding by an 8TX UE, Ng=2,

- At least the following combinations of layer splitting are supported

  - For rank>4, all the layers for each CW is mapped to only one antenna group

| Rank | All layers in one Antenna Group | Layers split across 2 Antenna Groups |
| --- | --- | --- |
| 2 | (2,0), (0,2) |  |
| 2 |  | (1,1) |
| 3 | (3,0), (0,3) |  |
| 3 |  | (1,2), (2,1) |
| 4 | (4,0), (0,4) |  |
| 4 |  | (2,2) |
| 5 |  | (2,3), (3,2) |
| 6 |  | (3,3) |
| 7 |  | (3,4), (4,3) |

# List of Companies’ Proposals

| ZTE | Proposal 1: Regarding full-coherent codebook design for UL 8-Tx based on Rel-15 DL type I codebook, - Definition and range of (i1,1, i1,2, i1,3, i2) in DL Type-I 8-Tx codebook should be reused. Proposal 2: For partially coherent uplink precoding by an 8TX UE, Ng=2, confirm the Working Assumptions with the following modification: Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (1,2), (2,1). 4; (4,0), (0,4). 4; (2,2). 5; (2,3), (3,2). 6; (3,3). 7; (3,4), (4,3). Proposal 3: For partially coherent uplink precoding by an 8TX UE, Ng=4, support the following down selections: Rank; All layers in one Antenna Group; Layers split across 4 Antenna Groups (All possible permutations). 3; Transmission by 2 of the 4 antenna groups: (2,1,0,0), (2,0,1,0), (2,0,0,1), (0,2,1,0), (0,2,0,1), (0,0,2,1), (1,2,0,0), (1,0,2,0), (1,0,0,2), (0,1,2,0), (0,1,0,2), (0,0,1,2) Transmission by 3 of the 4 antenna groups: (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1). 4; Transmission by 3 of the 4 antenna groups: (2,1,1,0), (0,2,1,1), (1,0,2,1), (1,1,0,2) (1,2,1,0), (1,1,2,0), (0,1,2,1), (0,1,1,2), (1,0,1,2), (2,0,1,1), (2,1,0,1), (1,2,0,1). 5; Transmission by 3 of the antenna groups: (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1), (2,1,2,0), (1,2,2,0), (2,1,0,2), (1,2,0,2), (2,0,1,2), (1,0,2,2), (0,2,1,2), (0,1,2,2) Transmission by 4 of the 4 antenna groups: (1,1,2,1), (1,1,1,2), (2,1,1,1), (1,2,1,1). 6; Transmission by 3 of the 4 antenna groups: (2,2,2,0), (2,2,0,2), (2,0,2,2), (0,2,2,2) Transmission by 4 of the 4 antenna groups: (2,1,2,1), (1,2,1,2), (1,2,2,1), (2,1,1,2), (2,2,1,1), (1,1,2,2. 7; Transmission by 4 of the 4 antenna groups: (2,1,2,2), {(2,2,2,1), (1,2,2,2), (2,2,1,2). Proposal 4: Regarding port index order, follow the same port index grouping scheme as AI 9.1.3.2 for different comb offsets for 8 port SRS - For Ng=2: two coherent groups of {0,2,4,6} and {2,3,5,7} (Alt 1) - For Ng=4: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7} (Alt 1) Rank; Ports selection. 1; 8: (0), (4), (2), (6), (1), (5), (3), (7). 2; 8: (0,4), (2,6), (1,5), (3,7), (0,2), (1,3), (4,6), (5,7). 3; 6: (0,4,2), (1,5,3) (2,6,0), (3,7,1), (0,1,2), (4,5,6). 4; 4: (0,4,2,6), (1,5,3,7), (0,1,2,3), (4,5,6,7). 5; 2: (0,4,2,6,1), (1,5,3,7,0). 6; 2: (0,4,2,6,1,5), (1,5,3,7,0,4). 7; 1: (0,4,2,6,1,5,3). 8; 1: (0,4,2,6,1,5,3,7). Total; 32. Proposal 5: Regarding non-coherent codebook design, only a subset of combinations from 8 non-coherent rank1 precoders are supported (Alt2), and the following port selection scheme can be studied and evaluated as a starting point. Proposal 6: Regarding codebook indication for 8-Tx, individual TPMI indication for each group based on indication of number of port groups should be supported: - For 1-group: one TPMI indicates one 8Tx precoder which needs new defined UL 8Tx codebook for rank 1-8 - For 2-group: 2 TPMI (rank and UL precoding info), and each for a respective port group - For 4-group: 4 TPMI (rank and UL precoding info), and each for a respective port group - For 8-group or non-coh: 8 or less bits indicate presence of respective ports Proposal 7: For codebook -based 8TX PUSCH transmission, Alt1 in the agreement in RAN1#112b-e as follows should be supported. - Alt1 - A fully-coherent UE (Ng =1) can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =1, 2, 4, 8 - FFS which combinations of Ng value(s), to be considered - A partially-coherent UE , with Ng =2 can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =2, 4, 8 - FFS which combinations of Ng value(s), to be considered - A partially-coherent UE , with Ng =4, can be configured with precoders considered for at least one or more Ng cases, i.e., Ng= 4, 8 - FFS which combinations of Ng value(s), if any, to be considered - A non-coherent UE , Ng =8, can only be configured with precoders considered for Ng = 8 Proposal 8: Regarding overhead reduction for codebook indication for 8-Tx: - A set of available Ng(s) can be configured by RRC, and then one from the set can be dynamically indicated for a scheduled PUSCH transmission. - For instance, for a UE supporting full-coherent 8-Tx ports, a list of candidate values of Ng=1, and Ng=2 can be configured by RRC, and DCI only needs to indicate one of them as UL codebook selection. Proposal 9: To support dual CW PUSCH operation by an 8TX UE, if CBG-based transmission is configured, the range N=[2, 4, 6 or 8] is supported for the CBGTI bit field. Proposal 10: To support 2 CWs for CG-PUSCH transmission with rank > 4, - For Type-1 CG-PUSCH, a second mcsAndTBS parameter is configured for the second CW. - For Type-2 CG-PUSCH, a second MCS field is indicated in DCI format activating the CG-PUSCH for the second TB. - FFS: Whether RV sequence parameter (repK-RV) is commonly or individually configured to two codewords. Proposal 11: Regarding full power mode 1 for Rel-18 UL 8-Tx, RAN1 strives to extend limited appropriate full coherent precoders for non-coherent UE,and partial-coherent UE after codebook design is completed. Proposal 12: Regarding full power mode 2 for Rel-18 UL 8-Tx, full power PA architecture should be discussed and determined as early as possible, and total number of full power PA architectures should be similar to that for Rel-16 full power mode 2. |
| --- | --- |
| InterDigital, Inc. | Proposal 1: Confirm the working assumption to support dual codeword when more than 4 layers, where it is applicable associated with considered UE types, coherency types, etc., based on UE capability. Proposal 2: Confirm that the range of values for N for the CBGTI bit field can be configured to be N = 2, 4, 6, or 8, in case when CBG-based transmission is configured. Proposal 3: Consider UE to report its capabilities on the number of antenna groups, supported type of antenna/panel structure or virtualization capability across UE antenna ports, etc. Proposal 4: If more than one Ng value can be reported as UE capability, decide on whether the UE can be configured with only one Ng value that generates one codebook or the UE can be configured to use multiple codebooks together, following the nested-like codebook construction based on multiple Ng values. Proposal 5: For codebook-based 8TX PUSCH transmission, support Alt 2 having non-nested codebook construction in that a fully-coherent UE (Ng=1) can only be configured with precoders considered for only one of Ng cases, i.e., Ng=1,2,4,8, and similarly for partially/non-coherent UEs, which does not allow multiple Ng values together for the precoder generation. Proposal 6: To reduce signaling overhead associated to SRI/TPMI indication for a 8TX UE, RAN1 studies partial update of TPMI/SRI information for 8TX UE. Proposal 7: Confirm both working assumptions on supporting full power transmission with Mode1 and Mode2, where for Mode2 at least the PA powers per TX chain of [-3 -3 -3 -3 -3 -3 -3 -3] dB relative to their power class can be considered for further discussion. |
| vivo | Proposal 1: For codebook design of an 8TX partial-coherent UE, configured with an 8-port SRS resource - For when Ng=2, the following convention for assumption of port coherency scheme is used - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7} - For when Ng=4, the following convention for assumption of port coherency scheme is used - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7} Proposal 2: For codebook -based 8TX PUSCH transmission, support alt2. - Alt2 - A fully-coherent UE (Ng =1) can only be configured with precoders considered for one of Ng cases, i.e., Ng =1, 2, 4, 8 - FFS which Ng value(s), to be considered - A partially-coherent UE , with Ng =2, can only be configured with precoders considered for one of Ng cases, i.e., Ng =2, 4, 8 - FFS which Ng value(s), to be considered - A partially-coherent UE , with Ng =4, can only be configured with precoders considered for one of Ng cases, i.e., Ng =4, 8 - FFS which Ng value(s), to be considered - A non-coherent UE , with Ng =8, can only be configured with precoders considered for Ng = 8 - FFS whether/how the configuration can be done via RRC or MAC-CE. Proposal 3: The TPMI for full-coherent precoders are indexed starting from rank 1 precoders then followed by precoders from higher rank values. Within a specific rank value, the precoders can be first indexed according to the parameters i1,1 then i1,2 then i1,3 then i2 or the sequence of precoders can be indexed according to the parameters i1,1 then i1,2 then i2 then i1,3. Proposal 4: for rank>4, if dual CW is supported, support one CW is transmitted from one antenna group Proposal 5: Confirm the working assumption, including the table, with following revision, For partially coherent uplink precoding by an 8TX UE, Ng=2, - At least the following combinations of layer splitting are supported - FFS: For rank>4, all the layers for each CW is mapped to only one antenna group Proposal 6: At least support precoders listed in section 2.1.4.1 for Ng=2 partial coherent 8Tx codebook. Proposal 7: For partially coherent uplink precoding by an 8TX UE codebook, Ng=4, - In addition to the previously agreed cases, support following cases Rank; All layers in one Antenna Group; Layers split across 4 Antenna Groups (All possible permutations). 3; Transmission by 3 of the 4 antenna groups: (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1). 5; Transmission by 4 of the 4 antenna groups: (1,1,2,1), (1,1,1,2), (2,1,1,1), (1,2,1,1). 6; Transmission by 3 of the 4 antenna groups: (2,2,2,0), (2,2,0,2), (2,0,2,2), (0,2,2,2). 7; Transmission by 4 of the 4 antenna groups: (2,1,2,2), {(2,2,2,1), (1,2,2,2), (2,2,1,2). - Further down select among the layer split combinations for rank 1~8, if necessary Proposal 8: At least support precoders listed in section 2.1.4.2 for Ng=4 partial coherent 8Tx codebook. Proposal 9: At least support precoders listed in section 2.1.5 for non coherent 8Tx codebook. Proposal 10: Support indicating 8Tx non-coherent precoder by 2 TPMI fields in DCI, further study how to select one of the 2 TPMI fields, e.g. for rank=1 transmission Proposal 11: for 8Tx UE supporting dual CW, when the second CW is enabled, the redundant bit in TPMI field is used as NDI bit. Proposal 12: Consider the following full power enhancement for CPE/FWA 8 Tx operation. - Depending on UE capability, UL full-power mode1 can be supported by introducing non-antenna selection matrices - For Ng=2 partial-coherent case, only full power transmission precoder for rank=1 is needed - For Ng=4 partial-coherent case, only full power transmission precoders for rank=1, 2 and 3 are needed - Depending on UE capability, further discuss UL full-power mode2 for partial and non-coherent UEs - For partial-coherent codebook, take Ng values {2, 4} into account for full-power precoders grouping Proposal 13: Further discuss PTRS-DMRS association indication when rank>4, if supported |
| Spreadtrum Comm. | Proposal 1: For a full-coherent uplink precoding by an 8TX UE, only support NR Rel-15 single panel DL Type I codebook. Proposal 2: For partial-coherent uplink precoding by an 8TX UE with Ng=2, support the combinations of layer splitting in working assumption as follows. Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (1,2), (2,1). 4; (4,0), (0,4). 4; (2,2). 5; (2,3), (3,2). 6; (3,3). 7; (3,4), (4,3). Proposal 3: For partial-coherent uplink precoding by an 8TX UE with Ng=4, support all combinations of layer splitting as follows. Rank; All layers in one Antenna Group; Layers split across 4 Antenna Groups (All possible permutations). 3; Transmission by 2 of the 4 antenna groups: (2,1,0,0), (2,0,1,0), (2,0,0,1), (0,2,1,0), (0,2,0,1), (0,0,2,1), (1,2,0,0), (1,0,2,0), (1,0,0,2), (0,1,2,0), (0,1,0,2), (0,0,1,2) Transmission by 3 of the 4 antenna groups: (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1). 4; Transmission by 3 of the 4 antenna groups: (2,1,1,0), (0,2,1,1), (1,0,2,1), (1,1,0,2) (1,2,1,0), (1,1,2,0), (0,1,2,1), (0,1,1,2), (1,0,1,2), (2,0,1,1), (2,1,0,1), (1,2,0,1). 5; Transmission by 3 of the antenna groups: (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1), (2,1,2,0), (1,2,2,0), (2,1,0,2), (1,2,0,2), (2,0,1,2), (1,0,2,2), (0,2,1,2), (0,1,2,2) Transmission by 4 of the 4 antenna groups: (1,1,2,1), (1,1,1,2), (2,1,1,1), (1,2,1,1). 6; Transmission by 3 of the 4 antenna groups: (2,2,2,0), (2,2,0,2), (2,0,2,2), (0,2,2,2) Transmission by 4 of the 4 antenna groups: (2,1,2,1), (1,2,1,2), (1,2,2,1), (2,1,1,2), (2,2,1,1), (1,1,2,2. 7; Transmission by 4 of the 4 antenna groups: (2,1,2,2), {(2,2,2,1), (1,2,2,2), (2,2,1,2). Proposal 4: For non-coherent uplink precoding with rank≤8 by an 8TX UE, support Alt1, i.e. All 255 combinations from 8 non-coherent rank1 precoders are supported. Proposal 5: Confirm the range of values for the CBGTI field for dual CW PUSCH transmission, i.e. support N=2, 4, 6 or 8. Proposal 6: Support dual CW CG PUSCH transmission for an 8TX UE - For Type 1 CG PUSCH: a second mcsAndTBS parameters is configured in rrc-ConfiguredUplinkGrant. - For Type 2 CG PUSCH: a second MCS field is added in DCI format activating a Type 2 CG PUSCH. Proposal 7: For partially coherent 8TX precoding with Ng =2, support Option3, i.e. up to two 4TX TPMIs are indicated. - When one antenna group is used, a reserved codepoint is used in the TPMI field corresponding to the other antenna group. Proposal 8: For partially coherent 8TX precoding with Ng =4, support up to four 2TX TPMIs. Proposal 9: For codebook configuration of codebook-based 8TX PUSCH transmission, support Alt1, i.e. - A full-coherent UE (Ng =1) can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =1, 2, 4, 8 - A partial-coherent UE , with Ng =2 can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =2, 4, 8 - A partial-coherent UE , with Ng =4, can be configured with precoders considered for at least one or more Ng cases, i.e., Ng= 4, 8 - A non-coherent UE , Ng =8, can only be configured with precoders considered for Ng = 8 Proposal 10: Support full power mode1 for an 8TX UE and introduce 8TX full power precoder(s). |
| Huawei, HiSilicon | Proposal 1: For SRS configuration required for non-codebook-based UL transmission by an 8TX UE, configuration of up to two, or four SRS resource sets is not supported. Proposal 2: Consider to use the reserved NDI field of the disabled transport block for rank<=4 to reduce SRI overhead. Proposal 3：For fully coherent precoder by an 8TX UE, support to enhance DL Type I SP codebook to mitigate the impact of phase alignment error. Proposal 4：For PUSCH transmission by an 8TX UE, support to separately indicate rank and TPMI for fully coherent precoder. Proposal 5：For the indication of fully coherent precoder by an 8TX UE, the #layer in {5, 6, 7, 8} is indicated if two TBs are enabled, and #layers in {1, 2, 3, 4} otherwise. Proposal 6: Consider to use the reserved NDI field of the disabled transport block for rank<=4 to reduce the indication overhead of UL 8TX fully coherent precoder. Proposal 7：For partially coherent precoder with Ng=2 by an 8TX UE, support all possible layer splitting schemes for each rank. Proposal 8：For PUSCH transmission by an 8TX UE, support to indicate two 4TX fully coherent precoder for partially coherent precoder with Ng=2. Proposal 9: Consider to use the reserved NDI field of the disabled transport block for rank<=4 to reduce the indication overhead of UL 8TX partially coherent precoder with Ng=2. Proposal 10: For partially coherent precoder with Ng=4 by an 8TX UE, for rank 1-3 and 7-8, supporting all layer splitting cases; for rank 4-6, supporting to prune layer splitting cases as - Rank 4: remove all layer splitting cases of transmission by 3 of the 4 antenna groups - Rank 5: only support (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1) of transmission by 3 of the 4 antenna groups and (1,1,2,1), (1,1,1,2) of transmission by 4 of the 4 antenna groups - Rank 6: only support all layer splitting cases of transmission by 3 of the 4 antenna groups and (2,1,2,1), (1,2,1,2), (1,2,2,1), (2,1,1,2) of transmission by 4 of the 4 antenna groups Proposal 11：For non-coherent precoder by an 8TX UE, support to design precoders based on Rel-15 UL 4TX non-coherent codebook. Proposal 12: The beamformed CSI-RS should be considered to indicate UL precoders to UE. |
| CATT | Proposal 1: For non-coherent uplink precoding with rank ≤ 8 by an 8TX UE, Alt 1 is supported: - Alt1 – All 255 combinations from 8 non-coherent rank1 precoders are supported. Proposal 2: For partially coherent uplink precoding by an 8TX UE codebook, Ng=2, the following rank and layer splitting cases are supported: Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (1,1). 3; (1,2). 4; (2,2). 5; (2,3). 6; (3,3). 7; (3,4). Proposal 3: For partially coherent uplink precoding by an 8TX UE codebook, Ng=2, - For rank 3, precoding design is based on Rel-15 UL 4TX codebook with a rank-1 full-coherent precoder and a rank-2 partial-coherent precoder used; - For rank 5, precoding design is based on Rel-15 UL 4TX codebook with a rank-2 partial-coherent precoder and a rank-3 partial-coherent precoder used; - For rank 7, precoding design is based on Rel-15 UL 4TX codebook with a rank-3 partial-coherent precoder and a rank-4 non-coherent precoder used. Proposal 4: For partially coherent uplink precoding by an 8TX UE codebook, Ng=4, - At least the following rank and layer splitting cases are supported: Rank; All layers in one Antenna Group; Layers split across 4 Antenna Groups (All possible permutations). 3; Transmission by 3 of the 4 antenna groups: (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1). 5; Transmission by 4 of the 4 antenna groups: (1,1,2,1), (1,1,1,2), (2,1,1,1), (1,2,1,1). 6; Transmission by 4 of the 4 antenna groups: (2,1,2,1), (1,2,1,2), (1,2,2,1), (2,1,1,2), (2,2,1,1), (1,1,2,2. 7; Transmission by 4 of the 4 antenna groups: (2,1,2,2), {(2,2,2,1), (1,2,2,2), (2,2,1,2). Proposal 5: For codebook -based 8TX PUSCH transmission, Alt 1 is supported: - Alt1 - A fully-coherent UE (Ng =1) can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =1, 2, 4, 8 - FFS which combinations of Ng value(s), to be considered - A partially-coherent UE , with Ng =2 can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =2, 4, 8 - FFS which combinations of Ng value(s), to be considered - A partially-coherent UE , with Ng =4, can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =4, 8 - FFS which combinations of Ng value(s), if any, to be considered - A non-coherent UE, Ng =8, can only be configured with precoders considered for Ng = 8. Proposal 6: For codebook-based 8TX PUSCH transmission, - The following Ng combinations are supported for fully-coherent UEs (Ng =1): - One Ng cases from Ng = 1,2, 4,8, i.e. (1), (2), (4), (8); - Combination of Ng = 8 and one or two Ng cases from Ng = 2,4, i.e. (2,8), (4,8), (2,4, 8); - Combination of Ng = 1, Ng =8 and one or two Ng cases from Ng = 2,4, i.e. (1,2,8), (1,4,8), (1,2,4, 8). - The following Ng combinations are supported for partially-coherent UEs with Ng =2: - One Ng cases from Ng = 2, 4,8, i.e. (2), (4), (8); - Combination of Ng = 8 and one or two Ng cases from Ng = 2,4, i.e. (2,8), (4,8), (2,4, 8); - The following Ng combinations are supported for partially-coherent UEs with Ng =4: - One Ng cases from Ng = 4,8, i.e. (4), (8); - Combination of Ng = 8 and Ng = 4, i.e. (4, 8). Proposal 7: On TPMI/TRI indication for precoders considered for Ng = 1, the legacy TPMI/TRI indication scheme as in Rel-15 (i.e., a single 8TX TPMI is jointly indicated with TRI) is adopted, i.e., a single 8TX TPMI is jointly indicated with TRI. - When maxRank = 1 is configured, one of the following TPMI/TRI indication scheme is adopted: - Option 1: A single 8Tx TPMI is indicated; - Option 2: - One bit is used to indicate the layer splitting combination; - 4 bits are used to indicate a UL 4TX rank-1 TPMI; - When maxRank = 2 is configured, the following TPMI/TRI indication scheme is adopted: - A single 8Tx TPMI is indicated; - When maxRank > 1 is configured, the following scheme is considered: - Two UL 4TX TPMI/TRIs are indicated, and the two UL 4TX TPMI/TRIs are indicated by two groups of bits separately; - The first TPMI/TRI is applied on one of antenna group, and the second TPMI/TRI is applied on the other antenna group - For each bit group, there is one value to indicate TRI= 0(i.e., the antenna ports corresponding to the group are unused). Proposal 8: On TPMI/TRI indication for precoders considered for Ng = 2, - When maxRank = 1 is configured, one of the following TPMI/TRI indication schemes is adopted: - Option 1: A single 8Tx TPMI is indicated; - Option 2: - One bit is used to indicate the layer splitting combination; - 4 bits are used to indicate a UL 4TX rank-1 TPMI; - When maxRank = 2 is configured, the following TPMI/TRI indication scheme is adopted: - A single 8Tx TPMI is indicated; - When maxRank > 1 is configured, the following scheme is considered: - Two UL 4TX TPMI/TRIs are indicated, and the two UL 4TX TPMI/TRIs are indicated by two groups of bits separately; - The first TPMI/TRI is applied on one of antenna group, and the second TPMI/TRI is applied on the other antenna group - For each bit group, there is one value to indicate TRI= 0 (i.e., the antenna ports corresponding to the group are unused). Proposal 9: On TPMI/TRI indication for precoders considered for Ng = 4, at least one of the following schemes is supported: - Option 1: The legacy TPMI/TRI indication scheme as in Rel-15, i.e., a single 8TX TPMI is jointly indicated with TRI; - Option 2: - Up to four 2TX TPMI/TRIs are indicated, and each 2TX TPMI/TRI is indicated by a group of dedicated bits; - For each bit group, there is one value to indicate TRI= 0 (i.e., the antenna ports corresponding to the group are unused); - Option 3: - One or more bits are used to indicate the layer splitting combination; - Up to four 2TX TPMI are indicated - Each 2TX TPMI is indicated by a group of dedicated bits; - The 2TX TPMIs are one-to-one mapped to the antenna groups, and each 2TX TPMI corresponding to a TRI indicated by the layer splitting combination. - FFS the applied option for each maxRank. Proposal 10: On TPMI/TRI indication for a UE configured with a codebook subset includes precoders for multiple Ng cases, one or two bits for Ng indication is considered in the TPMI/TRI indication field. Proposal 11: Confirm the following working assumption: - For uplink transmission with rank>4, support dual CW transmission. Proposal 12: For PUSCH transmission with rank>4, UE always applies 2 CWs transmission. Proposal 13: For codebook based 8TX PUSCH transmission, maxRank and maxRankDCI-0-2 are used to indicate whether two CWs transmission is enabled for DCI format 0_1 and 0_2 respectively. Proposal 14: For non-codebook based 8TX PUSCH transmission, maxMIMO-Layers, if is configured, is used to indicate whether 2 CWs transmission is enabled; if maxMIMO-Layers is not configured, if the maximum number of layers for PUSCH supported by the UE is larger than 4, 2 CWs transmission is enabled, and if the maximum number of layers for PUSCH supported by the UE is no more than 4, 2 CWs transmission is disabled. Proposal 15: To support dual CW PUSCH transmission for rank>4 by an 8TX UE, two new fields are used to indicate the second set of NDI (1 bit) and RV (2 bits) fields respectively. Proposal 16: For an 8TX UE with UE Capability 1, scaling factor s=1 for all precoders is supported. Proposal 17: For full power PUSCH transmission by an 8TX UE, delay the discussion on whether and how to support full power transmission Mode 1 and Mode 2 until the codebook design for UL 8TX transmission is finished. |
| Google | Proposal 1: Support the UE to report the UE capability indicating whether the antenna port groups sharing the same QCL properties or not. Proposal 2: Support to divide the partial-coherent and non-coherent precoders into several groups - The precoders with non-zero-power coefficients from the same antenna port groups are within the same group - Support the gNB to activate/deactivate the precoder groups, and the indicated precoder is based on the activated precoder groups. Proposal 3: Support the 8Tx UE to report the UE capability with regard to spatial domain fallback operation, including: - Supported UL full power mode when it is configured as 2Tx, 4Tx and 8Tx based UL transmission - Supported codebook coherency subset when it is configured as 2Tx, 4Tx and 8Tx based UL transmission Proposal 4: For precoder indication, support option 4 (a single 8Tx TPMI is indicated). Proposal 5: For codebook coherency subset configuration, support Alt1. - A fully-coherent UE (Ng =1) can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =1, 2, 4, 8 - FFS which combinations of Ng value(s), to be considered - A partially-coherent UE, with Ng =2 can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =2, 4, 8 - FFS which combinations of Ng value(s), to be considered - A partially-coherent UE, with Ng =4, can be configured with precoders considered for at least one or more Ng cases, i.e., Ng= 4, 8 - FFS which combinations of Ng value(s), if any, to be considered - A non-coherent UE, Ng =8, can only be configured with precoders considered for Ng = 8 |
| xiaomi | Proposal 1: Confirm the Working Assumption to support dual CW for RANK>4 8Tx transmission for DG PUSCH. Proposal 2: Support at least CG Type 2 PUSCH for the dual CW operation, with the following enhancements, - A second MCS field in the activating DCI. Proposal 3: If CG Type 1 PUSCH is supported for dual CW operation, also consider the following enhancement, - Rep_RV can be configured separately to apply to each CW. Proposal 4: Introduce a UE capability to report the supported antenna configuration to gNB. Proposal 5: For Rel-18 UL 8Tx fully-coherent codebook, support the union set of codebook with (N1,N2) = (4,1) and codebook with (N1,N2) = (2,2), and the cardinality of Rel-18 UL 8Tx fully-coherent codebook is 224. Proposal 6: For Rel-18 UL 8Tx codebook, the target size can be designed as 512 and a single 8Tx TPMI with 9 bits signalling overhead can be used to indicate the codewords if nested codebook is supported. Proposal 7: For Rel-18 UL 8Tx fully-coherent codebook, support the following configuration parameters to reduce the signalling overhead. - (N1,N2,O1,O2) = (4,1,1,1) - Rank=1: i1,1={0,1,2,3}, i1,2={0}, i2={0,1,2,3} - Rank=2,3,4: i1,1={0,1,2,3}, i1,2={0}, i1,3={0}, i2={0,1} - Rank=5,6: i1,1={0,2}, i1,2={0}, i2={0,1} - Rank=7,8: i1,1={0}, i1,2={0}, i2={0,1} - (N1,N2,O1,O2) = (2,2,2,2) - Rank=1: i1,1={0,1}, i1,2={0,1}, i2={0,1,2,3} - Rank=2,3,4: i1,1={0,1}, i1,2={0,1}, i1,3={0}, i2={0,1} - Rank=5,6: i1,1={0,1}, i1,2={0}, i2={0,1} - Rank=7,8: i1,1={0}, i1,2={0}, i2={0,1} Proposal 8: For Rel-18 UL 8Tx fully-coherent codebook, to reduce the signalling overhead, support the union set of codebook subset with (N1,N2) = (4,1) and codebook subset with (N1,N2) = (2,2), and the cardinality of Rel-18 UL 8Tx fully-coherent codebook is 92. Proposal 9: For Rel-18 UL 8Tx fully-coherent codebook, fixing the candidates of {i1,1, i1,2, i1,3, i2} can be used to further reduce the signalling overhead. Proposal 10: Support Alt2 for two coherent groups of {0,1,4,5} and {2,3,6,7} for Ng=2. Support Alt1 for four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7} for Ng=4. Proposal 11: Row-interleaving operation should be used for Rel-18 UL 8Tx partially-coherent codewords to satisfy different port coherency schemes. Proposal 12: Support the following rank splitting rules for Ng=2. Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (1,2). 4; (4,0), (0,4). 4; (2,2). 5; (2,3). 6; (3,3). 7; (3,4). Proposal 13: For partially-coherent codebook with Ng=2, using the following Rel-15 UL 4Tx fully-coherent codewords. - Rank=1 with TMPI indices [12 14 15 17 20 22 23 25] (size 8) - Rank=2 with TMPI indices [14 17 18 21] (size 4) - Rank=3 with TMPI indices [3 5] (size 2) - Rank=4 with TMPI index [3] (size 1) Proposal 14: For PUSCH transmission by a partially-coherent 8Tx UE with Ng=2, support the precoding structure A104×rank(A2)04×rank(A1)A28×rank where A1 and A2 are precoding matrices taken from the selected Rel-15 UL 4Tx fully-coherent codebook or empty matrix, the rank=rank(A1) + rank(A2) is one of 1, 2, 3, 4, 5, 6, 7 or 8, and rank(A1) and rank(A2) are determined based on the rank splitting rule for Ng=2. Proposal 15: In addition to the previously agreed cases, support the following rank splitting rule for Ng=4. Rank; All layers in one Antenna Group; Layers split across 4 Antenna Groups (All possible permutations). 3; Transmission by 2 of the 4 antenna groups: (2,1,0,0), (2,0,1,0), (2,0,0,1), (0,2,1,0), (0,2,0,1), (0,0,2,1), (1,2,0,0), (1,0,2,0), (1,0,0,2), (0,1,2,0), (0,1,0,2), (0,0,1,2) Transmission by 3 of the 4 antenna groups: (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1). 4; Transmission by 3 of the 4 antenna groups: (2,1,1,0), (0,2,1,1), (1,0,2,1), (1,1,0,2) (1,2,1,0), (1,1,2,0), (0,1,2,1), (0,1,1,2), (1,0,1,2), (2,0,1,1), (2,1,0,1), (1,2,0,1). 5; Transmission by 3 of the antenna groups: (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1), (2,1,2,0), (1,2,2,0), (2,1,0,2), (1,2,0,2), (2,0,1,2), (1,0,2,2), (0,2,1,2), (0,1,2,2) Transmission by 4 of the 4 antenna groups: (1,1,2,1), (1,1,1,2), (2,1,1,1), (1,2,1,1). 6; Transmission by 3 of the 4 antenna groups: (2,2,2,0), (2,2,0,2), (2,0,2,2), (0,2,2,2) Transmission by 4 of the 4 antenna groups: (2,1,2,1), (1,2,1,2), (1,2,2,1), (2,1,1,2), (2,2,1,1), (1,1,2,2). 7; Transmission by 4 of the 4 antenna groups: (2,1,2,2), (2,2,2,1), (1,2,2,2), (2,2,1,2). Proposal 16: For partially-coherent codebook with Ng=4, using the following Rel-15 UL 2Tx fully-coherent codewords. - Rank=1 with TMPI indices [2 3] (size 2) - Rank=2 with TMPI index [2] (size 1) Proposal 17: For PUSCH transmission by a partially-coherent 8TX UE with Ng=4, support the precoding structure A102×rank(A2)02×rank(A1)A202×rank(A3)02×rank(A4)02×rank(A3)02×rank(A4)02×rank(A1)02×rank(A2)02×rank(A1)02×rank(A2)A302×rank(A4)02×rank(A3)A48×rank where A1, A2, A3 and A4 are precoding matrices taken from the selected Rel-15 2TX UL fully-coherent codebook or empty matrix, the rank= rank(A1) + rank(A2) + rank(A3) + rank(A4) is one of 1, 2, 3, 4, 5, 6, 7 or 8, and rank(A1), rank(A2), rank(A3), and rank(A4) are determined based on the rank splitting rule. Proposal 18: Support the following Rel-18 UL 8Tx non-coherent codebook for rank=2-8 with scaling factor of 122. Rank=2. Rank=3; Rank=4. Rank=5; Rank=6. Rank=7; Rank=8. Proposal 19: For codebook-based 8TX PUSCH transmission, support Alt 1 and support that a UE with Ng can be configured with precoders considered for all Ng cases, i.e., - A fully-coherent UE, with Ng =1, can be configured with precoders considered for Ng =1, 2, 4, and 8 - A partially-coherent UE, with Ng =2 can be configured with precoders considered for Ng =2, 4, and 8 - A partially-coherent UE, with Ng =4, can be configured with precoders considered for Ng= 4, and 8 - A non-coherent UE, with Ng =8, can only be configured with precoders considered for Ng = 8 Proposal 20: For Rel-18 UL 8Tx codebook, support joint indication of TRI and TPMI as legacy, and the bitwidth of the filed precoding information and number of layers for 8 antenna ports can be designed as 9 bits. Proposal 21: For Rel-18 UL 8Tx partially-coherent codebook with Ng=2, if nested codebook is not supported, support using two 4Tx TPMIs to indicate the precoding matrix and each 4Tx TPMI includes an empty matrix. Proposal 22: For Rel-18 UL 8Tx partially-coherent codebook with Ng=4, if nested codebook is not supported, support using four 2Tx TPMIs to indicate the precoding matrix and each 2Tx TPMI includes an empty matrix. |
| Lenovo | Proposal 1: Use antenna grouping to represent different UL Tx coherence assumptions, with the following conditions - Antennas within an antenna group are coherent. - Antennas within an antenna group are uniformly spaced. - Antenna configurations of different antenna groups are identical. - Coherence assumptions of two antennas across two antenna groups are the same. Proposal 2: A number of antenna coherence groups Nc is used to characterize the coherence assumption across Ng antenna groups, where Nc is the number of antenna groups in which all antennas of the antenna groups are coherent, and Nc≤Ng. Proposal 3: For coherence-based antenna grouping under partial-coherent UEs: - Nc=2: Support Alt-2, i.e.,: two coherent groups of {0,1,4,5} and {2,3,6,7} - Nc=4: Support Alt-1, i.e.,: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7} Proposal 4: Do not support layer splitting (3,2) or (4,3) for Ng=2 partilly-coherent UE. Proposal 5: A single TPMI field indicates a combination of two precoding matrices of the corresponding rank applied to the two antenna groups. Proposal 6: The TPMI field in the DCI scheduling PUSCH signals a 2TX codeword for each antenna group separately. Proposal 7: Use only mode 1 of Rel-15 DL Type 1 codebook for full coherent 8TX UE and study mechanism to indicate the paramters for the full coherent precoding matrix. Proposal 8: For full coherent UE, list all the precoders in a table and signal the index of the scheduled precoder as TPMI. Proposal 9: Support all 255 combinations from 8 non-coherent rank 1 precoders. Proposal 10: Introduce a bitmap based TPMI indication scheme for non-coherent 8Tx UE. Proposal 11: A fully coherent UE can only use the fully-coherent codebook. No RRC configuration is required. Proposal 12: A partially coherent UE with Ng=4 can only use the partial-coherent codebook for Ng=4. No RRC configuration is required. Proposal 13: A non-coherent UE can only the non-coherent codebook. No RRC configuration is required. Proposal 14: Study the benefit for a partially-coherent UE with Ng=2 to use the partially-coherent codebook for Ng=4; if the benefit can be justified, a Ng=2 UE can be configured in RRC to use either the Ng=2 or the Ng=4 codebook, but not both at the same time. Proposal 15: Study to use a subset of SRS resource combinations for reduced SRI overhead as a UE capability. Proposal 16: Use two SRI fields for transmission rank higher than 4. Proposal 17: When dual CWs PUSCH transmission is enabled by the NW, the UL grant should indicate the MCS, NDI, RV indication for the second CW. Proposal 18: Support permutation of the layers in the precoding matrix for transmission rank higher than 4. Proposal 19: Do not define or configure full power transmission mode for fully-cohernet UE. Proposal 20: Confirm the working agreement on Mode 1 and Mode 2 from the RAN1#112-bis meeting. Proposal 21: Mode 1 and Mode 2 need to be discussed separately for partial coherent UE (Ng=2 and Ng=4) and non-coherent UE. Proposal 22: Discuss further details for Mode 1 and Mode 2 after the codebooks for partial and non-coherent UE have stabalized. Proposal 23: Study possible UE PA architectures for Mode 1 and Mode 2 for partial coherent UE (Ng=2 and Ng=4) and non-coherent UE. Proposal 24: For partial coherent UE and non-coherent UE, focus on transmission rank no more than 4 for Mode 1 and Mode 2. Proposal 25: Study the performance benefits, signaling overhead and specification impact of supporting frequency-selective precoding for 8Tx UE. |
| NEC | Proposal 1: For partially coherent 8TX precoding with Ng =2 and Ng=4, support Option 4 (A single 8TX TPMI is indicated) for precoder indication. Proposal 2: For 8Tx partial coherent UE, it can further report one value of Ng for capability. Proposal 3: For full coherent UE, it can configured with precoders of Ng=1 (full-coherent) and Ng=8 (non-coherent) and one of Ng=2 or Ng=4 (partial-coherent); For partial coherent UE reporting capability of Ng=2, it can configured with precoders of Ng=2 (partial-coherent) and Ng=8 (non-coherent) and FFS whether Ng=4 (partial-coherent) can be configured; For partial coherent UE reporting capability of Ng=4, it can configured with precoders of Ng=4 (partial-coherent) and Ng=8 (non-coherent); For non coherent UE, it can configured with precoders of Ng=8 (non-coherent). |
| Sony | Proposal 1: RAN1 should confirm the working assumption - For uplink transmission with rank>4, support dual CW transmission. Proposal 2: A second set of NDI and RV fields for the second CW should be explicitly indicated by UL grant. Proposal 3: Configured grant PUSCH with dual CW transmission should be supported - For Type-1 CG: A second mcsAndTBS parameters is configured in rrc-ConfiguredUplinkGrant - For Type-2 CG: A second MCS field is added in DCI format activating a Type-2 CG-PUSCH Proposal 4: For codebook design of an 8TX partial-coherent UE, the following alternative for coherent groups should be supported - For when Ng=2, the following convention for assumption of port coherency scheme is used - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7} - For when Ng=4, the following convention for assumption of port coherency scheme is used - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7} |
| CMCC | Proposal 1: For codebook design of an 8 TX partial-coherent UE, when Ng=2, support Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7}, when Ng=4, support Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7}. Proposal 2: For partially coherent uplink precoding by an 8 TX UE, Ng=2, confirm the Working Assumptions with updates: Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (1,2), (2,1). 4; (4,0), (0,4). 4; (2,2). 5; (2,3), (3,2). 6; (3,3). 7; (3,4), (4,3). Proposal 3: For codebook design of an 8 TX partial-coherent UE, one precoding matrix W0 and phase offset α are indicated to UE for generating 8 TX codebook W8×8=W000α*W0. Proposal 4: For codebook design of an 8 TX partial-coherent UE, the phase offset equals to 0 could support to indicate the codebook of antenna groups selection. Proposal 5: Support joint indication of rank and precoding information, where RI is the total number of transmission layers from different antenna groups. Proposal 6: Support Alt1, and the combinations of Ng value(s) can be configured by RRC. - A fully-coherent UE (Ng =1) can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =1, 2, 4, 8 - A partially-coherent UE, with Ng =2 can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =2, 4, 8 - A partially-coherent UE, with Ng =4, can be configured with precoders considered for at least one or more Ng cases, i.e., Ng= 4, 8 - A non-coherent UE, Ng =8, can only be configured with precoders considered for Ng = 8 Proposal 7: For fullpowerMode1, at least one full coherent precoder per rank is selected for full power transmission. Proposal 8: For fullpowerMode2 with antenna virtualization, a maximum of 4 SRS resources with 8 ports, 4 ports, 2 ports, or 1 port can be supported for usage set to ‘codebook’ in a set to not increase the overhead of SRI field in DCI. Proposal 9: For full-coherent precoders, TPMI overhead is 7 bits for (N1, N2) = (2, 2) and (N1, N2) = (4, 1) values. |
| Intel Corporation | Proposal 1: Port mapping for Ng=2 is to have two coherent groups of {0,1,4,5} and {2,3,6,7} for both antenna layouts 2-a and 2-b. Port mapping for Ng=4 is to have four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7} for both antenna layouts 3-a and 3-b Proposal 2: For partially coherent uplink precoding by an 8TX UE, Ng=2, when rank>4, layers for each CW can be mapped to either or both of the antenna groups Proposal 3: Support nested-like codebook configuration - Alt2 (Nested-like codebook but needs re-configuration) - A fully-coherent UE (Ng=1) can only be configured with precoders associated with one of Ng cases, i.e., Ng=1, 2, 4, 8 - A partially-coherent UE, with Ng=2, can only be configured with precoders associated with one of Ng cases, i.e., Ng=2, 4, 8 - A partially-coherent UE, with Ng=4, can only be configured with precoders associated with one of Ng cases, i.e., Ng=4, 8 - A non-coherent UE, with Ng=8, can only be configured with precoders associated with Ng = 8 Proposal 4: TPMI indication of a full-coherent codebook is by a single index mapped to 𝒊𝒙 components. In order to reduce TPMI overhead consider introducing configurable restrictions on TPMI parameters 𝒊𝟏,𝟏, 𝒊𝟏,𝟐, 𝒊𝟏,𝟑, and 𝒊𝟐. Proposal 5: TPMI indication of a partial-coherent codebook (Ng=2 or Ng=4) is by a single index mapped to 2 or 4 Rel-15 TPMI components. Consider restrictions on layer-split options in order to reduce TPMI overhead. Proposal 6: Consider flexible rank restriction (bitmap) to reduce the DCI overhead for TPMI indication. Proposal 7: For full power Mode 1 operation, consider similar design principle as Rel-16, i.e., adding full coherent precoder to the non-coherent codebook subset and the partial coherent codebook subset. For full power Mode 2 operation, consider extending the Rel-16 framework to 8Tx UE. |
| Apple | Proposal 1: For fully-coherent codebook with 8Tx UL, use separate indication of RI (3 bits) and TPMI (5 bits), where the TPMI is a concatenation of 𝒊𝟏,𝟏,𝒊𝟏,𝟐,𝒊𝟏,𝟑(if needed) and 𝒊𝟐 as defined in Rel-15 single panel DL Type I codebook. Proposal 2: For partially-coherent codebook with 8Tx UL and Ng=2, adopt Option 3, using two 4TX TPMIs (10 bits in total). - Add one entry for TPMI to indicate no transmission from an antenna group - The first TPMI is applied on one antenna group, and the second TPMI is applied on the other antenna group. - All the combinations of layer splitting can be supported (i.e. the working assumption can be reverted). Proposal 3: For partial coherent codebook design with Ng = 2, there is no restriction that one codeword is mapped to one antenna group only. Proposal 4: For partially-coherent codebook with 8Tx UL and Ng=4, down-select between the following options: - Option 1: use 4 TPMIs to indicate 2Tx FC precoders for the 4 antenna groups, respectively (12 bits in total) - Option 2: a single 8Tx TPMI is provided, with the total number of precoders <= 1024 (<= 10 bits) o An example of down-selection for the layer splitting combinations is provided in Table 2. Proposal 5: For non-coherent codebook design, Alt 1 is adopted (i.e., all 255 combinations of the 8 antenna ports are supported). TPMI indication reuses the mechanism for SRI for non-codebook based transmission, which requires up to 8 bits. Proposal 6: A fully-coherent UE can be configured with at least one of the following subject to UE capability: - Fully-coherent precoders only - Fully-coherent precoders and (a subset of) non-coherent precoders - FFS: Fully-coherent precoders and (a subset of) partially-coherent precoders for Ng=2 - FFS: Fully-coherent precoders and (a subset of) partially-coherent precoders for Ng=4 Proposal 7: A partially-coherent UE with Ng=2 can be configured with one of the following subject to UE capability: - Partially-coherent precoders for Ng=2 - Partially-coherent precoders for Ng=2 and (a subset of) non-coherent precoders - FFS: Partially-coherent precoders for Ng=2 and (a subset of) partially-coherent precoders for Ng=4 Proposal 8: A partially-coherent UE with Ng=4 can only be configured with partially-coherent precoders for Ng=4. Proposal 9: For a partially-coherent UE with Ng=2 with full power Mode 1, add fully-coherent precoder(s) using all 8 Tx for rank = 1. Proposal 10: For a partially-coherent UE with Ng=4 with full power Mode 1, add fully-coherent precoder(s) using all 8 Tx for rank = 1/2/3. Proposal 11: For a non-coherent UE with full power Mode 1, add one fully-coherent precoder using all 8 Tx for each of rank = 1 to 7. Proposal 12: Deprioritize the specification work for full power transmission with Mode2. |
| LG Electronics | Proposal 1: For full coherent codebook, do not additionally support precoders from Alt2a. Proposal 2: Support Ng=2 and Ng=4 for fully coherent codebook. Proposal 3: Support two-level partial coherency for codebook based 8Tx UL transmission. - Level-1: 2-group 4Tx coherency (Ng=2) - Coherency PUSCH port groups consist of {1000, 1001, 1004, 1005} and {1002, 1003, 1006, 1007}. - Level-2: 4-group 2Tx coherency (Ng=4) - Coherency PUSCH port groups consist of {1000, 1004}, {1001, 1005}, {1002, 1006}, and {1003, 1007}. Proposal 4: For Ng=2 partial coherent codebook, confirm working assumption regarding layer splitting. Proposal 5: Support Alt 1 or Alt 2 (with MAC-CE) for codebook subset of CB based UL transmission. Proposal 6: Rank-1 uplink codebook for DFT-s-OFDM is supported in 8 Tx UL transmission. Proposal 7: Reuse UL DMRS antenna port indication field for NDI and/or RV field indication of 2nd CW in order to reduce DCI overhead. |
| Qualcomm Inc. | Proposal 1: For codebook -based 8TX PUSCH transmission, adopt Alt 1 with the following modification. - Alt1 - A fully-coherent UE (Ng =1) can be configured with precoders with at least Ng=1, 8. considered for at least one or more Ng cases, i.e., Ng =1, 2, 4, 8 - FFS whether support additional Ng(s). which combinations of Ng value(s), to be considered - A partially-coherent UE , with Ng =2 can be configured with precoders with Ng=2, 8. considered for at least one or more Ng cases, i.e., Ng =2, 4, 8 - FFS whether support additional Ng(s). which combinations of Ng value(s), to be considered - A partially-coherent UE , with Ng =4, can be configured with precoders with Ng=4, 8. considered for at least one or more Ng cases, i.e., Ng= 4, 8 - FFS which combinations of Ng value(s), if any, to be considered - A non-coherent UE , Ng =8, can only be configured with precoders considered for Ng = 8. Proposal 2: For 8 Tx PUSCH in Rel-18, Ng=2, 4 are not applicable to fully coherent 8 Tx. Proposal 3: For a fully coherent uplink precoding by an 8TX UE - Support NR Rel-15 single panel DL Type I codebook mode 1 as the starting point for design of the codebook - Do not support sub-band based precoding. Proposal 4: Update the agreement made in RAN1 #111 as the following. For a fully coherent uplink precoding by an 8TX UE, - Support NR Rel-15 single panel DL Type I codebook as the starting point for design of the codebook - FFS: For a constructed codebook with size M based on above method, unless M=2N; otherwise, round up the codebook size to the smallest integer 2N>M by adding 2N-M precoders generated via Alt 2a. - No LS to RAN4 will be needed Proposal 5: For fully coherent 8 Tx codebook, further study the following hybrid codebook design. - Reduce the number of DFT precoders for rank 1,2,3,4 generated based on NR Rel-15 single panel DL Type I codebook. - Add nonDFT precoders generated via Alt 2a to rank 2, 3, 4. Proposal 6: Following Table 5 and Table 6, NR Rel-18 concatenate existing Rel-15 4 Tx or 2 Tx PUSCH precoders to support 8 Tx PUSCH precoders with partial coherent or noncoherent 8 Tx. - FFS how to reduce the size of the codebook. Proposal 7: For partially coherent uplink precoding by an 8TX UE codebook, - When Ng=2, do not support using Rel-15 UL 4 TX partial-coherent precoders to construct 8 Tx partial coherent codebook. - When Ng=4, do not support using Rel-15 UL 2 TX partial-coherent precoders to construct 8 Tx partial coherent codebook. Proposal 8: Confirm the following working assumption made in RAN1#112bis-e with the following modification. For partially coherent uplink precoding by an 8TX UE, Ng=2, - At least the following combinations of layer splitting are supported - FFS: For rank>4, all the layers for each CW is mapped to only one antenna group Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (1,2), (2,1). 4; (4,0), (0,4). 4; (2,2). 5; (2,3), (3,2). 6; (3,3). 7; (3,4), (4,3). Proposal 9: Update the agreement made in RAN1#112bis-e with the following down-selection. For partially coherent uplink precoding by an 8TX UE codebook, Ng=4, - In addition to the previously agreed cases, down-select from support the rank and layer splitting cases listed below Rank; All layers in one Antenna Group; Layers split across 4 Antenna Groups (All possible permutations). 3; Transmission by 2 of the 4 antenna groups: (2,1,0,0), (2,0,1,0), (2,0,0,1), (0,2,1,0), (0,2,0,1), (0,0,2,1), (1,2,0,0), (1,0,2,0), (1,0,0,2), (0,1,2,0), (0,1,0,2), (0,0,1,2) Transmission by 3 of the 4 antenna groups: (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1). 4; Transmission by 3 of the 4 antenna groups: (2,1,1,0), (0,2,1,1), (1,0,2,1), (1,1,0,2) (1,2,1,0), (1,1,2,0), (0,1,2,1), (0,1,1,2), (1,0,1,2), (2,0,1,1), (2,1,0,1), (1,2,0,1). 5; Transmission by 3 of the 4 antenna groups: (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1), (2,1,2,0), (1,2,2,0), (2,1,0,2), (1,2,0,2), (2,0,1,2), (1,0,2,2), (0,2,1,2), (0,1,2,2) Transmission by 4 of the 4 antenna groups: (1,1,2,1), (1,1,1,2), (2,1,1,1), (1,2,1,1). 6; Transmission by 3 of the 4 antenna groups: (2,2,2,0), (2,2,0,2), (2,0,2,2), (0,2,2,2) Transmission by 4 of the 4 antenna groups: (2,1,2,1), (1,2,1,2), (1,2,2,1), (2,1,1,2), (2,2,1,1), (1,1,2,2. 7; Transmission by 4 of the 4 antenna groups: (2,1,2,2), {(2,2,2,1), (1,2,2,2), (2,2,1,2). Proposal 10: Following Table 5 and Table 6, a single 8 Tx TPMI is used to signal the precoder index for partial coherent 8 Tx PUSCH, i.e., adopt option 4 agreed in RAN1#112bis-e. Proposal 11: For codebook design of an 8TX partial-coherent UE, configured with an 8-port SRS resource - For when Ng=2, the following convention for assumption of port coherency scheme is used - Alt 3: two coherent groups of {0,1,2,3} and {4,5,6,7} - For when Ng=4, the following convention for assumption of port coherency scheme is used - Alt 2: four coherent groups of {0,1}, {2,3}, {4,5}, and {6,7} Proposal 12: 8 Tx noncoherent codebook size reduction can be considered only if minor performance reduction is observed. Proposal 13: For SRS configuration for non-codebook UL transmission for an 8TX UE, further support configuration of up to two, or four SRS resource sets, each configured with up to 4, or 2 single-port SRS resources, respectively. Proposal 14: For SRI indication for NCB-based 8TX PUSCH transmission with NSRS>4, support Option 2: Use a legacy-based solution, as in Rel-15. Proposal 15: For a rank>4 PUSCH without UL-SCH, UCIs are multiplexed on the CW with the higher MCS index (if MCS indices are the same, UCI is multiplex on the first CW). The MCS of the selected CW is used to determine rate matching for the multiplexed UCIs. Proposal 16: in Rel-18, for a PUSCH transmission with 2 TBs/CWs, the two TBs/CW are with a same PHY layer priority. Proposal 17: Confirm the scope of Rel-18 8Tx PUSCH includes CG-PUSCH with 2 TBs/CWs and up to 8 layers. Proposal 18: For CG-PUSCH with 2 TBs/CWs, support new CG-UCI fields of “Redundancy version” and “new data indicator” for the second TB/CW. Proposal 19: For CG-PUSCH with 2 TBs/CWs, study the following - How to enable/disable the second CW of the CG-PUSCH - How to multiplex the CG-UCIs for the two TBs/CWs onto the CG-PUSCH. Proposal 20: Full power operation for a partial/non-coherent 8TX UE should support at least PA architecture which does not have full rated PA on each of the 8 Tx chains. Proposal 21: In addition to reusing Rel-16 full power mode 0/1/2, support a new mode 0A for full power transmission for PUSCH with 8 Tx. - Mode 0A set the power scaling factor α = min(1,i=18αiδi) for a PUSCH transmission, where αi is the power scaling factor the i-th Tx port. δi=1 if i-th Tx port is used in the PUSCH transmission, δi=0 otherwise. Proposal 22: Confirm the working assumption for full power mode 1 and 2, with the following modification. - To support full power transmission with Mode1, Rel-16 Mode1 (fullPowerMode1) is re-used. - FFS if more than Only one of the 8TX full coherent precoders is used per rank. - To support full power transmission with Mode2, Rel-16 Mode2 (fullPowerMode2) is re-used. - FFS definition of precoder groups (G0, G1, …) FFS enhancements for SRS configuration. |
| OPPO | Proposal 1: At least part of the following parameters can be configured by RRC for 8Tx codebook - (N1, N2), which is the same as UE reported values - RI restriction, which would impact the TPMI size - CBSR, which indicates the available codebook parameters for 8Tx CB - (k1, k2), for DFT vector offset among layers (if not indicated by DCI) - Proposal 2: For partial-coherent uplink codebook with Ng=2, only support the following layer splitting in the working assumption: Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (1,2), (2,1). 4; (4,0), (0,4). 4; (2,2). 5; (2,3), (3,2). 6; (3,3). 7; (3,4), (4,3). Proposal 3: For partial-coherent uplink codebook with Ng=4, support the following layer splitting cases: Rank; All layers in one Antenna Group; Layers split across 4 Antenna Groups (All possible permutations). 3; Transmission by 2 of the 4 antenna groups: (2,1,0,0), (2,0,1,0), (2,0,0,1), (0,2,1,0), (0,2,0,1), (0,0,2,1), (1,2,0,0), (1,0,2,0), (1,0,0,2), (0,1,2,0), (0,1,0,2), (0,0,1,2) Transmission by 3 of the 4 antenna groups (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1). 4; Transmission by 3 of the 4 antenna groups: (2,1,1,0), (0,2,1,1), (1,0,2,1), (1,1,0,2) (1,2,1,0), (1,1,2,0), (0,1,2,1), (0,1,1,2), (1,0,1,2), (2,0,1,1), (2,1,0,1), (1,2,0,1). 5; Transmission by 3 of the antenna groups: (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1), (2,1,2,0), (1,2,2,0), (2,1,0,2), (1,2,0,2), (2,0,1,2), (1,0,2,2), (0,2,1,2), (0,1,2,2) Transmission by 4 of the 4 antenna groups: (1,1,2,1), (1,1,1,2), (2,1,1,1), (1,2,1,1). 6; Transmission by 3 of the 4 antenna groups: (2,2,2,0), (2,2,0,2), (2,0,2,2), (0,2,2,2) Transmission by 4 of the 4 antenna groups: (2,1,2,1), (1,2,1,2), (1,2,2,1), (2,1,1,2), (2,2,1,1), (1,1,2,2). 7; Transmission by 4 of the 4 antenna groups: (2,1,2,2), (2,2,2,1), (1,2,2,2), (2,2,1,2). Proposal 4: For non-coherent 8Tx codebook, all antenna port combinations are supported. Proposal 5: On the codebook configuration for different coherent capabilities, support Alt.2. Proposal 6: Strive for unified TPMI signaling at least for partial coherent codebook with different Ng and full-coherent codebook. Proposal 7: For Ng=2, support two coherent groups of {0,1,4,5} and {2,3,6,7}; For Ng=4, support four coherent groups of {0,4}, {1,5}, {2,6} and {3,7}. Proposal 8: Multiple SRS resource sets for CB or NCB based 8-port uplink transmission is not needed. Proposal 9: one SRI/TRI/TPMI indication is sufficient for uplink codebook based 8-port transmission. Proposal 10: Confirm the working assumption for Rel-16 full power Mode 1 for uplink 8Tx transmission. - Rel-16 full power Mode 2 for uplink 8Tx transmission can be deprioritized. |
| Ericsson | Proposal 1: For UL 8 Tx partial-coherent codebooks with two antenna groups, i.e., Ng=2, ‘Group-Selection + Balanced’ partial-coherent precoders are used with layer distributions according to Table 5, including (a) precoders where a single antenna group is selected and a single UL Rel-15 4 Tx precoder is used for rank ≤4 and additionally (b) precoders where combinations of two UL Rel-15 4 Tx precoders with a nearly equal number of layers are used. The following additional restrictions are applied to limit the codebook size: i. Restrict the UL Rel-15 4 Tx precoders to the precoders with the oversampling factor of O1=1, and ii. Restrict the layer distribution over the antenna groups to match the DL codeword-to-layer mapping for r>4, i.e., the larger number of layers is mapped to the second group. Proposal 2: For UL 8 Tx partial-coherent codebooks with four antenna groups, i.e., Ng=4, ‘Group-Selection + Balanced’ partial-coherent precoders are used with layer distributions according to Table 12, including (a) precoders that strive to minimize the number of active antenna groups and one or more Rel-15 2 Tx UL precoders are used and additionally (b) precoders where combinations of Rel-15 2 Tx UL precoders with a nearly equal number of layers are used. The following further restrictions are used to limit the codebook size: i. Restrict the 2 Tx FC precoders to the precoders with co-phasing factors between the two antenna ports restricted to 1,-1, and ii. Form two antenna group pairs, each with two antenna groups and restrict the layer distribution over the antenna group pairs to match the DL codeword-to-layer mapping for r>4, i.e., the larger number of layers is mapped to the second antenna group pair. Proposal 3: 8 Tx codebook subset design uses at least fully- and non-coherent precoders, targeting power saving, coherence fallback, and directional antennas with the non-coherent precoders. Proposal 4: 8 Tx fully coherent precoders are not constructed by cophasing across Rel-15 precoders Proposal 5: Full power mode 1 can be configured with at most one TPMI per each of ranks 1-8. Proposal 6: Full power mode 2 with full power TPMIs can be configured with at most one TPMI per each of ranks 1-8. Proposal 7: Full power mode 2 with multiple SRS resources can be configured with one or more of 1, 2, 4, or 8 SRS port resources in an SRS resource set Proposal 8: Defer discussion of support for configured grant operation until dynamic grant operation for 8 Tx is stable. At that time, address specification impacts including those of multiple codewords. Proposal 9: Unless multi-SRS resource set operation is defined for 8 Tx CB-based operation, it is not defined for 8 Tx NCB-based operation in Rel-18. Proposal 10: Focus RAN1 discussions on precoder structure and codebook size rather than how to signal TPMIs for as yet undefined codebooks. Proposal 11: A PDCCH carries a single TPMI/TRI field for 8 Tx operation, where the indicated precoder corresponds to one SRS resource. Proposal 12: Identify Rel-18 8 Tx fully coherent precoders for each rank 1-8 and each of (N1, N2) = (4, 1) and (N1, N2) = (2, 2) according to and i1,1, i1,2, i1,3, and i2. Further discuss how to indicate the precoders in a TPMI. |
| Samsung | Proposal 1: antennae within a group are coherent, and antennae across multiple groups are non-coherent - Do not support full coherent precoders with Ng=2,4 Proposal 2: reuse DL Type I codebook parameters (Ng,N1,N2) to describe/configure 8Tx UL codebook for different coherence types - FC: Ng,N1,N2,P=1,4,1,2, 1,2,2,2 - PC: Ng,N1,N2,P=2,2,1,2,(4,1,1,2) - NC: Ng,N1,N2,P=8,-,-,- - FC/PC precoders: comprises two components - selection of antenna group(s) - precoder associated with the selected antenna group(s) - NC precoders: selection of antenna group(s), where a group comprises single antenna Proposal 3: to reduce 8Tx TPMI payload, reuse the legacy principle (e.g. Rel.15 4Tx UL codebook) in which number of precoders for higher rank values is restricted to a small value. Proposal 4: support the following regarding full-coherent precoder design - CodebookMode1 (L=1) - Codebook subsampling: a subset of Rel. 15 Type I codebook is used as FC precoders in 8Tx UL codebook as shown in Table 2 - (i1,1,i1,2): subsampling for rank > 4 - i1,3: only i1,3=0 - i2: all values Proposal 5: for 8Tx PC precoders, support separate indication of (i) up to Ng FC TPMIs and (ii) the mapping/ordering of the TPMIs to Ng groups - For (i), the UE is indicated with n∈{1,…,Ng} Ng-Tx FC TPMI(s), where n-th TPMI corresponds to rank lxn - For (ii), the UE is indicated with the mapping/ordering x1,…,xn via higher layer or MAC CE Proposal 6: for Ng=2 - Support confirming the working assumption on the layer splitting values - Support indication of {lxn} values according to Table 3 and the mapping x1,x2∈{1,2,2,1}. - Do not support the restriction that all the layers for each CW is mapped to only one antenna group Proposal 7: for Ng=4 - Do not support {lxn} values for n=3 - Support indication of {lxn} values according to Table 5 and the mapping/ordering x1,x2,x3,x4. Proposal 8: for NC precoders, - support Alt2 and number of NC precoders is at most 32 - support one of the following schemes - Scheme 1: 8Tx NC precoders are based on selecting consecutive antenna ports - Scheme 2: 8Tx NC precoders are based on legacy Rel.15 NC 4Tx precoders, as shown in Table 7 Proposal 9: support Alt2 regarding the codebook configuration - A fully-coherent UE (Ng =1) can only be configured with precoders considered for one of Ng = 1, 2, 8 or 1, 4, 8 subject to UE capability - A partially-coherent UE , with Ng =2, can only be configured with precoders considered for one of Ng =2, 8 - A partially-coherent UE , with Ng =4, can only be configured with precoders considered for one of Ng =4, 8 - A non-coherent UE , with Ng =8, can only be configured with precoders considered for Ng = 8 - MAC CE indication to indicate one of the configured Ng values Proposal 10: - support TPMI payload reduction based on efficient signalling for the indication of (A) antenna group(s), and (B) UL precoding matrix - support Option 3 regarding partially coherent 8TX precoding with Ng =2 - When one 4Tx TPMI is indicated, the corresponding antenna group is indicated separately Proposal 11: - Discussion on full power modes can start after the 8Tx codebook design is sufficiently mature - Do not support any change in specification for Mode0 - If WA on Mode1 is confirmed, support only one FC TPMI for a given rank, and do not support more than one FC TPMIs - If WA on Mode2 is confirmed, - TPMI group: for NC TPMIs, support TPMI groups based on legacy TPMI groups; and for PC TPMIs, support TPMI groups corresponding to precoders associated with n=1,…,Ng-1 antenna ports selected for the precoders. - SRS resource set: extend legacy by including a value 8 in addition to {1,2,4} Proposal 12: regarding 8Tx NCB based UL transmission, - Specify new SRI indication for NSRS≥5 and Lmax≥5 - Do not support NSRS≥5 and Lmax≤4 - support two SRS resource sets - SRI indication is based on legacy SRI tables Proposal 13: regarding 2 CWs for > 4 layers, confirm the working assumption |
| NTT DOCOMO, INC. | Proposal 1: Confirm the working assumption to support dual CW for uplink transmission with rank>4. Proposal 2: For CBG-based transmission for dual CW PUSCH operation, support N=2, 4, 6 or 8 for the CBGTI bit field. Proposal 3: Support CG for dual CW PUSCH transmission by an 8TX UE, - For Type-1 CG, a second mcsAndTBS parameters is configured in rrc-ConfiguredUplinkGrant, - For Type-2 CG, the special fields setting with second fields of MCS/NDI/RV for Type-2 CG-PUSCH activation and release should be discussed. - Further study whether to support activation/release of one transmission block or always two transmission blocks for a CG configuration. Proposal 4: For codebook subset configuration for 8TX UE, Alt2 is supported, where - A fully-coherent UE (Ng =1) can only be configured with precoders considered for one of Ng cases, i.e., Ng =1, 2, 4, 8. - At least support Ng=1, 2, 8 for the considered Ng cases. Open for Ng=4 case. - A partially-coherent UE, with Ng =2, can only be configured with precoders considered for one of Ng cases, i.e., Ng =2, 4, 8. - At least support Ng=2, 8 for the considered Ng cases. Open for Ng=4 case. - A partially-coherent UE, with Ng =4, can only be configured with precoders considered for one of Ng cases, i.e., Ng =4, 8. - Support Ng=4, 8 for the considered Ng cases. - A non-coherent UE, with Ng =8, can only be configured with precoders considered for Ng = 8. - The configuration is based on RRC signaling. Proposal 5: For codebook subset configuration for 8TX UE, additional UE capability is not needed at least for codebook subset configuration of non-coherent precoders. - A fully-coherent UE or partially-coherent UE with Ng=2 or Ng=4 can be configured with non-coherent precoders without additional UE capability. Proposal 6: For non-coherent precoders, support Alt1. - Alt1: All 255 combinations from 8 non-coherent rank1 precoders are supported. Proposal 7: For partially-coherent precoders with Ng=2, - Support to confirm the working assumption. - The candidates of fully-coherent precoders from Rel-15 UL 4TX codebook should be reduced. Proposal 8: For partially-coherent precoders with Ng=4, - First preference is to support all the listed layer split cases in following table. - The candidates of fully-coherent precoders from Rel-15 UL 2TX codebook should be reduced. Rank; All layers in one Antenna Group; Layers split across 4 Antenna Groups (All possible permutations). 3; Transmission by 2 of the 4 antenna groups: (2,1,0,0), (2,0,1,0), (2,0,0,1), (0,2,1,0), (0,2,0,1), (0,0,2,1), (1,2,0,0), (1,0,2,0), (1,0,0,2), (0,1,2,0), (0,1,0,2), (0,0,1,2) Transmission by 3 of the 4 antenna groups: (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1). 4; Transmission by 3 of the 4 antenna groups: (2,1,1,0), (0,2,1,1), (1,0,2,1), (1,1,0,2) (1,2,1,0), (1,1,2,0), (0,1,2,1), (0,1,1,2), (1,0,1,2), (2,0,1,1), (2,1,0,1), (1,2,0,1). 5; Transmission by 3 of the antenna groups: (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1), (2,1,2,0), (1,2,2,0), (2,1,0,2), (1,2,0,2), (2,0,1,2), (1,0,2,2), (0,2,1,2), (0,1,2,2) Transmission by 4 of the 4 antenna groups: (1,1,2,1), (1,1,1,2), (2,1,1,1), (1,2,1,1). 6; Transmission by 3 of the 4 antenna groups: (2,2,2,0), (2,2,0,2), (2,0,2,2), (0,2,2,2) Transmission by 4 of the 4 antenna groups: (2,1,2,1), (1,2,1,2), (1,2,2,1), (2,1,1,2), (2,2,1,1), (1,1,2,2. 7; Transmission by 4 of the 4 antenna groups: (2,1,2,2), {(2,2,2,1), (1,2,2,2), (2,2,1,2). - Second preference is to consider down-selection of supported layer splitting cases from those with same antenna port selection. One example is shown in following table. - The candidates of fully-coherent precoders from Rel-15 UL 2TX codebook should be reduced. Rank; All layers in one Antenna Group; Layers split across 4 Antenna Groups (All possible permutations). 3; Transmission by 2 of the 4 antenna groups: (2,1,0,0), (2,0,1,0), (2,0,0,1), (0,2,1,0), (0,2,0,1), (0,0,2,1), (1,2,0,0), (1,0,2,0), (1,0,0,2), (0,1,2,0), (0,1,0,2), (0,0,1,2) Transmission by 3 of the 4 antenna groups: (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1). 4; Transmission by 3 of the 4 antenna groups: (2,1,1,0), (0,2,1,1), (1,0,2,1), (1,1,0,2) (1,2,1,0), (1,1,2,0), (0,1,2,1), (0,1,1,2), (1,0,1,2), (2,0,1,1), (2,1,0,1), (1,2,0,1). 5; Transmission by 3 of the antenna groups: (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1), (2,1,2,0), (1,2,2,0), (2,1,0,2), (1,2,0,2), (2,0,1,2), (1,0,2,2), (0,2,1,2), (0,1,2,2) Transmission by 4 of the 4 antenna groups: (1,1,2,1), (1,1,1,2), (2,1,1,1), (1,2,1,1). 6; Transmission by 3 of the 4 antenna groups: (2,2,2,0), (2,2,0,2), (2,0,2,2), (0,2,2,2) Transmission by 4 of the 4 antenna groups: (2,1,2,1), (1,2,1,2), (1,2,2,1), (2,1,1,2), (2,2,1,1), (1,1,2,2). 7; Transmission by 4 of the 4 antenna groups: (2,1,2,2), {(2,2,2,1), (1,2,2,2), (2,2,1,2). Proposal 9: For full power PUSCH transmission by an 8TX UE, confirm the working assumptions with following update, - To support full power transmission with Mode1, Rel-16 Mode1 (fullPowerMode1) is re-used. - Discuss the number of supported fully-coherent precoders for each rank separately for codebook subset configuration of partially-coherent precoders with Ng=2, partially-coherent precoders with Ng=4, or non-coherent precoders. - To support full power transmission with Mode2, Rel-16 Mode2 (fullPowerMode2) is re-used. - Discuss the definition of precoder groups separately for different identified PA architectures. - Open to consider enhanced SRS configuration with different number of SRS ports for multiple SRS resources within an SRS resource set. Proposal 10: For codebook subset configuration of fully-coherent precoders for each (N1, N2) case, support joint indication of TRI and TPMI (an example is shown in Table 8). - The detailed indexing between TPMI per rank and the {i1,1,i1,2,i1,3,i2} in Rel-15 DL Type I codebook can be up to editor. Proposal 11: For codebook subset configuration of non-coherent precoders, support joint indication of TRI and TPMI (similar as Table 8). - The detailed indexing between TPMI per rank and the port selection precoder can be up to editor. Proposal 12: For codebook subset configuration of partially-coherent precoders with Ng=2, support joint indication of TRI and TPMI (an example is shown in Table 10). - The candidates of 4TX precoders can be predefined with candidate index (an example is shown in Table 11). The detailed indexing between TPMI per rank and the candidate index can be up to editor. Proposal 13: For codebook subset configuration of partially-coherent precoders with Ng=4, the TRI/TPMI indication method is discussed after codebook design is finished. - Note that if all the layer splitting cases are supported and the candidates of 2TX precoders is equal to 2N-1, the bit size of Option 3 and Option 4 is the same. |
| MediaTek Inc. | Proposal 1: Support possible layer distributions across coherency groups on Table IV for Ng=2. Proposal 2: We propose to restrict to use a subset of the rank-1 full coherent 4TX precoders that can be used for 8TX Ng=2 for rank 2 and rank 3 scenarios. Proposal 3: Consider 2TX basis and co-phasing factors to generate co-phased beams which will serve as the precoders of partially coherent UE (Ng = 2 or 4). The following predefined basis set and co-phasing factors can be considered to generate all possible Legacy full coherent 2TX/4TX precoders thereby used for 8TX Partial coherent CBs. - 11;1-1;1j;1-j; 111-1;11j-j; ∅∈{1,j,-1,-j} Proposal 4: Rank-wise mathematical expressions-based approach as presented in Table II can be adopted for partial coherent CBs. Restrictions on possible co-phasing factors can be considered for each rank to have uniformity (in TPMI sizes) with the full coherent CB approach. Proposal 5: TPMI indication should include the information of basis, co-phasing factor, and the coherency group selection. Proposal 6: Support possible layer distributions, as described in Table VI, across coherency groups for Ng=4. Proposal 7: For non-coherent UE, support precoders with only a subset of antenna selection vectors for each rank. Proposal 8: Support to reuse Rel-16 Mode 0 full power operation for UL 8TX without any further update in the specifications apart from scaling of codebook. Proposal 9: Support to reuse Rel-16 Mode 1 full power operation for UL 8TX and identify the full coherent 8TX CBs that works for mode-1 operation. |
| Sharp | Proposal 1: The codebookSubset should be separated by one coherent capability. Proposal 2: We don’t support the working assumption for the layer splitting. Proposal 3: Support the all layer combination in case of Ng = 2. Proposal 4: Support the Alt3 for the codebook configuration. - Alt3 - A fully-coherent UE (Ng =1) can only be configured with precoders considered for Ng =1 - A partially-coherent UE, with Ng =2, can only use precoders considered for Ng =2 - A partially-coherent UE, with Ng =4, can only use precoders considered for Ng =4 - A non-coherent UE, with Ng =8, can only use precoders considered for Ng = 8 Proposal 5: Support the full power transmission Mode 1. Proposal 6: Discussion on full power transmission Mode 2 should be postponed until the codebook design specification is completed. |
| Nokia, Nokia Shanghai Bell | Proposal 1: Considering feedback overhead for 8Tx uplink, support Type-I codebookMode=1 CB as fully coherent codebook. Proposal 2: For (N1, N2) = (4,1), consider supporting at least (O1, O2) = (4,1) based on UE capability. For (N1, N2) = (2,2), consider supporting at least (O1, O2) = (4,4) based on UE capability. Proposal 3: Consider limiting certain 4Tx precoder combinations to reduce overhead for Ng=2 partially coherent CB design. Proposal 4: Consider limiting certain 2Tx precoder combinations to reduce overhead for Ng=4 partially coherent CB design. Proposal 5: Study other possible implementations other than Ng=2 and Ng=4 Proposal 6: Subject to its capability, an 8TX UE may report more than one Ng value as part of its UE capability. Based on which, gNB may select and configure UE with one of supported Ng values. Based on UE indicated capability of supporting Ng, UE can be configured to operate with its supported Ng values with RRC. Proposal 7: Further study is needed to mitigate the spec impact on SRI table to support 8TX NCB Tx. Proposal 8: Study channel models that capture the effect of differing average per layer BLER performance for link level evaluation of 8TX. Proposal 9: Confirm the WA and support Rel-16 full power mode 1 and mode 2 for 8Tx support. Proposal 10: Use these two antenna layouts with Ng=2 and Ng=4 to support model-1 and model-2 for full Tx power feature. Proposal 11: Support CG for dual CW PUSCH transmission for 8Tx. |
| FGI | Proposal 1: Support configuration of at least one SRS resource set, configured with 𝟖/𝑴 of M-port SRS resources for SRS configuration supporting codebook-based UL transmission by an 8TX UE (𝑴=𝟐,𝟒) Proposal 2: For codebook design of an 8TX partial-coherent UE - Support antenna numbering with two coherent groups with {0,1,2,3} and {4,5,6,7} when 𝑵𝒈=𝟐 - Support antenna numbering with four coherent groups with {0,1} and {2,3} and {4,5} and {6,7} when 𝑵𝒈=𝟒 ▪ A fully-coherent UE (Ng=1) can be configured with precoders considered for Ng cases from one of two groups: {1, 2, 8} and {1, 4, 8} Proposal 3: Support joint indication of TRI and TPMI for CB-based 8TX PUSCH transmission Proposal 4: For codebook-based 8TX PUSCH transmission, support - A fully-coherent UE (Ng=1) can be configured with precoders considered for Ng cases from one of two groups: {1, 2, 8} and {1, 4, 8} - A partially-coherent UE, with Ng =2 can be configured with precoders considered for Ng cases from the group {2, 8} - A partially-coherent UE, with Ng =4, can be configured with precoders considered for Ng cases from the group {4, 8} - A non-coherent UE, Ng =8, can only be configured with precoders considered for Ng = 8 |
| CEWiT | Proposal 1: Confirm the working assumption for partial coherent uplink precoding by an 8Tx UE, 𝑵𝒈=𝟐 to support different combinations for layer splitting. Proposal 2: For 𝑵𝒈=𝟐, additional combinations should be studied for rank > 4. Proposal 3: At least in the case of layer splitting for 𝑵𝒈=𝟐 partially coherent uplink precoding, support all the layers of each codeword is mapped to only one antenna group. Proposal 4: Support all 255 combinations from 8 non-coherent rank1 precoders for non-coherent uplink precoding with rank ≤ 8. Proposal 5: Support all possible permutations for layer splitting across antenna groups in the case of partially coherent uplink precoding by an 8TX UE when 𝑵𝒈=4. Proposal 6: At least for 𝑵𝒈=𝟒, each antenna group should not contain layers corresponding to more than one codeword. |
| KDDI Corporation | Proposal 1: MCS, NDI and RV indications for the 2nd CW are specified in DCI format 0_1. Proposal 2: Type 2 configured grant transmission is supported in the case where maxRank and maxMIMO-Layers for uplink are set to be more than 4. Proposal 3: MCS of the 2nd CW is included in the DCI addressed to CS-RNTI in the case of Type-2 configured grant transmission. Also, if Type-1 configured grant transmission is supported, it is included in rrc-ConfiguredUplinkGrant. Proposal 4: In the agreement reached at the RAN1 #112-bis meeting regarding the CBGTI DCI field, N is 4 or 8 if the downlink scheme is diverted. |

# RAN1 Agreements for Sub-agenda 9.1.4.2

RAN1 Meeting #112b-e Conclusion For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook (CodebookMode=1), there is no consensus to support any optional over-sampling ratio. Working Assumption For partially coherent uplink precoding by an 8TX UE, Ng=2, - At least the following combinations of layer splitting are supported - FFS: For rank>4, all the layers for each CW is mapped to only one antenna group Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (1,2), (2,1). 4; (4,0), (0,4). 4; (2,2). 5; (2,3), (3,2). 6; (3,3). 7; (3,4), (4,3). Agreement To configure PUSCH transmission by an 8TX UE, - Alt2: Max number of MIMO layers is RRC configured by extending the range of the legacy parameter maxRank and maxMIMO-Layers to 8 Agreement To support dual CW PUSCH operation by an 8TX UE, if CBG-based transmission is configured, the DL principle for CBGTI DCI field is reused where, - The first half of CBGTI field bits is used to indicate the transmission state of CBGs of the first transport block, while the second half of CBGTI field bits is used to indicate the transmission state of CBGs of the second transport block. - The bit field may be configured to have a length of N bits that can support operation of N/2 CBGs , where N=[2, 4, 6 or 8]. Agreement Framework for full power PUSCH transmission by an 8TX UE - To support full power transmission with Mode0, Rel-16 Mode0 (fullPower ) is re-used. - FFS if any change is required in the specifications. - Working Assumption To support full power transmission with Mode1, Rel-16 Mode1 (fullPowerMode1) is re-used. - FFS if more than one of the 8TX full coherent precoders is used per rank. - Working Assumption To support full power transmission with Mode2, Rel-16 Mode2 (fullPowerMode2) is re-used. - FFS definition of precoder groups (G0, G1, …) - FFS enhancements for SRS configuration Agreement For 8TX UE supporting dual CW PUSCH (Maximum number of layers configured for the UE is larger than 4) - Alt1 – DL principle is reused for disabling transmission of a transport block, where - The combination of IMCS = 26 and rvid = 1 indicated for a CW is used as an indication to disable (when transmission rank<=4) transmission of its corresponding TB - The enabled transport block is mapped to the first CW. - Note: When the transmission of a transport block is disabled, the number of layers is ≤ 4. - Note: the first CW refers to the enabled CW. Agreement For partially coherent uplink precoding by an 8TX UE codebook, Ng=4, Alt1 is supported where - Precoding design is based on Rel-15 UL 2TX codebook, - Full-coherent precoders are used - Further study codebook size reduction Agreement For partially coherent uplink precoding by an 8TX UE codebook, Ng=4, - The following rank and layer splitting cases are supported, Rank; All layers in one Antenna Group; Layers split across 4 Antenna Groups. 1; (1,0,0,0), (0,1,0,0), (0,0,1,0), (0,0,0,1). 2; (2,0,0,0), (0,2,0,0), (0,0,2,0), (0,0,0,2). 2; Transmission by 2 of the 4 antenna groups: (1,1,0,0), (1,0,1,0), (1,0,0,1) (0,1,1,0), (0,1,0,1), (0,0,1,1). 4; (1,1,1,1). 4; Transmission by 2 of the 4 antenna groups: (2,2,0,0), (2,0,2,0), (2,0,0,2) (0,2,2,0), (0,2,0,2), (0,0,2,2). 8; (2, 2, 2, 2). Note: Above is not relevant to how precoders are indicated. Agreement For non-coherent uplink precoding with rank≤8 by an 8TX UE, down-select from - Alt1. – All 255 combinations from 8 non-coherent rank1 precoders are supported - Alt2. – Only a subset of Alt1. is supported, striving for a substantial reduction in the number of precoders Agreement For partially coherent uplink precoding by an 8TX UE codebook, Ng=4, - In addition to the previously agreed cases, down-select from the rank and layer splitting cases listed below Rank; All layers in one Antenna Group; Layers split across 4 Antenna Groups (All possible permutations). 3; Transmission by 2 of the 4 antenna groups: (2,1,0,0), (2,0,1,0), (2,0,0,1), (0,2,1,0), (0,2,0,1), (0,0,2,1), (1,2,0,0), (1,0,2,0), (1,0,0,2), (0,1,2,0), (0,1,0,2), (0,0,1,2) Transmission by 3 of the 4 antenna groups: (1,1,1,0), (1,1,0,1), (1,0,1,1), (0,1,1,1). 4; Transmission by 3 of the 4 antenna groups: (2,1,1,0), (0,2,1,1), (1,0,2,1), (1,1,0,2) (1,2,1,0), (1,1,2,0), (0,1,2,1), (0,1,1,2), (1,0,1,2), (2,0,1,1), (2,1,0,1), (1,2,0,1). 5; Transmission by 3 of the antenna groups: (2,2,1,0), (2,2,0,1), (2,0,2,1), (0,2,2,1), (2,1,2,0), (1,2,2,0), (2,1,0,2), (1,2,0,2), (2,0,1,2), (1,0,2,2), (0,2,1,2), (0,1,2,2) Transmission by 4 of the 4 antenna groups: (1,1,2,1), (1,1,1,2), (2,1,1,1), (1,2,1,1). 6; Transmission by 3 of the 4 antenna groups: (2,2,2,0), (2,2,0,2), (2,0,2,2), (0,2,2,2) Transmission by 4 of the 4 antenna groups: (2,1,2,1), (1,2,1,2), (1,2,2,1), (2,1,1,2), (2,2,1,1), (1,1,2,2. 7; Transmission by 4 of the 4 antenna groups: (2,1,2,2), {(2,2,2,1), (1,2,2,2), (2,2,1,2). Agreement For NCB-based 8TX PUSCH transmission with NSRS>4, where NSRS is the number of configured single-port SRS resources in a resource set, - Support Option 2 where a legacy-based solution is used by extending the existing SRI indication tables to include NSRS=8 and lmax=8 Agreement To support UCI multiplexing on PUSCH for transmission with rank>4 by an 8TX UE, UCI is always multiplexed only on one of the scheduled CWs - Alt2: The CW with the higher MCS index (if MCS indices are the same, UCI is multiplex on the first CW) - Note: in case of PUSCH retransmission, the initial MCS is used for CW selection. Agreement For partially coherent 8TX precoding with Ng =2, the precoder is based on up to two full-coherent 4TX precoders. Down-select one of the following options for precoder indication, - Option 3 – Up to two 4TX TPMIs are indicated, - When two TMPIs are indicated, the first is applied on one of antenna group, and the second is applied on the other antenna group, - FFS : details of TPMI indication when one antenna group is used - Option 4 – A single 8TX TPMI is indicated - Other options are not precluded Agreement For codebook -based 8TX PUSCH transmission, down-select from, - Alt1 - A fully-coherent UE (Ng =1) can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =1, 2, 4, 8 - FFS which combinations of Ng value(s), to be considered - A partially-coherent UE, with Ng =2 can be configured with precoders considered for at least one or more Ng cases, i.e., Ng =2, 4, 8 - FFS which combinations of Ng value(s), to be considered - A partially-coherent UE, with Ng =4, can be configured with precoders considered for at least one or more Ng cases, i.e., Ng= 4, 8 - FFS which combinations of Ng value(s), if any, to be considered - A non-coherent UE, Ng =8, can only be configured with precoders considered for Ng = 8 - Alt2 - A fully-coherent UE (Ng =1) can only be configured with precoders considered for one of Ng cases, i.e., Ng =1, 2, 4, 8 - FFS which Ng value(s), to be considered - A partially-coherent UE , with Ng =2, can only be configured with precoders considered for one of Ng cases, i.e., Ng =2, 4, 8 - FFS which Ng value(s), to be considered - A partially-coherent UE , with Ng =4, can only be configured with precoders considered for one of Ng cases, i.e., Ng =4, 8 - FFS which Ng value(s), to be considered - A non-coherent UE , with Ng =8, can only be configured with precoders considered for Ng = 8 - FFS whether/how the configuration can be done, via RRC or MAC-CE. - Alt3 - A fully-coherent UE (Ng =1) can only be configured with precoders considered for Ng =1 - A partially-coherent UE, with Ng =2, can only use precoders considered for Ng =2 - A partially-coherent UE, with Ng =4, can only use precoders considered for Ng =4 - A non-coherent UE, with Ng =8, can only use precoders considered for Ng = 8 - Other alternatives are not precluded Note: For an 8TX UE , Ng =8 can represent a non-coherent UE. RAN1 Meeting #112 Agreement For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook, the following pairs of (N1, N2) values are supported, - (N1, N2) = (4, 1) - (N1, N2) = (2, 2)` A pair of (N1, N2) can be configured with subject to UE capability. Agreement Fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook - Precoding matrices generated according to (O1, O2) = (1, 1) is supported - Further study additional support of precoding matrices generated according to (O1, O2) where O1>1 or O2>1 - Subject to UE capability - FFS: Different O1, O2 values for different ranks Agreement To support dual CW PUSCH transmission for rank>4 by an 8TX UE, for MCS indication, support - Alt.2: A second MCS field (5 bits) is indicated for the second codeword Agreement To support dual CW PUSCH transmission for rank>4 by an 8TX UE, a second set of NDI (1 bit) and RV (2 bits) fields are indicated. - FFS: Details on how to signal Agreement To support dual CW PUSCH transmission for rank>4 by an 8TX UE, reuse DL PDSCH scrambling mechanism to initialize the scrambling sequence generator for codeword q∈{0,1}, cinit=nRNTI∙215+q∙214+nID where nRNTI, and nID are defined similar to the legacy single CW PUSCH transmission. Agreement For fully coherent uplink precoding by an 8TX UE, based on NR Rel-15 single panel DL Type I codebook (CodebookMode=1), - Study whether/how to support (O1, O2) = (2,1), (2,2) - whether for all rank, or rank 1-2, or rank 3-8 - applicability of different (O1, O2) values per agreed (N1, N2) - companies are encouraged to submit simulation results Agreement To support UCI multiplexing on PUSCH for transmission with rank>4 by an 8TX UE, UCI is always multiplexed only on one of the CWs, down-select from, - Alt1: First CW - Alt2: The CW with the highest MCS (if MCSs are the same, UCI is multiplex on the first CW) Agreement For non-coherent uplink precoding by an 8TX UE, following precoders are supported for 1 layer transmission. 10000000,01000000,00100000,00010000,00001000,00000100,00000010,00000001 with the scaling factor of 122. Agreement For NCB-based 8TX PUSCH transmission with NSRS>4, where NSRS is the number of configured single-port SRS resources in a resource set, - All SRS port combinations are supported - For SRI indication, down-select from, - Option 1: Use an NSRS bit length bitmap - Option 2: Use a legacy-based solution - Consideration of Lmax for SRI indication For NSRS≤4, Rel-15 SRI indication is reused Agreement For CB-based 8TX PUSCH transmission, where Mode 2 uplink full power transmission (if supported) is not used, re-use legacy Rel-15 mechanism, that is - when only one SRS resource in a resource set is configured, the SRI field in DCI is absent, - when two SRS resources are configured in a resource set, 1 bit of SRI field in DCI is used to indicate the selected SRS resource in the set. Agreement For partially coherent uplink precoding by an 8TX UE codebook, - When Ng=2 - Precoding design is based on Rel-15 UL 4TX codebook, - Full-coherent precoders are used - FFS whether partial-coherent precoders are needed - When Ng=4, down-select from, - Alt1: - Precoding design is based on Rel-15 UL 2TX codebook, - Full-coherent precoders are used - Alt2: - Precoding design is based on Rel-15 UL 4TX codebook, - Partial-coherent precoders are used Agreement For partially coherent uplink precoding by an 8TX UE codebook, Ng=2, - Following rank and layer splitting cases are supported Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 1; (1,0), (0,1). 8; (4,4). - Select from the following cases based on the performance and overall DCI overhead Rank; All layers in one Antenna Group; Layers split across 2 Antenna Groups. 2; (2,0), (0,2). 2; (1,1). 3; (3,0), (0,3). 3; (2,1), (1,2). 4; (4,0), (0,4). 4; (2,2), (3,1), (1,3). 5; (4,1), (1,4), (2,3), (3,2). 6; (4,2), (2,4), (3,3). 7; (4,3), (3,4). Note: Above is not relevant to how precoders are indicated. RAN1 Meeting #111 Agreement For a fully coherent uplink precoding by an 8TX UE, - Support NR Rel-15 single panel DL Type I codebook as the starting point for design of the codebook - FFS: For a constructed codebook with size M based on above method, unless M=2N; otherwise, round up the codebook size to the smallest integer 2N>M by adding 2N-M precoders generated via Alt 2a. - No LS to RAN4 will be needed Agreement For PUSCH transmission with rank>4 by an 8TX UE, to support dual CW transmission, - specify MCS, NDI, RV indication for the second CW - specify PUSCH Scrambling for the second CW - specify UCI multiplexing on PUSCH for dual CW transmission - study whether/how Enabling/Disabling the second CW FFS: Optimization of DCI to indicate the above Note: Strive to reuse Rel-15 NR DL schemes where possible. Agreement For PUSCH transmission with rank>4 by an 8TX UE, to support UCI multiplexing on PUSCH, down-select at least one of the following options in RAN1#112, - Option1: UCI is always multiplexed on one of the CWs - Option2: UCI is multiplexed on both CWs - Option3: Based on UCI (e.g., type, payload size, etc.) UCI is multiplexed on one or both CWs - Option4: UCI is multiplexed only when single CW is enabled - Option5: UCI is repeated across the two CWs - Other options are not precluded Agreement For CB-based 8TX PUSCH transmission, for rank indication, down-select among the following - Separate indication of TRI and TPMI - Joint indication of TRI and TPMI Agreement Study full TX power uplink codebook-based transmission by a partially/non-coherent 8TX precoder, - Reuse Rel-16 UE capability definitions for discussion purpose, i.e., UE Capability 1, 2 and 3 - For full TX power transmission by UE Capability 2/3, at least, following exemplary PA architectures can be considered - Other cases of interest are not precluded, down-select preferred potential architecture for the purpose of 8TX full power study in RAN#112. - This can be used for other UE Power Classes as well. 8TX UE, Power class 3 (23 dBm) Pi= Nominal power rating of each PA. Regular UE; P1=P2= …=P8=14 dBm (Full power supported by Mode1). Full-power capable UE; Full power capability with any PA comb. (CAP1) Example: P1=P2= …=P8= 23 dBm. Full power capability with 1 PA (CAP3) Example: P1=P2= …=P7= 14 dBm P8= 23 dBm. (lower priority) Full power capability with 2 PAs (CAP2) Example 2a: P1=P2= …=P6= 14 dBm, P7=P8 ≥ 20 dBm Example 2b: P1=P2= …= P8= 20 dBm. (lower priority) Full power capability with 4 PAs (CAP2) Example 3a: P1=P2= …=P4= 14 dBm, P5=P6= …=P8 ≥ 17 dBm Example 3b: P1=P2= …= P8 = 17 dBm. (lower priority) Full power capability with 6 PAs (CAP2) Example 4a: P1=P2= 14 dBm, P3=P4= …=P8 ≥ 15.3 dBm Example 4b: P1=P2= … = P8≥ 15.3 dBm. Agreement For an 8TX partial/non-coherent precoder, for study on full power codebook-based PUSCH transmissions, use Rel-16 full power modes as the starting point for the design. Note: This does not mandate support of all Rel-16 modes. RAN1 Meeting #110bis-e Agreement Support the following cases for codebook design for 8TX precoders - Full coherent precoders with Ng=1 - FFS: Full coherent precoders with Ng=2, Ng=4 - Partial coherent precoders with Ng=2 and Ng=4 - This does not imply any relation with the number of TPMI indications for 8TX precoder - Non-coherent precoders Agreement For codebook design of an 8TX partial-coherent UE, configured with an 8-port SRS resource - For when Ng=2, down-select of the following convention for assumption of port coherency scheme is used - Alt 1: two coherent groups of {0,2,4,6} and {1,3,5,7} - Alt 2: two coherent groups of {0,1,4,5} and {2,3,6,7} - Alt 3: two coherent groups of {0,1,2,3} and {4,5,6,7} - For when Ng=4, down-select of the following convention for assumption of port coherency scheme is used - Alt 1: four coherent groups of {0,4}, {1,5}, {2,6}, and {3,7} - Alt 2: four coherent groups of {0,1}, {2,3}, {4,5}, and {6,7} - Alt3: four coherent groups of {0, 2}, {4, 6}, {1, 3} and {5, 7} - Note: Other alternatives which are not foreseen are not precluded Agreement For SRI and/or transmitter precoder matrix indication for codebook-based uplink transmission by an 8TX UE, study - Whether/how to indicate one or multiple TPMI/SRI, according to the number of antenna groups, coherence capability, codebooksubset configuration, etc. - Whether/how to extend Rel-17 framework, e.g., TPMI/SRI indication in MTRP PUSCH - Whether/how to separate/joint indication of rank and precoding information. - Whether/how to indicate n (<=Ng) selected antenna group(s) separately from TPMI/TRI indication Agreement In Rel-18, on support of full power operation by a partial/non-coherent 8TX UE configured with codebook-based transmission, - Identify and agree on at least one potential PA architecture by RAN1 meeting #111 Agreement For 8TX UE codebook-based uplink transmission, - For partially/non-coherent precoding, support NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of codebook Agreement For SRS configuration required for non-codebook-based UL transmission by an 8TX UE, Alt1 is supported, that is - Alt1: A single SRS resource set configured with up to 8 single-port SRS resources - FFS: Configuration of up to two, or four SRS resource sets, each configured with up to 4, or 2 single-port SRS resources, respectively. Agreement For SRS configuration supporting codebook -based UL transmission for an 8TX UE , - Support configuration of 1 SRS resource set containing up to X 8-port SRS resource(s), where X = 2 - FFS : Other values for X, if needed - FFS : Configuration of at least one SRS resource set, configured with more than one SRS resources where each SRS resource may have the same or different number of SRS ports, e.g., for support full power operation, if supported - FFS : Configuration of at least one SRS resource set, configured with 8/M of M-port SRS resources, for example, - Configuration of an SRS resource set, configured with at least 4 of 2-port SRS resources - Configuration of an SRS resource set, configured with at least 2 of 4-port SRS resources Working Assumption For uplink transmission with rank>4, support dual CW transmission. Agreement If dual CW is supported for uplink transmission with Rank>4 by an 8TX UE, reuse DL Rel-15 codeword to layer mapping for both codebook-based and non-codebook-based transmission. Agreement For SRS configuration supporting codebook -based UL transmission for an 8TX UE , - Support configuration of 1 SRS resource set containing up to X 8-port SRS resource(s), where X = 2 - FFS : Other values for X, if needed - FFS : Configuration of at least one SRS resource set, configured with more than one SRS resources where each SRS resource may have the same or different number of SRS ports, e.g., for support full power operation, if supported - FFS : Configuration of at least one SRS resource set, configured with 8/M of M-port SRS resources, for example, - Configuration of an SRS resource set, configured with at least 4 of 2-port SRS resources - Configuration of an SRS resource set, configured with at least 2 of 4-port SRS resources RAN1 Meeting #110 Agreement 8TX PUSCH is supported in Rel-18 Agreement For 8TX PUSCH, at least support - Ng=1, 2, 4 Note: The above does not restrict the Ng for the non-coherent case Agreement For evaluation purpose of codebook alternatives when a precoder based on Rel-15 DL Type I is used, following oversampling ratios are assumed - (O1, O2) = (1,1), (2,1), (2,2) - Note: Other values may be used and reported by companies - Note: When deciding the supported O1, O2 combination, the signalling overhead, performance, UE complexity, etc should be considered Agreement RAN1 further studies Alt1b and Alt2a for down-selection of one of the two in RAN1 meeting #110b-e. - Transmission using one or multiple precoders corresponding to one or multiple SRS resources can be studied as part of the above alternatives. Agreement Support up to X layers for codebook and non-codebook UL transmission for 8TX UE where X=4, 8 is determined based on separate UE capability - For uplink transmission with rank<=4, single CW is supported - For uplink transmission with rank>4, whether single or dual CW is used will be decided in RAN1 meeting #110b-e The above applies only with regards to the work scope of this agenda item. Agreement For SRS configuration for non-codebook UL transmission for an 8TX UE, down-select from - Alt1: A single SRS resource set configured with up to 8 single-port SRS resources - Alt2: Up to two SRS resource sets, each configured with up to 4 single-port SRS resources - Alt3: Support both alternatives. Agreement Study low overhead solutions for SRI and/or transmitter precoder matrix indication for codebook-based, and SRI indication for non-codebook-based UL transmission by an 8TX UE, - FFS using single or separate (exiting or new) fields for the indication, other solutions are not precluded. - Note: Low overhead schemes for study include those using Rel-15 SRI/TPMI indication mechanisms RAN1 Meeting #109-e Agreement Study fully-coherent, partially-coherent and non-coherent UEs for uplink transmission with 8TX UEs. Agreement Study full power transmission for 8TX UEs. - Details are FFS upon completion of codebook design Agreement Adopt the following Table as the reference EVM for LLS evaluation - Companies may provide additional evaluation results per their case of interest - LLS is optionally used for 8Tx UL evaluation, if needed Parameter; Value. Carrier Frequency; 3.5 GHz. Waveform; CP-OFDM. SCS; 30 KHz. System bandwidth; 20 MHz, 100 MHz. Scheduled PRBs; 5, 25, 50, 260 PRBs. gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝); (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ. UE TX antenna configuration; To be defined according to outcome of Proposal 2.1. UE speed; 3 Km/h. Number of Layers; Adaptive, Fixed (reported by company). AMC; Adaptive, Fixed (reported by company). DMRS configuration; Type 1; 1 front loaded + 1 additional symbol. Channel estimation; Real. Channel Model; CDL-A (30ns), CDL-B (100ns), CDL-C (300ns). Agreement For 8TX UE uplink transmission, study codebook- and non-codebook-based transmission with maximal layer number of both 4 and 8 layers. Agreement Adopt the following Table as the reference EVM for SLS evaluation. - Companies may provide additional evaluation results per their case of interest. Parameter; Value. Frequency range; 3.5 GHz. Multiple access; OFDMA. Numerology; 14 OFDM symbol slot SCS , 30 KHz. Scenario; Outdoor FWA (38.901): UMa (ISD = 500 m), 100% Outdoor, 3Km/h. Indoor FWA (38.901): UMi (ISD = 200 m), 100% Indoor, 3Km/h. Industrial (38.901): Indoor Office (Inh ), 3Km/h. Channel model; 38.901. System bandwidth; 20 MHz, 100 MHz. gNB RX antenna setup and port layouts (𝑀,𝑁,𝑃,𝑀𝑔,𝑁𝑔,𝑀𝑝,𝑁𝑝); Outdoor FWA : (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Indoor FWA : (8,8,2,1,1,4,8) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 (4,4,2,1,1,4,4) with (𝑑H, 𝑑V) = (0.5, 0.8)𝜆 Industrial: (2,2,2,1,1,2,2) with (dH , dV ) = (0.5, 0.5)λ. gNB antenna radiation pattern parameters; Outdoor/Indoor FWA : 38.901 Table 7.3-1, 8 dBi , 65° HPBW Industrial: IMT.2412 Table 10,5 dBi , 90° HPBW. gNB receiver noise figure; 5dB. gNB receiver; MMSE-IRC. gNB scheduler; Single user with proportional fair. Modulation; - Up to 64 QAM - Up to 256QAM. MIMO scheme; SU-MIMO with rank adaptation. UE speed; 3 Km/h. UE TX antenna configuration; To be defined according to outcome of Proposal 2.1. Traffic model; - FTP model 1: Packet size 500KB, RU= 50% and suggested low/high RU of values of 20% and 70% - Full buffer (optional). Suggested benchmarking; R15 UL 4-Tx codebook , Eigen-based, companies report PRG assumption. Precoder granularity; Wideband. Power control; Open loop, - alpha = 0.8 - P0 = -50, -80 dBm to be selected according to the deployment scenario. UE power rating; 23 dBm (UE, 38.101) 32 dBm (FWA, 38.101). Metric; UL mean-user throughput, 5%-ile and 95%-ile UPT. Agreement For 8TX UE, consider the following UE antenna layouts for codebook design, · For non-coherent UEs, consider linear array (1D/2D) of cross-polarized or single-polarized antenna configuration · For fully/partial-coherent UEs, consider linear array (1D/2D) o Where the array is either cross-polarized antenna configuration or single polarized antenna configuration o Ng>=1 antenna groups can be considered where each group comprises coherent antennas, and across groups, antennas can be non-coherent/coherent depending on device types § An example of an antenna group is a panel o Within an antenna group, antenna elements are uniformly spaced. Across different antenna groups, companies to provide details. - Additional information for definition of antenna layout - Based on the number of coherent groups, following exemplary cases can be considered where, within each group, antenna elements are spaced by 0.5λ, and then dG-H, dG-V represent the horizontal and vertical spacings between the centers of adjacent antenna groups, respectively - Further down-selection can be done in the next meeting, if needed - The shown exemplary placing of antenna groups can be used for evaluation purpose, but the codebook design is not restricted to shown cases. - Other antenna layouts for other use cases are not precluded. - To start companies may report their results according to their preferred layout. Case; Ng; (M, N, P) per group; Antenna Layout; Antenna Pattern/Antenna Element Gain. 1; 1; (2, 2, 2), (1, 4, 2); Isotropic (Indoor/Outdoor FWA & Industrial) 8 dBi, 65° HPBW(Outdoor FWA). 2; 2; (1, 2, 2); Isotropic (Indoor/Outdoor FWA & Industrial) 8 dBi, 65° HPBW(Outdoor FWA). 3; 4; (1, 1, 2); Isotropic (Indoor/Outdoor FWA & Industrial) 4 dBi, 110° HPBW(Indoor FWA & Industrial. o Other UE antenna assumption for the purpose of evaluation Outdoor FWA; Indoor FWA; Industrial. UE antenna height; 6, 3 m (To start); According to 36.873; According to 38.901. Agreement For 8TX UE codebook-based uplink transmission, down-select one of - Alt1-a: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of the codebook for non-coherent UEs - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully/partially-coherent UEs - Alt1-b: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of the codebook for partially/non-coherent UEs - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully-coherent UEs - Alt2-a: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of codebook for fully/partially/non-coherent UEs - Alt2-b: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) in combination with those based on NR Rel-15 DL Type I codebooks as the starting point for design of codebook for fully/partially/non-coherent UEs - Alt3: - Study NR Rel-15 DL Type I codebook as the starting point for design of codebook for fully/partially/non-coherent UEs - Transmission using one or multiple precoders corresponding to one or multiple SRS resources can be studied as part of the above alternatives.

# References

- RP-213598, New WI: MIMO evolution for Downlink and Uplink, Samsung, 3GPP RAN Meeting #94e, Dec.6-17, 2021

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #112b-e, April 2023

- R1-2302310, Recommended Direction on SRI/TPMI Enhancements for RAN1#113, Moderator (InterDigital), 3GPP TSG RAN WG1 Meeting #112b, April 2023

- R1-2304398	SRI/TPMI enhancement for enabling 8 TX UL transmission, ZTE

- R1-2304427	Details on 8TX UE Operations, InterDigital, Inc.

- R1-2304469	Further discussion on enabling 8 TX UL transmission	vivo

- R1-2304548	Discussion on SRI/TPMI enhancement for enabling 8 TX UL transmission, Spreadtrum Comm.

- R1-2304642	Discussion on SRI/TPMI enhancement for enabling 8 TX UL transmission, Huawei, HiSilicon

- R1-2304711	SRI/TPMI enhancement for 8Tx UL transmission, CATT

- R1-2304840	On SRI/TPMI Indication for 8Tx Transmission, Google

- R1-2304879	Enhancements on 8Tx uplink transmission	xiaomi

- R1-2304956	SRI/TPMI enhancement for enabling 8TX UL transmission, Lenovo

- R1-2304998	Discussion on SRI/TPMI enhancement, NEC

- R1-2305030	Considerations on SRI/TPMI enhancement for enabling 8 TX UL transmission, Sony

- R1-2305083	Discussion on SRI/TPMI enhancement for enabling 8 TX UL transmission, CMCC

- R1-2305173	Discussion on enhancement for 8Tx UL transmission, Intel Corporation

- R1-2305232	Views on SRI/TPMI enhancement for enabling 8 TX UL transmission, Apple

- R1-2305294	SRI/TPMI enhancement for enabling 8 TX UL transmission, LG Electronics

- R1-2305324	Enhancements for 8 Tx UL transmissions, Qualcomm Incorporated

- R1-2305407	SRI TPMI enhancement for 8 TX UL transmission, OPPO

- R1-2305482	SRI/TPMI Enhancement for Enabling 8 TX UL Transmission, Ericsson

- R1-2305503	Views on TPMI/SRI enhancements for 8Tx UL transmission, Samsung

- R1-2305589	Discussion on 8 TX UL transmission, NTT DOCOMO, INC.

- R1-2305671	SRI/TPMI enhancement for enabling 8 Tx UL transmission, MediaTek Inc.

- R1-2305740	Views on 8 TX UL transmission, Sharp

- R1-2305754	UL enhancements for enabling 8Tx UL transmission,	Nokia, Nokia Shanghai Bell

- R1-2305779	Discussion on SRI/TPMI Enhancements for 8 TX UL Transmission, FGI

- R1-2305893	SRI/TPMI enhancement for enabling 8 TX UL transmission, CEWiT

- R1-2305906	Discussion on SRI/TPMI enhancement for enabling 8 TX UL transmission, KDDI Corporation