3GPP TSG-RAN WG1 Meeting #113		R1-2304457

Incheon, Korea, May 22nd – May 26th, 2023

Source:	vivo

Title:	Discussion on Rel-18 Tx switching across 3/4 bands

Agenda Item:	5

Document for:	Discussion and Decision

- Introduction

RAN4 sent an LS to RAN1 regarding Rel-18 Tx switching across 3/4 bands in [1] and asked the following question to RAN1.

RAN4 would like to ask RAN1 one question: In sub-clause 6.1.6 of TS 38.214, it is specified that: If an uplink switching is triggered for an uplink transmission starting at T0, after T0-Toffset, the UE is not expected to cancel the uplink switching, or to trigger any other new uplink switching occurring before T0 for any other uplink transmission that is scheduled after T0-Toffset, where Toffset is the UE processing procedure time defined for the uplink transmission triggering the switch given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213]. On the definition and understanding of T0: - When gNB provides sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier, in RAN4 understanding, the T0 is the starting time for an uplink transmission on the switched-to carrier from network scheduling perspective and the starting time of actual uplink transmission from UE perspective. - When gNB does not provide sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier, and the switching period is located at the switch-to carrier according to the RRC signalling uplinkTxSwitchingPeriodLocation, UE may omit uplink transmission on certain symbol(s) on the switch-to carrier. In this case, is T0 the starting time of uplink transmission from network scheduling perspective, or the starting time of actual uplink transmission from UE perspective?

In this contribution, we discuss on potential RAN1 reply to the above question.

- Discussion

In Rel-16/17 Tx switching, the PUSCH preparation time Tproc,2 guarantees a sufficient gap between the UL grant and the start of the corresponding UL transmission so that UE has sufficient time to prepare PUSCH and execute Tx switching. If a UL transmission starting from T0 may initiate Tx switching, then T0-Toffset (where Toffset is equal to the UL processing procedure time Tproc,2) serves as the deadline for modifying the initiated Tx switching. Any DCIs after the deadline for canceling or changing the Tx switching would be considered an error case because the UE would not be able to complete the corresponding changes.

In our understanding, the procedure is executed as below:

- Step1. When a UE receives a UL grant scheduling a UL transmission starting from T0, the UE checks if it may trigger Tx switching. If Tx switching is needed for the scheduled UL transmission, go to step2.

- Step2. The UE checks all DCIs received before T0-Toffset to determine whether and how to perform Tx switching. If UE receives a DCI changing the Tx switching before the deadline, UE updates T0 and Toffset correspondingly. If the Tx switching is not canceled or changed after checking the DCIs, go to step3.

- Step3. The UE determines the location of the switching period and performs Tx switching.

It can be noted that T0-Toffset is used to determine the deadline before determining whether and how to perform Tx switching. Thus, in Rel-16 Tx switching, T0 is always the starting time for uplink transmission on the switched-to carrier from a network scheduling perspective, regardless of whether there is a sufficient time period between UL grant and the UL transmission, and regardless of whether the UL transmission is interrupted by the switching period or not. If the network does not provide a sufficient time gap, UE may drop part of the UL transmission until the completion of the Tx switching. In this case, the actual starting time of PUSCH transmission from the UE perspective can be later than T0.

It is also worth noting that in Rel-16/17 Tx switching, only single Tx switching instance involving two bands is supported. T0 is always the start of the UL transmission after a single Tx switching instance.

Observation 1. In Rel-16/17 Tx switching, T0 is defined per single Tx switching instance, and T0 is the starting time for uplink transmission on the switched-to carrier from a network scheduling perspective regardless of whether there is a sufficient time period between the UL grant and the UL transmission and regardless of whether the UL transmission would be interrupted by the switching period or not.

When it comes to Rel-18 Tx switching, the same principle should be reused as much as possible, i.e., T0 should be defined per single Tx switching instance and T0-Toffset is the deadline for DCI canceling or changing a single Tx switching instance for UL transmission(s). However, as the Rel-18 Tx switching may involve two band pairs, some further clarifications on T0 and Toffset are needed due to ambiguity on one switching and two switching.

Proposal 1. In Rel-18 Tx switching, T0 should be defined per single Tx switching instance, T0-Toffset is the deadline for DCI cancelling or changing a single Tx switching instance for scheduled PUSCH transmission(s).

At the previous RAN1 meeting, the following proposal was discussed in [2].

- When a UE is triggered to perform TX switching between a band pair, and the start of the UL transmission after TX switching is T0, UE uses grants received before T0-Toffset to determine how to perform switching, where Toffset is the UE processing procedure time defined for the uplink transmission triggering. - To determine the Toffset which is composed of N2 and Tswitch, the minimum SCS among the downlink carriers where DCI triggers the UL transmission for Tx switching is used as µDL and the minimum SCS among the UL carriers after Tx switching is used as µUL to determine N2, additionally, the minimum SCS among the UL carriers involved in Tx switching is used as µUL to determine Tswitch - If the two Tx chains are triggered to switch between two different band pairs (e.g., band A + band C->band B + band D), when the two UL transmissions after TX switching are at least partially overlapped in time domain, UE perform it as one TX switching involving more than 2 bands, otherwise as twice of TX switching. - For UE having capability to use Tx chain for transmission even during the switching performed at another Tx chain, if the two Tx chains are triggred to switch between two different band pairs (e.g., band A + band C->band B + band D), when there is time domain overlapping between two bands Tx switching period, UE perform it as one TX switching involving more than 2 bands, otherwise as twice of TX switching.

First, for Rel-18 Tx switching involving only two bands, the same principle can be directly reused. The following switching cases for Rel-18 Tx switching between two bands (e.g., switch from Band A to Band B) are considered:

- Case1-1. Band A 1P or 2P-> Band B 2P, where the UE is under an operation state of supporting 2port transmission on Band A

- Case1-2. Band A 1P or 2P -> Band B 1P, where Band B has no associated band, or the UE does not support dual UL for Band B and any other bands in the reported band combination.

In other words, for Rel-18 Tx switching involving only two bands, T0 is the starting time for uplink transmission on the switched-to carrier from a network scheduling perspective regardless of whether there is a sufficient time between UL grant and the UL transmission, and regardless of whether the switching period is located on the switched-to carrier or not according to the RRC configuration.

Figure 1. Example of single Tx switching involving 2 bands in Rel-18

Secondly, for Rel-18 Tx switching for two band pairs, according to our companion paper [3], how to perform Tx switching should be clarified, otherwise the time location for UL interruption as well as time location for DL interruption is ambiguous.

For Case2 where Band A 1P+Band B 1P -> Band C 1P or 2P and the UE is expected to transfer into an operation state of supporting 2port transmission on Band C, single Tx switching instance is always performed, and T0 is the starting time of the UL transmission on Band C.

When two overlapped UL transmissions are to be transmitted on different bands and when the two UL transmissions trigger switching of two TX chains for two different band pairs, UE should perform it as single Tx switching as illustrated in Figure 2, otherwise, the later UL transmission will be interrupted by the first Tx switching.

The following switching cases for switching between two band pairs are considered:

- Case3-1. Band A 1P or 2P-> Band B 1P+Band C 1P, where the UE is under an operation state of supporting 2port transmission on Band A

- Case3-2. Band A 1P+Band C 1P -> Band B 1P+Band D 1P.

Figure 2. Example of single Tx switching involving more than 2 bands in Rel-18

Therefore, proposal 5-1 in [2] should be agreed first to have a clear picture of Tx switching, and it needs to be revised as below to clarify the T0 for single Tx switching involving 3 or 4 bands. Additionally, as more than two bands are involved in the Tx switching, the minimum UL SCS and minimum DL SCS to derive the Toffset should be clarified as well.

Proposal 2. Clarify the condition of a single Tx switching instance that, when a UE is triggered to perform Tx switching between a band pair, and the start of the UL transmission after Tx switching is T0, the UE uses the grants received before T0-Toffset to determine how to perform switching, where Toffset is the UE processing procedure time defined for the uplink transmission triggering.
-  To determine the Toffset which is composed of N2 and Tswitch, the minimum SCS among the downlink carriers where DCI triggers the UL transmission for Tx switching is used as µDL and the minimum SCS among the UL carriers after Tx switching is used as µUL to determine N2, additionally, the minimum SCS among the UL carriers involved in Tx switching is used as µUL to determine Tswitch
-  If the two Tx chains are triggered to switch between two different band pairs (e.g., band A + band C->band B + band D), when the two UL transmissions after Tx switching are at least partially overlapped in the time domain, the UE performs it as one Tx switching instance involving more than 2 bands, otherwise as twice of Tx switching.
  -  For one Tx switching instance involving more than 2 bands, T0 is the starting time of the earlier transmission of the two UL after Tx switching

Based on the above discussion, the draft reply LS is as below:

Proposal 3. Reply to RAN4 that, in Rel-18 Tx switching, T0 is used to determine the deadline (i.e., T0-Toffset) for DCI canceling or changing a Tx switching that can be triggered by UL transmission(s), and T0 is defined as below:
-  Case1. For Tx switching involving only two bands (e.g., Band A ->Band B), T0 is the starting time for an uplink transmission on the switched-to carrier from a network scheduling perspective regardless of whether there is a sufficient time period between UL grant and the UL transmission, and regardless of whether the switching period is located on the switched-to carrier or not according to the RRC configuration.
-  Case2. For Tx switching involving two switched-from bands and one switched-to band (e.g., Band A + Band B->Band C), T0 is the starting time for an uplink transmission on the switched-to carrier from a network scheduling perspective regardless of whether there is a sufficient time period between UL grant and the UL transmission, and regardless of whether the switching period is located on the switched-to carrier or not according to the RRC configuration.
-  Case3. For Tx switching involving the two Tx chains to switch between two different band pairs (e.g., Band A + Band C->Band B + Band D, or Band A-> Band B + Band C), T0 is the starting time for earlier uplink transmission on the two switched-to carriers from network scheduling perspective regardless of whether there is a sufficient time period between UL grants and the UL transmissions, and regardless of whether the switching period is located on the switched-to carriers or not according to the RRC configuration(i.e., BandPriority).
  -  Note that if the two Tx chains are triggered to switch between two different band pairs, and when the two UL transmissions after Tx switching are at least partially overlapped in the time domain, the UE performs it as one Tx switching instance involving more than 2 bands.

- Conclusion

In this contribution, we have the following observations and proposals, and a draft LS reply can be found in [4]:

Observation 1. In Rel-16/17 Tx switching, T0 is defined per single Tx switching instance, and T0 is the starting time for uplink transmission on the switched-to carrier from a network scheduling perspective regardless of whether there is a sufficient time period between the UL grant and the UL transmission and regardless of whether the UL transmission would be interrupted by the switching period or not.

Proposal 1. In Rel-18 Tx switching, T0 should be defined per single Tx switching instance, T0-Toffset is the deadline for DCI cancelling or changing a single Tx switching instance for scheduled PUSCH transmission(s).

Proposal 2. Clarify the condition of a single Tx switching instance that, when a UE is triggered to perform Tx switching between a band pair, and the start of the UL transmission after Tx switching is T0, the UE uses the grants received before T0-Toffset to determine how to perform switching, where Toffset is the UE processing procedure time defined for the uplink transmission triggering.
-  To determine the Toffset which is composed of N2 and Tswitch, the minimum SCS among the downlink carriers where DCI triggers the UL transmission for Tx switching is used as µDL and the minimum SCS among the UL carriers after Tx switching is used as µUL to determine N2, additionally, the minimum SCS among the UL carriers involved in Tx switching is used as µUL to determine Tswitch
-  If the two Tx chains are triggered to switch between two different band pairs (e.g., band A + band C->band B + band D), when the two UL transmissions after Tx switching are at least partially overlapped in the time domain, the UE performs it as one Tx switching instance involving more than 2 bands, otherwise as twice of Tx switching.
  -  For one Tx switching instance involving more than 2 bands, T0 is the starting time of the earlier transmission of the two UL after Tx switching

Proposal 3. Reply to RAN4 that, in Rel-18 Tx switching, T0 is used to determine the deadline (i.e., T0-Toffset) for DCI canceling or changing a Tx switching that can be triggered by UL transmission(s), and T0 is defined as below:
-  Case1. For Tx switching involving only two bands (e.g., Band A ->Band B), T0 is the starting time for an uplink transmission on the switched-to carrier from a network scheduling perspective regardless of whether there is a sufficient time period between UL grant and the UL transmission, and regardless of whether the switching period is located on the switched-to carrier or not according to the RRC configuration.
-  Case2. For Tx switching involving two switched-from bands and one switched-to band (e.g., Band A + Band B->Band C), T0 is the starting time for an uplink transmission on the switched-to carrier from a network scheduling perspective regardless of whether there is a sufficient time period between UL grant and the UL transmission, and regardless of whether the switching period is located on the switched-to carrier or not according to the RRC configuration.
-  Case3. For Tx switching involving the two Tx chains to switch between two different band pairs (e.g., Band A + Band C->Band B + Band D, or Band A-> Band B + Band C), T0 is the starting time for earlier uplink transmission on the two switched-to carriers from network scheduling perspective regardless of whether there is a sufficient time period between UL grants and the UL transmissions, and regardless of whether the switching period is located on the switched-to carriers or not according to the RRC configuration(i.e., BandPriority).
  -  Note that if the two Tx chains are triggered to switch between two different band pairs, and when the two UL transmissions after Tx switching are at least partially overlapped in the time domain, the UE performs it as one Tx switching instance involving more than 2 bands.

- Reference

- R1-2304313, LS on Rel-18 Tx switching across 3/4 bands (ChinaTelecomm)

- R1-2302221, Summary#3 of discussion on multi-carrier UL Tx switching scheme (NTT DOCOMO, INC.)

- R1-2304517, Discussion on Rel-18 Multi-carrier enhancements, eDSS and NCR (vivo)

- R1-2304456, Draft LS reply on Rel-18 Tx switching across 3/4 bands (vivo)