3GPP TSG RAN WG1 #113			R1-2304691

Incheon, Korea, May 22nd – May 26th, 2023

Source:	CATT

Title:	Discussion on RAN2 LS on report of switching periods in Rel-18 UL Tx switching

Agenda Item:	5

Document for:	Discussion and Decision

- Introduction

In [1], the issue on report of switching periods in Rel-18 UL Tx switching was discussed by RAN2, and one question was asked to RAN1 as follows:

RAN2 could not conclude whether the UE needs to explicitly report if it supports 2Tx-2Tx switching for every band pair used for Rel-18 UL Tx switching. RAN2 is not sure which is the correct understanding: - The UE always supports 2Tx-2Tx switching on a pair of bands if the UE supports 2 layers/ports UL MIMO on the two bands - The UE may not support 2Tx-2Tx switching on a pair of bands even if the UE supports 2 layers/ports UL MIMO on the two bands (i.e., per-band-pair UE capability to report whether to support 2Tx-2Tx switching is needed, e.g. based on the presence/absence of 2Tx-2Tx switching period).

In this document, the understanding of 2Tx-2Tx switching on a pair of bands is discussed, and one proposal is provided to answer the question of RAN2.

- Discussion

In RAN1#104b [2], it was discussed Rel-17 1Tx-2Tx switching states between two different bands and reached the following agreements as follows.

Agreements (RAN1#104b): For Rel-17 1Tx-2Tx switching between 1 carrier on Band A and 2 contiguous carriers on Band B, the mapping between UL transmission ports and Tx chain for SUL and UL CA Option 1 is defined as follows. Number of Tx chains in WID (band A + band B); Number of antenna ports for UL transmission (band A (carrier 1) + band B (carrier 2 + carrier 3)). Case 1; 1T+1T; 1P+(0P+0P). Case 2; 0T+2T; 0P+(2P+0P), 0P+(0P+2P), 0P+(2P+2P), 0P+(1P+0P), 0P+(0P+1P), 0P+(1P+1P), 0P+(1P+2P), 0P+(2P+1P).

For Rel-17 1Tx-2Tx switching state between two different bands, only one band (e.g. band B) supports 2 layers/ports UL transmission, the other band (e.g. band A)  does not support 2 layers/port transmission UL transmission and only supports 1 layer/ port. The switching gap NTx1-Tx2 is used for the Tx switching between case 1 and case 2 in above table.

Observation 1: For Rel-17 1Tx-2Tx switching states, only one band support supports 2 layers/ports UL transmission.

Moreover, Rel-17 2Tx-2Tx switching states between two different bands were also discussed. The corresponding proposal is shown as follows.

Agreements:(RAN1#104b): For Rel-17 2Tx-2Tx switching between two uplink carriers, the mapping between UL transmission ports and Tx chain for SUL and UL CA option 1 is defined as follows. Number of Tx chains in WID (carrier 1 + carrier 2); Number of antenna ports for UL transmission (carrier 1 + carrier 2). Case 2; 0T+2T; 0P+2P, 0P+1P. Case 3; 2T+0T; 2P+0P, 1P+0P. Agreements: For Rel-17 2Tx-2Tx switching between two uplink carriers, the mapping between UL transmission ports and Tx chain for UL CA option 2 is defined as follows. Number of Tx chains in WID (carrier 1 + carrier 2); Number of antenna ports for UL transmission (carrier 1 + carrier 2). Case 1; 1T+1T; 1P+0P, 1P+1P, 0P+1P. Case 2; 0T+2T; 0P+2P, 0P+1P. Case 3; 2T+0T; 2P+0P, 1P+0P.

For Rel-17 2Tx-2Tx switching between two different bands, both two bands (e.g. band B and band B) support 2 layers/ports UL transmission, and Tx switching is allowed between any two different cases. It means that the UE always supports 2Tx-2Tx switching on a pair of bands if the UE supports 2 layers/ports UL MIMO on the two bands. It is noted that the switching gap NTx2-Tx2 is used for the following three switching cases:

- UL Tx switching between case 1 and case 2

- UL Tx switching between case 1 and case 3

- UL Tx switching between case 2 and case 3

Observation 2: UE always supports 2Tx-2Tx switching on a pair of bands if the UE supports 2 layers/ports UL MIMO on the both of two bands for Rel-17.

For the RAN2’s question, the followings reply is suggested

- UE always supports 2Tx-2Tx switching on a pair of bands if the UE supports 2 layers/ports UL MIMO on the two bands.

Proposal 1: For the RAN2’s question, the followings reply is suggested

- UE always supports 2Tx-2Tx switching on a pair of bands if the UE supports 2 layers/ports UL MIMO on the two bands.

- Conclusion

In this document, the understanding of 2Tx-2Tx switching on a pair of bands is discussed, and one proposal is provided as followings.

Observation 1: For Rel-17 1Tx-2Tx switching states, only one band support supports 2 layers/ports UL transmission.

Observation 2: UE always supports 2Tx-2Tx switching on a pair of bands if the UE supports 2 layers/ports UL MIMO on both of two bands for Rel-17.

Proposal 1: For the RAN2’s question, the followings reply is suggested

- UE always supports 2Tx-2Tx switching on a pair of bands if the UE supports 2 layers/ports UL MIMO on the two bands.

- References

R1-2304333 LS on report of switching periods in Rel-18 UL Tx switching RAN2, NTT DOCOMO

R1-2104151 Final Report of 3GPP TSG RAN WG1 #104bis-e v1.0.0 (Online meeting, 12th - 20th April 2021)