3GPP TSG RAN WG1 #113											     	 	        R1-2304759

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.14.1

Source:	Thales

Title:	Consideration on IMT-2020 Satellite self-evaluation

Document for:	Discussion and decision

# Introduction

As per the approved SID  [1, RP-230736] on IMT-2020 satellite radio interface evaluation, it is expected that 3GPP will be actively contributing to the satellite component of IMT-2020 radio interface(s) development and make the submission to ITU-R WP 4B at the appropriate time. Therefore, it is necessary for 3GPP to complete the description and compliance templates and conduct a self-evaluation to complete the submission and evaluation process for IMT-2020.

Detailed objectives of this study item include:

- Complete all required submission templates as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc] - Provide self-evaluation results against technical performance requirements for eMBB-s as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2], including - Peak data rate - Peak spectral efficiency - User experienced data rate - 5th percentile user spectral efficiency - Average spectral efficiency - Area traffic capacity - Latency, including user plane latency and control plane latency - Energy efficiency, including both network and device - Mobility - Mobility interruption time - Provide self-evaluation results against technical performance requirements for mMTC-s as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2], including - Connection density - Provide self-evaluation results against technical performance requirements for HRC-s as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2], including - Reliability - Provide self-evaluation results for other requirements (including bandwidth) as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2, RAN4] This study shall start with evaluating features that are supported by Rel-17 NTN (NR NTN + IoT NTN), as relevant for the above aspects.

In this contribution we discuss the methodology from RAN1 perspective for the evaluation of control plane latency, and user plane latency in NR NTN.

# Defining evaluation assumptions

The following table is used in RAN1#112bis-e as the starting point for defining evaluation assumptions:

| Reference number | Characteristic for evaluation | High-level assessment method | Requirement description in ITU-R M.2514 | Usage Scenario | Needed assumptions |
| --- | --- | --- | --- | --- | --- |
| #1 | Peak data rate | Analytical | § 7.2.1 | eMBB-s | Yes (modulation, #layers, etc) (NOTE 1) |
| #2 | Peak spectral efficiency | Analytical | § 7.2.2 | eMBB-s | Yes (modulation, #layers, etc) (NOTE 1) |
| #3 | User experienced data rate | Simulation and Analytical | § 7.2.3 | eMBB-s | Derived from #4 |
| #4 | 5th percentile spectral efficiency | Simulation | § 7.2.4 | eMBB-s | Yes |
| #5 | Average spectral efficiency | Simulation | § 7.2.5 | eMBB-s | Yes |
| #6 | Area traffic capacity | Simulation and Analytical | § 7.2.6 | eMBB-s | Derived from #5 (May need discussion on how to compute the area) |
| #7 | User plane latency | Analytical and Inspection | § 7.2.7.1 | eMBB-s | NOTE 2 |
| #8 | Control plane latency | Analytical and Inspection | § 7.2.7.2 | eMBB-s | NOTE 2 |
| #9 | Connection density | Simulation | § 7.2.8 | mMTC-s | Yes |
| #10 | Energy efficiency | Inspection | § 7.2.9 | eMBB-s | No |
| #11 | Reliability | Simulation | § 7.2.10 | HRC-s | Yes |
| #12 | Mobility | Simulation | § 7.2.11 | eMBB-s | Yes |
| #13 | Mobility interruption time | Analytical | § 7.2.12 | eMBB-s | NOTE 2 |
| #14 | Bandwidth | Inspection | § 7.2.13 | N/A | No |
| NOTE 1: How to determine the appropriate parameters (MCS, bandwidth, etc.) may be subject to evaluations. NOTE 2: To be evaluated by RAN2. RAN2 may need to develop assumptions for these metrics. RAN1 can provide input on aspects such as UE and gNB processing time. |  |  |  |  |  |

Table 1: IMT-2020 self-evaluation assumptions

Based on the above table, the following technical performance requirements should be evaluated in RAN2:

- Control plane latency

- User plane latency

- Mobility interruption time

However, in our view RAN1 should provide several inputs as will be discussed in the following sections:

# Control plane latency

The CP latency is defined in [2] as follows:

Control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state). Control plane latency should be equal to or less than 40 ms. This requirement is defined for the eMBB-s usage scenario and the above target applies to handheld devices. The proponent should provide the elements and their values in the calculation of the control plane latency. Table 2 in §7.2.5 of Report ITU-R M.2412 provides an example of the elements in the calculation of the control plane latency that should be followed with needed adaptions to e.g. include the assumed time-of-flight between the terminal and the base station. In addition to compliance with the above requirement, the proponent may provide indication that the Satellite Radio Interface is able to support larger latencies, e.g. up to 1.15 s, and may operate with a range of relevant satellite orbits.

Observation 1:  Feeder and service links propagation delays are not included in the time budget to satisfy the 40 ms control plane latency requirement.

For the analysis of NR CP latency, the following control plane procedure used for NR can be also used for NR NTN:

Figure 1: Control Plane procedure latency

By considering the different components of CP procedure in the above diagram, we have the following proposal:

Proposal 1: The Table 2 is used for NR NTN Control plane latency analysis

| Step | Description | CP Latency for UL data transfer [ms] |
| --- | --- | --- |
| 1 | Delay due to RACH scheduling period | 0 |
| 2 | Transmission of RACH Preamble (see NOTE 3) | Length of the preamble according to the PRACH format as specified in [38.211] |
| 3 | Preamble detection and processing in gNB | Tproc,2 (assuming d2,1=0) as starting point, note: The value of Tproc,2 is used only for evaluation. gNB processing delay may vary depending on implementation. |
| 4 | Transmission of RA response | Ts (the length of 1 slot ) NOTE1: the length of 1 slot or 1 non-slot include PDCCH and PDSCH (the first OFDM symbol of PDSCH is frequency multiplexed with PDCCH). |
| 5 | UE Processing Delay (decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Connection Resume Request) | NT,1+NT,2+0.5 ms |
| 6 | Transmission of RRC Resume Request | Ts (the length of 1 slot ) NOTE1: the length of 1 slot or 1 non-slot is equal to PUSCH allocation length. |
| 7 | Processing delay in gNB (L2 and RRC) | 3 |
| 8 | Transmission of RRC Resume | Ts (the length of 1 slot / non-slot) NOTE1 and NOTE 2 in Step 4 apply |
| 9 | Processing delay in UE of RRC Resume including grant reception | 7 |
| 10 | Transmission of RRC Resume Complete and UP data | 0 |
|  | Total delay [ms] |  |

Table 2: Control plane procedure latency

We made the following notes on the different steps/inputs in Table 2:

- Transmission delay includes service link propagation delay, feeder link propagation delay and satellite processing delay but only the satellite processing delay is including in the time budget as explained in Observation 1.

- For step 1, the procedure for transition from a most “battery efficient” state haas yet not begun, hence this step is not relevant for the latency of the procedure which is illustrated by a '0' in the above.

- For step 5, the latency of NT,1+NT,2+0.5ms is used according to Section 8.3 of TS 38.213. NT,1 is a time duration of N1 symbols corresponding to a PDSCH reception time for PDSCH processing capability 1 or 2 when additional PDSCH DM-RS is configured; and NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for PUSCH processing capability 1 or 2. The value of N1 and N2 are shown in Table 5.3-1 and Table 6.4-1 of TS38.214, respectively..

- For step 7, the processing delay in gNB (L2 and RRC) has been reduced to 3 ms. The delays due to inside-gNB or inter-gNB communication are not included in Step 7. Such delays may exist depending on deployment, but are not within the scope of this evaluation.

- For step 9 for UL data transfer, the processing delay in the UE (L2 and RRC) is considered, i.e., from reception of RRC Connection Resume to the reception of UL grant. The transmission of UL grant by gNB and processing delay in the UE (processing of UL grant and preparing for UL tx) are also considered. The RRCConnectionResume message only includes MAC and PHY configuration. No DRX, SPS, CA, or MIMO re-configuration will be triggered by this message. Further, the UL grant for transmission of RRC Connection Resume Complete and the data is transmitted over common search space with DCI format 0.

- For step 9 for DL data transfer, only the processing delay in the UE (L2 and RRC) is considered, i.e., from reception of RRC Connection Resume to the reception of DL grant. The RRCConnectionResume message only includes MAC and PHY configuration. No DRX, SPS, CA, or MIMO re-configuration will be triggered by this message. Further, the UL grant for transmission of RRC Connection Resume Complete and the data is transmitted over common search space with DCI format 0.

- For step 10, the beginning of this subframe is considered to be "the start of continuous data transfer", hence this step is not relevant for the latency of the procedure which is illustrated by a '0' in the above.

Proposal 2:   Send LS to RAN2 to provide RAN1 inputs on Control plane procedure latency

# User plane latency

The UP latency is defined in [2] as follows:

User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state. User plane latency should be equal to or less than 10 ms, for both downlink and uplink, assuming unloaded conditions (i.e. a single user) and small IP packets (e.g. 0 byte payload + IP header). This requirement is defined for the eMBB-s usage scenario and the above target applies to handheld devices. The proponent should provide the elements and their values in the calculation of the user plane latency, for both UL and DL. Table 3 in § 7.2.6 of Report ITU-R M.2412 provides an example of the elements in the calculation of the user plane latency that should be followed with needed adaptions to e.g. include the assumed time-of-flight between the terminal and the base station. In addition to compliance with the above requirement, the proponent may provide indication that the Satellite Radio Interface is able to support larger latencies, e.g. up to 650 ms, and may operate with a range of relevant satellite orbits.

Observation 2: Feeder and service links propagation delay are not included in the time budget to satisfy the 10 ms user plane latency requirement.

The evaluation of NR NTN user plane latency in Downlink (DL) and Uplink (UL) is based on the procedures illustrated in the following Figures:

Figure 2a: Downlink (DL) user plane (UP) data transfer procedure latency

Figure 2b: Uplink (UL) UP data transfer procedure latency

The detailed assumptions of each step are provided in Table 3 and Table 4 for downlink and uplink, respectively:

Proposal 3: The components in Table 3 are considered for DL user plane procedure latency analysis for NR NTN with HARQ enabled.

FFS: HARQ is disabled and radio retransmission handled by RLC ARQ.

| ID | Component |  | Notations | Value |
| --- | --- | --- | --- | --- |
| 1 | DL data transfer |  | T1 = (tBS,tx + tFA,DL) + tDL_duration + tUE,rx |  |
| 1.1 | BS processing delay |  | tBS,tx The time interval between the data is arrived, and packet is generated. | Tproc,2/2, with d2,1= d2,2= d2,3=0. (Tproc,2 is defined in Section 6.4 of TS38.214) (NOTE1) |
| 1.2 | DL Frame alignment (transmission alignment) |  | tFA,DL It includes frame alignment time, and the waiting time for next available DL slot | TFA + Twait, TFA is the frame alignment time within the current DL slot; Twait is the waiting time for next available DL slot if the current slot is not DL slot. |
| 1.3 | DL data packet transmission |  | tDL_duration | Length of one slot (14 OFDM symbol length) or non-slot (2/4/7 OFDM symbol length), depending on slot or non-slot selected in evaluation. |
| 1.4 | UE processing delay |  | tUE,rx The time interval between the PDSCH is received and the data is decoded; | Tproc,1/2 (Tproc,1 is defined in Section 5.3 of TS38.214), d1,1=0; d1,2 should be selected according to resource mapping type and UE capability. N1=the value with “No additional PDSCH DM-RS configured”. |
| 2 | HARQ retransmission |  | THARQ = T1 + T2 T2 = (tUE,tx + tFA,UL) + tUL_duration + tBS,rx (For Steps 2.1 to 2.4) |  |
| 2.1 | UE processing delay |  | tUE,tx The time interval between the data is decoded, and ACK/NACK packet is generated. | Tproc,1/2 (Tproc,1 is defined in Section 5.3 of TS38.214), d1,1=0; d1,2 should be selected according to resource mapping type and UE capability. N1=the value with “No additional PDSCH DM-RS configured”. (NOTE2) |
| 2.2 | UL frame alignment (transmission alignment) |  | tFA,UL It includes frame alignment time, and the waiting time for the next available UL slot | TFA + Twait, TFA is the frame alignment time within the current UL slot; Twait is the waiting time for next available UL slot if the current slot is not UL slot |
| 2.3 | HARQ ACK/NACK transmission |  | tUL_duration | 1 OFDM symbol |
| 2.4 | BS processing delay |  | tBS,rx The time interval between the ACK is received and the ACK is decoded. | Tproc, 2/2 with d2,1= d2,2= d2,3=0. (NOTE3) |
| 2.5 | Repeat DL data transfer from 1.1 to 1.4 |  | T1 |  |
| - | Total one way user plane latency for DL [ms] | LEO600 | Twait is the waiting time for next available UL slot if the current slot is not UL slot |  |
|  |  | GEO | 1 OFDM symbol |  |
| NOTE: Transmission delay includes service link propagation delay, feeder link propagation delay and satellite processing delay but only the satellite processing delay is including in the time budget as explained in Observation 2. |  |  |  |  |

Table 3: User plane DL procedure latency

1. The value is used for evaluation only; gNB processing delay may vary depending on implementation.

2. For the above case, the value of this step is Tproc,1(  =15kHz) - Tproc,1(  =30kHz)/2.

3. For the above case, the value of this step is Tproc,2(  =15kHz)/2.

Proposal 4: The components in Table 4 are considered for UL user plane procedure latency analysis for NR NTN. FFS HARQ is disabled and radio retransmission handled by RLC ARQ.

| Step | Component |  | Notations | Value |
| --- | --- | --- | --- | --- |
| 1 | UL data transfer |  | T1 = (tUE,tx + tFA,UL) + tUL_duration + tBS,rx |  |
| 1.1 | UE processing delay |  | tUE,tx The time interval between the data is arrived, and packet is generated; | Tproc,2/2 (Tproc,2 is defined in Section 6.4 of TS38.214), with d2,1= d2,2= d2,3=0 |
| 1.2 | UL Frame alignment (transmission alignment) |  | tFA,UL It includes frame alignment time, and the waiting time for next available UL slot | TFA + Twait, TFA is the frame alignment time within the current UL slot, Twait is the waiting time for next available UL slot if the current slot is not UL slot. |
| 1.3 | UL data packet transmission (see NOTE 5) |  | tUL_duration | Length of one slot (14 OFDM symbol length) or non-slot (2/4/7 OFDM symbol length), depending on slot or non-slot selected in evaluation. |
| 1.4 | BS processing delay |  | tBS,rx The time interval between the PUSCH is received and the data is decoded; | Tproc,1/2 (Tproc,1 is defined in Section 5.3 of TS38.214), d1,1=0; d1,2 should be selected according to resource mapping type and UE capability. N1=the value with “No additional PDSCH DM-RS configured”; It is assumed that BS processing delay is equal to UE processing delay as for PDSCH (Note2) |
| 2 | HARQ retransmission |  | THARQ = T2 + T1 T2 = (tBS,tx + tFA,DL) + tDL_duration + tUE,rx (For Steps 2.1 to 2.4) |  |
| 2.1 | BS processing delay |  | tBS,tx The time interval between the data is decoded, and PDCCH preparation | Tproc,1/2 (Tproc,1 is defined in Section 5.3 of TS38.214), d1,1=0; d1,2 should be selected according to resource mapping type and UE capability. N1=the value with “No additional PDSCH DM-RS configured”. |
| 2.2 | DL Frame alignment (transmission alignment) |  | tFA,DL It includes frame alignment time, and the waiting time for next available DL slot | TFA + Twait, TFA is the frame alignment time within the current DL slot; Twait is the waiting time for next available DL slot if the current slot is not DL slot; |
| 2.3 | PDCCH transmission (see NOTE 5) |  | tDL_duration | 1 OFDM symbols for PDCCH scheduling the retransmission. |
| 2.4 | UE processing delay |  | tUE,rx The time interval between the PDCCH is received and decoded. | Tproc,2/2 (Tproc,2 is defined in Section 6.4 of TS38.214), with d2,1= d2,2= d2,3=0 |
| 2.5 | Repeat UL data transfer from 1.1 to 1.4 |  | T1 |  |
|  | Total one way user plane latency for UL [ms] | LEO600 | TUP= T1 + n×THARQ where n is the number of re-transmissions (n≥0) |  |
|  |  | GEO |  |  |
| NOTE 1 : Transmission delay includes service link propagation delay, feeder link propagation delay and satellite processing delay but only the satellite processing delay is including in the time budget as explained in Observation 2. Note 2: The value is used for evaluation only; gNB processing delay may vary depending on implementation. |  |  |  |  |

Table 4: User plane UL procedure latency

Proposal 5:   Send LS to RAN2 to provide RAN1 inputs on User plane procedure latency

# Mobility interruption time

The mobility interruption time is defined in [2] as follows:

Mobility interruption time is the shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any satellite and/or gateway node during transitions.

The mobility interruption time includes the time required to execute any radio access network procedure, radio resource control signalling protocol, or other message exchanges between the mobile station and the radio access network, as applicable to the candidate RIT/SRIT.

The requirement for mobility interruption time is 50 ms.

The procedure of exchanging user plane packets with base stations during transitions shall be described based on the proposed technology including the functions and the timing involved.

The requirement value above pertains to the eMBB-s usage scenario and applies to handheld devices.

Network controlled mobility applies to UEs in RRC_CONNECTED is categorized into two types of mobility: cell level mobility and beam level mobility.

## Cell level mobility interruption time

Cell level mobility requires explicit RRC signaling to be triggered, i.e. handover. For inter-gNB handover, handover request, handover acknowledgement, handover command, handover complete procedure are supported between source gNB and target gNB. The release of the resources at the source gNB during the handover completion phase is triggered by the target gNB.

The user plane data transfer interruption (Tdata_interrupt) during handover execution is defined as the interruption: From reception of rrcreconfiguration (HO command) within the serving cell to the transmission of rrcreconfigurationcomplete to the target cell:

Tdata_interrupt =  RRC procedure delay + Tinterrupt + 3 * Propagation delay + UE processing + L1 encoding [ms]

Where: Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin [ms] (see clause §6.1.1.2.2 in TS 38.133) and RRC procedure delay = 10 ms (see clause §12 in TS 38.331).

The different delays are depicted on the following signaling flow:

Figure 3: Data transfer interruption during Handover execution

The Table 5 shows the user plane data transfer interrupt (ms) for cell at nadir (Scenario C: Transparent payload):

|  | Intra-frequency handover from FR1 to FR1 | Inter-frequency handover from FR1 to FR1 |
| --- | --- | --- |
| RRC procedure delay | 10 | 10 |
| Tinterrupt | 210 | 270 |
| Propagation delay (PRACH UE to gNB) | 4 | 4 |
| Preamble detection | 2,25 | 2,25 |
| Propagation delay (RAR gNB to UE) | 4 | 4 |
| UE Processing and L1 encoding | 5 | 5 |
| Propagation delay (RRC Reconfig Complete UE to gNB) | 4 | 4 |
| UP plane data transfer interrupt [ms] | 239.25 | 299.25 |

Table 5: User plane data transfer interruption duration for cell at nadir

The following table shows the user plane data transfer interrupt (ms) the cell at Edge of Cell (EoC) (Scenario C: Transparent payload):

|  | Intra-frequency handover from FR1 to FR1 | Inter-frequency handover from FR1 to FR1 |
| --- | --- | --- |
| RRC procedure delay | 10 | 10 |
| Tinterrupt | 210 | 270 |
| Propagation delay (PRACH UE to gNB) | 12,88 | 12,88 |
| Preamble detection | 2,25 | 2,25 |
| Propagation delay (RAR gNB to UE) | 12,88 | 12,88 |
| UE Processing and L1 encoding | 5 | 5 |
| Propagation delay (RRC Reconfig Complete UE to gNB) | 12,88 | 12,88 |
| UP plane data transfer interrupt [ms] | 265.89 | 325.89 |

Table 6: User plane data transfer interruption duration for cell at Edge of Cell

## Beam level mobility interruption time

Beam level mobility does not require explicit RRC signaling to be triggered - it is dealt with at lower layers - and RRC is not required to know which beam is being used at a given point in time.

Proposal 6: Beam level mobility in same cell with no RRC involvement should be considered for mobility interruption time evaluation.

# Conclusion

In this contribution we made the following observations and proposals:

Observation 1:  Feeder and service links propagation delays are not included in the time budget to satisfy the 40 ms control plane latency requirement.

Observation 2: Feeder and service links propagation delay are not included in the time budget to satisfy the 10 ms user plane latency requirement.

Proposal 1: The Table 2 is used for NR NTN Control plane latency analysis

Proposal 2:   Send LS to RAN2 to provide RAN1 inputs on Control plane procedure latency

Proposal 3: The components in Table 3 are considered for DL user plane procedure latency analysis for NR NTN with HARQ enabled.

FFS: HARQ is disabled and radio retransmission handled by RLC ARQ.

Proposal 4: The components in Table 4 are considered for UL user plane procedure latency analysis for NR NTN.

FFS HARQ is disabled and radio retransmission handled by RLC ARQ.

Proposal 5:   Send LS to RAN2 to provide RAN1 inputs on User plane procedure latency

Proposal 6: Beam level mobility in same cell with no RRC involvement should be considered for mobility interruption time evaluation.

# References

- RP-230736 - New SID: Study on Self-Evaluation towards the 3GPP submission of an IMT-2020 Satellite Radio Interface Technology, Ericsson, Qualcomm, Thales, MediaTek Inc.

- Report ITU-R M.2514 (09/2022) - Vision, requirements and evaluation guidelines for satellite radio interface(s) of IMT-2020 – PDF, Word

- R1-2302871: SI work plan for Study on self-evaluation towards the IMT-2020 submission of the 3GPP Satellite Radio Interface Technology, Rapporteur (Ericsson)