3GPP TSG RAN WG1 #113		R1- 2305732

Incheon, Korea, May 22nd – 26th, 2023

Agenda item:	9.16.12

Source: 	Nokia (WI rapporteur)

Title: 	Initial UE feature list for NR support for dedicated spectrum less than 5MHz

Document for:	Discussion/Decision

- Introduction

This document provides the initial UE feature list for NR support for dedicated spectrum less than 5MHz for FR1 WI.

# 51.	NR_FR1_lessthan_5MHz_BW

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51. NR_FR1_lessthan_5MHz_BW | 51-x | Support for 3 MHz channel bandwidth | 1. Reception of [12 or 15] RB PBCH. 2. [Reception of 12 or 15 RB CORESET]. 3. [Disabling of PUCCH Frequency hopping] |  | Yes | N/A | UE not able to support 3 MHz channel bandwidth | Per Band | No | No | N/A |  | Optional |

- Annex – Agreements

RAN1#111 (November 2022)

Agreement In an LS to RAN4, in addition to reuse 5 MHz channel bandwidth, RAN1 suppose only 3 MHz channel bandwidth is supported, and would like to get RAN4 responses on the maximum transmission bandwidth (the number of PRBs) for this channel BW. Agreement RAN1 would like to ask RAN4 if finer sync. raster for the 3MHz and/or 5MHz channel bandwidth is feasible, as well as any input from RAN1 for RAN4’s answer to this question. Agreement Before getting RAN4 responses, RAN1 assume maximum transmission bandwidth, 15RBs or 16RBs for 3 MHz channel BW for evaluation and analysis. Note: include agreement into the LS Agreement Before getting RAN4 responses, RAN1 assume that the UE could know which RBs are used for SSB transmission after PSS/SSS is detected for evaluation and analysis. Note: it does not mean indication signaling is needed. Note: include this agreement into the LS Agreement For transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth, a subset of PRBs of 20-PRB PBCH are used for PBCH transmission if the transmission BW of a channel is less than 20PRBs. - FFS which PRBs are used and how to use the PRBs - Note: PRBs for PSS/SSS are not punctured. Agreement For CORESET#0 configuration for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidth, following options are for study, - Opt.1: Existing configuration table for 15kHz SCS, 5MHz minimum channel BW (i.e., table 13-1 in TS38.213) is reused for configuration - Opt.2: A new CORESET#0 configuration table is to be introduced for the configuration. Conclusion No enhancements are required for PRACH to operate NR on transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. - Note: PRACH formats and configurations not fitting into the transmission BW are not applicable Agreement Short PRACH formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS are supported for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidth. Conclusion No enhancements are needed for PUCCH to support transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth, - FFS: the necessity for PUCCH FH disabling. Agreement Study whether and how to recover PDCCH detection performance of CORESET#0 for transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. The following options are considered, - Opt.1: Power boosting - Opt.2: Non-interleaved CCE-to-REG mapping - Opt.3: A new interleaver to ensure PDCCH is fully mapped in the spectrum - Opt.4: New aggregation level(s) for fit in the spectrum - Opt.5: PDCCH rate matching - Opt.6.: no enhancement specified Agreement Study whether and how to recover PBCH detection performance for transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. The following options are considered, - Opt.1: Power boosting - Opt.2: Multiple PBCH receptions - Opt.3: PBCH remapping - Opt.4: PBCH payload reduction - Opt.5: PBCH rate matching around the punctured PRBs - Opt.6: no enhancement specified

RAN1#112

Conclusion For transmission bandwidths of <5MHz for 3MHz channel bandwidth, for CSI-RS other than for RRM measurements, no enhancements are needed. FFS: CSI-RS for RRM Agreement - For transmission BWs for 3MHz and 5MHz channel BW, send an LS to RAN plenary for operators input for the following and RAN plenary guidance, - For 5MHz channel BW, whether to allow/support transmission BW(s) for physical channels of approximate 3 MHz up to below 5 MHz. What is the recommended transmission BW(s) to consider? - For 3MHz channel BW, whether to allow/support transmission BW(s) for physical channels of approximate 3 MHz. What is the recommended transmission BW(s) to consider? - No intention to change the WID scope and TU Working Assumption For transmission bandwidth[s] of <5MHz, for PBCH, in the case[s] that available PRBs for PBCH transmission is less than 20PRB, - PBCH based on RB-level puncturing (i.e., PBCH encoding is based on 20PRB. The encoded bits and DMRS are mapped to 20PRBs based on legacy SSB structure, and those PRBs that fall outside of available PRBs for PBCH transmission are punctured) - Note: No other optimization is needed