3GPP TSG WG RAN1 #113	R1-2306249

Incheon, May 22nd – 26th, 2023

# Title:	Reply on LS 2TA for multi-DCI multi-TRP

# Response to:	R1-2304326 LS on 2TA for multi-DCI multi-TRP

# Release:	Rel-18

# Work Item:	NR_MIMO_evo_DL_UL-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

Name:	Siva Muruganathan

E-mail Address:	siva.muruganathan@ericsson.com

1. Overall Description:

RAN1 thanks RAN2 for the LS on 2TA for multi-DCI multi-TRP and would like to provide the following response.

Question 1 on TAG grouping

RAN2 discussed how the cells/TRPs configured for the UE, are to be grouped if UE is configured with two TA groups per serving cell. Currently, NR does not impose any requirements in configuring the association of serving cells and TAGs.

Q1a:   For the 2TA operation, are there any restrictions on the association of serving cells and/or TRPs to the TAGs?

Answer:  Apart from the agreements RAN1 has sent in LS R1-2302226 to RAN2 before, RAN1 has not agreed to any further restrictions on the association of serving cells and/or TRPs to the TAGs at this point. If RAN1 agrees to such restrictions, RAN1 will inform RAN2.

Q1b: NR currently supports up to 4 TAGs per cell group. Are the 4 TAGs enough or does RAN1 see a need to increase the number of TAGs per cell group?

Answer:  RAN1 has not reached consensus to increase the current number of TAGs per cell group.

Question 2 on operation

Q2: When the time-alignment timer associated with one of the TRPs of a serving cell expires, are certain UL or DL operation only impacted towards that TRP while they are not impacted towards the another TRP? If so, which UL or DL operation?

Answer: RAN1 confirms that when the TA timer associated to one TRP expires for a TAG associated with a TCI state, UL or DL operation associated to the another TRP is not impacted. This further depends on PTAG/STAG definition, which is up to RAN2 to decide.

Which UL or DL operation is impacted have not been discussed in RAN1.

2. Actions:

To RAN2 group:

ACTION:

RAN1 respectfully asks RAN2 to take the above answers into consideration.

3. Date of Next RAN1 Meetings:

TSG WG RAN1 #114		21st– 25th August 2023	Toulouse, France

TSG WG RAN1 #114-bis		9th – 13th October 2023	Xiamen, China