3GPP TSG RAN WG1 #114bis	R1-2308850

Xiamen, China, 9th – 13th October, 2023

Agenda item:		8.16.5

Source:	Nokia, Nokia Shanghai Bell

Title:	UE features for NR network energy savings

WI code:	Netw_Energy_NR-Core

Document for:		Discussion and Decision

# Introduction

UE features for Rel-18 NR NES were first discussed in RAN1#114.  The rapporteur has compiled the UE feature groups and features for each WI and the outcome of previous discussion for these feature groups including Rel-18 NR NES is captured in [1]. In this contribution, we provide our inputs to the UE features for Rel-18 NR NES enhancements in RAN1#114bis

# Discussion

## Spatial Domain Adaptation for NES in NR

In RAN1#114, a new FG was added for Spatial domain adaptation with CSI feedback based on CSI report sub-configuration for periodic and aperiodic CSI reporting. While details related to UE capabilities have not been finalized, the need for capability indication of supported number of sub-configurations per CSI report configuration, number of measurements reported per configuration, and number of CSI-RS resources configurations are necessary and should be added as components to the newly added FG 42-1; however the usage scenarios and transmission methods for periodic and aperiodic measurement reporting are different.  For this reason, the maximum number of supported sub-configurations should be different for periodic and aperiodic reporting.

Proposal 1:	Separate components 1 and 2 into different components for periodic and aperiodic reporting for FG 42-1, Spatial domain adaptation with CSI feedback based on CSI report sub-configuration.

Similarly, number of sub-configurations per CSI report configuration and number of measurements per sub-configuration should be added for semi-persistent CSI reporting as well; however, semi-persistent CSI reports activated by MAC CE are transmitted on PUCCH, while semi-persistent CSI reports activated by DCI are transmitted on PUSCH. Under these circumstances, the maximum number of sub-configurations and CSI measurements per sub-configuration should be reported as separate capabilities for reporting on PUCCH and PUSCH respectively.

Proposal 2:	Separate components 1 and 2 into different components for reporting on PUCCH and PUSCH respectively for FG 42-1a, Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting.

Likewise, a similar structure is necessary for power domain adaptation with CSI feedback for aperiodic and periodic CSI reporting.

Proposal 3:	Separate components 1 and 2 into different components for periodic and aperiodic reporting for FG 42-2, Power domain adaptation with CSI feedback based on CSI report sub-configuration.

Similarly, number of sub-configurations per CSI report configuration and number of measurements per sub-configuration should be added for semi-persistent CSI reporting as well.

Proposal 4:	Separate components 1 and 2 into different components for reporting on PUCCH and PUSCH respectively for FG 42-2a, Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting.

In RAN1#114, consensus could not be reached on whether FG 42-5, Cell DTX/DRX operation triggered by DCI format, should be merged with FG 42-4, Cell DTX or DRX operation based on RRC configuration.  Companies, however, may prefer not to use both mechanisms and so for flexibility, the FGs should not be merged.  Additionally, the description of FG 42-5, should be updated to indicate that operation is triggered by DCI 2_9.

Proposal 5:	Keep FG 42-5, Cell DTX/DRX operation triggered by DCI format as a separate FG and update to clarify that operation is triggered by DCI format 2_9.

A proposed table to be used as a baseline for UE feature groups discussion related to NES in NR in RAN1#114bis is provided in Table 1.

Proposal 6: Adopt Table 1 as baseline for discussion on UE feature groups related to Rel-18 NES in NR.

Table 1: Proposed baseline for UE feature groups for Rel-18 NES in NR

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 42. Netw_Energy_NR | 42-1 | Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) [for periodic and aperiodic CSI reporting] | Support of CSI feedback based on CSI report sub-configuration(s), each containing one [port subset configuration/list of CSI-RS IDs] [for each of periodic and aperiodic CSI reporting] 1. The max number of sub-configurations L in one aperiodic CSI report configuration 2. The max number of sub-configurations M in one periodic CSI report configuration 3. Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration in an aperiodic report configuration 4. Report of P CSI(s) in one CSI report where each CSI corresponds to one sub-configuration in a periodic report configuration 5. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs |  | Yes |  | UE does not support spatial domain adaptation [for periodic and aperiodic CSI reporting] | [Per UE, Per band] | No | No | N/A | [Component 1 candidate value for P-CSI report: FFS Component 1 candidate values for A-CSI report: FFS Component 2 candidate value(s): FFS] [Component 3 candidate value(s): FFS Component 4 candidate value(s): FFS Component 5 candidate value(s): FFS Component 6 candidate value: FFS] FFS: merge FG 42-2 with FG 42-1 FFS: whether to have separate rows for type 1 or 2 | Optional with capability signaling |
| 42. Netw_Energy_NR | 42-1a | Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting | Support of CSI feedback based on CSI report sub-configuration(s), each containing one [port subset configuration/list of CSI-RS IDs] for semi-persistent CSI reporting 1. The max number of sub-configurations L in one CSI report configuration in PUCCH 2. The max number of sub-configurations M in one CSI report configuration in PUSCH 3. Report of N CSI(s) in one SP-CSI report where each CSI corresponds to one sub-configuration in PUCCH report configuration 4. Report of P CSI(s) in on SP-CSI report where each CSI corresponds to one sub-configuration in PUSCH report configuration | FFS | Yes |  | UE does not support spatial domain adaptation for semi-persistent CSI reporting | [Per UE, Per band] | No | No | N/A | Component 1 candidate value for SP-CSI report: FFS Component 2 candidate value(s): FFS] FFS: whether to have separate rows for type 1 or 2 | Optional with capability signaling |
| 42. Netw_Energy_NR | 42-2 | Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) [for periodic and aperiodic CSI reporting] | Support of CSI feedback based on CSI report sub-configuration(s), each containing one power offset [for each of periodic and aperiodic CSI reporting] 1. The max number of sub-configurations L in one aperiodic CSI report configuration 2. The max number of sub-configurations M in one periodic CSI report configuration 3. Report of N CSI(s) in one CSI report where each CSI corresponds to one sub-configuration in an aperiodic report configuration 4. Report of P CSI(s) in one CSI report where each CSI corresponds to one sub-configuration in a periodic report configuration 5. Supported maximum number of simultaneous NZP-CSI-RS resources per CC 6. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources per CC 7. Supported maximum number of total CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 8. Supported maximum number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs |  | Yes |  | UE does not support power domain adaptation [for periodic and aperiodic CSI reporting] | [Per UE, Per band] | No | No | N/A | [Component 1 candidate value for P-CSI report: FFS Component 1 candidate values for A-CSI report: FFS Component 2 candidate value for L: FFS] [Component 2 candidate value for N: FFS]] [Component 3 candidate value: FFS Component 4 candidate value: FFS Component 5 candidate value: FFS Component 6 candidate value: FFS] FFS: merge FG 42-2 with FG 42-1 | Optional with capability signaling |
| 42. Netw_Energy_NR | 42-2a | Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting | Support of CSI feedback based on CSI report sub-configuration(s), each containing one power offset for semi-persistent CSI reporting 1. The max number of sub-configurations L in one CSI report configuration 2. The max number of sub-configurations M in one CSI report configuration in PUSCH 3. Report of N CSI(s) in one SP-CSI report where each CSI corresponds to one sub-configuration in PUCCH report configuration. 4. Report of P CSI(s) in on SP-CSI report where each CSI corresponds to one sub-configuration in PUSCH report configuration |  | Yes |  | UE does not support power domain adaptation for semi-persistent CSI reporting | [Per UE, Per band] | No | No | N/A | [Component 1 candidate value for SP-CSI report: FFS] Component 2 candidate value for L: FFS] [Component 2 candidate value for N: FFS]] FFS: merge FG 42-2a with FG 42-1a | Optional with capability signaling |
| 42. Netw_Energy_NR | 42-4 | Cell DTX or DRX operation based on RRC configuration [with one DTX/DRX configuration per cell] | Support of cell DTX/DRX operation [with one DTX/DRX configuration per cell] [1) During non-active period of Cell DTX, UE does not receive and/or process the following channels/signals from the gNB: - Periodic/Semi-persistent CSI-RS configuration in CSI-ReportConfig with reportQuantity including RI 2) During non-active period of Cell DRX, UE does not transmit the following channels/signals to the gNB: - Periodic/Semi-persistent CSI report - Periodic/Semi-persistent SRS] |  | Yes |  | UE does not support Cell DTX/DRX operation [with one DTX/DRX configuration per cell] | FFS | No | No | N/A | Note: RAN2 may add additional details | Optional with capability signaling |
| 42. Netw_Energy_NR | 42-5 | Cell DTX/DRX operation triggered by DCI format 2_9 | 1) Support of Cell DTX/DRX configuration activation and deactivation via DCI 2_9 | 42-4 | Yes |  | UE does not support dynamic Cell DTX/DRX operation triggered by L1 signalling | FFS | No | No | N/A | FFS: merge this FG with FG 42-4 | Optional with capability signaling |

# Summary

The following are proposed for discussion on UE features related to Rel-18 NR network energy saving enhancements in RAN1#114bis:

Proposal 1:	Separate components 1 and 2 into different components for periodic and aperiodic reporting for FG 42-1, Spatial domain adaptation with CSI feedback based on CSI report sub-configuration.

Proposal 2:	Separate components 1 and 2 into different components for reporting on PUCCH and PUSCH respectively for FG 42-1a, Spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting.

Proposal 3:	Separate components 1 and 2 into different components for periodic and aperiodic reporting for FG 42-2, Power domain adaptation with CSI feedback based on CSI report sub-configuration.

Proposal 4:	Separate components 1 and 2 into different components for reporting on PUCCH and PUSCH respectively for FG 42-2a, Power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for semi-persistent CSI reporting.

Proposal 5:	Keep FG 42-5, Cell DTX/DRX operation triggered by DCI format as a separate FG and update to clarify that operation is triggered by DCI format 2_9.

Proposal 6: Adopt Table 1 as baseline for discussion on UE feature groups related to Rel-18 NES in NR.

# References

- R1-2308521, “Updated RAN1 UE features list for Rel-18 NR after RAN1#114,”	Moderators (AT&T, NTT DOCOMO, INC.)