3GPP TSG RAN WG1 #114bis	                              	         R1-2308992

Xiamen, China, October 9th – October 13th, 2023

Agenda Item:     8.6.1

Source:	Spreadtrum Communications

Title:	Remaining issues on XR-specific capacity enhancements

Document for:	Discussion and decision

Introduction

In this contribution, we provide our views on remaining issue of XR-specific capacity enhancements and propose associated text proposals to rectify missing or incorrectly captured agreements.

Discussion

Remaining issue on UTO-UCI applicability under the “multi-PUSCH” CG and the legacy CG

Regarding to the description in section 9.3.1 of the endorsed CR [1], as quoted as the following text. One sentence was deleted as the statements on UTO-UCI do not explicitly mention “multi-PUSCH” CG. As there is no agreement on the dependency between UTO-UCI and multi-PUSCH CGs in RAN1, we support that there is no limitation of UTO-UCI to multi-PUSCH CG. UTO-UCI should also be applicable on legacy CGs, too, if gNB configures UTO-UCI for a legacy CG configuration.

- 9.3.1 UE procedure for reporting UTO-UCI A UE can be indicated, by nrofSlots_InCGperiod in configuredGrantConfig, more than one TO for CG-PUSCH transmission within a period of a CG-PUSCH configuration [6, TS 38.214]. If the UE is provided nrof_UTO_UCI with value equal to OUTO-UCI in configuredGrantConfig of a CG-PUSCH configuration, the UE multiplexes UTO-UCI represented by a bitmap of OUTO-UCI bits in each CG-PUSCH transmission for the CG-PUSCH configuration.

Proposal 1: UTO-UCI can be applicable on both of “multi-PUSCH” CG and the legacy CGs.

The definition of an invalid CG PUSCH

RAN2 have discussed the specification impact of HARQ process ID determination for multi-PUSCH configured grant. RAN2 sent a LS to RAN1 asking the definition of an invalid CG PUSCH and where the definition will be captured in the RAN1 specifications (so that RAN2 specifications can refer to that definition) [2].

Note: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB.

From RAN1 specification perspective, the definition of an invalid CG PUSCH have not been clearly captured in the RAN1 specifications.

There is some statements for the collision/dropping rules of PUSCH in 38.213 clause 11.1. But there is no explicitly definition for an invalid or valid CG PUSCH. In 38.213 clause 11.1, PUSCH is dropped when DL symbol configured by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB symbol, but also some other droppings like dynamic canceled by PDCCH. So there would be some ambiguity if only refers to the current spec in 38.213 clause 11.1, because it is not clear which part is counted for invalid/valid CG-PUSCH occasions. DL and SSB symbol only, or PDCCH scheduled DL reception also impact.

38.213, 11.1 Slot configuration The statement for DL symbol and SSB symbol /************************ Omitted**************************/ For a set of symbols of a slot that are indicated to a UE as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit PUSCH, PUCCH, PRACH, or SRS when the PUSCH, PUCCH, PRACH, or SRS overlaps, even partially, with the set of symbols of the slot. /************************ Omitted**************************/ For operation on a single carrier in unpaired spectrum, for a set of symbols of a slot indicated to a UE for reception of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon or, if the UE is not provided dl-OrJoint-TCIStateList, by ssb-PositionsInBurst in SSB-MTCAdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH blocks configured for L1 beam measurement/reporting, the UE does not transmit PUSCH, PUCCH, PRACH in the slot if a transmission would overlap with any symbol from the set of symbols and the UE does not transmit SRS in the set of symbols of the slot. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE.

The statement for dynamic cancelled PUSCH by PDCCH /************************ Omitted**************************/ For operation on a single carrier in unpaired spectrum, if a UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH, or PRACH in a set of symbols of a slot and the UE detects a DCI format indicating to the UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - If the UE does not indicate the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in the set of symbols if the first symbol in the set occurs within Tproc,2 relative to a last symbol of a PDCCH reception where the UE detects the DCI format; otherwise, the UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from clauses 9, 9.2.5 and 9.2.6 or clause 6.1 of [6, TS 38.214], or the PRACH transmission in the set of symbols. - If the UE indicates the capability of [partialCancellation], the UE does not expect to cancel the transmission of the PUCCH or PUSCH or PRACH in symbols from the set of symbols that occur within Tproc,2 relative to a last symbol of a PDCCH reception where the UE detects the DCI format. The UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from clauses 9, 9.2.5 and 9.2.6 or clause 6.1 of [6, TS 38.214], or the PRACH transmission in remaining symbols from the set of symbols. /************************ Omitted**************************/

Thus, our preference is to give explicit statement for invalid CG PUSCH, which can be capture in 38.214 clause 6.1.2.3. HARQ process determination for multi-PUSCH CG and UTO-UCI indication can both refer to it.

6.1.2.3 Resource allocation for uplink transmission with configured grant If at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, the configured PUSCH grant is invalid, otherwise it is valid.

Proposal 2: Adopt the following TP for section 6.1.2.3 in TS 38.214.

| Reason for change | The definition of an invalid CG PUSCH have not been clearly captured in the RAN1 specifications. |
| --- | --- |
| Summary of change | 6.1.2.3 Resource allocation for uplink transmission with configured grant If at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, the configured PUSCH grant is invalid, otherwise it is valid. |
| Consequences if not approved | The definition of an invalid CG PUSCH is not clear. |
| Text proposal | TS 38.214 6.1.2.3 Resource allocation for uplink transmission with configured grant ************** Unchanged parts omitted************** If at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, the configured PUSCH grant is invalid, otherwise it is valid. ************** Unchanged parts omitted************** |

Conclusion

In this contribution, we discuss on remaining issue of XR-specific capacity enhancements. The following proposals are achieved:

Proposal 1: UTO-UCI can be applicable on both of “multi-PUSCH” CG and the legacy CGs.

Proposal 2: Adopt the following TP for section 6.1.2.3 in TS 38.214.

| Reason for change | The definition of an invalid CG PUSCH have not been clearly captured in the RAN1 specifications. |
| --- | --- |
| Summary of change | 6.1.2.3 Resource allocation for uplink transmission with configured grant If at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, the configured PUSCH grant is invalid, otherwise it is valid. |
| Consequences if not approved | The definition of an invalid CG PUSCH is not clear. |
| Text proposal | TS 38.214 6.1.2.3 Resource allocation for uplink transmission with configured grant ************** Unchanged parts omitted************** If at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, the configured PUSCH grant is invalid, otherwise it is valid. ************** Unchanged parts omitted************** |

Reference

- R1-2308740, “Introduction of XR Enhancements for NR”, RAN1#114, Toulouse, France, August 21st – August 25th, 2023.

- R1-2308825, “Reply LS on XR capacity enhancements”, RAN1#114, Toulouse, France, August 21st – August 25th, 2023.