3GPP TSG RAN WG1 #114bis	R1-2309130

# Xiamen, China, October 9th – October 13th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | [DRAFT] | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Type-1 HARQ-ACK codebook for MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, CBN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For Type-1 codebook generation, the maximum number of repetitions is used to determine whether to exclude a TDRA from the TDRA set. For MBS, the gNB can configure the number of repetitions per G-RNTI/G-CS-RNTI. For the Type-1 codebook for MBS, the maximum number of repetitions among all the G-RNTI/G-CS-RNTI should be used. For the joint Type-1 codebook for MBS and unicast, the maximum number of reptitions of the MBS and unicast should be used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use the maximum number of repetitions for Type-1 codebook generation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE cannot determine the Type-1 codebook correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

9.1.2	Type-1 HARQ-ACK codebook determination

<Unchanged parts are omitted>

If the UE

-	is not configured to monitor PDCCH for multicast DCI formats

-	if the UE is provided pdsch-AggregationFactor-r16 in SPS-Config, or pdsch-AggregationFactor in PDSCH-Config and no entry in pdsch-TimeDomainAllocationList and pdsch-TimeDomainAllocationListDCI-1-2 includes repetitionNumber in PDSCH-TimeDomainResourceAllocation-r16, NPDSCHrepeat,max is a maximum value of pdsch-AggregationFactor-r16 in SPS-Config, or pdsch-AggregationFactor in PDSCH-Config;

-	otherwise NPDSCHrepeat,max=1.

-	is configured to minotor PDCCH for multicast DCI formats,

-	if the UE is provided fdmed-ReceptionMulticast

-	if the UE is provided pdsch-AggregationFactor-r16 in SPS-Config, or pdsch-AggregationFactor in PDSCH-Config and no entry in pdsch-TimeDomainAllocationList and pdsch-TimeDomainAllocationListDCI-1-2 includes repetitionNumber in PDSCH-TimeDomainResourceAllocation-r16, NPDSCHrepeat,max is a maximum value of pdsch-AggregationFactor-r16 in SPS-Config, or pdsch-AggregationFactor in PDSCH-Config; otherwise NPDSCHrepeat,max=1 for the set SU.

-	if the UE is provided pdsch-AggregationFactor-r16 in SPS-Config for a G-CS-RNTI or pdsch-AggregationFactor in MBS-RNTI-SpecificConfig for a G-CS-RNTI, NPDSCHrepeat,max is a maximum value of pdsch-AggregationFactor-r16 in SPS-Config, or pdsch-AggregationFactor in PDSCH-Config for all the G-RNTI and G-CS-RNTI; otherwise NPDSCHrepeat,max=1 for the set SM.

-	if the UE is not provided fdmed-ReceptionMulticast and is provided type1CodebookGenerationMode = 'mode1',

-	if the UE is provided pdsch-AggregationFactor-r16 in SPS-Config, pdsch-AggregationFactor in PDSCH-Config, pdsch-AggregationFactor-r16 in SPS-Config for a G-CS-RNTI or pdsch-AggregationFactor in MBS-RNTI-SpecificConfig for a G-CS-RNTI and no entry in pdsch-TimeDomainAllocationList and pdsch-TimeDomainAllocationListDCI-1-2 includes repetitionNumber in PDSCH-TimeDomainResourceAllocation-r16 NPDSCHrepeat,max is a maximum value of pdsch-AggregationFactor-r16 in SPS-Config, pdsch-AggregationFactor in PDSCH-Config, or pdsch-AggregationFactor in MBS-RNTI-SpecificConfig; otherwise NPDSCHrepeat,max=1 for the first K1,UM set.

-	if the UE is provided pdsch-AggregationFactor-r16 in SPS-Config, or pdsch-AggregationFactor in PDSCH-Config and no entry in pdsch-TimeDomainAllocationList and pdsch-TimeDomainAllocationListDCI-1-2 includes repetitionNumber in PDSCH-TimeDomainResourceAllocation-r16, NPDSCHrepeat,max is a maximum value of pdsch-AggregationFactor-r16 in SPS-Config, or pdsch-AggregationFactor in PDSCH-Config; otherwise NPDSCHrepeat,max=1 for the second K1,U\M set.

-	if the UE is provided pdsch-AggregationFactor-r16 in SPS-Config for a G-CS-RNTI or pdsch-AggregationFactor in MBS-RNTI-SpecificConfig for a G-CS-RNTI, NPDSCHrepeat,max is a maximum value of pdsch-AggregationFactor-r16 in SPS-Config, or pdsch-AggregationFactor in PDSCH-Config for all the G-RNTI and G-CS-RNTI; otherwise NPDSCHrepeat,max=1 for the third K1,M\U set.

-	if the UE is not provided fdmed-ReceptionMulticast and is not provided type1CodebookGenerationMode = 'mode1',

-	if the UE is provided pdsch-AggregationFactor-r16 in SPS-Config, pdsch-AggregationFactor in PDSCH-Config, pdsch-AggregationFactor-r16 in SPS-Config for a G-CS-RNTI or pdsch-AggregationFactor in MBS-RNTI-SpecificConfig for a G-CS-RNTI and no entry in pdsch-TimeDomainAllocationList and pdsch-TimeDomainAllocationListDCI-1-2 includes repetitionNumber in PDSCH-TimeDomainResourceAllocation-r16 NPDSCHrepeat,max is a maximum value of pdsch-AggregationFactor-r16 in SPS-Config, pdsch-AggregationFactor in PDSCH-Config, or pdsch-AggregationFactor in MBS-RNTI-SpecificConfig; otherwise NPDSCHrepeat,max=1.

The UE reports HARQ-ACK information for a PDSCH reception

-	from DL slot nD−NPDSCHrepeat+1 to DL slot nD, if NPDSCHrepeat is provided by pdsch-AggregationFactor or pdsch-AggregationFactor-r16 [6, TS 38.214], or

-	from DL slot  nD−repetitionNumber+1 to DL slot nD, if the time domain resource assignment field in the DCI format scheduling the PDSCH reception indicates an entry containing repetitionNumber, or

-	in DL slot nD, otherwise

only in a HARQ-ACK codebook that the UE includes in a PUCCH or PUSCH transmission in slot n+k, where n is

-	an UL slot overlapping with the end of the PDSCH reception in DL slot nD if the UE is provided subslotLengthForPUCCH for the HARQ-ACK codebook

-	the UL slot is on the primary cell if the UE is provided pucch-sSCellPattern; otherwise, the UL slot is on the serving cell of the PUCCH transmission

-	the last UL slot for PUCCH transmission overlapping with DL slot nD if the UE is not provided subslotLengthForPUCCH for the HARQ-ACK codebook

-	the last UL slot is on the primary cell if the UE is provided pucch-sSCellPattern; otherwise, the last UL slot is on the serving cell of the PUCCH transmission

and k is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format, or provided by dl-DataToUL-ACK or dl-DataToUL-ACK-r16 or dl-DataToUL-ACK-DCI-1-2 or dl-DataToUL-ACK-r17 or dl-DataToUL-ACK-DCI-1-2-r17 if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI format. If the UE reports HARQ-ACK information for the PDSCH reception in a slot other than slot n+k, the UE sets a value for each corresponding HARQ-ACK information bit to NACK.

<Unchanged parts are omitted>