3GPP TSG RAN WG1 #114bis 	                                                                                           R1-2309438

Xiamen, China, October 9th – 13th, 2023

Agenda item:	8.14.2

Source:	            		Xiaomi

Title:	Remaining issues on specification impact for CSI feedback based on AI/ML

Document for:	Discussion and Decision

Introduction

In this contribution, the remaining issues on potential specification impact on AI/ML based CSI compression feedback with two-sided model and time domain CSI prediction are further discussed.

Remaining issues on specification impact for CSI compression feedback with two-sided AI/ML model

AI/ML model training collaboration type

According to the achieved agreements in the last meeting [1], the training type on Type 1 training at UE/NW/ neutral site with 3GPP transparent model delivery to UE and NW respectively is not needed to considered. In addition, the training type on type 1 with unknown model structure at UE followed by retraining at UE side is not required to discuss as well.

During the RAN1#114 meeting, the pros and cons of type 1, type 2 and type 3 have been discussed, and the discussed pros and cons on some items have been identified by companies. However, there are still controversies on some items.  In this subsection, we provide our views on these remaining issues.

Discussion on the pros and cons of type 1

- Flexibility to support cell/site/scenario/configuration specific model

No matter whether NW side knows the model structure of UE or not, NW side can collect dataset from different users in a cell, scenario or configuration. This helps NW can train different models corresponding to different cells/scenarios/configurations. Hence, NW side can flexibly to support cell/scenario/configuration specific model for type 1 training at network side.

- Feasibility of allowing UE side and NW side to develop/update models separately

Different from type 2 and type 3, both mode at UE side and model at NW side are jointly trained at one side. Once the model at UE side is updated, the model at NW side is updated as well due to compatible requirement for two-side model. Thus, it is not feasible to develop/update model separately.

- Extendibility: to train new UE-side model compatible with NW-side model in use

For UE side type 1, the CSI generation model can be independently updated during training two-side model, since UE side knows the CSI reconstruction model. During training the new UE-side model, the NW-side model can be fixed, such that the new trained UE-side model can be compatible with NW-side model in use. Otherwise, it is impossible that the new trained UE-side model is compatible with NW-side model in use.

- Extendibility: To train new NW-side model compatible with UE-side model in use

The analysis of this question is similar to extendibility on training new UE-side model compatible with NW-side model in use.

- Whether training data distribution can match the inference device

One side which training two-side model has known the data distribution. After model training, the training data distribution can match the inference device. Correspondingly, the training data distribution may not match the inference device for the opposite side, since the opposite side does not involve the model training. Therefore, training data distribution can match the inference device for gNB when training model at NW side. While the training data distribution cannot match the inference device for UE. Similarly, training data distribution can match the inference device for UE when training model at UE side.

- Software/hardware compatibility (Whether device capability can be considered for model development)

If one side that implementing joint training has known model structure at one side, the software/hardware compatibility can be considered for model development. Otherwise, one side cannot consider software/hardware compatibility due to not knowing the model structure at the side. Hence, we think software/hardware compatibility cannot be guaranteed if the training side does not know the model structure of the opposite site.

Proposal 1: In CSI compression using two-sided model use case, the following table captures the pros/cons of training collaboration types 1:

Table 1： The pros and cons of Type 1

| Training types Characteristics | NW side type 1 |  | UE side type 1 |  |
| --- | --- | --- | --- | --- |
|  | Unknown model structure at UE | Known model structure at UE | Unknown model structure at NW | Known model structure at NW |
| Whether model can be kept proprietary | No | No | No | No |
| Whether require privacy-sensitive dataset sharing | No (Note2) | No (Note2) | No (Note2) | No (Note2) |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | Yes | Semi-flexibility, if assistance information is supported. No otherwise. | Yes, if assistance information is supported. No otherwise |
| Whether gNB/device specific optimization is allowed | gNB: Yes UE: No | Yes (UE can compile the model) | gNB: No UE: Yes | Yes (gNB can compile the model) |
| Model update flexibility after deployment (note 3) | Flexible | Flexible | Flexible Less flexible than Type 1 NW side | Flexible Less flexible than Type 1 NW side |
| Feasibility of allowing UE side and NW side to develop/update models separately | No | No | No | No |
| Whether gNB can maintain/store a single/unified model over different UE vendors for a CSI report configuration | Yes | Yes | No | No |
| Whether UE device can maintain/store a single/unified model over different NW vendors for a CSI report configuration | No | No | Yes | Yes |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | No | No | Yes | Yes |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Yes | Yes | No | No |
| Whether training data distribution can match the inference device | gNB: Yes UE: No | gNB: Yes UE: No | UE: Yes gNB: No | UE:Yes gNB: No |
| Software/hardware compatibility (Whether device capability can be considered for model development) | No | Yes | No | Yes |
| Model performance based on evaluation in 9.2.2.1 | Performance refers to 9.2.2.1 observations | Performance refers to 9.2.2.1 observations | Performance refers to 9.2.2.1 observations | Performance refers to 9.2.2.1 observations |

Note 1: Type 2 Sequential training assumes NW-first training, since Type 2 Sequential UE-first training would have similar pros/cons as Type 3 UE-first training

Note 2: Assume information on model structure disclosed in training collaboration does not reveal proprietary information.

Note 3: Assume precoding matrix is not privacy sensitive data. FFS: other information such as channel matrix and assisted information.

Note 4: Flexibility after deployment is evaluated by the amount of offline cross-vendor co-engineering effort. Flexible indicates minimum additional co-engineering between vendors, semi-flexible indicates additional co-engineering effort between vendors.

Remaining issue on the pros and cons of type 2 and type 3

- Whether UE device can maintain/store a single/unified model over different NW vendors for a CSI report configuration

The following provides the observations provided for type 2 and type 3 when pairing between 1 UE part model and N>1 separate NW part models in 9.2.2.1.

Observation: For the evaluation of Type 2 training between 1 UE part model and N>1 separate NW part models (Case 3), as compared to joint training between 1 NW part model and the 1 UE part model, - 2 sources observe minor degradation of -0%~-0.8% or positive gain; - 1 source observe moderate degradation of -1.4%~-4.2%. Note: among the above sources, 1 source adopts simultaneous training. Observation: For the evaluation of NW first separate training with dataset sharing manner for CSI compression, for the pairing between 1 UE part model and N>1 separate NW part models (Case 3), when taking 1-on-1 joint training between the NW part model and the UE part model as benchmark, larger performance loss is observed in general than the case of NW first separate training with 1 UE part model and 1 NW part model pairing (Case 1): - 6 sources observe minor loss of -0%~-1.6% compared to the 1-on-1 joint training. - 3 sources observe moderate loss of -1.9%~-6.64% compared to the 1-on-1 joint training. - 5 sources observe significant loss of -37.9%~-87% compared to the 1-on-1 joint training. - Note: as opposed to companies which observe significant loss, the minor loss observed by other companies may due to the fact that special handling (e.g., adaptation layer) is performed to pair with N>1 NW part models during the training at the UE side. - Note: the dataset sharing behavior from above sources follows the example of the agreement, where “the set of information includes the input and output of the Network side CSI generation part, or includes the output of the Network side CSI generation part only”.

According to the provided observation, we can see that UE device can maintain a single model over different NW vendors, although some system performance is degraded. For UE side first separating training with multiple NW vendors, the performance is same to that of one UE device and one NW vendor with separate training. The reason is that the training UE part model and NW part model are same.

- Extendibility: to train new UE-side model compatible with NW-side model in use;

For UE-first of type 3, the model at NW needs to be updated due to the update of model at UE side. This leads the updated UE-side model is not compatible with original model at NW side. However, if the parameters of CSI reconstruction model are fixed during model training, the retrained new UE side part model can still be compatible with NW side part model.

- Whether training data distribution can match the inference device

For type 3 with NW first separate training, the training dataset may be collected from different inference divides. The training data distribution can match the devices only if the training dataset is from those inference devices. For type 2 with sequential training, it is similar with type 3 with NW first separate training whether training data distribution matching the inference device.

Proposal 2: The pros and cons of collaboration training type 2 and type 3 are respectively provided in Table 2.

Table 2： The pros and cons of type 2 and type 3

| Training types Characteristics | Type 2 |  | Type 3 |  |
| --- | --- | --- | --- | --- |
|  | Simultaneous | Sequential NW first (Note 1) | NW first | UE first |
| Whether model can be kept proprietary | Yes (note 2) | Yes (note 2) | Yes (note 2) | Yes (note 2) |
| Whether require privacy-sensitive dataset sharing | No (Note 3) | No (Note 3) | No (Note 3) | No (Note 3) |
| Flexibility to support cell/site/scenario/configuration specific model | Difficult. | Semi-flexible. Less flexible compared to type 3. | Semi-flexible. | Semi-flexible. If assistance information is supported. Not flexible otherwise. |
| Whether gNB/device specific optimization is allowed | Yes | Yes | Yes | Yes |
| Model update flexibility after deployment (note 4) | Not flexible | Semi-flexible. Less flexible compared to type 3. | Semi-flexible | Semi-flexible |
| Feasibility of allowing UE side and NW side to develop/update models separately | Infeasible | Feasible | Feasible | Feasible |
| Whether gNB can maintain/store a single/unified model over different UE vendors for a CSI report configuration | Yes. Performance loss refers to 9.2.2.1 observations | Yes. Performance loss refers to 9.2.2.1 observations | Yes. Performance loss refers to 9.2.2.1 observations | Yes. Performance loss refers to 9.2.2.1 observations |
| Whether UE device can maintain/store a single/unified model over different NW vendors for a CSI report configuration | Yes. Performance loss refers to 9.2.2.1 observations | Yes. Performance loss refers to 9.2.2.1 observations | Yes. Performance loss refers to 9.2.2.1 observations | Yes. Performance is same as that of one UE device and one NW vendor with UE side separate training |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | Not support | Support | Support | Not support; Support if NW side part model is fixed during training. |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Not support | Not support | Not support; | Support |
| Whether training data distribution can match the inference device | More limited | Limited | Limited | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | Compatible | Compatible | Compatible | Compatible |
| Model performance based on evaluation in 9.2.2.1 | Performance refers to 9.2.2.1 observations | Performance refers to 9.2.2.1 observations | Performance refers to 9.2.2.1 observations | Performance refers to 9.2.2.1 observations |

Note 1: Assume precoding matrix is not privacy sensitive data. FFS: other information such as channel matrix and assisted information.

Note 2: Assume information on model structure disclosed in training collaboration does not reveal prioprotiy information

Note 3: Flexibility after deployment is evaluated by the amount of offline cross-vendor co-engineering effort. Flexible indicates minimum additional co-engineering between vendors, semi-flexible indicates additional co-engineering effort between vendors.

Note 4: Under the assumption that the vendor training first has engineering freedom to design its own model, the condition follows naturally. [To understand the effects of long-term evolution of the AI/ML model in the eco-system, further studies are needed].

Note 5: Additional assistance signaling may be needed. Once the first side has done training, a model defines a mapping between latent space codeword and CSI, i.e., implicitly defining a codebook. If multiple vendors are part in the first-side training, those multiple models may represent multiple codebooks. For the second side to train a unified model, it would require assistance information to ensure that a unified model compresses/decompresses according to the correct codebook.

Note 6: The need for matching the inference device in training can be limited, when mixing datasets from different device Types are used.

Data collection

It has discussed and agreed to study the necessity, feasibility and potential specification impact of UE side or NW side dataset collection enhancement. The collected dataset is used to train two-side model or monitor model performance. For training type 3, UE side or NW side needs to collect training dataset of CSI generation model or training dataset of CSI reconstruction model. How to collect training dataset for type 3 has not been agreed. In this subsection, we discuss the potential specification impact of training dataset collection for type 3.

For UE first training type 3, CSI reconstruction model training dataset needs to be delivered from UE side to NW side. The training dataset can be delivered via 3GPP transparent way or air interface. Note that there is no specification impact if dataset is delivered in 3GPP transparent way. If dataset is transferred through air interface, the type of dataset needs to defined. E.g., whether dataset type is precoding matrix or raw channel matrix. It also needs to study how to deliver the dataset. As discussed on ground-truth dataset reporting, scalar or codebook-based method can be adopted to report the training dataset.

For NW first training type 3, CSI generation model training dataset needs to be delivered from NW side to UE side. Similarly, the corresponding data set format or type needs to be defined, and the potential dataset delivery methods are required to study as well.

Proposal 3: The potential specification impact on training dataset delivery methods, dataset format or type need to study for collaboration training type 3.

CSI configuration and reporting

UE-side model selection for compatibility with NW-side model

In the RAN1#114 meeting [1], it has been observed that multiple options on pairing information were proposed by companies to enable the UE to select a CSI generation model that is compatible with the CSI reconstruction model used by gNB. Correspondingly, the paring information can also be used to enable the gNB to select a CSI reconstruction model that is compatible with the CSI generation model used by UE.

Observation In CSI compression using two-sided model use case, at least the following options have been proposed by companies to define the pairing information used to enable the UE to select a CSI generation model(s) that is compatible with the CSI reconstruction model(s) used by the gNB: - Option 1: The pairing information is in the forms of the CSI reconstruction model ID that NW will use. - Option 2: The pairing information is in the forms of the CSI generation model ID that the UE will use. - Option 3: The pairing information is in the forms of the paired CSI generation model and CSI reconstruction model ID. - Option 4: The pairing information is in the forms of by the dataset ID during type 3 sequential training. - Option 5: The pairing information is in the forms of a training session ID to a prior training session (e.g., API) between NW and UE. - Option 6: The pairing information is up to UE/NW offline co-engineering alignment, transparent to 3GPP specification. - Note: the disclosure of the vendor information during the model pairing procedure and model identification procedure should be considered. - Note: If each UE side model is compatible with all NW side model, the information is not needed for the UE. - Note: Above does not imply there is a need for a central entity for defining/storing/maintaining the IDs.

One remaining issue is how to select the compatible model by UE or gNB when there are multiple candidate models or part models at UE side or gNB side. In our view, there are potential following two options to implement model selection.

- Option 1: Model selected at UE side

- Option 2: Model selected at gNB side

The interactions or model selection procedures between UE and gNB are different for the two options. Assume that M>1 UE-side and gNB-side part models have been deployed at UE side and gNB side respectively, and the model identification between UE and gNB have been implemented at model training stage or UE capability reported. For Option 1, one straightforward way is that UE selects a part model depending on parameter configuration. E.g., the configured payload size of model output or application scenarios of model. The other way is that UE calculates the intermediate KPI or eventual KPI based on the output of CSI reconstruction model. Obviously, the computation complexity is scale with the number of part models. In order to address the issue, UE can preclude some part models according to some assistance information or configuration parameters from gNB, such that UE only calculates and selects one model from the less candidate models.

For Option 2, the input of CSI generation part model and CSI compression, i.e., the latent space codeword needs to report gNB so that gNB can calculate and select the best model. Since the input of CSI generation part model does not need to persistently report, gNB can indicate UE through downlink dynamic signalling, e.g., DCI to report if needed. There is still issue on computation complexity which is scale with the number of candidate models. In addition, the feedback overhead is increased as well with the number of candidate models increases. gNB can configure or indicate less candidate models to track the problem. We can see that the processing procedure of Option 2 is different from that of Option 1. Different procedures will incur different specification impacts. Hence, it is necessary to study specification impact on model selection when there are multiple candidate pair models at UE side and gNB side.

Proposal 4: If there are multiple candidate part models at UE side and gNB side, the interactions or procedures for model selection between UE and gNB should be studied.

CSI omission

When there is no enough uplink resource for transmitting all contents in a CSI reporting, CSI omission will occur. I.e., partial contents in the CSI reporting will omitted. However, if some important CSI parts are dropped, it will result significant performance loss. In order to address this issue, CSI Part 2 for eType II codebook are divided into three groups. It has agreed that CSI reporting based on AI/ML model also include two parts as a starting point. In our view, the CSI Part 2 based on AI/ML model should also be divided into N>1 groups to implement CSI omission. The question is how to divide the compressed quantization information into N groups.

Different from eType II codebook, AI/ML-based CSI compression feedback may not include indication information of SD basis, FD basis and non-zero coefficients. The legacy group method for eType II codebook cannot be directly adopted. Before providing the group method, it should firstly discuss the payload structure of CSI compression information. According to discussion in AI 9.2.2.1, for the two-sided AI/ML model, it includes layer-common model, layer-specific model, rank-common model and rank-specific. For different models, the payload structures of CSI compression information are different as well. In our view, the payload structure of layer-specific and layer-common is similar, while the structure of rank-specific and rank-common may be same. For layer-specific and layer-common, the payload structure can be the payload of layer by layer, or half payload of layer by layer which is arranged in one order.  For rank-specific and rank-common, the payload structure can be included two parts. One part is that half payload of one rank. The other part is that the remaining half payload of the rank.

Proposal 5: The compressed information located CSI part 2 is divided into N>1 groups for CSI omission, where the values N and how to divide into N groups depends on payload structures for different model types, e.g., layer-specific, layer-common, rank-specific or rank-common.

Performance monitoring

In [2]-[3], the following agreements on NW-side or UE-side performance monitoring were identified.

Agreement In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact for intermediate KPIs based monitoring including at least: - NW-side monitoring based on the target CSI with realistic channel estimation associated to the CSI report, reported by the UE or obtained from the UE-side. - UE-side monitoring based on the output of the CSI reconstruction model, subject to the aligned format, associated to the CSI report, indicated by the NW or obtained from the network side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - UE-side monitoring based on the output of the CSI reconstruction model at the UE-side - Note: CSI reconstruction model at the UE-side can be the same or different comparing to the actual CSI reconstruction model used at the NW-side. - Network may configure a threshold criterion to facilitate UE to perform model monitoring. - FFS: Other solutions, e.g., UE-side uses a model that directly outputs intermediate KPI. Network-side monitoring based on target CSI measured via SRS from the UE. Note: Monitoring approaches not based on intermediate KPI are not precluded Note: the study of intermediate KPIs based monitoring should take into account the monitoring reliability (accuracy), overhead, complexity, and latency. Agreement In CSI compression using two-sided model use case, for NW-side monitoring, further study the necessity, feasibility and potential specification impact to enable performance monitoring using an existing CSI feedback scheme as the reference. - The association between AI/ML scheme and existing CSI feedback scheme for monitoring - Note: The metric for monitoring and comparison includes intermediate KPI and eventual KPI. - Other aspects are not precluded. Agreement In CSI compression using two-sided model use case, for UE-side monitoring, further study potential specification impact on triggering and means for reporting the monitoring metrics, including periodic/semi-persistent and aperiodic reporting, and other reporting initiated from UE.

For UE-side model performance monitoring, it has agreed that UE-side monitoring can be based on the output of the CSI reconstruction model, and NW may configure a threshold criterion to facilitate UE to perform model monitoring. The remaining issues are how to deliver the output of the CSI reconstruction model, i.e., output-CSI-UE, and configure the threshold for UE-side monitoring.

UE side performance monitoring

For UE-side performance monitoring based on output-CSI-UE transmission from gNB to UE, the following issues needs to be studied.

- Issue 1: The procedures of output-CSI-UE transmission

Output-CSI-UE transmission can be initiated by UE if UE wants to monitor model performance. Then, UE will send signalling to require gNB transmits output -CSI-UE. The other way is that gNB indicates UE to implement performance monitoring and transmit output-CSI-UE to UE. UE will monitor model performance based on received indication signalling and output-CSI-UE. The time-domain behaviour of indicating UE implementing performance monitoring can be periodic, semi-persistent or aperiodic.

- Issue2: The method of output-CSI-UE transmission

Output-CSI-UE can be indicated to UE in an implicit or explicit method. The explicit method is that output-CSI-UE is transmitted to UE from gNB after output-CSI-UE quantization. The quantization method can reuse the approach of ground-truth CSI reporting, i.e., scalar quantization or codebook-based quantization. The implicit method is that output-CSI-UE is transmitted through beamforming CSI-RS. Assume output-CSI-UE is precoding matrix. Output-CSI-UE can be designed as the beam of CSI-RS ports. UE will calculate SINR or throughput based on estimated effective downlink channel through beamformed CSI-RS. Then, UE determines the model performance according to threshold and the calculated SINR or throughput.

- Issue3: The robust of UE-side performance monitoring based on output-CSI-UE

If UE determines the model performance based on intermediate or eventual KPI measured at a time, this may lead UE reporting a wrong alarm. This degraded intermediated or eventual KPI may be caused by degraded channel. In wireless communication scenario, it is possible that the channel of UE is complexed and variation due to environment change of UE experience. Thus, the robust of UE-side performance monitoring should be considered. In order to address issue, the following two options can be considered.

- Option1: Legacy CSI reporting, e.g., Rel-16 Type II codebook as a reference

- Option2: Performance monitoring based on the calculated intermediate or eventual KPIs of multiple different instances

The intermediated or eventual KPI can be calculated by using Option1.The calculated results can be referred to help UE provide reliable performance monitoring. But it will increase the computation complexity of UE. For Option 2, UE can determine model performance based on multiple measurement results for multiple different instances. This may improve the accuracy or robust performance of model monitoring.

Proposal 6: The following potential specification impact on UE-side monitoring based on output-CSI-UE should be studied:

- The procedure of output-CSI-UE transmission

- The method of output-CSI-UE transmission

- The robust of monitoring performance

Inactive model monitoring

Assume that multiple two-side part CSI compression models have been deployed in UE and gNB side. It is not necessary to active all these models. Usually, only one two-side model is active at a time due to hardware restriction or power saving.  This implies that some models are not active. One question is how to monitor these inactive models such that best model can be active once network needs. In our view, the following options can be considered to monitor inactive model.

- Option 1: Monitor inactive model at training server, e.g., OTT or OAM server.

- Option 2: Monitor inactive model at UE or NW side.

For Option 1, network or UE may indicate the training server to monitor those inactive models. Due to powerful capability of server, these inactive models can be monitored at the same by using the same input data of model or a single signalling. Then, the monitoring result calculated by server is sent to network for model activation, deactivation, fallback and so on. Since the input data of model may not be transferred through air interface, the signalling or data transmission overhead can be saved.

For Option 2, it needs to firstly make these inactive models be active for model inference. Due to hardware restriction, only one inactive model become active at a time. More signalling overhead may require for model activation. In order to compare fairly, the input data of these inactive model may need to use the same data. This implies that input data of these inactive models needs to be stored in memory, which will consume a lot of memory. For UE side or gNB side monitoring, the monitoring procedure or method proposed for performance monitoring can be reused. However, due to the impact of some other factors of UE, e.g., power consume, some inactive models cannot be supported any more. In order to save signalling overhead and reduce computation complexity, UE needs to report its states or supported inactive model. Those unsupported inactive models by UE will not be monitored. For Option 1 and Option2, the interaction between UE and gNB for monitoring inactive model is necessary. The details of the interaction or procedure can be further studied.

Proposal 7: The specification impact on how to monitor inactive model needs to study.

Remaining issues on specification impact for time domain CSI prediction

Performance monitoring and reporting

In the RAN1#114 meeting, the following agreement on performance monitoring for CSI prediction has been achieved [1].

Agreement For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM: - Type 1: - UE calculate the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at the network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 2: - UE reports predicted CSI and/or the corresponding ground truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 3: - UE calculate the performance metric(s) - UE report performance metric(s) to the NW - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Functionality selection/activation/ deactivation/switching what is defined for other UE side use cases can be reused, if applicable. - Configuration and procedure for performance monitoring - CSI-RS configuration for performance monitoring - Performance metric including at least intermediate KPI (e.g., NMSE or SGCS) - UE report, including periodic/semi-persistent/aperiodic reporting, and event driven report. - Note: down selection is not precluded. Note: UE may make decision within the same functionality on model selection, activation, deactivation, switching operation transparent to the NW.

For performance monitoring type 1 and type 2, UE needs to report performance monitoring output and performance metric. The remaining issue is how to the output and metric. For performance monitoring, the latency requirement can be relaxed. Usually, several tens milliseconds or a few seconds is enough for performance monitoring. Therefore, RRC signalling and/or L1 signalling can be considered as a container to enable identification of AI/ML model performance. For monitoring result reporting, legacy aperiodic or periodic report can be reused. UE-event report can be considered as well since the model is deployed at UE side and UE knows when to report monitoring results according to UE side measurement.

Proposal 8: Studying the following potential specification impact on performance monitoring output or performance metric report for model performance monitoring:

- RRC signaling and/or L1 signaling can be considered as a container to enable identification of AI/ML model performance

- Legacy aperiodic, periodic or UE-event can be considered to report performance monitoring output or performance metric.

Conclusions

In this contribution, the proposals on CSI compression feedback and time domain CSI prediction are summarised as follows:

Proposal 1: In CSI compression using two-sided model use case, the following table captures the pros/cons of training collaboration types 1:

Table 1： The pros and cons of Type 1

| Training types Characteristics | NW side type 1 |  | UE side type 1 |  |
| --- | --- | --- | --- | --- |
|  | Unknown model structure at UE | Known model structure at UE | Unknown model structure at NW | Known model structure at NW |
| Whether model can be kept proprietary | No | No | No | No |
| Whether require privacy-sensitive dataset sharing | No (Note2) | No (Note2) | No (Note2) | No (Note2) |
| Flexibility to support cell/site/scenario/configuration specific model | Yes | Yes | Semi-flexibility, if assistance information is supported. No otherwise. | Yes, if assistance information is supported. No otherwise |
| Whether gNB/device specific optimization is allowed | gNB: Yes UE: No | Yes (UE can compile the model) | gNB: No UE: Yes | Yes (gNB can compile the model) |
| Model update flexibility after deployment (note 3) | Flexible | Flexible | Flexible Less flexible than Type 1 NW side | Flexible Less flexible than Type 1 NW side |
| Feasibility of allowing UE side and NW side to develop/update models separately | No | No | No | No |
| Whether gNB can maintain/store a single/unified model over different UE vendors for a CSI report configuration | Yes | Yes | No | No |
| Whether UE device can maintain/store a single/unified model over different NW vendors for a CSI report configuration | No | No | Yes | Yes |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | No | No | Yes | Yes |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Yes | Yes | No | No |
| Whether training data distribution can match the inference device | gNB: Yes UE: No | gNB: Yes UE: No | UE: Yes gNB: No | UE:Yes gNB: No |
| Software/hardware compatibility (Whether device capability can be considered for model development) | No | Yes | No | Yes |
| Model performance based on evaluation in 9.2.2.1 | Performance refers to 9.2.2.1 observations | Performance refers to 9.2.2.1 observations | Performance refers to 9.2.2.1 observations | Performance refers to 9.2.2.1 observations |

Proposal 2: The pros and cons of collaboration training type 2 and type 3 are respectively provided in Table 2.

Table 2： The pros and cons of type 2 and type 3

| Training types Characteristics | Type 2 |  | Type 3 |  |
| --- | --- | --- | --- | --- |
|  | Simultaneous | Sequential NW first (Note 1) | NW first | UE first |
| Whether model can be kept proprietary | Yes (note 2) | Yes (note 2) | Yes (note 2) | Yes (note 2) |
| Whether require privacy-sensitive dataset sharing | No (Note 3) | No (Note 3) | No (Note 3) | No (Note 3) |
| Flexibility to support cell/site/scenario/configuration specific model | Difficult. | Semi-flexible. Less flexible compared to type 3. | Semi-flexible. | Semi-flexible. If assistance information is supported. Not flexible otherwise. |
| Whether gNB/device specific optimization is allowed | Yes | Yes | Yes | Yes |
| Model update flexibility after deployment (note 4) | Not flexible | Semi-flexible. Less flexible compared to type 3. | Semi-flexible | Semi-flexible |
| Feasibility of allowing UE side and NW side to develop/update models separately | Infeasible | Feasible | Feasible | Feasible |
| Whether gNB can maintain/store a single/unified model over different UE vendors for a CSI report configuration | Yes. Performance loss refers to 9.2.2.1 observations | Yes. Performance loss refers to 9.2.2.1 observations | Yes. Performance loss refers to 9.2.2.1 observations | Yes. Performance loss refers to 9.2.2.1 observations |
| Whether UE device can maintain/store a single/unified model over different NW vendors for a CSI report configuration | Yes. Performance loss refers to 9.2.2.1 observations | Yes. Performance loss refers to 9.2.2.1 observations | Yes. Performance loss refers to 9.2.2.1 observations | Yes. Performance is same as that of one UE device and one NW vendor with UE side separate training |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | Not support | Support | Support | Not support; Support if NW side part model is fixed during training. |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Not support | Not support | Not support; | Support |
| Whether training data distribution can match the inference device | More limited | Limited | Limited | Yes |
| Software/hardware compatibility (Whether device capability can be considered for model development) | Compatible | Compatible | Compatible | Compatible |
| Model performance based on evaluation in 9.2.2.1 | Performance refers to 9.2.2.1 observations | Performance refers to 9.2.2.1 observations | Performance refers to 9.2.2.1 observations | Performance refers to 9.2.2.1 observations |

Proposal 3: The potential specification impact on training dataset delivery methods, dataset format or type need to study for collaboration training type 3.

Proposal 4: If there are multiple candidate part models at UE side and gNB side, the interactions or procedures for model selection between UE and gNB should be studied.

Proposal 5: The compressed information located CSI part 2 is divided into N>1 groups for CSI omission, where the values N and how to divide into N groups depends on payload structures for different model types, e.g., layer-specific, layer-common, rank-specific or rank-common.

Proposal 6: The following potential specification impact on UE-side monitoring based on output-CSI-UE should be studied:

- The procedure of output-CSI-UE transmission

- The method of output-CSI-UE transmission

- The robust of monitoring performance

Proposal 7: The specification impact on how to monitor inactive model needs to study.

Proposal 8: Studying the following potential specification impact on performance monitoring output or performance metric report for model performance monitoring:

- RRC signaling and/or L1 signaling can be considered as a container to enable identification of AI/ML model performance

- Legacy aperiodic, periodic or UE-event can be considered to report performance monitoring output or performance metric.

References

- 3GPP RAN1#114 meeting, Chairman’s notes.

- 3GPP RAN1#112bis e-meeting, Chairman’s notes.

- 3GPP RAN1#110bis e-meeting, Chairman’s notes.

- 3GPP RAN1#113 meeting, Chairman’s notes.