3GPP TSG RAN WG1 #114bis		R1-2309522

Xiamen, China, October 9th – October 13th, 2023

Source:	CATT, CICTCI

Title:	Maintenance on NR sidelink CA operation

Agenda Item:	8.2.3

Document for:	Discussion and Decision

Introduction

In RAN#101 meeting, it is announced that RAN1 work on NR sidelink evolution WI has been done [1]. But further discussions are still needed at the maintenance stage. In this contribution, we will discuss the remaining issues on NR SL CA.

Remaining issues on NR SL CA

Remaining issues on resource selection procedure

In RAN1#114 meeting, the following agreement was achieved that resource selection procedure is performed independently per carrier [2].

Agreement In NR SL CA, Rel-16/17 SL resource (re)selection procedure is independently performed for each SL carrier.

In LTE SL CA, such feature is captured as the highlight part in TS 36.213, so a similar description can be added in the spec of NR SL CA.

In sidelink transmission mode 4, when requested by higher layers in subframe n for a carrier, the UE shall determine the set of resources to be reported to higher layers for PSSCH transmission according to the steps described in this Clause.

The following TP is proposed.

- TP#1

| Reason for change: | Corrections on NR sidelink evolution. |
| --- | --- |
| Summary of change: | In clause 8.1.4, adding the description that resource selection procedure is performed independently per carrier. |
| Consequences if not approved: | Previous agreement is not captured. |

------------------------------------------ Start of text proposal to TS 38.214 v18.0.0------------------------------------------

<<< UNCHANGED PARTS OMITTED >>>

8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission for a carrier. To trigger this procedure, in slot n for this carrier, the higher layer provides the following parameters for this PSSCH/PSCCH transmission:

<<< UNCHANGED PARTS OMITTED >>>

-------------------------------------------End of text proposal to TS 38.214 v18.0.0------------------------------------------

Proposal 1: Modify the description of current specification regarding resource selection procedure and adopt TP#1.

Remaining issues on power control for PSCCH/PSSCH transmission

In RAN1#114 meeting, the following agreement was achieved regarding power control for PSCCH/PSSCH transmission [2].

Agreement To reuse LTE SL CA PSCCH/PSSCH power control for NR SL CA PSCCH/PSSCH power control across all the aggregated SL carriers, - The existing PSCCH/PSSCH power control in Rel-16/17 is used for PSCCH/PSSCH power control for each SL carrier.

In legacy NR SL, the power control on PSCCH transmission and PSSCH transmission is performed jointly, where the power of PSCCH transmission is determined based on the power of PSSCH transmission, i.e., the fully independent power control on PSCCH transmission or PSSCH transmission is not supported. In current spec of NR SL CA, the “PSCCH(s) or PSSCH(s)” is used in several places, which are highlighted in the follows.

If a UE would transmit PSSCHs and PSCCHs on multiple carriers, the UE determines a power for each PSSCH and PSCCH transmission as described in Clauses 16.2.1 and 16.2.2, respectively. If the UE would transmit PSCCHs or PSSCHs that would overlap in time on respective carriers and a total power for the transmission of the PSCCHs or PSSCHs would exceed PCMAX, the UE reduces a power for a transmission of a PSCCH or PSSCH that has the largest priority value as determined by SCI formats provided by the PSCCHs scheduling the respective PSSCHs. If more than one PSCCH/PSSCH transmissions have the largest priority value, the UE autonomously selects one of the more than one PSCCH/PSSCH transmissions to reduce a respective power. If, after the reduction of the power for the transmission of the PSCCH or the PSSCH with the largest priority value, a total power does not exceed PCMAX, the UE transmits the PSCCHs or the PSSCHs, respectively. If, after the reduction of the power of the PSCCH or the PSSCH with the largest priority value, a total power exceeds PCMAX, the UE drops the PSCCH or the PSSCH with the largest priority value, respectively, and repeats the procedure over the remaining PSCCHs or PSSCHs.

If the “PSCCH(s) or PSSCH(s)” is used, it may lead to some misunderstanding, e.g. the UE can reduce the transmission power of either PSCCH or PSSCH to meet the PCMAX limitation, and UE can drop the transmission of either PSCCH or PSSCH to  meet the PCMAX limitation. If this is the situation, then the UE may transmit PSCCH only or PSSCH only according to the “PSCCH(s) or PSSCH(s)” description. We think it would be better to avoid this ambiguous issue. Since the “PSCCH-PSSCH” has been used in section 16.2 of TS 38.213, it is preferred to replace the “PSCCH(s) or PSSCCH(s)” by “PSCCH-PSSCH(s)” to avoid the ambiguous issue.

The following TP is proposed.

- TP#2

| Reason for change: | Corrections on NR sidelink evolution. |
| --- | --- |
| Summary of change: | In clause 16.2.5, makings some updates on the description of “PSCCH(s) or PSSCH(s)”. |
| Consequences if not approved: | Causing some ambiguities for PSCCH/PSSCH power control and PSCCH/PSSCH transmission. |

------------------------------------------ Start of text proposal to TS 38.213 v18.0.0------------------------------------------

<<< UNCHANGED PARTS OMITTED >>>

16.2.5	SL Carrier Aggregation

If a UE would transmit PSCCH-PSSCHs on multiple carriers, the UE determines a power for each PSSCH and PSCCH transmission as described in Clauses 16.2.1 and 16.2.2, respectively. If the UE would transmit PSCCH-PSSCHs that would overlap in time on respective carriers and a total power for the transmission of the PSCCH-PSSCHs would exceed PCMAX, the UE reduces a power for a transmission of a PSCCH-PSSCH that has the largest priority value as determined by SCI formats provided by the PSCCHs scheduling the respective PSSCHs. If more than one PSCCH/PSSCH transmissions have the largest priority value, the UE autonomously selects one of the more than one PSCCH/PSSCH transmissions to reduce a respective power. If, after the reduction of the power for the transmission of the PSCCH-PSSCH with the largest priority value, a total power does not exceed PCMAX, the UE transmits the PSCCH-PSSCHs, respectively. If, after the reduction of the power of the PSCCH-PSSCH with the largest priority value, a total power exceeds PCMAX, the UE drops the PSCCH-PSSCH with the largest priority value, respectively, and repeats the procedure over the remaining PSCCH-PSSCHs.

<<< UNCHANGED PARTS OMITTED >>>

-------------------------------------------End of text proposal to TS 38.213 v18.0.0------------------------------------------

Proposal 2: Modify the description of current specification regarding power control for NR SL CA and adopt TP#2.

Conclusion

In this contribution, the design aspects on NR SL CA are discussed. Partially, we have the following proposals.

Proposal 1: Modify the description of current specification regarding resource selection procedure and adopt TP#1.

Proposal 2: Modify the description of current specification regarding power control for NR SL CA and adopt TP#2.

References

- RP-231583, Status report for WI NR sidelink evolution, OPPO, Sep. 2023.

- 3GPP TSG RAN WG1 Meeting RAN1 #114 Chairman's Notes, Aug. 2023.