3GPP TSG RAN WG1 #114bis	R1-2309713

Xiamen, China, October 9th  – 13th, 2023

Source:	Panasonic

Title: 	Evaluation results on NR-NTN for self-evaluation of IMT-2020 satellite radio interface technology

Agenda Item:		8.15.2

Document for:	Discussion and Decision

Introduction

Study item on Self-Evaluation towards the 3GPP submission of an IMT-2020 Satellite Radio Interface Technology was agreed in RAN#99 [1]. The aim of SI is to provide the description of the self-evaluation results towards IMT-2020 submission to ITU-R WP 4B against the technical performance requirements defined by Report ITU-R M.2514. Our initial evaluation results were submitted in RAN1#114. In this document, we update our evaluation results based on the latest evaluation assumptions agreed in RAN1#114.

Peak spectral efficiency and peak data rate

The peak spectral efficiency and peak data rate are defined as the maximum values under ideal conditions in ITU-R M.2514. The requirement is as follows.

Table 1 Peak spectral efficiency requirement

| Peak spectral efficiency (DL) | 3 bit/s/Hz (1) |
| --- | --- |
| Peak spectral efficiency (UL) | 1.5 bit/s/Hz (1) |
| (1) Requirements were derived using an assignable bandwidth of up to 30 MHz over one satellite beam. |  |

Table 2 Peak data rate requirement

| Peak data rate (DL) | 70 Mbit/s (1) |
| --- | --- |
| Peak data rate (UL) | 2 Mbit/s (1) |
| (1) Requirements were derived using an assignable bandwidth of up to 30 MHz over one satellite beam. |  |

The following evaluation assumptions were agreed in RAN1#112b/113.

- Reference scenario:

Transparent payload

S-band (2GHz)

LEO-600

Handheld UEs

- Peak spectral efficiency is calculated as the following equation

SEp=vLayers⋅Qm⋅f⋅RmaxNPRBBW,μ⋅12Tsμ1-OHBW

- The parameters are chosen based on “ideal conditions”: 90degree elevation angle, 0dB atmospheric loss, 0dB shadow fading margin, 0dB scintillation loss, 0dB polarization loss, 0dB additional losses.

- For uplink peak spectral efficiency and peak data rate, a bandwidth of 8 RBs is used.

- For downlink peak spectral efficiency and peak data rate, a transmission bandwidth of 160 PRBs out of a channel bandwidth of 30 MHz is used. Channel bandwidth of 30 MHz is used as denominator for calculation of spectrum efficiency

Regarding the overhead, 0.14 for DL and 0.08 for UL was agreed in RAN1#114.

We have evaluated the peak spectral efficiency and peak data rate using above assumptions. CNR (dB) is calculated based on the link budget calculations and achievable SE for the CNR is derived. We confirm the peak spectral efficiency and peak data rate requirements are satisfied.

Table 3 Link budget and SE calculation

|  | Downlink | Uplink |
| --- | --- | --- |
| Frequency [GHz] | 2.00 | 2.00 |
| Elevation angle [deg] | 90 | 90 |
| Distance [km] | 600 | 600 |
| Bandwidth [MHz] | 30 | 1.44 |
| TX: EIRP [dBm] | 78.8 | 23.0 |
| RX: G/T [dB/T] | -31.6 | 1.1 |
| Free space path loss [dB] | 154.0 | 154.0 |
| Atmospheric loss [dB] | 0.0 | 0.0 |
| Shadow fading margin [dB] | 0.0 | 0.0 |
| Scintillation Loss [dB] | 0.0 | 0.0 |
| Polarization loss [dB] | 0.0 | 0.0 |
| Additional losses [dB] | 0.0 | 0.0 |
| CNR [dB] | 16.95 | 7.08 |
| MCS | 25 (64QAM, R=822/1024) | 13 (16QAM, R=490/1024) |
| SE (w/ overhead) [bit/s/Hz] | 3.71 | 1.58 |
| Peak data rate (Mbps) | 111.3 | 2.27 |

Evaluation of average spectral efficiency, 5%-tile user spectral efficiency, user experienced data rate and area traffic capacity

User experienced data rate is calculated as Ruser=W×SEuser where SEuser is the 5%-tile user spectral efficiency and W is the channel bandwidth. Area traffic capacity is calculated as Carea=ρ×W×SEavg where SEavg is the average spectral efficiency.

The requirements for DL and UL are shown in Table 4 and Table 5, respectively.

Table 4 Requirements for DL

| Average spectral efficiency (DL) | 0.5 bit/s/Hz |
| --- | --- |
| 5th percentile user spectral efficiency (DL) | 0.03 bit/s/Hz |
| User experienced data rate (DL) | 1 Mbit/s |
| Area traffic capacity (DL) | 8 kbit/s/km2 |

Table 5 Requirements for UL

| Average spectral efficiency (UL) | 0.1 bit/s/Hz |
| --- | --- |
| 5th percentile user spectral efficiency (UL) | 0.003 bit/s/Hz |
| User experienced data rate (UL) | 100 kbit/s |
| Area traffic capacity (UL) | 1.5 kbit/s/km2 |

The SLS assumptions (as agreed in the previous meetings) are shown in Table 6. Satellite and UE configurations are same as TR38.821 and shown in Annex A.

Table 6 eMBB SLS parameters

| Parameter | Value |
| --- | --- |
| Frequency band | S-band (i.e. 2 GHz) |
| Maximum Bandwidth per beam (DL + UL) | S-band: DL 30 MHz and UL 30 MHz For FRF=3, 30 MHz is the total maximum bandwidth (i.e., each beam will have 10 MHz maximum bandwidth). |
| Subcarrier spacing | 15 kHz |
| Satellite antenna | Circular |
| Frequency re-use factor | FRF=1 or 3 |
| Polarization re-use | Disable |
| Channel model | Large scale model of Section 6.6 in 38.811 (NOTE2) LOS probability is assumed 100%. |
| Deployment scenarios | Rural, UE speed 3 km/h |
| Propagation conditions | Clear Sky and Line of Sight |
| Beam layout | As described in 6.1.1.1 of TR 38.821 For FRF = 1, two additional tiers of beams are considered in the simulation surrounding the 19-beam layout. For FRF > 1, four additional tiers of beams are considered in the simulation surrounding the 19-beam layout. Center beam elevation 90 degree |
| UEs outdoor/indoor distribution | 100% outdoor distribution for UEs |
| UE distribution | 10 UEs per beam with uniform distribution in all the Voronoi cell area associated to each beam.The cell area associated to a given beam is defined as the Voronoi cell associated with the corresponding beam centers. |
| UE orientation | Random |
| UE antenna height | 1.5 m |
| Handover Margin | 0 dB |
| UE attachment | RSRP |
| Scheduler | PF |
| Traffic model | Full buffer |
| Number of HARQ processes | Up to 32 (as in Rel.17) |
| Receiver type | MMSE-IRC |
| UE speed | 3 km/h |
| Frequency offset | 0 ppm |
| Frequency drift | 0 ppm (perfect pre-compensation) |
| HARQ-ACK delay | N+4 slot |
| Retransmission delay | 26(RTT)+4 slot (assuming the maximum RTT for LEO 600km) |
| CSI feedback | CQI, 20 ms periodicity |
| CSI delay | 26 slots |
| Overhead | Same as peak rate calculation (DL 14%, UL 8%) |

Geometry distribution obtained by SLS is shown in Figure 1. This is in line with the geometry obtained in the NTN study item phase.

FRF=1

FRF=3

Figure 1 geometry distribution

< Downlink >

SLS results for DL 5%-tile user spectral efficiency, user experienced data rate, average user spectral efficiency and area traffic capacity for frequency reuse factor 1 (FRF 1) and 3 (FRF 3) are show in Table 7. Area traffic capacity is calculated based on the area of 1415 km2 as agreed in RAN1#114.

FRF 3 can achieve higher 5%-tile user spectral efficiency than FRF 1 because of the interference mitigation effect, but average user spectral efficiency is lower because only 1/3 of the channel bandwidth is available per cell.

All values satisfy the requirements.

Table 7 SLS results (DL)

| DL | 5%-tile user spectral efficiency (bps/Hz) | User experienced data rate (Mbps) | Average user spectral efficiency (bps/Hz) | Area traffic capacity (kbps/km2) |
| --- | --- | --- | --- | --- |
| Requirement | 0.03 | 1 | 0.5 | 8 |
| FRF 1 | 0.043 | 1.28 | 0.80 | 16.96 |
| FRF 3 | 0.047 | 1.41 | 0.65 | 13.78 |

FRF=1

FRF=3

Figure 2 CDF of User throughput for DL FRF 1 and FRF 3

< Uplink >

SLS results for UL average user spectral efficiency, 5%-tile user spectral efficiency and user experienced data rate are show in Table 7. For UL, FRF 3 can achieve average user spectral efficiency than FRF 1 unlike DL. This is because bandwidth reduction due to FRF 3 does not likely influence the throughput since less PRBs are used per UE due to lack of transmission power.

All values satisfy the requirements.

Table 8 SLS results (UL)

| UL | 5%-tile user spectral efficiency (bps/Hz) | User experienced data rate (kbps) | Average user spectral efficiency (bps/Hz) | Area traffic capacity (kbps/km2) |
| --- | --- | --- | --- | --- |
| Requirement | 0.003 | 100 | 0.1 | 1.5 |
| FRF 1 | 0.007 | 209 | 0.13 | 2.76 |
| FRF 3 | 0.012 | 360 | 0.21 | 4.45 |

FRF=1

FRF=3

Figure 3 CDF of User throughput for UL FRF 1 and FRF 3

Reliability evaluation

Reliability requirement for IMT-2020 satellite component is 1-10−3 (i.e. 99.9%) for both DL and UL. The following steps based on the ITU evaluation guideline [2] are used for the evaluation.

1. Run the full buffer system level simulation and obtain 5%-tile SINR value.

2. Run the link level simulation and obtain the success probability at the 5%-tile SINR.

3. Check if the success probability is higher or equal to the requirement, 1-10−3 (i.e. 99.9%).

Downlink reliability

Reliability for downlink is evaluated. Evaluation assumptions (based on the agreement) for downlink LLS are shown in Table 10.

Table 10  downlink LLS parameters for reliability

| NR Downlink | Values |
| --- | --- |
| Physical channel | PDSCH (OFDM) |
| Simulation bandwidth | 8 PRB |
| Number of users in simulation | 1 |
| Link-level Channel model | NTN TDL-C Rural, elevation angle 90 degree lgDS = -8.19, K = 3.59 |
| Antenna configuration at Satellite | 1Tx |
| Antenna configuration at UE | 1Rx |
| Transmission mode | SISO |
| Transmission rank | 1 |
| TBS | 256 |
| Modulation order | QPSK |
| Number of repetitions | Up to 8 |
| Channel estimation | LMMSE |
| Channel coding scheme | LDPC |
| HARQ | Disable/Enable |
| Doppler spread | 5Hz |
| DL DMRS config | Type 1, 2 symbol DMRS |

5%-tile SINR values obtained by SLS for FRF=1 and FRF=3 are shown below.

| Frequency reuse factor | 5%-tile SINR |
| --- | --- |
| 1 | -3.17 dB |
| 3 | 6.61 dB |

BLER performance is obtained by the LLS. The success rate for the 5%-tile SINR is shown in Table 12. For FRF 1, the requirement of the success rate 99.9% (1-10-3) is satisfied by 2 repetitions with HARQ up to 8 transmissions. For FRF 3, it is satisfied by 8 repetitions without HARQ.

Table 12 PDSCH success rate for the 5%-tile SINR

| DL | 5%-tile SINR (dB) | Repetition | HARQ | Success rate |
| --- | --- | --- | --- | --- |
| Requirement |  |  |  | 99.9% |
| FRF 1 | -3.17 | 2 | Enable (up to 8Tx) | 99.98% |
| FRF 3 | 6.61 | 8 | Disable | 99.92% |

Uplink reliability

Reliability for downlink is evaluated. Evaluation assumptions (based on the agreement) for downlink LLS are shown in Table 10.

Table 10  uplink LLS parameters for reliability

| NR Uplink | Values |
| --- | --- |
| Physical channel | PUSCH (DFT-s-OFDM) |
| Simulation bandwidth | 2 PRB |
| Number of users in simulation | 1 |
| Link-level Channel model | NTN TDL-C Rural, elevation angle 90 degree lgDS = -8.19, K = 3.59 |
| Antenna configuration at Satellite | 1Tx |
| Antenna configuration at UE | 1Rx |
| Transmission mode | SISO |
| Transmission rank | 1 |
| TBS | 256 |
| Modulation order | QPSK |
| Number of repetitions | Up to 32 |
| Frequency hopping | ON |
| Channel estimation | LMMSE |
| Channel coding scheme | LDPC |
| HARQ | Disable/Enable |
| Doppler spread | 5Hz |
| DL DMRS config | Type 1, 2 symbol DMRS |

5%-tile SINR values obtained by SLS for FRF=1 and FRF=3 are shown below.

| Frequency reuse factor | 5%-tile SINR |
| --- | --- |
| 1 | -4.02 dB |
| 3 | 6.72 dB |

BLER performance is obtained by the LLS. The success rate for the 5%-tile SINR is shown in Table 12. For FRF 1, the requirement of the success rate 99.9% (1-10-3) is satisfied by 32 repetitions with HARQ up to 8 transmissions. For FRF 3, it is satisfied by 8 repetitions with HARQ up to 4 transmissions.

Table 12 PUSCH success rate for the 5%-tile SINR

| UL | 5%-tile SINR (dB) | Repetition | HARQ | Success rate |
| --- | --- | --- | --- | --- |
| Requirement |  |  |  | 99.9% |
| FRF 1 | -4.02 | 32 | Enable (up to 8Tx) | 99.95% |
| FRF 3 | 6.72 | 8 | Enable (up to 4Tx) | 99.92% |

Conclusion

This contribution provides our initial evaluation results for self-evaluation of IMT-2020 satellite radio interface technology. The following performance metrics are covered in this document. All evaluation results satisfy the requirements.

  - Peak data rate

  - Peak spectral efficiency

  - User experienced data rate

  - 5th percentile user spectral efficiency

  - Average spectral efficiency

  - Area traffic capacity

  - Reliability

The evaluation results are summarized below.

- Peak spectral efficiency and Peak data rate

Downlink

| DL | Peak spectral efficiency [bit/s/Hz] | Peak data rate (Mbps) |
| --- | --- | --- |
| Requirement | 3 | 70 |
| DL | 3.71 | 111.3 |

Uplink

| UL | Peak spectral efficiency [bit/s/Hz] | Peak data rate (Mbps) |
| --- | --- | --- |
| Requirement | 1.5 | 2 |
| UL | 1.58 | 2.27 |

- 5%-tile user spectral efficiency, User experienced data rate, Average spectral efficiency and Area traffic capacity

Downlink

| DL | 5%-tile user spectral efficiency (bps/Hz) | User experienced data rate (Mbps) | Average user spectral efficiency (bps/Hz) | Area traffic capacity (kbps/km2) |
| --- | --- | --- | --- | --- |
| Requirement | 0.03 | 1 | 0.5 | 8 |
| FRF 1 | 0.043 | 1.28 | 0.80 | 16.96 |
| FRF 3 | 0.047 | 1.41 | 0.65 | 13.78 |

*FRF: Frequency Reuse Factor

Uplink

| UL | 5%-tile user spectral efficiency (bps/Hz) | User experienced data rate (kbps) | Average user spectral efficiency (bps/Hz) | Area traffic capacity (kbps/km2) |
| --- | --- | --- | --- | --- |
| Requirement | 0.003 | 100 | 0.1 | 1.5 |
| FRF 1 | 0.007 | 209 | 0.13 | 2.76 |
| FRF 3 | 0.012 | 360 | 0.21 | 4.45 |

- Reliability

Downlink

| DL | 5%-tile SINR (dB) | Repetition | HARQ | Success rate |
| --- | --- | --- | --- | --- |
| Requirement |  |  |  | 99.9% |
| FRF 1 | -3.17 | 2 | Enable (up to 8Tx) | 99.98% |
| FRF 3 | 6.61 | 8 | Disable | 99.92% |

Uplink

| UL | 5%-tile SINR (dB) | Repetition | HARQ | Success rate |
| --- | --- | --- | --- | --- |
| Requirement |  |  |  | 99.9% |
| FRF 1 | -4.02 | 32 | Enable (up to 8Tx) | 99.95% |
| FRF 3 | 6.72 | 8 | Enable (up to 4Tx) | 99.92% |

- Reference

- RP-230736, “New SID: Study on Self-Evaluation towards the 3GPP submission of an IMT-2020 Satellite Radio Interface Technology”, Ericsson, Qualcomm, Thales, MediaTek

- ITU-R M.2514

- TR38.821 V16.0.0

- TR38.811 V15.4.0

- Annex A: Satellite and UE configuration for SLS

Satellite configuration

Satellite parameter set 1 for LEO 600km in TR38.821 (copied below)

Table 9 Satellite characteristics (parameter set 1 of LEO600km, S-band)

| Parameters | Value |
| --- | --- |
| Satellite altitude | 600km |
| Satellite antenna pattern | Section 6.4 in TR38.811 [4] |
| Parameters for DL transmission |  |
| Equivalent satellite antenna aperture | 2 m |
| Satellite EIRP density | 34 dBW/MHz |
| Satellite Tx max Gain | 30 dBi |
| 3dB beamwidth | 4.4127 deg |
| Satellite beam diameter (Note 2) | 50 km |
| Parameters for UL reception |  |
| Equivalent satellite antenna aperture | 2 m |
| G/T | 1.1 dB K-1 |
| Satellite Rx max Gain | 30 dBi |

Terminal configuration

Handheld terminal described in TR38.821. in addition, 4Rx case can also be evaluated.

Table 10 Terminal characteristics (handheld terminal)

| Parameters | Value |
| --- | --- |
| Frequency band | S band (i.e. 2 GHz) |
| Antenna type and configuration | (1, 1, 2) with omni-directional antenna element |
| Polarisation | Linear: +/-45°X-pol |
| Rx Antenna gain | 0 dBi per element |
| Antenna temperature | 290 K |
| Noise figure | 7 dB |
| Tx transmit power | 200 mW (23 dBm) |
| Tx antenna gain | 0 dBi per element |