3GPP TSG RAN WG1 #114bis		        R1-2309864

Xiamen, China, October 9th – October 13th, 2023

Source: 	Apple Inc.

Title:                     On UE features for less than 5MHz FR1

Agenda item:	8.16.9

Document for:	Discussion and Decision

# 1 Introduction

In this contribution, we provide our views on the open issues of UE features for dedicated spectrum less than 5MHz in accordance with the agreements made in RAN1 114 e-meeting.

# Discussions

New Feature Group 51-z

In RAN1 114 meeting, the following was agreed to support 20-PRBs CORESET #0 configuration for 5MHz channel BW [1]:

Agreement For 5MHz channel BW, 20 PRBs CORESET#0 transmission BW is supported. - The upper 4 PRBs of the 𝑁RB CORESET = 24 CORESET#0 are punctured to obtain 20 PRBs CORESET#0. - Table 13-0 is used for configuring 20 PRBs CORESET#0 - Maximum number of CORESET#0 symbols is 3. Minimum number of CORESET#0 symbols is 2. - PRB offset = 0 - Only interleaved CCE to REG mapping is supported. - REG bundle size = 6 - Kssb follows legacy configuration. - Note: The 20 PRBs CORESET#0 is only valid for the new sync. raster (=921.45 MHz) for band n100, 5MHz channel BW - Note 1: UE can be configured with 20 PRBs BWP for UE capable of 20 PRBs CORESET#0

A corresponding UE feature needs to be added to indicate whether UE supports it or not.

We therefore proposed:

Proposal 1: To add the following FG 51-z for 20 RB CORESET 0 reception

| Features | Index | Feature group | Components | Prerequisite feature groups | Type | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 51. NR_FR1_lessthan_ 5MHz_BW | 51-z | Support 20 RB CORESET0 for 5MHz channel bandwidth | 1) Reception of 12 RB CORESET0 |  | Per Band | Optional with capability signalling |

FG 51-x

In RAN1 114 meeting, there were discussions regarding the need of new components for FG 51-x to describle the corresponding synchronization rasters without consesus. Note that the FG 51-x and 51-y are sort of ‘implicit’ UE capabilities as they are defined for RRC_IDLE UEs for cell access procedure and can not be indicated as part of UE capability report. Instead, a UE implicitly indicates the support of a respective FG by searching the predefined synchronization rasters for system acqusition. More imporantly, these new CORESET #0 configurations should be strictly limited to the new synchronization rasters defined by RAN4 for the target bands and prohabit using them on the legacy synchronization rasters and causing any impact on the legacy UEs. We prefer to clarify this in ‘Note’ column of FG 51-x and FG 51-y to avoid any mis-understanding and debating in the furture.

We therefore proposed:

Proposal 2: Support to add the following in the ‘Note’ column for FG 51-x:

- Reception of 15 RB CORESET0 at the sync-raster points derived from the new sync-raster formula as defined in TS 38.101, for n100, n106, n26, n28 and n85

FG 51-y

The simliar ‘note’ needs to be added for FG 51-y with the same reason:

Proposal 3: Support to add the following in the ‘Note’ column for FG 51-y:

- Reception of 12 RB CORESET0 at one sync-raster point defined in NR band n100 (i.e., 920.73 MHz)”

There is one highlighted FFS aspect in the ‘Note’ column for FG 51-y as follows:

This FG is supported for 15 kHz SCS only [This FG is applicable to the case when transmission BW is limited within 12 PRB]

With adding the note proposed earlier, we do not see the need of keeping the highlighted sentence as 12-RB CORESET is limited to be used for NR band n100 where the single sync-raster point was newly defined.

We therefore proposed:

Proposal 4: Delete the following hightlighted Note:

This FG is supported for 15 kHz SCS only

Others

One more FFS aspect for all these FGs are the applicability for Redcap UEs. Note that Redcap UE was introduced in Rel-17 and is capable of 20MHz BW in FR1. Same as other Rel-18 features support, we do not see any justification to exclude Redcap UE from support these new Rel-18 features, especailly considering the channel BW is up to 5MHz that is signficiantly smaller than Redcap UE capability.

Proposal 5: The UE features 51-x/51-y/51-z are applicable for Redcap UEs.

# 3. Conclusion

In the previous sections, we have discussed different open issues of UE features for less than 5MHz BW. Based on the discussions above, the following proposals were made:

Proposal 1: To add the following FG 51-z for 20 RB CORESET 0 reception

| Features | Index | Feature group | Components | Prerequisite feature groups | Type | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- |
| 51. NR_FR1_lessthan_ 5MHz_BW | 51-z | Support 20 RB CORESET0 for 5MHz channel bandwidth | 1) Reception of 12 RB CORESET0 |  | Per Band | Optional with capability signalling |

Proposal 2: Support to add the following in the ‘Note’ column for FG 51-x:

- Reception of 15 RB CORESET0 at the sync-raster points derived from the new sync-raster formula as defined in TS 38.101, for n100, n106, n26, n28 and n85

Proposal 3: Support to add the following in the ‘Note’ column for FG 51-y:

- Reception of 12 RB CORESET0 at one sync-raster point defined in NR band n100 (i.e., 920.73 MHz)”

Proposal 4: Delete the following hightlighted Note:

This FG is supported for 15 kHz SCS only

Proposal 5: The UE features 51-x/51-y/51-z are applicable for Redcap UEs.

# References

- R1-2308521	Updated RAN1 UE features list for Rel-18 NR after RAN1#114	Moderators (AT&T, NTT DOCOMO, INC.)

- NR_FR1_lessthan_5MHz_BW

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 51. NR_FR1_lessthan_5MHz_BW | 51-x | Support for 3 MHz channel bandwidth | 1) Reception of 12 RB PBCH based on RB-level puncturing 2) Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS 3) Reception of 15 RB and CORESET0 |  | Yes | N/A | UE is not able to support 3 MHz channel bandwidth | Per Band | FDD only | FR1 only | N/A | This FG is supported for 15 kHz SCS only | Optional with capability signalling |
| 51. NR_FR1_lessthan_5MHz_BW | 51-y | Support 12 RB CORESET0 | 1) Reception of 12 RB CORESET0 | 51-x | Yes | N/A | UE is not able to support 3 MHz channel bandwidth with 12 RB CORESET0 | Per Band | FDD only | FR1 only | N/A | This FG is supported for 15 kHz SCS only [This FG is applicable to the case when transmission BW is limited within 12 PRB] | Optional with capability signalling |