3GPP TSG-RAN WG1 Meeting #114bis	R1-2310085

Xiamen, China, October 9th – October 13th, 2023

Agenda Item:	8.1.2

Source:	Ericsson

Title:	Remaining issues on CSI enhancements for Rel-18 NR MIMO evolution

Document for:	Discussion, Decision

# 1	Introduction

In [1], the Rel-18 work item for NR MIMO evolution was agreed. The following two objectives of the work item concern CSI enhancements:

- Study, and if justified, specify CSI reporting enhancement for high/medium UE velocities by exploiting time-domain correlation/Doppler-domain information to assist DL precoding, targeting FR1, as follows:

  - Rel-16/17 Type-II codebook refinement, without modification to the spatial and frequency domain basis

  - UE reporting of time-domain channel properties measured via CSI-RS for tracking

…

4. Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

In this contribution, we discuss some remaining issues on TRS based TDCP reporting, CSI enhancement for high/ medium UE velocities, and CSI enhancement for coherent JT.

# 2	TRS-based time-domain channel property reporting

In the RAN4 LS R1-2308807, RAN4 mentions the following RAN4 agreements:

- If receiver diversity is used, the method to combine TDCP measurement across RX branches shall be left to UE implementation. The reported TDCP amplitude is expected to be no lower than the minimum and no higher than the maximum measured values across the Rx branches if receiver diversity is used.

- RAN4 is discussing the feasibility of introducing accuracy requirements for TDCP amplitude measurement. Based on the conclusion of the feasibility study, if further update is needed on the definition, RAN4 shall inform RAN1 in future meetings.

The second sentence of the first bullet defines the expected behaviour for TDCP amplitude reporting if receiver diversity is used.  We propose that this expected behaviour be captured as part of the TDCP measurement definition in TS 38.215.

###############  Start of TP1 for TS 38.215 V18.0.0 #################

### 5.1.45	Time domain channel property (TDCP)

| Definition | Time domain channel property (TDCP) is defined as wideband normalized correlation between two CSI-RS transmission occasions, corresponding to CSI-RS resourece(s) from NZP-CSI-RS-ResourceSet(s) configured with higher layer parameter trs-info, that are separated by Dn symbols or slots, depending on the configuration, where Dn is the n-th delay configured value among [placeHolderForRrcParameter-D] configured delay values {D1, …, DY} and Y is number of configured delay values. The wideband normalized correlation value in TDCP is quantized in amplitude and if configured by [placeHolderForRrcParameter-phaseQuanOn], in phase, as described in Clause 5.2.1.4.5 of [6, TS38.214]. For frequency range 1, the reference point for the TDCP shall be the antenna connector of the UE. For frequency range 2, TDCP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. The reported TDCP amplitude is expected to be no lower than the minimum and no higher than the maximum measured values across the Rx branches if receiver diversity is used. |
| --- | --- |
| Applicable for | RRC_CONNECTED |

################  End of TP1 for TS 38.215 V18.0.0 #################

Adopt TP1 for TS 38.215 V18.0.0.

# 4	Type II Codebook Refinement for CJT

In this section, we discuss our views on some remaining issues related to Type II codebook refinement for CJT.

## 4.1 Inconsistency notation on number of selected TRPs in 38.212 and 38.214

In 38.212, “N0”  is used to indicate the number of selected NZP CSI-RS resources for type II CSI while “N” is used in 38.214 as shown below.

--------------------------------------------------38.212 v18.0.0 section 6.3.2.1.2 --------------------------------------------

6.3.2.1.2	CSI

If cqi-BitsPerSubband is configured, this Clause 6.3.2.1.2 applies by taking Subband CQI as Subband differential CQI and replacing the corresponding number of bits 2 by 4.

--- unrelated text omitted ---

The bitwidth for PMI of codebookType=typeII-CJT is provided in Tables 6.3.2.1.2-1B, where the values of (N1, N2), (O1, O2), O3, KNZ, N3, N0, Ln, σ(n) and Mll=1,…, υ are given by Clause 5.2.2.2.8 in [6, TS 38.214].

Table 6.3.2.1.2-1B: PMI of codebookType= typeII-CJT

--- unrelated text omitted ---

The bitwidth for PMI of codebookType=typeII-CJT-PortSelection is provided in Tables 6.3.2.1.2-2C, where the values of PCSI-RS, K1,n, KNZ, N3, N, N0, M and σn are given by Clause 5.2.2.2.9 in [6, TS 38.214].

Table 6.3.2.1.2-2C: PMI of codebookType= typeII-CJT-PortSelection

--- unrelated text omitted ---

--------------------------------38.214 v18.0.0 section 5.2.2.2.8 -------------------------------------------------------------	The UE may be configured with higher layer parameter restrictedCMR-Selection. If restrictedCMR-Selection is configured, the number of selected CSI-RS resources is N=NTRP. Otherwise, the UE is expected to select N CSI-RS resources, with 1≤N≤NTRP, and the selection is reported with an NTRP-bit bitmap, bNTRP, …,b1, where the CSI-RS resources are mapped from bit b1 to bit bNTRP by their ordering in the resource set and the first of the N selected CSI-RS resources corresponds to the nonzero bit with lowest index.

--------------------------------38.214 v18.0.0 section 5.2.2.2.9 -------------------------------------------------------------

-	The UE may be configured with higher layer parameter restrictedCMR-Selection. If restrictedCMR-Selection is configured, the number of selected CSI-RS resources is N=NTRP. Otherwise, the UE is expected to select N CSI-RS resources, with 1≤N≤NTRP, and the selection is reported with an NTRP-bit bitmap, bNTRP, …,b1, where the CSI-RS resources are mapped from bit b1 to bit bNTRP by their ordering in the resource set and the first of the N selected CSI-RS resources corresponds to the nonzero bit with lowest index.

Note that N is also used in Rel-17 further enhanced type II port selection codebook to indicate the window size for selecting the second frequency domain basis vector, where N can be 2 or 4.  To avoid potential ambiguity, one possible option is to replace N by N0 in 38.214 for CJT.  In addition, the range of the selected TRP, σn, may also be aligned between 38.212 and 38.214, either from 0 to N0-1 or from 1 to N0.  We propose the following TP for TS 38.214 V18.0.0:

###############  Start of TP2 for TS 38.214 V18.0.0 #################

##### 5.2.2.2.8	Enhanced Type II codebook for CJT

--- unaffected text omitted ---

- The UE may be configured with higher layer parameter restrictedCMR-Selection. If restrictedCMR-Selection is configured, the number of selected CSI-RS resources is N0=NTRP. Otherwise, the UE is expected to select N0 CSI-RS resources, with 1≤N0≤NTRP, and the selection is reported with an NTRP-bit bitmap, bNTRP, …,b1, where the CSI-RS resources are mapped from bit b1 to bit bNTRP by their ordering in the resource set and the first of the N selected CSI-RS resources corresponds to the nonzero bit with lowest index.

--- unaffected text omitted ---

##### 5.2.2.2.9	Further enhanced Type II port selection codebook for CJT

--- unaffected text omitted ---

- The UE may be configured with higher layer parameter restrictedCMR-Selection. If restrictedCMR-Selection is configured, the number of selected CSI-RS resources is N0=NTRP. Otherwise, the UE is expected to select N0 CSI-RS resources, with 1≤N0≤NTRP, and the selection is reported with an NTRP-bit bitmap, bNTRP, …,b1, where the CSI-RS resources are mapped from bit b1 to bit bNTRP by their ordering in the resource set and the first of the N selected CSI-RS resources corresponds to the nonzero bit with lowest index.

################  End of TP2 for TS 38.214 V18.0.0 #################

The notation for the number of selected TRP in Rel-18 CJT is not aligned between 38.212, where N0 is used, and 38.214, where N is used.

Adopt TP2 for TS 38.214 V18.0.0.

## 4.2 Inconsistency notation on priority formula in 38.212 and 38.214

Priority function Pril,n, i,f is referred for type II CJT in 38.212, while Pril,ij,f,j is used in 38.214 as shown below. They are not aligned.

----------------------------------------------------------38.212 section 6.3.2.1.2---------------------------------------------------------

Table 6.3.2.1.2-5E: Mapping order of CSI fields of one CSI report, CSI part 2 of codebookType=typeII-CJT

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Table 6.3.2.1.2-1B, if reported; |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1B: i2,3,l:l=1,…,υ, i1,5, i1,6,l:l=1,…,υ, i1,9, and max⁡(0,KNZ2-υ)×3 highest priority bits of i2,4,l:l=1,…,υ,max⁡(0,KNZ2-υ)×4 highest priority bits of {i2,5,l:l=1,…,υ} and ν*2∙n=0N0-1Lσ(n)Mυ-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,n, i,f defined in clause 5.2.3 of TS38.214, if present and if reported; |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-1B: minKNZ-v, KNZ2×3 lowest priority bits of i2,4,l:l=1,…,υ,minKNZ-v, KNZ2×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,n, i,f defined in clause 5.2.3 of TS38.214, if reported; |

Table 6.3.2.1.2-5G: Mapping order of CSI fields of one CSI report, CSI part 2 of codebookType=typeII-CJT-PortSelection

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n CSI part 2, group 0 | PMI fields X1, from left to right as in Table 6.3.2.1.2-2C, if reported |
| CSI report #n CSI part 2, group 1 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-2C: i2,3,l:l=1,…,υ (max⁡(0,KNZ2-υ))×3 highest priority bits of i2,4,l:l=1,…,υ,(max⁡(0,KNZ2-υ))×4 highest priority bits of {i2,5,l:l=1,…,υ}, i1,9 and ν*n=0N-1K1,nM-KNZ/2 highest priority bits ofi1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,n, i,f defined in clause 5.2.3 of TS38.214, if present and if reported |
| CSI report #n CSI part 2, group 2 | The following PMI fields X2, from left to right, as in Table 6.3.2.1.2-2C: (minKNZ-v, KNZ2)×3 lowest priority bits of i2,4,l:l=1,…,υ,(minKNZ-v, KNZ2)×4 lowest priority bits of i2,5,l:l=1,…,υ and KNZ/2 lowest priority bits of i1,7,l:l=1,…,υ, in decreasing order of priority based on the corresponding function Pril,n, i,f defined in clause 5.2.3 of TS38.214, if reported |

----------------------------------------------38.214 section 5.2.3 CSI reporting using PUSCH--------------------------------------

-	For Enhanced Type II for CJT reports, for a given CSI report n, each reported element of i2,4,l i2,5,l and i1,7,l, indexed by l,ij, f and j, is associated with a priority value Pril,ij,f,j=2⋅k=1NLσk⋅υ⋅πf+υ⋅2k=1j-1Lσk+ij+l, with π(f)=min⁡(2⋅n3,lf,2⋅N3-n3,l(f)-1), for l=1,…,υ, ij=0,1,…,2Lσj-1, f=0,1,…,Mυ-1 and j=1,…,N,  and where n3,l(f) and Lσj are defined in Clause 5.2.2.2.8. The element with the highest priority has the lowest associated value Pril,ij,f,j. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where

-	For Further Enhanced Type II Port Selection for CJT reports, for a given CSI report n, each reported element of i2,4,l i2,5,l and i1,7,l, indexed by l, ij, f and j, is associated with a priority value Pril,ij,f,j=k=1NK1,k⋅υ⋅f+υ⋅k=1j-1K1,k+ij+l, for l=1,…,υ, ij=0,1,…,K1,j-1, f=0,…,M-1 and j=1,…,N,  and where K1,j is defined in Clause 5.2.2.2.8. The element with the highest priority has the lowest associated value Pril,ij,f,j. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where:

The notation for the priority formula for Rel-18 CJT is not aligned between 38.212, where Pril,n, i,f is used, and 38.214, where Pril,ij,f,j is used.

Align the priority function for Rel-18 CJT between 38.212 and 38.214.

# 6	Conclusion

In the previous sections we made the following observations:

Observation 1	The notation for the number of selected TRP in Rel-18 CJT is not aligned between 38.212, where N0 is used, and 38.214, where N is used.

Observation 2	The notation for the priority formula for Rel-18 CJT is not aligned between 38.212, where Pril,n, i,f is used, and 38.214, where Pril,ij,f,j is used.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt TP1 for TS 38.215 V18.0.0.

Proposal 2	Adopt TP2 for TS 38.214 V18.0.0.

Proposal 3	Align the priority function for Rel-18 CJT between 38.212 and 38.214.

# 6. References

RP-213598, Revised WID: MIMO evolution for downlink and uplink, Samsung, RAN#94-e, Dec 2021.