3GPP TSG RAN WG1 #114bis	Tdoc R1-2310364

Xiamen, China, October 9th – October 13th, 2023

Agenda Item:	8.14.3

Source:	Moderator (Ericsson)

Title:	Summary #1 on Remaining Aspects of Evaluating AI/ML for Positioning Accuracy Enhancement

Document for:	Discussion, Decision

# 1	Introduction

This contribution discusses the remaining issues on the evaluation of positioning accuracy enhancement.

# 2	Discussion on evaluation of positioning accuracy enhancement

## 2.1	New observations on positioning accuracy

### 2.1.1 	1st round discussion

In RAN1#114, the following observations were made for direct AI/ML positioning, which are summary type of observations based on a list of observations on fine-tuning performance.

Observation (RAN1#114) For direct AI/ML positioning, evaluation results show that: - Fine-tuning/re-training a previous model with dataset of the new deployment scenario improves the model performance for the new deployment scenario. For details on the amount of improvement, see other observations. - After fine-tuning/re-training a previous model with dataset of the new deployment scenario, the performance of the updated model degrades for the previous deployment scenario (e.g., previous clutter parameter setting) that the previous model was trained for. - Examples of the deployment scenario include: different drops, different clutter parameter, different InF scenarios Observation (RAN1#114) For direct AI/ML positioning, - if the new deployment scenario is significantly different from the previous deployment scenario the model was trained for (e.g., different drops, different clutter parameter, different InF scenarios), fine-tuning a previous model requires similarly large training dataset size as training the model from scratch, in order to achieve the similar performance for the new deployment scenario. - If the new deployment scenario is NOT significantly different from the previous deployment scenario the model was trained for (e.g., 2ns difference in network synchronization error between the previous and the new deployment scenario), fine-tuning a previous model requires a small (e.g., x%=10%) training dataset size as compared to training the model from scratch, in order to achieve the similar performance for the new deployment scenario.

Considering that similar list of observations on fine-tuning performance were made for AI/ML assisted positioning also, the same summary type of observation can be made for AI/ML assisted positioning as well. Thus the observations in RAN1#114 can be updated to the following. It is intended that they replace the corresponding observations made in RAN1#114.

Furthermore, with the above updates, the observation should be captured under a new heading "Both direct AI/ML positioning and AI/ML assisted positioning". The text proposal is shown below to capture these points.

- Proposal 2.1.1

- Capture the following observations in TR 38.843, which are updated from the corresponding observations in RAN1#114.

For both direct AI/ML positioning and AI/ML assisted positioning, evaluation results show that:

- Fine-tuning/re-training a previous model with dataset of the new deployment scenario improves the model performance for the new deployment scenario. For details on the amount of improvement, see other observations.

- After fine-tuning/re-training a previous model with dataset of the new deployment scenario, the performance of the updated model degrades for the previous deployment scenario (e.g., previous clutter parameter setting) that the previous model was trained for.

  - Examples of the deployment scenario include: different drops, different clutter parameter, different InF scenarios

For both direct AI/ML positioning and AI/ML assisted positioning,

- if the new deployment scenario is significantly different from the previous deployment scenario the model was trained for (e.g., different drops, different clutter parameter, different InF scenarios), fine-tuning a previous model requires similarly large training dataset size as training the model from scratch, in order to achieve the similar performance for the new deployment scenario.

- If the new deployment scenario is NOT significantly different from the previous deployment scenario the model was trained for (e.g., 2ns difference in network synchronization error between the previous and the new deployment scenario), fine-tuning a previous model requires a small (e.g., x%=10%) training dataset size as compared to training the model from scratch, in order to achieve the similar performance for the new deployment scenario.

======================= Start of text proposal to TR 38.843 v1.0.0 ==================== #### 6.4.2.3 Fine-tuning Observations: Direct AI/ML positioning ... As a summary of the observations above, for direct AI/ML positioning, evaluation results show that: - Fine-tuning/re-training a previous model with dataset of the new deployment scenario improves the model performance for the new deployment scenario. For details on the amount of improvement, see the observations listed above. - After fine-tuning/re-training a previous model with dataset of the new deployment scenario, the performance of the updated model degrades for the previous deployment scenario (e.g., previous clutter parameter setting) that the previous model was trained for. - Examples of the deployment scenario include: different drops, different clutter parameter, different InF scenarios For direct AI/ML positioning, - if the new deployment scenario is significantly different from the previous deployment scenario the model was trained for (e.g., different drops, different clutter parameter, different InF scenarios), fine-tuning a previous model requires similarly large training dataset size as training the model from scratch, in order to achieve the similar performance for the new deployment scenario. - If the new deployment scenario is NOT significantly different from the previous deployment scenario the model was trained for (e.g., 2ns difference in network synchronization error between the previous and the new deployment scenario), fine-tuning a previous model requires a small (e.g., x%=10%) training dataset size as compared to training the model from scratch, in order to achieve the similar performance for the new deployment scenario. AI/ML assisted positioning ... Both direct AI/ML positioning and AI/ML assisted positioning As a summary of the observations above, for both direct AI/ML positioning and AI/ML assisted positioning, evaluation results show that: - Fine-tuning/re-training a previous model with dataset of the new deployment scenario improves the model performance for the new deployment scenario. For details on the amount of improvement, see the observations listed above. - After fine-tuning/re-training a previous model with dataset of the new deployment scenario, the performance of the updated model degrades for the previous deployment scenario (e.g., previous clutter parameter setting) that the previous model was trained for. - Examples of the deployment scenario include: different drops, different clutter parameter, different InF scenarios For both direct AI/ML positioning and AI/ML assisted positioning, - if the new deployment scenario is significantly different from the previous deployment scenario the model was trained for (e.g., different drops, different clutter parameter, different InF scenarios), fine-tuning a previous model requires similarly large training dataset size as training the model from scratch, in order to achieve the similar performance for the new deployment scenario. - If the new deployment scenario is NOT significantly different from the previous deployment scenario the model was trained for (e.g., 2ns difference in network synchronization error between the previous and the new deployment scenario), fine-tuning a previous model requires a small (e.g., x%=10%) training dataset size as compared to training the model from scratch, in order to achieve the similar performance for the new deployment scenario. <Unchanged text is omitted> ======================= End of text proposal to TR 38.843 v1.0.0 ====================

Please share your view on Proposal 2.1.1 and the corresponding text proposal.

| Company | Comments |
| --- | --- |
| CATT | OK |
| LG | Fine with capturing the observation |
| ZTE | OK |

## 2.2	New observations on AI/ML complexity for positioning evaluations

### 2.2.1 	1st round discussion

For the AI/ML evaluation work, AI/ML complexity (both model complexity and computational complexity) for model inference is an important KPI.  Companies have reported the complexity values of the models used their simulations. In this section, figures are drawn to summarize the model inference complexity as reported by participating companies.

Specifically, Figure 1-5 show the reported range of complexity values by companies. Figure 1-4 each show the range of complexity for a given scheme: (1) direct positioning; (2) assisted positioning with multi-TRP; (3) assisted positioning with single-TRP and one-model for N TRPs; (4) assisted positioning with single-TRP and N models for N TRPs. Figure 5 collects the complexity data of all schemes in one plot. The complexity values shown correspond to those of  Table 1 - Table 4 in the Appendix.

In each figure, three lines are drawn to show the trend of the complexity values among companies, one solid line for the linear regression line, two dashed lines for +σ and -σ lines relative to the linear regression line.

For the data points in the figures, if several different complexities are reported by a given company, two data points (i.e., the minimum and maximum) reported by companies are selected, as shown in Table 1 - Table 4. This is to reflect all companies' input in a fair manner, and to avoid the figures being dominated by companies who has used many variations of the AI/ML complexity.

Additionally, there is already an observation made on complexity, which is currently captured under "6.4.2.2 Generalization Aspects". This observation can be moved under the new section 6.4.2.6 as well.

The text proposal is shown below to reflect the points above.

- Proposal 2.2.1

Capture in TR 38.843 Figures 1-5 for model inference complexity for the positioning use case, which shows the (a) model complexity in number of real parameters (millions) and (b) computational complexity in FLOPs (millions).

- Include a Note for the figures: "Note: For a given positioning scheme, the information captured in Figures 1-5 indicates the highest complexity and lowest complexity used in individual company's evaluation, if models of multiple complexity levels are evaluated by a company for the given scheme."

======================= Start of text proposal to TR 38.843 v1.0.0 ==================== ### 6.4.2 Performance results ... #### 6.4.2.2 Generalization Aspects Observations: Direct AI/ML positioning ... For AI/ML based positioning method, companies have submitted evaluation results to show that for their evaluated cases, for a given company’s model design, a lower complexity (model complexity and computational complexity) model can still achieve acceptable positioning accuracy (e.g., <1m), albeit degraded, when compared to a higher complexity model. ... #### 6.4.2.6 Model complexity and computational complexity For AI/ML based positioning method, companies have submitted evaluation results to show that for their evaluated cases, for a given company’s model design, a lower complexity (model complexity and computational complexity) model can still achieve acceptable positioning accuracy (e.g., <1m), albeit degraded, when compared to a higher complexity model. In Figure 6.4.2.6-1 to Figure 6.4.2.6-5 below, the model inference complexity for the positioning use case as reported by companies are shown, including (a) on the x-axis: model complexity in number of real parameters (millions) and (b) on the y-axis: computational complexity in FLOPs (millions). Figure 6.4.2.6-1 to Figure 6.4.2.6-4 each show the range of complexity for a given scheme: (1) direct positioning; (2) assisted positioning with multi-TRP; (3) assisted positioning with single-TRP and one-model for N TRPs; (4) assisted positioning with single-TRP and N models for N TRPs. Figure 6.4.2.6-5 collects the complexity data of all schemes in one plot. The complexity numbers are taken from POS_Table 1. For the three schemes of AI/ML assisted positioning, the complexity is calculated according to Table 6.4.1-2. Note: For a given positioning scheme, the information captured in Figure 6.4.2.6-1 to Figure 6.4.2.6-5 indicates the highest complexity and lowest complexity used in individual company's evaluation, if models of multiple complexity levels are evaluated by a company for the given scheme. Figure 1. Model complexity and computational complexity for AI/ML direct positioning, based on companies' evaluations in Table 1. Figure 2. Model complexity and computational complexity for AI/ML assisted positioning with multiple-TRP, based on companies' evaluations in Table 2. Figure 3. Model complexity and computational complexity for AI/ML assisted positioning with single-TRP and one-model for N TRPs, based on companies' evaluations in Table 3. Figure 4. Model complexity and computational complexity for AI/ML assisted positioning with single-TRP and N models for N TRPs, based on companies' evaluations in Table 4. Figure 5. Model complexity and computational complexity for schemes of AI/ML based positioning. Details of the data are in Table 1-Table 4. <Unchanged text is omitted> ======================= End of text proposal to TR 38.843 v1.0.0 ====================

Please share your view on Proposal 2.2.1 and the related text proposal.

| Company | Comments |
| --- | --- |
| HW/HiSi | Similar figures have been proposed in other agenda item, e.g. BM. Whether to include them or not could be handled in the same manner for all aspects. Maybe a general decision is needd? |
| CATT | OK |
| mtk | 1, okay to have a new section for the model complexity and computation complexity for positioning 2, the complexity aspects for different applications (CSI BM POS) may not be easy to compare even when putting together. Positioning may have input from multiple TRPs, however other applications may not have. Then we still prefer to put complexity analysis of different applications in different sections |
| vivo | We don’t see the need to have Proposal 2.2.1 at all. Looking at all the figures, it’s not clear to us what kind of insight can we gain from these figures. For figures 1 to 4, the values are from different companies which made the observation/comparison less meaningful since different companies may take different optimization in terms of model complexity. This is espeacially confusing for Figure 5 where the reported complexity values are all over the place and hence no concrete conclusion/observsation can be drawn. In general, what is the point of capturing this proposal? |
| LG | Fine |
| Nokia/NSB | We tend to agree with vivo. It is not clear if all companies used the same procedure to calculate the computational complexity. If the criteria was not the same, the information may not be clear in this observation. In 3GPP RAN1#111 we got the following conclusion: Conclusion Companies describe how their computational complexity values are obtained. - It is out of 3GPP scope to consider computational complexity values that have platform-dependency and/or use implementation (hardware and software) optimization solutions. If possible, we suggest to provide an explanation of the procedure used to obtain computational complexity in the TR or unless provide a common reference used by companies. |
| Qualcomm | Our understanding is that optimizing model complexity was not within the scope of the evaluation. People looking at the TR need to be clear that these complexity plots do not necessary indicate optimized models. Please capture a note on this. Optimizing model complexity (i.e., computational complexity and number of learnable parameters) was not within the scope of the evaluation item. The captured complexity plots are not meant to indicate complexities of optimized models. |
| Baicells | We tend to agree with HW that it can be discussed considering these figures with other use cases. |
| ZTE | We share similar view as Huawei. |

## 2.3	Texts to clarify the evaluation methodology

### 2.3.1 1st round discussion

In RAN1#109e, the following agreement was made:

Agreement

When reporting evaluation results with direct AI/ML positioning and/or AI/ML assisted positioning, proponent company is expected to describe if a one-sided model or a two-sided model is used.

- If one-sided model (i.e., UE-side model or network-side model), the proponent company report which side the model inference is performed (e.g. UE, network), and any details specific to the side that performs the AI/ML model inference.

- If two-sided model, the proponent company report which side (e.g., UE, network) performs the first part of interference, and which side (e.g., network, UE) performs the remaining part of the inference.

The evaluation study by companies show that the positioning methods can be applied equally on the UE side or network side. In the excel sheets capturing the evaluation results, there was no need to distinguish evaluation results using uplink measurements from those using downlink measurements. It is suggested that this is clarified in the TR.

Proposal 2.3.1

Adopt the text proposal to clarify that the AI/ML positioning methods can be used on the network side or the UE side. Evaluation results have been submitted for both by companies.

======================= Start of text proposal to TR 38.843 v1.0.0 ==================== <Unchanged text is omitted> ### 6.4.1 Evaluation assumptions, methodology and KPIs For AI/ML-based positioning evaluation, RAN1 does not attempt to define any common AI/ML model as a baseline. The evaluated positioning schemes can be used either on the network side or the UE side. The evaluated direct AI/ML positioning methods are applicable to Case 1/2b/3b, and the AI/ML assisted positoning methods are applicable to Case 1/2a/3a. For Case 1/2a/2b, the model input are UE measurements of PRS on the downlink. For Case 3a/3b, the model input are gNB measurements of SRS on the uplink. In the evaluation, some results use UE measurements as model input, other results assume gNB measurements as model input, and they are not distinguished. <Unchanged text is omitted> ======================= End of text proposal to TR 38.843 v1.0.0 ====================

Please share your view on Proposal 2.3.1 and the related text proposal.

| Company | Comments |
| --- | --- |
| HW/HiSi | Seems ok |
| CATT | OK |
| vivo | We don’t understand why the first sentense „The evaluated positioning schemes can be used either on the network side or the UE side.“ is needed here when the section is about evaluation assumptions, methodology and KPIs. What’s the reason to exclude a positioning scheme at both side? For example, an assisted AI/ML positoing scheme can be used at both UE side (Case 2a) and network side (Case 3a) where the AI/ML model outputs can be feed into the LMF so that LMF can use the conventional M-RTT positioing. |
| Nokia/NSB | We suggest the following update/rewording : “ The evaluated positioning schemes can be used either on the network side or the UE side. The evaluated direct AI/ML positioning methods are applicable to Case 1/2b/3b, and the AI/ML assisted positoning methods are applicable to Case 1/2a/3a. For Case 1/2a/2b, the model input are UE measurements of PRS on the downlink. For Case 3a/3b, the model input are gNB measurements of SRS on the uplink. In the evaluation, some results use UE measurements as model input, other results assume gNB measurements as model input The evaluation results, provided in the study item phase, are agnostic of using uplink measurements or downlink measurements, and they are not distinguished.“ |
| ZTE | We also support to remove the first sentence “ The evaluated positioning schemes can be used either on the network side or the UE side.”, Nokia’s update is also fine. |

## 2.4	Correct the placement of observations which cover both direct and assisted approaches

In section 6.4.2, some observations misplaced.

- For section 6.4.2.1, heading "Direct AI/ML positioning" should be deleted, since the observations are generic.

- For section 6.4.2.2 and 6.4.2.4, the performance results are grouped under “Direct AI/ML positioning” and “AI/ML assisted positioning”. On the other hand, some agreements are made to cover both. A new heading "Both direct AI/ML positioning and AI/ML assisted positioning" can be created for such observations.

- For section 6.4.2.5, some observations are made for semi-supervised learning or labels from existing NR-RAT methods. Such observations should be put under a new heading rather than “Direct AI/ML positioning”.

A text proposal is provided below to reflect the comments above. These editorial changes can be considered by the TR editor when updating TR 38.843.

======================= Start of text proposal to TR 38.843 v1.0.0 ==================== ### 6.4.2 Performance results #### 6.4.2.1 Training Data Collection Observations: Direct AI/ML positioning ... #### 6.4.2.2 Generalization Aspects Observations: Direct AI/ML positioning ... AI/ML assisted positioning ... Both direct AI/ML positioning and AI/ML assisted positioning For both direct AI/ML and AI/ML assisted positioning, evaluation results submitted show that with CIR model input for a trained model, - For two SNR/SINR values S1 (dB) and S2 (dB), S1 ≥ S2 + 15 dB, positioning error of a model trained with data of S1 (dB) and tested with data of S2 (dB) is more than 5.75 times that of the model trained and tested with data of S1 (dB). - For two SNR/SINR values S1 (dB) and S2 (dB), S1 ≤ S2 – 10 dB, the generalization performance of a model trained with data of S1 (dB) and tested with data of S2 (dB) is better than the performance of a model trained with data of S2 (dB) and tested with data of S1 (dB). Positioning error of a model trained with data of S2 (dB) and tested with data of S1 (dB) is more than 2.97 times that of the model trained with data of S1 (dB) and tested with data of S2 (dB). Note: here the positioning error is the horizonal positioning error (meters) at CDF=90%. #### 6.4.2.3 Fine-tuning ... #### 6.4.2.4 Model-input Size Reduction Observations: Direct AI/ML positioning ... AI/ML assisted positioning ... Both direct AI/ML positioning and AI/ML assisted positioning Evaluation of TRP reduction for both direct AI/ML positioning and AI/ML assisted positioning shows that: identification of the active TRPs is beneficial for Approach 2-B. Otherwise, the model suffers from poor performance in terms of positioning accuracy. For example, evaluation results from 4 sources show that the horizontal positioning accuracy is greater than 10 m if TRP identification is not included as model input. #### 6.4.2.5 Non-ideal label(s) Observations: Direct AI/ML positioning Evaluation shows that direct AI/ML positioning is robust to certain label error based on evaluation results of L in the range of (0, 5) meter. The exact range of label error that can be tolerated depends on the positioning accuracy requirement, where tighter positioning accuracy requirement demands smaller label error. For AI/ML based positioning, evaluation results show that semi-supervised learning is helpful for improving the positioning accuracy when the same amount of ideal labelled data is used for supervised learning, and the number of ideal labelled data is limited. Regarding ground truth label generation for AI/ML based positioning, multiple sources submitted evaluation results on the impact of ground truth label for training obtained by existing NR RAT-dependent positioning methods. Feasibility and performance benefit of utilizing ground truth label for training estimated by existing NR RAT-dependent positioning methods are observed. - Source 1 evaluated in InF-DH {40%, 2, 2} and showed that AI/ML model can be trained with noisy labels along with the corresponding quality estimated by the legacy positioning methods, to improve positioning performance from 3.73m@90% (5k ideal label) to 1.72m @90% (5k ideal label + 20k noisy label). It also showed that the performance benefit compared to semi-supervised training of 2.78m @90% (5k ideal label + 20k unlabeled data). Note that training data weighting is used with label quality indicator. - Source 2 evaluated in InF-DH {60%, 6, 2} and showed that the performance of direct AI/ML positioning with 1k clean labelled samples improves from 13.76m to 8.72m when considering additional 350 samples that are labelled using NR-RAT positioning method. Note that the label error is up to 3.5m. - Source 3 evaluated in both InF-DH {60%, 6, 2} and InF-DH {40%, 2, 2} and showed performance loss when compared to all ideal label case. For example it showed in InF-DH {40%, 2, 2} the accuracy degrades from 0.39m @90% (100% ideal label) to 2.10m @90% (50% ideal label and 50% label obtained by existing DL-TDOA scheme). Note that noisy label is treated the same as ideal label in training. ... AI/ML assisted positioning ... Other For AI/ML based positioning, evaluation results show that semi-supervised learning is helpful for improving the positioning accuracy when the same amount of ideal labelled data is used for supervised learning, and the number of ideal labelled data is limited. Regarding ground truth label generation for AI/ML based positioning, multiple sources submitted evaluation results on the impact of ground truth label for training obtained by existing NR RAT-dependent positioning methods. Feasibility and performance benefit of utilizing ground truth label for training estimated by existing NR RAT-dependent positioning methods are observed. - Source 1 evaluated in InF-DH {40%, 2, 2} and showed that AI/ML model can be trained with noisy labels along with the corresponding quality estimated by the legacy positioning methods, to improve positioning performance from 3.73m@90% (5k ideal label) to 1.72m @90% (5k ideal label + 20k noisy label). It also showed that the performance benefit compared to semi-supervised training of 2.78m @90% (5k ideal label + 20k unlabeled data). Note that training data weighting is used with label quality indicator. - Source 2 evaluated in InF-DH {60%, 6, 2} and showed that the performance of direct AI/ML positioning with 1k clean labelled samples improves from 13.76m to 8.72m when considering additional 350 samples that are labelled using NR-RAT positioning method. Note that the label error is up to 3.5m. - Source 3 evaluated in both InF-DH {60%, 6, 2} and InF-DH {40%, 2, 2} and showed performance loss when compared to all ideal label case. For example it showed in InF-DH {40%, 2, 2} the accuracy degrades from 0.39m @90% (100% ideal label) to 2.10m @90% (50% ideal label and 50% label obtained by existing DL-TDOA scheme). Note that noisy label is treated the same as ideal label in training.

## 2.5	KPIs not evaluated

### 2.5.1 1st round discussion

In RAN1#110 and RAN1#110bis-e, a list of common KPIs were agreed, and the list is captured in TR38.843 v1.0.0 section 6.1.

For the positioning use case, the yellow-highlighted common KPIs are not evaluated or reported by companies. For inference complexity for pre- and post-processing, this KPI is not evaluated or reported by companies when pre-processing or post-processing may be necessary, e.g., pre-processing to select N't samples from the time window of Nt samples for model input.

The blue-highlighted KPI ("Link and system level performance") is not applicable to positioning.

Agreement The following is an initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML - Performance - Intermediate KPIs - Link and system level performance - Generalization performance - Over-the-air Overhead - Overhead of assistance information - Overhead of data collection - Overhead of model delivery/transfer - Overhead of other AI/ML-related signaling - Inference complexity - Computational complexity of model inference: FLOPs - Computational complexity for pre- and post-processing - Model complexity: e.g., the number of parameters and/or size (e.g. Mbyte) - Training complexity - LCM related complexity and storage overhead - FFS: specific aspects - FFS: Latency, e.g., Inference latency Note: Other aspects may be added in the future, e.g. training related KPIs Note: Use-case specific KPIs may be additionally considered for the given use-case. Agreement The following are additionally considered for the initial list of common KPIs (if applicable) for evaluating performance benefits of AI/ML - Clarification on inference complexity - Note: Inference complexity includes complexity for pre- and post-processing. - LCM related complexity and storage overhead - Storage/computation for training data collection. - Storage/computation for training and model update - Storage/computation for model monitoring. - Storage/computation for other LCM procedures, e.g., model activation, deactivation, selection, switching, fallback operation. - FFS: Power consumption, latency (e.g., Inference latency)

The model monitoring KPIs were agreed in RAN1#110bis-e as shown below. In the Rel-18 study item, no rigorous evaluations were done to examine the model monitoring KPIs listed, except some accuracy results submitted by individual companies based on their preferred model monitoring methods.

Model monitoring is an important component in the life cycle management procedure. While it can be up to implementation how model monitoring metric is calculated, the model monitoring decision (e.g., activate or deactivate a model) has to be reliable. Rel-19 WI may further investigate how to ensure that the model monitoring entity provide reliable model monitoring decisions. This may involve both RAN1 and RAN4 work.

Agreement (RAN1#110bis-e) Study performance monitoring approaches, considering the following model monitoring KPIs as general guidance - Accuracy and relevance (i.e., how well does the given monitoring metric/methods reflect the model and system performance) - Overhead (e.g., signaling overhead associated with model monitoring) - Complexity (e.g., computation and memory cost for model monitoring) - Latency (i.e., timeliness of monitoring result, from model failure to action, given the purpose of model monitoring) - FFS: Power consumption - Other KPIs are not precluded. Note: Relevant KPIs may vary across different model monitoring approaches. FFS: Discussion of KPIs for other LCM procedures

Considering the above situation, RAN1 should discuss and decide how to treat the KPIs not evaluated in TR 38.843. This is because such KPIs are explicitly described in the TR for evaluations, see section 6.1 "Common evaluation methodology and KPIs" of [1]. One solution is to spell out in the TR the KPIs not evaluated for each use case. As shown below, the KPIs in the previous RAN1 agreement but are not evaluated for the positioning use case are listed in Proposal 2.5.1-1 below.

Proposal 2.5.1

Capture in TR38.843 that for the positioning use case, the following KPIs are not evaluated in the Rel-18 study item:

- Over-the-air Overhead

  - Overhead of assistance information

  - Overhead of model delivery/transfer

  - Overhead of other AI/ML-related signaling

- Inference complexity

  - Computational complexity for pre- and post-processing

- Training complexity

- LCM related complexity and storage overhead

  - Storage/computation for training data collection.

  - Storage/computation for training and model update

  - Storage/computation for model monitoring.

  - Storage/computation for other LCM procedures, e.g., model activation, deactivation, selection, switching, fallback operation.

- Model monitoring

  - Overhead (e.g., signaling overhead associated with model monitoring)

  - Complexity (e.g., computation and memory cost for model monitoring)

  - Latency (i.e., timeliness of monitoring result, from model failure to action, given the purpose of model monitoring)

Please share your view on Proposal 2.5.1.

| Company | Comments |
| --- | --- |
| HW/HiSi | Could be ok to mention which KPIs have not been evaluated. But since we already report the metrics that we have evaluated, isn’t it then obvious which have not been evalauted? |
| CATT | Seems true, but do we really need to spell it out since we already capture what we have evaluated? |
| vivo | We don’t understand the motivation of this proposal. Looking at the agreements, the wording used are „an initial list of common KPIs (if applicable)” and “additionally considered for the initial list of common KPIs (if applicable)”, “considering the following model monitoring KPIs as general guidance”. To us, they are all guidance, rather than “agreed” KPIs. Listing them as not evaluated KPIs in the TR may give the wrong impression that they are identified KPIs and are unfinished tasks of this SI. |
| LG | Similar understanding with vivo |
| Nokia/NSB | We suggest to list only the KPIs used in the study item. |
| Baicells | Agree with Nokia. |
| ZTE | We also don’t see strong necessity to have this propsal to clarify which KPTs are used or not, anyway it can be found in companies‘ assumptions. Maybe Nokia’s proposal can be the middle ground. |

## 2.6	Aspects without observations due to insufficient evaluation results

### 2.6.1 1st round discussion

For the generalization aspect of SNR/SINR, an observation was made in RAN1#112bis for CIR as model input. No  observation was made for PDP or DP as model input, since there were insufficient evaluation results on this aspect.

Observation (RAN1#112bis) For both direct AI/ML and AI/ML assisted positioning, evaluation results submitted to RAN1#112bis show that with CIR model input for a trained model, - For two SNR/SINR values S1 (dB) and S2 (dB), S1>=S2 + 15 dB, positioning error of a model trained with data of S1 (dB) and tested with data of S2 (dB) is more than 5.75 times that of the model trained and tested with data of S1 (dB). - For two SNR/SINR values S1 (dB) and S2 (dB), S1<=S2 – 10 dB, the generalization performance of a model trained with data of S1 (dB) and tested with data of S2 (dB) is better than the performance of a model trained with data of S2 (dB) and tested with data of S1 (dB). Positioning error of a model trained with data of S2 (dB) and tested with data of S1 (dB) is more than 2.97 times that of the model trained with data of S1 (dB) and tested with data of S2 (dB). Note: here the positioning error is the horizonal positioning error (meters) at CDF=90%.

For the length of Nt consecutive time domain samples used as model input, the following observation was made for CIR and PDP as model input. No observation was made for DP as model input, since there were insufficient evaluation results on this aspect.

Observation (RAN1#113) For AI/ML assisted positioning, with Nt consecutive time domain samples used as model input, evaluation results submitted to RAN1#113 show that when CIR or PDP are used as model input, using different Nt while holding other parameters the same, - Reducing Nt from 256 to 128 does not appreciably degrade the positioning accuracy, while the measurement size and signaling overhead shrink to (approximately) 1/2 that of Nt=256. - Positioning error of Nt=128 is 1.00 ~ 1.42 times the positioning error of Nt=256; - Reducing Nt from 256 to 64~32 may degrade the positioning accuracy, while the measurement size and signaling overhead shrink to (approximately) 1/4 ~1/8 that of Nt=256, respectively. - Positioning error of Nt=64 is 1.09 ~ 3.02 times the positioning error of Nt=256; - Positioning error of Nt=32 is 2.43 ~ 5.10 times the positioning error of Nt=256;

Since the evaluation work has concluded, no further action is to be taken for the above in the study item phase. On the other hand, evaluations may be necessary in the work item phase for the missing aspects if decisions need to be taken based on such evaluation results, for example, to decide on the desired range of Nt when DP is used as model input.

For the purpose of TR writing, there is no need to explicitly point out potential observations which were not drawn due to insufficient evaluation results. The TR only need to summarize the observations RAN1 was able to draw. No further action is needed in the study item phase.

Conclusion 2.6.1

For aspects that no observations were drawn due to insufficient evaluation results, there is no need to explicitly state it in the TR. No further action is needed in the study item phase.

Please share your view whether you can support the proposed conclusion 2.6.1.

| Company | Comments |
| --- | --- |
| HW/HiSi | Ok |
| vivo | We don’t need this conclusion and no need to discuss further on any evaluation/observations not captured in TR. |
| LG | Fine |

# 3	High-level descriptions for evaluation of positioning accuracy enhancement

## 3.1 1st round discussion

In this section, high-level descriptions for improving readability of TR 38.843 are provided.

- Proposal 3.1-1

Adopt the text proposal below to describe the AI/ML methods used in evaluation.

======================= Start of text proposal to TR 38.843 v1.0.0 ==================== ## 6.4 Positioning accuracy enhancements ### 6.4.1 Evaluation assumptions, methodology and KPIs For AI/ML based positioning, the following methods are evaluated. - Direct AI/ML positioning, as illustrated in Figure 6.4.1-1. - Assisted AI/ML positioning. - Assisted AI/ML positioning with multi-TRP construction, as illustrated in Figure 6.4.1-2. - Assisted positioning with single-TRP construction and one model for N TRPs, as illustrated in Figure 6.4.1-3. - Assisted positioning with single-TRP construction and N models for N TRPs, as illustrated in Figure 6.4.1-4. Figure 6.4.1-1. Direct AI/ML positioning Figure 6.4.1-2. Assisted positioning with multi-TRP construction Figure 6.4.1-3. Assisted positioning with single-TRP construction, and one model for N TRPs. Figure 6.4.1-4. Assisted positioning with single-TRP construction, and N models for N TRPs. ======================= End of text proposal to TR 38.843 v1.0.0 ====================

Please share your view whether you can support the text proposal in Proposal 3.1-1.

| Company | Comments |
| --- | --- |
| Hw/HiSi | In principle fine with the text and figures. One question for clarification: In our understanding, the channel measurements can be used as part of the model input. For example, if the model input is PDP of Nt values, the channel measurement could be N’t values with power out of the Nt values and the rest would be zeros. Is it common understanding that this is also covered by the figures? |
| CATT | Almost fine with the proposal. One comment for the figure of AI/ML-assisted positioning. Currently only ‘e.g. xxx’ is used for model output, making it lack of definition. Can we consider one of the following as model output for AI/ML-assisted positioning: (1) ‘New measurement and/or enhancement of existing measurement’, as has been used in Huaming’s summary. (2) ‘Intermediate positioning measurement’ used in Taesang’s summary for reply LS to RAN2 on Part B. |
| mtk | 1, We may treat “channel measurement” as a general term. The format of channel measurement may be discussed in WI phase. For example it is based on absolute time or relative time, and the case mentioned by HW |
| vivo | We don’t see a strong need to capture figures when the definitions of direct AI/ML and AI/ML assisted positioning are clear alrady. Furthermore, it’s not clear to us what’s the difference between Figure 2 and 3. |
| LG | Fine with the proposal. To our understanding, the figures above represents the high-level concept for AI model input/output for multi-TRP construction as FL mentioned and no need to include the details on those in each figure. Regarding Figure 6.4.1-2, just correct the typo on the second channel measurement input (i.e. TRP0TRP 1) |
| Qualcomm | Our understanding these figures are example to help people reading the TR better picture the intention for each approach. Please add the wording (potential example) to figure captions and to text description. |
| Baicells | Thanks FL for the efforts. Maybe some notes can be added to address concerns from other companies. In addition, support CATT’s wording suggestion for model output. |

- Proposal 3.1-2

Adopt the text proposal below to describe the issues evaluated for AI/ML based positioning in the study item.

======================= Start of text proposal to TR 38.843 v1.0.0 ==================== For the use case of positioning accuracy enhancement, extensive evaluations have been carried out. Both direct AI/ML positioning and AI/ML assited positioning are evaluated using one-sided model (either UE-side model or network-side model). The following areas are investigated. - Basic performance, where the AI/ML model is trained and tested with dataset of the same deployment scenario. - For the basic performance without generalization consideration, AI/ML based positioning can significantly improve the positioning accuracy compared to existing RAT-dependent positioning methods. For example, in InF-DH with clutter parameter setting {60%, 6m, 2m}, AI/ML based positioning can achieve horizontal positioning accuracy of <1m at CDF=90%, as compared to >15m for conventional positioning method. - Impact of training data sample density (i.e., training dataset size for a given evaluation area). Evaluation with uniform UE distribution show that, the larger the training dataset size (i.e., higher sample density), the smaller the positioning error (in meters), until a saturation point is reached where additional training data does not bring further improvement to the positioning accuracy. - AI/ML complexity. For a given company’s model design, a lower complexity (model complexity and computational complexity) model can still achieve acceptable positioning accuracy (e.g., <1m), albeit degraded, when compared to a higher complexity model. - Generalization study. Evaluations are carried to investigate various generalization aspects, where the AI/ML model is trained with dataset of one deployment scenario, while tested with dataset of a different deployment scenario. The generalization aspects include: different drops; different clutter parameters; different InF scenarios; network synchronization error; UE/gNB RX and TX timing error; SNR mismatch; Channel estimation error. - Methods to handel generalization issues are extensively evaluated. - Better training dataset construction (e.g., mixed dataset), where the training dataset is composed of data from multiple deployment scenarios, which include data from the same deployment scenario as the test dataset. - Fine-tuning/re-training, where the model is re-trained/fine-tuned with a dataset from the same deployment scenario as the test dataset. The impact of the amount of fine-tuning data on the positioning accuracy of the fine-tuned model is evaluated for the various generalization aspects. Evaluation results are obtained for two experiments: - The AI/ML model is (a) previously trained for scenario A with a dataset of sample density N (#samples/m2), (b) followed by fine-tuning for scenario B with a dataset of sample density x% N (#samples/m2), (c) then tested under scenario B and the horizontal accuracy at CDF=90% is E meters. - The AI/ML model is (a) previously trained for scenario A with a dataset of sample density N (#samples/m2), (b) followed by fine-tuning for scenario B with a dataset of sample density x% N (#samples/m2), (c) then tested under scenario A and the horizontal accuracy at CDF=90% is E meters. - Model input size reduction. Evaluations are carried out to examine various ways to change the model input size. The model size can be varied by using: - Different model input type, for example, CIR, PDP, DP. - Different time window size, Nt, during which the model input is collected. - Different number of non-zero samples N't collected within the time window.. - Different number of active TRPs, N'TRP, where measurements are collected for the model input. - Fixed TRP pattern vs dynamic TRP pattern. Evaluation results show that, approaches supporting dynamic TRP pattern can achieve comparable horizontal positioning accuracy as approaches supporting fixed TRP pattern, when other design parameters are held the same - Model output of AI/ML assisted positioning. For AI/ML assisted positioning, evaluations are carried out where the model output includes timing information and/or LOS/NLOS indicator. - Non-ideal label in the training dataset. Evaluations are carried out to show the impact of: - Label error, where the label in the training dataset is degraded from ground truth label by an error. - For direct AI/ML positioning and AI/ML assisted positioning with timing information as model output, location error in each dimension of x-axis and y-axis is modelled as a truncated Gaussian distribution. - For AI/ML assisted positioning where the model output includes the LOS/NLOS indicator, random LOS/NLOS label error is applied. - Absent label, where some data samples in the training dataset do not have associated labels. Semi-supervised learning is evaluated for this case. - Model monitoring. Performance of model monitoring methods are evaluated, including: - Label based methods, where ground truth label (or its approximation) is provided for monitoring the accuracy of model output. - Label-free methods, where model monitoring does not require ground truth label (or its approximation). - Measurement size of model input. The measurement size for model input type (CIR, PDP, DP) and parameters (N'TRP, Nt, N't, Nport) is analyzed. ======================= End of text proposal to TR 38.843 v1.0.0 ====================

Please share your view whether you can support the text proposal in Proposal 3.1-2.

| Company | Comments |
| --- | --- |
| HW/HiSi | For clarification: - For Fixed vs dynamic pattern, not sure if we can agree. For direct pos, the accuracy with dynamic TRP is about 0.8-2.15 times the accuracy of fixed TRP. And for assisted it is 1.03 to 1.74. This is not really comparable, since for assisted pos the dynamic approach is always worse, and for direct pos there is a large uncertainty. Or am I missing something here? |
| CATT | Content is good. Just need to clarify which section/clause should capture this part. |
| vivo | Which section of TR is this proposed TP targeting for? In currect TR, there’re already descriptions for each evaluted aspects in section 6.4.2. We don’t see the necessity to have duplicate descriptions. |
| LG | Fine with the proposal but further discussion seems to be needed for capturing this in TR |
| Qualcomm | We have a few concerns with the current version. It is good in general to capture such a summary, however, the current version does not properly capture details of some bullets. The bullet on generalization need to include model switching (as captured in multiple agreements). In addition, the model input reduction needs to use notation description as was mentioned in the agreement. There is also no summary on model input size increase/reduction and related overhead and achieved accuracy. Observation RAN1#112 (9.2.4.1) Evaluation of the following generalization aspects show that the positioning accuracy of direct AI/ML positioning deteriorates when the AI/ML model is trained with dataset of one deployment scenario, while tested with dataset of a different deployment scenario. - The generalization aspects include: - Different drops - Different clutter parameters - Different InF scenarios - Network synchronization error - Companies have provided evaluation results which show that the positioning accuracy on the test dataset can be improved by better training dataset construction and/or model fine-tuning/re-training. - Better training dataset construction: The training dataset is composed of data from multiple deployment scenarios, which include data from the same deployment scenario as the test dataset. - Model fine-tuning/re-training: the model is re-trained/fine-tuned with a dataset from the same deployment scenario as the test dataset. Note: ideal model training and switching may provide the upper bound of achievable performance when the AI/ML model needs to handle different deployment scenarios. Agreement: RAN1#110be (9.2.4.1) For the model input used in evalutions of AI/ML based positioning, if time-domain channel impulse response (CIR) or power delay profile (PDP) is used as model input in the evaluation, companies report the input dimension NTRP * Nport * Nt, where NTRP is the number of TRPs, Nport is the number of transmit/receive antenna port pairs, Nt is the number of time domain samples. - Note: CIR and PDP may have different dimensions. Note: Companies provide details on their assumption on how PDP is constructed and how (if applicable) it is mapped to Nt samples Agreement RAN1-111-9.2.4.1 For reporting the model input dimension NTRP * Nport * Nt of CIR and PDP, Nt refers to the first Nt consecutive time domain samples. - If N’t (N’t < Nt) samples with the strongest power are selected as model input, with remaining (Nt ‒ N’t) time domain samples set to zero, then companies report value N’t in addition to Nt. It is also assumed that timing info for the N’t samples need to be provided as model input. Please check the following revised bullets: - Generalization study. Evaluations are carried to investigate various generalization aspects, where the AI/ML model is trained with dataset of one deployment scenario, while tested with dataset of a different deployment scenario. The generalization aspects include: different drops; different clutter parameters; different InF scenarios; network synchronization error; UE/gNB RX and TX timing error; SNR mismatch; Channel estimation error, time varying errors - Methods to handel generalization issues are extensively evaluated. - Better training dataset construction (e.g., mixed dataset), where the training dataset is composed of data from multiple deployment scenarios, which include data from the same deployment scenario as the test dataset. - Fine-tuning/re-training, where the model is re-trained/fine-tuned with a dataset from the same deployment scenario as the test dataset. The impact of the amount of fine-tuning data on the positioning accuracy of the fine-tuned model is evaluated for the various generalization aspects. Evaluation results are obtained for two experiments: - The AI/ML model is (a) previously trained for scenario A with a dataset of sample density N (#samples/m2), (b) followed by fine-tuning for scenario B with a dataset of sample density x% N (#samples/m2), (c) then tested under scenario B and the horizontal accuracy at CDF=90% is E meters. - The AI/ML model is (a) previously trained for scenario A with a dataset of sample density N (#samples/m2), (b) followed by fine-tuning for scenario B with a dataset of sample density x% N (#samples/m2), (c) then tested under scenario A and the horizontal accuracy at CDF=90% is E meters. - Note: ideal model training and switching may provide the upper bound of achievable performance when the AI/ML model needs to handle different deployment scenarios. - Model input size reduction. Evaluations are carried out to examine various ways to change the model input size:. The model size can be varied by using: - Different model input type, for example, CIR, PDP, DP. - Different time window size number of time domain measurements, Nt, during which the model input is collected. - Different number of non-zero measurements samples N't from Nt (N’t < Nt) collected within the time window.. - Different number of active TRPs, N'TRP, where measurements are collected for the model input. - Measurement size and reporting overhead/signaling. Evaluation results show that, different measurement types and sizes of model input can have different reporting overhead and positioning accuracy - Model output of AI/ML assisted positioning. For AI/ML assisted positioning, evaluations are carried out where the model output includes timing information (hard- and soft information) and/or LOS/NLOS indicator. |
| ZTE | To us, this is a summary of observation, fine to have it in the TR. But same question as other comapnies, which section of the TR will capture this proposal? |
| Nokia/NSB | In the sub-item related to Model monitoring, the text “performance of model monitoring“ is confusing. We propose to use “performance monitoring“ only. In addition, we believe that it is important to highligh in the sumary that only few evaluations were done on monitoring and there was not consensus between companies to use a similar KPI. |

# 4	Potential conclusions/recommendations for AI/ML based positioning

## 4.1 1st round discussion

Based on the evaluation results performed by RAN1 for the Rel-18 study item, recommendations are provided below by FL for the use case of positioning accuracy enhancement.

From RAN1 perspective:

- Support both direct AI/ML and AI/ML assisted positioning approaches. Direct AI/ML and AI/ML assisted positioning can achieve comparable positioning accuracy. Thus the down-selection of sub-use cases 1/2a/2b/3a/3b (if desired) depends on other considerations, e.g., measurement report sizes for training data collection, signaling overhead for model inference.

- Support both UE-side model and NW-side model. Evaluations by companies show that the AI/ML based positioning methods can be equally applied to UE-side and NW-side model. Thus both UE-side model and NW-side model can be supported from RAN1 perspective. UE-side model and NW-side model have different ramification to architecture, protocol, signaling when considering the various LCM stages. Such issues may need input from other RAN groups (e.g., RAN2 and RAN3).

- Model life-cycle management needs to handle the generalization issues if present in the deployment. Evaluation results show that most trained models tend to be sensitive to changes in deployment scenario, where the exact performance impact varies depending on many factors. The AI/ML model is robust only for certain AI/ML approaches, for example, some companies observed that AI/ML assisted positioning with single TRP is robust to environment changes. Thus model life cycle management needs to take this into account. For example, support mixed training dataset in training data collection, properly define conditions in functionality-based LCM.

- Training data collection need to support collecting a sufficiently large quantity of training data. In the evaluations, companies typically used 2.7~8.8 training data samples per m2 for training. That is, a dataset of (19,000~63,000) training data samples for a factory floor size of 120mx60m. Since evaluations show that the achievable positioning accuracy is directly affected by the training dataset size, it is important that the training data collection procedure is properly designed to collect a sufficiently large training dataset for the targeted use case.

- Training data collection need to ensure adequate label quality. Evaluation results have show that positioning error grows approximately linearly with label error for both AI/ML direct positioning and AI/ML assisted positioning with timing information as output.  Thus it is very important to have high-quality label in order to achieve high positioning accuracy with the AI/ML model.

- Training data collection should support mixed training dataset for the anticipated deployment scenarios. Extensive evaluation results have been submitted by companies to demonstrate that mixed training dataset allow the AI/ML model to support different deployment scenarios without increasing the model complexity. This can be accomplished by a properly designed training data collection procedure.

- Deprioritize model fine-tuning in Rel-19. Evaluation results show that fine-tuning a previously trained model with a small dataset is only useful to handle minor environment change. Otherwise, re-training the model with a full dataset is required. Considering the difficulty in implementing model fine-tuning, it is not wise to support fine-tuning in Rel-19. Rel-19 is the first release to support AI/ML in PHY and only the most useful and critical procedure should be considered.

- Rel-19 WI further investigate the most appropriate model input type (e.g., CIR, PDP, DP) considering the tradeoff of positioning accuracy, signaling overhead, and AI/ML complexity. While evaluations have been done in the study item, further study is needed before the model input can be standardized. It is noted that CIR does not exist in current specification. Compared to PDP and DP, CIR has more standardization workload and more specification impact.

- Rel-19 WI further investigate the most appropriate model input size to specify, e.g., parameters (N'TRP, Nt, N't, Nport). While evaluations have been done in the study item, further study is needed before they can be standardized. These parameters affect the training data collection for all cases, and affect the signaling overhead for Case 2b/3b for model inference.

- Rel-19 WI further investigate how to ensure that the model monitoring entity provide reliable model monitoring decisions (e.g., activate or deactivate a model).

- Proposal 4.1

Based on RAN1 evaluations of AI/ML based positioning, the following is recommended from RAN1 perspective:

      - Support both direct AI/ML and AI/ML assisted positioning approaches.

      - Support both UE-side model and NW-side model.

      - Model life-cycle management needs to handle the generalization issues if generalization issue(s) are present in the targeted deployment.

      - Training data collection need to support collecting a sufficiently large quantity of training data for the desired positioning accuracy.

      - Training data collection need to ensure adequate label quality for the desired positioning accuracy.

      - Training data collection should support mixed training dataset for the anticipated deployment scenarios.

      - Deprioritize model fine-tuning in Rel-19.

      - If AI/ML based positioning is to be adopted for a given scenario, it is necessary to

        - further investigate the most appropriate model input type (e.g., CIR, PDP, DP) considering the tradeoff of positioning accuracy, signaling overhead, and AI/ML complexity.

        - further investigate the most appropriate model input size to specify, e.g., parameters (N'TRP, Nt, N't, Nport).

        - further investigate how to ensure that the model monitoring entity provide reliable model monitoring decisions (e.g., activate or deactivate a model).

Please share your view on the recommendations in Proposal 4.1.

| Company | Comments |
| --- | --- |
| HW/HiSi | We do not fully agree with the bullets before the proposal, but it is our understanding that these bullets are not intended to be captured in TR, only the Proposal 4.1. Therefore, our comments here refer to Proposal 4.1: 1) For the 1st and 2nd bullets, they are overlapping with the FL recommendations in „pos other aspects“. We should discuss them at one place to avoid duplicated discussion. Temporaly cross them out here. 2) For the 3rd bullet, it is covering the 6th bullet in some sense“Training data collection should support mixed training dataset, since mixing the training data set is a tool to improve the generalization . BTW, the dataset mixing seems to be implementation behavior, only functionality switching/selection may have spec impact. Therefore, we are not sure if bullet #6 should be kept. At least it should be modified as commented below. 3) For the 4th~6th bullets, it seems how to construct the dataset is implementation level recommendation. We may need to clarify this. 4) For the 7th bullet, as fine tuning is implementation (same as dataset mixing) on dataset obtainment/construction, there seems no need to mention it in Rel-19. But we would not object it since we do not identify spec impact to fine-tuning. 5) For the first and second sub-bullet of bullet #8, fine in general. 6) For the last sub-bullet, we are not clear about the intention of “investigate” – does it mean a further study in R19? In addition, it seems this part is duplicated with the FL recommendation in pos other aspects. We should discuss it at one place to avoid duplicated discussion. Temproraly cross it out. We are therefore making the following suggestion for an update: - Support both direct AI/ML and AI/ML assisted positioning approaches. - Support both UE-side model and NW-side model. - Model life-cycle management needs to handle the generalization issues if generalization issue(s) are present in the targeted deployment. - Training data collection need to support collecting a sufficiently large quantity of training data in implementation for the desired positioning accuracy. - Training data collection need to ensure adequate label quality in implementation for the desired positioning accuracy. - Training data collection should support mixed training dataset for the anticipated deployment scenarios in implementation. - [Deprioritize model fine-tuning in Rel-19.] - If AI/ML based positioning is to be adopted for a given scenario, it is necessary to - further investigate the most appropriate model input type (e.g., CIR, PDP, DP) considering the tradeoff of positioning accuracy, signaling overhead, and AI/ML complexity. - further investigate the most appropriate model input size to specify, e.g., parameters (N'TRP, Nt, N't, Nport). - further investigate how to ensure that the model monitoring entity provide reliable model monitoring decisions (e.g., activate or deactivate a model). |
| CATT | Some comments from our side: Model life-cycle management needs to handle the generalization issues if present multiple scenarios/configurations are targeted in the deployment If it is targeting at only one specific scenario, generalization is not a sevious issue. Training data collection need to support collecting a sufficiently large quantity of training data. For offline training, there is no latency on data collection, and thus a sufficiently large quantity of training data can also be achieved offline (no spec impact) Training data collection should support mixed training dataset for the anticipated deployment scenarios. Same comment as previous one for ‘generalization’. Depends on whether we are targeting at multiple scenarios/configurations. And this can be done by implementation. [Deprioritize model fine-tuning in Rel-19.] This should be a general discussion for all use case. |
| mtk | 1, have typo Training data collection need to ensure adequate label quality. Evaluation results have show shown 2, for ”Rel-19 WI further investigate the most appropriate model input type (e.g., CIR, PDP, DP)”, we notice that there is an agreement in August meeting, Agreement For evaluation of AI/ML based positioning, when timing information is included in model input (e.g., in CIR/PDP/DP), training dataset and test dataset use the same timing format (i.e., both are absolute time, or both are relative time) unless explicitly stated otherwise. Then we suggest to change the wording as “ Rel-19 WI to further investigate the model input type (e.g., CIR, PDP, DP), and the timing format (e.g., absolute time or relative time) ” Or to have a separate bullet with the wording “Rel-19 WI to further investigate the timing format (e.g., absolute time or relative time) of the model input” Or add a sub-bullet for, - If AI/ML based positioning is to be adopted for a given scenario, it is necessary to - further investigate the most appropriate model input type (e.g., CIR, PDP, DP) considering the tradeoff of positioning accuracy, signaling overhead, and AI/ML complexity. - further investigate the most appropriate model input size to specify, e.g., parameters (N'TRP, Nt, N't, Nport). - further investigate how to ensure that the model monitoring entity provide reliable model monitoring decisions (e.g., activate or deactivate a model). - Further investigate the timing format (e.g., absolute time or relative time) of the model input 3, We suggest NOT to drop model fine-tuning. The reason is, it seems to us that there may be relationship bwtween model fine-tuning and model monitoring. Through the model monitoring, we may learn whether the small amount data for fine-tuning, or large amount data for re-training is needed |
| vivo | 1. Is this proposal intended to be captured into TR? If so, which section? 2. We prefer to discuss conclusion and recommendation on AI/ML positioning together with potential specification impact, rather than simply making recommendations „Based on RAN1 evaluations of AI/ML based positioning“ only. 3. It is very unual to have statements such as „needs to handle“, „need to support“ and „need to ensure“ in the TR as recommendation where no solution(s) is(are) identified. Does this mean the study is not completed? |
| LG | Generally fine but not support on 7th bullet of the proposal. It seems not reasonable for deprioritizing model fine-tuning, since model LCM aspect also related to model monitoring and the corresponding model fine-tuning from a perspective of generalization. |
| Nokia/NSB | We believe that to provide a recommendation from RAN1 side, we should achieve an homogeneous aligment between Evaluations for AIML positioning (previously 9.2.4.1 agenda item) and Spec. Impact for AIML positioning (previously 9.2.4.2 agenda item). |
| Qualcomm | We think the summary in the previous proposal is better reflecting the status in evaluation item than this recommendation listing. Many listed agreements have no supporting agreements. See examples below: - Training data collection need to support collecting a sufficiently large quantity of training data for the desired positioning accuracy. We do not have any supporting numbers in any agreements that indicates the “sufficiently large quantity”. We can better say the amount of training data to be collected depends on multiple factors, including desired positioning accuracy, AI/ML positioning approach (e.g., direct/assisted), clutter settings, model input type and size. - Training data collection should support mixed training dataset for the anticipated deployment scenarios. Same here, we do not have an agreement that training data collection should support mixed training dataset. Let’s consider a more recommendation wording. Please add model switching as one of the recommended approaches to allow generalization. - Deprioritize model fine-tuning in Rel-19. We have no agreement on this one. - If AI/ML based positioning is to be adopted for a given scenario, it is necessary to - further investigate the most appropriate model input type (e.g., CIR, PDP, DP) considering the tradeoff of positioning accuracy, signaling overhead, and AI/ML complexity. - further investigate the most appropriate model input size to specify, e.g., parameters (N'TRP, Nt, N't, Nport). - further investigate how to ensure that the model monitoring entity provide reliable model monitoring decisions (e.g., activate or deactivate a model). Same here. We have not discussed this in the evaluation item. |
| ZTE | In this agenda, the conclusion/recommendation should be based on the simulation results. Then it seems the proposal in section 3.1 is more aligned with this. As pointed by other companies, the current formulation of this proposal has some overlapping with “ pos other aspects”. |

# 5. Proposals for online sessions

## 5.1 Proposals for Tuesday online session

# Conclusion

TBD

# References

RP-231766, TR 38.843 v1.0.0 (2023-09).

# Appendix. Tables for AI/ML Complexity

Table 1. From excel "POS_Table1 Pos-Supervised-NoLabelerr-Train-Test-Same-Setting_vfinal", tab "Direct AIML"

|  |  |  |  | Settings (e.g., drops, clutter parameters, mixed dataset) |  | AI/ML complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source | Model input | Model output | UE distribution area | Train | Test | Model complexity (Millions) | Computational complexity (Millions) | Horizontal accuracy @90% (meters) |
| China Telecom R1-2306811 | DL-TDOA;size:18*1 | UE coordinates | 120x60 m2 | {60%, 6m, 2m} | same as training | 0.08 | 0.08 | 0.73 |
| China Telecom R1-2306811 | RSRP+DL-TDOA; size:18*2 | UE coordinates | 120x60 m2 | {60%, 6m, 2m} | same as training | 0.2 | 0.2 | 0.38 |
| ZTE R1-2306799 | PDP {Nport, NTRP, Nt, Nt'} = {2, 18, 256, 256} | 2-D UE position (1x2) | InF-DH: 120x60 m(grid distribution for training dataset; the width of the square grid is 0.5 m) | clutter parameters:{60%, 6m, 2m} | same as training | 9.5 | 158.66 | 0.26 |
| ZTE R1-2306799 | CIR {Nport, NTRP, Nt, Nt'} = {2, 18, 256, 256} | 2-D UE position (1x2) | InF-DH: 120x60 m(grid distribution for training dataset; the width of the square grid is 0.5 m) | clutter parameters:{60%, 6m, 2m} | same as training | 9.69 | 172.77 | 0.18 |
| Samsung R1-2303124 | CIR with 18x256x2 | UE 2D position estimate | 120x60 m2 | SH | SH | 0.076 | 9.5 | 0.37 |
| Samsung R1-2307672 | SIG With 18*6 | UE 2D position estimate | 120x60 m2 | DH662 | DH662 | 0.074 | 0.21 | 1.69 |
| Xiaomi R1-2307379 | 18*256*1 CIR | 2*1 UE coordinates | 120*60 | {0.6，6，2} | same as training | 21.28 | 5760 | 0.4462 |
| OPPO R1-2307568 | RSTD + RSRP (18, 1,1,1) | UE coordination | 120x60 m | 1 drop, 80,000 UEs per drop {60%, 6, 2} | same as training | 0.24 | 0.47 | 0.48 |
| OPPO R1-2307568 | Normalized CIR + RSRP (18, 1, 256, 2) | UE coordination | 120x60 m | 1 drop, 80,000 UEs per drop {60%, 6, 2} | same as training | 2.66 | 5.32 | 0.33 |
| MediaTek R1-2308056 | CIR [18,2,256] | UE pos [x,y] | 120x60m | InF-DH{60%, 6m, 2m} | same as training | 0.46424 | 266 | 0.896 |
| MediaTek R1-2308056 | PDP [18,2,256] | UE pos [x,y] | 120x60m | InF-DH{60%, 6m, 2m} Nt=64,N't=25 | same as training | 0.24305 | 66 | 0.942 |
| NVIDIA R1-2306479 | CIR | Position | 120x60 m | {60%, 6m, 2m} | same as training | 1.8 | 90.9 | 0.75 |
| vivo R1-2306744 | CIR | Pos. | 120x60 m | DH {0.6, 6,2} | same as training | 1.65 | 22.3 | 0.99 |
| CMCC R1-2307187 | CIR;size：18*1*256 | UE coordinates | 120x60 m2 | {60%, 6m, 2m}, Drop 1 | same as training | 3.71 | 7.42 | 0.38 |
| CMCC R1-2307187 | CIR;size：18*1*256 | UE coordinates | 120x60 m2 | {60%, 6m, 2m} | same as training | 6.4 | 12.8 | 0.386 |
| CATT R1-2307080 | Type: CIR; Size: 18*1*256*2 | Type:Position Size:1*2 | 120m*60m | InF-DH {60%,6m,2m} with perfect network synchonzation | same as training | 11.2 | 2780 | 0.54m |
| CATT R1-2307080 | Type: CIR; Size: 18*1*32*2 | Type:Position Size:1*2 | 120m*60m | InF-DH {60%,6m,2m} with perfect network synchonzation | same as training | 11.2 | 350 | 0.62m |
| Ericsson R1-2302335 | Time-domain CIR, 18x2x256, complex array | UE 2D position estimate | 120x60 m2 | {60%,6m,2m} | same as training | 11 | 410 | 0.155 |
| Ericsson R1-2304339 | Time-domain PDP, 6x1x256, real array | UE 2D position estimate | 120x60 m2 | {60%,6m,2m} | same as training | 0.05 | 1 | 1.191 |
| NOKIA R1-2307242 | PDP (NTRP =18* Nt = 128*Real Number=1) | horizontal 2d positioning | 120x60 m2 | {40%,2m,2m} | same as training | 0.4669 | 6.39 | 0.732 |
| NOKIA R1-2307242 | PDP (NTRP =8* Nt = 128*Real Number=1) | horizontal 2d positioning | 120x60 m2 | {40%,2m,2m} | same as training | 0.464 | 0.72 | 1.23 |
| Qualcomm R1-2307920 | CIR (18,1, 256) | 2D | Baseline evaluation area for InF-DH = 120x60 m | Drop A {60%, 6m, 2m} | same as training | 0.029 | 29.7 | 1.08 |
| Qualcomm R1-2307920 | CIR (18,4, 400) | 2D | Baseline evaluation area for InF-DH = 120x60 m | Drop A {60%, 6m, 2m} | same as training | 1.5 | 1540 | 2.24 |
| InterDigital R1-2307582 | CIR (18*256* 2) | UE position | 120x60 m2 | clutter parameters:{60%, 6m, 2m} | same as training | 37 | 843 | 0.98 |
| InterDigital R1-2307582 | RSRP +RSTD | UE position | 120x60 m2 | clutter parameters:{60%, 6m, 2m} | same as training | 0.334 | 11.41 | 1.69 |
| Apple-R12308248 | CIR [18,1,256,2] | UE coordinates [1x2] | 120 x 60m | InF-DH{60%, 6m, 2m} | same as training | 12.37 | 103 | 0.745 |
| Apple-R12308248 | CIR [18,1,256,2] | UE coordinates [1x2] | 100 x 40 m | InF-DH{60%, 6m, 2m} | same as training | 1.48 | 2.75 | 0.844 |
| Fraunhofer R1-2307236 | CIR with 18x120x2 | UE 2D position estimate | 60x30 m2 | DH662 | DH662 | 0.462 | 637 | 0.37 |
| Huawei-R1-2306515 | CIR 18*4*256 | UE coordinates | 120x60 m | {60%, 6m, 2m}, Drop 1 | same as training | 0.052 | 25.81 | 0.62 |
| Huawei-R1-2306515 | PDP 2*4*256 TRP(7,10) | UE coordinates | 120x60 m | {60%, 6m, 2m}, Drop 1 | same as training | 0.005 | 2.05 | 2.8 |

Table 2. From excel "POS_Table1 Pos-Supervised-NoLabelerr-Train-Test-Same-Setting_vfinal", tab "Assisted, multi-TRP"

|  |  |  |  | Settings (e.g., drops, clutter parameters, mixed dataset) |  | AI/ML complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source | Model input | Model output | UE distribution area | Train | Test | Model complexity (Millions) | Computational complexity (Millions) | Horizontal accuracy @90% (meters) |
| ZTE R1-2306799 | CIR {Nport, NTRP, Nt, Nt'} = {1, 18, 256, 256} | RSTD values(1x18) | InF-DH: 120x60 m(grid distribution for training dataset; the width of the square grid is 0.5 m) | clutter parameters:{60%, 6m, 2m} | same as training | 9.5 | 158.66 | 0.28 |
| Xiaomi R1-2307379 | 18*256*1 CIR | 18*1 UE TOA | 120*60 | {0.6，6，2} | same as training | 21.29 | 5760 | 0.6778 |
| OPPO R1-2307568 | Normalized CIR (18, 1, 256, 2) | TOA | 120x60 m | 1 drop, 80,000 UEs per drop {60%, 6, 2} | same as training | 1.48M | 2.96 MFLOPs | 0.52 |
| MediaTek R1-2308056 | PDP[18,8,256] | TOA | 120x60m | InF-DH{60%, 6m, 2m} | same as training | 0.205 | 77 | 1.05 |
| MediaTek R1-2308056 | PDP[18,8,256] | TOA | 120x60m | InF-DH{40%, 2m, 2m} Nt=64 N't=64 | same as training | 0.181 | 19.3 | 1.7 |
| Huawei-R1-2306515 | CIR 18*1*256 | TOA | 120x60 m | {60%, 6m, 2m}, Drop 1 | same as training | 0.177 | 45.19 | 1.25 |
| vivo R1-2306744 | CIR | TOA | 120x60 m | DH{0.6, 6, 2} | same as training | 1.65 | 22.3 | 1.08 |
| CMCC R1-2307187 | CIR;size：18*1*256 | UE coordinates | 120x60 m2 | {60%, 6m, 2m} | same as training | 3.71M | 7.42M | 0.537 |
| CATT R1-2307080 | Type: CIR; Size: 18*1*256*2 | Type:TOA Size:1*18 | 120m*60m | InF-DH {60%,6m,2m} with perfect network synchonzation | same as training | 0.7M | 60.8M FLOPs | 0.655m |
| CATT R1-2307080 | Type: CIR; Size: 18*1*32*2 | Type:TOA Size:1*18 | 120m*60m | InF-DH {60%,6m,2m} with perfect network synchonzation | same as training | 0.7M | 7.60M FLOPs | 1.104m |
| Ericsson R1-2302335 | Time-domain CIR, 18x2x256, complex array | 18 direct path ToA estimates | 120x60 m2 | {60%,6m,2m} | same as training | 11 M real parameters | 410 M FLOPs | 0.156 |
| Ericsson R1-2302335 | Time-domain PDP, 18x1x256, real array | 18 direct path ToA estimates | 120x60 m2 | {60%,6m,2m} | same as training | 0.36 M real parameters | 9 M FLOPs | 0.426 |
| NOKIA R1-2307242 | PDP (NTRP =18* Nt = 128*Real Number=1) | LOS/NLOS indicator | 120x60 m2 | {40%,2m,2m} | same as training | 2.364186 | 0.476 | 0.9818 |
| NOKIA R1-2307242 | PDP (NTRP =18* Nt = 128*Real Number=1) | ToA estimation | 120x60 m2 | {40%,2m,2m} | same as training | 1.4131 | 1.82 | 0.61 |
| Qualcomm R1-2307920 | CIR (18,4, 400) | 17 RSTDs | Baseline evaluation area for InF-DH = 120x60 m | Drop A | same as training | 1.5 | 1540 | 2.92 |
| InterDigital R1-2307582 | CIR (18,256,2) | 18 direct path TOA | 100x40 m2 | {60%,2m,2m} | same as training | 23.15 | 551 | 1.27 |
| InterDigital R1-2307582 | RSRP | 18 direct path RSTD | 120x60 m2 | {60%,2m,2m} | same as training | 0.332 | 11.42 | 10.6 |

Table 3. From excel "POS_Table1 Pos-Supervised-NoLabelerr-Train-Test-Same-Setting_vfinal", tab "Assisted, single-TRP, one model"

|  |  |  |  | Settings (e.g., drops, clutter parameters, mixed dataset) |  | AI/ML complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source | Model input | Model output | UE distribution area | Train | Test | Model complexity (Millions) | Computational complexity (Millions) | Horizontal accuracy @90% (meters) |
| ZTE R1-2306799 | CIR {Nport, NTRP, Nt, Nt'} = {1, 1, 256, 256} | LOS&NLOS indicator | InF-DH: 120x60 m(grid distribution for training dataset; the width of the square grid is 0.5 m) | clutter parameters:{40%, 2m, 2m} | same as training | 1.18 | 7.6 | N/A |
| OPPO R1-2307568 | Normalized CIR (1, 1, 256, 2) | TOA | 120x60 m | 1 drop, 80,000 UEs per drop {40%, 2, 2} | same as training | 0.33M | 0.66 MFLOPs | 0.35 |
| MediaTek R1-2308056 | PDP[18,8,256] | TOA Soft- decision | 120x60m | InF-DH{40%, 2m, 2m} | same as training | 0.187 | 15*18 | 5.6 |
| MediaTek R1-2308056 | CIR[18,8,256] | LOS/NLOS | 120x60m | InF-DH{40%, 2m, 2m} | same as training | 0.186 | 29.4*18 | N/A |
| MediaTek R1-2308056 | CIR[18,8,256] | LOS/NLOS | 120x60m | InF-DH{40%, 2m, 2m} Nt=64 N't=64 | same as training | 0.173 | 7.39*18 | N/A |
| Huawei-R1-2306515 | PDP 1*4*256 TRP(0~17) | LOS probability | 120x60 m | InF-DH{40%, 2m, 2m}, Drop 1 | same as training | 0.001 | 192K | 0.353 |
| vivo R1-2306744 | CIR | TOA | 120x60 m | DH{0.6, 6, 2} | same as training | 11.92 | 23.79 | 0.83 |
| CATT R1-2307080 | Type: CIR; Size: 1*1*256*2 | Type:TOA Size:1 | 120m*60m | InF-DH {60%,6m,2m} with perfect network synchonzation | same as training | 3.84M | 19.48M FLOPs | 0.52m |
| Ericsson R1-2210854 | Time-domain CIR, 1x2x256, complex array | LoS/NLoS classification, ToA estimates | 120x60 m2 | {40%,2m,2m} | same as training | 0.07 M real parameters | 7 M FLOPs | 0.109 |
| Qualcomm R1-2307920 | CER (1, 2, 256) | RSTD distribution | Baseline evaluation area for InF-DH = 120x60 m | {60%, 6m, 2m} | same as training | 2.1 | 37.59 (per TRP) | 4.74 |
| Qualcomm R1-2307920 | CER (1, 2, 64) | RSTD distribution | Baseline evaluation area for InF-DH = 120x60 m | {40%, 4m, 2m} | same as training | 0.02 | 0.21 (per TRP) | 0.53 |

Table 4. From excel "POS_Table1 Pos-Supervised-NoLabelerr-Train-Test-Same-Setting_vfinal", tab "Assisted, single-TRP, N models"

|  |  |  |  | Settings (e.g., drops, clutter parameters, mixed dataset) |  | AI/ML complexity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source | Model input | Model output | UE distribution area | Train | Test | Model complexity (Millions) | Computational complexity (Millions) | Horizontal accuracy @90% (meters) |
| Huawei-R1-2306515 | CIR 1*1*256 TRP(0~17) | TOA | 120x60 m | {60%, 6m, 2m}, Drop 1 | same as training | 0.175 | 7.98 | 0.72 |
| vivo R1-2306744 | CIR | TOA | 120x60 m | DH{0.6, 6, 2} | same as training | 4.26x18 | 8.50x18 | 0.73 |
| CATT R1-2307080 | Type: CIR; Size: 1*1*256*2 | Type:TOA Size:1 | 120m*60m | InF-DH {60%,6m,2m} with perfect network synchonzation | same as training | 23.04M | 116.88M FLOPs | 2.37m |
| Ericsson R1-2304339 | Time-domain PDP, 1x1x256, real array | 1 direct path ToA estimate | 120x60 m2 | {60%,6m,2m} | same as training | 17 M real parameters | 420 M FLOPs | 0.72 |
| Apple-R12308248 | DP [18,1,256,1] | Direct path ToA [1x1] | 100x40 m | InF-DH{60%, 6m, 2m} Drop1 | same as training | 1.6 | 3.1 | 0.9183 |
| Apple-R12308248 | DP [18,1,256,1] | Direct path ToA [1x1] | 100x40 m | InF-DH{60%, 6m, 2m} Drop1 | same as training | 3.88 | 24.95 | 0.8657 |
| Apple-R12308248 | DP [18,1,256,1] | Direct path ToA [1x1] | 100x40 m | InF-DH{60%, 6m, 2m} Drop1 | same as training | 12.37 | 103 | 0.5794 |