3GPP TSG RAN WG1 #116		R1-2400048

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:     9.1.3.3

Source:	Spreadtrum Communications

Title:	             Discussion on other aspects of AI/ML model and data

Document for:	Discussion and decision

Introduction

In Rel-18, the application of AI/ML techniques to NR air interface has been studied. In RAN#102 meeting, it has been agreed to do the normative support for the general framework for AI/ML for air interface, as well as, enable the recommended use cases in the preceding study[1]. In addition, we also agree to further study some issues identified during the Rel-18 study in an attempt to deepen the understanding in view of future normative work.

In this paper, we would share our views on the general part, including data collection for UE-sided model training, model transfer/delivery and model identification/procedure.

Discussion

Data collection for UE-sided model training

In Rel-18 study stage, several of UE data collection mechanisms have been studied and identified. They also have been captured in TR38.843 [2] shown below.

The following proposals were discussed in RAN2: - UE collects and directly transfers training data to the Over-The-Top (OTT) server; 1a) OTT (3GPP transparent) 1b) OTT (non-3GPP transparent) - UE collects training data and transfers it to Core Network. Core Network transfers the training data to the OTT server. - UE collects training data and transfers it to OAM. OAM transfers the needed data to the OTT server. RAN2 did not study or analyse these proposals and did not agree to requirements or recommendations.

In our mind, since the purpose of UE’s data collection is for UE-sided model training, we have not seen any strong justification to let the privacy data exposure to Network. Thus, we prefer mechanism 1a. Given what we have said, the issue is more related to RAN2. We can further discuss about it after some progress of RAN2.

Proposal 1: For data collection for UE-side model training, support 1a or we could wait the progress of RAN2.

Model transfer/delivery

In R18, several of model transfer/delivery cases have been identified, and pros and cons for each case also have been studied in RAN1. With the assumption of model transfer/delivery, RAN2 has done studies from the perspective of signaling procedure.

In [1], it has clearly stated the scope of the study in Rel-19 as shown below.

- Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

For one-sided model, we have not seen the necessarity of model transfer/delivery in 3GPP non-transparent way. Although even if with model transfer/delivery, it may alleviate the burden of model maintenance/storage at the device side. However, the issue of model maintenance/storage would not exist for UE’s server where the model training would happen.

For two-sided model, although model transfer/delivery from NW to UE seems to be beneficial in terms of device storage requirement, and model paring. However, it would face more challenges. There would exist the implementation feasibility issue if NW not aware of the structure at UE. If NW wants to know the structure at UE, obviously proprietary design disclosure concern would arise. In addition, for model pairing, there are other solutions other than model delivery/transfer. Thus, we don’t think it is necessary to consider model transfer/delivery in 3GPP non-transparent way, at least in 5G.

Proposal 2: Not suggest to support model transfer/delivery in 3GPP non-transparent way in 5G.

Model identification

In [1], it has stated that we would continue to study model identification, including necessity and details in Rel-19.

- Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1]

Since collaboration level x is implementation-based AI/ML operation without any dedicated AI/ML-specific enhancement collaboration between network and UE, we focus on level y and level z where there is signaling/procedure related to AI/ML operation. Model/functionality identification/LCM can be applicable only for collaboration level y and level z.

For UE-sided model, and two-sided model, in our understanding, it is necessary to do model/functionality identification, to make NW well understand UE’s capability. For example, if gNB can be aware that UE is capable of beam prediction in spatial domain, RS overhead reduction can be achieved. For NW-sided model, we have not seen clear motivation to do model identification since the management of AI/ML model can be up to NW’s implementation.

In general, functionality identification can be applied at least for one-sided model, since for this case, model information is not necessary and real physical model can be transparent to another side. Correspondingly, functionality level LCM is applied. For two-sided use cases, model based identification can be considered to enable model alignment between two sides.

Proposal 3: At least for two-sided model, model identification can be considered, for the sake of providing pairing of two-sided models

      - Model-ID-based LCM can be considered and provides more granular, model-level management by NW

For model-based identification, in Rel-18 study stage and also in TR38.843 [2], we have agreed three types of model identification where Type A is for offline identification, and Type B1 and Type B2 are for online identification.

- Type A: Model is identified to NW (if applicable) and UE (if applicable) without over-the-air signalling - The model may be assigned with a model ID during the model identification, which may be referred/used in over-the-air signalling after model identification. - Type B: Model is identified via over-the-air signalling, - Type B1: - Model identification initiated by the UE, and NW assists the remaining steps (if any) of the model identification - the model may be assigned with a model ID during the model identification - Type B2: - Model identification initiated by the NW, and UE responds (if applicable) for the remaining steps (if any) of the model identification - the model may be assigned with a model ID during the model identification - Note: This study does not imply that model identification is necessary.

For Type A, a large of air interface overhead can be saved while offline engineering is required. It can be applicable for two-sided model and one-sided model when w/o model delivery/transfer. To some degree, Type B1 and Type B2 can be interpreted as online identification. It is obvious that offline co-engineering can be avoided but it would bring in air interface overhead, spec effort and so on.

For Type B1, it can be used for UE to identify a model developed offline or is available at UE, referring to a specified list of parameters and candidate values, conditions, and/or additional conditions.

For Type B2, one example use case is model identification in model transfer from NW to UE. Another example is model identification with data collection related configuration(s) and/or indication(s) and/or dataset transfer. Based on these, we think Type B2 can be further classified into the following sub-types:

- B2-1: Used along with model transfer from NW to UE

- B2-2: Used for NW to indicate data collection at UE. In this case, a model ID is a logical ID associated with the underlying conditions and additional conditions for the indicated data collection.

- B2-3: Used along with dataset transfer from NW to UE. In this case, a model ID is a logical ID referring to the dataset.

Proposal 4: For model identification Type B2, it can be further classified into three sub-types:

- B2-1: Used along with model transfer from NW to UE

- B2-2: Used for NW to indicate data collection at UE. In this case, a model ID is a logical ID associated with the underlying conditions and additional conditions for the indicated data collection.

- B2-3: Used along with dataset transfer from NW to UE. In this case, a model ID is a logical ID referring to the dataset.

Conclusion

In this contribution, we provide our opinions on general aspects of AI/ML:

Proposal 1: For data collection for UE-side model training, support 1a or we could wait the progress of RAN2.

Proposal 2: Not suggest to support model transfer/delivery in 3GPP non-transparent way in 5G.

Proposal 3: At least for two-sided model, model identification can be considered, for the sake of providing pairing of two-sided models

      - Model-ID-based LCM can be considered and provides more granular, model-level management by NW

Proposal 4: For model identification Type B2, it can be further classified into three sub-types:

- B2-1: Used along with model transfer from NW to UE

- B2-2: Used for NW to indicate data collection at UE. In this case, a model ID is a logical ID associated with the underlying conditions and additional conditions for the indicated data collection.

- B2-3: Used along with dataset transfer from NW to UE. In this case, a model ID is a logical ID referring to the dataset.

Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- TR38.843 V2.0.1, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface