3GPP TSG RAN WG1 #116                                                        R1-2400063

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.5.2

Source:	Spreadtrum Communications

Title: 	Discussion on on-demand SIB1 for idle/inactive mode UEs

Document for:	Discussion and decision

# Introduction

It was agreed in WID [1] to specify on-demand SIB1 for UEs in idle/inactive state.

- Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105

According to SI stage, it is common understanding that on-demand SIB1 can enable a cell switching between on and off, which provides energy saving gain.

# Scenarios

According to WID [1], there is only one triggering method, i.e. UE uplink wake-up-signal (UL WUS) using an existing signal/channel.

There could be some scenarios shown as follows.

    - Scenario-1: After UE is power on, UE perform cell search to get MIB and requests for SIB1 for the current cell.

    - Scenario-2: UE in idle/inactive state camps on a cell without SIB1 (at least receiving paging), and then requests for on-demand SIB1 for the cell.

    - Scenario-3: When UE in idle/inactive state performs cell re-selection to a target cell, UE transmits UL WUS autonomously to the target cell to requests for SIB1 of the target cell.

    - Scenario-4: When UE in idle/inactive state performs random access, gNB commands UE to transmit UL WUS to the target cell to requests for SIB1 of the target cell.

Above scenarios can be illustrated as follows.

(a) Scenarios-1                                                    (b) Scenarios-2

(c) Scenario-3

(d) Scenario-4

Figure 1: Illustration of scenarios for on-demand SIB1

For Scenario-1, it is not so feasible. In general, UE needs to get SIB1 to get UL WUS configuration for the target cell. It seems that UL WUS can be network specific, e.g. common UL WUS occasions and common sequences with cell ID scrambled. However, it will cause some problems, e.g.

    - Lack of flexibility, e.g. predefined UL WUS resource allocation

    - Large spec impact, e.g. new sequence design

    - High complexity at gNB, e.g. new sequence detection

As another way, UE can get UL WUS configuration from the target cell according SIB1 PDCCH or “fake SIB1” PDSCH. However, no matter transmission of SIB1 PDCCH or “fake SIB1” PDSCH by the target cell provides limited network energy saving gain, since the target cell should send PDCCH and/or PDSCH with SIB1 repetition periodicity, i.e. 20ms.

For Scenario-2, it is not feasible. UE cannot camp on a cell without SIB1, e.g. UE cannot get paging configuration and UL WUS configuration for the target cell.

For Scenario-3 and Scenarios-4, it seems feasible. UE can get UL WUS configuration from the source cell.

Observation 1: Scenario-3 and Scenario-4 may be feasible.

From perspective of necessity, Scenario-3 and Scenario-4 can provide network energy saving gain by reducing SIB1 transmission in target cells.

Comparisons between Scenario-3 and Scenario-4 are listed in the following table.

Table 1: Comparisons between Scenario-3 and Scenario-4

|  | Scenario-3 | Scenario-4 |
| --- | --- | --- |
| Event | Cell re-selection | Random access |
| Energy saving gain | Slightly smaller, since UE may perform cell re-selection often | Slightly larger, since the target cell is turned on only when network decides to offload traffic |
| Spec impact | Similar Including UL WUS design, UL WUS configuration, and condition of UL WUS transmission | Similar Including UL WUS design, UL WUS configuration, and condition of UL WUS transmission (including gNB commands) |

It can be observed that Scenario-3 and Scenario-4 have Pros and Cons respectively. Down-selection between Scenario-3 and Scenario-4 can be FFS.

As discussed above, we have the following proposal.

Proposal 1: Study the scenarios:

    - When UE in idle/inactive state performs cell re-selection, UE transmits UL WUS autonomously to the target cell to requests for SIB1.

    - When UE in idle/inactive state performs random access, gNB commands UE to transmit UL WUS to the target cell to requests for SIB1.

Moreover, especially for Scenario-3, condition of UL WUS transmission should be considered, since unconditioned autonomous transmission of UL WUS makes UL interference uncontrollable.

Proposal 2: Condition of UL WUS transmission should be studied.

# UL WUS configuration

As mentioned above, for Scenario-3 and Scenario-4, UL WUS can be configured by the source cell. For Scenario-3, UL WUS configuration can be broadcasted by SIB1 in the source cell, and UE applies UL WUS configuration if UE want to re-selects the target cell. For Scenario-4, UL WUS configuration can be also broadcasted by SIB1 in the source cell, and UE applies UL WUS configuration once upon receiving gNB commands.

Proposal 3: UL WUS can be configured by the source cell.

# UL WUS design

Since UE is in idle/inactive state, it is natural that UE transmits PRACH (preamble) to request for on-demand SIB1. Moreover, UE has only coarse DL sync to target cell and has no UL sync (no TA adjustment) to the target cell, so PRACH is robust for gNB to handle UL interference.

Proposal 4: PRACH (preamble) is candidate of UL WUS for on-demand SIB1.

Since preambles and PRACH occasions (time/frequency resource) are flexible enough, a segment from all preambles and PRACH occasions (ROs in short) can be used for UL WUS. The segment from all preamble is simpler than the segment from all ROs.

Proposal 5: A segment for all preambles can be used for UL WUS for on-demand SIB1.

# Conclusion

We have the following proposals.

Scenarios

Proposal 1: Study the scenarios:

    - When UE in idle/inactive state performs cell re-selection, UE transmits UL WUS autonomously to the target cell to requests for SIB1.

    - When UE in idle/inactive state performs random access, gNB commands UE to transmit UL WUS to the target cell to requests for SIB1.

Proposal 2: Condition of UL WUS transmission should be studied.

UL WUS configuration

Proposal 3: UL WUS can be configured by the source cell.

UL WUS design

Proposal 4: PRACH (preamble) is candidate of UL WUS for on-demand SIB1.

Proposal 5: A segment for all preambles can be used for UL WUS for on-demand SIB1.

# Reference

- RP-234065, “New WID: Enhancements of network energy savings for NR”, Ericsson, RAN#102, Dec.  11th - 15th, 2023.