3GPP TSG RAN WG1 Meeting #116	R1-2400087

Athens, Greece, February 26 – March 1, 2024

Agenda Item:	9.4.1.1

Source:	Futurewei

Title:	Discussion on evaluation assumptions and results for Ambient IoT devices

Document for:	Discussion and Decision

# Introduction

In the approved new SID for Ambient IoT [1], the study objectives, where RAN1 is responsible, are as follows:

The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. - Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR base station. For Topology 2, no difference in physical layer design from Topology 1.

In this contribution, we present our views about Ambient IoT evaluation assumptions taking into account deployment scenarios, and connectivity topologies as outlined in the general scope of the SID. Based on the evaluation assumptions, we provide initial results for link budget analysis.

# Discussion

An overview of the main characteristics of the Rel-19 Ambient IoT general scope is summarized in Table 1. For a detailed description of the Rel-19 Ambient IoT general scope, refer to [1].

Table 1: Rel-19 Ambient IoT general scope overview

| Device peak power consumption | Low-power device | 1 µW |  |
| --- | --- | --- | --- |
|  | High-power device | ≤ a few hundred microwatts |  |
| Use case |  | rUC1 (indoor inventory) | rUC4 (indoor command) |
| Traffic type |  | DO-DTT | DT |
| Spectrum |  | NR FR1 licensed frequency bands |  |
| Duplex scheme |  | FDD (uplink transmission including backscattering can occur on uplink frequency bands) |  |
| Deployment scenario and topology |  | Deployment Scenario 1 with Topology 1 | Deployment Scenario 2 with Topology 2 (UE as an intermediate node) |

Referring to TR 38.848, the representative use cases for rUC1 and rUC4 are presented in Table 2.

Table 2: Use cases for rUC1 and rUC4

| rUC | Applicable SA1 use cases |
| --- | --- |
| rUC1 (indoor inventory) | 5.1 Automated warehousing 5.2 Medical instruments inventory management and positioning 5.4 Non-Public Network for logistics 5.5 Automobile manufacturing 5.7 Airport terminal / shipping port 5.15 Smart laundry 5.16 Automated supply chain distribution 5.18 Fresh food supply chain 5.27 End-to-end logistics 6.1 Flower auction 6.3 Electronic shelf label |
| rUC4 (indoor command) | 5.11 Online modification of medical instruments status 5.17 Device activation and deactivation 5.26 Elderly Health Care 5.29 Device Permanent Deactivation 6.3 Electronic shelf label |

## Ambient IoT Devices

### Type of Devices

Two types of Ambient IoT devices are defined for the study [1]. To align the discussion in the study we have the following proposal:

Proposal 1: For future meetings on Ambient IoT use the following terminology for discussion,

Low-power Ambient IoT devices (LP-Device): ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

High-power Ambient IoT devices (HP-Device): ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device or be backscattered on a carrier wave provided externally.

In gNB and LP-Device communication, the gNB modulates the RF signal to transmit message in the downlink direction. The LP-device processes the message and sends response back by modulating the reflection coefficient of its antenna, so-called backscattering an information signal to the gNB in the uplink direction. Note that an RF CW source is needed to enable LP-Device to backscatter. The RF source can be either from the gNB or from another node.

A typical block diagram of an LP-Device is shown in Figure 1. The main components are antenna, Analog Front-End (AFE), baseband processing unit and EEPROM.

Baseband Processing Unit

Analogy FE

Demodulator

Decoding/

Encoding

Control Logic

Oscillator

EEPROM

Modulator

Rectifier/DC-Supply

Figure 1: LP-Device function block diagram

In one possible implementation, the LP-Device first converts an incoming RF signal into RF energy and stores it in a charge capacitor to get a DC supply voltage to the device. The AFE demodulates the downlink signal with an RF envelope detector and the signal is then digitized with a comparator. In baseband processing unit the digitized signal is decoded. This decoded message is processed and the response from the device is generated and sent back through backscattering by modulating the incoming CW which is provided externally to the device.

Observation 1: LP-Device is likely powered by RF.

An HP-Device can operate similarly to an LP-Device. In addition, the HP-Device can also harvest other sources of energy, and it may have downlink and uplink RF amplification capability. Our companion contribution has more discussion on the HP-Device block diagram and functionalities [3].

FR1 licensed spectrum in FDD is selected for the study [1]. Usually in a typical NR FDD configuration, UE works in a full-duplex mode. Due to backscattering, Ambient IoT devices work in half duplex mode.

Observation 2: Ambient IoT devices, which rely on backscattering for UL transmission, work in half duplex mode even in FDD band, meaning they can only either receive or transmit at a given time.

According to SID [1] for the uplink, an HP-Device may backscatter an external CW or generate an internal uplink signal. To harmonize the uplink interface, it is proposed that all HP-Devices, whether or not having internal uplink signal generation capability, support the same uplink modulation waveforms as LP-Devices. For example, for an HP-Device that can generate uplink signal internally it will generate the uplink signal in the same modulation form as if it backscatters an external CW source.

Proposal 2: To harmonize uplink interface for both LP-Device and HP-Device it is proposed that HP-Devices, whether or not having capability to generate UL internally, support the same UL transmission waveform as low-power Ambient IoT devices.

Based on [1] HP-Devices may have downlink and uplink amplification. This will enhance HP-Device’s receive sensitivity and downlink/uplink coverage for transmitting and receiving.

Observation 3: HP-Device that has internal DL and UL amplification provides increased coverage compared to LP-Devices.

Due to its low power consumption level, an LP-Device can have very limited frequency selectivity, with possibly some sort of low pass filtering at best. On the other hand, it is envisioned that HP-Devices can have some channel selection [3].

Observation 4: HP-Device likely has channel selectivity capability while LP-Device likely does not. This will impact coexistence characteristics with NR devices.

### Sampling Frequency Offset

As shown in Fig. 1, the clock generator is a vital element in the signal front-end for decoding the downlink data, encoding the uplink data and clocking the command handler in the baseband processor. One of the critical challenges of the clock generator design is the stringent frequency accuracy requirement needed for the uplink data in the protocol.

To generate the required clock, a viable solution is to extract it from the incoming RF signal. However, such an approach requires a chain of dividers to convert the ultra-high-frequency (UHF) carrier (860 MHz to 960 MHz) to the required baseband clock frequency. The complexity and power overhead imposed by the dividers typically rules out this scheme in LP-Devices and suggests using a local oscillator instead. Passive LC oscillators and ring oscillators (ROs) are the two main categories of oscillators in CMOS technology. Ring oscillators are not inherently dependent upon passive components (e.g., inductors) which is greatly desirable for silicon area considerations.

A 3-stage ring oscillator is shown in Figure 2[15].

Figure 2: A 3-stage ring oscillator.

The challenges concerning the oscillator are the ultra-low power consumption and the lack of post-fabrication trimming procedure to tune the target frequency. Low-cost demand is only possible with bulk CMOS technology without modifications after the manufacturing process [14-15].

A typical oscillator can have as much as ±14% frequency deviation due to process, supply voltage and temperature variations. Reported solutions to cope with the frequency deviation of clock generators include clock recovery circuit, temperature compensation, among others. The frequency deviation can be reduced to 3% to 5% [12-15].

Observation 5: The sampling frequency deviation for a local oscillator is about 3% to 5% after compensation.

## Link budget analysis

### Evaluation Assumptions

To conduct a link budget analysis initial assumptions are made on the evaluation parameters. These assumptions are based on technical papers and industrial product datasheets.

The following are these initial assumptions:

Operation scenario: Indoor Factory, InF pathloss model is per 3GPP TS 38.901 [4].

Carrier Frequency: we choose FDD Band n8 as the evaluation. Band n8 is 880 to 915MHz for the UL band and 925 to 960MHz for the DL band. 920MHz is selected as the carrier frequency for both UL/DL for simplicity.

gNB Antenna gain: 6 dB is chosen for gNB transmit and receive antenna gain. This number could be higher depending on deployments and configurations.

UE Antenna gain: 0 dB is chosen for UE transmit and receive antenna gain.

A IoT antenna gain: 0 dB is chosen for Ambient IoT device transmit and receive antenna gain.

A IoT receive sensitivity/activation: for an LP-Device, it is the activation level where the LP-Device is powered on by the RF to start demodulating the DL signal. The reported numbers are from -20 dBm to -25.5 dBm from the technical papers and industrial datasheets [7-9]. -24 dBm is used in the analysis. For the HP-Device, this represents the receive sensitivity. The reported numbers are from -35 dBm to -42 dBm. -40 dBm is used in the analysis [9-11].

Loss Margin (Downlink and Uplink): 5 dB is used for the loss margin for both DL and UL. The loss is accounted for fading and polarization mismatching etc.

Return Loss/back scatter efficiency: this is the loss during backscattering. It is also called modulation factor. 6 dB is used in analysis.

Reflection Amplifier gain factor: this is the amplification stage for backscattering. An HP-Device may have this capability. 10 dB is used in the analysis [16].

gNB receive sensitivity: it is based on TS 38.104 medium range BS reference sensitivity assuming a 3MHz bandwidth. -98.6 dBm is used in analysis.

UE receiver sensitivity: it is based on TS 38.101-1 for Band n8. -97 dBm is used in analysis.

### Deployment Scenario 1 with Topology 1

According to TR 38.848 for Deployment Scenario 1, both the base stations (or gNBs) and Ambient IoT devices are situated in the indoor environment. The structure of Topology 1 is illustrated in Figure 3.

Figure 3: Topology 1

For this scenario two cases are analyzed. The first case is for gNB and CW node being integrated (monostatic configuration) while the second case assumes that the gNB and CW node are separate (bistatic configuration).

#### gNB in monostatic configuration

The following table is the link budget for both LP and HP-Devices. The gNB transmission power levels are adjusted to make the coverage at 10 meters for LP-Device.

Table 3: Link budget for monostatic configuration coverage

| Indoor Factory (pathloss takes the worse of LOS and NLOS) |  |  |  |
| --- | --- | --- | --- |
|  | LP-Device (RF powered) | LP-Device (Pre-charged) | HP-Device |
| Downlink parameters |  |  |  |
| Carrier Frequency (GHz) | 0.92 | 0.92 | 0.92 |
| gNB Tx power (dBm) | 30.00 | 30.00 | 30.00 |
| Tx antenna gain (dB) | 6 | 6 | 6 |
| A IoT receive sensitivity/activation (dBm) | -24 | -40 | -40 |
| A IoT antenna gain (dB) | 0 | 0 | 0 |
| Loss margin (Downlink) (dB) | 5 | 5 | 5 |
| Path loss (dB) Downlink | 55.00 | 71.00 | 71.00 |
| Maximum coverage distance Downlink (m) | 10.33 | 51.26 | 51.26 |
| Uplink link parameters |  |  |  |
| Return Loss/back scatter efficiency (dB) | 6 | 6 | 6 |
| A IoT antenna gain (dB) | 0 | 0 | 0 |
| Reflection Amplifier gain factor (dB) | 0 | 0 | 10 |
| Rx antenna gain (dB) | 6 | 6 | 6 |
| Loss margin (Uplink) (dB) | 5 | 5 | 5 |
| gNB receive sensitivity (dBm) | -98.6 | -98.6 | -98.6 |
| Path loss (dB) Uplink if DL coverage limited | 69.6 | 53.6 | 63.6 |
| Path loss (dB) Uplink if UL coverage limited | 62.30 | 62.30 | 67.30 |
| Path loss (dB) Uplink | 69.60 | 62.30 | 67.30 |
| Maximum coverage distance Uplink (m) | 44.55 | 21.45 | 35.39 |
| Monostatic configuration coverage (m) | 10.33 | 21.45 | 35.39 |

Notes:

- On the UL the path loss is evaluated for both DL limited case and UL limited case. The difference is the power level at the backscatter point.

- For the DL limited case, the power level at the backscatter point is the LP-Device activation level, for example -24 dBm in the LP-Device column of this table.

- For the UL limited case, the power level at the backscatter point is the power level that makes UL and DL coverage same. For example, it is -31.3 dBm in the LP-Device of this table.

- The final Uplink path loss is the higher value of the two cases. For example, in the LP-Device column the final path loss is 69.6dB which is the higher value of 62.3 and 69.6.

Observation 6: (a) RF powered low-power Ambient IoT devices have the DL coverage limited due to the -24 dBm activation level;

(b) High power and pre-charged low-power Ambient IoT devices have increased DL coverage compared to RF powered low-power devices;

(c) High-power Ambient IoT devices have increased UL coverage due to reflection amplifier gain.

Proposal 3: For coverage of Deployment Scenario 1 with Topology 1, adopt the evaluation assumptions listed in Table 3 for low-power and high-power Ambient IoT devices.

#### gNB with separate carrier wave node

In this scenario the gNB cell is divided into multiple zones and each zone is covered by a carrier wave node.

The following is the link budget for LP/HP-Device with carrier wave node assisting.

Table 4: Link budget for bistatic configuration coverage

| Indoor Factory (pathloss takes the worse of LOS and NLOS) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | LP-Device |  | HP-Device |  |
| Downlink parameters | CW | DL | CW | DL |
| Carrier Frequency (GHz) | 0.92 | 0.92 | 0.92 | 0.92 |
| gNB/RF source Tx power (dBm) | 30.00 | 30.00 | 30.00 | 30.00 |
| Tx antenna gain (dB) | 6 | 6 | 6 | 6 |
| A IoT receive sensitivity/activation (dBm) | -24 | -40 | -24 | -40 |
| A IoT antenna gain (dB) | 0 | 0 | 0 | 0 |
| Loss margin (Downlink) (dB) | 5 | 5 | 5 | 5 |
| Path loss (dB) Downlink | 55.00 | 71.00 | 55.00 | 71.00 |
| Maximum coverage distance Downlink (m) | 10.33 | 51.26 | 10.33 | 51.26 |
| Uplink parameters |  |  |  |  |
| Carrier wave power at A IoT device (dBm) |  | -24 |  | -24 |
| Return Loss/back scatter efficiency (dB) |  | 6 |  | 6 |
| A IoT antenna gain (dB) |  | 0 |  | 0 |
| Reflection Amplifier gain factor (dB) |  | 0 |  | 10 |
| Rx antenna gain (dB) |  | 6 |  | 6 |
| Loss margin (Uplink) (dB) |  | 5 |  | 5 |
| gNB receive sensitivity (dBm) |  | -98.6 |  | -98.6 |
| Path loss (dB) Uplink |  | 69.60 |  | 79.60 |
| Maximum coverage distance Uplink (m) |  | 44.55 |  | 121.24 |
| Bistatic configuration coverage (m) |  | 44.55 |  | 51.26 |

Table 4: Link budget for bistatic configuration coverage

Notes:

- Two columns are used to represent the link budget for CW node link and gNB signal link.

- CW node transmission power is adjusted to have 10-meter DL coverage.

- CW node provides the backscatter signal for both LP-Device and HP-Device.

- LP-Device’s receive sensitivity is assumed to -40 dBm if it is pre-charged by CW node.

Observation 7: Compared to monostatic configuration, with same gNB transmission power of 30 dBm, gNB coverage for LP-Devices is extended from ~10 meter to ~45 meter, a result by configuring CW nodes. In other words, HP and LP devices can have similar coverage under a dense CW node deployment.

### Deployment Scenario 2 with Topology 2

According to TR 38.848 for Deployment Scenario 2, the base stations (or gNBs) are outdoors and the Ambient IoT devices are situated in the indoor environment. The structure of Topology 2 is illustrated in Figure 4. As stated in Table 1, the UE, which is indoor, serves as an intermediate node between the Ambient IoT device and the base station, and is under network control.

(UE)

Figure 4: Topology 2

A complete link budget for the low-power and high-power Ambient IoT devices is shown in Table 4.

Table 5: Link budget for Deployment Scenario 2 with Topology 2

| Indoor Factory (pathloss takes the worse of LOS and NLOS) |  |  |  |
| --- | --- | --- | --- |
|  | LP-Device (RF powered) | LP-Device (pre-charged) | HP-Device |
| Downlink parameters |  |  |  |
| Carrier Frequency (GHz) | 0.92 | 0.92 | 0.92 |
| UE Tx power (dBm) | 36.00 | 29.00 | 36.00 |
| Tx antenna gain (dB) | 0 | 0 | 0 |
| A IoT receive sensitivity/activation (dBm) | -24 | -40 | -40 |
| A IoT antenna gain (dB) | 0 | 0 | 0 |
| Loss margin (Downlink) (dB) | 5 | 5 | 5 |
| Path loss (dB) Downlink | 55.00 | 64.00 | 71.00 |
| Maximum coverage distance Downlink (m) | 10.33 | 25.43 | 51.26 |
| Uplink parameters |  |  |  |
| Return Loss/back scatter efficiency (dB) | 6 | 6 | 6 |
| A IoT antenna gain (dB) | 0 | 0 | 0 |
| Reflection Amplifier gain factor (dB) | 0 | 0 | 10 |
| Rx antenna gain (dB) | 0 | 0 | 0 |
| Loss margin (Uplink) (dB) | 5 | 5 | 5 |
| gNB receive sensitivity (dBm) | -97 | -97 | -97 |
| Path loss (dB) Uplink if DL coverage limited | 62 | 46 | 56 |
| Path loss (dB) Uplink if UL coverage limited | 58.50 | 55.00 | 63.50 |
| Path loss (dB) Uplink | 62.00 | 55.00 | 63.50 |
| Maximum coverage distance Uplink (m) | 20.82 | 10.33 | 24.19 |
| Monostatic configuration coverage (m) | 10.33 | 10.33 | 24.19 |

Observation 8: For Scenario 2 Topology 2, significant intermediate UE transmit power may be required to keep coverage in the 10-50m range with the assumption that no transmit or receive antenna gains

Observation 9: For pre-charged LP-Devices, a lower UE transmit power is needed to reach 10m coverage.

Proposal 4: For coverage of Deployment Scenario 2 with Topology 2, adopt the evaluation assumptions listed in Table 5 for low-power and high-power Ambient IoT devices.

# Conclusion

This contribution has discussed

Proposal 1: For future meetings on Ambient IoT use the following terminology for discussion,

Low-power Ambient IoT devices (LP-Device): ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

High-power Ambient IoT devices (HP-Device): ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device or be backscattered on a carrier wave provided externally.

Proposal 2: To harmonize uplink interface for both LP-Device and HP-Device it is proposed that HP-Devices, whether or not having capability to generate UL internally, support the same UL transmission waveform as low-power Ambient IoT devices.

Proposal 3: For coverage of Deployment Scenario 1 with Topology 1, adopt the evaluation assumptions listed in Table 3 for low-power and high-power Ambient IoT devices.

Proposal 4: For coverage of Deployment Scenario 2 with Topology 2, adopt the evaluation assumptions listed in Table 5 for low-power and high-power Ambient IoT devices.

Observation 1: LP-Device is likely powered by RF.

Observation 2: Ambient IoT devices, which rely on backscattering for UL transmission, work in half duplex mode even in FDD band, meaning they can only either receive or transmit at a given time.

Observation 3: HP-Device that has internal DL and UL amplification provides increased coverage compared to LP-Devices.

Observation 4: HP-Device likely has channel selectivity capability while LP-Device likely does not. This will impact coexistence characteristics with NR devices.

Observation 5: The sampling frequency deviation for a local oscillator is about 3% to 5% after compensation.

Observation 6: (a) RF powered low-power Ambient IoT devices have the DL coverage limited due to the -24 dBm activation level;

(b) High power and pre-charged low-power Ambient IoT devices have increased DL coverage compared to RF powered low-power devices;

(c) High-power Ambient IoT devices have increased UL coverage due to reflection amplifier gain.

Observation 7: Compared to monostatic configuration, with same gNB transmission power of 30 dBm, gNB coverage for LP-Devices is extended from ~10 meter to ~45 meter, a result by configuring CW nodes. In other words, HP and LP devices can have similar coverage under a dense CW node deployment.

Observation 8: For Scenario 2 Topology 2, significant intermediate UE transmit power may be required to keep coverage in the 10-50m range with the assumption that no transmit or receive antenna gains

Observation 9: For pre-charged LP-Devices, a lower UE transmit power is needed to reach 10m coverage.

# References

RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, Huawei.

3GPP TR 38.848, “Study on Ambient IoT (Internet of Things) in RAN”, v18.0.0 (2023-09).

R1-2400088, Discussion on Ambient IoT device architectures, RAN1#116, Futurewei, February 26 – March 1, 2024.

3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz (Release 17)”, v17.0.0 (2022-03)

3GPP TR 38.875, “Study on support of reduced capability NR devices (Release 17)”, v17.0.0 (2021-03)

3GPP TR 38.901, “Study on low power wake up signal and receiver for NR (Release 18)”, v2.0.0 (2023-12)

NXP UCODE-9xm UCODE 9xm | NXP Semiconductors

Datasheet “IMPINJ M830/M850 SERIES TAG CHIPS”.

X. Lu, P. Wang, D. Niyato, D. I. Kim, and Z. Han, “Wireless networks with RF energy harvesting: A contemporary survey,” IEEE Commun. Surveys Tuts., vol. 17, no. 2, pp. 757–789, 2nd Quart., 2015.

P. H an, N. Mei, and Z. Zhang, “A UHF semi-passive RFID system with photovoltaic/thermoelectric energy harvesting for wireless sensor networks,” in Proc. IEEE 13th Int. Conf. ASIC (ASICON), Oct. 2019, pp. 1–4.

Gang Li, Mengqi Chen, Fading Zhao, Zhenbing Li, Daniele Inserra, Gang Li, Guangjun Wen, "A Reflection-Amplifier-Based Long-Range Semi-Passive UHF RFID Tag with Standard Compatibility", IEEE Microwave and Wireless Technology Letters, vol.33, no.9, pp.1373-1376, 2023.

V. Pillai, H. Heinrich, D. Dieska, P. V. Nikitin, R. Martinez, and K. V. S. Rao, “An ultra-low-power long range battery/passive RFID tag for UHF and microwave bands with a current consumption of 700 nA at 1.5 V,” IEEE Trans. Circuits Syst. I, Reg. Papers, vol. 54, no. 7, pp. 1500–1512, Jul. 2007

N. Cho, S.-J. Song, J.-Y. Lee, S. Kim and H.-J. Yoo, " A 8- uW 0.3- mm2 RF-powered transponder with temperature sensor for wireless environmental monitoring ", Proc. IEEE Int. Symp. Circuits and Systems, vol. 5, pp. 4763-4766, 2005-May.

C.-F. Chan, K.-P. Pun, K.-N. Leung, J. Guo, L.-K. L. Leung, and C.-S. Choy, “A low-power continuously-calibrated clock recovery circuit for uhf rfid epc class-1 generation-2 transponders,” IEEE Journal of solidstate circuits, vol. 45, no. 3, pp. 587–599, 2010.

L. Dai, R. Harjani, “Design of Low-Phase-Noise CMOS Ring Oscillators”, IEEE Trans. On Circuit and Systems, vol. 49, no. 5, pp. 328-338, May 2002.

J. Kimionis, A. Georgiadis, A. Collado and M. M. Tentzeris, "Enhancement of RF tag backscatter efficiency with low-power reflection amplifiers", IEEE Trans. Microw. Theory Techn., vol. 62, no. 12, pp. 3562-3571, Dec. 2014.