3GPP TSG-RAN WG1 Meeting #116	R1-2400104

Athens, Greece, February 26 – March 1, 2024

Agenda Item:	9.2.1

Source:	Huawei, HiSilicon

Title:	On UE initiated/event-driven beam management

Document for:  Discussion and Decision

# Introduction

In RAN#102 [1], it was agreed to specify UE-initiated/event-driven beam management in Rel-19:

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In this paper, we discuss the use cases, and other critical issues relevant to this objective.

# Discussion

In current spec, both P2 Tx and P3 Rx beam refinements are supported in gNB-initiated beam management procedures where the former uses Tx beam sweeping to achieve Tx beam refinement and the latter uses Rx beam sweeping to achieve Rx beam refinement. Rel-19 UE-initiated beam management should aim at enhancing the overall efficiency and reducing the overhead/latency of current Tx and Rx beam refinement procedures.

## UE-initiated BM for Tx beam refinement

For P2 Tx beam refinement procedure, in current spec, a CSI-RS resource set is configured or activated for UE to measure a set of Tx beams. Due to the large number of gNB Tx beams (e.g., 128+), it is not practical for the UE to measure all supported Tx beams in every direction. Therefore, CSI-RS resources measured in P2 procedure correspond to only a subset of Tx beams. Once the quality of this CSI-RS resource set gets worse, gNB needs to update the CSI-RS resource set measured in the P2 procedure. One approach to such CSI-RS resource set update is RRC reconfiguration which causes a large latency to the P2 Tx beam refinement procedure. An alternative approach to reduce the latency is to configure multiple SP CSI-RS resource sets corresponding to multiple report configuration and, when the UE reported quality of current CSI-RS resource set is poor, activate another CSI-RS resource set for the UE measurement. The activation delay is much smaller than the RRC reconfiguration delay. However, finding the best Tx-Rx beam pair still requires performing UE measurements on the newly-activated CSI-RS resource set over several CSI-RS periodicities using different Rx beams only after reported measurements on the formerly-activated CSI-RS resource set was deemed poor. Such sequential measurements on the CSI-RS resource sets is a major contributing factor for the latency of the P2 Tx beam refinement procedure.

Observation 1: In the legacy gNB-initiated beam management, reconfiguration and measurement of CSI-RS resources for Tx beam refinement cause a large latency.

UE-initiated BM procedure can be used to resolve the large latency of Tx beam refinement procedure. For instance, the UE may be configured with two report configurations which are associated with two  CSI-RS resource sets, respectively. The second resource set may be transmitted such that it corresponds to a set of candidate beams, for instance, within the beam span of the second best SSB. To save UL resources and UE power, UE reports only the measurement results of the first CSI-RS set according to the first report configuration and is not required to periodically report the measurement results of the second CSI-RS set. Instead, the reporting of the measurement results of the second CSI-RS set can be initiated by UE only when it is deemed necessary, e.g., when one Tx beam corresponding to the second CSI-RS set is better than the best Tx beam corresponding to the first CSI-RS set. Such solution can reduce UL resource and power consumption. In addition, compared to the legacy procedure, it supports a fast Tx beam tracking as the UE can measure the set of candidate CSI-RS resources in advance. Based on the above discussion, we have the following proposal.

Proposal 1: For UE-initiated/event-driven beam management, support UE-initiated CSI report based on current CSI framework.

## UE-initiated BM for Rx beam refinement

For Rx beam refinement procedure, in current spec, gNB may transmit a CSI-RS resource set with the higher layer parameter repetition set to “ON” and UE uses different Rx beams to measure the same Tx beam and determine the corresponding best Rx beam. In practice, Rx beam refinement needs to be performed when best Rx beam changes due to UE movement or rotation. However, as gNB is not aware of the Rx beam change at UE side, it is usually not possible for gNB to trigger/activate the measurement of a AP/SP CSI-RS resource set with repetition “ON” to facilitate refinement of UE Rx beam in a timely manner. This results in a large latency of Rx beam refinement. To reduce the Rx beam refinement latency, gNB may transmit a P-CSI-RS resource set with repetition “ON”, however, this entails a large RS overhead and unnecessary UE measurements.

Observation 2: In the legacy gNB-initiated beam management, Rx beam refinement procedure causes a large latency and/or large RS overhead and unnecessary UE measurements since gNB does not know when UE needs to refine its Rx beam.

This problem can be solved by UE-initiated BM. For instance, when UE finds that the Rx beam should be modified due to its movement or rotation, it can send a request to gNB to perform P3 procedure; letting gNB know that it needs to trigger/activate the measurement of a CSI-RS resource set with repetition “ON” and, therefore, resulting in a substantial reduction in the Rx beam refinement latency. The above UE-initiated P3 procedure includes both UE-initiated beam measurement and the corresponding beam reporting (if any) and is comprised of the following 4 steps as shown in Figure 1:

- Step 1: UE send a BM request message to gNB to request to perform particular beam management, e.g., request to perform Rx beam sweeping when the Rx beam needs to be changed at the UE side.

- Step 2: gNB send a response message to inform UE whether the requested beam management is allowed.

- Step 3: If the BM request from UE is allowed by gNB, gNB can subsequently send BM RS to UE to perform beam measurement.

- Step 4: After beam measurement, UE reports the measurement results to gNB.

Figure 1. UE-initiated beam measurement/reporting procedure

Compared to the legacy BM procedure, above procedure introduces two new steps: UE request and gNB response. These two steps are to ensure that, similar to other procedures in the network, gNB still controls the UE-initiated BM procedure. Otherwise, gNB cannot know when to transmit the BM RS and to schedule the beam report.

Proposal 2: For UE-initiated beam measurement/reporting procedure, support the following two steps prior to the beam measurement and the subsequent reporting: 1) UE BM request; 2) gNB response.

## Other critical issues

In UE-initiated BM, the corresponding signalling, e.g., UE request, gNB response, and beam report could be transmitted in the CC where the measurement is performed. However, since UE-initiated BM is usually performed when the used Tx-Rx beam pair of the CC is compromised, i.e., Rx beam needs to be changed or the Tx beam quality is degraded, signalling transmission in the same CC may not be reliable. For instance, UE may send a request message to initiate a beam measurement when the quality of its serving beam of a FR2 CC falls below a threshold. In such situation, the transmission of the UE request message and the following gNB response and UE report message may fail due to the bad quality of the serving beam. In this case, it is better to transmit the signalling via other CCs, e.g., a FR1 CC, to ensure the reliability of the UE-initiated BM procedure.

Proposal 3: To ensure the reliability of UE-initiated beam management, corresponding signalling including UE request, gNB response, and beam report can be transmitted via a CC1 (e.g., FR1 CC) while the RS measurements are performed on CC2.

Another issue is that current beam management is based on pre-defined beam pattern (e.g., DFT). UE can only report one of the pre-defined beams for data transmission. When the channel path is located in the center of two beams, neither beam is good enough as shown at the left subfigure of Figure 2. Hence, no matter which one is reported to the gNB for data transmission, performance of data transmission may be degraded.

To solve this problem, a new report quantity may be introduced to enable UE to report additional beam-related information besides RSRP and the fixed beam ID. For example, UE can report a suggested combination coefficient of the two beams. With such combination factor, the gNB can calculate a new beam which is better than the two original beams. For example, as shown at the right subfigure of Figure 2, with reported beam ID #1 and #2 and the combination coefficient of beam #1 and #2, gNB can calculate the weight of a new beam which would match the channel path better.

Proposal 4: Study a new report quantity, i.e., beam combination factor, for Tx beam refinement.

Figure 2. Example of combined beam

# Summary and conclusion

In this contribution, we have the following observations and proposals:

Observation 1: In the legacy gNB-initiated beam management, reconfiguration and measurement of CSI-RS resources for Tx beam refinement cause a large latency.

Observation 2: In the legacy gNB-initiated beam management, Rx beam refinement procedure causes a large latency and/or large RS overhead and unnecessary UE measurements since gNB does not know when UE needs to refine its Rx beam.

Proposal 1: For UE-initiated/event-driven beam management, support UE-initiated CSI report based on current CSI framework.

Proposal 2: For UE-initiated beam measurement/reporting procedure, support the following two steps prior to the beam measurement and the subsequent reporting: 1) UE BM request; 2) gNB response.

Proposal 3: To ensure the reliability of UE-initiated beam management, corresponding signalling including UE request, gNB response, and beam report can be transmitted via a CC1 (e.g., FR1 CC) while the RS measurements are performed on CC2.

Proposal 4: Study a new report quantity, i.e., beam combination factor, for Tx beam refinement.

# References

[1] RP-234007, New WID: NR MIMO Phase 5, RAN #102, December 11-15, 2023