3GPP TSG-RAN WG1 Meeting #116                                                                                                                                                                                                                                                                                    	R1-2400131

Athens, Greece, February 26th -March 1st, 2024

Agenda Item:	8.12.2

Source:	Huawei, HiSilicon

Title:	UE features for NR sidelink evolution

Document for:	Discussion and Decision

- Introduction

This contribution discusses the UE feature list for R18 NR SL, including SL-U Channel Access, SL-U PHY design, co-channel coexistence for LTE sidelink and NR sidelink and SL CA. We give our views based on the latest endorsed UE feature list after RAN1#115 in R1-2312572 [1] (the proposed full list is in Appendix 1, our proposed changes are marked in red).

- Discussion

- SL-U channel access mechanism

FG 47-k1 SL channel access for dynamic channel access mode

In the UE features list after RAN1#115, FG 47-k1 is as follows.

| 47. NR_SL_enh2 | 47-k1 | SL channel access for dynamic channel access mode | UE supports 1. SL Type 1 channel access and contention window size adjustment 2. SL Type 2A channel access 3. SL Type 2B channel access 4. SL Type 2C channel access 5. 20MHz LBT bandwidth 6. CP extension up to 1 symbol in 15kHz SCS if the UE supports 15 kHz SCS 7. CP extension up to 2 symbols in 30kHz SCS 8. CP extension up to 2 symbols if the UE supports 60kHz SCS [9. Monitoring SCI to decode] FFS whether to merge with 47-k3 | At least one of {15-25, 15-3, [32-4, 32-4a]} | Yes | No | UE does not support channel access for NR sidelink operation in shared spectrum. | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. Note: Component 8 is applicable in regions without OCB requirements. Note1: If UE supports 15-25, the UE is not required to support Component 3 and 4 in 15-2. Note2: If UE supports 15-3, the UE is not required to support Component 3 in 15-3, and FR2 parts of Component 7 in 15-3. Note: It is up to RAN2 whether/how to implement the above Notes 1/2 and whether/how to update the prerequisite FGs | Optional with capability signalling For UE supports NR SL in unlicensed spectrum and when shared spectrum channel access must be used, UE must indicate this FG is supported |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The columns with yellow highlights can be updated as below:

- FG 47-k3 for COT sharing needs to be kept separate because it is not a basic FG in unlicensed spectrum as FG 47-k1, i.e., a UE may not support COT sharing. Thus, the FFS in the components should be removed.

- Component 9 should be removed. FG 47-k1 is the UE feature for SL Type 1/2A/2B/2C channel access and CPE, which is applicable to sharing cases and non-sharing cases. For sharing cases, no need to introduce Component 9 because it has been defined clearly in FG 47-k3. For non-sharing cases, e.g. short control signalling transmission, the capability of monitoring SCI is unnecessary.

- The brackets of prerequisites can be removed because SL channel access is also applicable to partial sensing and random selection.

FG 47-k2 SL multi-channel access for dynamic channel access mode

In the UE features list after RAN1#115, FG 47-k2 is as follows.

| 47. NR_SL_enh2 | 47- k2 | SL multi-channel access for dynamic channel access mode | 1. UE supports multi-channel access procedures for PSCCH/PSSCH/S-SSB/PSFCH transmission(s) in multiple RB sets in a slot 2. UE supports Type A and Type B multi-channel access procedures for PSFCH transmissions in multiple RB sets in a slot 4) UE supports multi-channel access procedure on N channel(s) with 20MHz LBT bandwidth for each channel. Candidate values of N: {2, 3, 4, 5} | 47-k1 | Yes | [No] | UE does not support multi-channel access in dynamic channel access mode for NR sidelink operation in shared spectrum. | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. Note: Support of S-SSB/PSFCH transmission(s) in multiple RB-sets in a slot is according to the support of {47-m11, 47-m11a} and {47-m12, 47-m12a} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The columns with yellow highlights can be updated as below:

- FG 47-k2 is not limited to unicast so it should not depend on exchange between UEs. The bracket for “No” should be removed.

FG 47-k3 and FG 47-k4 Transmitting /Receiving UE to UE COT sharing information

In the UE features list after RAN1#115, FG 47-k3 and 47-k4 are as follows.

| 47. NR_SL_enh2 | 47- k3 | Receiving UE to UE COT sharing information | 1. UE supports monitoring SCI to read COT sharing information 2. UE supports transmitting NR SL based on COT sharing information subject to COT sharing conditions FFS whether to merge with 47-k1 | 47-k1 | No | [No] | UE does not support using UE-to-UE COT sharing information contained in SCI for sharing COT for NR sidelink operation in shared spectrum. | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | [Optional with capability signalling] [For UE supports NR SL in unlicensed spectrum where shared spectrum channel access must be used, UE must indicate this FG is supported] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-k4 | Transmitting UE to UE COT sharing information | 1. UE supports using ue-toUE-COT-SharingED-Threshold for Type 1 channel access for UE to UE COT sharing 2. UE supports indicating COT sharing information in SCI | 47-k1 | No | [No] | UE does not support transmitting UE-to-UE COT sharing information for sharing COT for NR sidelink operation in shared spectrum. | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |

The columns with yellow highlights for FG 47-k3 can be updated as below:

- Behaviours for FG 47-k1 and FG 47-k3 are different. FG 47-k1 includes channel access via RSSI measurement and transmission based on the result of channel access, and, FG 47-k3 includes COT-SI monitoring and transmission based on the COT-SI.  Thus, it is not necessary to merge with FG 47-k1 and the FFS should be removed.

- Since the COT sharing operation is performed by UE, there is no need to report the FG to gNB.

- COT sharing is not limited to unicast and FG 47-k3 is a basic FG (see below), so it should not depend on exchange between UEs. Exchange between UEs should be “No”.

- Since it is unnecessary to report to gNB and exchange between UEs, FG 47-k3 is optional without capability signalling.

- There is no agreement to limit COT sharing to unicast only, so that receiving COT sharing information should be a basic FG. Otherwise, Tx UE is uncertain whether Rx UE is able to decode COT sharing information and may only send COT sharing information after capability exchange via PC5 RRC, which is only possible in unicast.

The columns with yellow highlights for FG 47-k4 can be updated as below:

- Similar to the analysis of FG 47-k3, other highlighted parts are reasonable and can be confirmed.

FG 47-k5 Multi-consecutive slots transmission

In the UE features list after RAN1#115, FG 47-k5 is as follows.

| 47. NR_SL_enh2 | 47-k5 | Resource allocation for multi-consecutive slots transmission | UE supports resource (re-)selection for PSCCH/PSSCH transmission on multiple consecutive slots | TBD | [Yes] | [No] | UE does not support resource (re-)selection for multi-consecutive slots transmission | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The columns with yellow highlights can be updated as below:

- Considering that the FG is applicable to SL mode2 in FR1 unlicensed spectrum, the prerequisites are 47-k1 and 15-3. And a Note can be added: If UE supports 15-3, the UE is not required to support Component 3 in 15-3, and FR2 parts of Component 7 in 15-3.

- FG 47-k5 is a capability of resource (re-)selection performed by Tx UE itself. There is no need to report to gNB.

- Since there is no difference in behaviours of Rx UE between MCS transmission or single slot transmission, there is no need to exchange between UEs.

- Per band is preferred. Per UE is not enough flexible, and finer granularity than per band is unnecessary. Because concurrent transmission/reception on multiple bands, e.g. inter-band CA, is not included in the scope in Rel-18 SL-U.

- Since SL-U is limited to n46 and n96/n102, the need of FDD/TDD differentiation and FR1/FR2 differentiation is N/A.

- Since it is unnecessary to report to gNB and exchange between UEs, FG 47-k5 is optional without capability signalling.

- Other highlighted parts are reasonable.

FG 47-k8 CW autonomous update for SL transmission without HARQ feedback

In the UE features list after RAN1#115, FG 47-k8 is as follows.

| 47. NR_SL_enh2 | 47-k8 | CW autonomous update for SL transmission without HARQ feedback | UE support autonomous update of the CWp to the next higher allowed value when the same CWp≠CWmax,p value is consecutively used for X times for generation of Ninit for PSCCH/PSSCH transmission without HARQ feedback. FFS whether to merge with 47-k1 | TBD | No | No | UE does not update CWp for PSCCH/PSSCH transmission without HARQ feedback. | Per band | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

We prefer to keep FG 47-k8 and the columns with yellow highlights can be updated as below:

- The FFS in components should be removed, because FG 47-k8 is not a basic feature.

- The prerequisites include 47-k1 and 15-11.

- Other highlighted parts are reasonable.

FG 47-k9 Sidelink mode 1 resource allocation in shared spectrum

In the UE features list after RAN1#115, FG 47-k9 is as follows.

| 47. NR_SL_enh2 | 47-k9 | Sidelink mode 1 resource allocation in shared spectrum | 1. UE can monitor DCI format 3_0 on a licensed band for NR sidelink dynamic scheduling and configured grant type 2 for transmitting interlaced RB-based PSCCH/PSSCH on a shared band] 2. UE supports reporting NACK to gNB when transmitting PSCCH/PSSCH on scheduled resource(s) is failed due to LBT failure | TBD | Yes | No | UE does not perform interlaced RB-based PSCCH/PSSCH based on mode 1 resource allocation. | Per band | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The columns with yellow highlights can be updated as below:

- The prerequisites include 47-k1 and 15-25 except Component 3 and 4 in 15-2. The components which should be excepted can be reflected in the notes.

- Other highlighted parts are reasonable.

- SL-U PHY channel design framework

FG 47-m1 Interlace RB-based PSCCH/PSSCH/PSFCH transmission/reception

In the UE features list after RAN1#115, FG 47-m1 is as follows.

| 47. NR_SL_enh2 | 47-m1 | Interlace RB-based SL transmission/reception | 1. UE supports interlace RB-based SL transmissions for the physical layer channels that it is capable of transmit 2. UE supports interlace RB-based SL receptions for the physical layer channels that it is capable of receive | 47-k1, at least one of [15-25 except Component 3 and 4 in 15-2] and [15-3 except Component 3] | Yes | No | UE does not support Interlace RB-based PSCCH/PSSCH/PSFCH transmission/reception | Per band | N/A | N/A |  | This is the basic FG for NR sidelink in shared spectrum, where PSD and/or OCB requirements are defined by regulation. | Optional with capability signalling For UE supports NR sidelink in shared spectrum, where PSD and/or OCB requirements are defined by regulation, UE must indicate this FG is supported. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The current prerequisites are reasonable and we suggest to remove the bracket. The components which should be excepted can be reflected in the notes.

FG 47-m3 and FG 47-m4 Transmitting/Receiving PSCCH/PSSCH from 2nd starting symbol in a slot

In the UE features list after RAN1#115, FG 47-m3 and 47-m4 are as follows.

| 47. NR_SL_enh2 | 47-m3 | Transmitting PSCCH/PSSCH from 2nd starting symbol in a slot | 1. UE supports transmitting PSCCH/PSSCH from 2nd starting symbol in a slot in addition to the first starting symbol | 47-k1, at least one of [15-25 except Component 3 and 4 in 15-2] and [15-3 except Component 3] | No | No | UE transmits PSCCH/PSSCH only from 1st starting symbol in a slot | Per band | N/A | N/A |  |  | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-m4 | Receiving PSCCH/PSSCH from 2nd starting symbol in a slot | 1. UE supports receiving PSCCH/PSSCH transmitted from 2nd starting symbol in a slot in addition to the first starting symbol 2. UE can monitor a total up to X PSCCHs in a slot in the 1st and 2nd starting symbols | [15-1 except Component 5] | No | No | UE receives PSCCH/PSSCH transmitted only from 1st starting symbol in a slot | Per band | N/A | N/A |  | [This is the basic FG for NR sidelink in unlicensed spectrum.] The value X is the same as the reported value in FG 15-1 | [Optional without capability signalling] [For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported.] |

The columns with yellow highlights for FG 47-m3 can be updated as below:

- The current prerequisites are reasonable and we suggest to remove the bracket. The components which should be excepted can be reflected in the notes.

- Per band is sufficient. Because concurrent transmission/reception on multiple bands, e.g. inter-band CA, is not included in the scope in Rel-18 SL-U.

- Since SL-U is limited to n46 and n96/n102, the need of FDD/TDD differentiation and FR1/FR2 differentiation is N/A.

The columns with yellow highlights for FG 47-m4 can be updated as below:

- The current prerequisites are reasonable and we suggest to remove the bracket. The components which should be excepted can be reflected in the notes.

- If receiving PSCCH/PSSCH from 2nd starting symbol in a slot is not a basic FG, Tx UE is uncertain whether Rx UE is able to receive from 2nd starting symbol. One way is that UE may transmit from 2nd starting symbol after capability exchange via PC5 RRC, which is only possible in unicast. However, there is no agreement to limit 2 starting symbol to unicast only. Thus, FG 47-m4 is basic and no need to exchange between UEs.

- Per band is sufficient. Because concurrent transmission/reception on multiple bands, e.g. inter-band CA, is not included in the scope in Rel-18 SL-U.

- Since it is unnecessary to report to gNB and exchange between UEs, FG 47-m4 is optional without capability signalling.

- Similar to the analysis of FG 47-k3, other highlighted parts are reasonable and can be confirmed.

FG 47-m5 Multiple PSFCH occasions per PSCCH/PSSCH

In the UE features list after RAN1#115, FG 47-m5 is as follows.

| 47. NR_SL_enh2 | 47-m5 | Multiple PSFCH occasions per PSCCH/PSSCH | 1. UE supports PSFCH transmission/reception on N PSFCH occasion(s) per PSCCH/PSSCH | 15-11, 47-k1 | Yes | No | UE supports only one PSFCH occasion per PSCCH/PSSCH transmission | Per band | N/A | N/A |  | Candidate values for N are {1,2,3,4} The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The columns with yellow highlights can be updated as below:

- The current prerequisites are reasonable and we suggest to remove the bracket.

- Per band is sufficient. Because concurrent transmission/reception on multiple bands, e.g. inter-band CA, is not included in the scope in Rel-18 SL-U.

- Since SL-U is limited to n46 and n96/n102, the need of FDD/TDD differentiation and FR1/FR2 differentiation is N/A.

FG 47-m6 Transmitting SSB repetitions within one RB set

FG 47-m6 from the UE features list after RAN1#115 is as follows.

| 47. NR_SL_enh2 | 47-m6 | Transmitting SSB repetitions within one RB set | 1. UE supports transmitting S-PSS/S-SSS/PSBCH multiple times by repetition in frequency domain within one RB set FFS whether to define the supported maximum number of repetitions in frequency domain within one RB set FFS whether/how to introduce Rx capability | 15-4, 47-k1 | No | No | UE does not support transmitting S-PSS/S-SSS/PSBCH multiple times by repetition in frequency domain within one RB set | Per band | N/A | N/A |  | This is the basic FG for NR sidelink in shared spectrum where PSD and/or OCB requirements are defined by regulation. [It is up to UE implementation whether S-SSB RX UE monitors more than one S-SSB repetition in frequency domain within one RB set as long as RAN4 requirements are satisfied] [The signaling is only expected for a band where shared spectrum channel access must be used.] | [Optional without capability signalling] For UE supports NR sidelink in unlicensed spectrum where PSD and/or OCB requirements are defined by regulation, UE must support this FG. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The columns with yellow highlights can be updated as below:

- For a UE supporting S-SSB repetition within one RB set, it is not necessary to limit or request the number of maximum repetitions once OCB requirement is satisfied. Thus, the first FFS in the components should be removed.

- FG 15-4 component 1) mentioned S-SSB Rx behaviour (copied below), this is mainly because if UE wants to perform SL Tx/Rx, it must support receiving S-SSB at the frequency position indicated by ARFCN for synchronization. For R18 SL-U, transmitting SSB repetitions within one RB set is to satisfy OCB requirement. Rx UE can choose only to receive one S-SSB repetition as indicated by ARFCN. So there is no need to introduce Rx capability for this feature.

- The current prerequisites are reasonable and we suggest to remove the bracket.

- gNB can know whether a UE supports operation in unlicensed spectrum via the report of 47-k1 which is the prerequisite of 47-m6, it is unnecessary to report 47-m6 to gNB.

- Per band is preferred. Per UE is not enough flexible, and finer granularity than per band is unnecessary. Because concurrent transmission of S-SSB on multiple bands, e.g. inter-band CA, is not included in the scope in Rel-18 SL-U.

- Since SL-U is limited to n46 and n96/n102, the need of FDD/TDD differentiation and FR1/FR2 differentiation is N/A.

- According to the analysis of whether/how to introduce Rx capability and regulatory requirements, it is acceptable to remove the brackets of the 2 notes.

- Since it is unnecessary to report to gNB and exchange between UEs, FG 47-m6 is optional without capability signalling.

(copied from FG 15-4 Synchronization sources for NR sidelink) 1) UE can receive S-SSB in NR sidelink if it supports 15-1. …

FG 47-m8/m9 Transmitting/Receiving SSB on additional S-SSB occasion(s)

FG 47-m8/m9 from the UE features list after RAN1#115 is as follows.

| 47. NR_SL_enh2 | 47-m8 | Transmitting S-SSB on additional S-SSB occasion(s) | 1. UE supports transmitting S-SSB on additional S-SSB occasion(s) | 15-4, 47-k1 | No | No | UE does not support transmitting S-SSB on additional S-SSB occasion(s) but supports to exclude those occasion(s) from SL resource pool(s) | Per band | N/A | N/A |  |  | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-m9 | Receiving S-SSB on additional S-SSB occasion(s) | 1. UE supports receiving S-SSB on additional S-SSB occasion(s) |  | No | No | UE does not support receiving S-SSB on additional S-SSB occasion(s) but supports to exclude those occasion(s) from SL resource pool(s) | Per band | N/A | N/A |  |  | Optional without capability signalling |

For FG 47-m8, the columns with yellow highlights can be updated as below:

- The current prerequisites are reasonable and we suggest to remove the bracket.

For FG 47-m9, the columns with yellow highlights can be updated as below:

- The prerequisite includes FG 15-4.

- Per band is sufficient.

FG 47-m10 Contiguous RB-based PSCCH/PSSCH transmission

In RAN1#115, FG 47-m10 from Moderator are as follows.

| 47. NR_SL_enh2 | 47-m10 | Contiguous RB-based PSCCH/PSSCH transmission | 1. UE supports contiguous RB-based PSCCH/PSSCH transmission 2. UE supports resource (re-)selection for contiguous RB-based PSCCH/PSSCH transmission FFS whether/how to define the capabilities for reception and/or PSFCH | TBD | [Yes] | [No] | UE does not support contiguous RB-based PSCCH/PSSCH transmission | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The columns with yellow highlights can be updated as below:

- Only new Tx behaviour is introduced, and the Rx behaviour is same as legacy NR SL. No need to define Rx capability.

- The UE behaviour for contiguous RB-based PSFCH in SL-U are the same to that in Rel-16/Rel-17. Thus, no need to define capability of PSFCH.

- Considering that the FG is limited in unlicensed spectrum and applicable to SL mode1 and mode2, the prerequisites are 47-k1, at least one of {15-3 except Component 3,15-25 except Component 3 and 4 in 15-2}. The components which should be excepted can be reflected in the notes.

- It is necessary to report to gNB because gNB need know the capability to determine how to allocate resources in SL mode1.

- According to the analysis above, the Rx behaviour is same as legacy NR SL and no need to exchange between UEs.

- Per band is preferred. Per UE is not enough flexible, and finer granularity than per band is unnecessary. Because concurrent transmission with contiguous PSCCH/PSSCH/PSFCH waveform on multiple bands, e.g. inter-band CA, is not included in the scope in Rel-18 SL-U.

- Since SL-U is limited to n46 and n96/n102, the need of FDD/TDD differentiation and FR1/FR2 differentiation is N/A.

- Other highlighted parts are reasonable.

Agreement Regarding “For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, support only option 3” and “Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission”: - Candidate resource, whose lowest sub-channel includes intra-cell guardband PRBs, is excluded - Such exclusion is performed in PHY layer, and such candidate resource is excluded in Step 1

FG 47-m11/m11a/m12/m12a Support of PSFCH/S-SSB transmission over contiguous and/or non-contiguous RB sets

In RAN1#115, FG 47-m11/m11a/m12/m12a from Moderator are as follows.

| 47. NR_SL_enh2 | 47-m11 | PSFCH transmissions in multiple contiguous RB sets | UE supports PSFCH transmissions in multiple contiguous RB sets | TBD | [No] | [No] | UE does not support PSFCH transmissions in multiple contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-m11a | PSFCH transmissions in multiple non-contiguous RB sets | UE supports PSFCH transmissions in multiple non-contiguous RB sets | TBD | [No] | [No] | UE does not support PSFCH transmissions in multiple non-contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m12 | S-SSB transmissions in multiple contiguous RB sets | UE supports S-SSB transmissions in multiple contiguous RB sets | TBD | [No] | [No] | UE does not support S-SSB transmissions in multiple contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m12a | S-SSB transmissions in multiple non-contiguous RB sets | UE supports S-SSB transmissions in multiple non-contiguous RB sets | TBD | [No] | [No] | UE does not support S-SSB transmissions in multiple non-contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |

UE can transmit PSFCH/S-SSB over multiple RB sets in 2 cases including initiating COT to transmit via multiple channel access procedure (FG 47-k2) and transmitting in shared COT via single channel access procedure in each RB set (FG 47-k1).

- FG 47-m11: For PSFCH, both cases are supported. Thus, the prerequisites include at least one of {47-k1, 47-k2}, 15-11.

- FG 47-m11a: It is not reasonable that a UE supports only PSFCH transmission over non-contiguous RB sets but not PSFCH transmission over contiguous RB sets. FG for transmission over contiguous RB sets should be pre-requisite of FG for transmission over non-contiguous RB sets. Thus, the prerequisite of FG 47-m12 includes FG 47-m11.

- FG 47-m12: For S-SSB, multiple channel access is not supported. Thus, the prerequisites include 47-k1, 15-4.

- FG 47-m12a: Similar as the analysis of FG 47-m11a, the prerequisite of FG 47-m12a includes FG 47-m12.

The columns with yellow highlights for FG 47-m11/m11a/m12/m12a can be updated as below:

- The behaviours are performed by UE itself and has no impact on Rx behaviour, the need of report to gNB and exchange between UEs are No.

- Similar to the analysis above, per band is preferred.

- Since SL-U is limited to n46 and n96/n102, the need of FDD/TDD differentiation and FR1/FR2 differentiation is N/A.

- Since it is unnecessary to report to gNB and exchange between UEs, these FGs are optional without capability signalling.

- Other highlighted parts are reasonable.

In addition, if SL-BWP includes multiple RB sets, FG 47-m11 should be supported as a basic capability to ensure the performance of PSFCH. There are two approaches to reflect this and either is ok as long as RAN1 has the same understanding.

- Approach1: Add a note in FG 47-m11 that this is the basic FG for NR sidelink in shared spectrum, when SL-BWP includes multiple RB sets.

- Approach2: Do not introduce FG 47-m11, which means that PSFCH transmissions in multiple contiguous RB sets is the basic FG for NR sidelink in shared spectrum.

FG 47-m13 Transmissions/receptions of multiple dedicated PRBs in interlace-based PSFCH

In RAN1#115, FG 47-m13 from Moderator are as follows.

| 47. NR_SL_enh2 | 47-m13 | Transmissions/receptions of multiple dedicated PRBs in interlace-based PSFCH | 1. UE can transmit PSFCH(s) on up to a total of K dedicated PRBs in a slot. 2. UE can receive PSFCH(s) on up to a total of L dedicated PRBs in a slot | TBD | No | No | UE does not support multiple transmissions/receptions of common interlace-based PSFCH. | Per band | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. Candidate values for K are FFS Candidate values for L are FFS | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The columns with yellow highlights can be updated as below:

- The prerequisite includes FG 47-k1.

- Per band is appropriate.

- The total number of PSFCH that UE can transmit/receive means the number of channels rather than the number of PRBs. For example, if a UE can transmit M PSFCH, it can transmit M PSFCH regardless of how many RBs each PSFCH occupies. Thus,

  - Candidate values for K are M*K3, where M is the same for each carrier and is reported by FG 15-11 component 3, and K3 is the number of dedicated PRBs of each PSFCH.

  - Candidate values for L are N*K3, where N is the same for each carrier and is reported by FG 15-11 component 2, and K3 is the number of dedicated PRBs of each PSFCH.

- Other highlighted parts are reasonable.

- Co-channel coexistence for LTE sidelink and NR sidelink

FG 47-s1 Transmission using dynamic resource pool sharing

In the UE features list after RAN1#114bis, FG 47-s1 is as follows.

| 47. NR_SL_enh2 | 47-s1 | Transmission/Reception using dynamic resource pool sharing | 1) Avoidance of NR PSCCH/PSSCH/PSFCH overlapping with EUTRA SL resources in dynamic resource pool sharing using LTE sidelink resource reservation information in NR mode2 resource (re)selection 2) UE supports NR sidelink TXs and RXs in a resource pool in 15kHz and 30kHz SCSs and uses the SCS that is (pre)configured for a SL BWP. | [15-3, 15-6, 15-11] | Yes | No | UE does not support transmission/reception using dynamic resource pool sharing | Per band | N/A | N/A |  | Component 2 does not imply that two different SCSs can be (pre)configured simultaneously in a SL BWP | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The columns with yellow highlights can be updated in accordance to the following:

- Regarding the prerequisite FGs, the brackets should be removed since dynamic resource pool sharing operates in Mode 2 (15-3), is based on Rel-16’s in-device coexistence (15-6) and supports PSFCH (15-11).

- Sidelink carrier aggregation operation

In the UE features list after RAN1#115, FG 47-v1/v2/v3 is as follows.

| 47. NR_SL_enh2 | 47-v1 | NR SL communication with SL CA | 1) UE supports transmitting/receiving PSCCH/PSSCH/PSFCH simultaneously over multiple X SL carriers: - 1-1) Maximum number of simultaneous PSCCH/PSSCH TX, equal to X and 1 per carrier - 1-2) For the number of PSCCH decodes: - UE can receive Z* floor (NRB,i /10 RBs) PSCCH in a slot on carrier i of the X carriers. - FFS whether to report Maximum number of non-overlapping RBs per slot across all carriers the UE can attempt to decode 2) UE can adjust the transmission power of the PSCCH/PSSCH/PSFCH across aggregated carriers such that its total transmission power does not exceed the maximum transmission power. FFS whether/how to merge FG for SL-CA | [15-3, 15-11] | Yes | No |  | Per band | N/A | N/A |  | Component 1: Candidate value of X = {2, 3, 4, 5, 6, 7, 8}, FFS on some BW restriction Component 1-2 candidate value set: Z={1, 2} NRB,i is the number of RBs defined per channel bandwidth of carrier i by RAN4 in 38.101-1 Table 5.3.2-1 for FR1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-v2 | Synchronization for SL CA | 1-1) UE supports transmitting S-SSB on one selected or all candidate synchronization carriers with the same sync reference from Set-B 1-2) UE supports receiving S-SSB from all candidate synchronization carriers with the same sync reference from Set-B 2) UE can adjust the transmission power of the S-SSB across aggregated carriers such that its total transmission power does not exceed the maximum transmission power. FFS whether/how to merge FG for SL-CA | 47-v1, [15-4] | Yes | No |  | Per band | N/A | N/A |  | Note: Option of UE selection of one selected SL synchronization carrier with the same sync reference from Set-B is not based on limited Tx capability Note: Component 1-2 does not require simultaneous reception of S-SSB on all candidate synchronization carriers with the same sync reference from Set-B | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-v3 | PSFCH for SL CA | 1) UE supports receiving X PSFCH resources in a slot over all aggregated SL carriers - 1-1) UE is capable of receiving at least one PSFCH resource on each of the aggregated carriers in a slot 2) UE supports transmitting Y PSFCH resources in a slot over all aggregated SL carriers according to PSFCH procedures - 2-1) UE is capable of transmitting at least one PSFCH resource on each of the aggregated carriers [3) UE can adjust the transmission number of PSFCH across aggregated carriers such that its total transmission number does not exceed the maximum transmission power.] FFS whether/how to merge FG for SL-CA | 47-v1, [15-11] | Yes | No |  | Per band | N/A | N/A |  | Candidate values for X are {FFS} Candidate values for Y are {FFS} Note: for component 1-1, it is up to UE implementation which PSFCH(s) to receive | Optional with capability signalling |

For the current structure of FG 47-v1/v2/v3, if a UE supports SL CA, all of the three FGs should be basic. From this point of view, separate FGs seem unnecessary. If the FGs are merged, there is no need to define it as a basic FG. If a UE supports SL CA, the FG should be supported. Thus, we propose to merge FG 47-v1/v2/v3 into one FG.

The columns with yellow highlights for the merged FG can be updated as follows:

- Since the feature is limited to Mode 2 and FR1, the prerequisites include 15-1 except Component 5, 15-3 except Component 3, 15-4, 15-11. If FG 15-1 is not added, it is unclear how to express the need to exclude its component 5, while the exclusion of component 3 in 15-3 is stated.

Part of R18 SL CA Scope - This feature is backwards compatible in the following regards - A Rel-16/Rel-17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH and transmits the corresponding sidelink HARQ feedback (when SL-HARQ is enabled in SCI)

For current FG 47-v1

- It is not necessary to report the maximum number of non-overlapping RBs per slot across all carriers the UE can attempt to decode. The number of non-overlapping RBs per slot that UE can attempt to decode is determined per SL carrier as with FG 15-1, but it is not reported, as per the same behaviour with FG 15-1. It may be useful to clarify with a new component how to interpret the FG 15-1 signalling:

  - “UE can attempt to decode Y=NRB non-overlapping RBs in a slot on carrier i of the X carriers, where Y is the value according to component 3 of FG 15-1”.

- The feature is backwards compatible, limited to packet duplication and not limited to unicast (copied below). A Rel-16/17 UE can receive Rel-18 sidelink broadcast/groupcast transmissions with CA for the carrier on which it receives PSCCH/PSSCH, and a Rel-18 UE can receive unicast/broadcast/groupcast transmissions in one or more CCs. It can work without needing to ensure Tx/Rx UEs have the same capability. Thus, exchange between UEs seems to be unnecessary.

- Modulation and demodulation are one of the main limiting factors of UE capabilities. For a UE, considering the cost and complexity of implementation, the capabilities of modulation and encoding in Tx, and the number of PSCCH decodes in a slot and the number of non-overlapping RBs in Rx are both limited. Since the capabilities of modulation and encoding are strongly correlated with bandwidths, and the number of PSCCH decodes and the number of non-overlapping RBs is also derived from bandwidths, bandwidth restriction including different candidate values needs to be introduced for UEs. The restriction is applicable to both transmission and reception for CA. A component can be introduced to report B, which is the maximum total bandwidth that the UE can aggregate. The set of candidate values for B include at least 20MHz, 40MHz and 70MHz, and FFS other values.

For current FG 47-v3

- In the current components, X and Y are the total number of PSFCHs in all of the aggregated carriers, where the UE is capable of receiving/transmitting at least one PSFCH on each of the aggregated carriers. In this case, the candidate values of X and Y can be the same as that of N and M from FG 15-11, which are {5, 15, 25, 32, 35, 45, 50, 64} and {4, 8, 16} respectively, where the selected value of X and Y should be greater than or equal to N and M. This would mean that the UE is capable of transmitting/receiving at least the same number of PSFCHs across multiple carriers as it can on a single carrier, in a given slot, but might be capable of more. We are also open to considering a higher value range of X and Y. If the UE supports SL CA, the values of X and Y signalled in 47-v3 would essentially override the values of N and M in 15-11, since in 15-11, the values of N and M are defined in a single carrier only. Hence it does not seem necessary to capture explicitly that X≥N, Y≥M, but this could also be done if preferred.

- Component 3 is the same as the power control mechanism for PSFCH in single carrier (FG 15-11). According to the agreement (copied below), Rel-16/17 PSFCH power control is performed across carriers for all PSFCH transmissions over all the aggregated SL carriers at the same time. Thus, Component 3 is reasonable in principle. However, because the prerequisites of FG 47-v3 include FG 15-11, we prefer to remove Component 3.

Agreement Rel-16/17 PSFCH power control and PSFCH TX/TX prioritization rule are performed across carriers for all PSFCH transmissions over all the aggregated SL carriers at the same time. - The UE does not expect to be provided with a (pre)configuration that would result in different transmit power per PSFCH on different carriers.

The complete FG list is given in Appendix 1.

- Summary

Based on above analysis, full list of R18 NR SL UE feature is given in Appendix 1.

Proposal 1: Support UE feature list in Appendix 1 for R18 NR SL.

- Conclusions

In this contribution, we have following proposals on UE features for Rel-18 NR sidelink.

Proposal 1: Support UE feature list in Appendix 1 for R18 NR SL.

- References

- R1-2312572, “Updated RAN1 UE features list for Rel-18 NR after RAN1#115 meeting”, NTT DOCOMO, INC., Nov. 2023.

- R1-2312583, “Summary on UE features for NR sidelink evolution”, NTT DOCOMO, INC., Nov. 2023.

# Appendix 1: UE feature list for R18 NR SL

47. NR_SL_enh2

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-k1 | SL channel access for dynamic channel access mode | UE supports 1. SL Type 1 channel access and contention window size adjustment 2. SL Type 2A channel access 3. SL Type 2B channel access 4. SL Type 2C channel access 5. 20MHz LBT bandwidth 6. CP extension up to 1 symbol in 15kHz SCS if the UE supports 15 kHz SCS 7. CP extension up to 2 symbols in 30kHz SCS 8. CP extension up to 2 symbols if the UE supports 60kHz SCS [9. Monitoring SCI to decode] FFS whether to merge with 47-k3 | At least one of {15-25, 15-3, [32-4, 32-4a]} | Yes | No | UE does not support channel access for NR sidelink operation in shared spectrum. | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. Note: Component 8 is applicable in regions without OCB requirements. Note1: If UE supports 15-25, the UE is not required to support Component 3 and 4 in 15-2. Note2: If UE supports 15-3, the UE is not required to support Component 3 in 15-3, and FR2 parts of Component 7 in 15-3. Note: It is up to RAN2 whether/how to implement the above Notes 1/2 and whether/how to update the prerequisite FGs | Optional with capability signalling For UE supports NR SL in unlicensed spectrum and when shared spectrum channel access must be used, UE must indicate this FG is supported |
| 47. NR_SL_enh2 | 47- k2 | SL multi-channel access for dynamic channel access mode | 1. UE supports multi-channel access procedures for PSCCH/PSSCH/S-SSB/PSFCH transmission(s) in multiple RB sets in a slot 2. UE supports Type A and Type B multi-channel access procedures for PSFCH transmissions in multiple RB sets in a slot 4) UE supports multi-channel access procedure on N channel(s) with 20MHz LBT bandwidth for each channel. Candidate values of N: {2, 3, 4, 5} | 47-k1 | Yes | [No] | UE does not support multi-channel access in dynamic channel access mode for NR sidelink operation in shared spectrum. | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. Note: Support of S-SSB/PSFCH transmission(s) in multiple RB-sets in a slot is according to the support of {47-m11, 47-m11a} and {47-m12, 47-m12a} | Optional with capability signalling |
| 47. NR_SL_enh2 | 47- k3 | Receiving UE to UE COT sharing information | 1. UE supports monitoring SCI to read COT sharing information 2. UE supports transmitting NR SL based on COT sharing information subject to COT sharing conditions FFS whether to merge with 47-k1 | 47-k1 | No | [No] | UE does not support using UE-to-UE COT sharing information contained in SCI for sharing COT for NR sidelink operation in shared spectrum. | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | [Optional without capability signalling] [For UE supports NR SL in unlicensed spectrum where and when shared spectrum channel access must be used, UE must indicate this FG is supported] |
| 47. NR_SL_enh2 | 47-k4 | Transmitting UE to UE COT sharing information | 1. UE supports using ue-toUE-COT-SharingED-Threshold for Type 1 channel access for UE to UE COT sharing 2. UE supports indicating COT sharing information in SCI | 47-k1 | No | [No] | UE does not support transmitting UE-to-UE COT sharing information for sharing COT for NR sidelink operation in shared spectrum. | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-k5 | Resource allocation for multi-consecutive slots transmission | UE supports resource (re-)selection for PSCCH/PSSCH transmission on multiple consecutive slots | TBD47-k1, 15-3 | [Yes]No | [No] | UE does not support resource (re-)selection for multi-consecutive slots transmission | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. Note: If UE supports 15-3, the UE is not required to support Component 3 in 15-3, and FR2 parts of Component 7 in 15-3. Note: It is up to RAN2 whether/how to implement the above Note and whether/how to update the prerequisite FGs | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-k6 | Type1 LBT blocking Option 1 | UE supports 1. avoid selection of N consecutive resource(s) before a reserved resource when the L1 SL priority value for the transmission is higher than the L1 SL priority value of the reserved resource. It is up to UE whether to do it 2. avoid selection of M consecutive resource(s) after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource. It is up to UE whether to do it | 47-k1 | Yes | No | UE does not support Type1 LBT blocking Option 1 | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k7 | Type1 LBT blocking Option 2 | UE supports 1. If transmission in slot(s) at least T_proc,0 before a reserved resource is able to share its initiated COT to the reservation, UE prioritize / select resource(s) in the slot(s) for transmission. It is up to UE whether to do it | 47-k1 | Yes | No | UE does not support Type1 LBT blocking Option 2 | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k8 | CW autonomous update for SL transmission without HARQ feedback | UE support autonomous update of the CWp to the next higher allowed value when the same CWp≠CWmax,p value is consecutively used for X times for generation of Ninit for PSCCH/PSSCH transmission without HARQ feedback. FFS whether to merge with 47-k1 | TBD47-k1, 15-11 | No | No | UE does not update CWp for PSCCH/PSSCH transmission without HARQ feedback. | Per band | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k9 | Sidelink mode 1 resource allocation in shared spectrum | 1. UE can monitor DCI format 3_0 on a licensed band for NR sidelink dynamic scheduling and configured grant type 2 for transmitting interlaced RB-based PSCCH/PSSCH on a shared band] 2. UE supports reporting NACK to gNB when transmitting PSCCH/PSSCH on scheduled resource(s) is failed due to LBT failure | TBD47-k1, 15-25 | Yes | No | UE does not perform interlaced RB-based PSCCH/PSSCH based on mode 1 resource allocation. | Per band | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. Note: If UE supports 15-25, the UE is not required to support Component 3 and 4 in 15-2. Note: It is up to RAN2 whether/how to implement the above Note and whether/how to update the prerequisite FGs | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m1 | Interlace RB-based SL transmission/reception | 1. UE supports interlace RB-based SL transmissions for the physical layer channels that it is capable of transmit 2. UE supports interlace RB-based SL receptions for the physical layer channels that it is capable of receive | 47-k1, at least one of [15-25 except Component 3 and 4 in 15-2] and [15-3 except Component 3] | Yes | No | UE does not support Interlace RB-based PSCCH/PSSCH/PSFCH transmission/reception | Per band | N/A | N/A |  | This is the basic FG for NR sidelink in shared spectrum, where PSD and/or OCB requirements are defined by regulation. Note1: If UE supports 15-25, the UE is not required to support Component 3 and 4 in 15-2. Note2: If UE supports 15-3, the UE is not required to support Component 3 in 15-3, and FR2 parts of Component 7 in 15-3. Note: It is up to RAN2 whether/how to implement the above Notes 1/2 and whether/how to update the prerequisite FGs | Optional with capability signalling For UE supports NR sidelink in shared spectrum, where PSD and/or OCB requirements are defined by regulation, UE must indicate this FG is supported. |
| 47. NR_SL_enh2 | 47-m3 | Transmitting PSCCH/PSSCH from 2nd starting symbol in a slot | 1. UE supports transmitting PSCCH/PSSCH from 2nd starting symbol in a slot in addition to the first starting symbol | 47-k1, at least one of [15-25 except Component 3 and 4 in 15-2] and [15-3 except Component 3] | No | No | UE transmits PSCCH/PSSCH only from 1st starting symbol in a slot | Per band | N/A | N/A |  | Note1: If UE supports 15-25, the UE is not required to support Component 3 and 4 in 15-2. Note2: If UE supports 15-3, the UE is not required to support Component 3 in 15-3, and FR2 parts of Component 7 in 15-3. Note: It is up to RAN2 whether/how to implement the above Notes 1/2 and whether/how to update the prerequisite FGs | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-m4 | Receiving PSCCH/PSSCH from 2nd starting symbol in a slot | 1. UE supports receiving PSCCH/PSSCH transmitted from 2nd starting symbol in a slot in addition to the first starting symbol 2. UE can monitor a total up to X PSCCHs in a slot in the 1st and 2nd starting symbols | [15-1 except Component 5] | No | No | UE receives PSCCH/PSSCH transmitted only from 1st starting symbol in a slot | Per band | N/A | N/A |  | [This is the basic FG for NR sidelink in unlicensed spectrum.] The value X is the same as the reported value in FG 15-1 Note: If UE supports 15-1, the UE is not required to support Component 5. Note: It is up to RAN2 whether/how to implement the above Note and whether/how to update the prerequisite FGs | [Optional without capability signalling] [For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported.] |
| 47. NR_SL_enh2 | 47-m5 | Multiple PSFCH occasions per PSCCH/PSSCH | 1. UE supports PSFCH transmission/reception on N PSFCH occasion(s) per PSCCH/PSSCH | 15-11, 47-k1 | Yes | No | UE supports only one PSFCH occasion per PSCCH/PSSCH transmission | Per band | N/A | N/A |  | Candidate values for N are {1,2,3,4} The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m6 | Transmitting SSB repetitions within one RB set | 1. UE supports transmitting S-PSS/S-SSS/PSBCH multiple times by repetition in frequency domain within one RB set FFS whether to define the supported maximum number of repetitions in frequency domain within one RB set FFS whether/how to introduce Rx capability | 15-4, 47-k1 | No | No | UE does not support transmitting S-PSS/S-SSS/PSBCH multiple times by repetition in frequency domain within one RB set | Per band | N/A | N/A |  | This is the basic FG for NR sidelink in shared spectrum where PSD and/or OCB requirements are defined by regulation. [It is up to UE implementation whether S-SSB RX UE monitors more than one S-SSB repetition in frequency domain within one RB set as long as RAN4 requirements are satisfied] [The signaling is only expected for a band where shared spectrum channel access must be used.] | [Optional without capability signalling] For UE supports NR sidelink in unlicensed spectrum where PSD and/or OCB requirements are defined by regulation, UE must support this FG. |
| 47. NR_SL_enh2 | 47-m8 | Transmitting S-SSB on additional S-SSB occasion(s) | 1. UE supports transmitting S-SSB on additional S-SSB occasion(s) | 15-4, 47-k1 | No | No | UE does not support transmitting S-SSB on additional S-SSB occasion(s) but supports to exclude those occasion(s) from SL resource pool(s) | Per band | N/A | N/A |  |  | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-m9 | Receiving S-SSB on additional S-SSB occasion(s) | 1. UE supports receiving S-SSB on additional S-SSB occasion(s) | 15-4 | No | No | UE does not support receiving S-SSB on additional S-SSB occasion(s) but supports to exclude those occasion(s) from SL resource pool(s) | Per band | N/A | N/A |  |  | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-m10 | Contiguous RB-based PSCCH/PSSCH transmission | 1. UE supports contiguous RB-based PSCCH/PSSCH transmission 2. UE supports resource (re-)selection for contiguous RB-based PSCCH/PSSCH transmission FFS whether/how to define the capabilities for reception and/or PSFCH | TBD47-k1, at least one of 15-3 and 15-25 | [Yes] | [No] | UE does not support contiguous RB-based PSCCH/PSSCH transmission | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. Note1: If UE supports 15-25, the UE is not required to support Component 3 and 4 in 15-2. Note2: If UE supports 15-3, the UE is not required to support Component 3 in 15-3, and FR2 parts of Component 7 in 15-3. Note: It is up to RAN2 whether/how to implement the above Notes 1/2 and whether/how to update the prerequisite FGs | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m11 | PSFCH transmissions in multiple contiguous RB sets | UE supports PSFCH transmissions in multiple contiguous RB sets | TBD At least one of {47-k1, 47-k2}, 15-11 | [No] | [No] | UE does not support PSFCH transmissions in multiple contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. This is the basic FG for NR sidelink in shared spectrum, when SL-BWP includes multiple RB sets. | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-m11a | PSFCH transmissions in multiple non-contiguous RB sets | UE supports PSFCH transmissions in multiple non-contiguous RB sets | TBD47-m11 | [No] | [No] | UE does not support PSFCH transmissions in multiple non-contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-m12 | S-SSB transmissions in multiple contiguous RB sets | UE supports S-SSB transmissions in multiple contiguous RB sets | TBD47-k1, 15-4 | [No] | [No] | UE does not support S-SSB transmissions in multiple contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-m12a | S-SSB transmissions in multiple non-contiguous RB sets | UE supports S-SSB transmissions in multiple non-contiguous RB sets | TBD47-m12 | [No] | [No] | UE does not support S-SSB transmissions in multiple non-contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-m13 | Transmissions/receptions of multiple dedicated PRBs in interlace-based PSFCH | 1. UE can transmit PSFCH(s) on up to a total of K dedicated PRBs in a slot. 2. UE can receive PSFCH(s) on up to a total of L dedicated PRBs in a slot | TBD47-k1 | No | No | UE does not support multiple transmissions/receptions of common interlace-based PSFCH. | Per band | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. Candidate values for K are FFS M*K3, where M is the same for each carrier and is reported by FG 15-11 component 3, and K3 is the number of dedicated PRBs of each PSFCH. Candidate values for L are FFS N*K3, where N is the same for each carrier and is reported by FG 15-11 component 2, and K3 is the number of dedicated PRBs of each PSFCH. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-s1 | Transmission/Reception using dynamic resource pool sharing | 1) Avoidance of NR PSCCH/PSSCH/PSFCH overlapping with EUTRA SL resources in dynamic resource pool sharing using LTE sidelink resource reservation information in NR mode2 resource (re)selection 2) UE supports NR sidelink TXs and RXs in a resource pool in 15kHz and 30kHz SCSs and uses the SCS that is (pre)configured for a SL BWP. | [15-3, 15-6, 15-11] | Yes | No | UE does not support transmission/reception using dynamic resource pool sharing | Per band | N/A | N/A |  | Component 2 does not imply that two different SCSs can be (pre)configured simultaneously in a SL BWP | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-v1 | NR SL communication with SL CA | 1) UE supports transmitting/receiving PSCCH/PSSCH/PSFCH simultaneously over multiple X SL carriers: - 1-1) Maximum number of simultaneous PSCCH/PSSCH TX, equal to X and 1 per carrier - 1-2) For the number of PSCCH decodes: - UE can receive Z* floor (NRB,i /10 RBs) PSCCH in a slot on carrier i of the X carriers. - FFS whether to report Maximum number of non-overlapping RBs per slot across all carriers the UE can attempt to decode - 1-2) For the Maximum number of non-overlapping RBs per slot across all carriers the UE can attempt to decode: - UE can attempt to decode Y=NRB non-overlapping RBs in a slot on carrier i of the X carriers, where Y is the value according to component 3 of FG 15-1. 2) UE can aggregate up to total bandwidth B 3) UE can adjust the transmission power of the PSCCH/PSSCH/PSFCH/S-SSB across aggregated carriers such that its total transmission power does not exceed the maximum transmission power. FFS whether/how to merge FG for SL-CA 4-1) UE supports transmitting S-SSB on one selected or all candidate synchronization carriers with the same sync reference from Set-B 4-2) UE supports receiving S-SSB from all candidate synchronization carriers with the same sync reference from Set-B 5) UE supports receiving X1 PSFCH resources in a slot over all aggregated SL carriers - 5-1) UE is capable of receiving at least one PSFCH resource on each of the aggregated carriers in a slot 6) UE supports transmitting Y1 PSFCH resources in a slot over all aggregated SL carriers according to PSFCH procedures - 6-1) UE is capable of transmitting at least one PSFCH resource on each of the aggregated carriers | [15-3, 15-11, 15-4] | Yes | No |  | Per band | N/A | N/A |  | Component 1: Candidate value of X = {2, 3, 4, 5, 6, 7, 8}, FFS on some BW restriction Component 1-2 candidate value set: Z = {1, 2} Component 2 candidate value set: B = {20Mhz, 40MHz, 70MHz, FFS other values} NRB,i is the number of RBs defined per channel bandwidth of carrier i by RAN4 in 38.101-1 Table 5.3.2-1 for FR1 Note: Option of UE selection of one selected SL synchronization carrier with the same sync reference from Set-B is not based on limited Tx capability Note: Component 4-2 does not require simultaneous reception of S-SSB on all candidate synchronization carriers with the same sync reference from Set-B Candidate values for X1 are {5, 15, 25, 32, 35, 45, 50, 64} Candidate values for Y1 are {4, 8, 16} Note: for component 5-1, it is up to UE implementation which PSFCH(s) to receive Note1: If UE supports 15-3, the UE is not required to support Component 3 in 15-3, and FR2 parts of Component 7 in 15-3. Note2: It is up to RAN2 whether/how to implement the above Note 1 and whether/how to update the prerequisite FGs Note 3: For components 5 and 6, the values of X and Y override the values of N and M, respectively, signaled in 15-11. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-v2 | Synchronization for SL CA | 1-1) UE supports transmitting S-SSB on one selected or all candidate synchronization carriers with the same sync reference from Set-B 1-2) UE supports receiving S-SSB from all candidate synchronization carriers with the same sync reference from Set-B 2) UE can adjust the transmission power of the S-SSB across aggregated carriers such that its total transmission power does not exceed the maximum transmission power. FFS whether/how to merge FG for SL-CA | 47-v1, [15-4] | Yes | No |  | Per band | N/A | N/A |  | Note: Option of UE selection of one selected SL synchronization carrier with the same sync reference from Set-B is not based on limited Tx capability Note: Component 1-2 does not require simultaneous reception of S-SSB on all candidate synchronization carriers with the same sync reference from Set-B | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-v3 | PSFCH for SL CA | 1) UE supports receiving X PSFCH resources in a slot over all aggregated SL carriers - 1-1) UE is capable of receiving at least one PSFCH resource on each of the aggregated carriers in a slot 2) UE supports transmitting Y PSFCH resources in a slot over all aggregated SL carriers according to PSFCH procedures - 2-1) UE is capable of transmitting at least one PSFCH resource on each of the aggregated carriers [3) UE can adjust the transmission number of PSFCH across aggregated carriers such that its total transmission number does not exceed the maximum transmission power.] FFS whether/how to merge FG for SL-CA | 47-v1, [15-11] | Yes | No |  | Per band | N/A | N/A |  | Candidate values for X are {FFS} Candidate values for Y are {FFS} Note: for component 1-1, it is up to UE implementation which PSFCH(s) to receive | Optional with capability signalling |