3GPP TSG RAN WG1 Meeting #116                                                                    R1-2400157

Athens, Greece, February 26th - March 1st, 2024

Source:	New H3C

Title:	Discussion on power control parameters for PHR of SCell

Agenda Item:	7.2

Document for:	Discussion and Decision

- Introduction

In the RAN1 #115 meeting, SCell Type 1 virtual PHR report derivation in Rel.17 was discussed. Some summary were made as below [1].

[115-R15/16-NR] Summary on SCell Type 1 virtual PHR report derivation	Moderator (Nokia)

Companies to check the following for final decision in RAN1#116:

- Adopt a clarification to TS 38.213 subclause 7.7.1 either based on the proposal in [5]

| Ref | Proposal |
| --- | --- |
| [5] | If the activated serving cell is an SCell and parameter preambleReceivedTargetPower is not configured for the cell, then the parameter preambleReceivedTargetPower configured for the primary cell is applied, where the parameter refers to the one configured for the non-supplementary uplink carrier if the primary cell is configured with two uplink carriers. |

This contribution continue to discuss about SCell Type 1 virtual PHR report derivation.

- Discussion

In terms of discussion in the RAN1#115  [1],  we agree with intention on  deriving the  for an SCell  by using the parameter preambleReceivedTargetPower configured for the primary cell when preambleRecceivedTargetPower is not provided for that Scell. Regarding 3 text proposals from Nokia, ZTE and Huawei, we prefer Huawei’s version which is recorded in Chairman’s Notes of  RAN1#115 as follows.

Proposal 1: For PHR of SCell, adopt the following TP.

TS 38.213 ### 7.7.1 Type 1 PH report < Unchanged parts are omitted > If the UE determines that a Type 1 power headroom report for an activated serving cell is based on a reference PUSCH transmission then, for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c, the UE computes the Type 1 power headroom report as [dB] where PCMAX,f,c(i) is computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB. TC = 0 dB. MPR, A-MPR, P-MPR and TC are defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2] and [8-3, TS 38.101-3]. The remaining parameters are defined in clause 7.1.1 and, if ul-powerControl is not provided, PO_PUSCH,b,f,c(j) and αb,f,cj are obtained using PO_NOMINAL,PUSCH,f,c0 and p0-PUSCH-AlphaSetId = 0, PLb,f,c(qd) is obtained using pusch-PathlossReferenceRS-Id = 0, and l=0. If ul-powerControl is provided, PO_PUSCH,b,f,c(j), αb,f,cj and l are obtained by p0AlphaSetforPUSCH associated with the indicated TCI-State or TCI-UL-State, PLb,f,c(qd) is obtained by PL-RS associated with the indicated TCI-State or TCI-UL-State. If the activated serving cell is an SCell and parameter preambleReceivedTargetPower is not configured for the cell, then the parameter preambleReceivedTargetPower configured for the primary cell is applied, where the parameter refers to the one configured for the non-supplementary uplink carrier if the primary cell is configured with two uplink carriers < Unchanged parts are omitted >

- Conclusion

In this contribution, we discussed about SCell Type 1 virtual PHR report derivation in Rel-17. The proposals are as follows:

Proposal 1: For PHR of SCell, adopt the following TP.

TS 38.213 ### 7.7.1 Type 1 PH report < Unchanged parts are omitted > If the UE determines that a Type 1 power headroom report for an activated serving cell is based on a reference PUSCH transmission then, for PUSCH transmission occasion i on active UL BWP b of carrier f of serving cell c, the UE computes the Type 1 power headroom report as [dB] where PCMAX,f,c(i) is computed assuming MPR=0 dB, A-MPR=0 dB, P-MPR=0 dB. TC = 0 dB. MPR, A-MPR, P-MPR and TC are defined in [8-1, TS 38.101-1], [8-2, TS 38.101-2] and [8-3, TS 38.101-3]. The remaining parameters are defined in clause 7.1.1 and, if ul-powerControl is not provided, PO_PUSCH,b,f,c(j) and αb,f,cj are obtained using PO_NOMINAL,PUSCH,f,c0 and p0-PUSCH-AlphaSetId = 0, PLb,f,c(qd) is obtained using pusch-PathlossReferenceRS-Id = 0, and l=0. If ul-powerControl is provided, PO_PUSCH,b,f,c(j), αb,f,cj and l are obtained by p0AlphaSetforPUSCH associated with the indicated TCI-State or TCI-UL-State, PLb,f,c(qd) is obtained by PL-RS associated with the indicated TCI-State or TCI-UL-State. If the activated serving cell is an SCell and parameter preambleReceivedTargetPower is not configured for the cell, then the parameter preambleReceivedTargetPower configured for the primary cell is applied, where the parameter refers to the one configured for the non-supplementary uplink carrier if the primary cell is configured with two uplink carriers. < Unchanged parts are omitted >

- Reference

- Chairman’s Notes, RAN1#115, final, November 13th –17th, 2023