3GPP TSG-RAN WG1 Meeting #116	R1-2400172

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.1.3.3

Source:	Ericsson

Title:	Discussion on other aspects of AI/ML

Document for:	Discussion

# Introduction

In RAN#102 meeting, a new WID is approved for Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface [2]. The study objectives with corresponding checkpoints in RAN#105 is copied below. In this contribution, we focus on the RAN1 other aspects excluding the CSI feedback enhancements.

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain). - Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. NOTE: Coordination with SA/SA WGs of the ongoing study/work as it may relate to their required work.

# Discussion on model identification

One of the WI objectives is to study necessity and details of model identification concept and procedure in the context of LCM [RAN2/RAN1]. In our understanding it's RAN1 role to conclude on the necessity of model identification, while RAN2 would investigate feasibility aspects and the necessary signaling and procedures to support model identification.

From the TR:

“For inference for UE-side models, to ensure consistency between training and inference regarding NW-side additional conditions (if identified), the following options can be taken as potential approaches (when feasible and necessary):

-	Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side

-	Model training at NW and transfer to UE, where the model has been trained under the additional condition

-	Information and/or indication on NW-side additional conditions is provided to UE

-	Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality)

-	Other approaches are not precluded

-	Note: 	the possibility that different approaches can achieve the same function is not denied”

During the SI phase, RAN1 has considered different options to ensure consistency between training and inference regarding NW-side additional conditions (if identified), and model identification can be one of the potential approaches.

Here we analyze this model identification approach for the purpose of ensuring consistency between training and inference regarding NW-side additional conditions.

Model identification associates a model to specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.  The conditions that have implications on the model performance should be identified and specified.

The first foreseen challenge is how to identify those conditions without exposing NW/UE configurations and implementations. Regardless of that, it’s impossible to capture all the conditions (NW sided and UE sided) that has implications on the model performance, which makes it necessary to have a fallback monitoring mechanism where the performance should be monitored to ensuring model applicability.

By identifying the model, the assumption is that the NW can take control over activation/deactivation/switching UE models based on knowledge of which NW additional conditions the model is trained on. But to be able to do that, the NW additional conditions must be provided to the UE during training, and therefore always available to the UE even during inference (e.g. NW broadcast/configures information that allows identification of a scenario/dataset, etc..). If this information is available to the UE, then the decision about the applicability of the model can be done by the UE itself, without the added complexity of model identification. In fact, model selection cannot be based only on the NW sided conditions, but also the UE sided conditions need to be checked. Otherwise, additional information should be reported by the UE to the NW during both model identification and inference so that the NW can ensure that the UE sided conditions are consistent between training and inference.

Information and/or indication on NW-side additional conditions provided to the UE is what ensures consistency between training and inference via model identification. If that is the case, the NW-side additional conditions that is revealed to the UE provides sufficient information to the UE to ensure this consistency on its own without the additional complexity of model identification.

If UE is provided with information and/or indication on NW-side additional conditions, which would be needed for model identification, RAN1 to conclude that model identification is no longer needed in terms of ensuring consistency between training and inference regarding NW-side additional conditions.

Nonetheless, model identification is needed if model transfer from NW to UE is supported. However, the necessity of model transfer should be agreed first before discussing the details for supporting model identification.

RAN1 to conclude that there is no need to discuss model identification in RAN1 for model transfer until the need of such transfer is confirmed.

For two-sided CSI compression use case, whether there will be a need of model identification depends on the model training methods and the conformance testing methods, which are to be continue studied in RAN1 and RAN4 respectively [3]. The closed Rel-18 SI did not conclude on the feasibility of any training collaboration types discussed in RAN1 nor the feasibility of any encoder conformance testing options discussed in RAN4. Hence, we propose to not discuss model identification in this agenda item for two-sided use cases until further progress has been made in RAN1 and RAN4 for the CSI compression use case.

RAN1 to conclude that there is no need to discuss model identification in RAN1 for two-sided use case until further progress has made in RAN1 and RAN4 for the CSI compression use case.

# Discussion on UE-sided model training data

CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

The second bullet refers to the section in the TR where RAN2 provided information on the UE data collection mechanisms. Naturally, RAN2 is tasked with continuing driving the second bullet on the mechanisms and entities responsible for data collection. RAN1 is directed to concentrate on the first sub-bullet, primarily responsible for identifying the contents necessary for data collection across each use case. For each use case, RAN1 should focus on the content of the data collection, it is up to RAN2 to define the procedure for enabling the UE to collect such content. This work was already conducted in the Rel-18 study item, and an LS was sent to RAN2. This should be sufficient to address the first bullet, further information on the content is to be handled via further LS requests from RAN2.

Conclude that the Rel-18 LS to RAN2 is sufficient for addressing the study objective on data content, at least as a starting point for Rel-19.

  - RAN2 can send an LS to RAN1 if there is a need to discuss any additional content, or any further details of the content.

# Discussion on model transfer/delivery

In Rel-18 study item stage, TR 38.843 has captured a set of challenges and benefits with respect to the various options of model transfer/delivery. Next, RAN1 should continue to study the following according to the WID:

Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

In the table below, the difficult aspects are mapped to the cases discussed in RAN1.

Table 1. The difficult aspects of various model delivery/transfer option.

| TR 38.843 | Difficult aspect | Cases with the corresponding difficulty |
| --- | --- | --- |
| For model delivery/transfer to UE (for UE-side models and UE-part of two-sided models): |  |  |
| - Model delivery/transfer to UE, if feasible, may be beneficial to handle scenario/configuration specific (including site-specific configuration/channel conditions) models (i.e., when a single model cannot generalize well to multiple scenarios/configurations/sites), to reduce the device storage requirement. | N/A |  |
| - Model delivery/transfer to UE after offline compiling and/or testing may be friendlier from UE’s implementation point of view compared to the case without offline compiling and/or testing. On the other hand, the case without offline compiling and/or testing (that can update parameter with known model structure), may have benefit at least in terms of shorter model parameter update timescale. | Cases without offline compiling and/or testing are not friendly for UE implementation | Case z3, z4, z5 |
| - Model transfer/delivery of an unknown structure at UE has more challenges related to feasibility (e.g. UE implementation feasibility) compared to delivery/transfer of a known structure at UE. | Unknow model structure | Case z5 |
| - For model trained at network side, Case y (w/ NW-side training) and Case z2 may incur the burden of offline cross-vendor collaboration such as sending a model to the UE-side and/or compiling a model. | Burden of offline cross-vendor collaboration | Case y w/ NW-side training, Case z2 |
| - For model trained at UE side/neutral site, Case z1 and Case z3 may incur the burden of offline cross-vendor collaboration to send the trained model from the UE-side to the network, compared to Case y (w/ UE-side training) which does not have such burden. | Burden of offline cross-vendor collaboration | Case z1, z3 |
| - Model storage at the 3GPP network, compared to storing the model outside the 3GPP network, may come with 3GPP network side burden on model maintenance/storage. | Network side burden of model maintenance/storage | Case z1 – z5 |
| - Proprietary design disclosure concern may arise from model training and/or model storage at the network side compared to other cases (such as case y with UE side training) which does not have such issue. | Proprietary design disclosure concern | Case y w/ NW-side training, Case z1-z5 |

As can be observed, each of cases z1-z5 poses at least 3 difficulties, while case y with NW-side training poses 2 difficulties. This left us with: case y with UE-side or neutral-side training. Thus for Rel-19, RAN groups should prioritize case y with UE-side or neutral-side training, and deprioritize the rest.

Rel-19 RAN groups prioritize case y with UE-side or neutral-side training, and deprioritize the rest.

Case y refers to "model delivery (if needed) over-the-top". This means that there is no specification impact from RAN1 perspective. If model delivery over-the-top is considered necessary, then there may be specification impact to higher layer (e.g., SA2).

Conclude that there is no need for RAN1 to work on any standardised solutions for model transfer/delivery in Rel-19.

Conclude that if model delivery (case y with UE-side or neutral-side training) is considered necessary, the potential specification impact is to higher layer.

# Conclusions

In the previous sections we made the following observations:

Observation 1	Information and/or indication on NW-side additional conditions provided to the UE is what ensures consistency between training and inference via model identification. If that is the case, the NW-side additional conditions that is revealed to the UE provides sufficient information to the UE to ensure this consistency on its own without the additional complexity of model identification.

Based on the discussion in the previous sections we propose the following:

Proposal 1	If UE is provided with information and/or indication on NW-side additional conditions, which would be needed for model identification, RAN1 to conclude that model identification is no longer needed in terms of ensuring consistency between training and inference regarding NW-side additional conditions.

Proposal 2	RAN1 to conclude that there is no need to discuss model identification in RAN1 for model transfer until the need of such transfer is confirmed.

Proposal 3	RAN1 to conclude that there is no need to discuss model identification in RAN1 for two-sided use case until further progress has made in RAN1 and RAN4 for the CSI compression use case.

Proposal 4	Conclude that the Rel-18 LS to RAN2 is sufficient for addressing the study objective on data content, at least as a starting point for Rel-19.

a.	RAN2 can send an LS to RAN1 if there is a need to discuss any additional content, or any further details of the content.

Proposal 5	Rel-19 RAN groups prioritize case y with UE-side or neutral-side training, and deprioritize the rest.

Proposal 6	Conclude that there is no need for RAN1 to work on any standardised solutions for model transfer/delivery in Rel-19.

Proposal 7	Conclude that if model delivery (case y with UE-side or neutral-side training) is considered necessary, the potential specification impact is to higher layer.

# References

3GPP TR 38.843 V2.0.1 (2023-12), Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18).

RP-234039, "New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface," Qualcomm (Moderator), 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11-15, 2023.

R1-2400166, “AI/ML for CSI compression”, 3GPP RAN1 Meeting #116, February 26 – March 1, 2024.