3GPP TSG-RAN WG1 #116			R1-2400175

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	9.7.1

Title:	Power Calibration for Radar Cross Section (RCS) Measurement

Source:	National Institute of Standards and Technology (NIST)

Document for:	Discussion and Information

1. Introduction

A channel parameter that is fundamental to the design of ISAC systems is the target Radar cross section (RCS). In this contribution, we describe a technique to calibrate the power that is measured by the channel sounder according to the practice recommended in [1]. The calibration ensures that RCS value reported is commensurate with the values reported across other measurement campaigns, to ensure a meaningful comparison when necessary. Also we provide details steps to implement the calibration procedure. From [1]

Pr=Pt GtGrλ2σL(4π)3R4

Where

Pt=Transmit powerW, Pr=Receive powerW, Gt=TX antenna gain, Gr=RX antenna gain

λ=Wavelengthmcycle, σ=RCSm2 and L=system loss.

2. Procedure for RCS calibration measurement

Theoretical Pr  is calculated with the known system parameters: power, antenna gains, λ ,

σ=πr2 sphere and L=1.  The receive power is measured.  Note that

Lmeasured=PrPr measured

This is receiver chain bias power offset using the substitution method from [1].  Please note that this is an example for a purely monostatic measurement system.  For deployed systems, the theoretical Pr equation needs to be changed depending on the measurement system geometry.

3. RCS measurement configuration

The measurement system is as described in [2] and [3].  The selection of a sphere and its diameter implies several factors.   The sphere RCS is angle independent.  Since both sphere diameter and array aperture size will dominate far field distance, a small sphere is preferred for reducing the distance between the system and the target. However the larger the sphere, the stronger the path reflected from it, improving the signal to noise and interference ratio and in turn the detectability of the sphere within the measurement environment. The sphere is placed inside our semi-anechoic chamber, with absorbers to cover the tripod to reduce scatters from the environment.

Measurements are done in the monostatic setup depicted in Figure 1.

Figure 1: RCS sphere measurement configuration

Measurement geometry is depicted Figure 2.  Measurements are done for spheres of 3 (7.62 cm), 5 (12.7 cm) and 6 in (15,4 cm) at 2.5, 5, 7.5 and 10 m. Only one sphere diameter at one distance outside of the far field are required for the measurement. However we use multiple sphere sizes and distances and incident angles for measurement systems validation.

Figure 2: Sphere measurement geometry

4. RCS calibration measurement result

Measurement procedure is as follow.

- First measure receive power with the sphere in the environment.

- Then measure the environment without the sphere.

- Use the background extraction method where receive power without the sphere is subtracted from the receive power of the sphere with the environment.

Figure 3 and Figure 4 show respectively the directional receiver power with the sphere, , in the measurement environment  and receive power of the sphere alone achieved by synthetically removing the sphere through background subtraction.  This is for a sphere 6in diameter and 10 m distance.

Figure 3: directional receiver power with the sphere in the measurement environment

Figure 4: Directional received power of the test environment with the sphere positioned at 5 m.

To mitigate residual environment MPCs, in addition to filtering by the semi-anechoic chamber, and background subtraction, super-resolution algorithm [4] is applied to extract mostly the multipath component (MPC) that corresponds to the sphere.

5. Conclusion

For your information, we described a procedure used for RCS measurement systems calibration based on [1].  This is not unique. There are other techniques that can be used. However this is an important step for   validation of the measured data for the scenarios presented in [2] and [3].

6. References

[1] “IEEE Recommended Practice for Radar Cross-Section Test Procedures” IEEE Std 1502™-2020

[2] “Channel modelling for ISAC”, RWS-230002

[3] “Measurement data for ISAC deployment scenarios”, R1-2400174,  NIST, Athens 02-2024

[4] Papazian, Peter B., et al. “Calibration of Millimeter-wave Channel Sounders for Super-resolution

Multipath Component Extraction” NIST/CTL . 2016 10th European Conference on Antennas and Propagation

(EuCAP). IEEE, 2016.