3GPP TSG-RAN WG1 Meeting #116		R1-2400218

Athens, Greece, February 26th – March 1st, 2024

Source:	vivo

Title:	Maintenance on Rel-18 SL-Evo

Agenda Item:	8.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide our views, as well as the TPs, on some remaining issues for R18 Sidelink evolution, including:

- Channel access mechanism of SLU

- Physical channel design framework of SLU

- Channel access mechanism

In this section, some remaining issues on the channel access mechanism are addressed.

Clarification on the remaining COT

In the RAN1#114 agreement as below, the remaining COT duration is expressed in physical slots. However, it is not correctly captured in TS 37.213, where the remaining channel occupancy duration is expressed in a number of slots K, without any clarification that whether the K is in physical slots or logical slots. Therefore, further clarification on the K should be captured as in below TP#1.

Agreement

“Remaining COT duration” is expressed in physical slots and it is carried in the 2nd stage SCI. The payload size is 4 bits in 15kHz, 5 bits in 30kHz and 6 bits in 60kHz

- If the indicated remaining COT duration is 0 slot, then the COT is not shared by the initiator UE.

- The starting slot for the remaining COT duration is the slot in which the COT-SI is transmitted.

  - Note, when the COT-SI is transmitted in slot n, and if the remaining COT duration is set to K, then the end of the COT duration to share is slot n+K.

Note: “Remaining COT duration” cannot be such that the COT exceeds the maximum COT duration.

Reason for change: It is not clear that whether the K is in physical slots or logical slots. Summary of change: Clarify that the remaining channel occupancy duration is in terms of physical slots. Consequence if not approved: There will be confusion on the slot types of the reaming channel occupancy duration.

*****TP#1 for TS 37.213***** ### 4.5.3 SL channel access procedures in a shared channel occupancy When a UE initiates a channel occupancy using the channel access procedures described in clause 4.5.1 on a channel to transmit SL transmission(s), the UE can provide a channel occupancy sharing information in SL control information that includes at least the Layer 1 source and destination IDs, the corresponding channel access priority class, the remaining channel occupancy duration , and the frequency domain information for the applicable RB set(s) of the channel occupancy. The channel occupancy sharing information can also include additional IDs and associated cast type. The additional IDs includes one pair of Layer 1 source and destination IDs for all cast types, where the source ID is set to the source ID of the UE initiating channel occupancy for unicast and to the reserved bits for groupcast and broadcast. The channel occupancy sharing information transmitted in slot n, can indicate the remaining channel occupancy duration in a number of physical slot(s) K. If K=0, the initiated channel occupancy by the UE shall not be shared for SL transmission(s) by other UE(s). Otherwise, the initiated channel occupancy by the UE can be shared for SL transmission(s) by other UE(s) within a duration starting from the end of slot n and ending at slot n+K. <unchanged part omitted> *****End of TP#1 for TS 37.213*****

Proposal 1: Adopt TP#1 for TS 37.213.

Insufficient time for type 1 LBT

Proposal 7-1 (II): To address the potential issue of insufficient time for a UE to perform Type 1 LBT before a selected resource due to LBT sensing time can be longer than T1 of resource selection window, further study at least the following options for down-selection in a future meeting. Other options are not precluded. - Option 1: A time offset is added to T1 of resource selection window. FFS the time offset length. - Option 2: MAC layer takes into account of a potential Type 1 LBT sensing duration and selects resources accordingly. - Option 3: Type 1 LBT sensing duration is determined firstly, then resource selection takes into account of the LBT duration is performed. - Option 4: Resource is selected firstly, then Type 1 LBT sensing duration is adjusted based on timing of the selected resource. - Option 5: Do nothing; Drop the SL transmission due to LBT failure, perform resource re-selection and attempt to access the channel for the next selected resource. - Option 6: At MAC layer, selection of resource(s) among the reported set of candidate resources from L1 is up to UE implementation in mode 2 for SL-U, instead of random selection. - Option 7: UE prioritizes resource in a detected shared COT.

In RAN1#112 meeting, several enhanced aspects were discussed in [7] to address the issue of insufficient time for a UE to perform Type 1 LBT before a selected resource, which is cited above.

In our opinion, at least the time gap between slot n (where resource selection is triggered) and the slot of the selected resource should be larger than the minimal/remaining LBT duration for the UE, and this behavior can be performed in MAC layer as a candidate UE implementation choice.

Proposal 2: In the resource selection procedure, MAC may prioritize selecting the resource after slot n+K, where K is the number of remaining slots for the UE to perform type 1 channel access.

CPE determination

For the CPE determination, two more cases need to be discussed: 1) the CPE for PSCCH/PSSCH transmission where the PSCCH/PSSCH is used to resume a COT, and 2) the CPE for S-SSB transmission in a S-SSB burst.

CPE determination for PSCCH/PSSCH transmission to resume a COT

For COT resumption using PSCCH/PSSCH transmission, the UE should determine CPE for the PSCCH/PSSCH transmission based on the rule of CPE determination for initiating a COT, because such rule can help mitigate inter-UE blocking. Therefore, the following proposal and TP are made.

Proposal 3: For COT resumption using PSCCH/PSSCH transmission, UE uses the same method to determine CPE for the PSCCH/PSSCH transmission as the case when UE performs Type 1 channel access to initiate a COT for PSCCH/PSSCH transmission.

Proposal 4: Adopt TP#2.

Reason for change: Specification needs a rule to determine the CPE for PSCCH/PSSCH transmission for the case of COT resumption using the PSCCH/PSSCH transmission. Summary of change: Add one more rule for CPE determination for a PSSCH/PSCCH transmission by a UE that follows another SL transmission by the same UE in a channel occupancy. Consequence if not approved: It is not clear how to set the CPE for PSCCH/PSSCH transmission for the case of COT resumption using the PSCCH/PSSCH transmission.

*****TP#2 for TS 38.214***** ##### 8.1.2.1 Resource allocation in time domain The UE shall transmit the PSSCH in the same slot as the associated PSCCH. <unchanged part omitted> - For operation with shared spectrum channel access in frequency range 1, for a PSSCH/PSCCH transmission by a UE that follows another SL transmission by the same UE in a channel occupancy, the UE determines the duration of a cyclic prefix extension Text as follows: - if theWhen gap between the PSSCH/PSCCH transmission and the previous SL transmission is 1 symbol and µ=0, the index Δiis set to ‘1’ for µ=0. - if theWhen gap between the PSSCH/PSCCH transmission and the previous SL transmission is 2 symbols and µ=1,3, the index Δiis set to ‘3’ for µ=1 and to 2 for µ=2. - otherwise, the index Δiis determined in the same way as the first SL transmission with PSSCH/PSCCH to initiate a channel occupancy for a slot. *****End of TP#2 for TS 38.214*****

CPE determination for S-SSB transmission in a S-SSB transmission burst

For consecutive S-SSB transmissions, it is believed that a S-SSB transmission burst should be maintained for more channel access opportunities. One way to formulate the S-SSB transmission burst is to (pre-)configure a default CPE for S-SSB that can reduce the gap between S-SSB transmissions until 16us. However, such default CPE may prevent the S-SSB from using type 2A channel access (i.e., there may be SL transmission in the prior slot), which is not preferred. Another way to formulate the S-SSB transmission burst is to reuse the CPE determination rule for PSCCH/PSSCH transmission in a PSCCH/PSSCH transmission burst, such mechanism can maintain S-SSB burst while still allowing the S-SSB to use a shorter (pre-)configured CPE to access channel using type 2A channel access. Therefore, the following proposal and TP are made.

Proposal 5: For a S-SSB transmission by a UE that follows another SL transmission by the same UE in a channel occupancy, the UE determines the duration of a cyclic prefix extension Text as follows:
-When the gap between the S-SSB transmission and the previous SL transmission is 1 symbol, the index Δiis set to ‘1’ for µ=0
-When the gap between the S-SSB transmission and the previous SL transmission is 2 symbols, the index Δiis set to ‘3’ for µ=1 and to 2 for µ=2.

Proposal 6: Adopt TP#3.

Reason for change: Specification only supports (pre-)configured CPE for S-SSB transmission, however when (pre-)configuring a default CPE for S-SSB to maintain S-SSB transmission burst, the default CPE prevents the S-SSB from using type 2A channel access if there is SL transmission in prior slot. Summary of change: Specification support an additional rule for CPE determination for S-SSB transmission to formulate a SL transmission burst using the S-SSB. Consequence if not approved: It is not clear how to set the CPE for PSCCH/PSSCH transmission for the case of COT resumption using the PSCCH/PSSCH transmission.

*****TP#3 for TS 38.213***** ##### 16.1 Synchronization procedures <unchanged part omitted> - For operation with shared spectrum channel access in frequency range 1, for a S-SS/PSBCH block transmission by a UE that follows another SL transmission by the same UE in a channel occupancy, the UE determines the duration of a cyclic prefix extension Text as follows: - When gap between the S-SS/PSBCH block and the previous SL transmission is 1 symbol, the index Δiis set to ‘1’ for µ=0. - When gap between the S-SS/PSBCH block and the previous SL transmission is 2 symbols, the index Δiis set to ‘3’ for µ=1 and to 2 for µ=2. *****End of TP#3 for TS 38.213*****

- Physical channel design framework

In this section, we discuss the remaining issues related to physical channel design framework.

Clarification on the starting position of PSCCH

The additional starting symbols within a slot has been discussed with the following agreement.

RAN1#112 Agreement For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - The location of 1st starting symbol can be (pre)configured from {#0,#1,#2,#3,#4,#5,#6} per BWP - By default (if no (pre)configuration), the location of the 1st starting symbol is symbol#0 - The location of 2nd starting symbol is (pre-)configured from {#3,#4,#5,#6,#7} per BWP - It shall be configured such that within a slot, the number of symbols used for PSCCH/PSSCH transmission from 2nd starting symbol is not smaller than 6 - It shall be configured such that within a slot, the 2nd starting symbol is later than the 1st starting symbol - PSCCH/PSSCH transmission starting from 1st or 2nd starting symbol shall have the same ending symbol within a slot - Note: assume symbol index in a slot starts from #0

In the TS 38.214, it is clearly stated that the starting position of the PSSCH transmission is sl-StartSymbol+1 if startingSymbolFirst and startingSymbolSecond are not provided for the SL-BWP, otherwise, there are 2 candidate starting symbols, given by startingSymbolFirst and startingSymbolSecond respectively, for PSSCH transmission for slots without PSFCH symbols; and there is one starting symbol, given by startingSymbolFirst, for PSSCH transmission for slots with PSFCH symbols. PSSCH resource allocation starts at the next symbol after each candidate starting symbol. For PSCCH transmission in TS 38.213, the starting position is described as a second symbol that is available for SL transmissions in a slot, which is confusing given two candidate starting symbols are supported. Since there are two candidate starting symbols in a slot, the second symbol that is available for SL transmission can be incorrectly interpreted as the second starting symbol, which is not the original intention of the specification. Therefore, the starting position of the PSCCH transmission should be clarified as in TP#4 below.

Reason for change: The starting symbol for PSCCH transmission is not clear. Summary of change: Clarify the starting symbol for PSCCH transmission in the cases when startingSymbolFirst and startingSymbolSecond are configured and not configured. Consequence if not approved: There are confusion on the starting symbol for PSCCH transmission.

*****TP#4 for TS 38.213***** ## 16.4 UE procedure for transmitting PSCCH A UE can be provided a number of symbols in a resource pool, by sl-TimeResourcePSCCH, starting from a second symbol that is available for SL transmissions in a slot startingSymbolFirst+1 or startingSymbolSecond+1 in a slot without PSFCH symbols or startingSymbolFirst+1 in a slot with PSFCH symbols when startingSymbolFirst and startingSymbolSecond are provided for the SL-BWP, or starting from symbol sl-StartSymbol+1 otherwise, and a number of PRBs in the resource pool, by sl-FreqResourcePSCCH, starting from the lowest PRB index of the lowest sub-channel index, in an RB-set with a lowest index if applicable, of the associated PSSCH for a PSCCH transmission with a SCI format 1-A. For operation with shared spectrum channel access, - if sl-TransmissionStructureForPSCCHandPSSCH = 'interlaceRB', the PRBs for PSCCH are within the sub-channel with the lowest index and within the RB-set with the lowest index among the RB-set(s) for the associated PSSCH transmission, - if sl-TransmissionStructureForPSCCHandPSSCH = 'contiguousRB', the PRBs for PSCCH are within the sub-channel with the lowest index in the RB-set with the lowest index among the RB-set(s) for the associated PSSCH transmission, and all PRBs in the sub-channel overlapping with intra-cell guard band [6, TS 38.214] are not used for PSCCH. <unchanged part omitted> *****End of TP#4 for TS 38.213*****

Proposal 7: Adopt TP#4 for TS 38.213.

Remaining issues on the multiple starting symbols

The additional starting symbols within a slot has been discussed with the following agreement.

RAN1#112 Agreement For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - The location of 1st starting symbol can be (pre)configured from {#0,#1,#2,#3,#4,#5,#6} per BWP - By default (if no (pre)configuration), the location of the 1st starting symbol is symbol#0 - The location of 2nd starting symbol is (pre-)configured from {#3,#4,#5,#6,#7} per BWP - It shall be configured such that within a slot, the number of symbols used for PSCCH/PSSCH transmission from 2nd starting symbol is not smaller than 6 - It shall be configured such that within a slot, the 2nd starting symbol is later than the 1st starting symbol - PSCCH/PSSCH transmission starting from 1st or 2nd starting symbol shall have the same ending symbol within a slot - Note: assume symbol index in a slot starts from #0 RAN1#112bis Agreement Regarding Tx UE behavior, at least when it initiates a COT: - For the 1st slot of a COT, the Tx UE chooses the earliest starting symbol for PSCCH/PSSCH transmission after clearing LBT. - Note: in the same slot, Tx UE can use the 2nd starting symbol only if LBT fails at the 1st starting symbol - FFS: whether/how to support that for the remaining slots of a COT, the Tx UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission. - FFS applicable scenarios - e.g., at least for MCSt with no greater than 16us gap - e.g., at least for transmission with no greater than 16us gap from the previous transmission by any UE - FFS: Rx UE behavior FFS: COT sharing case RAN1#113 Agreement If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission, for TBS determination and 2nd SCI overhead, in TS 38.214 Clause 8.1.3.2: - L_ref replaces sl-LengthSymbols - Value range of L_ref is {7, 8, 9, 10, 11, 12, 13, 14} symbols - NsymbPSFCH is determined in the same way as in legacy NR SL RAN1#114 Agreement For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, the PSCCH/PSSCH transmission has at least 1 AGC symbol - Regarding Rx UE behaviour: - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot - Note: AGC symbol is duplication of next symbol

In SLU, there are two candidate starting positions for PSSCH/PSCCH transmission. When a UE operates as a receiving UE, since different Tx UEs may transmit to the Rx UE from different starting positions, the Rx UE has to detect at both the 1st or the 2nd starting symbol within a slot.  Otherwise, the Rx UE may miss the transmissions starting from the 2nd starting symbol. Therefore, it should be clearly stated that the Rx UE may detect from both the 1st and the 2nd starting symbol within a slot.

Reason for change: To avoid the missed detection from the second starting position. Summary of change: Clarify that the Rx UE shall detect from both the 1st and the 2nd starting symbol within a slot Consequence if not approved: The Rx UE may miss the transmission from the 2nd starting symbol, and Tx UE will perform retransmission, which degrades the system performance.

*****TP#5 for TS 38.214***** 8.3 UE procedure for receiving the physical sidelink shared channel For sidelink resource allocation mode 1, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B, 2-C and 2-D, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate. For sidelink resource allocation mode 2, a UE upon detection of SCI format 1-A on PSCCH can decode PSSCH according to the detected SCI formats 2-A, 2-B, 2-C and 2-D, and associated PSSCH resource configuration configured by higher layers. The UE is not required to decode more than one PSCCH at each PSCCH resource candidate. A UE is required to decode neither the corresponding SCI formats 2-A, 2-B, 2-C nor the PSSCH associated with an SCI format 1-A if the SCI format 1-A indicates an MCS table that the UE does not support. The UE may receive at both startingSymbolFirst and startingSymbolSecond if they are provided for the SL-BWP. <unchanged part omitted> *****End of TP#5 for TS 38.214*****

Proposal 8: Adopt TP#5 for TS 38.214.

Another issue is that, if PSCCH/PSSCH transmission starts from 1st starting symbol, the PSCCH/PSSCH transmission has at least 1 AGC symbol. When there are 2 AGC symbols in a slot, the second AGC symbol may conflict with the DMRS symbol for some DMRS patterns. If the collided DMRS symbol is not transmitted, the Rx UE may not correctly decode the PSSCH. To minimize the spec impact, it is preferred that the UE does not expect the DMRS symbol to conflict with the AGC symbol. Furthermore, when the second AGC symbol overlaps with PSSCH, it should be excluded from the PSSCH reference resources when determining the TBS. The following TP should be applied.

Reason for change: When there are two AGC symbols within a slot, the DMRS symbol may conflict with the second AGC symbol. Summary of change: Clarify that the UE does not expect the DMRS symbol to be overlapped with the startingSymbolSecond when there are two AGC symbols within the slot. Consequence if not approved: The confliction between the DMRS symbol and the AGC symbol may cause decoding failure of the PSSCH.

*****TP#6 for TS 38.211***** 8.4.1.1.2 Mapping to physical resources <unchanged part omitted> The position(s) of the DM-RS symbols is given by l according to Table 8.4.1.1.2-1 where the number of PSSCH DM-RS is indicated in the SCI, and ld is the duration of the scheduled resources for transmission of PSSCH according to clause 8.1.2.1 of [6, TS 38.214] and the associated PSCCH, including the OFDM symbol duplicated as described in clauses 8.3.1.5 and 8.3.2.3. The UE does not expect the DMRS symbol to be overlapped with the startingSymbolSecond if configured within a slot. <unchanged part omitted> *****End of TP#6 for TS 38.211*****

Proposal 9: Adopt TP#6 for TS 38.211.

RSSI calculation

Agreement: - For congestion control purpose, SL-RSSI measurements use the OFDM symbols starting from the next symbol of the 2nd candidate starting symbol.

In RAN4#109 meeting, the above agreement was achieved, and RAN4 asked RAN1 to take the information into account. It can be seen that if the 2nd candidate starting symbol is configured, SL-RSSI measurement would be started from the next symbol of the 2nd candidate starting symbol. Hence, to reflect this change, the following TP is proposed:

Reason for change: If the 2nd candidate starting symbol is configured, SL-RSSI measurement would be started from the next symbol of the 2nd candidate starting symbol, which is different from the current specification that SL-RSSI measurements always start from the 2nd OFDM symbol. Summary of change: Clarify that SL-RSSI measurement would be started from the next symbol of the 2nd candidate starting symbol if the 2nd candidate starting symbol is configured. Consequence if not approved: The SL-RSSI measurement result would be wrong if the 2nd candidate starting symbol is configured.

*****TP#7 for TS 38.215***** ### 5.1.25 Sidelink received signal strength indicator (SL RSSI) Definition; Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH, starting from the next symbol of the startingSymbolSecond symbol if the startingSymbolSecond is configured for operation with shared spectrum channel access, otherwise the 2nd OFDM symbol. For frequency range 1, the reference point for the SL RSSI shall be the antenna connector of the UE. For frequency range 2, SL RSSI shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SL RSSI value shall not be lower than the corresponding SL RSSI of any of the individual receiver branches.. Applicable for; Sidelink. *****End of TP#7 for TS 38.215*****

Proposal 10: Adopt TP#7 for TS 38.215.

Intra-cell guard band

Agreement For SL-U PHY, following higher layer parameters are endorsed. row; Parameter name in the text; Description; Value range; Default value aspect. 3; intraCellGuard BandsSL-List; List of intra-cell guard bands for operation with shared spectrum channel access. If not configured, the guard bands are defined according to 38.101-1 [15], see TS 38.214 [19], clause 7. For operation in licensed spectrum, this field is absent, and no UE action is required.; SEQUENCE (SIZE (1..maxSCSs)) OF IntraCellGuardBandsPerSCS-r16; N/A.

In the previous meetings, the intraCellGuardBandsSL-List was agreed, together with the Rel-16 guard band configuration (IntraCellGuardBandsPerSCS-r16) that can be 0 – 15 PRB. In the last meeting, someone proposed to reverse the agreement by removing the  zero value for intra-cell guard band. In NRU, there are UEs only capable of performing transmission on the BWP as a whole, thus the configuration of the guard band with zero value was then introduced, and the corresponding channel procedure was also specified. In SLU, to achieve better flexibility in configuration and avoid the potential limitation in deployment, such kind of UE operation, i.e., supporting single subchannel in a SL BWP, should still be supported, thus the zero value should also be supported in SLU. If there were some cases in which a non-zero guard band is needed, the network can configure a proper value, which anyway is already supported by the current agreement. Hence, there is no reason to reverse the previous agreement – even unnecessary to spend time to re-discuss this.

Proposal 11: Keep the current agreement of intra-cell guard band configuration as it is (i.e., including zero guard band) and do not further re-discuss this.

PSFCH resource mapping

It is agreed that two types of PSFCH transmission are introduced to meet the OCB requirement in SLU:

·Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

·Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

Currently, the sequence is generated with the length of NscRB (i.e., the number of subcarriers per PRB) and mapped to a PRB occupied by the PSFCH in TS 38.211. However, the sequence generation and mapping to physical resources for these two types of PSFCH transmission have not been specified yet. In NRU, for interlaced PUCCH transmission, the mapping operation is repeated for each assigned PRB with the resource-block-dependent sequence. A similar operation as NRU should be applied for PSFCH in SL-U. Hence, the following TP#8 is proposed.

Additionally, the cyclic shift of these two types of PSFCH was agreed in the previous RAN1 meeting. In Alt 2-3a, the cyclic shift of each PRB of dedicated interlace is determined by the PSFCH transmission with HARQ-ACK information, while in Alt 1-1b, although the cyclic shift on each of K3 dedicated PRB(s) is same as that in Alt 2-3a, the cyclic shift on each PRB of common interlace is up to UE implementation. However, the current Spec implies that the cyclic shift is only determined by the PSFCH transmission with HARQ-ACK information, which is not aligned with the agreement. Hence, we suggest capturing the cyclic shift on each PRB of common interlace for Alt 1-1b as follows.

Proposal 12: The resource-block dependent sequence generation should be supported and the mapping operation should be repeated for each PRB occupied by the PSFCH in SL-U. Adopt TP #8 in TS 38.211.

Reason for change: The current spec states that the sequence is generated with the length of NscRB (i.e., the number of subcarriers per PRB) and mapped to a PRB occupied by the PSFCH. However, the sequence generation and mapping to physical resources for two types of PSFCH transmission in SLU have not been specified. Summary of change: Add the resource-block dependent sequence generation and the mapping operation for two types of PSFCH transmission in SLU. Consequence if not approved: It is not clear how to generate the sequence of the PSFCH and map the PSFCH to physical resources.

*****TP#8 for TS 38.211***** ##### 8.3.4.2.1 Sequence generation The sequence xn shall be generated according to xn=ru,vα,δn n=0,1,…,NscRB-1 where ru,vα,δ(n) is given by clause 6.3.2.2 with the following exceptions: - mcs is given by clause 16.3 of [5, TS 38.213]; - m0 is given by clause 16.3 of [5, TS 38.213]; - mint is given by - mint=5nIRBμ if the higher-layer parameter transmissionStructureForPSFCH is configured and set to ‘dedicated interlace’ and where nIRBμ is the resource block number within the interlace; - mint=0 otherwise - l=0; - l' is the index of the OFDM symbol in the slot that corresponds to the second OFDM symbol of the PSFCH transmission in the slot given by [5, TS 38.213]; - u=nID mod 30 and v=0 with nID given by the higher-layer parameter sl-PSFCH-HopID if configured; otherwise, u=0. - cinit=nID with nID given by the higher-layer parameter sl-PSFCH-HopID if configured; otherwise, cinit=0. ##### 8.3.4.2.2 Mapping to physical resources The sequence xn shall be multiplied with the amplitude scaling factor βPSFCH in order to conform to the transmit power specified in [5, TS 38.213] and mapped in sequence starting with x0 to resource elements k,lp,μ assigned for transmission of the second PSFCH symbol according to clause 16.3 of [5, TS 38.213] in increasing order of the index k over the assigned physical resources on antenna port p=5000. The resource elements used for the PSFCH in the OFDM symbol in the mapping operation above shall be duplicated in the immediately preceding OFDM symbol. If the higher-layer parameter transmissionStructureForPSFCH is configured and set to ‘dedicated interlace’, the mapping operation shall be repeated for each resource block in the interlace and in the RB set over the assigned physical resource blocks according to clause 16.3 of [5, TS 38.213], with the resource-block dependent sequence generated according to clause 8.3.4.2.1 If the higher-layer parameter transmissionStructreForPSFCH is configured and set to ‘dedicated PRB’, the mapping operation shall be repeated for each resource block assigned for transmission of the common interlace and for PSFCH transmission with HARQ-ACK information over the assigned physical resource according to clause 16.3 of [5, TS 38.213], with the resource-block dependent sequence generated according to clause 8.3.4.2.1, where the cyclic shift α on each resource block assigned for transmission of the common interlace is up to UE implementation. *****End of TP#8 for TS 38.211*****

PSFCH prioritization

- PSFCH prioritization in COT sharing case

Agreement For UE-to-UE COT sharing, - When performing S-SSB transmission(s), a responding UE can utilize a COT shared by a COT initiating UE (using type 1 channel access) when the responding UE is intended to transmit S-SSB within RB set(s) corresponding to the shared COT. - When performing PSFCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when at least one of the responding UE’s PSFCH transmissions in a symbol/slot within RB set(s) corresponding to the shared COT is intended for the COT initiating UE. - FFS: whether a responding UE can transmit PSFCH(s) to UE(s) other than the initiator - When performing PSSCH/PSCCH transmission(s), a responding UE can utilize a COT shared by a COT initiating UE at least when the responding UE’s PSSCH/PSCCH transmission(s) within RB set(s) corresponding to the shared COT is intended for the COT initiating UE - FFS whether to support the case if a responding UE transmits PSSCH/PSCCH to destination ID other than the source ID of the COT initiating transmission, where the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) can be different from the source/destination IDs of COT initiating UE’s PSSCH/PSCCH transmission when sharing the COT information. - FFS: how to determine / what are the restrictions to the destination ID of the responding UE’s PSSCH/PSCCH transmission(s) to utilize the COT shared by the initiating UE. - FFS whether the responding UE can utilize the COT when at least the responding UE’s PSCCH transmission in the reserved resources within the shared COT or MCSt is intended for the COT initiating UE and what are the restrictions (e.g., priority, etc.) and indication to the responding UE. - FFS: UE forwarding/relaying information about a COT initiated by another UE.

It was agreed that a responding UE can utilize a COT shared by a COT initiating UE at least when at least one of the responding UE’s PSFCH transmissions is intended for the COT initiating UE. However, such aspect has not been reflected in the PSFCH prioritization procedure. In the current spec, UE only performs PSFCH prioritization according to the PSFCH priority, which may result in the responding UE cannot share the COT to transmit PSFCH if all of the PSFCH transmissions intended for the COT initiating UE are dropped by prioritization. Therefore, when the responding UE performs PSFCH prioritization, it should ensure that at least one of the PSFCH transmissions intended for the COT initiating UE is selected in the shared COT.

Proposal 13: During the PSFCH prioritization, the responding UE should preserve at least one of the PSFCH transmissions intended for the COT initiating UE in the shared COT. Adopt TP #9 in TS 38.211.

Reason for change: The responding UE should select at least one of the PSFCH transmissions intended for the COT initiating UE to share the COT, which has not been reflected in the PSFCH prioritization procedure. Summary of change: UE can select at least one of the PSFCH transmissions intended for the COT initiating UE in the shared COT. Consequence if not approved: The responding UE cannot legally share the COT to transmit PSFCH if the PSFCH transmission intended for the COT initiating UE is dropped by prioritization.

*****TP#9 for TS 38.213***** ##### 16.2.4.2 Simultaneous PSFCH transmission/reception For a PSFCH transmission or reception with HARQ-ACK information, a priority value for the PSFCH is equal to the priority value indicated by an SCI format 1-A associated with the PSFCH. For PSFCH transmission with conflict information, a priority value for the PSFCH is equal to the smallest priority value determined by the corresponding SCI format(s) 1-A for the conflicting resource(s). For PSFCH reception with conflict information, a priority value for the PSFCH is equal to the priority value determined by the corresponding SCI format 1-A for the conflicting resource. If a UE - would transmit Nsch,Tx,PSFCH PSFCHs and receive Nsch,Rx,PSFCH PSFCHs, and - transmissions of the Nsch,Tx,PSFCH PSFCHs would overlap in time with receptions of the Nsch,Rx,PSFCH PSFCHs the UE transmits or receives only a set of PSFCHs corresponding to the smallest priority field value, as determined by a first set of SCI format 1-A and/or a second set of SCI format 1-A [5, TS 38.212] that are respectively associated with PSFCHs with HARQ-ACK information from the Nsch,Tx,PSFCH PSFCHs and PSFCHs with HARQ-ACK information from the Nsch,Rx,PSFCH PSFCHs when one or more of the PSFCHs provide HARQ-ACK information. If none of the Nsch,Tx,PSFCH PSFCHs and none of the Nsch,Rx,PSFCH PSFCHs provide HARQ-ACK information, the UE transmits or receives only a set of PSFCHs corresponding to the smallest priority value of the first set of PSFCHs and the second set of PSFCHs that are respectively associated with the Nsch,Tx,PSFCH PSFCHs and the Nsch,Rx,PSFCH PSFCHs when the PSFCHs provide conflict information. If a UE would transmit Nsch,Tx,PSFCH PSFCHs in a PSFCH transmission occasion, the UE first transmits PSFCHs with HARQ-ACK information from NTx,PSFCH PSFCHs corresponding to the smallest priority field values from the NTx,PSFCH priority field values, if any. Subsequently, the UE transmits remaining PSFCHs with conflict information corresponding to the smallest remaining priority field values from the NTx,PSFCH priority field values, if any. For operation with shared spectrum channel access, if a UE does not support PSFCH transmission in non-contiguous RB sets, the UE selects for PSFCH transmission any contiguous RB set(s) that include PSFCH with the smallest priority value. For operation with shared spectrum channel access, for the PSFCH transmission by a UE within a channel occupancy, the UE selects at least one of the PSFCH transmission intended for the UE initiating the channel occupancy. If a UE indicates a capability to receive NRx,PSFCH PSFCHs in a PSFCH reception occasion [18, TS 38.306], the UE first receives PSFCHs with HARQ-ACK information, if any, and subsequently receives PSFCHs with conflict information, if any. *****End of TP#9 for TS 38.211*****

- Clarification of the maximum number of simultaneous PSFCH transmissions

In R16/17 SL, UE may drop some PSFCHs based on the number of PSFCH(s) resources that the UE can transmit/receive in a slot indicated in UE’s capability. Since each PSFCH occupies one PRB in R16/17 SL, there is no ambiguity about the number of PSFCH resources and the number of PSFCHs. However, regarding R18 SLU, it is not clear how to perform PSFCH prioritization, especially to determine the simultaneous PSFCH transmissions based on UE’s capability, since both types of PSFCH transmission occupy more than one PRB. Therefore, a clarification of the UE’s capability about the UE can transmit/receive PSFCHs in a slot is needed in SLU.

Option 1: the UE’s capability indicates the number of PSFCH(s) PRBs that the UE can transmit/receive in a slot.

Option 2: the UE’s capability indicates the number of PSFCH(s) interlaces that the UE can transmit/receive in a slot.

Option 3: the UE’s capability indicates the number of PSFCH resources with valid HARQ-ACK information in response to a PSSCH reception or with conflict information that the UE can transmit/receive in a slot.

In option 1, the number of simultaneous PSFCHs transmissions is subject to the PSFCH(s) PRB numbers. As the PRB number of each PSFCH transmission increases, the number of simultaneous PSFCH transmission decreases. It is noted that each PSFCH interlace contains 10/11 PRBs, thus the number of simultaneous HARQ-ACK or IUC transmission is approximately 1/10 compared with Rel-16. When the UE’s capability is 20 PRBs, the number of PSFCH transmissions is shown in Table 1. The decreased number of simultaneous PSFCH transmission may result in a decline in system performance.

| Rel-16: 20 PSFCHs | Alt 1-1b: 2 PSFCHs(20 PSFCH RBs) | Alt 2-3a: 2 PSFCHs(20 PSFCH RBs) |
| --- | --- | --- |

Table 1 the number of PSFCH transmissions when UE’s capability is 20 PRBs

For Alt 2-3a, option 2 implies that the number of simultaneous PSFCH transmission is equal to the interlace number indicated in UE’s capability. However, in Alt 1-1b, each interlace may contain multiple sets of dedicated K3 PRBs, resulting in the number of simultaneous PSFCH transmission being much more than the interlace number indicated in UE’s capability. For example, UE can transmit 20 different HARQ-ACK/IUC in 3 interlaces (including common interlace) when K3 is equal to 1. When the UE’s capability is 3 interlaces, the number of PSFCH transmissions is shown in Table 2. In this case, the processing complexity will be dramatically increased compared with Rel-16.

| Rel-16: 30 PSFCHs | Alt 1-1b: 20 PSFCHs(K3=1) | Alt 2-3a: 3 PSFCHs |
| --- | --- | --- |

Table 2 the number of PSFCH transmissions when UE’s capability is 3 interlaces

In Option 3,  the number of PSFCH resources is determined excluding the common interlace, and the number of simultaneous PSFCH transmission is equal to the indication in UE’s capability. The processing complexity slightly increases due to the PRB repetition of the PSFCH transmission compared with Rel-16. When the UE’s capability is 3 PSFCH resources, the number of PSFCH transmissions is shown in Table 3. Moreover, since the UE may drop the PRB of common interlace in Alt 1-1a, it is reasonable that the UE performs PSFCH prioritization based on the PSFCH resource without common interlace. Therefore, option 3 is preferred.

| Rel-16: 3 PSFCHs | Alt 1-1b: 3 PSFCHs | Alt 2-3a: 3 PSFCHs |
| --- | --- | --- |

Table 3 the number of PSFCH transmissions when UE’s capability is 3 interlaces

Proposal 14: The UE’s capability of the supported PSFCH indicates the number of PSFCH resources with valid HARQ-ACK information in response to a PSSCH reception or with conflict information that the UE can transmit/receive in a slot.

PSFCH power control

114-b Agreement: Agreement In “Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace”: - The power for a PSFCH transmission is equally allocated over the occupied PRBs. Working assumption - In “Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)”: - Assume the UE transmits N PSFCH - Denote the final Tx power on one common PRB is P_common - Denote the final Tx power on one dedicated PRB is P_dedicated - P_common <= P_dedicated - (pre-)configure an offset between P_common and P_dedicated - Send an LS to RAN4 asking whether there is any difficulty for supporting the following cases - P_common < P_dedicated - P_common = P_dedicated 115 Agreement: Agreement Regarding “SL-U PSFCH occupies more than 1 PRB”: - PPSFCH,one refers to the target power on one dedicated PRB for a PSFCH transmission.

Two PSFCH transmission schemes, namely Alt 1-1b and Alt 2-3a, are agreed. The power control for these schemes have been discussed in several meetings, and some agreements have been made as above. More specifically, there are four main cases to determine the PSFCH transmit power:

- Case1: UE is scheduled to transmit no more than Nmax,PSFCH PSFCHs and the total target power of all scheduled Nsch,Tx,PSFCH PSFCHs is no larger than PCMAX

- Case2: UE is scheduled to transmit no more than Nmax,PSFCH PSFCHs and the total target power of all scheduled Nsch,Tx,PSFCH PSFCHs is larger than PCMAX

- Case3: UE is scheduled to transmit more than Nmax,PSFCH PSFCHs and the total target power of all Nmax,PSFCH PSFCHs is no larger than PCMAX

- Case4: UE is scheduled to transmit more than Nmax,PSFCH PSFCHs and the total target power of all Nmax,PSFCH PSFCHs is larger than PCMAX

Since a PSFCH transmission in SLU occupies more than one PRBs, PPSFCH,one refers to the target power on one dedicated PRB for a PSFCH transmission. The target power of N PSFCHs can be determined by formula (1), where N refers to Nsch,Tx,PSFCH or Nmax,PSFCH, and NPSFCH,one,kinterlace refers to the number of PRBs in the interlace for a PSFCH transmission.

PPSFCH,target=PPSFCH,one+10log10k=1NNPSFCH,one,kinterlacedBm#1

Based on the current agreements and specification, the PSFCH power control procedure is clear for Alt 2-3a, as summarized below:

1): When a UE is scheduled to transmit Nsch,Tx,PSFCH PSFCHs, the UE first checks whether the number of Nsch,Tx,PSFCH PSFCHs exceed the maximum number of PSFCHs (i.e., Nmax,PSFCH).

1-1) If no, go step2. (The actual PSFCHs is Nsch,Tx,PSFCH PSFCHs)

1-2) If yes, UE determines the Nmax,PSFCH PSFCHs based on the PSFCH priority, then go step2; (The actual PSFCHs is Nmax,PSFCH PSFCHs)

2) After step1, UE further checks whether the total target power of all actual PSFCHs exceed PCMAX

2-2) If no, the transmitted PSFCHs is the actual PSFCHs and the transmit power of each PSFCH is determined by formula (2).

PPSFCH,ki=PPSFCH,one+10log10NPSFCH,one,kinterlacedBm#2

2-1) If yes, UE determines the NTx,PSFCH PSFCH based on the PSFCH priority. The transmitted PSFCHs is NTx,PSFCH PSFCH and the transmit power of each PSFCH is determined by formula (3);

PPSFCH,ki=minPCMAX-10log10k=1NTx,PSFCHNPSFCH,one,kinterlace,PPSFCH,one +10log10NPSFCH,one,kinterlacedBm#3

On the other hand, regarding the PSFCH power control for Alt 1-1b, some additional issues should be discussed.

One remaining issue is whether the case of the case of P_common < P_dedicated can be supported for Alt 1-1b. According to the latest RAN4’s LS, P_common < P_dedicated is feasible when the evaluated MPR value is not higher than that for P_common = P_dedicated for all PSFCH transmissions with Alt 1-1b. However, starting from Rel-16, RAN4 made the requirement for PSFCH transmission such as MPR based on the assumption that the power per RB is the same in PSFCH transmission. The configuration of P_common < P_dedicated is in practice possible only when RAN4 defines the corresponding requirements for at least one possible power offset value. Whehther and how to define such requirements, as well as the reasonable power offset between RBs, can be left to RAN4, thus no additional RAN1 work is needed right now.

Another remaining issue is whether/how to handle the case that different numbers of PSFCH transsmion are stacked on different PSFCH RBs of the dedicate interlace, e.g., 4 PSFCHs on one RB while 2 PSFCHs on another RB. It is equivalent to having another power offset between different RBs on the dedicated interlace. Similar to the discussion above, there is no RAN4 requirement defined for this case, and whehther and how to define such requirements, as well as the reasonable power offset between RBs, can be left to RAN4. Nonetheless, a simple workaround is to configure no more than one cyclic shift pairs in the resource pool. Then, no additional RAN1 specificaiton change is expected; or if spec change is really needed, simply it can be clarified in the specification that a UE does not expect more than 1 PSFCH transmission on the same PRB.

Observation 1: There is no RAN4 requirement defined for the case that different numbers of PSFCH transsmion are stacked on different PSFCH RBs of the dedicate interlace, how to handle this case is up to RAN4.

Proposal 15: For PSFCH power control procedure, the power control scheme of PSFCH Alt 2-3a can be reused for PSFCH Alt 1-1b.

- Conclusion

In this contribution, we focus on the remaining issues for R18 Sidelink evolution, and have the following observations and proposals:

Observation 1: There is no RAN4 requirement defined for the case that different numbers of PSFCH transsmion are stacked on different PSFCH RBs of the dedicate interlace, how to handle this case is up to RAN4.

Proposal 1: Adopt TP#1 for TS 37.213.

Proposal 2: In the resource selection procedure, MAC may prioritize selecting the resource after slot n+K, where K is the number of remaining slots for the UE to perform type 1 channel access.

Proposal 3: For COT resumption using PSCCH/PSSCH transmission, UE uses the same method to determine CPE for the PSCCH/PSSCH transmission as the case when UE performs Type 1 channel access to initiate a COT for PSCCH/PSSCH transmission.

Proposal 4: Adopt TP#2.

Proposal 5: For a S-SSB transmission by a UE that follows another SL transmission by the same UE in a channel occupancy, the UE determines the duration of a cyclic prefix extension Text as follows:
-When the gap between the S-SSB transmission and the previous SL transmission is 1 symbol, the index Δiis set to ‘1’ for µ=0
-When the gap between the S-SSB transmission and the previous SL transmission is 2 symbols, the index Δiis set to ‘3’ for µ=1 and to 2 for µ=2.

Proposal 6: Adopt TP#3.

Proposal 7: Adopt TP#4 for TS 38.213.

Proposal 8: Adopt TP#5 for TS 38.214.

Proposal 9: Adopt TP#6 for TS 38.211.

Proposal 10: Adopt TP#7 for TS 38.215.

Proposal 11: Keep the current agreement of intra-cell guard band configuration as it is (i.e., including zero guard band) and do not further re-discuss this.

Proposal 12: The resource-block dependent sequence generation should be supported and the mapping operation should be repeated for each PRB occupied by the PSFCH in SL-U. Adopt TP #8 in TS 38.211.

Proposal 13: During the PSFCH prioritization, the responding UE should preserve at least one of the PSFCH transmissions intended for the COT initiating UE in the shared COT. Adopt TP #9 in TS 38.211.

Proposal 14: The UE’s capability of the supported PSFCH indicates the number of PSFCH resources with valid HARQ-ACK information in response to a PSSCH reception or with conflict information that the UE can transmit/receive in a slot.

Proposal 15: For PSFCH power control procedure, the power control scheme of PSFCH Alt 2-3a can be reused for PSFCH Alt 1-1b.

- References

- RP-213678, WID on NR sidelink evolution, RAN #94e, Electronic Meeting, Dec. 6-17, 2021.

- ETSI EN 301 893, Harmonised Standard covering the essential requirements Harmonised Standard covering the essential requirements, v2.1.1, 2017-05.

- 3GPP RAN1#109-e, Chairman’s notes.

- 3GPP RAN1#110bis, Chairman’s notes.

- 3GPP RAN1#111, Chairman’s notes

- 3GPP RAN1#112, Chairman’s notes

- 3GPP RAN1#112bis, Chairman’s notes

- R4-2318997, Discussion on PSFCH power control, RAN #109, Nov.13-17,2023.