3GPP TSG-RAN WG1 Meeting #116		R1-2400227

Athens, Greece, February 26th – March 1st, 2024

Source:	vivo

Title:	Discussion on Rel-18 sidelink UE features

Agenda Item:	8.12.2

Document for:	Discussion and Decision

- Introduction

In the previous meeting, the UE features for Rel-18 sidelink evaluation were discussed and agreed in [1], and copied in Annex. In this contribution, we provide our view on the remaining issues of UE features for Rel-18 NR sidelink evolution.

- Discussion

  - Sidelink on unlicensed spectrum

For SLU, many features are introduced to meet the regulatory requirements such as LBT, OCB, PSD, etc. In the RAN1#114bis meeting, the basic framework of the UE feature list for SLU was updated in [1] and copied in the Annex. In this section, the remaining details of UE features for SLU are discussed.

Firstly, the FG 47-k9 should be introduced to indicate the support of mode-1 resource allocation for SLU. It seems some companies think that the Rel-16 FG 15-2 can be reused. However, it should be noted that the Rel-16 UE FG for mode-1 transmission (i.e., FG 15-2) requires that the Uu and SL are in the same band (i.e., “UE can monitor DCI format 3_0 for NR sidelink dynamic scheduling and configured grant type 2 on the same carrier as sidelink”), which is not aligned with the Rel-18 WID objective where the SLU mode-1 resource allocation is supported with the restriction that Uu operation for mode 1 is limited to licensed spectrum only. Besides, the FG 15-25 (“Transmitting NR sidelink mode 1 scheduled by NR Uu on a different carrier”) cannot be used either, as it requires FG 15-2 as the prerequisite feature. Therefore, the UE feature 47-k9 indicating the support of NR SL mode-1 should be introduced.

Proposal 1: The UE capability 47-k9 should be introduced to indicate the support of NR SL mode-1 scheduling SL transmission on an unlicensed band.

Secondly, in the RAN1#114bis meeting, the CW autonomous update feature is introduced as below.

Agreement For the CWp autonomous update to the next higher allowed value when the same value is consecutively used for X times for generation of Ninit, - The (pre-)configuration provides 1 value for X among a value range of {1, 8, 16, 32, ‘infinity’}. - This operation is restricted only to PSCCH/PSSCH transmission with HARQ feedback indicator in SCI-2 is set to disabled, regardless of PSFCH resources being configured in a resource pool.

The corresponding candidate UE FG 47-k8 is defined for this feature. Considering that the feature is a new channel access behavior that is not defined for NRU, and especially given that this is a late feature that was agreed during the maintenance phase (i.e., after the RAN1 frozen time), it is not reasonable to mandate every UE to implement.

Proposal 2: The UE capability 47-k8 is introduced for CW autonomous update of SL transmission without HARQ feedback is introduced.

Additionally, the FG 47-m13 is proposed to indicate the support of PSFCH transmission with 1 common interlace and K3 dedicated PRB(s). Considering that the behavior of this PSFCH scheme is quite different from the dedicated-interlace based scheme, it seems beneficial to introduce the FG 47-m13.

Proposal 3: The UE capability 47-m13 can be introduced for the PSFCH transmission scheme with 1 common interlace and K3 dedicated PRB(s).

Moreover, S-SSB repetition on time domain (multiple occasions) and frequency domain (multiple RB sets) are supported in SLU, and the respective transmission capabilities are defined. There were some discussions on whether to introduce the UE FGs for S-SSB reception (on additional time and/or frequency locations). In our view, whether and how to receive and decode the additional S-SSB, either in time or frequency location, can be up to UE implementation. The benefits of introducing and/or reporting this UE FG are not clear.

Proposal 4: It is not necessary to introduce Rx capabilities for S-SSB reception on additional time and/or frequency occasions.

Regarding the agreed UE FGs, in our view, FG 47-k2, 47-k3, 47-k4, and 47-m4 should be reported to peer UE. For unicast transmission, a UE should be aware of whether the peer UE supports multiple RB sets (and multi-channel access) so that it transmits a TB over multiple RB sets and is expected to correctly receive the PSFCH that may span multiple channels. The UE-A should be aware that the peer UE-B is capable of receiving COT SI, so that it may determine to share the COT to the peer UE. On the other hand, the UE-B should be aware that the peer UE-A supports transmitting UE-to-UE COT sharing information, so that it can provide the ue-toUE-COT-SharingED-Threshold to peer UE-A when it performs in COT sharing case. Similarly, a UE should be aware that the peer UE is capable of receiving SL transmission from the 2nd starting symbol in a slot, so that it may determine to transmit the TB to that UE from the 2nd starting symbol when LBT failed in the 1st starting symbol but succeeded before the 2nd starting symbol.

Proposal 5: The UE FG 47-k2, 47-k3, 47-k4, and 47-m4 should be reported to peer UE.

On the other hand, in our view, FG 47-k3, 47-k4, 47-k9 (obviously), 47-m10, 47-m11, 47-m11a, 47-m12, and 47-m12a should be reported to the network for mode-1 scheduling. For the COT sharing case (i.e., 47-k3/k4), the gNB needs to know whether the two SL UEs support UE to UE COT sharing. Only when they support, the gNB can schedule two SL transmissions of these two UEs in a back-to-back manner in the unlicensed band, otherwise, the second SL transmission would always be dropped due to LBT failure if the first SL transmission occupied the channel. If these FGs are not reported to gNB, mode-1 scheduling would be very inefficient. For the contiguous RB-based PSCCH/PSSCH transmission, this capability is needed so that network can provide a suitable SL carrier configuration in unlicensed band, as the FG 47-m1. Similarly, the FG 47-m11, 47-m11a, 47-m12, and 47-m12a should be reported to the gNB for proper S-SSB and PSFCH configurations.

Proposal 6: The UE FG 47-k3, 47-k4, 47-k9, 47-m10, 47-m11, 47-m11a, 47-m12, and 47-m12a should be reported to the network.

A list of the detailed UE capabilities for SLU including the updates to the agreed FG is provided in Annex.

  - Co-channel coexistence for LTE Sidelink and NR Sidelink

In this section, the UE features for co-channel coexistence of LTE sidelink and NR sidelink are discussed.

The UE capability (FG 47-s1) of indicating the support of dynamic resource pool sharing for co-channel coexistence between LTE SL and NR SL has almost been finalized. The only remaining issues of FG 47-s1 are the details of the prerequisites. The FG 15-3 and 15-11 can be considered as the prerequisites, while 15-6 seems not necessary as it is designed in Rel-16 for inter-frequency case. Moreover, the FG-15-6 is per BC, i.e., a finer granularity than 47-s1, thus it should not be the prerequisite of 47-s1 according to RAN2’s guidelines [2].

Proposal 7: FG 47-s1 is reported with a prerequisite of FG 15-3 and 15-11.

On the other hand, dynamic resource pool sharing is not the only solution for co-channel coexistence between LTE SL and NR SL. It is concluded that TDM-based semi-static resource pool partitioning can be used for co-channel coexistence. It should be noted that co-channel coexistence between LTE and NR SL is not a basic SL UE feature. Even in rel-16, the support of in-device coexistence is optional. Moreover, co-channel coexistence is not the basic assumption of in-device coexistence. Instead, separate operating channels between RATs are assumed, as, e.g., described in the TR 37.985 [3], where frequency spacing is always assumed between LTE and NR SL:

It is envisaged that there will exist devices that support both LTE-V2X and NR-V2X, and which will be operating in both systems concurrently. If the two RATs are widely spaced in frequency, e.g. being in different bands, then there need be no particular issues to consider since it is assumed that a separate RF chain will be provided for each band. If, however, a sufficiently close frequency spacing is deployed, then it is desirable to enable a single RF chain to be used in the implementation, and also to adhere to the sidelink half-duplex principle established in LTE-V2X, i.e. that the UE is not required to simultaneously transmit and receive on sidelink. The former constraint means that interference between the two RATs' receptions can occur in the device if they are placed sufficiently close together in the frequency domain, and that simultaneous transmission on both RATs is prevented by the UE's single power budget. The latter constraint implies that one RAT cannot be received/transmitted while the other RAT is doing the opposite.

Moreover, the co-channel coexistence of LTE and NR with different SCSes (i.e., assuming simultaneous transmission and reception using different SCSes in the same channel for a SL UE), is not considered even for TDM-based semi-static resource pool partitioning.

Therefore, a UE cannot be assumed to mandatorily support the TDM-based co-channel coexistence. A separate UE capability should be defined to indicate whether the UE supports TDM-based semi-static resource pool partitioning for co-channel coexistence.

Proposal 8: A capability of TDM-based semi-static resource pool partitioning for co-channel coexistence of LTE sidelink and NR sidelink with different SCS(es), e.g., 15kHz SCS for LTE SL and 30kHz SCS for NR SL, is introduced.

A list of the detailed UE capabilities for co-channel coexistence including the updates to the agreed FG is provided in Annex.

  - Sidelink CA operation

In this section, the UE features for sidelink CA operation are discussed.

In the RAN1#114bis meeting, several UE FGs, i.e., 47-v1, 47-v2, and 47-v3, were agreed for SL CA [1]. In our view, the UE FG should be defined clearly for each feature. A ‘Mega’ FG combining different features is not good for product implementation, nor future-proof for specification development. Thus, merging 47-v1, 47-v2, and 47-v3 is not preferable.

Proposal 9: FG 47-v1, FG 47-v2 and FG 47-v3 should be separated FGs.

Regarding 47-v1, considering that in the case of unicast, whether CA is supported needs to be known between peer UEs, this FG should also be reported to UE. Regarding the prerequisites, in our view, the CA capability can be standalone without prerequisites. The current candidate prerequisites, e.g., [15-3, 15-11], seem to preclude the UE using partial-sensing/random-selection to support CA, as the partial-sensing/random-selection UE may not support 15-3 or 15-11. Such a restriction is not desirable. If the prerequisites are really necessary, the acceptable compromise could be one of FG {15-3, 15-11, 32-4, 32-4a}.

Proposal 10: FG 47-v1 should be reported between UEs. and the candidate value of supported SL carriers can be {2, 3, 4, 5}.

Proposal 11: Prerequisite is not necessary for FG 47-v1. If it is really desirable, the acceptable compromise could be one of FG {15-3, 15-11, 32-4, 32-4a}.

Regarding the prerequisites of 47-v2, similar to the discussion above for 47-v1, additional prerequisites beyond 47-v1 is not necessary. Adding FG 15-4 may unnecessarily preclude the partial-sensing/random-selection UE to support CA, as this kind of UE is not required to support 15-4. The potential compromise could be one of FG {15-4, 32-2b, 32-4b}.

Proposal 12: Additional prerequisite beyond 47-v1 is not necessary for FG 47-v2. If it is really desirable, the acceptable compromise could be one of FG {15-4, 32-2b, 32-4b}.

Regarding 47-v3, one remaining issue is the candidate number of PSFCH transmission and reception, i.e., X and Y. For the single carrier case, a UE can report up to M={4, 8, 16} PSFCH transmissions and up to N={5, 15, 25, 32, 35, 45, 50, 64} PSFCH receptions. In the CA case, up to K={2, 3, 4, 5, 6, 7, 8} carriers can be supported. Thus, the candidate number of X and Y can be X=K*N, Y=K*M, where the value K is the number of SL carriers that the UE supports.

Proposal 13: For FG 47-v3, the candidate number of PSFCH receptions X and PSFCH transmission Y can be X=K*N, Y=K*M, where the value K is the number of SL carriers that the UE supports.

Regarding the prerequisites of 47-v2, additional prerequisites beyond 47-v1 are not necessary.

Proposal 14: Additional prerequisite beyond 47-v1 is not necessary for FG 47-v3.

Additionally, in Rel-16, the capabilities of simultaneous transmission and reception of Uu and SL are defined for single carrier cases. When CA is supported for SL and these capabilities are reported by a UE, it would be confusing whether the simultaneous transmission/reception capabilities include a single carrier or multiple carriers. To avoid any potential confusion, it is better to introduce the corresponding capabilities for CA.

- Simultaneous transmission of uplink and sidelink for CA operation

- Simultaneous reception of downlink and sidelink for CA operation

Proposal 15: A capability of simultaneous transmission of uplink and sidelink for CA operation is introduced.

Proposal 16: A capability of simultaneous reception of downlink and sidelink for CA operation is introduced.

A list of the detailed UE capabilities for CA including the updates to the agreed FG is provided in Annex.

- Conclusion

In the contribution, we provide our view on UE features for Rel-18 NR sidelink evolution, with the following proposals. The list of the detailed UE capabilities is provided in Annex.

Proposal 1: The UE capability 47-k9 should be introduced to indicate the support of NR SL mode-1 scheduling SL transmission on an unlicensed band.

Proposal 2: The UE capability 47-k8 is introduced for CW autonomous update of SL transmission without HARQ feedback is introduced.

Proposal 3: The UE capability 47-m13 can be introduced for the PSFCH transmission scheme with 1 common interlace and K3 dedicated PRB(s).

Proposal 4: It is not necessary to introduce Rx capabilities for S-SSB reception on additional time and/or frequency occasions.

Proposal 5: The UE FG 47-k2, 47-k3, 47-k4, and 47-m4 should be reported to peer UE.

Proposal 6: The UE FG 47-k3, 47-k4, 47-k9, 47-m10, 47-m11, 47-m11a, 47-m12, and 47-m12a should be reported to the network.

Proposal 7: FG 47-s1 is reported with a prerequisite of FG 15-3 and 15-11.

Proposal 8: A capability of TDM-based semi-static resource pool partitioning for co-channel coexistence of LTE sidelink and NR sidelink with different SCS(es), e.g., 15kHz SCS for LTE SL and 30kHz SCS for NR SL, is introduced.

Proposal 9: FG 47-v1, FG 47-v2 and FG 47-v3 should be separated FGs.

Proposal 10: FG 47-v1 should be reported between UEs. and the candidate value of supported SL carriers can be {2, 3, 4, 5}.

Proposal 11: Prerequisite is not necessary for FG 47-v1. If it is really desirable, the acceptable compromise could be one of FG {15-3, 15-11, 32-4, 32-4a}.

Proposal 12: Additional prerequisite beyond 47-v1 is not necessary for FG 47-v2. If it is really desirable, the acceptable compromise could be one of FG {15-4, 32-2b, 32-4b}.

Proposal 13: For FG 47-v3, the candidate number of PSFCH receptions X and PSFCH transmission Y can be X=K*N, Y=K*M, where the value K is the number of SL carriers that the UE supports.

Proposal 14: Additional prerequisite beyond 47-v1 is not necessary for FG 47-v3.

Proposal 15: A capability of simultaneous transmission of uplink and sidelink for CA operation is introduced.

Proposal 16: A capability of simultaneous reception of downlink and sidelink for CA operation is introduced.

- Reference

- R1-2312572, “Updated RAN1 UE features list for Rel-18 NR after RAN1#115”, Moderators (AT&T, NTT DOCOMO, INC.), RAN1 meeting #115.

- R2-2306810, “Further Guidelines on UE capability definitions”, RAN2.

- 3GPP TR 37.985, “Overall description of Radio Access Network (RAN) aspects for Vehicle-to-everything (V2X) based on LTE and NR”, V17.1.1 (2022-03).

- RP-230077, “WID revision: NR sidelink evolution”, OPPO, RAN meeting #99.

Annex.

Updated Rel-18 NR SL UE feature list

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-k1 | SL channel access for dynamic channel access mode | UE supports 1. SL Type 1 channel access and contention window size adjustment 2. SL Type 2A channel access 3. SL Type 2B channel access 4. SL Type 2C channel access 5. 20MHz LBT bandwidth 6. CP extension up to 1 symbol in 15kHz SCS if the UE supports 15 kHz SCS 7. CP extension up to 2 symbols in 30kHz SCS 8. CP extension up to 2 symbols if the UE supports 60kHz SCS [9. Monitoring SCI to decode] FFS whether to merge with 47-k3 | At least one of {15-25, 15-3, [32-4, 32-4a]} | Yes | No | UE does not support channel access for NR sidelink operation in shared spectrum. | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. Note: Component 8 is applicable in regions without OCB requirements. Note1: If UE supports 15-25, the UE is not required to support Component 3 and 4 in 15-2. Note2: If UE supports 15-3, the UE is not required to support Component 3 in 15-3, and FR2 parts of Component 7 in 15-3. Note: It is up to RAN2 whether/how to implement the above Notes 1/2 and whether/how to update the prerequisite FGs | Optional with capability signalling For UE supports NR SL in unlicensed spectrum and when shared spectrum channel access must be used, UE must indicate this FG is supported |
| 47. NR_SL_enh2 | 47- k2 | SL multi-channel access for dynamic channel access mode | 1. UE supports multi-channel access procedures for PSCCH/PSSCH/S-SSB/PSFCH transmission(s) in multiple RB sets in a slot 2. UE supports Type A and Type B multi-channel access procedures for PSFCH transmissions in multiple RB sets in a slot 4) UE supports multi-channel access procedure on N channel(s) with 20MHz LBT bandwidth for each channel. Candidate values of N: {2, 3, 4, 5} | 47-k1 | Yes | [No] | UE does not support multi-channel access in dynamic channel access mode for NR sidelink operation in shared spectrum. | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. Note: Support of S-SSB/PSFCH transmission(s) in multiple RB-sets in a slot is according to the support of {47-m11, 47-m11a} and {47-m12, 47-m12a} | Optional with capability signalling |
| 47. NR_SL_enh2 | 47- k3 | Receiving UE to UE COT sharing information | 1. UE supports monitoring SCI to read COT sharing information 2. UE supports transmitting NR SL based on COT sharing information subject to COT sharing conditions FFS whether to merge with 47-k1 | 47-k1 | No | [No] | UE does not support using UE-to-UE COT sharing information contained in SCI for sharing COT for NR sidelink operation in shared spectrum. | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | [Optional with capability signalling] [For UE supports NR SL in unlicensed spectrum where shared spectrum channel access must be used, UE must indicate this FG is supported] |
| 47. NR_SL_enh2 | 47-k4 | Transmitting UE to UE COT sharing information | 1. UE supports using ue-toUE-COT-SharingED-Threshold for Type 1 channel access for UE to UE COT sharing 2. UE supports indicating COT sharing information in SCI | 47-k1 | No | [No] | UE does not support transmitting UE-to-UE COT sharing information for sharing COT for NR sidelink operation in shared spectrum. | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k5 | Resource allocation for multi-consecutive slots transmission | UE supports resource (re-)selection for PSCCH/PSSCH transmission on multiple consecutive slots | TBD | [Yes] | [No] | UE does not support resource (re-)selection for multi-consecutive slots transmission | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k6 | Type1 LBT blocking Option 1 | UE supports 1. avoid selection of N consecutive resource(s) before a reserved resource when the L1 SL priority value for the transmission is higher than the L1 SL priority value of the reserved resource. It is up to UE whether to do it 2. avoid selection of M consecutive resource(s) after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource. It is up to UE whether to do it | 47-k1 | Yes | No | UE does not support Type1 LBT blocking Option 1 | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k7 | Type1 LBT blocking Option 2 | UE supports 1. If transmission in slot(s) at least T_proc,0 before a reserved resource is able to share its initiated COT to the reservation, UE prioritize / select resource(s) in the slot(s) for transmission. It is up to UE whether to do it | 47-k1 | Yes | No | UE does not support Type1 LBT blocking Option 2 | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k8 | CW autonomous update for SL transmission without HARQ feedback | UE support autonomous update of the CWp to the next higher allowed value when the same CWp≠CWmax,p value is consecutively used for X times for generation of Ninit for PSCCH/PSSCH transmission without HARQ feedback. FFS whether to merge with 47-k1 | TBD | No | No | UE does not update CWp for PSCCH/PSSCH transmission without HARQ feedback. | Per band | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k9 | Sidelink mode 1 resource allocation in shared spectrum | 1. UE can monitor DCI format 3_0 on a licensed band for NR sidelink dynamic scheduling and configured grant type 2 for transmitting interlaced RB-based PSCCH/PSSCH on a shared band] 2. UE supports reporting NACK to gNB when transmitting PSCCH/PSSCH on scheduled resource(s) is failed due to LBT failure | TBD | Yes | No | UE does not perform interlaced RB-based PSCCH/PSSCH based on mode 1 resource allocation. | Per band | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m1 | Interlace RB-based SL transmission/reception | 1. UE supports interlace RB-based SL transmissions for the physical layer channels that it is capable of transmit 2. UE supports interlace RB-based SL receptions for the physical layer channels that it is capable of receive | 47-k1, at least one of [15-25 except Component 3 and 4 in 15-2] and [15-3 except Component 3] | Yes | No | UE does not support Interlace RB-based PSCCH/PSSCH/PSFCH transmission/reception | Per band | N/A | N/A |  | This is the basic FG for NR sidelink in shared spectrum, where PSD and/or OCB requirements are defined by regulation. | Optional with capability signalling For UE supports NR sidelink in shared spectrum, where PSD and/or OCB requirements are defined by regulation, UE must indicate this FG is supported. |
| 47. NR_SL_enh2 | 47-m3 | Transmitting PSCCH/PSSCH from 2nd starting symbol in a slot | 1. UE supports transmitting PSCCH/PSSCH from 2nd starting symbol in a slot in addition to the first starting symbol | 47-k1, at least one of [15-25 except Component 3 and 4 in 15-2] and [15-3 except Component 3] | No | No | UE transmits PSCCH/PSSCH only from 1st starting symbol in a slot | Per band | N/A | N/A |  |  | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-m4 | Receiving PSCCH/PSSCH from 2nd starting symbol in a slot | 1. UE supports receiving PSCCH/PSSCH transmitted from 2nd starting symbol in a slot in addition to the first starting symbol 2. UE can monitor a total up to X PSCCHs in a slot in the 1st and 2nd starting symbols | [15-1 except Component 5] | No | No | UE receives PSCCH/PSSCH transmitted only from 1st starting symbol in a slot | Per band | N/A | N/A |  | [This is the basic FG for NR sidelink in unlicensed spectrum.] The value X is the same as the reported value in FG 15-1 | [Optional without capability signalling] [For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported.] |
| 47. NR_SL_enh2 | 47-m5 | Multiple PSFCH occasions per PSCCH/PSSCH | 1. UE supports PSFCH transmission/reception on N PSFCH occasion(s) per PSCCH/PSSCH | 15-11, 47-k1 | Yes | No | UE supports only one PSFCH occasion per PSCCH/PSSCH transmission | Per band | N/A | N/A |  | Candidate values for N are {1,2,3,4} The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m6 | Transmitting SSB repetitions within one RB set | 1. UE supports transmitting S-PSS/S-SSS/PSBCH multiple times by repetition in frequency domain within one RB set FFS whether to define the supported maximum number of repetitions in frequency domain within one RB set FFS whether/how to introduce Rx capability | 15-4, 47-k1 | No | No | UE does not support transmitting S-PSS/S-SSS/PSBCH multiple times by repetition in frequency domain within one RB set | Per band | N/A | N/A |  | This is the basic FG for NR sidelink in shared spectrum where PSD and/or OCB requirements are defined by regulation. [It is up to UE implementation whether S-SSB RX UE monitors more than one S-SSB repetition in frequency domain within one RB set as long as RAN4 requirements are satisfied] [The signaling is only expected for a band where shared spectrum channel access must be used.] | [Optional without capability signalling] For UE supports NR sidelink in unlicensed spectrum where PSD and/or OCB requirements are defined by regulation, UE must support this FG. |
| 47. NR_SL_enh2 | 47-m8 | Transmitting S-SSB on additional S-SSB occasion(s) | 1. UE supports transmitting S-SSB on additional S-SSB occasion(s) | 15-4, 47-k1 | No | No | UE does not support transmitting S-SSB on additional S-SSB occasion(s) but supports to exclude those occasion(s) from SL resource pool(s) | Per band | N/A | N/A |  |  | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-m9 | Receiving S-SSB on additional S-SSB occasion(s) | 1. UE supports receiving S-SSB on additional S-SSB occasion(s) |  | No | No | UE does not support receiving S-SSB on additional S-SSB occasion(s) but supports to exclude those occasion(s) from SL resource pool(s) | Per band | N/A | N/A |  |  | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-m10 | Contiguous RB-based PSCCH/PSSCH transmission | 1. UE supports contiguous RB-based PSCCH/PSSCH transmission 2. UE supports resource (re-)selection for contiguous RB-based PSCCH/PSSCH transmission FFS whether/how to define the capabilities for reception and/or PSFCH | TBD | [Yes] | [No] | UE does not support contiguous RB-based PSCCH/PSSCH transmission | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m11 | PSFCH transmissions in multiple contiguous RB sets | UE supports PSFCH transmissions in multiple contiguous RB sets | TBD | [No] | [No] | UE does not support PSFCH transmissions in multiple contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m11a | PSFCH transmissions in multiple non-contiguous RB sets | UE supports PSFCH transmissions in multiple non-contiguous RB sets | TBD | [No] | [No] | UE does not support PSFCH transmissions in multiple non-contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m12 | S-SSB transmissions in multiple contiguous RB sets | UE supports S-SSB transmissions in multiple contiguous RB sets | TBD | [No] | [No] | UE does not support S-SSB transmissions in multiple contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m12a | S-SSB transmissions in multiple non-contiguous RB sets | UE supports S-SSB transmissions in multiple non-contiguous RB sets | TBD | [No] | [No] | UE does not support S-SSB transmissions in multiple non-contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m13 | Transmissions/receptions of multiple dedicated PRBs in interlace-based PSFCH | 1. UE can transmit PSFCH(s) on up to a total of K dedicated PRBs in a slot. 2. UE can receive PSFCH(s) on up to a total of L dedicated PRBs in a slot | TBD | No | No | UE does not support multiple transmissions/receptions of common interlace-based PSFCH. | Per band | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. Candidate values for K are FFS Candidate values for L are FFS | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-s1 | Transmission/Reception using dynamic resource pool sharing | 1) Avoidance of NR PSCCH/PSSCH/PSFCH overlapping with EUTRA SL resources in dynamic resource pool sharing using LTE sidelink resource reservation information in NR mode2 resource (re)selection 2) UE supports NR sidelink TXs and RXs in a resource pool in 15kHz and 30kHz SCSs and uses the SCS that is (pre)configured for a SL BWP. | [15-3, 15-6, 15-11] | Yes | No | UE does not support transmission/reception using dynamic resource pool sharing | Per band | N/A | N/A |  | Component 2 does not imply that two different SCSs can be (pre)configured simultaneously in a SL BWP | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-v1 | NR SL communication with SL CA | 1) UE supports transmitting/receiving PSCCH/PSSCH/PSFCH simultaneously over multiple X SL carriers: - 1-1) Maximum number of simultaneous PSCCH/PSSCH TX, equal to X and 1 per carrier - 1-2) For the number of PSCCH decodes: - UE can receive Z* floor (NRB,i /10 RBs) PSCCH in a slot on carrier i of the X carriers. - FFS whether to report Maximum number of non-overlapping RBs per slot across all carriers the UE can attempt to decode 2) UE can adjust the transmission power of the PSCCH/PSSCH/PSFCH across aggregated carriers such that its total transmission power does not exceed the maximum transmission power. FFS whether/how to merge FG for SL-CA | [15-3, 15-11] | Yes | No |  | Per band | N/A | N/A |  | Component 1: Candidate value of X = {2, 3, 4, 5, 6, 7, 8}, FFS on some BW restriction Component 1-2 candidate value set: Z={1, 2} NRB,i is the number of RBs defined per channel bandwidth of carrier i by RAN4 in 38.101-1 Table 5.3.2-1 for FR1 | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-v2 | Synchronization for SL CA | 1-1) UE supports transmitting S-SSB on one selected or all candidate synchronization carriers with the same sync reference from Set-B 1-2) UE supports receiving S-SSB from all candidate synchronization carriers with the same sync reference from Set-B 2) UE can adjust the transmission power of the S-SSB across aggregated carriers such that its total transmission power does not exceed the maximum transmission power. FFS whether/how to merge FG for SL-CA | 47-v1, [15-4] | Yes | No |  | Per band | N/A | N/A |  | Note: Option of UE selection of one selected SL synchronization carrier with the same sync reference from Set-B is not based on limited Tx capability Note: Component 1-2 does not require simultaneous reception of S-SSB on all candidate synchronization carriers with the same sync reference from Set-B | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-v3 | PSFCH for SL CA | 1) UE supports receiving X PSFCH resources in a slot over all aggregated SL carriers - 1-1) UE is capable of receiving at least one PSFCH resource on each of the aggregated carriers in a slot 2) UE supports transmitting Y PSFCH resources in a slot over all aggregated SL carriers according to PSFCH procedures - 2-1) UE is capable of transmitting at least one PSFCH resource on each of the aggregated carriers [3) UE can adjust the transmission number of PSFCH across aggregated carriers such that its total transmission number does not exceed the maximum transmission power.] FFS whether/how to merge FG for SL-CA | 47-v1, [15-11] | Yes | No |  | Per band | N/A | N/A |  | Candidate values for X are {FFS} Candidate values for Y are {FFS} Note: for component 1-1, it is up to UE implementation which PSFCH(s) to receive | Optional with capability signalling |

Proposed Rel-18 NR SL UE feature list

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47. NR_SL_enh2 | 47-k1 | SL channel access for dynamic channel access mode | UE supports 1. SL Type 1 channel access and contention window size adjustment 2. SL Type 2A channel access 3. SL Type 2B channel access 4. SL Type 2C channel access 5. 20MHz LBT bandwidth 6. CP extension up to 1 symbol in 15kHz SCS if the UE supports 15 kHz SCS 7. CP extension up to 2 symbols in 30kHz SCS 8. CP extension up to 2 symbols if the UE supports 60kHz SCS [9. Monitoring SCI to decode] FFS whether to merge with 47-k3 | At least one of {15-25, 15-3, [32-4, 32-4a]} | Yes | No | UE does not support channel access for NR sidelink operation in shared spectrum. | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. Note: Component 8 is applicable in regions without OCB requirements. Note1: If UE supports 15-25, the UE is not required to support Component 3 and 4 in 15-2. Note2: If UE supports 15-3, the UE is not required to support Component 3 in 15-3, and FR2 parts of Component 7 in 15-3. Note: It is up to RAN2 whether/how to implement the above Notes 1/2 and whether/how to update the prerequisite FGs | Optional with capability signalling For UE supports NR SL in unlicensed spectrum and when shared spectrum channel access must be used, UE must indicate this FG is supported |
| 47. NR_SL_enh2 | 47- k2 | SL multi-channel access for dynamic channel access mode | 1. UE supports multi-channel access procedures for PSCCH/PSSCH/S-SSB/PSFCH transmission(s) in multiple RB sets in a slot 2. UE supports Type A and Type B multi-channel access procedures for PSFCH transmissions in multiple RB sets in a slot 4) UE supports multi-channel access procedure on N channel(s) with 20MHz LBT bandwidth for each channel. Candidate values of N: {2, 3, 4, 5} | 47-k1 | Yes | [No] Yes | UE does not support multi-channel access in dynamic channel access mode for NR sidelink operation in shared spectrum. | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. Note: Support of S-SSB/PSFCH transmission(s) in multiple RB-sets in a slot is according to the support of {47-m11, 47-m11a} and {47-m12, 47-m12a} | Optional with capability signalling |
| 47. NR_SL_enh2 | 47- k3 | Receiving UE to UE COT sharing information | 1. UE supports monitoring SCI to read COT sharing information 2. UE supports transmitting NR SL based on COT sharing information subject to COT sharing conditions FFS whether to merge with 47-k1 | 47-k1 | No Yes | [No] Yes | UE does not support using UE-to-UE COT sharing information contained in SCI for sharing COT for NR sidelink operation in shared spectrum. | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | [Optional with capability signalling] [For UE supports NR SL in unlicensed spectrum where shared spectrum channel access must be used, UE must indicate this FG is supported] |
| 47. NR_SL_enh2 | 47-k4 | Transmitting UE to UE COT sharing information | 1. UE supports using ue-toUE-COT-SharingED-Threshold for Type 1 channel access for UE to UE COT sharing 2. UE supports indicating COT sharing information in SCI | 47-k1 | No Yes | [No] Yes | UE does not support transmitting UE-to-UE COT sharing information for sharing COT for NR sidelink operation in shared spectrum. | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k5 | Resource allocation for multi-consecutive slots transmission | UE supports resource (re-)selection for PSCCH/PSSCH transmission on multiple consecutive slots | TBD | [Yes] No | [No] | UE does not support resource (re-)selection for multi-consecutive slots transmission | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k6 | Type1 LBT blocking Option 1 | UE supports 1. avoid selection of N consecutive resource(s) before a reserved resource when the L1 SL priority value for the transmission is higher than the L1 SL priority value of the reserved resource. It is up to UE whether to do it 2. avoid selection of M consecutive resource(s) after a reserved resource when the transmitting symbols of the reserved resource overlap with LBT of the selected resource. It is up to UE whether to do it | 47-k1 | Yes | No | UE does not support Type1 LBT blocking Option 1 | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k7 | Type1 LBT blocking Option 2 | UE supports 1. If transmission in slot(s) at least T_proc,0 before a reserved resource is able to share its initiated COT to the reservation, UE prioritize / select resource(s) in the slot(s) for transmission. It is up to UE whether to do it | 47-k1 | Yes | No | UE does not support Type1 LBT blocking Option 2 | Per band | n/a | n/a |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k8 | CW autonomous update for SL transmission without HARQ feedback | UE support autonomous update of the CWp to the next higher allowed value when the same CWp≠CWmax,p value is consecutively used for X times for generation of Ninit for PSCCH/PSSCH transmission without HARQ feedback. FFS whether to merge with 47-k1 | TBD | No | No | UE does not update CWp for PSCCH/PSSCH transmission without HARQ feedback. | Per band | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-k9 | Sidelink mode 1 resource allocation in shared spectrum | 1. UE can monitor DCI format 3_0 on a licensed band for NR sidelink dynamic scheduling and configured grant type 2 for transmitting interlaced RB-based PSCCH/PSSCH on a shared band] 2. UE supports reporting NACK to gNB when transmitting PSCCH/PSSCH on scheduled resource(s) is failed due to LBT failure | TBD | Yes | No | UE does not perform interlaced RB-based PSCCH/PSSCH based on mode 1 resource allocation. | Per band | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m1 | Interlace RB-based SL transmission/reception | 1. UE supports interlace RB-based SL transmissions for the physical layer channels that it is capable of transmit 2. UE supports interlace RB-based SL receptions for the physical layer channels that it is capable of receive | 47-k1, at least one of [15-25 except Component 3 and 4 in 15-2] and [15-3 except Component 3] | Yes | No | UE does not support Interlace RB-based PSCCH/PSSCH/PSFCH transmission/reception | Per band | N/A | N/A |  | This is the basic FG for NR sidelink in shared spectrum, where PSD and/or OCB requirements are defined by regulation. | Optional with capability signalling For UE supports NR sidelink in shared spectrum, where PSD and/or OCB requirements are defined by regulation, UE must indicate this FG is supported. |
| 47. NR_SL_enh2 | 47-m3 | Transmitting PSCCH/PSSCH from 2nd starting symbol in a slot | 1. UE supports transmitting PSCCH/PSSCH from 2nd starting symbol in a slot in addition to the first starting symbol | 47-k1, at least one of [15-25 except Component 3 and 4 in 15-2] and [15-3 except Component 3] | No | No | UE transmits PSCCH/PSSCH only from 1st starting symbol in a slot | Per band | N/A | N/A |  |  | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-m4 | Receiving PSCCH/PSSCH from 2nd starting symbol in a slot | 1. UE supports receiving PSCCH/PSSCH transmitted from 2nd starting symbol in a slot in addition to the first starting symbol 2. UE can monitor a total up to X PSCCHs in a slot in the 1st and 2nd starting symbols | [15-1 except Component 5] | No | No Yes | UE receives PSCCH/PSSCH transmitted only from 1st starting symbol in a slot | Per band | N/A | N/A |  | [This is the basic FG for NR sidelink in unlicensed spectrum.] The value X is the same as the reported value in FG 15-1 | [Optional without capability signalling] [For UE supports NR sidelink in unlicensed spectrum, UE must indicate this FG is supported.] |
| 47. NR_SL_enh2 | 47-m5 | Multiple PSFCH occasions per PSCCH/PSSCH | 1. UE supports PSFCH transmission/reception on N PSFCH occasion(s) per PSCCH/PSSCH | 15-11, 47-k1 | Yes | No | UE supports only one PSFCH occasion per PSCCH/PSSCH transmission | Per band | N/A | N/A |  | Candidate values for N are {1,2,3,4} The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m6 | Transmitting SSB repetitions within one RB set | 1. UE supports transmitting S-PSS/S-SSS/PSBCH multiple times by repetition in frequency domain within one RB set FFS whether to define the supported maximum number of repetitions in frequency domain within one RB set FFS whether/how to introduce Rx capability | 15-4, 47-k1 | No | No | UE does not support transmitting S-PSS/S-SSS/PSBCH multiple times by repetition in frequency domain within one RB set | Per band | N/A | N/A |  | This is the basic FG for NR sidelink in shared spectrum where PSD and/or OCB requirements are defined by regulation. [It is up to UE implementation whether S-SSB RX UE monitors more than one S-SSB repetition in frequency domain within one RB set as long as RAN4 requirements are satisfied] [The signaling is only expected for a band where shared spectrum channel access must be used.] | [Optional without capability signalling] For UE supports NR sidelink in unlicensed spectrum where PSD and/or OCB requirements are defined by regulation, UE must support this FG. |
| 47. NR_SL_enh2 | 47-m8 | Transmitting S-SSB on additional S-SSB occasion(s) | 1. UE supports transmitting S-SSB on additional S-SSB occasion(s) | 15-4, 47-k1 | No | No | UE does not support transmitting S-SSB on additional S-SSB occasion(s) but supports to exclude those occasion(s) from SL resource pool(s) | Per band | N/A | N/A |  |  | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-m9 | Receiving S-SSB on additional S-SSB occasion(s) | 1. UE supports receiving S-SSB on additional S-SSB occasion(s) |  | No | No | UE does not support receiving S-SSB on additional S-SSB occasion(s) but supports to exclude those occasion(s) from SL resource pool(s) | Per band | N/A | N/A |  |  | Optional without capability signalling |
| 47. NR_SL_enh2 | 47-m10 | Contiguous RB-based PSCCH/PSSCH transmission | 1. UE supports contiguous RB-based PSCCH/PSSCH transmission 2. UE supports resource (re-)selection for contiguous RB-based PSCCH/PSSCH transmission FFS whether/how to define the capabilities for reception and/or PSFCH | TBD | [Yes] | [No] | UE does not support contiguous RB-based PSCCH/PSSCH transmission | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m11 | PSFCH transmissions in multiple contiguous RB sets | UE supports PSFCH transmissions in multiple contiguous RB sets | TBD | [No] Yes | [No] | UE does not support PSFCH transmissions in multiple contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m11a | PSFCH transmissions in multiple non-contiguous RB sets | UE supports PSFCH transmissions in multiple non-contiguous RB sets | TBD | [No] Yes | [No] | UE does not support PSFCH transmissions in multiple non-contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m12 | S-SSB transmissions in multiple contiguous RB sets | UE supports S-SSB transmissions in multiple contiguous RB sets | TBD | [No] Yes | [No] | UE does not support S-SSB transmissions in multiple contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m12a | S-SSB transmissions in multiple non-contiguous RB sets | UE supports S-SSB transmissions in multiple non-contiguous RB sets | TBD | [No] Yes | [No] | UE does not support S-SSB transmissions in multiple non-contiguous RB sets | [Per band] | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-m13 | Transmissions/receptions of multiple dedicated PRBs in interlace-based PSFCH | 1. UE can transmit PSFCH(s) on up to a total of K dedicated PRBs in a slot. 2. UE can receive PSFCH(s) on up to a total of L dedicated PRBs in a slot | TBD | [No] Yes | No | UE does not support multiple transmissions/receptions of common interlace-based PSFCH. | Per band | N/A | N/A |  | The signaling is only expected for a band where shared spectrum channel access must be used. Candidate values for K are FFS Candidate values for L are FFS | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-s1 | Transmission/Reception using dynamic resource pool sharing | 1) Avoidance of NR PSCCH/PSSCH/PSFCH overlapping with EUTRA SL resources in dynamic resource pool sharing using LTE sidelink resource reservation information in NR mode2 resource (re)selection 2) UE supports NR sidelink TXs and RXs in a resource pool in 15kHz and 30kHz SCSs and uses the SCS that is (pre)configured for a SL BWP. | [15-3, 15-6, 15-11] | Yes | No | UE does not support transmission/reception using dynamic resource pool sharing | Per band | N/A | N/A |  | Component 2 does not imply that two different SCSs can be (pre)configured simultaneously in a SL BWP | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-s2 | TDM-based semi-static resource pool partitioning for co-channel coexistence of LTE sidelink and NR sidelink with mix SCSes | - UE supports TDM-based semi-static resource pool partitioning for co-channel coexistence between LTE sidelink and NR sidelink with 15 kHz SCS and/or 30kHz SCSs. Candidate value sets: {[15KHz, 30kHz, both]}. - Combination A (Mode 2 NR SL with Mode 4 LTE SL) is supported. - Device type A (the NR SL module uses the sensing and resource reservation information shared by the LTE SL module) is supported. | None | Yes | No |  | Per band | N/A | N/A |  |  | Optional with capability signalling. |
| 47. NR_SL_enh2 | 47-v1 | NR SL communication with SL CA | 1) UE supports transmitting/receiving PSCCH/PSSCH/PSFCH simultaneously over multiple X SL carriers: - 1-1) Maximum number of simultaneous PSCCH/PSSCH TX, equal to X and 1 per carrier - 1-2) For the number of PSCCH decodes: - UE can receive Z* floor (NRB,i /10 RBs) PSCCH in a slot on carrier i of the X carriers. - FFS whether to report Maximum number of non-overlapping RBs per slot across all carriers the UE can attempt to decode 2) UE can adjust the transmission power of the PSCCH/PSSCH/PSFCH across aggregated carriers such that its total transmission power does not exceed the maximum transmission power. FFS whether/how to merge FG for SL-CA | [15-3, 15-11] N/A or one of {15-3, 15-11, 32-4, 32-4a} | Yes | No Yes |  | Per band | N/A | N/A |  | Component 1: Candidate value of X = {2, 3, 4, 5, 6, 7, 8}, FFS on some BW restriction Component 1-2 candidate value set: Z={1, 2} NRB,i is the number of RBs defined per channel bandwidth of carrier i by RAN4 in 38.101-1 Table 5.3.2-1 for FR1 | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-v2 | Synchronization for SL CA | 1-1) UE supports transmitting S-SSB on one selected or all candidate synchronization carriers with the same sync reference from Set-B 1-2) UE supports receiving S-SSB from all candidate synchronization carriers with the same sync reference from Set-B 2) UE can adjust the transmission power of the S-SSB across aggregated carriers such that its total transmission power does not exceed the maximum transmission power. FFS whether/how to merge FG for SL-CA | 47-v1, [15-4] or one of {one of FG {15-3, 15-11, 32-4, 32-4a} | Yes | No |  | Per band | N/A | N/A |  | Note: Option of UE selection of one selected SL synchronization carrier with the same sync reference from Set-B is not based on limited Tx capability Note: Component 1-2 does not require simultaneous reception of S-SSB on all candidate synchronization carriers with the same sync reference from Set-B | Optional with capability signalling |
| 47. NR_SL_enh2 | 47-v3 | PSFCH for SL CA | 1) UE supports receiving X PSFCH resources in a slot over all aggregated SL carriers - 1-1) UE is capable of receiving at least one PSFCH resource on each of the aggregated carriers in a slot 2) UE supports transmitting Y PSFCH resources in a slot over all aggregated SL carriers according to PSFCH procedures - 2-1) UE is capable of transmitting at least one PSFCH resource on each of the aggregated carriers [3) UE can adjust the transmission number of PSFCH across aggregated carriers such that its total transmission number does not exceed the maximum transmission power.] FFS whether/how to merge FG for SL-CA | 47-v1, [15-11] | Yes | No |  | Per band | N/A | N/A |  | Candidate values for X are {FFS} K*{5, 15, 25, 32, 35, 45, 50, 64}, where the value K is the number of SL carriers that the UE supports Candidate values for Y are {FFS} K*{4, 8, 16}, where the value K is the number of SL carriers that the UE supports Note: for component 1-1, it is up to UE implementation which PSFCH(s) to receive | Optional with capability signalling |