3GPP TSG RAN WG1 #116	R1-2400254

Athens, Greece, February 26th – March 1st, 2024

Source:	vivo

Title:	Discussion on LP-WUS operation in IDLE/INACTIVE modes

Agenda Item:	9.6.2

Document for:	Discussion and Decision

- Introduction

In RAN#102 meeting, the following objectives of R19 LP-WUS work item have been agreed [1].

- To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - This objective is to be further refined in RAN#103

Based on the R19 LP-WUS WID, we focus on the discussion for LP-WUS operation in RRC idle/inactive mode in this contribution including the procedure and the configurations of LP-WUS monitoring.

- Procedure of LP-WUS indicating paging monitoring

  - UE LP-WUS monitoring behavior

In R17 PEI discussion, the specified UE behaviour for PEI monitoring is: PEI indicates UE should monitor a PO if UE’s group/subgroup is paged. And UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO.

For the UE behaviour of LP-WUS monitoring, we can comply with the same UE behaviour of R17 PEI for the sake of saving LP-WUS resource overhead. But having to say, since LP-WUS may not achieve the same detection performance as that of PDCCH based PEI, the case that UE fails to detect LP-WUS due to out of LP-WUS coverage should be separately considered as provided in section 2.4 of the entry/exit condition for LP-WUS monitoring. Moreover, the UE behaviour both considering LP-WUS and PEI could be further discussed if any benefit can be verified.

Proposal 1: Support the following UE behavior for LP-WUS monitoring:

- UE needs to monitor PO, if UE’s group/subgroup is woken up by LP-WUS.

- UE is not required to monitor a PO, if UE does not detect LP-WUS at all LP-WUS occasion(s) for the PO.

- FFS: if PEI is also configured.

  - Subgrouping indication of LP-WUS

With larger number of subgroups carried by LP-WUS i.e., the finer the granularity of the subgrouping., false alarm rate caused by other UEs in the same group will be better reduced and more power saving gain can be achieved as well. To achieve similar or better wake-up granularity as PEI, at least 8 subgroups are required for LP-WUS design. And regarding how many subgroup indication bits can be carried by LP-WUS which highly depends on the subgrouping method, signal structure and required SNR to achieve desirable coverage of LP-WUS will be discussed in A.I. 9.6.1 in detail [2].

Besides, as analyzed before, in RRC idle/inactive mode, the network cannot be aware of whether UE does LP-WUS monitoring or not, that is, it is up to UE implementation to monitor LP-WUS and/or PEI. In this regard, the subgrouping indication of LP-WUS should be separately designed without in conjunction with PEI. That is, LP-WUS subgroup is a further grouping of the PO group.

Proposal 2: Support no less than 8 subgroups per PO carried by LP-WUS.

  - Entry/exit condition for LP-WUS monitoring

According to the discussion in R18 LP-WUS SI, the network cannot be aware of whether a certain UE is monitoring LP WUS or not in RRC idle/inactive mode. And it is also difficult for the network to be aware of the UE’s channel condition, e.g., whether the UE is in LP-WUS coverage or not. For example, when the UE is out of LP-WUS coverage, UE should exit the LP-WUS monitoring and fallback to monitor PEI or PO. When the UE is enters the LP-WUS coverage again, the UE can resume/activate the LP-WUS monitoring again. Therefore, similar as for R17 PEI monitoring, for RRC idle/inactive mode, the entry/exit of UE LP-WUS monitoring is transparent to gNB, i.e. explicitly signaling is not necessary.

However, there are some differences between PEI and LP-WUS. PDCCH-based PEI is detected by UE’s main radio and the detection performance of PEI is the same as that of paging PDCCH. So, there is no need for the network to warry about the coverage of PEI. While for LP-WUS which is received by LP-WUR, the detection and coverage performance are totally different. Some criteria need to be preconfigured so that it can help UE decide to enter or exit LP-WUS monitoring. In this way, UE can distinguish between the cases that it is out of coverage and no LP-WUS is transmitted by the network.

Proposal 3: For RRC idle/inactive mode, whether UE enters/exits LP-WUS monitoring is based on preconfigured criteria by the Network, i.e.

- If the entry criteria is satisfied, UE starts LP-WUS monitoring, and legacy PO is monitored only when UE is woken up by LP-WUS.

- If the exit criteria is satisfied, UE exits LP-WUS monitoring and fallback to legacy monitoring behavior e.g., UE performs PEI or PO monitoring.

Additionally, as in WID, RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR should be supported. Hence, when UE enters or exits LP-WUS monitoring, the UE behavior on performing RRM measurement should be considered. For example, RRM measurement via MR could be relaxed (including fully offload to LR) once UE enters LP-WUS monitoring. That is, the relaxation of RRM measurement via MR or offloading from MR to LR can use the same criteria with the entry criteria of LP-WUS monitoring. And UE will perform LR based RRM measurement when it enters LP-WUS monitoring. Similarly, the same criteria can apply for both resuming RRM measurement from LR to MR and the exit of LP-WUS monitoring.

Proposal 4: Support the following UE behavior on RRM measurement:

- If the entry criteria of LP-WUS monitoring is satisfied, UE performs LR RRM measurement;

- If the exit criteria of LP-WUS monitoring is satisfied, UE performs MR RRM measurement.

Besides, for the determination of the entry and exit criteria of LP-WUS monitoring, generally, there can be based on RRM measurement via MR and/or LR. Since before UE enters LP-WUS monitoring, RRM measurement by UE MR is performed, so sufficient serving cell quality measured at least based on MR needs to be guaranteed for the design of entry criteria. Besides, RRM measurement via LR could be further considered into the determination of the entry criteria if UE enables LR in advance to receive LP-SS before entering LP-WUS monitoring. And for the exit criteria, in case the serving cell quality based on LR RRM measurement is lower than a threshold during LP-WUS monitoring, UE can decide to exit the LP-WUS monitoring. Additionally, if the RRM measurement is not fully offloaded to LR i.e., UE still does relaxed MR RRM measurement, the exit criteria can also consider MR RRM measurement.

Proposal 5: The entry /exit criteria of LP-WUS monitoring can be based on MR and/or LR RRM measurement.

- Entry criteria: serving cell quality at least based on MR RRM measurement.

- FFS: also based on LR RRM measurement.

- Exit criteria: serving cell quality at least based on LR RRM measurement.

- FFS: also based on MR RRM measurement.

- LP-WUS monitoring configuration

  - Determination of LP-WUS offset and monitoring occasions

According to the discussion in R18 SI on the required wake-up delay including both ramp up time and sync time from detecting LP-WUS to be ready for PO monitoring [3], it is explicit that the of wake-up delay is vary from different UE capabilities. For example, for UE1, due to its high capability, the wake-up delay can be shortened to 400ms, while for UE2 with low capability, 1 second wake-up delay may be required. Hence, UE should report its required wake-up delay from woken up by LP-WUS to be ready for PO reception. And the network can determine the location of LP-WUS occasion accordingly.

Proposal 6: Support to report UE capability on wake-up delay, which is the time offset between woken up by LP-WUS and be ready for PO reception.

The basic functionality of LP-WUS is to trigger PO monitoring. Since the location of UE’s PO/PF is fixed, it will be efficient to monitor the LP-WUS only before its associated PO/PF. Similar as PEI configuration, it is natural to configure LP-WUS relative to PO/PF. Furthermore, considering the subgrouping functionality of LP-WUS, it will be more beneficial to configure LP-WUS relative to PO/PF, so that the LP-WUS can do further grouping based on the existing PO group. Also, the duty cycle of LP-WUS monitoring will be the same with paging cycle. For the offset between LP-WUS and PO/PF, ramp up time from ultra-deep sleep state and the sync time after woken up by LP-WUS needs to be considered.

For each LP-WUS duty cycle, within the LP-WUS monitoring window/duration, there can be multiple LP-WUS monitoring occasions (MO) considering multiple beam operation. And the offset between adjacent LP-WUS MOs can be fixed.

Proposal 7: Support to configure LP-WUS relative to PO/PF. And the offset between LP-WUS and PO/PF is decided by

- Ramp up time from ultra-deep sleep state.

- Sync time

  - Duty-cycled or continuous LP-WUS monitoring

As mentioned before, LP-WUS is assumed to be configured relative to PO/PF. Hence, if the time offset between LP-WUS monitoring and PO is equal to the required wake up delay of UE, the smallest latency can be achieved already. In this sense, it seems no need to configure continuous LP-WUS monitoring for RRC idle/inactive mode. Besides, considering continuous LP-WUS monitoring will potentially result in the risk of high false alarm, duty-cycled LP-WUS monitoring will be more effective for RRC idle/inactive mode.

Proposal 8: Support duty-cycled LP-WUS monitoring for RRC idle/inactive mode.

- Conclusion

In this contribution, the procedure of LP-WUS monitoring in RRC idle/inactive mode is discussion on the aspect of UE monitoring behavior, entry/exit condition, subgrouping indication and LP-WUS monitoring configuration etc., with the following proposals:

Proposal 1: Support the following UE behavior for LP-WUS monitoring:

- UE needs to monitor PO, if UE’s group/subgroup is woken up by LP-WUS.

- UE is not required to monitor a PO, if UE does not detect LP-WUS at all LP-WUS occasion(s) for the PO.

- FFS: if PEI is also configured.

Proposal 2: Support no less than 8 subgroups per PO carried by LP-WUS.

Proposal 3: For RRC idle/inactive mode, whether UE enters/exits LP-WUS monitoring is based on preconfigured criteria by the Network, i.e.

- If the entry criteria is satisfied, UE starts LP-WUS monitoring, and legacy PO is monitored only when UE is woken up by LP-WUS.

- If the exit criteria is satisfied, UE exits LP-WUS monitoring and fallback to legacy monitoring behavior e.g., UE performs PEI or PO monitoring.

Proposal 4: Support the following UE behavior on RRM measurement:

- If the entry criteria of LP-WUS monitoring is satisfied, UE performs LR RRM measurement;

- If the exit criteria of LP-WUS monitoring is satisfied, UE performs MR RRM measurement.

Proposal 5: The entry /exit criteria of LP-WUS monitoring can be based on MR and/or LR RRM measurement.

- Entry criteria: serving cell quality at least based on MR RRM measurement.

- FFS: also based on LR RRM measurement.

- Exit criteria: serving cell quality at least based on LR RRM measurement.

- FFS: also based on MR RRM measurement.

Proposal 6: Support to report UE capability on wake-up delay, which is the time offset between woken up by LP-WUS and be ready for PO reception.

Proposal 7: Support to configure LP-WUS relative to PO/PF. And the offset between LP-WUS and PO/PF is decided by

- Ramp up time from ultra-deep sleep state

- Sync time

Proposal 8: Support duty-cycled LP-WUS monitoring for RRC idle/inactive mode.

# References

- RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR).

- R1-2400253, Discussion on LP-WUS and LP-SS design, vivo.

- 3GPP TR 38.869 V2.0.0, Study on low-power wake up signal and receiver for NR, Release 18.