3GPP TSG RAN WG1 Meeting # 116				                                                 R1-2400272

Athens, Greece, Feb 26th – Mar 1st, 2024

Agenda Item:	9.6.1

Source:	TCL

Title:	LP-WUS and LP-SS Design

Document for:	Discussion and Decision

# Introduction

In 3GPP RAN#102 meeting, a working item for NR LP-WUS and LP-WUR was endorsed [1], with a focus on specifying the LP-WUS signal design and procedure as mentioned in the objective of the working item below.

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - This objective is to be further refined in RAN#103

In this contribution we provide our views, on the waveform of LP-SS and LP-WUS, the synchronization of LP-WUR, the bandwidth of LP-WUS and LP-SS, and the location of LP-WUS and LP-SS in both time domain and frequency domain.

# LP-WUS and LP-SS waveform

In Rel-18, several options of MC-ASK and MC-FSK waveforms for LP-WUS were investigated. However, no study was conducted on the LP-SS waveform. We believe that studying the LP-SS waveform is also necessary to consider because existing LP-WUS waveform such as OOK or FSK based waveform, only transmit binary data by turning the carrier on and off or varying its frequency, respectively. They do not provide any information about the carrier signal's phase or frequency, which may not be suitable for LP-SS. Instead, QPSK or OFDM based waveform provide more information about the carrier signal's phase and frequency, which can be used for accurate time/frequency synchronization. However, when designing the LP-SS waveform, the limited bandwidth of the LP-WUS carrier and the simple architecture of the LP-WUR can be taken into account.

Observation 1: The LP-WUS waveform, which uses OOK or FSK modulation schemes, may not be suitable for LP-SS because it does not provide any information about the carrier signal's phase or frequency.

# Synchronization of LP-WUR

Periodic LP-SS can be broadcasted, similar to SSB, to carry synchronization and acquisition information. This enables all UEs within the coverage area to synchronize their LP-WUR with the network, establish communication, and decode LP-WUS. The periodicity of LP-SS can follow the existing SSB-based signal. Furthermore, using the same waveform for LP-SS and LP-WUS can reduce specification effort and LP-WUR architecture requirements. However, the LP-WUS waveform uses OOK and FSK modulation schemes that only transmits binary data by turning the carrier on and off, and by varying the frequency of the carrier signals respectively. It does not provide any information about the phase or frequency of the carrier signal, which may not be suitable candidates for the waveform of LP-SS.

Regarding the structure of LP-SS, two alternatives were discussed in Rel-18: Alt1, sequence-based, and Alt2, sequence + message with encoded bits. Each alternative has its own advantages and disadvantages. Alt1 may have less overhead but cannot carry extra information. On the other hand, Alt2 may increase overhead but can enable/disable continuous monitoring behavior through an encoded message in the LP-SS payload. This capability can help reduce LP-WUR power consumption if a UE is not triggered.

Observation 2:  Using the same waveform for LP-SS and LP-WUS may reduce the specification effort and LP-WUR architecture requirements. However, the OOK or FSK based LP-WUS waveform may not be suitable for LP-SS, as they do not provide information about the phase and frequency of the carrier signal

Observation 3: The sequence + message with encoded bits structure of LP-SS is more beneficial in terms of enabling/disabling continuous monitoring behavior of LP-SS.

Proposal 1: The structure of LP-SS can contains a sequence for synchronization and a message with encoded bits for activation or de-activation of LP-WUS monitoring.

# Bandwidth of LP-WUS and LP-SS

In Rel-18, it was agreed that at least for idle/inactive mode the recommended BW size can be <= 5MHZ [2]. However, there was no discussion regarding the bandwidth of LP-SS.  Since the LP-SS will also be processed by the LP-WUR of the UE, therefore it may necessary to define the bandwidth of LP-SS.  In our view, a configurable BW size with a lower limit of 5 MHz and an upper limit of 20 MHz can be considered for the bandwidth of both LP-WUS and LP-SS. This would offer flexibility in selecting a suitable bandwidth between 5 MHz and 20 MHz, which is useful in situations where power consumption and complexity are main concerns of LP-WUR architecture. In addition, the configurable BW size can be considered for both idle state and connected state. Moreover, The LP-WUR of the UE shall know this configured bandwidth, and it can be configured to the UE in SIB1 or SIBX during initial/random access.

Observation 4: Configurable bandwidth for LP-WUS and LP-SS within the range of 5MHz and 20MHz may offer flexibility and keep the low power consumption of LP-WUR.

Proposal 2: Consider, a configurable BW size within the range of 5MHz and 20MHz for LP-WUS and LP-SS in both idle/inactive and connected state.

LP-WUS association with BWP

According to the current specification, a signal/channel is typically transmitted to the UEs within a BWP. However, since LP-WUS is a wake-up signal, it may not necessarily follow the existing specification rules and can be transmitted independently of BWPs. However, in case periodic LP-SS is used for synchronization of LP-WUR, and the LP-SS is transmitted within a BWP (such as initial DL BWP) and LP-WUS are configured in independent of BWP or within an active DL BWP, then the LP-WUR has to perform RF retuning and BWP switching for synchronization before LP-WUS detection as shown in Figure 1. This behavior may increase the LP-WUR power consumption.  In order to solve this issue, a dedicated downlink BWP which may contains both the LP-SS and LP-WUS with a configurable BW size within the range of 5MHz to 20 MHz, can be configured to the UE. Since the dedicated downlink BWP contains the LP-SS to synchronize the LP-WUR with the network and LP-WUS to trigger the MR to receive data/signaling, therefore the LP-WUR may avoid the RF retuning for LP-SS.

Figure 1 LP-WUR RF retuning for LP-SS and LP-WUS

Observation 5: The transmission of LP-SS and LP-WUS in different BWP, or the transmission of LP-WUS independent of BWP, may let the LP-WUR to perform RF retuning for LP-SS, and increase its power consumption.

Proposal 3: Study a dedicated BWP for the placement of LP-WUS and LP-SS, with the maximum bandwidth within the range of 5MHz to 20MHz.

Proposal 4: The configurable BW of LP-WUS and LP-SS and its associated dedicated BWP can be configured to the UE during initial access.

# Location of LP-WUS and LP-SS

## 5.1 Placement/location of LP-WUS and LP-SS in Frequency domain

In Rel-18, the carrier of LP-WUS and the detail location of LP-WUS within a carrier was discussed as given below [2].

- Whether LP-WUS and signals/channels used by MR can be different carriers in the band - Details on the LP-WUS location within a carrier

However, no concrete agreement was achieved in the study item phase. In case the LP-WUS is considered to use the same carrier as the MR signal/channel, the LP-WUR need to support all the existing bands and their associated carriers of NR, which may complicate its design. On the other hand, if the LP-WUS as well as the LP-SS and signal/channel used by MR are on different carriers in the band, a dedicated carrier may be required for LP-WUS transmission, which reduces flexibility and wastes carrier resources.

In case, the LP-WUS and LP-SS uses the other NR carrier channels, then a flexibly configured location within a carrier which is used for both LP-WUS and LP-SS is more feasible. This approach allows to place the LP-WUS and LP-SS at the lower edge or upper edge of the carrier BW, or utilizing empty PRBs in the carrier that do not impact the legacy NR channel. However, it is important for the LP-WUR of the UE to have knowledge of the exact location of the LP-WUS to avoid searching the entire carrier for LP-WUS detection.

Observation 6: LP-WUS/LP-SS and MR’s signal/channel on the same carrier in the band need to support all NR bands and it may complicate the design of LP-WUR. If LP-WUR and MR’s signal/channel use different carriers, a dedicated carrier for LP-WUS transmission may be needed.

Observation 7: A flexibly configured location within a carrier for LP-WUS and LP-SS can efficiently utilize the frequency resources of the carrier without impacting the legacy NR channel.

## 5.2 Placement/location of LP-WUS in time domain

Similar to the placement of LP-WUS in frequency resources, it may also be necessary to consider the time domain resources of the LP-WUS waveform. This will help minimize the continuous monitoring behavior of the LP-WUR for LP-WUS detection, and ensure that the LP-WUR maintains a simple architecture.

LP-WUS occasion: To improve the detection of LP-WUS in the time domain and enhance its coverage, the LP-WUS monitoring occasions, where the LP-WUR performs detection attempts to decode the LP-WUS can be considered as shown in figure 2. The total number of monitoring occasions for LP-WUS can be configured in higher layers such as SIB1. During the LP-WUS occasion, the LP-SS is not configured to avoid any time domain conflicts between LP-SS and LP-WUS.

Figure 2 LP-WUS Occasion with K monitoring occasion

Location of the LP-WUS occasion in time domain: In order for the LP-WUR of a UE to detect the LP-WUS in the time domain, the location of the LP-WUS occasion must be known. This can be achieved by using a predefined reference point known to the UE, along with an offset value in terms of slots/symbols, to determine the exact timing of the first monitoring occasion of the LP-WUS occasion. There are several options for finding the location of the LP-WUS occasion in time domain.

For instance, for idle/inactive UEs, the incoming/target PO or SSB can be used as a reference point, and an SSB level or symbols/slots level offset can be applied to locate the LP-WUS occasion, as shown in Figure 3. For connected UEs, the SSB can be used as a reference point, or if periodic LP-SS is transmitted to the LP-WUR, the LP-SS can be considered as a reference point, and a symbols/slots level offset can be used from SSB or LP-SS to identify the LP-WUS occasion location, as shown in Figure 4.

Figure 3 LP-WUS occasion location w.r.t target PO or SSB for idle/inactive UEs

Figure 4 LP-WUS occasion location w.r.t SSB or LP-SS for idle/inactive or connected UEs

Observation 8: The placement of LP-WUS in the time domain resources should also be considered to minimize the continuous monitoring and maintain a simple architecture.

Observation 9: LP-WUS can be defined as a set of consecutive monitoring occasions, and during this time, LP-SS is not configured to avoid conflicts.

Proposal 5: To locate LP-WUS in the time domain, a reference point and an offset from the reference point be used:

- For idle/inactive UEs, the incoming PO or SSB can be used as a reference point

- For connected UEs, SSB or periodic LP-SS can be used as reference point.

## 5.3 Configuration of the LP-WUS and LP-SS

Configuring the frequency and time domain parameters of the LP-WUS and LP-SS to the UE through the MR can reduce power consumption and blind detection complexity for the UEs LP-WUR. This enables the LP-WUR to perform detection in specific frequency resources and avoid continuous monitoring. The configuration parameters may include carrier, location within the carrier, BWP of LP-WUS and LP-SS, location of LP-WUS in the time domain, and periodicity of LP-SS. SIB can be used to configure these parameters.

Proposal 6: Study the configuration of time and frequency resources for LP-WUS and LP-SS to the UE.

# LP-WUS Indication

Due to the simple architecture and low sensitivity of the LP-WUR, the coverage of LP-WUS may be degraded as compared to the legacy NR channel. In order to ensure the reliability of the LP-WUS an indication from the MR of UE can be used to let the gNB know that the LP-WUS has successfully received and decoded by the LP-WUR of the UE. This indication can be Implicit, where the gNB implicitly derive from the first ACK message (such as HARQ-ACK) received from the MR of UE after the UE receives data/signaling as shown in Figure 5a, or it can be an explicit indication, where the MR of UE sends an ACK message to the gNB and let the gNB know that the LP-WUS has successfully received as shown in Figure 5b. The implicit indication can reduce the false rate of ACK message to the gNB. However, the physical resources (time/frequency resources) may be wasted in case the gNB transmit the data/signaling after LP-WUS without knowing the MR on/off status.  The explicit indication of LP-WUS reception will enhance the resource utilization, however it may increase the rate of false ACK message to the gNB in case the LP-WUR of a UE decode a noise signal and consider it as an LP-WUS.

Figure 5a Implicit Indication of LP-WUS reception        Figure 5b Explicit Indication of LP-WUS reception

Observation 10: The implicit or explicit indication for the LP-WUS reception from the MR of a UE may enhance the LP-WUS reliability.

Proposal 7: Study the following indication method for LP-WUS successful detection;

- Implicit derivation of LP-WUS detection from the first ACK message received from the MR, which is sent by the UE for receiving the data/signaling.

- Explicit derivation of LP-WUS detection, where the MR sent ACK message before receiving the signaling/data.

# Conclusion

In this contribution we discussed, the waveform, bandwidth, location in time domain, location in frequency domain for both LP-WUS and LP-SS, and made the following observations and proposals.

Observation 1: The LP-WUS waveform, which uses OOK or FSK modulation schemes, may not be suitable for LP-SS because it does not provide any information about the carrier signal's phase or frequency.

Observation 2:  Using the same waveform for LP-SS and LP-WUS may reduce the specification effort and LP-WUR architecture requirements. However, the OOK or FSK based LP-WUS waveform may not be suitable for LP-SS, as they do not provide information about the phase and frequency of the carrier signal

Observation 3: The sequence + message with encoded bits structure of LP-SS is more beneficial in terms of enabling/disabling continuous monitoring behavior of LP-SS.

Observation 4: Configurable bandwidth for LP-WUS and LP-SS within the range of 5MHz and 20MHz may offer flexibility and keep the low power consumption of LP-WUR.

Observation 5: The transmission of LP-SS and LP-WUS in different BWP, or the transmission of LP-WUS independent of BWP, may let the LP-WUR to perform RF retuning for LP-SS, and increase its power consumption.

Observation 6: LP-WUS/LP-SS and MR’s signal/channel on the same carrier in the band need to support all NR bands and it may complicate the design of LP-WUR. If LP-WUR and MR’s signal/channel use different carriers, a dedicated carrier for LP-WUS transmission may be needed.

Observation 7: A flexibly configured location within a carrier for LP-WUS and LP-SS can efficiently utilize the frequency resources of the carrier without impacting the legacy NR channel.

Observation 8: The placement of LP-WUS in the time domain resources should also be considered to minimize the continuous monitoring and maintain a simple architecture.

Observation 9: LP-WUS can be defined as a set of consecutive monitoring occasions, and during this time, LP-SS is not configured to avoid conflicts.

Observation 10: The implicit or explicit indication for the LP-WUS reception from the MR of a UE may enhance the LP-WUS reliability.

Proposal 1: The structure of LP-SS can contains a sequence for synchronization and a message with encoded bits for activation or de-activation of LP-WUS monitoring.

Proposal 2: Consider, a configurable BW size within the range of 5MHz and 20MHz for LP-WUS and LP-SS in both idle/inactive and connected state.

Proposal 3: Study a dedicated BWP for the placement of LP-WUS and LP-SS, with the maximum bandwidth within the range of 5MHz to 20MHz.

Proposal 4: The configurable BW of LP-WUS and LP-SS and its associated dedicated BWP can be configured to the UE during initial access.

Proposal 5: To locate LP-WUS in the time domain, a reference point and an offset from the reference point be used:

- For idle/inactive UEs, the incoming PO or SSB can be used as a reference point

- For connected UEs, SSB or periodic LP-SS can be used as reference point.

Proposal 6: Study the configuration of time and frequency resources for LP-WUS and LP-SS to the UE.

Proposal 7: Study the following indication method for LP-WUS successful detection;

- Implicit derivation of LP-WUS detection from the first ACK message received from the MR, which is sent by the UE for receiving the data/signaling.

- Explicit derivation of LP-WUS detection, where the MR sent ACK message before receiving the signaling/data.

# References

RP-234056, “New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, 3GPP TSG RAN Meeting #102, Edinburgh, GB, December 11-15, 2023

3GPP TR 38.869, Study on low-power wake up signal and receiver for NR, (Release 18)