3GPP TSG-RAN WG1 #116	R1-2400295

Athens, Greece, February 26th – March 1st, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PDSCH resource mapping and DMRS reception |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW-Core |  |  |  |  |  | Date: |  |  | 2024-2-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is agreed that SS/PBCH blocks should be punctured to 12 PRBs for 3 MHz channel bandwidth. In the existing specification, when PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, gNB should guarantee that no REs for reception of the PDSCH is overlapping with resources for SS/PBCK block transmission. For other PDSCHs, the resources should be mapped around resources for SSB transmission. For dedicated spectrum with 3 MHz channel bandwidth, it needs to clarify that the SSB is after puncturing for the PDSCH resource mapping. Similarly, the UE shall not expect to receive DM-RS in resource elements that overlap with those of the SS/PBCH block. It also needs a clarification that the SSB is after puncturing for dedicated spectrum with 3 MHz channel bandwidth. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the SS/PBCH block is after puncturing if applicable for PDSCH resource mapping and DM-RS reception. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect UE behavior on PDSCH resource mapping and DM-RS reception for dedicated spectrum less than 5 MHz. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.4, 5.1.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated impact analysis: This CR is based on RAN1’s common understanding. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

### 5.1.4	PDSCH resource mapping

When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block, or no SS/PBCH block after puncturing if applicable, is transmitted in REs used by the UE for a reception of the PDSCH.

When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MSGB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources, or SS/PBCH block transmission resources after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted.

A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1.

When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, G-RNTI, G-CS-RNTI, MCCH-RNTI, multicast-MCCH-RNTI or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, or SS/PBCH block transmission resources after puncturing if applicable, and the UE shall assume that the PRBs containing SS/PBCH block transmission resources, or SS/PBCH block transmission resources after puncturing if applicable, are not available for PDSCH in the OFDM symbols where SS/PBCH block associated with the same PCI is transmitted.

<Unchanged parts are omitted>

#### 5.1.6.2	DM-RS reception procedure

<Unchanged parts are omitted>

If the UE receives the DM-RS for PDSCH and an SS/PBCH block associated with the same PCI in the same OFDM symbol(s), then the UE may assume that the DM-RS and SS/PBCH block are quasi co-located with 'typeD', if 'typeD' is applicable. Furthermore, the UE shall not expect to receive DM-RS in resource elements that overlap with those of the SS/PBCH block, or SS/PBCH block after puncturing if applicable, associated with the same PCI as the DM-RS, and the UE can expect that the same or different subcarrier spacing is configured for the DM-RS and SS/PBCH block in a CC except for the case of 240 kHz where only different subcarrier spacing is supported. A DM-RS for PDSCH is said to be associated with an additional PCI if the indicated TCI state for the PDSCH is associated with the additional PCI, otherwise a DM-RS for PDSCH is associated with serving cell PCI.

<Unchanged parts are omitted>