3GPP TSG RAN WG1 #116 	                               R1-2400311

Athens, Greece, February 26th – March 1st, 2024

Source: 	CMCC

Title:	Remaining issues on Multi-carrier enhancements for NR

Agenda item:	8.8

Document for:	Discussion & Decision

- Introduction

The remaining issues on Rel-18 Multi-carrier enhancements have been discussed during previous RAN1 meetings, and draft CRs are endorsed after last RAN1 meeting [1]. In this contribution, we mainly discuss and share our views on the remaining issues and CRs of multi-cell PUSCH/PDSCH scheduling with a single DCI.

- Discussion

2.1 Interpretation of Type-1A fields

According to the design principle of different field types of DCI format 0_3/1_3, the Type-1A fields could indicate common information shared by all co-scheduled cells, and the size of each Type-1A field is determined by the maximum field size of active BWPs among all scheduled cells within the set of cells. Considering UE will interpret the field independently for each scheduled cell for some Type-1A fields, the issue on invalid values/codepoints of the field for a scheduled cell when the maximum configurable size for the scheduled cell is smaller than the field size in DCI format 0_3/1_3 has been discussed. And the higher layer configuration or rules for UE to interpret invalid codepoints for some of Type-1A fields have been achieved. The corresponding agreements for BWP indicator, antenna port(s) field, TPMI field, SRI field when configured as Type-1A field are shown as below.

RAN1#115 Agreement - When Antenna port(s) field in DCI format 1_3 is configured as type1a, UE expects to be configured with a common table from Tables 7.3.1.2.2-1/2/3/4 in TS38.212 is used for all cells in set of cells. - The DMRS mapping type should be the same across the cells in set of cells. - When Antenna port(s) field in DCI format 0_3 is configured as type1a, UE expects to be configured with a common table from Tables 7.3.1.1.2-6, 7.3.1.1.2-6A, 7.3.1.1.2-7, 7.3.1.1.2-7A, 7.3.1.1.2-8, 7.3.1.1.2-9, 7.3.1.1.2-10, 7.3.1.1.2-11, 7.3.1.1.2-12, 7.3.1.1.2-13, 7.3.1.1.2-14, 7.3.1.1.2-15, 7.3.1.1.2-16, 7.3.1.1.2-17, 7.3.1.1.2-18, 7.3.1.1.2-19, 7.3.1.1.2-20, 7.3.1.1.2-21, 7.3.1.1.2-22, 7.3.1.1.2-23, 7.3.1.1.2-24, and 7.3.1.1.2-25 in TS38.212 is used for all cells in set of cells. - The DMRS mapping type should be the same across the cells in set of cells. - When TPMI field in DCI format 0_3 is configured as type1a, UE expects to be configured with a common table from Tables 7.3.1.1.2-2, 7.3.1.1.2-2A, 7.3.1.1.2-B, 7.3.1.1.2-3, 7.3.1.1.2-3A, 7.3.1.1.2-4, 7.3.1.1.2-4A, 7.3.1.1.2-5, and 7.3.1.1.2-5A in TS38.212 is used for all cells in set of cells. - When SRI field in DCI format 0_3 is configured as type1a, UE expects to be configured with a common table from Tables 7.3.1.1.2-28, 7.3.1.1.2-29, 7.3.1.1.2-30, 7.3.1.1.2-31, 7.3.1.1.2-32, 7.3.1.1.2-32A, and 7.3.1.1.2-32B in TS38.212 is used for all cells in set of cells. RAN1#115 Agreement - BWP indicator in a DCI format 0_3/1_3 applies only to the scheduled cell(s) with valid FDRA value(s). - For a cell scheduled by DCI format 0_3/1_3 with valid FDRA value, if the BWP indicator indicates a code point that does not correspond to a configured BWP for the cell, the UE does not perform dynamic BWP switching based on the BWP indicator and transmits/receives data on the current active BWP of the cell.

For BWP indicator, UE will ignore the invalid value and not perform BWP switching for the scheduled cell to avoid ambiguity issue on BWP switching. And for antenna port(s) field, TPMI field and SRI field when configured as Type-1A, a common higher layer table is expected to be configured, and there is no invalid states/codepoints issue as a consequence. Furthermore, considering some other Type-1A fields interpreted independently for each cell and the size of which is 0 or 1 bit determined by higher layer parameters, such as frequency hopping flag, VRB-to-PRB mapping, PRB bundling size indicator, the current description that the field is applied to the applicable scheduled cells configured with higher layer parameter independently in the 38.212 CR [2] is clear. And no further clarification is necessary.

While for the Open-loop power control parameter set indication in DCI format 0_3, the size of the field can be 0, 1, or 2 bits determined by the configuration of p0-PUSCH-SetList, SRS resource indicator, or olpc-ParameterSetDCI-0-1 for each scheduled cell. Due to the field size is determined based on the maximum size of this field in legacy formats across cells in the set of scheduled cells, the issue that invalid codepoints configured for some scheduled cells still exists. To ensure UE correctly interprets the field for each co-scheduled cell, pre-defined rule is needed, which can effectively align the understanding of the indication between gNB and UE. And for simplicity, UE interprets LSBs of the indication in the field based on the available configuration for the co-scheduled cell can be considered. When the field size is beyond the indication range of the configuration for a scheduled cell, UE will only interpret LSBs of the field based on the RRC configuration for that scheduled cell, which can also avoid impact on network configuration flexibility. The text proposal is provided as the following:

Proposal 1. Take the following TP for TS 38.212 section 7.3.1.1.4.

  - Reason for change: Clarify the UE behaviour on interpreting Open-loop power control parameter set indication field for each scheduled cell.

  - Summary of change: Add the clarification that UE interprets Mor LSB(s) of the Open-loop power control parameter set indication field in DCI format 0_3 for each co-scheduled cell independently.

  - Consequence if not approved: The interpretation of the field is not clear when the required field size for the scheduled cell is smaller than the determined field size in DCI format 0_3 in TS 38.212.

7.3.1.1.4 Format 0_3 <Unchanged text is omitted> - Open-loop power control parameter set indication - maxr∈{1,2,…,NcellUL,2}Mor bits applying to the scheduled cells with Mor>0 independently, where r is mapped to the cells according to an ascending order of a serving cell index with r=1 corresponding to the cell with the smallest serving cell index, and Mor is defined by the following: - 0 bit if the higher layer parameter p0-PUSCH-SetList is not configured; - 1 or 2 bits otherwise, - 1 bit if SRS resource indicator is present in the DCI format 0_3; - 1 or 2 bits as determined by higher layer parameter olpc-ParameterSetDCI-0-1 if SRS resource indicator is not present in the DCI format 0_3. If maxr∈{1,2,…,NcellUL,2}Mor > Mor for a scheduled cell, the UE interprets Mor LSB(s) of the field for this cell. <Unchanged text is omitted>

2.2 Coverage enhancements with multi-cell PUSCH scheduling

For PUSCH transmission scheduled by DCI format 0_3 with PUSCH repetitions, there is one conclusion that PUSCH repetition Type B operation is not supported with DCI format 0_3 (i.e., UE cannot be configured with PUSCH repetition Type B applicable for DCI format 0_3). While whether Rel-17 PUSCH coverage enhancements features or dynamic waveform switching in Rel-18 coverage enhancements can be combined with multi-cell PUSCH scheduling simultaneously has not been decided yet. The issue has been discussed in the last RAN1 meeting and no consensus to support TPI field for DCI format 0_3 in Rel-18.

From our perspective, the Rel-17/18 coverage enhancement mechanisms are specified to improve the PUSCH coverage performance of bottleneck channels in realistic deployment scenarios, such as TBoMS, available slot counting of PUSCH repetition Type A and dynamic waveform switching. However, multi-cell PUSCH scheduling with a single DCI is introduced to improve the utilization of scattered spectrums and save DCI signaling overhead in CA scenario with heavy traffic, which is more suitable to be used in good coverage cases. Besides, to avoid more complex design of DCI fields in DCI format 0_3, it seems not necessary to simultaneously support coverage enhancement features combined with multi-cell PUSCH scheduling. Thus, we think such PUSCH coverage enhancement features are not necessary to be introduced in multi-carrier scheduling DCI.

Proposal 2. Simultaneous multi-cell PUSCH scheduling with DCI format 0_3 and Rel-17/18 PUSCH coverage enhancements (DWS, TBoMS, Available Slot counting, DM-RS bundling) is not supported in Rel-18.

- Conclusions

In this contribution, remaining issues on multi-cell PUSCH/PDSCH scheduling with a single DCI are discussed, and the following proposals are made.

Proposal 1. Take the following TP for TS 38.212 section 7.3.1.1.4.

  - Reason for change: Clarify the UE behaviour on interpreting Open-loop power control parameter set indication field for each scheduled cell.

  - Summary of change: Add the clarification that UE interprets Mor LSB(s) of the Open-loop power control parameter set indication field in DCI format 0_3 for each co-scheduled cell independently.

  - Consequence if not approved: The interpretation of the field is not clear when the required field size for the scheduled cell is smaller than the determined field size in DCI format 0_3 in TS 38.212.

7.3.1.1.4 Format 0_3 <Unchanged text is omitted> - Open-loop power control parameter set indication - maxr∈{1,2,…,NcellUL,2}Mor bits applying to the scheduled cells with Mor>0 independently, where r is mapped to the cells according to an ascending order of a serving cell index with r=1 corresponding to the cell with the smallest serving cell index, and Mor is defined by the following: - 0 bit if the higher layer parameter p0-PUSCH-SetList is not configured; - 1 or 2 bits otherwise, - 1 bit if SRS resource indicator is present in the DCI format 0_3; - 1 or 2 bits as determined by higher layer parameter olpc-ParameterSetDCI-0-1 if SRS resource indicator is not present in the DCI format 0_3. If maxr∈{1,2,…,NcellUL,2}Mor > Mor for a scheduled cell, the UE interprets Mor LSB(s) of the field for this cell. <Unchanged text is omitted>

Proposal 2. Simultaneous multi-cell PUSCH scheduling with DCI format 0_3 and Rel-17/18 PUSCH coverage enhancements (DWS, TBoMS, Available Slot counting, DM-RS bundling) is not supported in Rel-18.

- References

- Chairman's Notes RAN1#115, November 13th – 17th, 2023.

- R1-2312742, Corrections on Rel-18 Multi-carrier enhancements in 38.212, Huawei.