3GPP TSG RAN WG1 #116	R1-2400366

Athens, Greece, February 26th - March 1st, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	Initial views on Waveform characteristics of carrier-wave provided externally to the Ambient IoT device

Agenda item:	9.4.2.4

Document for:	Discussion and Decision

# Introduction

At RAN#102, a new study item “Study on solutions for Ambient IoT (Internet of Things) in NR” (FS_Ambient_IoT_solutions) was approved [1].

The following objective(s) is/are relevant for the present agenda item:

2. Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination (coordination with SA3 is required for privacy aspects).  RAN1-led: For the Ambient IoT DL and UL: o Frame structure, synchronization and timing, random access o Numerologies, bandwidths, and multiple access o Waveforms and modulations o Channel coding o Downlink channel/signal aspects o Uplink channel/signal aspects o Scheduling and timing relationships o Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1.

# Discussion

The following deployment scenarios and topologies are within the scope of the study [1]:

- Deployment scenario 1 with Topology 1

  - Basestation and coexistence characteristics: Micro-cell, co-site

- Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control

  - Basestation and coexistence characteristics: Macro-cell, co-site

  - The location of intermediate node is indoor

Both topologies can be implemented in a mono-static or bi-static setup. The Activator (activating device) and the Reader (receiving device) are co-located in a mono-static setup and they are expected to be capable of operating in full duplex mode, i.e., transmitting and receiving at the same time, to support Ambient IoT devices that backscatter their responses on the same frequency as the activation signal, like Ambient IoT device type (i) and (ii). Bi-static setups can remove the requirement for full duplex support as the Activator and the Reader are not co-located and separate devices, which will enable generic 3GPP gNBs & UEs to act as Activators or Readers (Focus evaluation case A, as defined in [2]). However, a bi-static set-up will suffer from interference issues as the transmitted activation signal will also be received by the AIoT receiving device.

Another option to avoid the full duplex requirement is for the Ambient IoT device to support FDD duplex frequency shift, which will only be an option for more advanced Ambient IoT device type (ii) having its own power storage charged by other means than the Ambient IoT activating signal alone.

## On activation signal interference at the Ambient IoT receiving device

The transmitted activation signal (for example a UL signal in a FDD configuration) will also be received at the reader in a bi-static set-up. Power level of that signal can be many orders of magnitude higher than the backscattered response from the Ambient IoT device, especially in Line of Sight channel (LoS) conditions. This is illustrated in Figure 1 below for a scenario where the Ambient IoT device type (i) is 5 m away from the Activator transmitting the activation signal and in the direct path (worst-case) between the Activator and Reader.

Figure 1: Received power levels at the Reader of the Ambient IoT device activation signal and the back scattered signal from the Ambient IoT for a LoS bi-static scenario.

It is seen from Figure 1 that the power level of the activation signal can be up to 50 dB higher than the received emitted response from the Ambient IoT device at the Reader. Such high level of interference can be problematic for common RF receiver architectures and even at baseband level, and will have to be addressed within RAN1. The interference level of the activation signal at the Reader will mostly be affected by the available antenna gain for an Ambient IoT device type (i) and/or the potential reflection gain of an Ambient IoT device type (ii). As such, every dB of antenna gain at the Ambient IoT device will in theory reduce the interference level by a factor twice as large as the antenna gain value. However, even a 10 dB antenna gain at the Ambient IoT device (very large device) or a reflection gain of 20 dB, will only reduce the potential interference level to around 30 dB, which is still a very high interference level. It is worth observing that the antenna gain of an Ambient IoT device is expected to be low (less than 3 dBi) as Ambient IoT devices are small in size and thereby also electrical small compared to the frequency of the activation signal. Hence, back-scattered signals from an Ambient IoT device type (i) can be expected to be received at the Reader with a high interference level in a bi-static setup.

Observation 1: Activation signal can induce very high interference levels (up to 50 dB) at the Reader in bi-static setup.

Different mitigation techniques already known within 3GPP could be adopted for interference mitigation for Ambient IoT, like adding a QCL relation between reference signals transmitted by the Activator and the activation signal itself, whereby the Reader can estimate the channel between itself and the Activator. This would enable the Reader to cancel out the activation signal at its receiver. Another option is to utilize Cross Link Interference (CLI) measurements at the Reader and reported back to the gNB, whereby the gNB can optimize the scheduling of Activators and Readers for the Ambient IoT session.

Proposal 1: RAN1 to study if mitigation procedures/techniques based on existing tools in RAN1 specification, e.g., QCL relationships between reference signals or the Cross Link Interference (CLI) framework, can be used for mitigating the Ambient IoT activation signal interference at the Reader in bi-static setup.

## On activation signal carrier-wave wave form

Within the frequency response range of the components, the Ambient IoT device can be activated by a carrier-wave with any waveform and backscatter (reflect) a modulated signal, whether the activation signal is single-tone continuous waveform or multiple-tone continuous waveform. The original activation signal may be already modulated or may be a constant amplitude signal. It is always possible to modulate the device’s data using at least the simplest method of OOK on top of, for example, the OFDM signal on which user data are modulated.

Furthermore, there is a clear difference with respect to how single-tone continuous waveform and multiple-tone continuous waveform should be supported, and we need to treat the design of each separately. For example, for the former, a new narrowband single-tone activation signal would be needed. For the latter, we can reuse the NR/LTE signals as the activation signal. A very intuitive example is to use UE's uplink transmission signal as activation for the Ambient IoT device. This would also motivate the support of coexistence of Ambient IoT and NR/LTE, since time-frequency resources used by NR/LTE physical channels and signals would be reused in this case.

Another aspect to consider is the relationship with the waveform used for the activation and the waveform that carries control information from activator to Ambient IoT device, if any. The complexity of the latter must be kept under control. Being able to demodulate an OFDM signal and parse its content, or detect and decode channels such as PDCCH/PUCCH may not be possible for the devices, at least those that communicate through backscattering.

Finally, it should be noted that in case the activation signal serves also as source of energy, i.e., it is used for RF energy harvesting, then its envelope and spectrum would strongly impact the AC-to-DC conversion efficiency, which in turn would affect the charging time of the device and its (albeit rudimentary) capabilities. This is particularly relevant for devices which do not have any energy storage, such as device type i. For instance, it is well-known that multi-carrier/tone waveforms who display high PAPR are better suited for the RF energy harvester, which in turn could charge the device more quickly. This can have a significant impact on aspects such as the delay at which a device can be become operative, the number of active Ambient IoT devices in a given area, the interference measured at the reader, the capability of the Ambient IoT device to satisfy certain timing/synchronization requirements and so on.

Proposal 2: RAN1 to identify the aspects and metrics that should drive the selection of the activation signal carrier-wave waveform, and study the impact that a certain waveform selection has on the main L1 aspects considered in this SI. Existing NR/LTE OFDM waveforms can be used as baseline for study, where considering new waveforms, if adequately justified by practical evidence, is also not precluded.

# Conclusion

In this contribution, we have made the following observations and proposals related to Ambient IoT:

Observation 1: Activation signal can induce very high interference levels (up to 50 dB) at the Reader in bi-static setup.

Proposal 1: RAN1 to study if mitigation procedures/techniques based on existing tools in RAN1 specification, e.g., QCL relationships between reference signals or the Cross Link Interference (CLI) framework, can be used for mitigating the Ambient IoT activation signal interference at the Reader in bi-static setup.

Proposal 2: RAN1 to identify the aspects and metrics that should drive the selection of the activation signal carrier-wave waveform, and study the impact that a certain waveform selection has on the main L1 aspects considered in this SI. Existing NR/LTE OFDM waveforms can be used as baseline for study, where considering new waveforms, if adequately justified by practical evidence, is also not precluded.

# References

- RP-234058, ” New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, RAN#102

- R1-2400361,” Initial views on Evaluation assumptions and results for Ambient IoT”, Nokia, RAN1#116