3GPP TSG RAN WG1 #116		R1-2400394

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.1.3.2

Source:	Google

Title:	ML based CSI prediction

Document for:	Discussion/Decision

# Introduction

In RP-234039, the following objectives for NR AI/ML enhancement are agreed.

Provide specification support for the following aspects: - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback - Identification related signalling is part of the above objective - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models - Signalling mechanism of applicable functionalities/models - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above two use cases. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain). - Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. NOTE: Coordination with SA/SA WGs of the ongoing study/work as it may relate to their required work.

In this contribution, we provide some discussion on AI/ML based CSI compression.

# Discussion

## CSI-RS configuration

Since the CSI prediction is agreed to be based on UE side model only, the input for CSI prediction should be based on the received CSI-RS instances. It is unnecessary to specify the exact input for CSI prediction. But the further study should focus on the CSI-RS configuration for the UE to measure the input for CSI prediction.

Different from the Rel-18 CSI, the UE may only support CSI prediction for CSI-RS with a certain configuration, e.g., a certain interval between every two consecutive CSI-RS instances, and number of CSI-RS instances for a CSI prediction. Thus, it is necessary for the NW to know the UE supported configuration for the CSI-RS for CSI prediction. Otherwise, there could be a CSI-RS configuration mismatch as shown in Figure 1.

Figure 1: CSI-RS configuration mismatch for CSI prediction

Proposal 1: Support the UE reports the preferred CSI-RS configuration for CSI prediction including at least the preferred intervals between every two consecutive CSI-RS instances and minimum number of CSI-RS instances for CSI prediction.

## CSI report content

Currently the Type1 codebook is widely implemented, which is a mandatory feature for Rel-15 UE. The CSI report in Rel-18 MIMO is based on Type2 codebook, which result in large report overhead and UE complexity. The CSI prediction in AI/ML could consider the Type1 codebook as the starting point, which is more practical from UE implementations perspective. The output for the CSI prediction could the predicted PMI based on a Type1 codebook. Then the CSI prediction could become a classification issue instead of a regression issue.

Further, with the help of the CSI prediction, the UE can also predict the CSI dwelling time, which could be much helpful for the network to determine when to trigger the CSI feedback. It is also helpful for the network to determine whether to trigger SRS to estimate the DL CSI or trigger a CSI report. Thus, for CSI prediction, it is necessary to consider the CSI dwelling time as an output.

In addition, for Type2 codebook, in addition to what is supported in Rel-18, one possible option is to report both predicted W1 and W2, since the AI/ML is able to predict CSI far from current CSI reference slot. It is possible that W1 could be different in different prediction slots.

Proposal 2: Support the following types of CSI report for AI/ML based CSI prediction:

- Type 1: Predicted RI/PMI/CQI based on Type1 codebook

- Type 2: Predicted RI/PMI/CQI based on Rel-16 eType2 codebook

- Type 3: Predicted RI/PMI/CQI based on Rel-18 eType2 codebook for PMI prediction (already supported)

- Type 4: Predicted CSI dwelling time

## UE side data collection

Similar to the UE side data collection for CSI compression, UE can also perform data collection for UE-side model training, finetuning, monitoring and so on for CSI prediction. Such data collection could require additional UE complexity. The NW still needs to know when the UE needs to perform the measurement for UE side data collection, as the NW needs to aware the additional UE complexity, e.g., additional CPU, for measurement for UE side data collection. Therefore, it is necessary to maintain the same understanding between the NW and UE on when to perform the measurement for UE side data collection, which can be based on NW configuration or UE request CSI-RS for data collection.

Proposal 3: Support to maintain the same understanding between the NW and UE on when to perform the measurement for UE side data collection for CSI prediction based on the following options:

- Option 1: The measurement for UE side data collection for CSI prediction is configured by the NW

- Option 2: UE request CSI-RS for data collection for CSI prediction

Proposal 4: Corresponding CPU(s) are occupied when UE performs CSI measurement for data collection

# Conclusion

In this contribution, we provided discussion on AI/ML based CSI prediction. Based on the discussion, the following proposals are provided.

Proposal 1: Support the UE reports the preferred CSI-RS configuration for CSI prediction including at least the preferred intervals between every two consecutive CSI-RS instances and minimum number of CSI-RS instances for CSI prediction.

Proposal 2: Support the following types of CSI report for AI/ML based CSI prediction:

- Type 1: Predicted RI/PMI/CQI based on Type1 codebook

- Type 2: Predicted RI/PMI/CQI based on Rel-16 eType2 codebook

- Type 3: Predicted RI/PMI/CQI based on Rel-18 eType2 codebook for PMI prediction (already supported)

- Type 4: Predicted CSI dwelling time

Proposal 3: Support to maintain the same understanding between the NW and UE on when to perform the measurement for UE side data collection for CSI prediction based on the following options:

- Option 1: The measurement for UE side data collection for CSI prediction is configured by the NW

- Option 2: UE request CSI-RS for data collection for CSI prediction

Proposal 4: Corresponding CPU(s) are occupied when UE performs CSI measurement for data collection