3GPP TSG RAN WG1 #116                                                                                             R1-2400436

Athens, Greece, February 26th – March 1st, 2024

# Source:             CATT

Title:                  Study of the Ambient IoT devices architecture

Agenda Item:    9.4.1.2

Document for:  Discussion and Decision

Introduction

In RAN#102 meeting, a new SID on Ambient IoT in NR was approved [1]. This study item will include the following objectives.

General Scope The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - X is to be decided in WGs. - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”. - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ. NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement. - Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Basestation and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Basestation and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor - FR1 licensed spectrum in FDD. - Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s). - Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command). - From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case. Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum. - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. - Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1. - RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential. - RAN3-led: - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable: - Paging - Device context management - Data transport - Identify RAN architecture aspects, including whether support for split architecture is necessary. - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network. - RAN4-led: - Coexistence study of Ambient IoT and NR/LTE. - RF requirements study for Ambient IoT: - Ambient IoT BS transmission and reception - Ambient IoT Device, as per the General Scope, transmission and reception - Intermediate node (UE), as per the General Scope, transmission and reception RAN2 and RAN3 are expected to identify RAN-CN functional split in coordination with SA2. Note: This study shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc. The study shall not aim to replace existing 3GPP LPWA technologies.

In this contribution, we discuss the Ambient IoT devices architecture and the aspects to consider in the architecture design of A-IoT.

Ambient IoT device characteristic for NR system design

Different from the traditional NR interactive UE device, A-IoT device is a response device in Rel-19 Ambient IoT study [1]. For responsive A-IoT communication, the response signal transmitted from A-IoT device depends on the energy and waveform of the excitation signal provided from the gNB or the intermediate node UE for devices Type-1 and Type-2 or self-generated internally for Type-2 devices. The general communication procedure for response device of A-IoT follows below steps. One example of A-IoT communication is shown in Figure 1.

- Step 1: The gNB or the intermediate node UE transmits the ‘interrogation’ signals with the control information to the Ambient IoT devices.

- Step 2: A-IoT devices would respond to the control information on the interrogation signal based on the pre-configured/pre-programed functions with the carrier wave for Type-1 and Type-2 devices.

Figure 1: The illustration of response communication for A-IoT

Proposal 1: The characteristic of Ambient IoT as a response device should be considered.

According to the objectives of the A-IoT study [1], RAN1 needs to discuss and identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity [4]. Based on the objectives of the A-IoT study [1], the A-IoT devices can be classified into two types according to their capabilities:

- Device Type-1: ~1 µW peak power consumption , has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - Device Type-2: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally.

For Type-1 device and Type-2 device, the operation band and the associated BW, the physical channels/signals, the channel coding, and the pre-configured/programed control function should have the common design. For Type-1 device, the carrier wave is provided externally, e.g., by the gNB or the intermediate node UE. For Type-2 device, the carrier wave could be generated internally, which the Type-2 device should be equipped with internal signal generation and transmission components or provided externally by the gNB or the intermediate node UE.

Proposal 2: The physical channel/signals and procedure and higher layer control function should have common design for Type-1 device and Type-2 device and additional components for Type-2 device with signal generation and transmission components should be supported.

Ambient IoT device architecture

A-IoT device architecture needs to meet A-IoT RAN design target and SI objective for power consumption and complexity. In general, the higher data transmission rates mean the higher power consumption and complexity for A-IoT device hardware implementation. Furthermore, the power consumption of device architecture is also related to the manufacturing process and the type of components selected. Some examples of implementations for passive IoT or ultra-low power IoT [4]~[16] are summarized in Table 1.

Table 1: The summary of data rates and power consumption of passive IoT or ultra-low IoT device

| Reference | Frequency | Modulation | Transmission rate | Power consumption |
| --- | --- | --- | --- | --- |
| [4] | 2.4GHz | GFSK | 125kbps | 10.6μW |
| [5] | 2.4GHz | DBPSK | 1Mbps | 2.5μW |
| [6] | 915MHz | FSK | 10Mbps | 753μW |
| [7] | 920MHz | FSK | 5Mbps | 700μW |
| [8] | 925MHz | FSK | 3Mbps | 367μW |
|  |  | OOK | 20Mbps | 314μW |
| [9] | 2.4GHz | MC-OOK | 62.5kbps | 495μW |
| [10] | 2.4GHz | QPSK | 2Mbps | 25μW |
| [11] | 2.4GHz | QPSK | 2Mbps | 28μW |
| [12] | 2.4GHz | FSK | 500kbps | 39μW |
| [13] | 800MHz | -- | -- | 1.0 μ W |
| [14] | 860~960MHz | ASK | -- | 1.0 μ W |
| [15] | 860~960MHz | -- | -- | 3.436 μ W |
| [16] | 915MHz | ASK | 20kbps | 4.28 μ W |

Proposal 3: A-IoT device architecture design needs to meet A-IoT RAN design target and SI objective for power consumption and complexity.

General architecture of Ambient IoT devices

Considering the power consumption and complexity design target of A-IoT in [1][2], the device architecture needs to consider at least the following main components[17]~[24] in Table 2.

Table 2: The main components of an Ambient IoT device

| Component | Function |
| --- | --- |
| Antenna | Receive radio signal and backscatter radio signal |
| RF energy harvesting | Capture the RF energy from receiver signals |
| RF energy storage | Storage the RF energy from RF energy harvesting |
| Receiver component | RF signal reception and processing |
| Backscatter/Tx component | Backscatter the RF signal or generate/Tx RF signal |
| Controller | Controlling the Rx and Tx functions |

An illustration of the general Ambient IoT architecture is shown in Figure 2.

Figure 2: The general architecture of A-IoT device

RF energy harvesting and storage module

The RF energy harvesting and storage components of Ambient IoT device are used to capture the RF energy to store or to provide a power supplement for the information stream processing of Ambient IoT device. One example of RF energy harvesting and storage [8] for A-IoT device is illustrated in Figure 3.

For RF energy harvesting and storage components, the antenna component is used to capture RF energy from carrier wave or generate electromagnetic wave. After the RF signal is received by the antenna, the load impedance of matching network is adjusted to fully match with the antenna impedance to maximum the RF signal reception strength. The received RF energy generates a DC output through a rectifier. In order to provide a stable voltage supply, a power management module is required. The stable voltage supply could provide all the power for subsequent information flow processing of A-IoT device. Furthermore, the harvesting RF energy can be stored through super capacitor [18] with sufficient power storage and to ensure the power supply quality of the load and maintain the stability of the power supply to subsequent processing circuits [16].

Figure 3: An example of the RF energy harvesting and storage architecture of A-IoT device

The activation threshold and energy conversion efficiency of the energy harvesting circuit are key design aspects in determining the performance of the A-IoT device, e.g., for A-IoT coverage performance.

- Activation threshold: The threshold value depends on the minimum power required to activate the semiconductor devices used in the energy harvesting circuit.

- RF-DC conversion efficiency: The performance index determined by the performance of the receiving antenna, the impedance matching network between the antenna and the rectifier circuit, etc.[20]. Table 3 summarizes examples of achievements in the design of RF energy harvesting circuits.

Table 3: The design of RF energy harvesting circuits [20]

For Type-1 device with ~1µW peak power consumption, the related activation threshold should be lower than -30dBm. For Type-2 device with a few hundred µW power consumption, the related activation threshold should be lower than 0 dBm.

The energy storage can be used to store sufficient energy for persistent RF signal processing during the time. Super capacitor [18] is usually used to store the relatively large amount of energy, especially for Type-2 device with a few hundred µW power consumption. Considering the RF energy harvesting and storage module is implemented with passive components, there is no power consumption problem.

Proposal 4: The RF energy harvesting and storage module should provide stable power supply for Type-1 device and Type-2 device of Ambient IoT.

Receiver module

For Type-1 device of A-IoT, ~1µw peak power consumption is defined in [1]. Therefore, RF receiver architecture with RF envelop detection of A-IoT device could be reused referring to LP-WUR RF receiver architecture. The illustration of Type-1 device receiver module is shown in Figure 4. As shown in Figure 4, the antenna receives RF signal and gets the ‘0/1’ bit through RF envelop detector and comparer in RF domain. The signal process is in the RF domain for A-IoT device since the down converter and multiple bit ADC component consumes a large amount of power, and thus may not be feasible to use in ultra-low-power systems, especially for Type-1 device with peak power consumption of ~1µw. The controller module is needed to provide a basic logic control for Rx and Tx function based on the control information in the interrogation signals. For Type-2 device of A-IoT, the similar receiver components of A-IoT with RF envelop detection could also be reused. Considering higher power consumption requirements, additional components could be used for Type-2 device of A-IoT to improve the detection performance of receiving signal.

Figure 4: The illustration of RF receiver module for A-IoT

The A-IoT receiver architecture could reduce power consumption by using ring oscillator or using FLL to replace PLL module. Although through specific low-power design manufacture processes, the power consumption of LO can be further reduced to less than 1µw [25], the local oscillator would still require persistent power to maintain the operation. It will drain out the power storage over short period of time for Type-1 device. Therefore, at least the Type-1 device of A-IoT needs to use RF envelop detector architecture without LNA module, multiple bit ADC module, or LO module. As summary, the power consumption of receiver module is summarized in Table 4.

Table 4: The power consumption of receiver module

| Components | Power consumption | Note |
| --- | --- | --- |
| Matching network | N.A. | Passive components |
| RF energy detector | N.A. | Passive components |
| Comparator | A few to several hundred nW | [25] |
| Controller | A few to several hundred nW | Tx/Rx controlling, decoder[23] |

Proposal 5: The receiver component of the ambient IoT could support a RF envelope detector architecture, at least for Type-1 A-IoT device.

Backscatter/Transmitter module

For backscattering communication system, the backscattering components are significantly important. The main backscattering components include the antenna, the matching network/impendence matching, the amplifier for reflected signals of Type-2 device, and the controller components. The antenna is an essential component used to receive and backscatter signals. Thus, the design of the antenna can significantly affect the performance of the backscatter communications system. Matching network is used to adjust the load impedance and antenna impedance, thereby adjusting the strength of the absorbed signal and the reflected signal. Reflected amplifier is used to amplify the reflected signal of Type-2 device of A-IoT. A reflected amplifier is achieved by adjusting the load impedance as a reflection coefficient magnitude larger than 1 [26]~ [30]. The typical power consumption of RA is several hundred µW in [26]~[30]. One example of backscattering module is shown in following Figure 5.

Figure 5: The illustration of A-IoT device backscattering architecture

Type-2 device of A-IoT could actively transmit the signals based on the objective of A-IoT study in Rel-19 [1]. An independent RF chain of active component for the transmission signal is needed, e.g., LO module, mixer, PA. Controller is used to control Tx and Rx function. Memory module or shift register is used to assist generation of backscattering signal or transmitter signal. One example of self-generate transmitter module for the active device is shown in following Figure 6

Figure 6: The illustration of A-IoT device transmitter architecture

Comparison with receiver module of A-IoT, the backscatter/transmitter module for A-IoT consuming most of power. As summary, the power consumption of backscatter/transmitter module is summarized in Table 5.

Table 5: The power consumption of A-IoT device backscattering/transmitter components

| Components | Power consumption | Note |
| --- | --- | --- |
| Reflected amplifier | A few to several hundred µW | -- |
| Impendence switching | N.A. | Passive components |
| Modulator& Coder | <1µW | -- |
| Controller & Memory | Several hundred µW | Tx/Rx function controlling, decoder, clock function, memory |
| Local oscillator | <100µW | Ring oscillator, or low oscillator frequency(32.768KHz) of RTC |
| Power amplifier | <1000µW | -- |

Proposal 6: The composition of backscatter/transmitter components in A-IoT devices should be strictly limited, considering that signal converters require more components and higher power, especially for Type-2 devices.

Architecture illustration of Type-1 and Type-2 A-IoT device

Based on the previous analysis of A-IoT main proponents, the examples of A-IoT architectures of Type-1 device and Type-2 device are shown in Figure 7, Figure 8 and Figure 9.

Figure 7: The illustration of A-IoT device architecture based on backscattering for Type-1 device

Figure 8: The illustration of A-IoT device architecture based on backscattering for Type-2 device

Figure 9: The illustration of A-IoT device architecture based on transmitter for Type-2 device

The power consumption of Type-1 device and Type-2 device is shown in Table 6.

Table 6: The summary of power consumption of Type-1 device and Type-2 device

| Components | Type-1 device | Type-2 device |
| --- | --- | --- |
| RF energy harvesting & storage | Passive | Passive |
| Matching network | Passive | Passive |
| Comparator | Several hundred nW | Several hundred nW |
| Decoder | Several hundred nW | Several hundred nW |
| Reflected amplifier | -- | Several hundred µW |
| Impendence switching | Passive | Passive |
| Controller | Several hundred nW | Several hundred µW |
| Memory | -- |  |
| Local oscillator | -- | <100µW |
| Power amplifier | -- | <1000µW |
| Total power consumption | ~1µW | <1000µW |

Observation 1: Considering the RAN design target of A-IoT, it is necessary to design appropriate architectures and component compositions for Type-1 and Type-2 devices, and limit power consumption.

Conclusion

In this contribution, the Ambient IoT devices architecture is discussed. We have the following proposals and observation:

Proposal 1: The characteristic of Ambient IoT as a response device should be considered.

Proposal 2: The physical channel/signals and procedure and higher layer control function should have common design for Type-1 device and Type-2 device and additional components for Type-2 device with signal generation and transmission components should be supported.

Proposal 3: A-IoT device architecture design needs to meet A-IoT RAN design target and SI objective for power consumption and complexity.

Proposal 4: The RF energy harvesting and storage module should provide stable power supply for Type-1 device and Type-2 device of Ambient IoT.

Proposal 5: The receiver component of the ambient IoT could support a RF envelope detector architecture, at least for Type-1 A-IoT device.

Proposal 6: The composition of backscatter/transmitter components in A-IoT devices should be strictly limited, considering that signal converters require more components and higher power, especially for Type-2 devices.

Observation 1: Considering the RAN design target of A-IoT, it is necessary to design appropriate architectures and component compositions for Type-1 and Type-2 devices, and limit power consumption.

References

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things), Huawei, December 11-15, 2023.

- 3GPP TR 38.848, V18.0.0 (2023-09), Study on Ambient IoT (Internet of Things) in RAN(Release 18).

- 3GPP TR 38.869, V2.0.0 (2023-12), Study on low-power wake up signal and receiver for NR (Release 18).

- K. A. Ahmed, R. Yang, P. Salamani, V. Rajanna and M. Alioto. Single-Antenna Backscattered BLE5 Transmitter with up to 97m Range, 10.6 μW Peak Power for Purely-Harvested Green Systems. ESSCIRC 2023- IEEE 49th European Solid State Circuits Conference (ESSCIRC), Lisbon, Portugal, 2023, pp. 49-52.

- L. Lin, K. A. Ahmed, P. S. Salamani and M. Alioto. Battery-Less IoT Sensor Node with PLL-Less WiFi Backscattering Communications in a 2.5-μW Peak Power Envelope. 2021 Symposium on VLSI Circuits, Kyoto, Japan, 2021, pp. 1-2.

- K. Tang et al.. A 75.3 pJ/b Ultra-Low Power MEMS-Based FSK Transmitter in ISM-915 MHz Band for Pico-IoT Applications. 2021 IEEE International Symposium on Circuits and Systems (ISCAS), Daegu, Korea, 2021, pp. 1-4.

- J. Bae, L. Yan and H. -J. Yoo. A Low Energy Injection-Locked FSK Transceiver With Frequency-to-Amplitude Conversion for Body Sensor Applications. IEEE Journal of Solid-State Circuits, vol. 46, no. 4, pp. 928-937.

- M. S. Jahan, J. Langford and J. Holleman. A low-power FSK/OOK transmitter for 915 MHz ISM band. 2015 IEEE Radio Frequency Integrated Circuits Symposium (RFIC), Phoenix, AZ, USA, 2015, pp. 163-166.

- R. Liu et al.. An 802.11ba-Based Wake-Up Radio Receiver With Wi-Fi Transceiver Integration. IEEE Journal of Solid-State Circuits, vol. 55, no. 5, pp. 1151-1164, May 2020.

- S. -K. Kuo, M. Dunna, H. Lu, A. Agarwal, D. Bharadia and P. P. Mercier. 21.5 An LTE-Harvesting BLE-to-WiFi Backscattering Chip for Single-Device RFID-Like Interrogation. 2023 IEEE International Solid-State Circuits Conference (ISSCC), San Francisco, CA, USA, 2023, pp. 320-322.

- P. -H. P. Wang, C. Zhang, H. Yang, D. Bharadia and P. P. Mercier. 20.1 A 28µW IoT Tag That Can Communicate with Commodity WiFi Transceivers via a Single-Side-Band QPSK Backscatter Communication Technique. 2020 IEEE International Solid-State Circuits Conference - (ISSCC), San Francisco, CA, USA, 2020, pp. 312-314.

- S. -K. Kuo, M. Dunna, D. Bharadia and P. P. Mercier. A WiFi and Bluetooth Backscattering Combo Chip Featuring Beam Steering via a Fully-Reflective Phased-Controlled Multi-Antenna Termination Technique Enabling Operation Over 56 Meters. 2022 IEEE International Solid-State Circuits Conference (ISSCC), San Francisco, CA, USA, 2022, pp. 1-3.

- A. Toccafondi, D. Zampilli, C. D. Giovampaola and V. Tesi. Low-power UWB transmitter for RFID transponder applications, 2012 IEEE International Conference on RFID-Technologies and Applications (RFID-TA). Nice, France, 2012, pp. 234-238.

- J. Essel, D. Brenk, J. Heidrich and R. Weigel. A highly efficient UHF RFID frontend approach. 2009 IEEE MTT-S International Microwave Workshop on Wireless Sensing, Local Positioning, and RFID, Cavtat, Croatia, 2009, pp. 1-4.

- A. S. W. Man, E. S. Zhang, H. T. Chan, V. K. N. Lau, C. Y. Tsui and H. C. Luong. Design and Implementation of a Low-power Baseband-system for RFID Tag. 2007 IEEE International Symposium on Circuits and Systems, New Orleans, LA, USA, 2007, pp. 1585-1588.

- Hongqiang Zong et al.. An ultra low power ASK demodulator for passive UHF RFID tag. 2011 9th IEEE International Conference on ASIC, Xiamen, 2011, pp. 637-640.

- D. Pathak, S. Vardhan G., N. Vamsi and A. Dutta. Wide Input Range Differential Dual Band RF Energy Harvesting System for Battery-Less RFID Applications. 2020 IEEE VLSI DEVICE CIRCUIT AND SYSTEM (VLSI DCS), Kolkata, India, 2020, pp. 484-488.

- Zhenbing L ,Jian L ,Jie Z , et al.. Ultra-Low Power High-Efficiency UHF-Band Wireless Energy Harvesting Circuit Design and Experiment[J].ZTE Communications,2018,16(01):2-10.

- J. Rosenthal and M. S. Reynolds. A 158 pJ/bit 1.0 Mbps Bluetooth Low Energy (BLE) Compatible Backscatter Communication System for Wireless Sensing. 2019 IEEE Topical Conference on Wireless Sensors and Sensor Networks, Orlando, FL, USA, 2019, pp. 1-3.

- D. A. Loku Galappaththige, F. Rezaei, C. Tellambura and S. Herath. Link Budget Analysis for Backscatter-Based Passive IoT. IEEE Access, vol. 10, pp. 128890-128922, 2022.

- [6] F. Amato, C. W. Peterson, B. P. Degnan and G. D. Durgin. A 45 μW bias power, 34 dB gain reflection amplifier exploiting the tunneling effect for RFID applications. 2015 IEEE International Conference on RFID (RFID), San Diego, CA, USA, 2015, pp. 137-144.

- G. Li et al.. A Reflection-Amplifier-Based Long-Range Semi-Passive UHF RFID Tag With Standard Compatibility. IEEE Microwave and Wireless Technology Letters, vol. 33, no. 9, pp. 1373-1376, Sept. 2023.

- José Carlos S. Palma,César A. M. Marcon, Hessel F, et al. A Passive 915 MHz UHF RFID Tag[C]. International Symposium on Quality Electronic Design. IEEE, 2008.

- D.-G. Lee, L. G. Salem and P. P. Mercier. Narrowband Transmitters: Ultralow-Power Design. IEEE Microwave Magazine, vol. 16, no. 3, pp. 130-142, April 2015.

- Silicore, Datasheet, Real-time Clock/Calendar D8563.

- J. Kimionis, A. Georgiadis, A. Collado and M. M. Tentzeris. Enhancement of RF Tag Backscatter Efficiency With Low-Power Reflection Amplifiers. IEEE Transactions on Microwave Theory and Techniques, vol. 62, no. 12, pp. 3562-3571, Dec. 2014.

- J. Kimionis, A. Georgiadis, Sangkil Kim, A. Collado, K. Niotaki and M. M. Tentzeris. An enhanced-range RFID tag using an ambient energy powered reflection amplifier. 2014 IEEE MTT-S International Microwave Symposium (IMS2014), Tampa, FL, USA, 2014, pp. 1-4.

- F. Farzami, S. Khaledian, B. Smida and D. Erricolo. Ultra-low power reflection amplifier using tunnel diode for RFID applications. 2017 IEEE International Symposium on Antennas and Propagation & USNC/URSI National Radio Science Meeting, San Diego, CA, USA, 2017, pp. 2511-2512.

- S. Khaledian, F. Farzami, D. Erricolo and B. Smida. A Full-Duplex Bidirectional Amplifier With Low DC Power Consumption Using Tunnel Diodes. IEEE Microwave and Wireless Components Letters, vol. 27, no. 12, pp. 1125-1127, Dec. 2017.

- F. Amato, C. W. Peterson, B. P. Degnan and G. D. Durgin. Tunneling RFID Tags for Long-Range and Low-Power Microwave Applications. IEEE Journal of Radio Frequency Identification, vol. 2, no. 2, pp. 93-103, June 2018.