3GPP TSG RAN WG1 #116                                                                                             R1-2400437

Athens, Greece, February 26th – March 1st, 2024

# Source:             CATT

Title:                  Discussion on general aspects of physical layer design

Agenda Item:    9.4.2.1

Document for:  Discussion and Decision

Introduction

In RAN#102 meeting, a new SID on Ambient IoT in NR was approved [1]. This study item will include the following objectives.

General Scope The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - X is to be decided in WGs. - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”. - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ. NOTE 1: It is to be understood that “≤ a few hundred µW” means WGs are not tasked with setting a particular value, and that it will be for WG discussions to determine if a presented design with corresponding power consumption satisfies the “≤ a few hundred µW” requirement. - Deployment Scenarios with the following characteristics, referenced to the tables in Clause 4.2.2 of TR 38.848: - Deployment scenario 1 with Topology 1 - Base station and coexistence characteristics: Micro-cell, co-site - Deployment scenario 2 with Topology 2 and UE as intermediate node, under network control - Base station and coexistence characteristics: Macro-cell, co-site - The location of intermediate node is indoor - FR1 licensed spectrum in FDD. - Spectrum deployment in-band to NR, in guard-band to LTE/NR, in standalone band(s). - Traffic types DO-DTT, DT, with focus on rUC1 (indoor inventory) and rUC4 (indoor command). - From RAN#104, the study will assess whether the harmonized air interface design (per bullet ‘A’ above) can address the DO-A (Device-originated autonomous) use case, only to identify which part(s) of the harmonized air interface design (per bullet ‘A’ above) is/are not sufficient for the DO-A use case. Transmission from Ambient IoT device (including backscattering when used) can occur at least in UL spectrum. - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. 2. Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1. - RAN2-led: - Study and decide which functions are needed for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission, and study those functions. For example: - Paging - Random access - Data transmission, including necessary radio resource control aspects, respecting the limitation in the General Scope - Interactions with upper layers For functionalities not listed above, they are studied only if found essential. - RAN3-led: - Identify necessary impacts on signaling and procedures for CN-RAN interface, to enable: - Paging - Device context management - Data transport - Identify RAN architecture aspects, including whether support for split architecture is necessary. - Identify potential solutions for locating an Ambient IoT device with no specification impact, e.g. reusing existing user location report, or minimal specification impact to convey location information to core network. - RAN4-led: - Coexistence study of Ambient IoT and NR/LTE. - RF requirements study for Ambient IoT: - Ambient IoT BS transmission and reception - Ambient IoT Device, as per the General Scope, transmission and reception - Intermediate node (UE), as per the General Scope, transmission and reception RAN2 and RAN3 are expected to identify RAN-CN functional split in coordination with SA2. Note: This study shall target for an IoT segment well below the existing 3GPP IoT technologies, e.g. NB-IoT, eMTC, RedCap, etc. The study shall not aim to replace existing 3GPP LPWA technologies.

In this contribution, we discuss the general aspects of physical layer design for NR ambient IoT communication system, including waveform, modulation, numerologies, bandwidths, multiple access, and channel coding.

Physical layer design for A-IoT

A-IoT communication characteristic for NR system design

According to the objectives of Rel-19 Ambient IoT study [1], two types of A-IoT devices with energy storage capability were identified. With energy harvesting, Type-1 battery-less A-IoT device and Type-2 A-IoT device support peak power consumption of ~1 µW and ≤ a few hundred µW, respectively. The NR physical layer design for A-IoT communication should take into the consideration of the A-IoT device characteristics.

The study of the Rel-19 A-IoT device is a responsive device and not an interactive device with triggering the UL transmission autonomously. In Rel-19 A-IoT study [1], the A-IoT device would support traffic types DT (Device Termination), DO-DTT (Device Origination-Device Transmission Triggered) only without DOA (Device Origination Autonomously). The NR physical layer should include the “interrogation” signals triggering mechanism for the responsive communication for Rel-19 A-IoT system design. The gNB or UE intermediate node transmits the DL “interrogation” signals to the A-IoT devices for network access control (DT) and triggering the device response to the control information (DO-DTT). As shown in Figure 1, the responsive A-IoT devices would backscatter/self-generate the carrier wave as the response signals back to the receiver. The response carrier wave arrives at the receiver within time D (in the range of µ seconds based on round trip propagation delay and device buffer time, where 1 µs Propagation delay = 300 m round trip distance). Duplex interference would be expected within CP with current NR SCS if the buffer time is not long. A-IoT devices would respond to the interrogation signal based on the pre-configured/pre-programed functions. The pre-configured/programed functions should be common for Type-1 and Type-2 devices. Additional functions could be supported by Type-2 devices with independent Tx power.

Figure 1: Interrogation signals triggering UL transmission of A-IoT device

For minimized difference in the common physical layer design for different A-IoT devices, DL interrogation signal/UL transmission should have unified/common design for Type-1 and Type-2 devices. In addition to the content of the control information and the associated format of the interrogation signal, DL interrogation channel/signal should have common design on the waveform, modulation, channel coding and DL synchronization signal for Type-1 and Type-2 devices. Regardless of whether carrier wave is generated internally or provided externally, UL transmission should have common design on waveform, modulation, channel coding and UL synchronization signal for Type-1 and Type-2 A-IoT devices. Considering that the complexity target is to be orders-of-magnitude lower than NB-IoT for Type-2 device with internal UL signal generation capability.

Proposal 1: The A-IoT device would support traffic types DT (Device Termination), DO-DTT (Device Origination-Device Transmission Triggered) only without DO-A (Device Origination Autonomously). The NR physical layer should include the “interrogation” signals triggering mechanism for the responsive communication triggering for Rel-19 A-IoT system design.

- The gNB or the intermediate node UE transmits the “interrogation” signals with the control information to the A-IoT devices.

- The A-IoT devices would respond to the control information in the interrogation signal based on the pre-configured/pre-programed functions by modulated the response information on the carrier wave provided externally from the transmitter or self-generated internally.

Proposal 2: DL interrogation signal should have unified/common design on waveform, modulation, channel coding, DL synchronization signal and the content of the interrogation signal for Type-1 and Type-2 devices.

Proposal 3: UL transmission should have unified/common design on waveform, modulation, channel coding and UL synchronization signal for Type-1 and Type-2 devices.

Wave form and modulation

The DL signal for the support of A-IoT devices should include DL interrogation signal and DL carrier wave provided to the A-IoT devices for the response. The UL signal should include the UL synchronization and response signals by backscattering or internal generation carrier wave. More details on design of DL/UL signal can be found in [4]. The design criteria of waveform and modulation for DL/UL channel/signal are discussed as follows.

Modulation and waveform for DL interrogation channel/signal

The low-power consumption passive or active devices could not have the component with high power consumption.   The waveform and modulation scheme for the A-IoT device should have simple RF envelop detector and modulation scheme. The complex and high power consumption components for digital signal processing, such as multi-bits ADC and high precision oscillator, are not feasible for the A-IoT device. A-IoT device would require higher power consumption in demodulating FSK modulation. Thus, ASK waveform/modulation could be considered for DL interrogation signal, as shown in Figure 2. Another candidate of waveform is OOK signal, e.g., OOK-1, which was discussed in low power WUS topic [3], as shown in Figure 3. ASK waveform has lower PAPR over OOK waveform, which is beneficial to improve the reliability of interrogation signal reception. Compared with ASK waveform, OOK waveform can have simple detector at the A-IoT device and reduce the required guard band for inter-channel interference.

Figure 2: ASK based interrogation signal transmission

Figure 3: OOK based interrogation signal transmission

Proposal 4: The following waveform/modulation candidates can be studied for DL interrogation channel/signal:

- Alt1: ASK waveform

- Alt2: OOK signal

Observation 1: ASK wave form has lower PAPR over OOK wave form, which is beneficial to improve the reliability of interrogation signal reception.

Modulation and waveform for CW channel/signal

The response to the control information in the Interrogation signals for both Type-1 and Type-2 A-IoT devices would be modulated on the carrier wave. The carrier wave of the response signal could be provided externally for both Type-1 and Type-2 A-IoT devices or generated internally by Type-2 A-IoT device. The carrier wave is the continuous unmodulated sinusoidal waveform for modulating the response information on the RF waveform to be used to transmit back to the reader on the UL channel.

Proposal 5: The carrier wave should be the continuous unmodulated sinusoidal waveform for modulating the response information on the RF waveform to be used to transmit back to the reader on the UL channel.

The carrier wave is a single tone continuous sinusoidal waveform in the A-IoT system similar to that in traditional RFID system [5][6]. The incident single tone is backscattered/transmitted by A-IoT devices to the gNB or the intermediate node UE after UL modulation. For single tone based carrier wave, the processing at the receiver is relatively simple but limited energy is provided to the devices depending on the BW of the carrier wave.

The transmission power of the carrier wave in UL would be restricted by the regulation. Multiple single tone carrier waves could provide higher total power to the A-IoT devices. The gNB transmits multiple single tones at different frequency positions and receives the backscattered signal corresponding to all/partial carrier waves based on A-IoT devices capability, respectively. The complexity of the A-IoT device would increase exponentially with multiple single tone carrier wave reception with the wideband matching network and RF bandpass filter for each band. The consideration of multiple single tone carrier waves should be excluded from Rel-19 study.

Proposal 6: The waveform of the carrier wave to be studied in Rel-19 A-IoT should focus on single tone continuous sinusoidal waveform.

Proposal 7: The consideration of multiple single tone carrier waves should be excluded from Rel-19 A-IoT study.

Modulation and waveform for UL channel/signal

UL transmission is enabled by passive A-IoT devices with backscatter modulation. The passive A-IoT device transmits UL data to gNB/UE intermediate node by adjusting the reflection coefficient of antenna with variation of its load impedance to reflect/modulate carrier wave, as shown in Figure 4. The complex reflection coefficient “”, which accounts for the impedance mismatch between the load impedance () and the antenna impedance () with   being its conjugate [10]-[14], is given by:

Figure 4: Backscatter modulation

For simplicity, two-state modulation (L=2) is typically used by switching between two loads impedance Z1 and Z2.

The reflection coefficient can be switched between impedance matching and impedance mismatching states, respectively. With impedance matching, carrier wave is absorbed, and reflection coefficient  corresponds to bit 0; otherwise, carrier wave is reflected with reflection coefficient  corresponds to bit 1.

With reflection coefficientand incident signal, backscattered signal can be written as

Refection coefficient determines what fraction of the incident power on passive A-IoT devices is reflected. The signal power received by device from the gNB is given by Friis’ equation:

where  is the output power of the Tx node; and  are the antenna gains of the Tx node, and the A-IoT device, respectively; c is the speed of light in free space ; f is the frequency of the signal; d is distance between the transmitter and device; L is system loss factor. Then, backscattered signal power can be written as

Instead of initiating their own RF transmissions as conventional wireless systems, backscatter can modulate its information bits to different RF modulation waveforms by tuning the load impedance of the antenna. The mapping of the reflection coefficient of antenna and load impedance can be characterized by the Smith chart, as shown in Figure 5. ASK, PSK and FSK modulation can be achieved with appropriate adjustment of the reflection coefficient of the antenna.

Figure 5: Smith chart

ASK/PSK modulation

By adjusting the load impedance, the reflection coefficient could fall on the real axis of the Smith chart. Multi-state backscatter ASK modulation can be achieved by modulating carrier wave with different amplitudes. 2-ASK signal can be written as

Where  is the carrier wave; is backscattered signal;  is impedance switching periodicity (symbol period);  is the reflection coefficient corresponding to two states and high level is associated with rectangular pulse within period .

By tuning the load impedance, backscatter ASK modulation can be achieved by modulating carrier wave with different phase . When phase and 0 are corresponded to high level and low level, respectively, 2-PSK modulation has similar operating with 2-ASK.

When carrier wave is single tone signal with frequency fc, spectrum of UL waveform with uncoded ASK and PSK modulation are shown in Figure 6 and Figure 7, respectively. We can see that ASK and PSK based backscattered signal have double sideband (DSB) spectrum with centre frequency fc and twice based band bandwidth, i.e. B=2/Ts.

Figure 6: 2-ASK spectrum (DSB)

Figure 7: 2-PSK spectrum (DSB)

Observation 2: ASK and PSK based backscattered signal have DSB spectrum with centre frequency fc and twice based band bandwidth, where fc is frequency of single tone carrier wave.

FSK modulation

FSK modulation is achieved according to switching between two distinct reflection coefficient values Γ0 and Γ1 with rate f0 corresponding to bit 0 and rate f1 corresponding to bit 1. To ensure orthogonality in non-coherent FSK, the spacing between the two modulation frequencies is f1-f0=k1Ts, k∈N*, where Ts is the symbol period.

As shown in Figure 8, the waveform of the reflection coefficient is a square wave with two amplitudes corresponding to the reflection coefficients determined by two loads impedances, and the frequency is the reflection coefficient switching rate f0.

Figure 8: The waveform of the reflection coefficients

The Fourier expansion of the reflection coefficient is as follows:

Гt=Γ1+Γ02+2(Γ1-Γ0)πn=1∞12n-1cos⁡(2π(2n-1)f0t)

The first harmonic component of the reflection coefficient is the largest except for the DC component. Thus, the carrier wave is modulated by utilizing the fundamental component in the square wave.

Soutt=ГtSint=Γ1+Γ02Sint+2(Γ1-Γ0)πSintcos⁡(2πf0t)

The spectral diagrams of FSK modulation for bit 0 with frequency shifting  f0 in the uplink transmission is shown in Figure 9. It can be observed that the backscatter FSK modulation uses double sideband which occupies more bandwidth compared with other modulation methods due to frequency shifting.

Figure 9: Spectral diagrams of DSB-FSK modulation

Single sideband FSK modulation (SSB-FSK) can be further studied to improve the spectrum utilization. SSB-FSK is modulated by ideal reflection coefficient ej2πfit which can not be achieved by switching load impendences [15]. A feasible approach is to use four complex reflection coefficients to simulate as shown in Figure 10. The spectral diagrams of FSK modulation with SSB is shown in Figure 11.

Figure 10: Complex reflection coefficients for SSB-FSK

Figure 11: Spectral diagrams of SSB-FSK modulation

The demodulation performance of DSB-FSK and SSB-FSK on TDL-C channel is shown in Figure 12. Obvious performance loss can be observed in SSB-FSK demodulation without combination gain compared with that of DSB-FSK. Further study on the tradeoff between spectrum overhead and demodulation performance of FSK is needed.

Figure 12: Demodulation performance for DSB-FSK and SSB-FSK

Observation 3: FSK based backscattered signal could have DSB spectrum or SSB spectrum.

Observation 4: DSB-FSK has better demodulation performance while poor spectrum utilization compared with SSB-FSK.

As reader has demodulation ability, ASK and PSK are widely used in tag modulation for RFID system [8]. FSK modulation is perfectly suitable for FDMA without performance degradation caused by self-interference. Considering common design should be supported for both Type-1 and Type-2 devices, ASK, PSK and FSK should be modulation candidates for UL transmission of A-IoT device. We also can see that UL spectrum is determined by carrier wave, modulation mode, and data rate. Then, for single tone carrier wave, ASK, PSK and FSK should be NR A-IoT UL waveform candidates.

Proposal 8: ASK, PSK and FSK should be modulation/waveform candidates for NR A-IoT UL channel/signal.

Receiver structure

For monostatic backscatter communications, the gNB receiving the backscattered signal from devices is the same as the gNB transmitting the CW to the A-IoT device. Figure 13 is an example for monostatic backscatter communication in Topology 1. The carrier wave is provided by a gNB. A-IoT devices then modulate and encode the received carrier wave before backscattering. Finally, the same gNB will receive and demodulate the backscatter signals based on the carrier wave.

Figure 13: Illustration of monostatic receiver structure

For bistatic backscatter communications, gNB receiving the backscattered signal from the A-IoT device is different from the gNB transmitting the CW to the A-IoT device as shown in Figure 14.

Figure 14: Illustration of bistatic receiver structure

When the transmission of backscattered signal and carrier wave are in the same spectrum, self-interference caused by carrier wave signal reflected by objects in the environment other than the A-IoT devices has impact on the backscattered signals. Due to the significantly higher carrier wave power received by a monostatic receiver compared to bistatic receiver, self-interference problem for monostatic backscatter communication is more obviously than that for bistatic backscatter communication. For bistatic receiver structure, although physical isolation of the transmitting and receiving antennas can alleviate interference, the interference of the carrier wave on backscatter signal from the direct link cannot be ignored. Self-interference cancelation should be studied in both bistatic backscatter communications and monostatic backscatter communication. Further discussion on self-interference issue is included in our contribution [7].

Proposal 9: Self-interference cancelation should be studied for both bistatic backscatter communication and monostatic backscatter communication.

Numerology design

The NR numerologies are defined based on the subcarrier spacing of the OFDM waveform and have been specified in FR1 and FR2 with the flexibility in different application under the different deployment scenarios. When the A-IoT communication is applied in NR system, the numerology design of A-IoT physical channel/signal need to be further considered including the following aspects:

- Reducing the signal processing complexity for A-IoT device.

- Minimizing the impacts to the NR signal transmission in the same band.

- Minimizing the inter-channel interference to the neighboring channels when multiplexing with the NR signal/channel.

- Supporting the large number of A-IoT devices in the area.

- The transmitted waveform of A-IoT carrier without OFDM spreading (no subcarrier spacing).

The numerology design for the NR A-IoT system should at least contain the time transmission interval (TTI) and A-IoT channel BW without OFDM spreading in relation to the NR subcarrier carrier space (SCS). The TTI of the A-IoT signals could be defined as the transmission time interval of the A-IoT signals carrying DL/UL information, including the interrogation signals and carrier wave in DL and the response signals in UL by backscattered/self-generated signals. The channel BW in relation to the NR SCS would be designed associated with the TTI and information bit rate of the DL and UL signals of NR A-IoT system.

With limited energy storage, passive A-IoT devices cannot process the interrogation signals with a long time duration. Thus, a TTI of the A-IoT signal would be shorter than the time duration of a NR symbol. For example, the length of an OFDM symbol is 71.43μs under the 15 kHz SCS in NR. If the interrogation signal is transmitted in a time duration of 71.43μs of one information bit per OFDM symbol, the A-IoT device would need to have enough power to process one bit of the DL signal of entire the OFDM symbol, which is a challenge for the A-IoT device due to its low energy storage capability. A TTI of 100 bits information would transmit over 100 symbols, which the span over 8 slots. In addition, the longer TTI of the interrogation signal would result in the larger number of state machine processing, which can increase the complexity of the signal processing for an A-IoT device. From the perspective of multiplexing with the NR signal/channel, when the interrogation signal is transmitted in NR band, the large time domain resource of the interrogation signal should not be allocated to avoid impacting the NR signal transmission in the same band and decreasing the NR spectrum efficiency. Thus, the length of the TTI of the DL A-IoT signals should be short enough in order to be transmitted within a short time, e.g. a few OFDM symbols.

In order for the common design for the DL signals/channels and the UL signals, the TTI of the “backscattered/self-generated” UL signals should be the same as that of DL interrogation signals.

The potential schemes of TTI design should be further considered as following:

- Option 1: The A-IoT dedicated TTI could be designed, which could be sub-multiple or multiple of the time duration of an OFDM symbol in NR FR1 band. The details need to be further studied.

- Option 2: The length of the TTI of the A-IoT DL channel could be one OFDM symbol with the scaling factor 2^k, k = 3, 4, 5, 6,…, in which is the multiple of sampling rate and could be reused by most of signal processing functions in the NR transmitter and receiver without additional signal processing.

Proposal 10: The numerology design for the A-IoT should at least contain the time transmission interval (TTI) and Channel BW without OFDM spreading in relation to the NR SCS.

Proposal 11: TTI (transmission time interval) of the A-IoT signals should be defined in the A-IoT system design.

Proposal 12: In order for the common design for the DL signals/channels and the UL signals, the TTI of the “backscattered/self-generated” UL signals should be the same as that of DL interrogation signals.

Proposal 13: The potential schemes of TTI design should be further considered as following:

- Option 1: The A-IoT dedicated TTI could be designed, which could be sub-multiple or multiple of the time duration of an OFDM symbol in NR FR1 band. The details need to be further studied.

- Option 2: The length of the TTI of the A-IoT DL channel could be one OFDM symbol with the scaling factor 2^k, k = 3, 4, 5, 6,…, in which is the multiple of sampling rate and could be reused by most of signal processing functions in the NR transmitter and receiver can be reused without additional signal processing.

The A-IoT channel BW in relation to the SCS would be designed associated with the TTI and data rate of the DL and UL signals of A-IoT, in which the details of channel bandwidth are discussed in Section 2.4.2. Furthermore, in order to support a large number of A-IoT devices, the system bandwidth of the A-IoT might be separated several sub-channel based on the SCS for multiple access, e.g. FDMA.

Proposal 14: The design of the channel BW in associated with NR SCS would be affected by the TTI and data rate of the DL and UL signals of A-IoT.

Bandwidth of NR A-IoT

For the bandwidth of the A-IoT communication, it would be considered from two aspects, including the system bandwidth configured for the A-IoT communication and the channel bandwidth used by the DL and UL channel/signals. The DL signals contain the interrogation signals in DL channel and the carrier wave, while the UL channel/signal is the backscattered/internal generated on the UL channel based on carrier wave provided by the transmitter externally or self-generated. The system bandwidth for the A-IoT communication is discussed in sub-section 2.4.1, while the channel bandwidth for the DL and UL channel/signals is analyzed in the sub-section 2.4.2.

The system bandwidth for the A-IoT communication

According to the study objective of A-IoT[1], the study of A-IoT system would focus on the deployment in FR1 licensed spectrum in FDD with in-band to NR, in guard-band to LTE/NR or in standalone band(s) deployment.

For the spectrum of the A-IoT communication deployed in guard-band to LTE/NR or in standalone band(s), the spectrum band only need to satisfy the requirements of the transmission bandwidth of the A-IoT signals as the description in Section 2.4.2 and the guard frequency gap for the coexistence of the A-IoT and LTE/NR systems, in which the detailed analysis is discussed in [16].

When the spectrum in-band to NR is considered for the A-IoT deployment, the spectrum deployments for the different A-IoT signals would be further discussed in the NR FDD system. Considering that the UL transmission backscattered/self-generation from A-IoT device could occur at least in UL spectrum [1], the potential spectrum deployment solutions are listed in Table 1.

Table 1: The potential spectrum deployment solutions for the A-IoT signals in NR FDD system

| The potential deployment solutions in NR FDD system | The spectrum deployment of the interrogation signal carrying the control information | The spectrum deployment of the carrier wave for the backscattered signals | The spectrum deployment of the transmission signal from Ambient IoT device |
| --- | --- | --- | --- |
| Option 1 | FDD DL band | FDD DL band | FDD UL band |
| Option 2 | FDD DL band | FDD UL band | FDD UL band |
| Option 3 | FDD UL band | FDD UL band | FDD UL band |
| Option 4 | FDD UL band | FDD DL band | FDD UL band |

For the Option 1 and Option 2, there are some non-negligible problems for the interrogation signal transmitted in DL band. Firstly, the A-IoT device is the ultra-low complexity device with ultra-low power consumption. When the interrogation signal is transmitted in the DL band multiplexed with NR DL channels/signals, there will be inter-channel interference between interrogation signal and NR DL signals/channels in the same band unless the interrogation signal multiplexed with NR DL channels/signals before IFFT. If the interrogation signal multiplexed with NR DL channels/signals before IFFT, the A-IoT device needs to have FFT processing and the sub-band filter to retrieve the interrogation signal from the multiplexed signals. If the interrogation signal multiplexed with NR DL channels/signals after IFFT, the ICI would be seen at the NR receiver and would affect the spectral efficiency of NR transmission. However, the regulation limitation is that the UE could not transmit on the DL band. If the interrogation is on the DL band, the interrogation signals could not be transmitted by intermediate node UE in Topology 2. Thus, the interrogation signal needs to be transmitted in NR FDD UL band as in the Option 3 and Option 4. For this deployment, the transmission details of the interrogation signal should be further discussed.

Observation 5: The interrogation signal needs to be transmitted in the FDD UL band.

When the CW is transmitted in FDD DL band, the A-IoT device should shift the frequency of CW into the FDD UL band to transmit the backscatter signal. The cheap frequency shifter might have the limit capability of shifting up to a few MHz. The frequency shifter in the A-IoT passive devices might not be capable of spectrum shifting 30-40 MHz from the DL spectrum to the UL spectrum for backscattered the response signal. According to the NR operating bands in FR1 specified in TS38.104, the minimum gap between DL band and UL band in FDD band spectrum is 45MHz in n8 [17]. For the A-IoT device without spectrum shifting capability, it is too hard to implement the spectrum shifting from the DL band to the UL band. If CW signal is transmitted in FDD DL band, the DL NR signal coexisting with carrier wave will be backscattered together with A-IoT signals in UL band, which may cause significant interference for UL transmission. Therefore, CW signal should be transmitted in FDD UL band. When the CW signal is transmitted in FDD UL band, the A-IoT devices can backscatter the signal based on the CW in the same band via the limited spectrum shifting capability.

Observation 6: If CW signal is transmitted in FDD DL band, the DL NR signal coexisting with carrier wave will be backscattered together with A-IoT signals in UL band, which may cause significant interference for UL transmission.

Proposal 15: The CW signal should be transmitted in FDD UL band for its continuous transmission.

Proposal 16: For the spectrum deployment for the A-IoT signals in NR FDD system, Option 3 would be further studied.

| The potential deployment solutions in NR FDD system | The spectrum deployment of the interrogation signal carrying the control information | The spectrum deployment of the carrier wave (CW) for the backscattered signals | The spectrum deployment of the transmission signal from Ambient IoT device |
| --- | --- | --- | --- |
| Option 3 | FDD UL band | FDD UL band | FDD UL band |

The channel bandwidth for the A-IoT signals

The channel bandwidth of A-IoT signal is affected by the waveform, the modulation, the symbol period and the transmission rate of the signals, which can be expressed as follows

BW A-IoT = F( fc, fi, Tsymbol, R);

where

BW A-IoT is the channel bandwidth of A-IoT signal,

fc is the frequency of carrier wave for the backscattered signals,

fi is the modulated frequency for FSK, i = 0, 1,…,

Tsymbol is the symbol period,

R is the data transmission rate.

For the interrogation signals, the channel bandwidth is affected by its waveform, modulation and the symbol period. The bandwidth based on the different waveforms and modulations are provided as following:

- If the interrogation signal is the single-carrier signal with ASK modulation,

  - For DSB-ASK: Bandwidth = 2*BWsignal = 2/Tsymbol, where Tsymbol is the symbol period.

  - For SSB-ASK: Bandwidth = BWsignal = 1/Tsymbol, where Tsymbol is the symbol period.

- If the interrogation signal is OOK signal,

  - Bandwidth = BWOFDM.

The bandwidth of the CW only depends on its wave form as following:

- For the CW is single tone signal, the bandwidth of the signal is single frequency.

Assume that the CW is the single tone signal with the frequency fc, the bandwidth of the UL signals under different modulation schemes are provided in Table 2.

Table 2: The bandwidth of the backscatter signal under the different modulations

| Modulation | Bandwidth |
| --- | --- |
| ASK | Bandwidth = 2*BWsignal = 2/Tsymbol, Tsymbol is the transmission symbol period. Tsymbol = 1/(Ndata*R), R is data rate, Ndata is number of encoded bits per data bit, which is determined by channel coding rate. |
| PSK | Bandwidth = 2*BWsignal = 2/Tsymbol, Tsymbol is the transmission symbol period. Tsymbol = 1/(Ndata*R), R is data rate, Ndata is number of encoded bits per data bit, which is determined by channel coding rate. |
| DSB-FSK | Bandwidth = 2*f0+ 2*BWsignal = 2*f0+ 2/Tsymbol, Tsymbol is the transmission symbol period. Tsymbol = 1/(Ndata*R), R is data rate, Ndata is number of encoded bits per data bit, which is determined by channel coding rate. A-IoT device switches the reflection coefficient at frequency f0. |
| SSB-FSK | Bandwidth = f0+ BWsignal = f0+ 1/Tsymbol, Tsymbol is the transmission symbol period. Tsymbol = 1/(Ndata*R), R is data rate, Ndata is number of encoded bits per data bit, which is determined by channel coding rate. A-IoT device switches the reflection coefficient at frequency f0. |

With 5kbps maximum date rate [2] and 1/16 channel coding rate, bandwidth of the UL signals with ASK modulation is 160kHz ((5kbps/(1/16))*2=160kHz). Considering that coding rate is not very low in case of supporting maximum data rate, 180kHz bandwidth of the UL signals is sufficient for 5kbps maximum data rate.

Observation 7: The channel bandwidth of the A-IoT signal is affected by the wave form, the modulation, the transmission symbol period and the data rate of the signals, which can be expressed as following formula:

BW A-IoT = F ( fc, fi, Tsymbol, R);

where

BW A-IoT is the channel bandwidth of A-IoT signal,

fc is the frequency of carrier wave for the backscattered signals,

fi is the modulated frequency for FSK, i = 0, 1,…,

Tsymbol is the transmission symbol period,

R is the data rate.

Proposal 17: The channel bandwidth of the A-IoT signal would be jointly considered with the waveform, the modulation, the transmission symbol period and the data rate of the signals.

Proposal 18: When the target of maximum data rate is 5kbps, 180kHz bandwidth of the UL signal is sufficient.

Multiple access

For the A-IoT communication, the multiple access of the responsive communication for large number A-IoT devices can minimize the collision of the response signals backscattered/generated to the receiver. For example, in the initial access procedure, the A-IoT devices would be queried by the network or UE intermediate node through “initial access interrogation” signals with devices feedback of UE Identification and encoded security challenge. Furthermore, the UL transmission of backscattered/generated UL signals could be scheduled through the DL multiple access procedure.

The common multiple access (MA) schemes, such as TDMA, FDMA, CDMA and SDMA could be considered in the A-IoT communication. There are two types of the MA schemes, i.e. the network controlled MA scheme and UE autonomous transmission as the certain MA manner.

For the network controlled MA scheme, the DL control information in interrogation signals can be used to indicate the resource and time for the A-IoT devices to backscatter/generate UL signal to as the certain MA manner. The potential network controlled MA schemes are listed as following, in which the transmission resource allocation information of UL signal can be carried in the interrogation signals.

- For TDMA, the UL transmission time at a specific TTI of the A-IoT device is allocated in interrogation signal.

- For FDMA, the UL transmission frequency among multiple channels is allocated for the A-IoT device in interrogation signal.

- For CDMA, the UL transmission resource with a specific code is allocated for the A-IoT device in interrogation signal.

- For SDMA, the UL transmission is responded to the same spatial domain of the carrier wave as the indication in the interrogation signal.

For the UE autonomous transmission scheme, A-IoT device can backscatter/generate the UL signal based on the device decision algorithm, such as Aloha, slot aloha or carrier sense etc. In these algorithms, the A-IoT device would determine the transmission time based on the uncooperative multiple access protocol with collision avoidance protocol to improve the system throughput.

Proposal 19: The MA schemes, such as TDMA, FDMA, CDMA and SDMA, could be considered in the A-IoT communication.

Proposal 20: Either of the two types of MA schemes, i.e. the network controlled MA scheme and UE autonomous transmission as the certain MA manner, should be further studied.

Channel coding

For A-IoT devices, coverage design target is the maximum distance of 10 -50 m for indoor scenario [1].

With limited Tx power, channel coding scheme should be studied for improving detection reliability of both reader and device sides.

DL channel coding

For NR transmission, quasi-cyclic LDPC codes with BG#1 and BG#2 and CRC-aided Polar cods are mainly used for data channel encoding and control channel encoding, respectively. Detailed NR channel coding schemes for DL and UL are given in Table 3.

Table 3: NR channel coding schemes

| PDSCH/PUSCH | UCI | DCI |
| --- | --- | --- |
| BG#1 based LDPC | 1bit Repetition code | Polar code |
|  | 2bits Simplex code |  |
| BG#2 based LDPC | 3~11bits RM code |  |
|  | >=12bis Polar code |  |

However, the passive A-IoT device does not have the capability in support of digital demodulation/decoder operating with limitation of the power supply in both Type-1 and Type-2 device. Simple coding schemes can be considered for ASK or OOK based DL interrogation channel/signal transmission. Manchester Code is adopted in DL transmission of RFID and Rel-18 low-power WUS study [3][8]. As shown in Figure 15, Manchester code maps “0” to bits “0, 1” and “1” to bits “1, 0”, with each bit is transmitted in a fixed time and 1/2 coding rate. The transitions at the midpoint of a period signify 0 or 1, which can be used for information transmission and self-synchronization. Furthermore, waveform transition detection is performed in Manchester code demodulation, which could provide better performance over amplitude threshold comparison.

Figure 15: Manchester coding

BER performance of DL interrogation signal with Manchester coding is evaluated for AWGN channel. Single side band ASK modulation is assumed in the evaluation and more parameters are provided in Table 4.

Table 4: Evaluation assumption for DL channel coding

| Information bits length | 44bits |
| --- | --- |
| Modulation | SSB ASK |
| Carrier | 900 MHz |
| Bandwidth | 112K |
| Channel | AWGN |
| Channel coding | Manchester code |

Figure 16: DL Manchester coding on AWGN channel

Although Manchester coding requires to use double the bit time duration to decode one bit information, Manchester coding has better BER performance with more than 3 dB gain than that of uncoded system on AWGN channel at BER=10-3. For DL interrogation channel/signal, Manchester coding should be considered for as the channel coding scheme.

Observation 8: Manchester coding has better BER performance with more than 3 dB gain than that of uncoded system on AWGN channel.

Proposal 21: Manchester code should be considered as the channel coding scheme for DL interrogation channel/signal channel coding.

UL channel coding

The UL channel coding performed by NR A-IoT device should have very low encoding complexity. Simple waveform coding, such as Miller code (mapping “0” to bits “0, 0” or “0, 0”, mapping“1” to bits “0, 1” or “1, 0”) and FM0 (mapping “0” to bits “0, 1” or “1, 0”, mapping“1” to bits “0, 0” or “1, 1”) are adopted in RFID system, as shown in Figure 17 and Figure 18, respectively. To improve performance of basis Miller code, Miller-M codes with low coding rates are generated with basis Miller waveform multiplied by a square-wave at M times the symbol rate [9]. Manchester code, Miller code and FM0 code should be considered for UL transmission and their coverage performance should be evaluated. Channel coding indication information could be included in DL interrogation channel/signal.

Figure 17: Miller coding

Figure 18: FM0 coding

Proposal 22: Manchester code, Miller code and FM0 code should be considered as the channel coding scheme of A-IoT UL response signals and their coverage performance should be evaluated.

Proposal 23: Channel coding indication information could be included in DL interrogation channel/signal.

To reach coverage design target of maximum distance of 10 - 50 m, channel error correction codes can be considered. The convolutional encoding is performed with simple shift register and few XOR operating which is widely used in wireless communication and space communications, e.g., LTE control channel. For convolutional code, gNB/ UE intermediate node can adopt complex decoding algorithm, such as soft decision based Viterbi decoding or list-decoding algorithm to improve coverage performance of NR A-IoT device. Advanced channel coding has been proved to achieve the performance close to Shannon limit for large information size, such as Turbo code, LDPC code and Polar code. With information bits size less than 1000bits, polar codes can outperform LDPC codes, Turbo codes and convolutional codes in BLER performance. For short information block size, BG#2 based LDPC code is designed for NR data channel transmission with low coding rate. However, encoding of Polar code and BG#2 LDPC code require multiplication, XOR operating and large memory, which is a challenge for Type-1/2 A-IoT devices. In addition, the channel coding scheme requires to have all the information bits in a block before entering the encoder at the A-IoT device, which the signal processing required high power consumption.

Proposal 24: To reach coverage design target, channel error correction codes with the feasibility study of power consumption can be investigated for NR A-IoT UL transmission, e.g., convolutional codes, or other advanced channel error correction codes.

Conclusion

In this contribution, general aspects of physical layer design for NR Ambient IoT communication system are discussed. We have the following observations and proposals:

Proposal 1: The A-IoT device would support traffic types DT (Device Termination), DO-DTT (Device Origination-Device Transmission Triggered) only without DO-A (Device Origination Autonomously).  The NR physical layer should include the “interrogation” signals triggering mechanism for the responsive communication triggering for Rel-19 A-IoT system design.

- The gNB or the intermediate node UE transmits the “interrogation” signals with the control information to the A-IoT devices.

- The A-IoT devices would respond to the control information in the interrogation signal based on the pre-configured/pre-programed functions by modulated the response information on the carrier wave provided externally from the transmitter or self-generated internally.

Proposal 2: DL interrogation signal should have unified/common design on waveform, modulation, channel coding, DL synchronization signal and the content of the interrogation signal for Type-1 and Type-2 devices.

Proposal 3: UL transmission should have unified/common design on waveform, modulation, channel coding and UL synchronization signal for Type-1 and Type-2 devices.

Proposal 4: The following waveform/modulation candidates can be studied for DL interrogation channel/signal:

- Alt1: ASK waveform

- Alt2: OOK signal

Observation 1: ASK wave form has lower PAPR over OOK wave form, which is beneficial to improve the reliability of interrogation signal reception.

Proposal 5: The carrier wave should be the continuous unmodulated sinusoidal waveform for modulating the response information on the RF waveform to be used to transmit back to the reader on the UL channel.

Proposal 6: The waveform of the carrier wave to be studied in Rel-19 A-IoT should focus on single tone continuous sinusoidal waveform.

Proposal 7: The consideration of multiple single tone carrier waves should be excluded from Rel-19 A-IoT study.

Observation 2: ASK and PSK based backscattered signal have DSB spectrum with centre frequency fc and twice based band bandwidth, where fc is frequency of single tone carrier wave.

Observation 3: FSK based backscattered signal could have DSB spectrum or SSB spectrum.

Observation 4: DSB-FSK has better demodulation performance while poor spectrum utilization compared with SSB-FSK.

Proposal 8: ASK, PSK and FSK should be modulation/waveform candidates for NR A-IoT UL channel/signal.

Proposal 9: Self-interference cancelation should be studied for both bistatic backscatter communication and monostatic backscatter communication.

Proposal 10: The numerology design for the A-IoT should at least contain the time transmission interval (TTI) and Channel BW without OFDM spreading in relation to the NR SCS.

Proposal 11: TTI (transmission time interval) of the A-IoT signals should be defined in the A-IoT system design.

Proposal 12: In order for the common design for the DL signals/channels and the UL signals, the TTI of the “backscattered/self-generated” UL signals should be the same as that of DL interrogation signals.

Proposal 13: The potential schemes of TTI design can be further considered as following:

- Option 1: The A-IoT dedicated TTI could be designed, which could be sub-multiple or multiple of the time duration of an OFDM symbol in NR FR1 band. The details need to be further studied.

- Option 2: The length of the TTI of the A-IoT DL channel could be one OFDM symbol with the scaling factor 2^k, k = 3, 4, 5, 6,…, in which is the multiple of sampling rate and could be reused by most of signal processing functions in the NR transmitter and receiver can be reused without additional signal processing.

Proposal 14: The design of the channel BW in associated with NR SCS would be affected by the TTI and data rate of the DL and UL signals of A-IoT.

Observation 5: The interrogation signal needs to be transmitted in the FDD UL band.

Observation 6: If CW signal is transmitted in FDD DL band, the DL NR signal coexisting with carrier wave will be backscattered together with A-IoT signals in UL band, which may cause significant interference for UL transmission.

Proposal 15: The CW signal should be transmitted in FDD UL band for its continuous transmission.

Proposal 16: For the spectrum deployment for the A-IoT signals in NR FDD system, Option 3 would be further studied.

| The potential deployment solutions in NR FDD system | The spectrum deployment of the interrogation signal carrying the control information | The spectrum deployment of the carrier wave (CW) for the backscattered signals | The spectrum deployment of the transmission signal from Ambient IoT device |
| --- | --- | --- | --- |
| Option 3 | FDD UL band | FDD UL band | FDD UL band |

Observation 7: The channel bandwidth of the A-IoT signal is affected by the wave form, the modulation, the transmission symbol period and the data rate of the signals, which can be expressed as following formula:

BW A-IoT = F ( fc, fi, Tsymbol, R);

where

BW A-IoT is the channel bandwidth of A-IoT signal,

fc is the frequency of carrier wave for the backscattered signals,

fi is the modulated frequency for FSK, i = 0, 1,…,

Tsymbol is the transmission symbol period,

R is the data rate.

Proposal 17: The channel bandwidth of the A-IoT signal would be jointly considered with the waveform, the modulation, the transmission symbol period and the data rate of the signals.

Proposal 18: When the target of maximum data rate is 5kbps, 180kHz bandwidth of the UL signals is sufficient.

Proposal 19: The MA schemes, such as TDMA, FDMA, CDMA and SDMA, could be considered in the A-IoT communication.

Proposal 20: Either of the two types of MA schemes, i.e. the network controlled MA scheme and UE autonomous transmission as the certain MA manner, should be further studied.

Observation 8: Manchester coding has better BER performance with more than 3 dB gain than that of uncoded system on AWGN channel.

Proposal 21: Manchester code should be considered as the channel coding schemefor DL interrogation channel/signal channel coding.

Proposal 22: Manchester code, Miller code and FM0 code should be considered as the channel coding scheme of A-IoT UL response signals and their coverage performance should be evaluated.

Proposal 23: Channel coding indication information could be included in DL interrogation channel/signal.

Proposal 24: To reach coverage design target, channel error correction codes with the feasibility study of power consumption can be investigated for NR A-IoT UL transmission, e.g., convolutional codes, or other advanced channel error correction codes.

References

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR, Huawei, December 11th - December 15th, 2023.

- TR 38.848, Study on Ambient IoT (Internet of Things) in RAN, V0.3.0.

- TR38.869, Study on low-power wake up signal and receiver for NR, v1.1.1.

- R1-2400439, DL and UL Physical Channels/signals design in support of Ambient IoT devices, CATT, RAN1 #116, February 26th – March 1st, 2024.

- Kargas N, Mavromatis F, Bletsas A. Fully-coherent reader with commodity SDR for Gen2 FM0 and computational RFID [J]. IEEE Wireless Communications Letters, 2015, 4(6): 617-620.

- D. Bharadia, K. R. Joshi, M. Kotaru, and S. Katti. BackFi: High throughput wifi backscatter [C]. Conference on Special Interest Group on Data Communication, 2015: 283-296.

- R1-2400440, Discussion on the waveform characteristics of carrier-wave for the Ambient IoT device, CATT, RAN1 #116, February 26th – March 1st, 2024.

- ISO/IEC18000-6, Parameters for air interface communications at 860 MHz to 960 MHz General.

- ISO/IEC18000-63, Parameters for air interface communications at 860 MHz to 960 MHz Type C.

- J. Kimionis, A. Georgiadis, A. Collado and M. M. Tentzeris. Enhancement of RF Tag Backscatter Efficiency With Low-Power Reflection Amplifiers [J]. IEEE Transactions on Microwave Theory and Techniques, 2014, 62(12): 3562-3571.

- N. Van Huynh, D. T. Hoang, X. Lu, D. Niyato, P. Wang and D. I. Kim. Ambient Backscatter Communications: A Contemporary Survey [J]. IEEE Communications Surveys & Tutorials, 2018, 20(4): 2889-2922.

- T. Jiang et al.. Backscatter Communication Meets Practical Battery-Free Internet of Things: A Survey and Outlook [J]. IEEE Communications Surveys & Tutorials, 2023, 25(3): 2021-20513.

- M. Alhassoun. Spectrally Efficient Backscatter Systems: A Hardware-Oriented Survey [J]. IEEE Access, 11: 8332-8344.

- R. Reed, F. L. Pour and D. S. Ha. An Energy Efficient RF Backscatter Modulator for IoT Applications [C]. 2021 IEEE International Symposium on Circuits and Systems, 2021:1-5.

- V. Iyer, V. Talla, B. Kellogg, S. Gollakota, and J. Smith. Intertechnology backscatter: Towards internet connectivity for implanted devices[C]. Conference on ACM SIGCOMM, 2016: 356-369.

- R1-2400436, Study of the Ambient IoT devices architecture, CATT, RAN1 #116, February 26th – March 1st, 2024.

- TS38.104, Base Station (BS) radio transmission and reception, v17.0.0.