3GPP TSG RAN WG1 #116		                                                      R1-2400452

Athens, Greece, February 26th – March 1st, 2024

Source:	CATT

Title:	Maintenance on NR mobility enhancements

Agenda Item:	8.5

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide 3 TPs aiming at the remaining issues on Rel-18 NR mobility enhancements.

- Discussion

  - Application time of the TCI state and/or TCI-UL-State and SCS configuration

In TS 38.213[1], it has defined that “A UE can be provided by a MAC CE in a PDSCH reception on the serving cell [11, TS 38.321] a TCI-State and/or TCI-UL-State in LTM-dl-OrJointTCI-StateToAddModList and/or LTM-ul-TCI-ToAddModList indicating a unified TCI state [6, TS 38.214] for applicable receptions or transmissions on a candidate cell from the number of candidate cells”, however, the application time of the TCI state and/or TCI-UL-State in the cell switch command MAC CE and the SCS configuration for μ are so far not specified.

In Section 5.1.5 of TS38.214[3], the beam application time is specified for unified TCI framework as follows:

“When a UE configured with dl-OrJointTCI-StateList would transmit a PUCCH with positive HARQ-ACK or a PUSCH with positive HARQ-ACK corresponding to the DCI carrying the TCI State indication and without DL assignment, or corresponding to the PDSCH scheduled by the DCI carrying the TCI State indication, and if the indicated TCI State(s) is/are different from the previously indicated one(s), the indicated TCI-State(s) and/or TCI-UL-State(s) should be applied starting from the first slot that is at least  beamAppTime symbols after the last symbol of the PUCCH or the PUSCH, and if the UE receives more than one indicated TCI state for a CC/BWP to be applied starting from the first slot that is at least  beamAppTime symbols after the last symbol of the PUCCH or the PUSCH, the indicated TCI state carried in the latest DCI in time corresponding to positive HARQ-ACK value is applied. The first slot and the  beamAppTime symbols are both determined on the active BWP with the smallest SCS among the BWP(s) from the CCs applying the indicated TCI-State(s) or TCI-UL-State(s) that are active at the end of the PUCCH or the PUSCH carrying the positive HARQ-ACK.”

The application time of the TCI state/TCI-UL-State in cell switch command can be defined similar to the beam application time specified for unified TCI framework in Section 5.1.5 of TS38.214.

Proposal 1: Adopt the following TP#1 for Section 21 of TS38.213:

TP #1:

| Reason for change: | In Section 21 of TS38.213[1], the application time of the TCI state and/or TCI-UL-State in the cell switch command MAC CE is not specified. Also the SCS configuration for μ is not specified. tThe application time of the TCI state/TCI-UL-State in cell switch command can be defined similar to the beam application time specified for unified TCI framework in Section 5.1.5 of TS38.214. |
| --- | --- |
| Summary of change: | Specify the application time of the TCI state and/or TCI-UL-State in the cell switch command MAC CE and the SCS configuration for μ as those specified for unified TCI framework. |
| Consequences if not approved: | The application time of the indicated TCI-state and/or TCI-UL-State for UE and the SCS configuration for μ are ambiguous. |

21 L1/L2-triggered mobility procedures <Unchanged parts are omitted> A UE can be provided by a MAC CE in a PDSCH reception on the serving cell [11, TS 38.321] a TCI-State and/or TCI-UL-State in LTM-dl-OrJointTCI-StateToAddModList and/or LTM-ul-TCI-ToAddModList indicating a unified TCI state [6, TS 38.214] for applicable receptions or transmissions on a candidate cell from the number of candidate cells. The UE applies the TCI-State and/or TCI-UL-State, if indicated by the MAC CE, from a first slot that is beamAppTime-r17 after the last symbol of a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE, and μ is the SCS configuration for the carrier with the smallest SCS among the carrier(s) applying the beam indication. If the MAC CE triggers a PRACH transmission [11, TS 38.321], the UE applies the TCI-State for receptions on the candidate cell, and applies a spatial domain filter corresponding to the TCI-State or the TCI-UL-State for transmissions on the candidate cell, that are after the completion of the random access procedure associated with the PRACH transmission on the candidate cell and before a new TCI state is indicated for the candidate cell. <Unchanged parts are omitted>

  - Alignment of LTM-CSI-ReportConfig Reporting Settings

In TS38.331[2], as shown in the following, each trigger state in CSI-AperiodicTriggerStateList, i.e., each CSI-AperiodicTriggerState, is only associated with one LTM-CSI-ReportConfig Reporting Setting.

CSI-AperiodicTriggerState ::=       SEQUENCE {

associatedReportConfigInfoList      SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo,

...,

[[

ap-CSI-MultiplexingMode-r17         ENUMERATED {enabled}                                          OPTIONAL  -- Need R

]],

[[

ltm-AssociatedReportConfigInfo-r18  LTM-CSI-ReportConfigId-r18                                    OPTIONAL  -- Need R

]]

}

However, in Section 5.2.1 of TS 38.214[3], it says a list of LTM-CSI-ReportConfig Reporting Settings can be associated with a trigger state. The descriptions in these two parts should be aligned.

“Each trigger state in CSI-AperiodicTriggerStateList contains a list of associated CSI-ReportConfigs or LTM-CSI-ReportConfigs indicating the Resource Set IDs for channel and optionally for interference where a Resource Set for interference can only be present for a Report Setting given by a CSI-ReportConfig and a trigger state additionally contains one or more [csi-ReportSubConfigID] if the associated CSI-ReportConfig configured with a list of sub-configurations, as described in Clause 5.2.1.1".

Proposal 2: Adopt the following TP#2 in section 5.2.1 of TS38.214:

TP #2:

| Reason for change: | In TS38.331[2], each trigger state in CSI-AperiodicTriggerStateList is only associated with one LTM-CSI-ReportConfig Reporting Setting. However, in Section 5.2.1 of TS 38.214[3], multiple LTM-CSI-ReportConfig Reporting Settings could be associated with one trigger state. The description in these two specifications should be aligned. |
| --- | --- |
| Summary of change: | Define each trigger state in CSI-AperiodicTriggerStateList is only associated with one LTM-CSI-ReportConfig Reporting Setting for alignment with TS 38.331 |
| Consequences if not approved: | The number of LTM-CSI-ReportConfig Reporting Settings associated with a trigger state in CSI-AperiodicTriggerStateList is incorrectly defined. |

##### 5.2.1 Channel state information framework <Unchanged parts are omitted> For CQI, PMI, CRI, SSBRI, LI, RI, L1-RSRP, L1-SINR, CapabilityIndex, TDCP a UE is configured by higher layers with N≥1 CSI-ReportConfig Reporting Settings and/or [X≥1 LTM-CSI-ReportConfig Reporting Settings], M≥1 CSI-ResourceConfig Resource Settings and/or [Y≥1 LTM-CSI-ResourceConfig Resource Settings], and one or two list(s) of trigger states (given by the higher layer parameters CSI-AperiodicTriggerStateList and CSI-SemiPersistentOnPUSCH-TriggerStateList). Each trigger state in CSI-AperiodicTriggerStateList contains a list of associated CSI-ReportConfigs and, optionally, one associated LTM-CSI-ReportConfig indicating the Resource Set IDs for channel and optionally for interference where a Resource Set for interference can only be present for a Report Setting given by a CSI-ReportConfig and a trigger state additionally contains one or more [csi-ReportSubConfigID] if the associated CSI-ReportConfig configured with a list of sub-configurations, as described in Clause 5.2.1.1. Each trigger state in CSI-SemiPersistentOnPUSCH-TriggerStateList contains one associated CSI-ReportConfig [or LTM-CSI-ReportConfig], and a trigger state additionally contain one or more [csi-ReportSubConfigID] if the associated CSI-ReportConfig is configured with a list of sub-configurations, as described in Clause 5.2.1.1. <Unchanged parts are omitted>

  - TP #3 in TS38.214

Proposal 3: Adopt the following TP#3 in section 5.2.1.4.2 of TS38.214:

TP #3:

| Reason for change: | There is a typo in section 5.2.1.4.2 of TS38.214[3], where different SSBRI mentioned in this part are not RRC parameters, which should not be written in italic type. |
| --- | --- |
| Summary of change: | Change the italic type of different SSBRI in section 5.2.1.4.2 to the normal type. |
| Consequences if not approved: | A typo exists in section 5.2.1.4.2. |

##### 5.2.1.4.2 Report quantity configurations <Unchanged parts are omitted> If a UE is configured with a [LTM-CSI-ReportConfig], - if the UE is configured with [spCellInclusion], the UE shall report in a single reporting instance [noOfReportedRS-PerCell] different SSBRI for the current SpCell and each of the [nOfReportedCells] -1 candidate cells. Otherwise, the UE shall report in a single reporting instance [noOfReportedRS-PerCell] different SSBRI for each of the [noOfReportedCell] candidate cells, <Unchanged parts are omitted>

- Conclusions

In this contribution, we have discussed the remaining issues for Rel-18 mobility enhancement and proposed the following TPs:

Proposal 1: Adopt the following TP#1 for Section 21 of TS38.213:

TP #1:

| Reason for change: | In Section 21 of TS38.213[1], the application time of the TCI state and/or TCI-UL-State in the cell switch command MAC CE is not specified. Also the SCS configuration for μ is not specified. tThe application time of the TCI state/TCI-UL-State in cell switch command can be defined similar to the beam application time specified for unified TCI framework in Section 5.1.5 of TS38.214. |
| --- | --- |
| Summary of change: | Specify the application time of the TCI state and/or TCI-UL-State in the cell switch command MAC CE and the SCS configuration for μ as those specified for unified TCI framework. |
| Consequences if not approved: | The application time of the indicated TCI-state and/or TCI-UL-State for UE and the SCS configuration for μ are ambiguous. |

21 L1/L2-triggered mobility procedures <Unchanged parts are omitted> A UE can be provided by a MAC CE in a PDSCH reception on the serving cell [11, TS 38.321] a TCI-State and/or TCI-UL-State in LTM-dl-OrJointTCI-StateToAddModList and/or LTM-ul-TCI-ToAddModList indicating a unified TCI state [6, TS 38.214] for applicable receptions or transmissions on a candidate cell from the number of candidate cells. The UE applies the TCI-State and/or TCI-UL-State, if indicated by the MAC CE, from a first slot that is beamAppTime-r17 after the last symbol of a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE, and μ is the SCS configuration for the carrier with the smallest SCS among the carrier(s) applying the beam indication. If the MAC CE triggers a PRACH transmission [11, TS 38.321], the UE applies the TCI-State for receptions on the candidate cell, and applies a spatial domain filter corresponding to the TCI-State or the TCI-UL-State for transmissions on the candidate cell, that are after the completion of the random access procedure associated with the PRACH transmission on the candidate cell and before a new TCI state is indicated for the candidate cell. <Unchanged parts are omitted>

Proposal 2: Adopt the following TP#2 in section 5.2.1 of TS38.214:

TP #2:

| Reason for change: | In TS38.331[2], each trigger state in CSI-AperiodicTriggerStateList is only associated with one LTM-CSI-ReportConfig Reporting Setting. However, in Section 5.2.1 of TS 38.214[3], multiple LTM-CSI-ReportConfig Reporting Settings could be associated with one trigger state. The description in these two specifications should be aligned. |
| --- | --- |
| Summary of change: | Define each trigger state in CSI-AperiodicTriggerStateList is only associated with one LTM-CSI-ReportConfig Reporting Setting for alignment with TS 38.331 |
| Consequences if not approved: | The number of LTM-CSI-ReportConfig Reporting Settings associated with a trigger state in CSI-AperiodicTriggerStateList is incorrectly defined. |

##### 5.2.1 Channel state information framework <Unchanged parts are omitted> For CQI, PMI, CRI, SSBRI, LI, RI, L1-RSRP, L1-SINR, CapabilityIndex, TDCP a UE is configured by higher layers with N≥1 CSI-ReportConfig Reporting Settings and/or [X≥1 LTM-CSI-ReportConfig Reporting Settings], M≥1 CSI-ResourceConfig Resource Settings and/or [Y≥1 LTM-CSI-ResourceConfig Resource Settings], and one or two list(s) of trigger states (given by the higher layer parameters CSI-AperiodicTriggerStateList and CSI-SemiPersistentOnPUSCH-TriggerStateList). Each trigger state in CSI-AperiodicTriggerStateList contains a list of associated CSI-ReportConfigs and, optionally, one associated LTM-CSI-ReportConfig indicating the Resource Set IDs for channel and optionally for interference where a Resource Set for interference can only be present for a Report Setting given by a CSI-ReportConfig and a trigger state additionally contains one or more [csi-ReportSubConfigID] if the associated CSI-ReportConfig configured with a list of sub-configurations, as described in Clause 5.2.1.1. Each trigger state in CSI-SemiPersistentOnPUSCH-TriggerStateList contains one associated CSI-ReportConfig [or LTM-CSI-ReportConfig], and a trigger state additionally contain one or more [csi-ReportSubConfigID] if the associated CSI-ReportConfig is configured with a list of sub-configurations, as described in Clause 5.2.1.1. <Unchanged parts are omitted>

Proposal 3: Adopt the following TP#3 in section 5.2.1.4.2 of TS38.214:

TP #3:

| Reason for change: | There is a typo in section 5.2.1.4.2 of TS38.214[3], where different SSBRI mentioned in this part are not RRC parameters, which should not be written in italic type. |
| --- | --- |
| Summary of change: | Change the italic type of different SSBRI in section 5.2.1.4.2 to the normal type. |
| Consequences if not approved: | A typo exists in section 5.2.1.4.2. |

##### 5.2.1.4.2 Report quantity configurations <Unchanged parts are omitted> If a UE is configured with a [LTM-CSI-ReportConfig], - if the UE is configured with [spCellInclusion], the UE shall report in a single reporting instance [noOfReportedRS-PerCell] different SSBRI for the current SpCell and each of the [nOfReportedCells] -1 candidate cells. Otherwise, the UE shall report in a single reporting instance [noOfReportedRS-PerCell] different SSBRI for each of the [noOfReportedCell] candidate cells, <Unchanged parts are omitted>

References

- 3GPP TS38.213 v18.1.0.

- 3GPP TS38.331 v18.0.0.

- 3GPP TS38.214 v18.1.0.