3GPP TSG-RAN WG1 Meeting #116	  R1-2400501

Athens, Greece, February 16th – March 1st, 2024

Agenda Item:	9.1.3

Source:	Comba

Title:	Discussion on AI/ML for CSI compression

Document for:	Discussion

Introduction

After the RAN1#115 meeting, most of aspects have been studied for the CSI feedback enhancement. But Ran1 hasn’t reached a consensus on the recommendation of CSI compression for the normative network. the following aspects are the reasons:

- Trade-off between performance and complexity/overhead.

- Issues related to inter-vendor training collaboration.

- Other aspects that require further study/conclusion are captured in the conclusion of TR38.843. This contribution will continue studying other aspects on CSI compression.

======================= Start of text proposal to TR 38.843 v1.2.0 ==================== #### 6.2.2.8 Summary of Performance Results for CSI feedback enhancement The following aspects have been studied for the evaluation on AI/ML based CSI compression in Rel-18: ... - From the perspective of generalization over various scenarios (assuming 1 on 1 joint training), - It has been studied with corresponding observations on (with the metric of SGCS): - the scenarios including various deployment scenarios, various outdoor/indoor UE distributions, various carrier frequencies, and various TxRU mappings - the approach of dataset mixing (generalization Case 3) - It has been studied but is lack of observations on: - other aspects of scenarios - the approach of fine-tuning - ... - 8 Conclusions - ... CSI feedback enhancement: CSI compression sub use case: The performance benefit and potential specification impact were studied for AI/ML based CSI compression sub use case. Evaluation has been performed to assess AI/ML based CSI compression from various aspects, including performance gain over non-AI/ML benchmark, model input/output type, CSI feedback quantization methods, ground-truth CSI format, monitoring, generalization, training collaboration types, etc. Some aspects were studied but not fully investigated, including the options of CQI/RI calculation, the options of rank>1 solution. Performance gain over baseline and computational complexity in FLOPs are summarized in clause 6.2.2.8. Potential specification impact on NW side/UE side data collection, dataset delivery, quantization alignment between CSI generation part at the UE and CSI reconstruction part at the NW, CSI report configuration, CSI report format, pairing information/procedure and monitoring approach were investigated but not all aspects were identified. The pros and cons are analysed for each training collaboration types, and each training collaboration type has its own benefits and limitations in different aspects. The study has investigated the feasibility of the studied training collaboration types and necessity of corresponding potential RAN1 specification impact. However, not all aspects have been concluded. Both NW side and UE side performance monitoring were studied, some but not all aspects were concluded. From RAN1 perspective, there is no consensus on the recommendation of CSI compression for normative work. At least the following aspects are the reasons for the lack of RAN1 consensus on the recommendation of CSI compression for normative work: - Trade-off between performance and complexity/overhead. - Issues related to inter-vendor training collaboration. Other aspects that require further study/conclusion are captured in the summary above. ======================= End of text proposal to TR 38.843 v1.2.0 ====================

Discussion about CSI compression.

Quantization alignment

In order to perform normal data transmission, quantization after CSI compression is essential. Different quantization functions usually produce different quantization noises, and different algorithms need to be designed to deal with different quantization noises. There is no quantitative method of consensus between different vendors, which will lead to a poor performance of the CSI compression model. Table 1 shows the different quantization methods process the output of the CSI generation part in different ways, which may lead to CSI reconstruction performance degradation. Uniform quantization is used to train the model, and the performance of the model is tested by uniform quantization. The performance is stable. The non-uniform quantization method is used to test the performance of the model, and the results show that the performance of the model is very poor. Therefore, a standard for quantization methods is needed.

Table 1 Comparison of different quantization method

|  | Training method | Test method |  |
| --- | --- | --- | --- |
| Compression rate | Uniform quantization | Uniform quantization | Non-uniform quantization |
| 1/4 | 0.933 | 0.9380 | 0.049 |
| 1/8 | 0.8882 | 0.8951 | 0.036 |
| 1/16 | 0.8455 | 0.8537 | 0.028 |
| 1/32 | 0.7676 | 0.7736 | 0.041 |

Proposal 1:Develop standard for quantization methods to mitigate the effect of quantization noise on performance fading.

Model-finetune

In model training stage, we tend to use a data set collected from our predefined scenario. In model application stage, the model compresses the data according to the data characteristics of the preset scene. There are two problems in this way of application. There are huge data characteristic differences between  training data set Configuration#A and application data set Configuration#B. The difference will lead to model generalization performance degrade. More importantly, the system parameters of Configuration#A and Configuration#B are also different, e.g. Antenna numbers and carrier frequency. In this case, the AI model simply cannot be applied. Fig.1 shows the model fine-tuning process. The input is data set with configuration#A, and the output is also the data set with configuration#A. Encoder and Decoder are trained with the data set with Configuration#B.

Figure 1 Model fine-tuning process

Evaluation results on CSI compression

Quantization Alignment evaluation

Multi-vendor quantization application evaluation

In this section, we evaluate the quantization application. In application stage, we want to know if training the CSI generation part and the CSI reconstruction part separately has a significant impact on the model.  So we train two model with the same quantization method in complete separation, but we only load the encoder part in model A and the decoder part in model B. Table 2 shows the evaluation result. The combination performance is so poor, which means the same model and the same training method can not train the same model.

Table 2 Model performance of separate training

|  | Training Stage |  | Application Stage |
| --- | --- | --- | --- |
| Compression rate | Model A | Model B | Combination |
| 1/4 | 0.9107 | 0.9349 | 0.039 |
| 1/8 | 0.8812 | 0.8875 | 0.026 |
| 1/16 | 0.8407 | 0.8527 | 0.057 |
| 1/32 | 0.7397 | 0.7377 | 0.061 |

Observation 1: Same model and the same training method can not train the same model.

AI-aided dequantization scheme

To address the issue of quantization noise, we evaluate the deep learning performance in dequantization. The dequantization module is applied after the data is received at the BS side. The module can use fully connected layers, transformer layers, or convolutional layers.

Figure 2 Dequantization Block

- (a) 	(b)	 (c)

Figure 3 dequantization blocks(a)Full connection block (b)Transformer block (c) Convolutional block

The complexity of 3 dequantization block is shown in table3. If we don't use the quantization module, the performance of our SGCS model is 0.9. Compared to the other two modules, the CNN module has extremely low parameter count and computational complexity, but it leads to a sharp decrease in model performance. The parameter count of the DNN module is only 1/12 of the Transformer module, and the computational complexity is only about 1/6. However, its performance in quantization is better than the Transformer module. Both modules have certain quantization performance, which can effectively reduce the noise introduced by quantization.

Table 3 Comparison of dequantization block complexity

| Model | Parameters | FLOPs | SGCS (Baseline is 0.98) |
| --- | --- | --- | --- |
| No dequantization |  |  | 0.9 |
| CNN block | 6 | 0.7k | 0.8 |
| DNN block | 75.2K | 76.13K | 0.9465 |
| Transformer block | 900K | 431.25K | 0.9461 |

Proposal 2:Start to develop dequantization network to address the noise issue introduced by quantization.

Model fine-tuning evaluation

Table 4 Link Level Simulation Assumptions for CSI Compression model fine-tuning

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, OFDM |
| Multiple access | OFDMA |
| Channel model | TDL |
| Scenario | Dense Urban |
| Carrier Frequency | 4 GHz |
| Subcarrier spacing | 30kHz |
| Simulation bandwidth | 20MHz |
| Inter-BS distance | 200m |
| Channel model | According to TR 38.901 |
| Antenna setup and port layouts at gNB | 4 ports: (1, 2, 2, 1, 1, 1, 1), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 4 Rx: (1, 2, 2, 1, 1, 1, 1), (dH,dV) = (0.5, 0.5)λ |
| Channel estimation | Ideal |

In this section, we evaluate the model fine-tuning method. The pre-training model is based on CDL channel model and the details is in Appendix. We derived data set from a TDL channel, whose detailed information is in table 4. According to the figure 4, the low-complexity channel data contains fewer data features than the high-complexity channel data. Therefore, it is possible to fine-tune a low-complexity channel-appropriate model with good performance from a model trained on high-complexity data.

Figure 4 model fine-tuning evaluation result

Observation 2:After fine-tuning, model trained with complex channel data can be applied to simple channel.

Proposal 3: Develop deep learning models for complex channels.

Proposal 4: Investigate fine-tuning methods for models applied to complex channels.

Conclusions

In this contribution, we discussed the other aspects on CSI compression. We give some conclusions and suggestions for the proposed problems, which are detailed as follows:

Observation 1: Same model and the same training method can not train the same model.

Observation 2:After fine-tuning, model trained with complex channel data can be applied to simple channel.

Proposal 1:Develop standard for quantifying problems to mitigate the effect of quantization noise on performance fading.

Proposal 2:Start to develop dequantization network to address the noise issue introduced by quantization.

Proposal 3: Develop deep learning models for complex channels.

Proposal 4: Investigate fine-tuning methods for models applied to complex channels.

- References

RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm (Moderator), RAN Meeting #102, Edinburgh, Scotland, Dec 11th - 15th, 2023.

R1- 2303174, Evaluation on AI/ML for CSI feedback enhancement, Comba, RAN WG1 #112bis-e, Online, April 17th - 26rd, 2023.

R1-2308954, Evaluation on AI/ML for CSI feedback enhancement, Comba, RAN WG1 #114bis-e, Xiamen, China, Oct 9th - 13th, 2023.

- Appendix

Table :  Link Level Simulation Assumptions for CSI Compression

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, OFDM |
| Multiple access | OFDMA |
| Scenario | Dense Urban |
| Carrier Frequency | 4 GHz |
| Subcarrier spacing | 30kHz |
| Simulation bandwidth | 20MHz |
| Inter-BS distance | 200m |
| Channel model | According to TR 38.901 |
| Antenna setup and port layouts at gNB | 32 ports: (8.8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE | 4 Rx: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| Channel estimation | Ideal |