3GPP TSG RAN WG1 #116		R1-2400511

Athens, Greece, February 26th – March 1st, 2024

Source:                 TCL

Title:                     Discussion on remaining issues for other aspects of AI/ML for CSI compression

Agenda Item:       9.1.3.2

Document For:    Discussion

- Introduction

CSI compression is a sub-use case within the broader scope of AI/ML for CSI feedback enhancement. Its architecture comprises a two-sided AI/ML model, featuring an encoder implemented on the UE side and a decoder on the NW side. In terms of technical standard research, significant findings have been achieved through discussions held across multiple meetings, ranging from RAN1#109 to RAN1#115. These findings encompass the configurations, scenarios, and procedures of AI/ML model training, as well as potential standard impacts. In Release 19, CSI compression will persist as a study item, aiming to optimize the balance between performance and complexity/overhead. Additionally, it seeks to address and resolve challenges associated with inter-vendor training collaboration, in line with the objectives of the new WID on AI/ML for the NR air interface proposed in RAN#102 [1].

Objective of AI/ML for CSI feedback enhancement in Rel-19 Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain). NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. NOTE: Coordination with SA/SA WGs of the ongoing study/work as it may relate to their required work.

To enhance the benefits of AI/ML-based CSI compression on uplink overhead redundancy and system throughput, there are several unresolved issues from Rel-18 pertaining to other aspects of AI/ML for CSI compression. These include training collaboration types, a two-sided model pairing mechanism, performance monitoring procedures, intermediate KPIs calculation, CSI quantization, and resource/report configuration for CSI compression model inference, among others. This contribution addresses a portion of the aforementioned open issues. Additionally, we provide our perspectives on the conclusions of the study item concerning the AI/ML-based CSI compression sub-use case from the RAN1 viewpoint.

- Study on the Specification Impact on CSI Compression Model Inference

2.1 Resource Set (RS) and CSI Report Configuration

The following consensus was reached regarding the CSI-RS resource and CSI report configuration for the CSI compression sub-use case [2][3]:

Agreement in RAN1#112 In CSI compression using two-sided model use case, further study the necessity, feasibility, and potential specification impact of UE side data collection enhancement including at least - Enhancement of CSI-RS configuration to enable higher accuracy measurement. - Assistance information for UE data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc. - The provision of assistance information needs to consider feasibility of disclosing proprietary information to the other side. - Signaling for triggering the data collection In CSI compression using two-sided model use case, further discuss the necessity, feasibility, and potential specification impact for NW side data collection including at least: - Enhancement of SRS and/or CSI-RS measurement and/or CSI reporting to enable higher accuracy measurement. - Contents of the ground-truth CSI including: - Data sample type, e.g., precoding matrix, channel matrix etc. - Data sample format: scaler quantization and/or codebook-based quantization (e.g., e-type II like). - Assistance information (e.g., time stamps, and/or cell ID, Assistance information for Network data collection for categorizing the data in forms of ID for the purpose of differentiating characteristics of data due to specific configuration, scenarios, site etc., and data quality indicator) - Latency requirement for data collection - Signaling for triggering the data collection Agreement in RAN1#112bis-e The study of AI/ML based CSI compression should be based on the legacy CSI feedback signaling framework. Further study potential specification enhancement on - CSI-RS configurations (No discussion on CSI-RS pattern design enhancements) - CSI reporting configurations - CSI report UCI mapping/priority/omission - CSI processing procedures. - Other aspects are not precluded. Agreement in RAN1#113 In CSI compression using two-sided model use case, further study the applicability and potential specification impact for CSI configuration and report: - For network to indicate CSI reporting related information, gNB can indicate the UE with the one or more of following information: - Information indicating CSI payload size - Information indicating quantization method/granularity. - Rank restriction - Other payload related aspects - For UE determination/reporting of the actual CSI payload size, UE reports related information as configured by the NW

The CSI framework, as established in NR, encompasses the setting of CSI-RS resources and report settings, as well as the configuration of the UE with CSI-ReportConfig via RRC signaling. In the use case of CSI compression using a two-sided model, three types of CSI-RS resources associated with the reportQuantity are defined for CSI measurement. These include channel measurement with the first CSI-RS resource configuration, interference measurement with the second CSI-IM resource configuration, and another form of interference executed by the NZP-CSI-RS resource configuration. The information required by the AI/ML model would significantly differ from the traditional CSI framework. Moreover, the conventional CSI-RS resource configuration methodology has not defined the AI or other usage scenarios. Therefore, the method of pairing the AI/ML model with the reference signal configuration needs to be elucidated.

To cater to the specific feedback requirements of the AI/ML model, several methods can be employed to differentiate the AI/ML CSI compression requirement:

- A dedicated reporting quantity has been defined, which is utilized for the operation of the AI/ML-specific model.

- A dedicated reporting configuration is in place, which incorporates information specific to the AI/ML model.

- A dedicated measurement configuration is established, which includes information specific to the AI/ML model.

In prior meetings, several proposals were put forth suggesting that the UE employs multiple AI/ML models to compress the CSI information. For each AI/ML model used for CSI compression, the corresponding CSI-RS resource configuration and CSI reporting configuration would be clarified to align the UE with LCM operations.

Proposal 1: In the case of CSI compression using a two-sided model, the design of an AI/ML-specific CSI-RS resource and CSI reporting configuration that may be compatible with the traditional CSI reporting scenario should be considered in the following aspects:

- AI/ML-specific CSI-RS resource configuration for CSI compression

- AI/ML-specific fields in CSI-ReportConfig IE

- Dedicated report quantities and report configurations for AI

2.2 Priority rules concerning CSI report collision

The following agreements were reached on the priority rules pertaining to CSI report collision for the CSI compression sub-use case:

Agreement In CSI compression using two-sided model use case, further study the feasibility and methods to support the legacy CSI reporting principles including at least: - The priority rule regarding CSI collision handling and CSI omission - Codebook subset restriction - CSI processing Unit Agreement The study of AI/ML based CSI compression should be based on the legacy CSI feedback signaling framework. Further study potential specification enhancement on - CSI-RS configurations (No discussion on CSI-RS pattern design enhancements) - CSI reporting configurations - CSI report UCI mapping/priority/omission - CSI processing procedures. - Other aspects are not precluded.

At times, a UE device may be triggered with multiple CSI reports based on the CSI report configurations. In such instances, it becomes necessary for the UE device to ascertain the priority of these multiple CSI reports. This is to determine whether some CSI reports or portions of the CSI report part 2 should be disregarded when the uplink symbols/slots for multiple CSI reports collide in the temporal domain. In other words, when the UE device is triggered with multiple AI/ML-based CSI reports and non-AI/ML CSI reports, the priority for such AI/ML-based and non-AI/ML-based CSI reports also needs to be established.

In the NR system, a priority rule for CSI reporting has been established, which is associated with multiple factors such as temporal domain behavior (periodic/semi-persistent/aperiodic), CSI report content (whether or not carrying L1-RSRP or L1-SINR), serving cell index, reporting config ID, and so on. Two CSI reports are considered to be in collision if the time occupancy of the physical channels scheduled to carry CSI reports overlaps in at least one OFDM symbol and is transmitted on the same carrier. When a UE is configured to transmit two colliding CSI reports to the gNB, the priority rule will be applied as follows:

- If the y values differ between the two colliding CSI reports, the following rules apply, except in the case where one of the y values is 2 and the other is 3. Here, the parameter y denotes different types of CSI reports, such as y=0 indicating aperiodic CSI reports to be carried on PUSCH, y=1 indicating semi-persistent CSI reports to be carried on PUSCH, y=2 indicating semi-persistent CSI reports to be carried on PUCCH, and y=3 indicating periodic CSI reports to be carried on PUCCH.

- The CSI report with the higher priority value should not be transmitted by the UE.

- Alternatively, the two CSI reports are either multiplexed or one is dropped, depending on the priority values, as outlined in [TS38.213].

At present, AI/ML-based CSI compression has been deliberated multiple times in RAN1/RAN2. For AI/ML models to function properly, dedicated CSI reporting, including AI/ML model life cycle management (LCM), is required. Both non-AI/ML based CSI compression and AI/ML-based CSI compression or related data collection would coexist for a specific radio link. Given the constraints on UCI payload, a new priority rule might be necessary to manage AI/ML and/or non-AI/ML based CSI compression. Consequently, an amendment to the existing CSI reporting priority rule is anticipated.

The establishment of an AI/ML-specific priority rule is essential within the system, distinct from the non-AI/ML specific CSI reporting priority rule currently employed in the NR system. In addition to the factors influencing the value of legacy CSI reporting priority, certain supplementary information may be taken into account as determinants for the value of AI/ML-specific CSI reporting priority. These factors include:

- AI/ML LCM procedures, signifying that CSI reporting associated with different procedures possess varying priority values, such as model monitoring, inference, and training.

- AI/ML characteristics for CSI compression, indicating that CSI reporting associated with different features carries different priority values, such as the model structure for CSI compression and quantization.

- AI/ML CSI reporting contents, suggesting that different CSI reporting information holds varying priority values, such as Rank and CQI, and the codebook subset.

Observation 1: Certain additional information may impact the priority of AI/ML CSI reporting for CSI compression:

- AI/ML LCM procedure

- AI/ML characteristics for CSI compression

- AI/ML reporting contents

- Other factors that may be pertinent to the value of AI/ML-specific priority should not be precluded.

Proposal 2: The definition of AI/ML-specific priority for CSI reporting in relation to CSI compression should be considered in comparison with the traditional CSI priority rules.

Given that both AI/ML CSI reporting and non-AI/ML CSI reporting for CSI compression are likely to coexist in the NR system, when an uplink resource collision occurs, the final priority between AI/ML-based and non-AI/ML-based should be determined. The approach to this determination is as follows:

- Option 1: The priority of AI/ML CSI is always higher than that of non-AI/ML CSI reporting for CSI compression. This implies that non-AI/ML can only be transmitted after the AI/ML CSI, and the CSI priority is determined by the specific priority value within either AI/ML CSI reporting or non-AI/ML CSI reporting.

- Option 2: The priority of AI/ML CSI is always lower than that of non-AI/ML CSI reporting for CSI compression. This implies that AI/ML can only be transmitted after the non-AI/ML CSI, and the CSI priority is determined by the specific priority value within either AI/ML CSI reporting or non-AI/ML CSI reporting.

- Option 3: If both AI/ML CSI and non-AI/ML CSI share the same priority value, the AI/ML CSI will be transmitted first. However, if the priority values differ, the transmission priority is determined by the respective priority value.

- Option 4: If both AI/ML CSI and non-AI/ML CSI share the same priority value, the non-AI/ML CSI will be transmitted first. However, if the priority values differ, the transmission priority is determined by the respective priority value.

Proposal 3: When the UE supports both AI/ML and non-AI/ML CSI reporting, it is necessary to redefine the priority rule considering different types of CSI reporting.

2.3 Mechanism for CSI Compression Processing with Limited UE Capability and Dedicated Timing

For traditional CSI measurement processing criteria, the concept of a CSI Processing Unit (CPU) is introduced to denote the capability of supported simultaneous CSI calculations implemented in a UE. This means that if a UE supports simultaneous CSI calculations, it is said to possess CSI processing units for processing CSI reports. Similarly, for AI/ML models implementing CSI measurement operations, the concept of an AI/ML-specific CPU can also be introduced. However, the requirements for an AI/ML-specific CPU differ significantly from those of a legacy CPU due to the distinct requirements of UE capabilities to implement AI/ML-specific and traditional CSI measurement operations. When AI/ML models are applied to the CSI measurement process for CSI prediction, channel information feedback, beam management, and positioning, the resources (including computing units, memory, storage, and other hardware resources) consumed by AI/ML models differ from those required by traditional CSI computation methods. Consequently, the current CPU and Occupied CPU quantified based on non-AI/ML models are unsuitable for AI/ML models. It is essential to redefine the relevant parameters specifically for AI/ML models.

The utilization of AI/ML models for CSI measurements and calculations may pose challenges to the capabilities of a UE to implement AI/ML models for inference. Firstly, the size of AI/ML models may be substantial due to the complex AI/ML model structures and a large number of AI/ML proprietary parameters, including hyperparameters, learnable parameters, and other types of parameters specific to an AI/ML model. Consequently, to facilitate the running of AI/ML models for inference in a UE, a significant amount of the UE's memory resources must be occupied, which may surpass the available memory resources of the UE specific to AI/ML model operation. Secondly, the computation requirements (e.g., FLOPs) of AI/ML models may be too high for a UE to support. This implies that running AI/ML models for inference in a UE may consume computational resources to a large extent. Lastly, running AI/ML models for inference in a UE may lead to substantial power consumption, which may exceed the tolerable limit of the UE's power-saving requirements.

Observation 2: The memory and computational resources as well as the power consumption may become bottlenecks for UE when implementing AI/ML models for CSI compression and restoration.

Proposal 4: The study of how to describe the capabilities of a UE to implement AI/ML models for inference on CSI compression and calculations should be undertaken.

Upon receipt of the CSI request, which instigates CSI measurements and calculations in the UE via DCI transmitted through PDCCH, the UE commences these measurements and calculations. It then conveys the CSI measurement results to the gNB via the PUSCH/PUCCH, the time slot for which has been predetermined. To ensure the most recent CSI measurement data is reported to the gNB, the CSI measurements and calculations must be completed prior to the first uplink symbol that carries the corresponding CSI report(s), taking into account the CSI computation time.

In the case of CSI measurements and calculations that employ AI/ML models for inference, the estimation of CSI computation time necessitates a reconfiguration for AI/ML. This differs from traditional CSI measurement and calculation operations, which can be executed immediately without any preliminary steps. AI/ML models, on the other hand, require activation within the UE before the implementation of inference, a process that may consume a certain amount of time. Moreover, the size of the AI/ML models may affect the model inference time.

The total operation time for the AI/ML model in CSI computation comprises two durations: the time taken for model activation (if applicable) and the time taken for model inference. This could result in different outcomes for the handling of CSI measurements and calculations (either providing a valid CSI report or disregarding the scheduling request) compared to the time duration from the last symbol of the DCI carrying the CSI request to the first uplink symbol carrying the corresponding CSI report(s).

Observation 3: The computational time for CSI restoration at NW side should be estimated and reconfigured to the UE. This may include the time duration for both model activation and model inference.

- Conclusions

This contribution has led to the following observations:

Observation 1: Certain additional information may impact the priority of AI/ML CSI reporting for CSI compression:

- AI/ML LCM procedure

- AI/ML characteristics for CSI compression

- AI/ML reporting contents

- Other factors that may be pertinent to the value of AI/ML-specific priority should not be precluded.

Observation 2: The memory and computational resources as well as the power consumption may become bottlenecks for UE when implementing AI/ML models for CSI compression and restoration.

Observation 3: The computational time for CSI restoration at NW side should be estimated and reconfigured to the UE. This may include the time duration for both model activation and model inference.

Furthermore, the following proposals have been put forward:

Proposal 1: In the case of CSI compression using a two-sided model, the design of an AI/ML-specific CSI-RS resource and CSI reporting configuration that may be compatible with the traditional CSI reporting scenario should be considered in the following aspects:

- AI/ML-specific CSI-RS resource configuration for CSI compression

- AI/ML-specific fields in CSI-ReportConfig IE

- Dedicated report quantities and report configurations for AI

Proposal 2: The definition of AI/ML-specific priority for CSI reporting in relation to CSI compression should be considered in comparison with the traditional CSI priority rules

Proposal 3: When the UE supports both AI/ML and non-AI/ML CSI reporting, it is necessary to redefine the priority rule considering different types of CSI reporting.

Proposal 4: The study of how to describe the capabilities of a UE to implement AI/ML models for inference on CSI compression and calculations should be undertaken.

References

- RP-234039 New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface.

- Chairman notes of 3GPP TSG RAN WG1 #112b-e meeting.

- Chairman notes of 3GPP TSG RAN WG1 #112 meeting.