3GPP TSG-RAN WG1 Meeting #116	R1-2400512

Greece, Athens, Feb 26th - Mar 01st, 2024

Agenda Item:	9.2.2

Source:	TCL

Title:	Discussion on CSI enhancement

Document for:	Discussion and Decision

Introduction

In RAN #102 meeting, there are the following objectives about CSI enhancement are approved in the WID for Rel-19 NR MIMO [1]:

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

In this contribution, we discuss on CSI enhancement for supporting up to 128 CSI-RS ports on FR1 and UE reporting enhancement for CJT.

CSI enhancement for supporting up to 128 CSI-RS antenna ports

Aspect of general

According to the scope of  WID for Rel-19 NR MIMO, Type-I codebook, Type-II codebook, and CRI-based CSI reporting refinement supporting up to a total of 128 CSI-RS antenna ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS antenna ports per resource).

Obviously, when the number of antenna ports is fixed, there can be multiple configurations for CSI-RS resources. For example, based on the existing single CSI-RS resource support the number of antenna ports {2,4,8,12,16,24,32}, for 64 CSI-RS antenna ports, there can be multiple possible configurations for CSI-RS resources, including the following:

- 2 CSI-RS resources, each with 32 CSI-RS antenna ports.

- 4 CSI-RS resources, each  with 16 CSI-RS antenna ports.

- 8 CSI-RS resources, each with 8 CSI-RS antenna ports.

- 16 CSI-RS resources, each with 4 CSI-RS antenna ports.

- 32 CSI-RS resources, each with 2 CSI-RS antenna ports.

- 3 CSI-RS resources, the possible antenna port combinations corresponding to CSI-RS resources are {8, 24, 32}.

It is worth noting that the above example does not cover all possible combinations of CSI-RS resources.

: When the number of antenna ports is fixed,  there are multiple configurations for CSI-RS resource combinations.

Based on the above observation and considering the existing CSI-RS resource configuration constrains for multi-TRP scenarios,  multiple CSI-RS resources used for channel measurement in a CSI-RS resource set  have the same number of antenna ports. In order to simplify the configuration of CSI-RS resources for channel measurement, we consider that this constrain should be adhered to. Therefore, the following proposal is provided:

:  For Type-I codebook and Type-II codebook refinement, multiple CSI-RS resource for channel measurement in a CSI-RS resource set should have the same number of antenna ports

Moreover, according to the aforementioned analysis, even when the number of antenna ports is fixed and multiple CSI-RS resources have the same number of antenna ports, there are still be multiple possible configurations for CSI-RS resource combinations. Different combinations of CSI-RS resources may achieve the same channel measurement effect. Therefore, it is necessary to consider the constraints of CSI-RS resource configuration based on the actual possible antenna deployments. Considering that 128 antenna ports can be achieved by configuring four sets of 32 antenna ports of CSI-RS resources within a single resource set, we provide the following proposals:

:  For Type-I codebook and Type-II codebook refinement supporting up to a total of 128 CSI-RS antenna ports across all CSI-RS resources, the maximum number of  CSI-RS resource is 4.

- In current spec, a single CSI-RS resource supports a variable number of antenna ports, which can be {2, 4, 8, 12, 16, 24, 32}. Considering multiple CSI-RS resources used for channel measurement in a CSI-RS resource set  have the same number of antenna ports. We believed that the number of CSI-RS antenna ports P∈{36,48,64,72,96,128} across all CSI-RS resources should be supported.

:  For Type-I codebook and Type-II codebook refinement, it supports a total of {36, 48, 64, 72, 96} CSI-RS antenna ports across multiple CSI-RS resources.

Type-I codebook refinement

For a massive MIMO antenna array, increasing the number of RF chains while keeping the number of antenna elements constant helps improve the spatial degrees of freedom of the antenna array. Moreover, simultaneously increasing the number of RF chains and the number of antenna elements helps improve the spatial degrees of freedom and angular resolution of beamforming, since the beamwidth is related to the aperture of the antenna array.

For the legacy Type-I single-panel codebook, when the number of antenna ports is greater than 4 and rank =1 or 2, the beam number of L selected by UE is configurable: L∈{1,4}. As the spatial degrees of freedom increase and the beam angle resolution improves, we believe that it is necessary to increase the number of beams in beam group for wide-band in order to increase the accuracy of beamforming.

:  For Type-I codebook refinement , the number of  beams in beam group for rank 1/2 should be increased, e.g. L∈{1,4,6,8,12,16}.

Type-II codebook refinement

In current spec, Type-II codebooks include Rel-15 Type-II codebook,  Rel-15 Type-II port selection codebook，Rel-16 eType-II codebook, Rel-16 eType-II port selection codebook, Rel-17 FeType-II port selection codebook, Rel-18 eType-II codebook for CJT, Rel-18 FeType-II port selection codebook for CJT, Rel-18 eType-II codebook for predicted PMI, and Rel-18 FeType-II port selection codebook for predicted PMI. Considering the high computational complexity and significant feedback overhead associated with some existing Type-II codebooks, we suggest prioritizing the enhancement of the Rel-16 eType-II codebook and Rel-17 FeType-II port selection codebook.

:  For Type-II codebook refinement, giving priority to refinement of the Rel-16 eType-II codebook and Rel-17 FeType-II port selection codebook.

Moreover, for Type-II codebook refinement, we believe that the following alternatives for the compression scheme of CSI corresponding to multiple CSI-RS resources should be considered：

- Alt 1：Separate compression of CSI associated with each CSI-RS resource.

- For example, for Rel-16 eType-II codebook refinement, assuming that the number of CSI-RS resources for channel measurement is N, and Hn indicates the CSI associated with the n-th CSI-RS resource, where n∈{1,...,N}. Then, Hn=W1,nW2,nWf,nH, where W1,n indicates the spatial domain beam selection matrix, W2,n is the frequency domain basis selection matrix, and Wf,n denotes the projection coefficient matrix. This means that the CSI associated with each CSI-RS resource can be expressed with different codebook parameter configurations.

- Alt 2：Joint compression of CSI associated with all CSI-RS resources.

- For example, for Rel-16 eType-II codebook refinement, assuming that the number of CSI-RS resources for channel measurement is N, and Hn indicates the CSI associated with the n-th CSI-RS resource, where n∈{1,...,N}. Then, H=[H1T,...,HNT] T =W1W2WfH, where W1 indicates the spatial domain beam selection matrix, W2 represents the frequency domain basis selection matrix, and Wf is the projection coefficient matrix. In other words, the CSI corresponding to all CSI-RS resources can be jointly compressed using a set of codebook parameters.

:  For Type-II codebook refinement, CSI compression scheme, down-select from:

- Alt1: Separate compression of CSI associated with each CSI-RS resource;

- Alt2: Joint compression of CSI associated with all CSI-RS resources.

CRI-based CSI reporting refinement

Hybrid beamforming structure can significantly reduce the number of RF chains in large-scale antenna arrays, effectively achieving a trade-off between performance and cost-effectiveness. In current spec, each UE report the CRI corresponding to the best analog beam. If multiple UEs report different CRIs , the base station cannot schedule these UE simultaneously, which greatly reduces the possibility of UE pairing. Obviously, UE reporting multiple CRIs helps to increase the probability of UE pairing. However, the more CRIs reported, the higher the reporting overhead. Therefore, the number of CRIs to be reported by UE should be  given priority for discussion. For this problem, we believe that the following two options should be considered:

Option 1:The base station determines the number of CRIs to be reported by the UE.

Option 2: The UE determines the number of CRIs to be reported.

For each UE, the channel quality of the analog beam corresponding to each CSI-RS resource is known. The UE can decide whether to report the corresponding CRI based on the channel quality of the analog beam. Therefore, the number of CRIs reported is determined by the UE is more reasonable.

:  For CRI-based CSI reporting refinement, UE determines the number of CRIs reported.

UE reporting enhancement for CJT

- Multi-TRP CJT is supported in Rel-18 with the assumption of ideal synchronization and backhaul among TRPs. However, in practical deployment, the frequency/time synchronization and calibration between TRPs may be difficult to be handled. In RAN #102 meeting, it was agreed that specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD. Furthermore, in this meeting, the following consensus has been reached: Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH.

- In our view, inter-TRP time misalignment and frequency/phase offset reporting are similar to CSI reporting. In current spec, aperiodic CSI reporting on PUSCH is triggered by DCI. For non-ideal backhaul scenarios,  inter-TRP time misalignment and frequency/phase offset reporting may require multiple DCI triggers. However, for ideal backhaul scenarios, time misalignment and frequency/phase offset reporting only require single DCI trigger. In other words, ideal and non-ideal backhaul scenarios may require different specification schemes. Although the WID of Rel-19 NR MIMO has been agreed that specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul. Considering the practical network deployments, it is possible that there could be ideal backhaul but non-ideal synchronization  between TRPs, or inter-TRP time delay can be ignored. We recommend conducting separate studies for the two scenarios mentioned above.

:  For UE reporting enhancement for CJT, consider the following two cases:

- Case 1: Non-ideal synchronization and backhaul between TRPs

- Case 2: Non-ideal synchronization but ideal backhaul between TRPs

Conclusions

This contribution provides our views on CSI enhancement for supporting up to 128 CSI-RS ports on FR1 and UE reporting enhancement for CJT.

: When the number of antenna ports is fixed,  there are multiple configurations for CSI-RS resource combinations.

:  For Type-I codebook and Type-II codebook refinement, multiple CSI-RS resource for channel measurement in a CSI-RS resource set should have the same number of antenna ports

:  For Type-I codebook and Type-II codebook refinement supporting up to a total of 128 CSI-RS antenna ports across all CSI-RS resources, the maximum number of  CSI-RS resource is 4.

:  For Type-I codebook and Type-II codebook refinement, it supports a total of {36, 48, 64, 72, 96} CSI-RS antenna ports across multiple CSI-RS resources.

:  For Type-I codebook refinement , the number of  beams in beam group for rank 1/2 should be increased, e.g. L∈{1,4,6,8,12,16}.

:  For Type-II codebook refinement, giving priority to refinement of the Rel-16 eType-II codebook and Rel-17 FeType-II port selection codebook.

:  For Type-II codebook refinement, CSI compression scheme, down-select from:

- Alt1: Separate compression of CSI associated with each CSI-RS resource;

- Alt2: Joint compression of CSI associated with all CSI-RS resources.

:  For CRI-based CSI reporting refinement, UE determines the number of CRIs reported.

:  For UE reporting enhancement for CJT, consider the following two cases:

- Case 1: Non-ideal synchronization and backhaul between TRPs

- Case 2: Non-ideal synchronization but ideal backhaul between TRPs

- References

[1]    RP-234007, New WID: NR MIMO Phase 5, Samsung, RAN#102, Dec 2023