3GPP TSG-RAN WG1 Meeting #116	R1-2400513

Greece, Athens, Feb 26th - Mar 01st, 2024

Agenda Item:	9.2.3

Source:	TCL

Title:	Discussion on SRI/TPMI enhancement for enabling 3TX UL transmission

Document for:	Discussion and Decision

Introduction

In RAN #102 meeting, there is the following objective to enable 3 TX codebook-based UL transmission in the WID for Rel-19 NR MIMO [1]:

- Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource - Note: UL full power transmission mode 1 and 2 are not supported.

In this contribution, we discuss non-coherent codebook design and SRI/TPMI enhancement for enabling 3TX codebook-based UL transmission.

Potential enhancement

According to the scope of the WID for Rel-19 NR MIMO, in this section, our views regarding non-coherent codebook design and SRI enhancement are provided for enabling 3TX UL transmission.

Non-coherent codebook

For a non-coherent UE, it can only perform multi-layer transmissions with multiple antennas in a non-coherent manner, where each antenna corresponds to one layer. Therefore, each precoding matrix for each layer in a non-coherent codebook shall be a port selection precoding matrix.

In view of this, for PUSCH transmission with different waveforms, the precoding matrix for different ranks can be expressed as shown in Table 1.

Table 1 Port selection precoding matrix for 3TX non-coherent UL transmission

| Rank value | The number of codeword | Codeword | Applicable waveforms |
| --- | --- | --- | --- |
| 1 | 3 | 13100，13010，13001 | DFT-s-OFDM & CP-OFDM |
| 2 | 3 | 131 00 010，13100 001，13010 001 | CP-OFDM |
| 3 | 1 | 131 00 010 001 | CP-OFDM |

: For codebook-based 3TX UL transmission, port selection precoding matrix in Table 1 should be included in non-coherent codebook.

For the non-coherent codebook of 2/4 TX for CP-OFDM waveform, the  precoding information and number of layer are indicated jointly, and the number of bits determined by the total number of precoding matrix across all ranks. We believe that the precoding information and number of layer indication for non-coherent codebook for 3 TX UL transmission should also adhere to the above method. Therefore, for codebook-based 3TX UL transmission, joint indication of TPMI and TRI, and the number of bits determined by the total number of precoding matrix across all ranks. For example, as shown in Table 1, the total number of codewords across all ranks is 7, and the joint indication TPMI and TRI requires 3 bits.

: For codebook-based 3TX UL transmission, joint indication of TPMI and TRI, and the number of bits determined by the total number of precoding matrix across all ranks.

SRI enhancement

To support  codebook-based 3TX UL transmission, one main aspect to consider is the configuration of SRS resources for channel measurement. This is particularly important because the current SRS resource configuration only support {1,2,4,8} antenna ports. According to the restriction ‘without enhancement on SRS resource’ of the WID for Rel-19 NR MIMO, we believe the following options for SRS resource configuration should be considered to support codebook-based 3TX UL transmission:

- Option 1: Configuration of an SRS resource set containing up to X 4-port SRS resource(s), where X = 2;

- Option 2: Configuration of at least one SRS resource set, configured with more than one SRS resources where each SRS resource may have the same or different number of SRS ports, for example,

- Option 2.1: Configuration of an SRS resource set, configured with at least 3 of 1-port SRS resources.

- Option 2.2:  Configuration of an SRS resource set, configured with at least 1 of 1-port SRS resource and 1 of 2-port SRS resource.

- Option 2.3: Support both option 2.1 and option 2.2.

: For SRS resource configuration supporting codebook-based UL transmission for an 3TX UE,  down-select from：

- Alt1: A single SRS resource set configured with X 4-port SRS resource(s), where X≥1;

- Alt2: A single SRS resource set configured with at least 3 of 1-port SRS resources.

- Alt3: A single SRS resource set configured with at least 1 of 1-port SRS resource and 1 of 2-port SRS resources

- Alt4: Support both Alt2 and Alt3.

- In current spec, for codebook-based UL transmission, when the number of SRS antenna port(s) is equal to the transmit radio frequency chain(s) of UE, the base station uses the SRI to indicate the selected SRS resource to the UE.However, according to the aforementioned analysis, under the premise of without enhancement on SRS resource, this is impossible for the number of antenna ports of one SRS resource to be equal to the number of RF chains of a 3TX UE. Therefore, the SRI should be refined, which is determined by the SRS resource configuration for codebook-based 3TX UL transmission.

: For codebook-based 3TX UL transmission, the SRI should be refined based on the SRS resource configuration.

Conclusions

This contribution provides our views on non-coherent codebook design and SRI refinement for 3TX codebook-based UL transmission. We have following proposals.

: For codebook-based 3TX UL transmission, port selection precoding matrix in Table 1 should be included in non-coherent codebook.

: For codebook-based 3TX UL transmission, joint indication of TPMI and TRI, and the number of bits determined by the total number of precoding matrix across all ranks.

: For SRS resource configuration supporting codebook-based UL transmission for an 3TX UE,  down-select from：

- Alt1: A single SRS resource set configured with X 4-port SRS resource(s), where X≥1;

- Alt2: A single SRS resource set configured with at least 3 of 1-port SRS resources.

- Alt3: A single SRS resource set configured with at least 1 of 1-port SRS resource and 1 of 2-port SRS resources

- Alt4: Support both Alt2 and Alt3.

: For codebook-based 3TX UL transmission, the SRI should be refined based on the SRS resource configuration.

- References

[1]    RP-234007, New WID: NR MIMO Phase 5, Samsung, RAN#102, Dec 2023