3GPP TSG-RAN WG1 Meeting #116	R1-2400514

26 Feburary – 01 March 2024, Athens, GR

Agenda item:	9.1.3.3

Source:	Dell Technologies

Title:	On AI model transfer

Document for:	Decision, Discussion

# 1	Introduction

Based on the latest agreements of RAN#102 plenary meeting, a WI on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (for Release-19) has been approved [1], with the following objectives:

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain). - Necessity and details of model Identification concept and procedure in the context of LCM [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

This contribution presents our initial views on AI model transfer/delivery support over CP RRC signaling.

# 2	AI/ML model transfer over CP RRC signaling

AI/ML model transfer implies the handling of AI/Ml model delivery over the radio interface. Based on the outcomes of RAN1#110-bis, two categories of AI/ML model transfer are defined as follows:

- Scheme 1: AI/ML model transfer transparent to 3GPP (i.e., level y)

- Scheme 2: AI/ML model transfer on 3GPP signaling (i.e., level z)

Scheme 2 denotes that AI/ML model payload is stored on servers, as OTTs or OEMs, which is outside 3GPP scope. Scheme 2 means that AI/ML model payload is stored at an arbitrary 3GPP entity (e.g., gNBs and/or various core network functions) and are delivered to devices based on 3GPP control and/or data signaling procedures, respectively.

Observation 1: Level Z of AI/ML model transfer based on 3GPP signaling is relevant to RAN2 scope, where AI/ML model payload is delivered based on 3GPP signaling. This calls for 3GPP CP or UP signaling and protocol enhancements to adapt to emerging AI/ML model delivery requirements.

Transferring the AI/ML model payload over the control plane (CP) allows the AI/ML model payload to be part of each UE context information, which offers multiple AI/ML model delivery benefits to each UE needs and AI/ML capabilities. Specifically, with this delivery option, the AI/ML model is transferred as part of the radio resource control (RRC) reconfiguration message.

Two key issues, which have been identified by companies for mode delivery over RRC, are (1) the RRC segmentation limitations in terms of segment size and number, and (2) RRC signaling not supporting mobility forwarding, respectively.

Observation 2: Delivering the AI/ML model over CP RRC signaling has the following issues: (1) the RRC segmentation limitations in terms of segment size and number, and (2) RRC signaling not supporting mobility forwarding, respectively.

Therefore, to avoid the mentioned RRC issues and to support efficient AI/ML model as part of the RRC signaling, we propose introducing a new RRC message signaling, which is specifically defined for carrying the AI/ML model payload. The new RRC message is treated in a different way than basic RRC signaling as:

- The new AI/ML model RRC signaling is separate from existing non-AI/ML RRC signaling such that it does not block critical RRC information.

- The new AI/ML model RRC signaling can adopt different RRC segmentation size thresholds and numbers compared to existing RRC signaling. This allows the RRC segment size to be match the AI/ML model sizes which can significantly vary depending on the AI/ML driven use case and model complexity.

- The new AI/ML model RRC signaling should support mobility forwarding, unlike existing RRC basic signaling. Thus, in case of handover, the RRC segments, carrying the AI/ML model payload, are exchanged among source and target cells, and hence, the remaining segments are only transmitted. This is beneficial specially for large AI/ML models by avoiding transmitting the whole RRC segment set carrying large AI/ML models.

Thus, we propose a two stage RRC signaling format, where a first RRC reconfiguration message carries the basic RRC information, while a second RRC reconfiguration message carries the actual AI/ML model information. The AI/ML RRC message is scheduled by new scheduling information/indication in the first RRC message. Thus, regardless of the size of the AI/ML model, carried over the second RRC message, the essential non-AI RRC information is not blocked by the AI/ML payload.

Proposal 1: A new RRC message signaling is defined, independently from existing RRCreconfig message, for carrying AI/ML model information as UE context information.

Proposal 2: The new AI/ML RRC message is semi-statically scheduled by basic RRCreconfig message. Hence, an additional scheduling indication/information must be included in basic RRC signaling.

Proposal 3: The new RRC signaling message, for carrying AI/ML model information, to support mobility forwarding, and different RRC segmentation configuration (segment size and number) compared to existing RRC messages, which matches the larger RRC model sizes.

# 4	Conclusions

This contribution has presented Dell’s views on AI/ML model transfer over CP RRC signaling, respectively. The following observations and proposals are therefore made:

Observation 1: Level Z of AI/ML model transfer based on 3GPP signaling is relevant to RAN2 scope, where AI/ML model payload is delivered based on 3GPP signaling. This calls for 3GPP CP or UP signaling and protocol enhancements to adapt to emerging AI/ML model delivery requirements.

Observation 2: Delivering the AI/ML model over CP RRC signaling has the following issues: (1) the RRC segmentation limitations in terms of segment size and number, and (2) RRC signaling not supporting mobility forwarding, respectively.

Proposal 1: A new RRC message signaling is defined, independently from existing RRCreconfig message, for carrying AI/ML model information as UE context information.

Proposal 2: The new AI/ML RRC message is semi-statically scheduled by basic RRCreconfig message. Hence, an additional scheduling indication/information must be included in basic RRC signaling.

Proposal 3: The new RRC signaling message, for carrying AI/ML model information, to support mobility forwarding, and different RRC segmentation configuration (segment size and number) compared to existing RRC messages, which matches the larger RRC model sizes.

# References

- RP-234078, “New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, Moderator (Huawei), 3GPP RAN#102, Dec 2023.