3GPP TSG-RAN WG1 Meeting #116	R1-2400517

26 Feburary – 01 March 2024, Athens, GR

Agenda item:	9.11.3

Source:	Dell Technologies

Title:	Disaggregated NTN uplink and downlink sessions

Document for:	Decision, Discussion

# 1	Introduction

Based on the latest agreements of RAN#102 plenary meeting, a WI on NR NTN phase-3 (for Release-19) has been approved [1], with the following objectives:

Uplink Capacity/Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Study then specify, if beneficial, DFT-s-OFDM PUSCH enhancements via Orthogonal Cover Codes (OCC) - Determine the achievable capacity improvement to be targeted taking into account realistic impairments (e.g. Doppler, time variation, phase distortion, etc) - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed - Note: The study can consider orthogonal cover codes across OFDM symbols, across slots, and/or within an OFDM symbol. - Note: the study phase is targeted to be completed by RAN#104

In this contribution, we present our views on NTN uplink coverage enhancements for transmission-power-limited devices and propose a disaggregated NTN downlink and uplink session support.

# 2	Disaggregated NTN uplink and downlink session establishment

The Uplink Non-Terrestrial Network (NTN) link budget is primarily influenced by the constrained uplink transmission power of devices. In a wireless communication system, the link budget is a comprehensive assessment of the gains and losses throughout the transmission path. In the case of NTN, which includes satellite and other non-terrestrial components, the uplink link budget is crucial for ensuring reliable communication between user devices and the satellite. The limited uplink transmission power of devices becomes a significant factor in this equation, as it directly impacts the signal strength and quality during the transmission process. Challenges arise due to the necessity to optimize power consumption in mobile devices while maintaining a sufficient signal strength to establish a robust communication link with non-terrestrial elements.

Observation 1: Uplink NTN link budget is majorly dictated by the limited device uplink transmission power.

To address this, 3GPP has focused (during Rel-18) on developing error correction techniques, uplink payload repetition techniques, and power control mechanisms to maximize the utilization of the available uplink transmission power within the constraints of device capabilities and energy efficiency requirements in the NTN link budget.  However, to truly ease NTN uplink link performance, the available uplink transmission power must be scaled up at the device. In this regard, 3GPP has started support for higher power UEs (HPUEs) to offer direct uplink connectivity to NTNs.

Observation 2: As an interim stage with a low HPUE device penetration, uplink direct-to-satellite becomes a critical limitation for establishing a reliable NTN bi-directional session.

Observation 3: Existing NTN uplink coverage enhancement solutions, e.g., packet repetitions, come with direct expense of a lower NTN spectral efficiency.

Figure 1: NTN uplink packet repetitions for NTN uplink coverage enhancements.

Thus, a hybrid Terrestrial-Satellite architecture is suggested for uplink coverage enhancement, combining terrestrial and satellite components in a hybrid network architecture allows for seamless handovers and provides an alternative communication path when terrestrial coverage is available. This approach helps in maintaining connectivity even in challenging scenarios. These solutions are often implemented in a synergistic manner to address the specific requirements and constraints of uplink communication in NTN scenarios, ensuring reliable and efficient data transmission from user devices to non-terrestrial elements.

Proposal 1: RAN1 to specify support for hybrid Terrestrial-Satellite architecture where, for power-limited devices, uplink session is handled over TN interface while downlink session is treated over NTN interface.

Hybrid Terrestrial-Satellite architecture

In a hybrid network scenario, UEs seeks to establishing seamless communication connections, simultaneously interacting with ground towers for the uplink connection and Non-Terrestrial Networks (NTN) for downlink sessions. In this architecture, the uplink connection involves UEs communicating with terrestrial ground towers, utilizing established cellular infrastructure. These ground towers serve as access points to the core network, facilitating bidirectional communication between mobile devices and centralized network resources.

Proposal 2: Support disaggregated uplink-only RRC connection establishment signaling over TN interface.

Proposal 3: Introduce sand extend available service cause list to include ‘’Uplink-only NTN session relaying’’, indicating UEs requesting an uplink-only connection establishment for relaying an active NTN uplink control and/or data payload.

On the downlink side, mobile phones engage in sessions with Non-Terrestrial Networks, which could include satellites or other non-ground-based components. This connection provides mobile users with enhanced coverage, reduced latency, and expanded network reach, making it particularly valuable in remote or challenging terrains. The downlink session with NTN enables mobile phones to receive data, content, and services from the broader network, complementing the terrestrial coverage and extending the network's capabilities.

This dual-mode operation, with uplink connections to ground towers and downlink sessions with NTN, ensures a versatile and resilient communication experience for mobile users. The system intelligently manages the network handovers, allowing mobile phones to seamlessly transition between ground-based and non-terrestrial connections based on signal strength, quality, and other relevant factors. This integrated approach optimizes the use of available resources, providing users with a reliable and efficient communication experience across diverse landscapes and scenarios.

Proposal 4: Study and support of TN uplink-only connection establishment with/without NTN UCI information relaying.

Proposal 5: RAN3 to specify and support TN-NTN backhaul exchange of disaggregated NTN uplink session information.

Figure 2: Hybrid Terrestrial-Satellite architecture with uplink/downlink session disaggregation.

# 3	Conclusions

In this contribution, the following observations and proposals are made:

Observation 1: Uplink NTN link budget is majorly dictated by the limited device uplink transmission power.

Observation 2: As an interim stage with a low HPUE device penetration, uplink direct-to-satellite becomes a critical limitation for establishing a reliable NTN bi-directional session.

Observation 3: Existing NTN uplink coverage enhancement solutions, e.g., packet repetitions, come with direct expense of a lower NTN spectral efficiency.

Proposal 1: RAN1 to specify support for hybrid Terrestrial-Satellite architecture where, for power-limited devices, uplink session is handled over TN interface while downlink session is treated over NTN interface.

Proposal 2: Support disaggregated uplink-only RRC connection establishment signaling over TN interface.

Proposal 3: Introduce sand extend available service cause list to include ‘’Uplink-only NTN session relaying’’, indicating UEs requesting an uplink-only connection establishment for relaying an active NTN uplink control and/or data payload.

Proposal 4: Study and support of TN uplink-only connection establishment with/without NTN UCI information relaying.

Proposal 5: send LS toRAN3 for supporting TN-NTN backhaul exchange of disaggregated NTN uplink session information.

# References

- RP-234078, “New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”, Moderator (Huawei), 3GPP RAN#102, Dec 2023.