3GPP TSG RAN WG1 #116			R1-2400519

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	8.1

Source:	Ericsson

Title:	Maintenance on NR MIMO evolution

Document for:	Discussion, Decision

# 1	Introduction

In this contribution we discuss a few remaining issues on Rel-18 MIMO.

# 2	Discussion

## 2.1	Unified TCI framework extension for multi-TRP

## 2.1.1	Default TCI state for single-TRP PDSCH reception

In RAN1#113, RAN1 agreed

Agreement

On unified TCI framework extension for S-DCI based MTRP, support the following:

- Using RRC configuration to indicate whether the first, second, or both of the indicated joint/DL TCI states is/are applied to PDSCH reception scheduled/activated by DCI format 1_0

  - If not configured, the first indicated joint/DL TCI state is applied

  - Only when the UE is configured with PDSCH-CJT and the UE supports two joint TCI states for PDSCH-CJT or the UE is configured with PDSCH-SFN, the RRC configuration can indicate both indicated joint/DL TCI states are applied.

    - For PDSCH-CJT and PDSCH-SFN, if the RRC configuration indicates both indicated joint/DL TCI states are applied, the UE shall apply both indicated joint/DL TCI states to PDSCH reception scheduled/activated by DCI format 1_0 on a search space other than Type0/0A/2 CSS on CORESET#0 (FFS: Other search space and/or CORESETs)

If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2, above applies regardless of the offset between the reception of the scheduling DCI format 1_0 and the scheduled/activated PDSCH reception

If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above applies when the offset between the reception of the scheduling DCI format 1_0 and the scheduled/activated PDSCH reception is equal to or larger than a threshold

In RAN1#113, the following was also agreed:

Agreement

On unified TCI framework extension for S-DCI based MTRP, for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the scheduled/activated PDSCH reception

- If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2, above applies regardless of the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception

- If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above applies when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold

This specifies how the UE would determine the default beams (i.e., when TCI state selection field is not present) for PDSCH for the case when there are two indicated TCI states and two PDSCH are transmitted. The agreement does not provide a solution for the case where there are two indicated TCI states and one PDSCH is transmitted from one TRP, i.e., for multi-TRP PDCCH repetition with single-TRP transmission for PDSCH, HST-SFN PDCCH, multi-TRP PUCCH/PUSCH repetition, and STxMP with single-TRP PDSCH transmission:

The UE cannot determine if the PDSCH is received from one or two TRPs in the absence of the TCI selection field.

In legacy, the UE determines if the PDSCH is received from one or two TRPs depending on how many TCI states are activated. This is not possible for the unified TCI framework, where two TCI states can be indicated even if the PDSCH is received from one TRP.

The issue is illustrated in Figure 1 for PDCCH repetition and HST-SFN PDCCH with single-TRP PDSCH transmission.

Figure 1: Illustration of PDCCH repetition and HST-SFN transmission. The default indicated TCI state for PDSCH is undetermined for these two cases.

To solve this issue, we propose to reuse the RRC configuration agreed for DCI format 1_0 also for the case where there are two indicated TCI states and single-TRP PDSCH transmission:

Reuse the RRC parameter controlling the default indicated TCI state for DCI format 1_0 also for DCI format 1_1 and 1_2 for single-TRP PDSCH transmission with two indicated TCI states.

## 2.1.2	TCI state for multi-DCI multi-TRP before first indication

Another remaining issue is related to initial operation for multi-DCI multi-TRP. According to RAN1 agreements, there is no possibility to perform cross-TRP TCI state indication for multi-DCI multi-TRP. Since the main intention with multi-DCI multi-TRP is to operate the TRPs independently, this is natural. However, this becomes a problem during the initial setup of the second TRP.

To set up the second TRP (corresponding to CORESETPoolIndex 1), the following steps are needed:

The NW decides to activate TRP2, based on L1 measurement reports received at TRP1

The NW sends a MAC CE over TRP1 to activate multiple TCI states for CORESETPoolIndex 1. This is possible, since the MAC CE contains a CORESETPoolIndex field.

One of the activated TCI states is indicated using DCI.

The problem is that the PDCCH in step 3 can only be sent over a CORESET associated with CORESETPoolindex 1, but since there is no indicated TCI state for that CORESETPoolIndex, the UE cannot receive a PDCCH associated with CORESETPoolIndex 1.

For single-TRP and single-DCI multi-TRP, this is not a problem, since the UE would continue to use the QCL assumption it used before the activation. But for multi-DCI multi-TRP, there is no such prior QCL assumption: the UE cannot guess how the NW would transmit the initial PDCCH from TRP2.

Hence, we realize that we need to define which QCL assumption the UE would use to receive the PDCCH carrying the initial beam indication for TRP2, i.e., for CORESETPoolIndex 1. In other words, which of the activated TCI states the UE should use to receive the PDCCH carrying the initial beam indication for TRP2:

Define a rule to determine which of the activated TCI states the UE would use before any TCI state has been indicated.

One simple rule could be that the UE uses the first activated TCI state before any TCI state has been indicated:

Before initial indication of a TCI state corresponding to CORESETPoolIndex 1, the UE assumes the first activated TCI state is indicated.

In the current specification, a work around would be

- The NW decides to activate TRP2, based on L1 measurement reports received at TRP1

The NW sends a MAC CE over TRP1 to activate a single TCI state for CORESETPoolIndex 1. The UE can directly receive using this single TCI state.

The NW sends a second MAC CE to activate multiple TCI states for CORESETPoolIndex 1.

One of the activated TCI states is indicated using DCI.

- Clearly, this solution is unnecessarily complicated.

## 2.2	SRI/TPMI enhancement for enabling 8 TX UL transmission

38.214 presently describes the configuration of 8 Tx codebooks with the following. However, there is no ‘ULcodebookFC-N1N2’ in the RRC spec, nor is ‘Ng=1’ explained (although Ng represents the number of port groups).

For codebook based transmission with eight antenna ports, the UE determines its codebook based upon the reception of higher layer parameter[s] CodebookType and ULcodebookFC-N1N2 if CodebookType is configured with Ng=1 in pusch-Config for PUSCH associated with DCI format 0_1 and 0_2, depending on the UE capability. According to the configured CodebookType, coherent UL MIMO operation applies within antenna port groups as defined in Table 6.3.1.5-8 of [4, TS 38.211].

In RAN1#115, antenna port groups were defined in part through the following agreement, which did not capture that  Ng is a number of port groups.

Agreement

- Adopt the following text proposal to TS 38.214.

-------------------------------------------Unchanged parts are omitted------------------------------------------- For codebook based transmission with eight antenna ports, the UE determines its codebook based upon the reception of higher layer parameter[s] CodebookType and ULcodebookFC-N1N2 if CodebookType is configured with Ng=1 in pusch-Config for PUSCH associated with DCI format 0_1 and 0_2, depending on the UE capability. According to the configured CodebookType, requirements for coherent UL MIMO in [38.101-1] and [38.101-2] apply within an antenna port group. -------------------------------------------Unchanged parts are omitted-------------------------------------------

Furthermore, CodebookType has the following values according to the current revision of 38.331, noting that 38.214 is incorrectly referencing the CodebookTypeUL IE as CodebookType.

CodebookTypeUL-r18 ::= CHOICE { codebook1-r18 ENUMERATED {ng1n4n1, ng1n2n2}, codebook2-r18 ENUMERATED {ng2}, codebook3-r18 ENUMERATED {ng4}, codebook4-r18 ENUMERATED {ng8} }

Since each of the possible values of CodebookTypeUL (i.e. {ng1n4n1,ng1n2n2,ng2,ng4,ng8}) have a corresponding table in 38.211 as exemplified with the Table captions from 38.211 below, the ASN.1 is clear with respect to which 8 Tx codebook to use, and there is no need for ULcodebookFC-N1N2.

Table 6.3.1.5-9: Intermediate precoding matrix W' for codebook1=ng1n4n1 and single-layer transmission using eight antenna ports.

Table 6.3.1.5-17: Intermediate precoding matrix W' for codebook1=ng1n2n2 and single-layer transmission using eight antenna ports.

38.214 refers to an undefined RRC parameter ULcodebookFC-N1N2 to be used to configure the 8 Tx UL MIMO codebook.  No such parameter is needed, since the IE CodebookTypeUL is sufficient to configure the codebook.

38.211 defines antenna port groups with the following, which does not explicitly tie to the number of antenna port groups Ng given by RRC.  Therefore, 38.214 should be clarified to say ‘According to the configured CodebookTypeUL and the value of Ng, coherent UL MIMO operation applies within antenna port groups’.

Table 6.3.1.5-8: The port mapping function fi for transmission using 8 antenna ports.

| i | Higher-layer parameter CodebookType |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | codebook1 |  | codebook2 |  | codebook3 |  | codebook4 |  |
|  | antenna port group | fi | antenna port group | fi | antenna port group | fi | antenna port group | fi |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 |  | 1 |  | 1 |  | 4 | 1 | 1 |
| 2 |  | 2 |  | 4 | 1 | 1 | 2 | 2 |
| 3 |  | 3 |  | 5 |  | 5 | 3 | 3 |
| 4 |  | 4 | 1 | 2 | 2 | 2 | 4 | 4 |
| 5 |  | 5 |  | 3 |  | 6 | 5 | 5 |
| 6 |  | 6 |  | 6 | 3 | 3 | 6 | 6 |
| 7 |  | 7 |  | 7 |  | 7 | 7 | 7 |

38.214 does not explicitly state how to identify which of the antenna port groups defined in 38.211 should be used for 8 Tx codebook based operation, but instead currently refers to ‘Ng=1’, which is not defined.

We therefore propose the following update to the 38.214 text:

For codebook based transmission with eight antenna ports, the UE determines its codebook and a corresponding number of antenna port groups, Ng, based upon the reception of higher layer parameter[s] CodebookTypeUL and ULcodebookFC-N1N2 if CodebookType is configured with Ng=1 in pusch-Config for PUSCH associated with DCI format 0_1 and 0_2, depending on the UE capability. According to the configured CodebookTypeUL and the value of Ng, coherent UL MIMO operation applies within antenna port groups as defined in Table 6.3.1.5-8 of [4, TS 38.211].

Correct 38.214 according to the text proposal herein to say a) that the UE determines its codebook and a corresponding number of antenna port groups, Ng, based on the RRC parameter CodebookTypeUL, and b) that the port groups are selected via the number of port groups determined from CodebookTypeUL.

## 2.3	On multi-user scheduling indicator for advanced MU-MIMO receiver

## 2.3.1 Previous LSs on advanced MU-MIMO receiver

RAN1 is requested by RAN4 to implement the DCI signaling for advanced MU-MIMO receiver with bit fields indicating MU-MIMO co-scheduling information in LS [1]. Questions from RAN1 have been further answered in LS [2]. Main contents of the two LSs are copied here for quick reference.

Requested fields in DCI 1_1:

| Bit field mapped to index | Content |
| --- | --- |
| 0 | No co-scheduled UE(s) which has same DMRS sequence as target UE exists |
| 1 | In all the PRBs allocated to the target UE, all the co-scheduled UE(s), which has the same DMRS sequence as the target UE, have QPSK scheduled |
| 2 | In all the PRBs allocated to the target UE, all the co-scheduled UE(s), which has the same DMRS sequence as the target UE, have 16QAM scheduled |
| 3 | In all the PRBs allocated to the target UE, all the co-scheduled UE(s), which has the same DMRS sequence as the target UE, have 64QAM scheduled |
| 4 | In all the PRBs allocated to the target UE, all the co-scheduled UE(s), which has the same DMRS sequence as the target UE, have 256QAM scheduled |
| 5 | In all the PRBs allocated to the target UE, all the co-scheduled UE(s), which has the same DMRS sequence as the target UE, have 1024QAM scheduled |
| 6 | Not covered by cases corresponding to index 0~5. In each individual PRB allocated to the target UE, the following condition is satisfied: Only single modulation order is allocated for the co-scheduled UE(s) which has the same DMRS sequence as the target UE, if the co-scheduled UE(s) exist |
| 7 | Others |

- The existence of MU-MIMO DCI signalling is configured by RRC signalling.

- The field is intended to be included in a DCI which can be based on the format 1_1.

RAN4’s answers to the questions from RAN1:

Question 1: Whether this new signaling in DCI is introduced in DCI format 1_2 in addition to format 1_1?

Answer: The understanding in RAN4 is that URLLC is not a common scenario for MU-MIMO, but if there are relevant use cases with MU-MIMO scheduling with DCI format 1_2, the signalling in DCI can be introduced in DCI format 1_2, otherwise not.

Question 2: Whether this new signaling in DCI is supported for one or more DL multi-TRP schemes?

Answer: The understanding in RAN4 is that there are limited scenarios for MU-MIMO with mTRP operation. RAN4 suggests that this new signalling in DCI is not supported for multi-TRP schemes.

Question3: Whether this new signaling in DCI is supported when the RRC parameter maxNrofCodeWordsScheduledByDCI is configured as 2?

Answer:  This new DCI is supported if RRC parameter maxNrofCodeWordsScheduledByDCI is configured as 2 and target UE is only scheduled 1 codeword.

Question 4: Whether the new signaling in DCI is supported when the RRC codeBlockGroupTransmission is configured?

Answer: This new DCI signalling can be supported if there are relevant use cases with MU-MIMO scheduling when the RRC codeBlockGroupTransmission is configured, otherwise not.

Question 5: Whether the new signaling in DCI is supported when Rel-18 DMRS is configured?

Answer: Yes. The new signaling can be supported for the UE with Rel-18 DMRS configured, and co-scheduled UE mentioned in DCI signaling includes both co-scheduled UEs on R15 DMRS ports and co-scheduled UEs on R18 DMRS ports

Question 6: In the content corresponding to “Bit field mapped to index” =6, whether or not the phrase “In each individual PRB allocated to the target UE, the following condition is satisfied” should be replaced by “In each individual PRB PRG allocated to the target UE, the following condition is satisfied”?

Answer: RAN4 has agreed that the revision suggested by RAN1 is not needed.

Question 7: For “Bit field mapped to index” =1/2/3/4/5, does “empty PRB without co-scheduled UE” is allowed “in all the PRBs” of the target UE.

Answer: Yes, “For bit field mapped to index”=1/2/3/4/5”, empty PRB without co-scheduled UE is allowed in all the PRB” of the target UE

Then in RAN1#114 meeting RAN1 made this agreement.

Agreement

Implement the DCI signaling in R1-2306361 (R4-2309895) in RAN1 specifications with the following assumptions.

- Scope of this DCI signaling at least applying to a PDSCH satisfying all the following conditions.

  - The PDSCH is scheduled by DCI format 1_1.

    - Support for this feature for other DCI format(s) can be later added depending on RAN4 input

  - Single TRP based scheme is configured for the PDSCH transmission.

  - Single codeword is configured for the PDSCH transmission.

  - CBG based transmission is not configured for the PDSCH transmission.

  - Rel-15/16/17 DMRS is configured for the PDSCH transmission.

- For “Bit field mapped to index” =0, the content “No co-scheduled UE(s) which has same DMRS sequence as target UE exists” is interpret as the following.

  - In all the PRBs allocated to the target UE, there is no co-scheduled UE or there is co-scheduled UE but with a different DMRS sequence.

- The terminology “the same DMRS sequence” in the DCI signaling table is interpret as the same root DMRS sequence r(n) in TS38.211 Section 7.4.1.1.1.

- “Bit field mapped to index” =7 in the DCI signaling table is interpret as including all the cases not covered by cases corresponding to “Bit field mapped to index” 0/1/2/3/4/5/6.

## 2.3.2 How to efficiently implement the DCI field

Next, we provide observations on network implementation and UE capability elaborate that in a practical network only a few selected indexes are needed.

One observation is, that 1024QAM is optional in FR1 and is not supported in FR2, in addition 256QAM is optional for both FR1 and FR2. As the supporting of these features is dependent on UE capability and frequency bands, the indexes for 1024QAM and 256QAM can be obsolete for UE not supporting the capability.

Second, even with high modulation schemes supported by the UE, the network may avoid configuring these high modulation schemes to the UE. Third, even network schedules single UE with high modulation schemes, it may avoid scheduling multiple UEs with high modulation schemes. Again, the corresponding modulation schemes and indexes become useless for network not using these indexes for MU-MIMO.

Apart from the modulation schemes, some of the UEs doesn’t support usage of advanced received if indicated with index 6. Thus the index 6 can be obsolete indication for some of the UEs.

In summary, depending on network deployment and UE capability, only a few of the indexes in the bit mapping table is applicable in a real network.

Which indexes to be used in the DCI as multi-user co-scheduling indicator shall be aligned with UE capability and network requests.

The table indexes to be used in the DCI for multi-user co-scheduling shall be configurable.

Number of bits for the DCI field can supported as configurable to improve the PDCCH resource efficiency. One example is to reduce the field to 2 bits by using only half of the indexes for UE not capable of supporting index 6, that the network only indicates index 0-3 to the UE.

Support 1,2,3 bits in the DCI for indicating multi-user co-scheduling.

When 2 bits is used for MU-MIMO co-scheduling, use index 0-3 in the table for UE not capable of index 6.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The UE cannot determine if the PDSCH is received from one or two TRPs in the absence of the TCI selection field.

Observation 2	38.214 refers to an undefined RRC parameter ULcodebookFC-N1N2 to be used to configure the 8 Tx UL MIMO codebook.  No such parameter is needed, since the IE CodebookTypeUL is sufficient to configure the codebook.

Observation 3	38.214 does not explicitly state how to identify which of the antenna port groups defined in 38.211 should be used for 8 Tx codebook based operation, but instead currently refers to ‘Ng=1’, which is not defined.

Observation 4	Which indexes to be used in the DCI as multi-user co-scheduling indicator shall be aligned with UE capability and network requests.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Reuse the RRC parameter controlling the default indicated TCI state for DCI format 1_0 also for DCI format 1_1 and 1_2 for single-TRP PDSCH transmission with two indicated TCI states.

Proposal 2	Define a rule to determine which of the activated TCI states the UE would use before any TCI state has been indicated.

Proposal 3	Before initial indication of a TCI state corresponding to CORESETPoolIndex 1, the UE assumes the first activated TCI state is indicated.

Proposal 4	Correct 38.214 according to the text proposal herein to say a) that the UE determines its codebook and a corresponding number of antenna port groups, Ng, based on the RRC parameter CodebookTypeUL, and b) that the port groups are selected via the number of port groups determined from CodebookTypeUL.

Proposal 5	The table indexes to be used in the DCI for multi-user co-scheduling shall be configurable.

Proposal 6	Support 1,2,3 bits in the DCI for indicating multi-user co-scheduling.

Proposal 7	When 2 bits is used for MU-MIMO co-scheduling, use index 0-3 in the table for UE not capable of index 6.

# References

R4-2309895, LS on required DCI signalling for advanced receiver on MU-MIMO scenario, RAN WG, RAN4#107, Incheon, Korea, May 2023

R4-2317011, Reply LS on required DCI signalling for advanced receiver on MU-MIMO scenario, RAN WG4, RAN4#118bis Xiamen China, October 2023