3GPP TSG-RAN WG1 Meeting #116	R1-2400522

# Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.2.2

Source:	Honor

Title:	Discussion on CSI enhancements

Document for:	Discussion and Decision

- Introduction

In RAN#102 meeting, a new work item for NR MIMO Phase 5 was approved [1]. This work item is intended to further enhance MIMO in 5G Advanced in terms of beam management latency, DL spectrum efficiency, inter-TRP calibration for coherent joint transmission, 3 Tx antennas for UL performance and asymmetric DL sTRP/UL mTRP deployment scenarios. In this contribution, we provide some preliminary views on CSI enhancements according to the following WID objectives.

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH

- Type I codebook refinements

Legacy codebooks

In FeType II codebook for CJT, as defined in Rel-18, UE can report up to 128-port precoding matrix. gNB can configure up 4 CSI-RS resources to for PMI acquisition. The codebook construction for CJT largely follows the eType II designs in Rel-16. The differences are mainly contributing to that:

- UE can select N0 CSI-RS resources out of NTRP CSI-RS resources is to enable dynamic TRP selection for joint transmission.

- UE reports SD basis selection independently for each selected CSI-RS resource to allow spatial diversity across distributed TRPs.

Observation 1: FeType II codebook for CJT supports dynamic TRP selection by UE for a CSI report.

Observation 2: In FeType II codebook for CJT, UE reports SD basis selection independently for each selected CSI-RS resource to allow spatial diversity across TRPs.

For Type I single-panel codebook, precoding matrix is generally constructed as follows:

pl=1 ej2πlO1N1…ej2πl(N1-1)O1N1

qm=1 ej2πmO2N2…ej2πm(N2-1)O2N2

vl, m=pl⊗qm

W1=w1∙w2=vl,m00vl,m1ejπφ1=(pl⊗qm)T00(pl⊗qm)T1ejπφ1

The Type I single-panel codebook adopts two-stage precoding, where the first stage is about SD basis selection in w1 and the second stage is to capture the phase variations between different antenna polarizations in w2. Meanwhile, the two antenna polarizations share the same SD basis to enable the same spatial direction.

Observation 3: Type I single-panel codebook adopts two-stage precoding, where the first stage includes the same SD basis for both antenna polarizations and second stage captures the phase variations between different antenna polarizations.

Moreover, for Type I multi-panel codebook, gNB should indicate the number of panels Ng in addition to N1 and N2. Therefore, all Ng panels are placed in one dimension, e.g., in a column or in a row. For Type I multi-panel codebook as cited below, the precoding matrix is divided into 2Ng sub-matrices, where each sub-matrix corresponds to a precoding matrix for an antenna polarization of a specific panel. Furthermore, unlike FeType II codebook for CJT, all the sub-matrices for Type I multi-panel codebook share the same SD basis across all panels and all antenna polarizations.

Wl,m,p,n1,2,1=1PCSI-RSvl,mφnvl,mφp1vl,mφnφp1vl,mWl,m,p,n2,2,1=1PCSI-RSvl,m-φnvl,mφp1vl,m-φnφp1vl,mWl,m,p,n1,4,1=1PCSI-RSvl,mφnvl,mφp1vl,mφnφp1vl,mφp2vl,mφnφp2vl,mφp3vl,mφnφp3vl,mWl,m,p,n2,4,1=1PCSI-RSvl,m-φnvl,mφp1vl,m-φnφp1vl,mφp2vl,m-φnφp2vl,mφp3vl,m-φnφp3vl,m

Observation 4: For Type I multi-panel codebook, gNB indicates the number of panels Ng in addition to N1 and N2. Therefore, all Ng panels are placed in one dimension.

Observation 5: For Type I multi-panel codebook, the precoding matrix is divided into 2Ng sub-matrices, where each sub-matrix corresponds to a precoding matrix for an antenna polarization of a specific panel and all the sub-matrices share the same SD basis across all panels and all antenna polarizations.

Type I codebook supporting up to 128 ports

As stated in the WID objective, Rel-19 is not going to enhance legacy CSI-RS design. Therefore, in order to support up to 128 ports for Type I codebook, network may configure multiple CSI-RS resources to jointly derive a precoding matrix, which is analogous to FeType II codebook for CJT defined in Rel-18. Each CSI-RS resource may correspond to either a TRP or a panel at network side.

For distributed TRPs, as shown in Figure 1. To ensure the spatial diversity, the TRPs may utilize different SD basis. It is similar as FeType II codebook for CJT that UE reports SD basis selection independently for each selected CSI-RS resource. Therefore, the mechanisms defined in Rel-18 can be simply reused for Type I codebook extension supporting up to 128 ports.

Figure 1. Joint transmission from distributed TRPs

Proposal 1: For Type I codebook supporting up to 128 ports, network can configure multiple CSI-RS resources to jointly get a PMI.

Proposal 2: For Type I codebook supporting up to 128 ports, further study whether to reuse the following mechanisms defined in Rel-18 for CJT:

- Dynamic CSI-RS resource selection by UE for a CSI report.

- SD basis selection can be reported independently for each selected CSI-RS resource.

- (b)

Figure 2. Joint transmission from multiple panels

To support Type I codebook supporting up to 128 ports, when multiple CSI-RS resources are configured, each CSI-RS resource may correspond to a panel. However, for Type I multi-panel codebook, SD basis is generated per panel, which prevents network from generating more granular SD beams by considering all antenna ports. In addition, all panels are placed in one dimension for Type I multi-panel codebook. To facilitate network deployment flexibility, the multiple panels can either be deployed as Figure 2a or Figure 2b for Type I codebook supporting up to 128 ports. Therefore, network may need to configure additional parameters Ng,1, Ng,2, where Ng,1 corresponds to the number of panels in first dimension and Ng,2 corresponds to the number of panels in second dimension. Based on the configurations, for example, UE can generate SD basis according to N1',N2' as follows:

N1'=Ng,1∙N1N2'=Ng,2∙N2

Proposal 3: For Type I codebook supporting up to 128 ports, when multiple CSI-RS resources are configured for a joint CSI report, network can inform UE about the panel layout associated with the multiple CSI-RS resources, e.g., the number of panels in first dimension and second dimension.

- CRI-based CSI reporting

The precoding methods generally include digital precoding, analog precoding and hybrid precoding. Digital precoding is performed in baseband by using different weighting coefficients (e.g., amplitude and/or phase) for various RF chains. The digital precoding is useful for multi-user multiplexing since different weighting coefficients can be used for different users at the same time. However, the deployment cost is very high due to massive RF chains. Therefore, the digital precoding is commonly deployed in low frequency with limited antennas. Analog precoding is conducted by phase shifters with low cost. Then, it is beneficial in massive MIMO deployment. However, a single panel cannot generate two analog beams simultaneously. In current specifications, digital precoding and analog precoding are supported separately via CSI report and beam report. In a CSI report, UE reports preferred RI/PMI/CQI to network. The beam report includes CRIs and the corresponding RSRPs to facilitate network to select appropriate analog precoding.

Observation 6: Digital precoding and analog precoding are supported via two separate reports in current specification, i.e., CSI report and beam report.

Figure 3. An example of hybrid precoding

As discussed above, both digital precoding and analog precoding have benefits and constraints. To move forward to 5G Advance and beyond, new spectrum especially for mid-range frequency will be further studied. In this scenario, as shown in Figure 3, hybrid precoding may be adopted to balance the deployment cost and system performance. In the remaining parts of this section, we discuss potential specification impacts to support hybrid precoding.

Codebook types:

There are two codebook types defined in NR specifications, i.e., Type I codebook and Type II codebook. Although Type II codebook has been continuously enhanced after Rel-15, there is still no commercial deployment due to its high UE calculation complexity and report overhead. However, Type I codebook only includes phase information in PMI, so the UE calculation complexity and report overhead are significantly lower than Type II codebook. Moreover, for CRI-based CSI reporting, UE may need to search the best RI/PMI/CQI among all configured reference signals, which also brings additional UE calculation complexity. Therefore, it’s proposed to prioritize Type I codebook over Type II codebook for CRI-based CSI reporting.

Proposal 4: Type I codebook is prioritized over Type II codebook for CRI-based CSI reporting.

Resource setting and report setting:

As stated in WID objective, hybrid beamforming should support up to a total of 128 CSI-RS ports across all resources. Since Rel-19 is not intended to enhance CSI-RS regarding the number of ports, network may need to configure multiple CSI-RS resources in a resource setting for CRI-based CSI reporting. According to Figure 3, the multiple CSI-RS resources may be transmitted with different analog precoding. Then, for a specific CSI-RS resource, UE has to report preferred RI/PMI/CQI, i.e., information related to digital precoding. Finally, based on the report, network can construct hybrid precoding to transmit the so-called hybrid beamforming.

Proposal 4: To support CRI-based CSI reporting for hybrid beamforming, network should configure multiple CSI-RS resources in a resource setting.

As discussed above, digital precoding and analog precoding are supported via CSI report and beam report, respectively. The hybrid beamforming needs to combine the CSI report and the beam report in a single report. Therefore, UE should at least report CRI and corresponding RI/PMI/CQI in the CRI-based CSI reporting, which basically reflects the digital precoding and analog precoding, respectively. In addition, like beam report, UE is required to report multiple CRIs and their corresponding CSI information in a report, which is to facilitate MU scheduling at network even in hybrid beamforming scenario.

Proposal 5: To support CRI-based CSI reporting for hybrid beamforming, network can configure UE to report multiple sets of CSI information in a single report, where each set of CSI information at least includes CRI and corresponding RI/PMI/CQI for a specific CSI-RS resource.

Moreover, network can also configure UE to report RSRP for a specific CRI. This is due to that RI/PMI/CQI only reflects some relative channel quality., e.g., CQI is derived based on the ratio between signal power and noise/interference. However, RSRP shows the absolute information of signal receiving power, which may help network to select a better analog precoding and to get some information of noise/interference level. In current specification, the RSRP is generally defined for single-port CSI-RS, which should be extended to support RSRP on a CSI-RS resource with multiple antenna ports.

Proposal 6: To support CRI-based CSI reporting for hybrid beamforming, network can additionally configure UE to report RSRP for a corresponding CRI.

Codebook subset restriction (CBSR):

CBSR is defined in specification to avoid the intra/inter cell interference. For example, network indicates the CBSR to avoid UE to report a specific SD basis for PMI report. In previous releases, the CBSR only includes the impact of digital precoding. When it comes to CRI-based CSI reporting, a hybrid beam is generated based on both digital precoding and analog precoding. Therefore, the configuration on CBSR should consider the intra/inter cell interference in hybrid beamforming scenario.

Proposal 7: For CRI-based CSI reporting, codebook subset restriction should be enhanced to support interference coordination in hybrid beamforming scenario.

UCI contents and report overhead reduction:

For legacy CSI report, UE can be configured to report wideband CSI information or subband CSI information. Regarding the subband CSI information, the CSI report is divided into two parts, where the overhead of second part CSI is determined by first part CSI. The CRI-based CSI reporting should reuse the legacy mechanisms to report the wideband CSI information or wideband CSI information. According to the discussion above, the CRI-based CSI report may include multiple sets of CSI information. The report overhead will be largely increased to report the multiple sets of CSI information. Therefore, the report overhead reduction should be studied for CRI-based CSI reporting. For example, UE should at least report one set of CSI information in a report. Whether and how to report other sets of CSI information to reduce report overhead can be studied in Rel-19.

Proposal 8: For CRI-based CSI reporting, at least reuse legacy CSI report mechanisms defined for wideband CSI information or subband CSI information.

Proposal 9: Study methods to reduce CSI report overhead for CRI-based CSI reporting.

- UE reporting enhancement for CJT deployments

In Rel-18, FeType II codebook for CJT has been defined, which is targeted on coherent transmission with ideal synchronization and backhaul. Moreover, Rel-16 supports two different mechanisms to compensate frequency offset between TRPs either at network side or UE side in HST SFN scenario. This case may be beneficial for TDD system where channel reciprocity is assumed. For FDD system, as shown in Figure 4, the compensation at network side strongly relies on UE reporting. As stated in WID objective, to support coherent transmission between TRPs, inter-TRP time offset, and frequency/phase offset should be defined and reported. In this section, we provide some preliminary views on how to define inter-TRP time offset and frequency/phase offset.

Figure 4. Coherent transmission using reported inter-TRP time offset and phase offset

Granular information of inter-TRP time offset and frequency/phase offset:

In NR positioning, there is a measurement defined for inter-TRP time offset, which is DL reference signal time difference (DL RSTD). As cited from 38.214 in the following table, it’s DL relative timing difference between TRPs determined according to first detected path in time domain. In some cases, the first detected path may not be the main component of a time delay path, e.g., the time delay path with highest power. If the DL RSTD is used to compensate the inter-TRP time offset, the performance for coherent transmission may not be as good as expected since DL RSTD does not capture the main component of time delay paths. However, DL RSTD can be a starting point for further discussion on how to define the inter-TRP time offset for coherent transmission.

| Definition | DL reference signal time difference (DL RSTD) is the DL relative timing difference between the Transmission Point (TP) [18] j and the reference TP i, defined as TSubframeRxj – TSubframeRxi, Where: TSubframeRxj is the time when the UE receives the start of one subframe from TP j. TSubframeRxi is the time when the UE receives the corresponding start of one subframe from TP i that is closest in time to the subframe received from TP j. Multiple DL PRS resources can be used to determine the start of one subframe from a TP. For frequency range 1, the reference point for the DL RSTD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSTD shall be the antenna of the UE. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE, RRC_IDLE |

Proposal 10: For the measurement on inter-TRP time offset, consider reusing the definition of DL reference signal time difference (DL RSTD) for NR positioning as a starting point.

The frequency offset depends on arrival angle, departure angle and UE velocity. As shown in Figure 4, the arrival angle and the departure angle are different for different TRPs, so the frequency offset is TRP dependent. Moreover, power delay profile generally reflects the paths with different delay, power, arrival angle and departure angle. Therefore, the frequency offset per TRP is also path dependent. For example, as depicted in Figure 5, the frequency offset per path can be measured based on the phase offset for the same path in two consecutive time occasions. However, UE may be hard to track the phase variation of the same path in two consecutive time occasions because of clock drift or birth and death of a path over time.

The above example is similar to DL reference signal carrier phase difference (DL RSCPD) defined for carrier phase-based positioning in Rel-18. The definition of DL RSCPD is cited in the following table. As the frequency offset is path dependent, it may not capture the main component of a time delay path, e.g., the time delay path with highest power. The performance may be degraded if the frequency offset is used to compensate the inter-TRP frequency/phase offset.

Figure 5. Frequency/phase offset calculation per path

| Definition | DL reference signal carrier phase difference (RSCPD) is defined as the difference of DL RSCPs measured from DL PRS transmitted in a DL PFL from the transmission point (TP) j and the reference TP i. If UE reports RSCPD measurements together with RSTD measurements in a measurement report element, the reference TP for RSCPD is the same as the reference TP reported for RSTD. For frequency range 1, the reference point for the DL RSCPD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSCPD shall be the antenna of the UE. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE, RRC_IDLE |

Proposal 11: For the measurement on inter-TRP frequency/phase offset, consider reusing the definition of DL reference signal carrier phase difference (RSCPD) for NR positioning as a starting point.

General information of inter-TRP time offset and frequency/phase offset:

In Rel-18, time domain channel property (TDCP) has been defined as follow. The TDCP not only benefits the network scheduling, but also reflects the channel variations over time. For example, the TDCP values are defined as wideband normalized correlation between two CSI-RS transmission occasions:

rτ,∆t=hτ+∆t∙h*τhτ+∆thτ

- hτ is the channel impulse response in first CSI-RS transmission occasion

- hτ+∆t is the channel impulse response in second CSI-RS transmission occasion

- ∆t is the time separation between two CSI-RS transmission occasions

- rτ,∆t is the TDCP value for a time separation ∆t

Take the Fourier transform of the above correlation function in the variable ∆t can get doppler power spectrum R(f). The first moment of the doppler power spectrum is mean excess doppler:

f=-∞∞f∙Rfdf-∞∞Rfdf

The second moment of the doppler power spectrum is rms doppler spread:

δf=-∞∞(f-f)2∙Rfdf-∞∞Rfdf

| Definition | Time domain channel property (TDCP) is defined as wideband normalized correlation between two CSI-RS transmission occasions, corresponding to CSI-RS resourece(s) from NZP-CSI-RS-ResourceSet(s) configured with higher layer parameter trs-info, that are separated by Dn symbols or slots, depending on the configuration, where Dn is the n-th delay configured value among [placeHolderForRrcParameter-D] configured delay values {D1, …, DY} and Y is number of configured delay values. The wideband normalized correlation value in TDCP is quantized in amplitude and if configured by [placeHolderForRrcParameter-phaseQuanOn], in phase, as described in Clause 5.2.1.4.5 of [6, TS38.214]. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported TDCP amplitude value shall be no lower than the minimum and no higher than the maximum measured values across the receiver branches. For frequency range 1, the reference point for the TDCP shall be the antenna connector of the UE. For frequency range 2, TDCP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. |
| --- | --- |
| Applicable for | RRC_CONNECTED |

With reported TDCP values, network can get the mean excess doppler and rms doppler spread. If TDCP values from multiple TRPs are reported, the inter-TRP frequency/phase offset can be derived based on the report. Compared to DL RSCPD discussed above, network can get whole picture on mean excess doppler and rms doppler spread, which will somehow increase the network compensation accuracy. However, with increased number of TDCP values, the overhead to report amplitude and phase of TDCP values will be significantly large.

Observation 7: With reported TDCP values, network can get the mean excess doppler and rms doppler spread. The TDCP values can benefit the inter-TRP frequency/phase offset compensation at network side.

Proposal 12: Study whether how to extend the TDCP report defined in Rel-18 to facilitate inter-TRP frequency/phase offset compensation at network side.

Regarding the inter-TRP time offset, network should get time domain information based on UE report. Similar to TDCP, we can define frequency domain channel property (FDCP), which can be normalized correlation between two frequency units in frequency domain as shown in Figure 6. For example, FDCP value can be defined as below:

Af,∆f=Hf+∆f∙H*fH(f+∆f)∙H(f)

- Hf is the channel frequency response in first frequency unit

- Hf+∆f is the channel frequency response in second frequency unit

- ∆f is the frequency separation between two frequency units

- Af,∆f is the FDCP value for a frequency separation ∆f

Figure 6. Frequency domain channel property (FDCP)

Take the Fourier transform of the above correlation function in the variable ∆f can get delay power spectrum a(τ). The first moment of the delay power spectrum is mean excess delay:

τ=-∞∞τ∙aτdτ-∞∞aτdτ

The second moment of the delay power spectrum is rms delay spread:

δτ=-∞∞(τ-τ)2∙aτdτ-∞∞aτdτ

With reported FDCP values, network can get the mean excess delay and rms delay spread. If FDCP values from multiple TRPs are reported, the inter-TRP time offset can be derived based on the report. Compared to DL RSTD discussed above, network can get whole picture on mean excess delay and rms delay spread, which will somehow increase the network compensation accuracy. However, with increased number of FDCP values, the overhead to report amplitude and phase of FDCP values will be significantly large.

Observation 8: With reported frequency domain channel property (FDCP) values, network can get the mean excess delay and rms delay spread. The FDCP values can benefit the inter-TRP time offset compensation at network side.

Proposal 13: Study whether how to define frequency domain channel property (FDCP) to facilitate inter-TRP time offset compensation at network side, where the FDCP is the normalized correlation between two frequency units in frequency domain.

- Conclusions

In this paper, some preliminary views on CSI enhancements are discussed. The following observations and proposals are made:

Observation 1: FeType II codebook for CJT supports dynamic TRP selection by UE for a CSI report.

Observation 2: In FeType II codebook for CJT, UE reports SD basis selection independently for each selected CSI-RS resource to allow spatial diversity across TRPs.

Observation 3: Type I single-panel codebook adopts two-stage precoding, where the first stage includes the same SD basis for both antenna polarizations and second stage captures the phase variations between different antenna polarizations.

Observation 4: For Type I multi-panel codebook, gNB indicates the number of panels Ng in addition to N1 and N2. Therefore, all Ng panels are placed in one dimension.

Observation 5: For Type I multi-panel codebook, the precoding matrix is divided into 2Ng sub-matrices, where each sub-matrix corresponds to a precoding matrix for an antenna polarization of a specific panel and all the sub-matrices share the same SD basis across all panels and all antenna polarizations.

Observation 6: Digital precoding and analog precoding are supported via two separate reports in current specification, i.e., CSI report and beam report.

Observation 7: With reported TDCP values, network can get the mean excess doppler and rms doppler spread. The TDCP values can benefit the inter-TRP frequency/phase offset compensation at network side.

Observation 8: With reported frequency domain channel property (FDCP) values, network can get the mean excess delay and rms delay spread. The FDCP values can benefit the inter-TRP time offset compensation at network side.

Proposal 1: For Type I codebook supporting up to 128 ports, network can configure multiple CSI-RS resources to jointly get a PMI.

Proposal 2: For Type I codebook supporting up to 128 ports, further study whether to reuse the following mechanisms defined in Rel-18 for CJT:

- Dynamic CSI-RS resource selection by UE for a CSI report.

- SD basis selection can be reported independently for each selected CSI-RS resource.

Proposal 3: For Type I codebook supporting up to 128 ports, when multiple CSI-RS resources are configured for a joint CSI report, network can inform UE about the panel layout associated with the multiple CSI-RS resources, e.g., the number of panels in first dimension and second dimension.

Proposal 4: Type I codebook is prioritized over Type II codebook for CRI-based CSI reporting.

Proposal 5: To support CRI-based CSI reporting for hybrid beamforming, network can configure UE to report multiple sets of CSI information in a single report, where each set of CSI information at least includes CRI and corresponding RI/PMI/CQI for a specific CSI-RS resource.

Proposal 6: To support CRI-based CSI reporting for hybrid beamforming, network can additionally configure UE to report RSRP for a corresponding CRI.

Proposal 7: For CRI-based CSI reporting, codebook subset restriction should be enhanced to support interference coordination in hybrid beamforming scenario.

Proposal 8: For CRI-based CSI reporting, at least reuse legacy CSI report mechanisms defined for wideband CSI information or subband CSI information.

Proposal 9: Study methods to reduce CSI report overhead for CRI-based CSI reporting.

Proposal 10: For the measurement on inter-TRP time offset, consider reusing the definition of DL reference signal time difference (DL RSTD) for NR positioning as a starting point.

Proposal 11: For the measurement on inter-TRP frequency/phase offset, consider reusing the definition of DL reference signal carrier phase difference (RSCPD) for NR positioning as a starting point.

Proposal 12: Study whether how to extend the TDCP report defined in Rel-18 to facilitate inter-TRP frequency/phase offset compensation at network side.

Proposal 13: Study whether how to define frequency domain channel property (FDCP) to facilitate inter-TRP time offset compensation at network side, where the FDCP is the normalized correlation between two frequency units in frequency domain.

Reference

- RP-234007, New WID: NR MIMO Phase 5, Samsung (Moderator).