3GPP TSG RAN WG1 #116	 R1-2400524

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.4.1.2

Source:	Honor

Title:	Discussion on Ambient IoT device architectures

Document for:	Discussion and Decision

Introduction

In RAN#102, a new Rel-19 SID “Study on solutions for Ambient IoT (Internet of Things)” has been approved with the following objectives [1]:

The objectives of the study item are the following: The following objectives are set, within the General Scope: - Evaluation assumptions - Conclude at least the following aspects of design targets left to WGs in Clause 5 (RAN design targets) of TR 38.848 [RAN1]. - Clause 5.3: Applicable maximum distance target values(s) - Clause 5.6: Refine the definition of latency suitable for use in RAN WGs - Clause 5.8: 2D distribution of devices - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations [RAN1, RAN4] - Identify basic blocks/components of possible Ambient IoT device architectures, taking into account state of the art implementations of low-power low-complexity devices which meet the RAN design target for power consumption and complexity. [RAN1] - Define link budget calculation for coverage, including whether/how to model carrier wave from node(s) inside or outside the connectivity topology. NOTE: Assessment performance of the design targets is within the study of feasibility and necessity of proposals in the following objectives, e.g. by inspection of reference implementations in the field, simulations, analytically. NOTE: strive to minimize evaluation cases in RAN1. - Study necessary and feasible solutions for Ambient IoT as prescribed in the General Scope, including decisions on which functions, procedures, etc. are needed and not needed, and ensuring at least the required functionalities in Section 6.2 of TR 38.848. Study of positioning in Rel-19 is RAN3-led, limited to functionalities which would have no, or minimal, specification impact (note: this does not imply any decision relating to WI creation). Study the feasibility and required functionalities for proximity determination (coordination with SA3 is required for privacy aspects). - RAN1-led: For the Ambient IoT DL and UL: - Frame structure, synchronization and timing, random access - Numerologies, bandwidths, and multiple access - Waveforms and modulations - Channel coding - Downlink channel/signal aspects - Uplink channel/signal aspects - Scheduling and timing relationships - Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. For Topology 2, no difference in physical layer design from Topology 1.

In this contribution, we provide input for the third sub-objective of the first objective. The purpose is to discuss possible architectures of devices that meet complexity and power consumption requirements for evaluation.

Discussion

  - Design Target

The design targets largely determine the design of the device architecture. This section will discuss the impact of the following design targets on device architecture.

    - Deployment Scenario

Deployment scenarios are related to channel conditions, interference, mobility, etc., which in turn affect the design of the device architecture. For example, RFID design mainly targets short-range scenarios of about 10m. In this scenario, the channel conditions are ideal and there is less interference in the environment. Therefore, some architectures with lower hardware complexity can be used.

It can be seen from [1] that the current deployment scenario of Ambient IoT research is the indoor scenario of cellular networks. This scenario obviously has higher requirements than RFID. Mainly reflected in the following aspects: more complex channel conditions, stronger interference in the environment, and larger coverage distance. These requirements require the device to use better RF components than RFID to meet them.

Observation 1: The complex deployment scenarios of Ambient IoT require better RF components to meet the requirements.

    - Coverage Target

The coverage design target given in SI objective is 10~50m. This is a very broad design target because every 10m coverage improvement may affect the design of the device architecture. Therefore, it is recommended to use coverage target as supplementary constraints to other design targets. For example, evaluate the coverage that the device architecture can achieve while meeting other design targets. If it can achieve a coverage range of 10 to 50 meters, this architecture meets the design requirements.

Observation 2: The coverage design target range is large, and the relationship with the receiver architecture cannot be directly determined.

    - Power Consumption Target

Power consumption requirements largely determine the device architecture. For example, the architecture that can be used for receivers and transmitters is directly related to power consumption. It can be seen from [1] that the power consumption of the device needs to meet the following requirements:

- ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally.

It can be seen from the above power consumption that the first power consumption requirement is similar to the requirement of RFID passive device. The second power consumption requirement is similar to the power consumption requirements of RFID's semi-passive and active devices. In addition, the application scenarios of Ambient IoT also have many overlaps with those of RFID. Therefore, in the next section we will discuss the device architecture of Ambient IoT with reference to the device architecture of RFID.

Observation 3: The power consumption requirements of Ambient IoT devices are similar to those of RFID device.

Proposal 1: Discuss the architecture of Ambient device with reference to the architecture of RFID device.

  - Ambient IoT Device Architectures

Referring to RFID, the device architecture mainly consists of matching network, power management, demodulation, modulation, and digital baseband components. Drawing reference from literature [2], a schematic device architecture in Figure 1 is given. The functions of each component are briefly introduced below.

Figure 1. Ambient IoT device architecture

- Matching Network

The role of the matching network is to ensure impedance matching between the signal source and load to ensure maximum energy transfer and minimum signal loss. Matching networks can improve the performance and stability of RF circuits while reducing signal interference and noise levels. Finally, an LC matching network is used to match the input impedance of the device to that of the antenna for maximum power transfer.

- Power Management

Power management mainly includes two circuits: Current recovery and power supply voltage stabilizing.

The power recovery circuit converts the ultra-high frequency signal received by the device antenna into a DC voltage through rectification, boosting, etc., to provide energy for the chip's operation. This circuit can also incorporate other energy harvesting circuits. For example, add photovoltaic and thermoelectric energy harvesting circuits.

The power supply voltage stabilizing circuit needs to meet the following requirements: when the input signal amplitude is high, it must be able to ensure that the output DC power supply voltage does not exceed the maximum voltage that the chip can withstand. When the input signal is small, the power consumed by the voltage stabilizing circuit should be as much as possible to reduce the total power consumption of the chip.

- Demodulation

In order to reduce chip area and power consumption, most passive RFID tags currently use ASK modulation. For the ASK demodulation circuit of the tag chip, the commonly used demodulation method is envelope detection. The R18 LPWUS SI topic has studied three types of basic architectures of ASK receivers, which can be used as a reference baseline [3].

- Modulation

The modulation process is the process of moving low-frequency signals to high-frequency bands. The modulation process uses the transmitted low-frequency signal to control the high-frequency oscillator, so that the parameters (amplitude, frequency, and phase) of the high-frequency oscillator output signal change with the low-frequency signal, thereby moving the low-frequency signal to the high-frequency band. Finally, the information of the low-frequency signal is carried and propagated by the high-frequency signal.

The implementation of modulation is the main point of difference between passive and active devices. Passive devices modulate their information by backscattering the carrier signal they receive from the reader. The device antenna should be able to extract energy from the incident RF signal and transmit the backscattered signal. Therefore, an active device usually contains its own internal power supply, which is used to power the device circuitry and generate the transmitted signal. The modulation module in Figure 1 can work in two modes. By switching, it can work in backscattering mode or in active mode that generates its own signal.

- Digital Baseband Parts

The main activities of the digital baseband in RFID are to decode the incoming data from the reader, to modify the memory contents and to execute the relevant commands to response to different reader interrogations. According to the RFID baseband architecture in literature [4], a reference baseband architecture for Ambient IoT is given. Active devices can modulate information onto signals they generate for transmission.

Figure 2. Device baseband architecture

As can be seen from the above functional description, each component is an essential function for the device architecture. However, under different design targets, each component has different implementation complexity or contains different sub-modules. Therefore, it is recommended to first determine the unified basic components, and then discuss the specific implementation of each component under different design targets.

Observation 4: Under different design targets, the basic components of the device architecture are basically the same, but the sub-modules and implementation complexity of each component will be different.

Proposal 2：Ambient IoT device architectures contain the following basic components: Matching network, power management, demodulation, modulation, and digital baseband.

- Conclusions

In this contribution, we discuss on Ambient IoT device architectures with the following observations and proposals.

Observation 1: The complex deployment scenarios of Ambient IoT require better RF components to meet the requirements.

Observation 2: The coverage design target range is large, and the relationship with the receiver architecture cannot be directly determined.

Observation 3: The power consumption requirements of Ambient IoT devices are similar to those of RFID device.

Observation 4: Under different design targets, the basic components of the device architecture are basically the same, but the sub-modules and implementation complexity of each component will be different.

Proposal 1: Discuss the architecture of Ambient device with reference to the architecture of RFID device.

Proposal 2：Ambient IoT device architectures contain the following basic components: Matching network, power management, demodulation, modulation, and digital baseband.

- References

- RP-234058, “Study on solutions for Ambient IoT (Internet of Things)”, RAN#102, Dec. 11 - 15, 2023.

- Peiqing Han, Niansong Mei and Zhaofeng Zhang, “A UHF Semi-Passive RFID System with Photovoltaic/Thermoelectric Energy Harvesting for Wireless Sensor Networks”, 2019 IEEE 13th International Conference on ASIC (ASICON)

- 3GPP TR 38.869

- Omar Abdelmalek, “Design and prototyping of robust architectures for UHF RFID Tags”