3GPP TSG RAN WG1 #116	 R1-2400525

Athens, Greece, February 26th – March 1st, 2023

Agenda Item:	9.5.1

Source:	Honor

Title:	Discussion on on-demand SSB SCell operation

Document for:	Discussion and Decision

Introduction

In RAN#102, a new Rel-19 WID “Enhancements of network energy savings for NR” has been approved with the following objectives [1] aim to specify further network energy saving techniques which were found to be beneficial in the study [2] and not be standardized yet in Rel-18.

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we will mainly discuss the first objective on-demand SSB SCell operation and provide some overall considerations and recommendations.

- Discussion

  - Overall considerations

On-demand SSB is an effective mechanism to reduce the transmission of SSB and thus to save the network energy. Since the transmission of SSB can be triggered by a signal, e.g. WUS, this means the SSB is only transmitted when it is required and the SSB could stay muted while no trigger signal is detected.

In the CA scenarios, there are two possible ways to trigger the on-demand SSB operation, network trigging and UE triggering. As shown in Figure 1, before adding a secondary carrier, the network may configure the UE to measure an SSB-less carrier, in which case the network triggers the SCell to send the SSB. When the UE needs to perform L1/L3 measurements or time-frequency synchronization on an SSB-less carrier, as shown Figure 2 and 3, the UE needs to trigger the SCell to send the SSB. Therefore, both Network triggered and UE triggered on-demand SSB SCell operation should be supported.

Observation 1: Both Network triggered and UE triggered scenarios exist in the CA case.

Proposal 1: Support both Network triggered and UE triggered on-demand SSB SCell operation.

  - Network triggered on-demand SSB SCell operation

For network triggered on-demand SSB SCell operation, such as a PCell configuring a UE to measure a secondary carrier, or after a SCell is blindly added but not activated, an indication via backhaul can be used as the WUS. It is therefore the job of RAN3 to further design the signaling. For RAN1, an indication to the UE can be considered after the network sends the WUS to trigger the SCell to send an SSB.

Figure 1. Illustration of a network triggered on-demand SSB operation

Observation 2: It is for RAN3 to design the indication via backhaul which can be used as the WUS for network triggered on-demand SSB SCell operation.

Proposal 2: RAN1 considers to design an indication to UE after network triggered on-demand SSB operation.

  - UE triggered on-demand SSB SCell operation

There are two possible ways for a UE to trigger the SCell to send SSB. The first is that the UE sends an on-demand request to the PCell and the PCell sends a WUS to the SCell as shown in Figure 2, especially before the SCell is activated. In this case, the PCell sends the WUS mentioned in the previous section can be reused. Regarding the on-demand request sent by the UE to the PCell, the UCI, e.g. the newly defined SR, is preferred and the PCell can configure the PUCCH resources it uses which both common and dedicated resources should be considered.

Observation 3: One way for UE triggered on-demand SSB SCell operation is to send the WUS via PCell.

Proposal 3: UCI, such as new SR can be used as on-demand request sent to the PCell.

UE directly sending WUS to the SCell is another possible way for UE triggered on-demand SSB SCell operation. As shown in Figure 3, considering SCell may not be configured with PUCCH, a dedicated PRACH similar to the on-demand SI supported by current specification or SRS can be used as the WUS.

Observation 4: UE directly sending WUS to the SCell is another possible way for UE triggered on-demand SSB SCell operation.

Proposal 4: Dedicated PRACH or SRS can be considered as the WUS sent to the SCell.

Figure 2. Illustration of a UE triggered on-demand SSB operation via PCell

Figure 3. Illustration of a UE directly triggered on-demand SSB operation

  - Other enhancement

In addition to the triggering methods and signaling discussed above, the following enhancements can also be considered. The first is whether to support extra info in the WUS signaling, such as on-demand cause values and beam indications, which can be added to the backhaul indication when using network triggering method, or different resources can be defined when using UE triggering method. The second enhancement is that, to further decrease the energy consumption, the network may not transmit a SIB1 after an on-demand SSB, in which case, it should be specified how to indicate whether or not a SIB1 exists after the SSB.

Observation 5: Consider the following enhancement, extra info in the WUS signaling, e.g. on-demand cause values and beam indications, and indication the presence of SIB1 after on-demand SSB.

- Conclusions

In this contribution, we discuss the on-demand SSB SCell operation and provide some overall considerations and recommendations. The following observations and proposals are given:

Observation 1: Both Network triggered and UE triggered scenarios exist in the CA case.

Observation 2: It is for RAN3 to design the indication via backhaul which can be used as the WUS for network triggered on-demand SSB SCell operation.

Observation 3: One way for UE triggered on-demand SSB SCell operation is to send the WUS via PCell.

Observation 4: UE directly sending WUS to the SCell is another possible way for UE triggered on-demand SSB SCell operation.

Observation 5: Consider the following enhancement, extra info in the WUS signaling, e.g. on-demand cause values and beam indications, and indication the presence of SIB1 after on-demand SSB.

Proposal 1: Support both Network triggered and UE triggered on-demand SSB SCell operation.

Proposal 2: RAN1 considers to design an indication to UE after network triggered on-demand SSB operation.

Proposal 3: UCI, such as new SR can be used as on-demand request sent to the PCell.

Proposal 4: Dedicated PRACH or SRS can be considered as the WUS sent to the SCell.

# References

- RP-234065, “Enhancements of network energy savings for NR”, RAN#102, Dec. 11 - 15, 2023.

- 3GPP TR 38.864 V18.1.0, Study on network energy savings for NR.