3GPP TSG RAN WG1 #116	 R1-2400526

Athens, Greece, February 26th – March 1st, 2024

Agenda Item:	9.5.3

Source:	Honor

Title:	Discussion on adaptation of common signal/channel transmission

Document for:	Discussion and Decision

Introduction

In RAN#102, a new Rel-19 WID “Enhancements of network energy savings for NR” has been approved with the following objectives [1]:

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we will mainly discuss the third objective. For each sub-topic, we will mainly discuss the necessity of research and possible solutions.

Discussion

  - Adaptation of SSB in time domain

NR SSB mainly has two characteristics in the time domain: one is that beam sweeping is required, as shown in Figure 1; the other is that for the initial access UE, the default period is 20ms. The time domain characteristics of SSB prevent the network side from shutting down and saving power on these symbols even when the network load is light. Therefore, the power consumption of the NR base station is high.

Figure 1. Schematic diagram of beam sweeping pattern

Obtaining energy-saving gains by adjusting SSB in the time domain is essentially reducing the number of SSB transmission symbols. However, since SSB is related to initial access, adjusting SSB is likely to affect the access of legacy users. Therefore, it is recommended to avoid introducing new SSB patterns.

Observation 1: Simply adjusting SSB in the time domain will affect the access of legacy users.

Proposal 1: New SSB pattern is not supported.

In actual network deployment, there are large overlaps between cells, especially in some dense urban areas. As shown in Figure 2 below, assuming that there is a 50% overlapping area between cells (depending on the setting of the overlap area threshold), then Cell 2 and Cell 4 are almost covered by the overlapping area with other cells. When the network is busy, dense deployment of the network can improve network capacity and user experience. However, during idle time, if all cells are still in working state, the energy consumption of the network will become a waste.

Observation 2: Dense networking wastes more energy when the load is light.

Observation 3: In dense networking, the overlapping areas between cells are larger.

Combined with the large overlapping area of dense networking, the SSB period of cells covered by the overlapping area can be lengthened to reduce network energy consumption when the load is light. For example, the SSB beams or part of the SSB beams in Cell 2 and Cell 4 cells can use a longer period, while Cell1/3/5 still uses the 20ms SSB period. This not only does not affect the access of Cell 2/4 legacy users, but also improves the energy saving effect.

Proposal 2: Support SSB time domain period adaptation without affecting legacy users.

Figure 2. SSB time domain period adaptation

  - Adaptation of PRACH

Since the base station side needs to blindly detect all configured PRACH resources, reasonable configuration of PRACH resources becomes particularly critical. The adaptation of existing PRACH resources is configured through SIB1, so it is difficult to quickly match load changes. This section will discuss how to reasonably configure PRACH resources from both the perspectives of the time domain and spatial domain to reduce the energy consumption of base station reception.

    - Adaptation of PRACH in time domain

The time domain resources of PRACH are configured through the PRACH Configuration Index parameter in the SIB1 message. The table corresponding to this parameter will provide detailed PRACH time domain resource information, including period, slot position, symbol position, and other information. Therefore, the update of time domain resources needs to be implemented through system message update, which is relatively complicated for both the network and the UE.

Observation 4: PRACH time domain resource update needs to be implemented through system message update, which consumes relatively high energy on both the network side and the UE side.

PRACH resources include contention-based access resources and contention-free access resources. Each type of resource can be used in multiple scenarios. For example, contention-based PRACH resources can be used for initial access, SR failure process, etc. Contention-free PRACH resources can be used for processes such as handover and reconfiguration requests. Therefore, the update of PRACH resources needs to consider users in RRC_IDLE, RRC-INACTIVE, and RRC_ACTIVE states respectively, and also needs to consider the impact on legacy users.

Observation 5: The update of PRACH time domain resources needs to be notified to all UEs in any possible RRC state and the impact on existing legacy users needs to be considered.

The PRACH time domain configuration table in the current specification is in Section 6.3.3.2 of 38.211[2], as shown in Figure 2. The table already contains time domain formats under various periods, so this information can be reused for time domain indication. But the table contains information in PRACH format, so the table has many rows.

Observation 6: The existing time domain table has many rows, which consumes a lot of resources if using other lightweight indication methods.

Figure 2. PRACH time domain configuration table

A simple way to implement PRACH time domain adaptation is to provide separate instructions to legacy users and users who support the new PRACH functions. Legacy users still use SIB1 for PRACH time domain resource indication, and new users can use a lightweight method for indication.

Proposal 3: Separately indicate the PRACH time domain resource information for legacy users and new users.

Proposal 4: Adopt lightweight time domain resource indication method for new users, e.g., Group common DCI.

    - Study adaptation of PRACH in spatial domain

In NR, the PRACH resources for initial access need to bear the function of indicating the best SSB beam selected by the UE. Therefore, the configured PRACH resources are increased by several times, as shown in Figure 3 below.

Figure 3. Correspondence between initial access PRACH resources and SSB beams

Observation 7: The need to indicate SSB beams causes PRACH resources to increase by several times.

In the operator network, the actual distribution of users is not uniform, especially when the load is light. Therefore, network energy consumption can be saved by turning off certain beams without user distribution. The corresponding PRACH resources of these beams do not need to be configured, and the network side can also reduce the detection of PRACH resources to improve energy-saving gains.

Observation 8: Turning off beams without user distribution can not only reduce downlink network energy consumption, but also reduce uplink energy consumption.

The resource allocation corresponding to PRACH and SSB beams itself is allocated first in the frequency domain and then in the time domain. A simple way to implement non-uniform PRACH resource allocation is to still refer to the existing uniform allocation method. However, in order to increase the deep sleep time of the base station, resources need to be reallocated according to the reduced number of beams to achieve the purpose of concentrating PRACH resources. For example, Figure 4 shows the resource allocation results when the number of SSB beams is reduced to 4. From the perspective of energy saving effect, it is actually similar to the PRACH period adjustment scheme, both of which reduce the energy consumption of the base station by receiving less PRACH resources in the time domain. However, from the perspective of protocol changes and implementation complexity, the impact of non-uniform distribution is greater, so it is recommended to lower the priority.

Observation 9: The adaptation of PRACH in the spatial domain is essentially similar to the adaptation in the time domain, and the standard impact and implementation are more complex.

Proposal 5: Deprioritize the study of adaptation of PRACH in spatial domain.

Figure 4. Non-uniform PRACH resource allocation method

  - Adaptation of paging occasions

According to the current specification 38.304[3], the formula of paging under DRX is as follows:

The PF and PO for paging are determined by the following formula:

SFN for the PF is determined by:

(SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

Index (i_s), indicating the index of the PO is determined by:

i_s = floor (UE_ID/N) mod Ns

The purpose of this formula is to evenly distribute UEs with different UE_IDs among N PFs, and the N PFs are also evenly distributed within T. This will cause the base station to wake up from sleep at any time to send paging messages, and thus may not be able to enter deep sleep.

Observation 10: The current uniform configuration of paging messages means that the base station may not be able to enter deep sleep and needs to be ready to send paging messages at any time.

The direct solution is to centralize paging messages when the load is light to increase the time the base station spends in deep sleep, thereby reducing energy consumption. As can be seen from Figures 5 and 6, overall, the delay of uniform distribution is better than concentrated distribution. However, from a single user perspective, there is no essential difference in latency. Because, no matter what the situation is, as long as the UE misses the PF frame in this cycle, it needs to wait until the PF frame in the next cycle before it can be paged. Therefore, it is recommended to support centralized distributed paging frames.

Figure 5. Uniformly distributed paging frames

Figure 6. Concentrated distributed paging frames

Proposal 6: Support centralized sending of paging messages.

- Conclusions

In this contribution, we discuss on adaptation of common signal/channel transmission with the following observations and proposals.

Observation 1: Simply adjusting SSB in the time domain will affect the access of legacy users.

Observation 2: Dense networking wastes more energy when the load is light.

Observation 3: In dense networking, the overlapping areas between cells are larger.

Observation 4: PRACH time domain resource update needs to be implemented through system message update, which consumes relatively high energy on both the network side and the UE side.

Observation 5: The update of PRACH time domain resources needs to be notified to all UEs in any possible RRC state and the impact on existing legacy users needs to be considered.

Observation 6: The existing time domain table has many rows, which consumes a lot of resources if using other lightweight indication methods.

Observation 7: The need to indicate SSB beams causes PRACH resources to increase by several times.

Observation 8: Turning off beams without user distribution can not only reduce downlink network energy consumption, but also reduce uplink energy consumption.

Observation 9: The adaptation of PRACH in the spatial domain is essentially similar to the adaptation in the time domain, and the standard impact and implementation are more complex.

Observation 10: The current uniform configuration of paging messages means that the base station may not be able to enter deep sleep and needs to be ready to send paging messages at any time.

Proposal 1: New SSB pattern is not supported.

Proposal 2: Support SSB time domain period adaptation without affecting legacy users.

Proposal 3: Separately indicate the PRACH time domain resource information for legacy users and new users.

Proposal 4: Adopt lightweight time domain resource indication method for new users, e.g., Group common DCI.

Proposal 5: Deprioritize the study of adaptation of PRACH in spatial domain.

Proposal 6: Support centralized sending of paging messages.

References

- RP-234065, “Enhancements of network energy savings for NR”, RAN#102, Dec. 11 - 15, 2023.

- 3GPP TS 38.211

- 3GPP TS 38.304